Research & Vehicle Technology

“Infotainment Systems Product Development”

Feature – Multi-Camera Client

APIM Infotainment Subsystem Part Specific Specification (SPSS)

Version 1.11

UNCONTROLLED COPY IF PRINTED

Version Date: October 2, 2019

FORD CONFIDENTIALF

# Revision History

| Date | Version | Notes |  |
| --- | --- | --- | --- |
| May 31, 2013 | 1.0 | Initial Release |  |
| January 16, 2015 | 1.1 | SPSS Updated to add soft button view implementation |  |
|  | RVCv2-IR-REQ-014080/B-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3) |  | rpaquet2 - Updated encodings in table |
|  | MD-REQ-014070/A-Feature_St (TcSE ROIN-282399-2) |  | sorris1: Updated the Personalization Index parameter encoding to match the CAN database. |
|  | DAFVCv1-IR-REQ-014066/B-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061) |  | rpaquet2 - Updated encodings |
|  | MD-REQ-014070/A-Feature_St (TcSE ROIN-282399-2) |  | sorris1: Updated the Personalization Index parameter encoding to match the CAN database. |
|  | STR-052389/B-Rear View Camera (TcSE ROIN-293312-1) |  | rpaquet2 - Added two new variants Camera image View v3 and Rear View Camera Zoom Settings v3 to cover the new way of zooming and soft button inputs. |
|  | RVC-FUR-REQ-014088/B-Deactivate RVC (TcSE ROIN-293328) |  | rpaquet2 - Added new text to clarify Forward gear and Park no prak for Manual transmission applications. |
|  | CAMERA-FUR-REQ-014093/B-Camera Image Priority (TcSE ROIN-264652-1) |  | rpaquet2 - Updated requirement to work for all camera views. no change to requirement intent. |
|  | RVCv3-UC-REQ-127872/A-Activate Rear Multicamera View |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128191/A-Enter CHMSL Delay Mode |  | rpaquet2 - New use case |
|  | TRG-UC-REQ-102959/A-Vehicle Not in RUN/START |  | rpaquet2 - Added use case to multi-camera APIM SPSS |
|  | TRG-UC-REQ-102960/A-Loss of communication with IPMB Module |  | rpaquet2 - Added use case to multi-camera APIM SPSS |
|  | TRG-UC-REQ-102961/A-Valid Camera Video Signal not present |  | rpaquet2 - Added use case to multi-camera APIM SPSS |
|  | RVCv3-UC-REQ-128173/A-Press Zoom Button from Rear 360 View |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128176/A-Press Zoom Button from Rear Normal View |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128179/A-Press Zoom Button from CHMSL View |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128188/A-Press Rear 360 Unzoom |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128189/A-Press Rear Normal Unzoom |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128190/A-Press CHMSL Unzoom |  | rpaquet2 - New use case |
|  | RVCv3-ACT-REQ-127095/A-Manual Zoom |  | rpaquet2: New Requirement |
|  | RVCv3-UC-REQ-127874/A-Press Rear 360 View Button |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128175/A-Press RVC Normal View Button |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128177/A-Press RVC Split View Button |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128178/A-Press CHMSL Camera View Button |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128180/A-Press Aux Camera view Button |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128181/A-Press Trailer Reverse Guidance Button |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128396/A-Press Hard Button to Enter CHMSL View |  | rpaquet2 - New Use Case for CHMSL |
|  | RVCv3-UC-REQ-128397/A-Press Hard Button to Enter RVC View |  | rpaquet2 - New use case |
|  | RVCv3-ACT-REQ-127096/A-Change Camera View Setting |  | rpaquet2: Diagram updated to reflect the change in HMI to go back to soft buttons |
|  | STR-052463/B-Driver Assist Front View Camera (TcSE ROIN-293325) |  | rpaquet2 - Added New function variant 2 for Driver Assist Front Camera Image View to capture the soft button implementation. |
|  | DAFVCv2-UC-REQ-128182/A-Entering Front Camera - 360 |  | rpaquet2 - New use case |
|  | DAFVCv2-UC-REQ-128184/A-Entering Front Camera – Non-360 |  | rpaquet2 - New use case |
|  | TRG-UC-REQ-102959/A-Vehicle Not in RUN/START |  | rpaquet2 - Added use case to multi-camera APIM SPSS |
|  | TRG-UC-REQ-102960/A-Loss of communication with IPMB Module |  | rpaquet2 - Added use case to multi-camera APIM SPSS |
|  | TRG-UC-REQ-102961/A-Valid Camera Video Signal not present |  | rpaquet2 - Added use case to multi-camera APIM SPSS |
|  | DAFVCv2-UC-REQ-128183/A-Press Front 360 View Button |  | rpaquet2 - New use case |
|  | DAFVCv2-UC-REQ-128185/A-Press Front Normal View Button |  | rpaquet2 - New use case |
|  | DAFVCv2-UC-REQ-128186/A-Press Front Split View |  | rpaquet2 - New use case |
|  | DAFVCv2-UC-REQ-128187/A-Press Rear button From Front camera |  | rpaquet2 - New use case |
|  | DAFVCv2-ACT-REQ-127100/A-Change Camera View Setting |  | rpaquet2: Diagram updated to reflect use of soft button implementation. |
| June 15, 2015 | 1.2 | Updates to Merge Off Road FVC into Multi-Camera and update Max Speed requirements |  |
|  | DAFVCv1-IIR-REQ-014069/B-DriverAssistFrontViewCameraClient_Rx (TcSE ROIN-282847-2) |  | rpaquet2 - Added Elocker_St, AwdRange_St and OffRoadMode_St in order to merge FVC Offroad Camera with MultiCamera. |
|  | STR-052387/B-General Requirements (TcSE ROIN-293327-1) |  | rpaquet2 - Added Determine Off Road Mode and DAFVC Malfunction and T_cameraMalfunctionDelay. And moved RVC Malfunction to this folder. |
|  | CAMERA-REQ-014077/B-Feature Maximum Speed (TcSE ROIN-290556) |  | wstephe1: Updated requirement for Max Speed per feature scenario types: camera activation (any feature), RVC active, and DAFVC active by configuration of front (off road) camera. Scenarios to continue application across camera features. |
|  | DAFVCv1-FUR-REQ-166648/A-Determine Off Road Mode |  | rpaquet2 - New requirement created during merge of Offroad and multi camera specs. |
|  | RVC-FUR-REQ-014087/B-RVC Malfunction (TcSE ROIN-146656-2) |  | rpaquet2 - Updated requirement per APIM team. |
|  | DAFVCv1-FUR-REQ-166722/A-DAFVC Malfunction |  | rpaquet2 - Added new requirement per APIM team. |
|  | RVC-TMR-REQ-166649/A-T_cameraMalfunctionDelay |  | rpaquet2 - Added new timer requirement for delay. |
|  | STR-052390/B-Rear View Camera General Requirements (TcSE ROIN-293310-1) |  | rpaquet2 - Moved RVC Malfunction from Rear View Camera general requirements folder to the general requirements folder. |
|  | RVC-FUR-REQ-014088/C-Deactivate RVC (TcSE ROIN-293328) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | RVC-UC-REQ-014099/B-Rear Camera Delay Mode is On (TcSE ROIN-289798) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | RVC-UC-REQ-014100/B-Active Park Assist is Active (TcSE ROIN-290554) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | RVC-UC-REQ-014101/B-Trailer Backup Assist is Active (TcSE ROIN-290555) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | RVC-UC-REQ-014099/B-Rear Camera Delay Mode is On (TcSE ROIN-289798) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | RVC-UC-REQ-014100/B-Active Park Assist is Active (TcSE ROIN-290554) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | RVC-UC-REQ-014101/B-Trailer Backup Assist is Active (TcSE ROIN-290555) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | DAFVCv1-FUN-REQ-014045/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-293385) |  | rpaquet2 - Added requirement folder with the off road requirement due to merging Offroad and Multicamera SPSS. |
|  | DAFVCv1-FUR-REQ-166641/A-Deactivate FVC |  | rpaquet2 - New requirement number created during merge of Offroad and multi camera specs due to signal change. |
|  | DAFVCv1-FUR-REQ-013997/B-E-Locker Deactivation Delay (TcSE ROIN-266607-1) |  | rpaquet2 - Changed FID to DAFVC due to merge of Offroad and Multicamera spec. |
|  | DAFVCv1-TMR-REQ-013998/B-T_eLockerDelay (TcSE ROIN-266609-1) |  | rpaquet2 - Changed FID to DAFVC due to merge of Offroad and Multicamera spec. |
|  | DAFVCv1-UC-REQ-014048/B-Activation Attempt During Overspeed Condition (TcSE ROIN-290145) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 rpaquet2 - Updated the precondition and Scenario description to combine the Offroad use case for deactivate with that from Multi Camera. |
|  | DAFVCv1-ACT-REQ-014043/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-282606-3) |  | rpaquet2 - Added note to diagram to state check Feature Maximum Speed requirement for exit speed value. |
|  | DAFVCv1-SD-REQ-014050/B-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2) |  | rpaquet2 - Update diagram to add note to check feature Maximum Speed requirement for exit speed value. |
|  | DAFVCv2-FUN-REQ-128309/B-Activate/Deactivate Driver Assist Front View Camera v2 |  | rpaquet2 - Added requirement folder with the off road requirement due to merging Offroad and Multicamera SPSS. |
|  | STR-246672/A-Requirements |  |  |
|  | DAFVCv1-FUR-REQ-166641/A-Deactivate FVC |  | rpaquet2 - New requirement number created during merge of Offroad and multi camera specs due to signal change. |
|  | DAFVCv1-FUR-REQ-013997/B-E-Locker Deactivation Delay (TcSE ROIN-266607-1) |  | rpaquet2 - Changed FID to DAFVC due to merge of Offroad and Multicamera spec. |
|  | DAFVCv1-TMR-REQ-013998/B-T_eLockerDelay (TcSE ROIN-266609-1) |  | rpaquet2 - Changed FID to DAFVC due to merge of Offroad and Multicamera spec. |
|  | DAFVCv2-UC-REQ-128182/B-Entering Front Camera - 360 |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | DAFVCv2-UC-REQ-128184/B-Entering Front Camera – Non-360 |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 rpaquet2 - Updated the precondition and Scenario description to combine the Offroad use case for deactivate with that from Multi Camera. |
|  | DAFVCv1-ACT-REQ-014043/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-282606-3) |  | rpaquet2 - Added note to diagram to state check Feature Maximum Speed requirement for exit speed value. |
|  | DAFVCv1-SD-REQ-014050/B-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2) |  | rpaquet2 - Update diagram to add note to check feature Maximum Speed requirement for exit speed value. |
| September 16, 2015 | 1.3 | Updates for Off Road FVC Activation and Max Speed |  |
|  | CAMERA-REQ-014077/C-Feature Maximum Speed (TcSE ROIN-290556) |  | tmertiri-Updated requirement to account for off road changes. |
|  | DAFVCv1-UC-REQ-014046/B-Activate Driver Assist Front View Camera (TcSE ROIN-290143) |  | tmertiri-Updated uses case to add Off Road Front Camera to the vehicle system indicated indications (via IPMB sending hard button press) |
|  | DAFVCv1-SD-REQ-014050/C-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2) |  | tmertiri-removed scenario of pre and post condition from sequence diagram requirement. Already defined in the use cases. |
|  | DAFVCv1-SD-REQ-014050/C-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2) |  | tmertiri-removed scenario of pre and post condition from sequence diagram requirement. Already defined in the use cases. |
| October 19, 2016 | 1.4 |  |  |
|  | RVC-FUR-REQ-014088/D-Deactivate RVC (TcSE ROIN-293328)+ |  | tmertiri: updated Reverse can signal name |
|  | RVC-FUR-REQ-014090/B-Display RVC Video (TcSE ROIN-194462-2)+ |  | tmertiri: replaces old signal name to new one. GearRvrse_D_Actl. |
|  | STR-052391/B-Use Cases (TcSE ROIN-293353) |  | tmertiri: updated use case |
|  | UC-REQ-238578/A-CHMSL Activation. Soft button press |  | tmertiri: updated use case |
|  | UC-REQ-238579/A-AUX Activation. Soft button press |  | tmertiri: updated use case |
|  | RVCv3-UC-REQ-128180/B-Press Aux Camera view Button |  | tmertiri: updated use case |
|  | RVCv3-UC-REQ-128396/B-Press Hard Button to Enter CHMSL View |  | tmertiri: updated use case |
| January 19, 2018 | 1.5 |  |  |
|  | RVC-REQ-292389/A-GearRvrse_D_Actl |  | tmertiri: Added new signal name |
|  | RVC-REQ-292387/A-GearPos_D_Trg |  | tmertiri: Added new signal name |
|  | RVC-REQ-292388/A-Veh_V_ActlEng |  | tmertiri: Added new signal name |
|  | RVC-FUR-REQ-014090/D-Display RVC Video (TcSE ROIN-194462-2) |  | tmertiri: updated with new signal names |
|  | RVCv2-SD-REQ-014104/B-Activate Rear View Camera (TcSE ROIN-282316-2) |  | Updated SD with new signals |
|  | RVCv2-SD-REQ-014105/B-Deactivate Rear View Camera (TcSE ROIN-282323-2) |  | Updated SD with new signals |
| February 1, 2018 | 1.6 |  |  |
|  | RVC-FUR-REQ-014090/E-Display RVC Video (TcSE ROIN-194462-2) |  | tmertiri: Update wording |
| July 27, 2018 | 1.7 |  |  |
|  | RVC-FUR-REQ-014090/F-Display RVC Video (TcSE ROIN-194462-2) |  | tm: Remove DE values details. |
| November 7, 2018 | 1.8 |  |  |
|  | RVCv2-IR-REQ-014080/C-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3) |  | tmertiri: added additonal camera views |
|  | MD-REQ-014071/B-CameraServerButton_St (TcSE ROIN-287063-1) |  | tmertiri: added signal name |
|  | DAFVCv1-IR-REQ-014066/C-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061) |  | tmertiri:Update encoding table |
|  | DAFVCv1-IIR-REQ-014067/B-DriverAsssitFrontViewCameraClient_Tx (TcSE ROIN-282846-1) |  | tmertiri: Added new signals |
|  | MD-REQ-331342/A-CamraFrntBttn_D_Stat3 |  | tmertiri: New signal added |
|  | DAFVCv1-IIR-REQ-014069/C-DriverAssistFrontViewCameraClient_Rx (TcSE ROIN-282847-2) |  | tmertiri: added LIN details |
|  | MD-REQ-331343/A-Camera Button LIN signal |  | tmertiri: new requirement added |
|  | MD-REQ-331847/A-CamraFrntOffRd_B_Stat |  | tmertiri: New signal |
|  | STR-052387/C-General Requirements (TcSE ROIN-293327-1) |  | tmertiri: Added new requirement. |
|  | REQ-331344/A-Button Press Logic |  | tmertiri: new requirement |
|  | DAFVCv1-FUR-REQ-166648/B-Determine Off Road Mode |  | tmertiri:Update requirement |
|  | REQ-331863/A-Lin Button Client Operation |  | tmertiri: New diagram |
|  | STR-052463/C-Driver Assist Front View Camera (TcSE ROIN-293325) |  | tmertiri:Modified diagrams for LIN button interface |
|  | STR-052307/B-Sequence Diagrams (TcSE ROIN-293389) |  | tmertiri: added new diagram |
|  | REQ-331856/A-Lin Button Camera Request |  | tmertiri: new diagram |
| December 18, 2018 | 1.9 |  |  |
|  | RVCv2-IR-REQ-014080/D-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3) |  | tmertiri: added additional camera views |
|  | MD-REQ-014084/B-ParkBrake_St (TcSE ROIN-287064-1) |  | tmertiri:no content change |
|  | MD-REQ-014085/B-ElectronicParkBrake_St (TcSE ROIN-287065-1) |  | tmertiri: add clarification verbiage |
|  | DAFVCv1-IR-REQ-014066/D-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061) |  | tmertiri: Update table |
|  | DAFVCv1-IIR-REQ-014067/C-DriverAssistFrontViewCameraClient_Tx (TcSE ROIN-282846-1) |  | tmertiri: fix typo in title |
|  | STR-052388/B-Functional Definition (TcSE ROIN-293326-1) |  | tmertiri: added offset view |
|  | REQ-331856/B-Lin Button Camera Request |  | tmertiri: verbiage correction in the boxlines |
|  | CAMERA-FUN-REQ-331348/A-Offset View |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331349/A-Press Rear Offset View Button from Rear 360 |  | tmertiri: new usecases Dec 14 |
|  | CAMERA-UC-REQ-331350/A-Press Rear Offset View Button from Front 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331351/A-Press Rear Left Corner View Button from Rear 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331352/A-Press Rear Left Corner View Button from Front 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331353/A-Press Rear Right Corner View Button from Rear 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331354/A-Press Rear Right Corner View Button from Front 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331356/A-Press Front Offset View Button from Rear 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331360/A-Press Front Offset View Button from Front 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331365/A-Press Front Left Corner View Button from Rear 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331369/A-Press Front Left Corner View Button from Front 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331373/A-Press Front Right Corner View Button from Rear 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331374/A-Press Front Right Corner View Button from Front 360 |  | tmertiri: new usecases |
| April 26, 2019 | 1.10 |  |  |
|  | RVCv2-IR-REQ-014080/E-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3) |  | tmertiri: added rock crawl views |
|  | RVC-MD-REQ-292389/B-GearRvrse_D_Actl |  | tmertiri: clarified what to consider as reverse |
|  | DAFVCv1-IR-REQ-014066/E-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061) |  | tmertiri: added rock crawl views |
|  | RVCv2-IIR-REQ-014086/C-VehicleInformation_Tx (TcSE ROIN-282400-2) |  | tmertiri: Structural change. Added to conent in this structure. |
|  | MD-REQ-347573/A-PowerMode |  | tmertiri: added for clarification. needed only in CGEA 1.2 |
|  | RVC-FUR-REQ-014090/G-Display RVC Video (TcSE ROIN-194462-2) |  | tmertiri: update the GeaRvrse_D_Actl New Strategy |
| October 2, 2019 | 1.11 |  |  |
|  | RVCv2-IR-REQ-014080/F-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3) |  | tmertiri: added Views at Speed Configs |
|  | DAFVCv1-IR-REQ-014066/F-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061) |  | tmertiri: adding Views At Speed |
|  | MD-REQ-331342/B-CamraFrntBttn_D_Stat3 |  | tmertiri: updating requirement. Adding Soft Press origins |
|  | CAMERA-REQ-331344/B-Button Press Logic |  | tmertiri: no content changes |
|  | CAMERA-REQ-331863/B-Lin Button Client Operation |  | tmertiri: no content change |
|  | STR-052388/C-Functional Definition (TcSE ROIN-293326-1) |  | tmertiri:revised to allow views at speed function |
|  | STR-669257/A-Views At Speed |  | tmertiri: new section |
|  | CAMERA-FUN-REQ-354982/A-Views At Speed |  | tmertiri: new section |
|  | STR-669258/A-Requirements |  | tmertiri: new section |
|  | CAMERA-REQ-354992/A-Views At Speed Availability |  | tmertiri: new req |
|  | CAMERA-REQ-354993/A-Views At Speed List |  | tmertiri: new req |
|  | CAMERA-REQ-358450/A-Off Road Mode Views At Speed |  | tmertiri: new req |
|  | STR-669259/A-Use Cases |  | tmertiri: new section |
|  | CAMERA-UC-REQ-355016/A-Views At Speed Trigger |  | tmertiri: new usecase |
|  | STR-669260/A-White Box Views |  | tmertiri: new section |
|  | STR-673941/A-Activity Diagram |  | tmertiri: new diagram |
|  | CAMERA-ACT-REQ-358468/A-Views At Speed |  | tmertiri: new diagram |
|  | STR-669262/A-Sequence Diagram |  | tmertiri: new section |
|  | CAMERA-SD-REQ-354994/A-Views At Speed |  | tmertiri: new SD |

Table of Contents

Revision History	2

1	Architectural Design	9

1.1	RVC-CLD-REQ-014079/A-Rear View Camera Client (TcSE ROIN-282382-1)	9

1.2	DAFVCv1-CLD-REQ-014064/A-Driver Assist Front View Camera Client (TcSE ROIN-282843-1)	9

1.3	RearViewCameraClient Interface	9

1.3.1	RVCv2-IR-REQ-014080/F-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3)	9

1.3.2	RVCv2-IIR-REQ-014081/A-RearViewCameraClient_Tx (TcSE ROIN-282395-3)	10

1.3.3	RVCv2-IIR-REQ-014082/B-RearViewCameraClient_Rx (TcSE ROIN-282396-3)	11

1.4	DriverAssistFrontViewCameraClient Interface	14

1.4.1	DAFVCv1-IR-REQ-014066/F-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061)	14

1.4.2	DAFVCv1-IIR-REQ-014067/C-DriverAssistFrontViewCameraClient_Tx (TcSE ROIN-282846-1)	16

1.4.3	DAFVCv1-IIR-REQ-014069/C-DriverAssistFrontViewCameraClient_Rx (TcSE ROIN-282847-2)	17

1.5	VehicleInformation Interface	20

1.5.1	RVCv2-IIR-REQ-014086/C-VehicleInformation_Tx (TcSE ROIN-282400-2)	20

2	General Requirements	24

2.1	REQ-331344/A-Button Press Logic+	24

2.2	CAMERA-REQ-014077/C-Feature Maximum Speed (TcSE ROIN-290556)	24

2.3	DAFVCv1-FUR-REQ-166648/B-Determine Off Road Mode	25

2.4	RVC-FUR-REQ-014087/B-RVC Malfunction (TcSE ROIN-146656-2)	25

2.5	DAFVCv1-FUR-REQ-166722/A-DAFVC Malfunction	25

2.6	RVC-TMR-REQ-166649/A-T_cameraMalfunctionDelay	25

2.7	REQ-331863/A-Lin Button Client Operation+	26

3	Functional Definition	27

3.1	Rear View Camera	27

3.1.1	Rear View Camera General Requirements	27

3.1.2	RVCv2-FUN-REQ-014094/A-Activate/Deactivate Rear View Camera (TcSE ROIN-293352)	29

3.1.3	RVCv3-FUN-REQ-128277/A-Activate/Deactivate Rear View Camera v3	38

3.1.4	RVCv2-FUN-REQ-014106/A-Rear View Camera Zoom Setting (TcSE ROIN-293363)	47

3.1.5	RVCv3-FUN-REQ-127106/A-Rear View Camera Zoom Setting v3	50

3.1.6	RVCv2-FUN-REQ-014111/A-Rear View Camera Delay Mode Setting (TcSE ROIN-293368)	54

3.1.7	RVCv2-FUN-REQ-014113/A-Camera Image View (TcSE ROIN-293370)	54

3.1.8	RVCv3-FUN-REQ-127111/A-Camera Image View v3	57

3.1.9	RVCv2-FUN-REQ-014119/A-Activate/Deactivate Enhanced Park Aids (TcSE ROIN-293375)	63

3.1.10	RVCv2-FUN-REQ-014124/A-Camera Overlay Setting (TcSE ROIN-293357)	64

3.2	Driver Assist Front View Camera	69

3.2.1	DAFVCv1-FUN-REQ-014045/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-293385)	69

3.2.2	DAFVCv2-FUN-REQ-128309/B-Activate/Deactivate Driver Assist Front View Camera v2	75

3.2.3	DAFVCv1-FUN-REQ-014052/A-Driver Assist Front Camera Image View (TcSE ROIN-293390)	82

3.2.4	DAFVCv2-FUN-REQ-127102/A-Driver Assist Front Camera Image View v2	86

3.3	CAMERA-FUN-REQ-331348/A-Offset View	91

3.3.1	CAMERA-UC-REQ-331349/A-Press Rear Offset View Button from Rear 360	91

3.3.2	CAMERA-UC-REQ-331350/A-Press Rear Offset View Button from Front 360	91

3.3.3	CAMERA-UC-REQ-331351/A-Press Rear Left Corner View Button from Rear 360	91

3.3.4	CAMERA-UC-REQ-331352/A-Press Rear Left Corner View Button from Front 360	91

3.3.5	CAMERA-UC-REQ-331353/A-Press Rear Right Corner View Button from Rear 360	92

3.3.6	CAMERA-UC-REQ-331354/A-Press Rear Right Corner View Button from Front 360	92

3.3.7	CAMERA-UC-REQ-331356/A-Press Front Offset View Button from Rear 360	92

3.3.8	CAMERA-UC-REQ-331360/A-Press Front Offset View Button from Front 360	93

3.3.9	CAMERA-UC-REQ-331365/A-Press Front Left Corner View Button from Rear 360	93

3.3.10	CAMERA-UC-REQ-331369/A-Press Front Left Corner View Button from Front 360	93

3.3.11	CAMERA-UC-REQ-331373/A-Press Front Right Corner View Button from Rear 360	93

3.3.12	CAMERA-UC-REQ-331374/A-Press Front Right Corner View Button from Front 360	94

3.4	Views At Speed	94

3.4.1	CAMERA-FUN-REQ-354982/A-Views At Speed	94

4	Appendix: Reference Documents	98

# Architectural Design

## RVC-CLD-REQ-014079/A-Rear View Camera Client (TcSE ROIN-282382-1)

Responsibility: The Rear View Camera Client is the interface of the Rear View Camera function. It acts with other system parts that control the Rear View Camera or need data from it.

## DAFVCv1-CLD-REQ-014064/A-Driver Assist Front View Camera Client (TcSE ROIN-282843-1)

Responsibility: The Driver Assist Front View Camera Client is the interface of the Driver Assist Front View Camera function. It acts with other system parts that control the Driver Assist Front View Camera or need data from it.

## RearViewCameraClient Interface

### RVCv2-IR-REQ-014080/F-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3)

The below table defines the Feature ID information related to the Feature Based Message Protocol command and control method used in the Rear View Camera feature.

| Feature Name | Feature ID | Config Number | Config Value |
| --- | --- | --- | --- |
| Rear Camera Visual Park Aid Overlays | 0x080A | 0x01 | On |
|  |  | 0x00 | Off |
| All Overlays | 0x0812 | 0x01 | On |
|  |  | 0x00 | Off |
| Rear Camera Static Overlays | 0x0818 | 0x01 | On |
|  |  | 0x00 | Off |
| Rear Camera Dynamic Overlays | 0x0819 | 0x01 | On |
|  |  | 0x00 | Off |
| Manual Zoom | 0x081A | 0x00 | Off |
|  |  | 0x01 | Level 1 |
|  |  | 0x02 | Level2 |
|  |  | 0x03 | Level3 |

| Feature Name | Feature ID | Config Number | Config Value |
| --- | --- | --- | --- |
| Camera Showing | 0x081B | 0x00 | Off |
|  |  | 0x01 | Front |
|  |  | 0x02 | Rear |
|  |  | 0x03 | Remote |
|  |  | 0x04 | Rear 360 |
|  |  | 0x05 | Rear Normal |
|  |  | 0x06 | Rear Split |
|  |  | 0x07 | Front 360 |
|  |  | 0x08 | Front Normal |
|  |  | 0x09 | Front Split |
|  |  | 0x0A | Rear Zoom |
|  |  | 0x0B | CHMSL |
|  |  | 0x0C | CHMSL Zoom |
|  |  | 0x0D | AUX |
|  |  | 0x0E | TRG |
|  |  | 0x0F | TRG Rear Normal |
|  |  | 0x10 | Straight Back Up Mode |
|  |  | 0x11 | TBA2 Rear Split |
|  |  | 0x12 | TBA2 Rear 360 |
|  |  | 0x13 | TBA2 CHMSL |
|  |  | 0x14 | TBA2 Aux |
|  |  | 0x15 | TBA2 50/50 |
|  |  | 0x16 | TBA2 Rear Normal |
|  |  | 0x17 | Auto Hitch Rear Normal |
|  |  | 0x18 | Rear Frwd Offset |
|  |  | 0x19 | Rear Rrwd Offset |
|  |  | 0x1A | Front Frwd Offset |
|  |  | 0x1B | Front Rrwd Offset |
|  |  | 0x1C | Rear FL Corner |
|  |  | 0x1D | Rear FR Corner |
|  |  | 0x1E | Rear RL Corner |
|  |  | 0x1F | Rear RR Corner |
|  |  | 0x20 | Front FL Corner |
|  |  | 0x21 | Front FR Corner |
|  |  | 0x22 | Front RL Corner |
|  |  | 0x23 | Front RR Corner |
|  |  | 0x24 | Rock Crawl Front |
|  |  | 0x25 | Rock Crawl Rear |
|  |  | 0x26 | Hitch |
|  |  | 0x27 | Rear Camera On Demand (360) |

### RVCv2-IIR-REQ-014081/A-RearViewCameraClient_Tx (TcSE ROIN-282395-3)

#### MD-REQ-014068/A-Feature_Rq (TcSE ROIN-282333-2)

Message Type: Request

Represents the request to command a feature change (select new feature, change feature setting, query features, etc.).

Included Parameters:

Operation

FeatureID

Configuration

PersIndex

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Operation | - | - | Type of operation being requested |
|  | Null | 0x0 |  |
|  | Query | 0x1 |  |
|  | Set | 0x2 |  |
|  | Upload | 0x3 |  |
|  | Restore | 0x4 |  |
|  | Copy | 0x5 |  |
|  | NotUsed | 0x6 – 0x7 |  |
| FeatureID | - | - | Feature number being requested |
|  |  | 0x0000 – 0xFFFF |  |
| Configuration | - | - | Configuration value being requested |
|  |  | 0x0000 – 0xFFFF |  |
| PersIndex | - | - | Indicates which personality profile is being accessed |
|  | PERS_1 | 0x0 |  |
|  | PERS_2 | 0x1 |  |
|  | PERS_3 | 0x2 |  |
|  | PERS_4 | 0X3 |  |
|  | VEHICLE | 0X4 |  |
|  | Not Used | 0x5 |  |
|  | Not Used | 0x6 |  |
|  | Not Used | 0x7 |  |

### RVCv2-IIR-REQ-014082/B-RearViewCameraClient_Rx (TcSE ROIN-282396-3)

#### MD-REQ-014070/A-Feature_St (TcSE ROIN-282399-2)

Message Type: Status

Represents the current status of a feature (feature selected, feature setting, etc.).

Included Parameters:

FeatureID

Configuration

PersIndex

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| FeatureID | - | - | Active feature number |
|  |  | 0x0000 – 0xFFFF |  |
| Configuration | - | - | Active configuration value |
|  |  | 0x0000 – 0xFFFF |  |
| PersIndex | - | - | Indicates which personality profile is active |
|  | PERS_1 | 0x0 |  |
|  | PERS_2 | 0x1 |  |
|  | PERS_3 | 0x2 |  |
|  | PERS_4 | 0X3 |  |
|  | VEHICLE | 0X4 |  |
|  | Not Used | 0x5 |  |
|  | Not Used | 0x6 |  |
|  | Not Used | 0x7 |  |

#### MD-REQ-014083/A-PJB_Bootlid_St (TcSE ROIN-282394-1)

Message Type: Status

Vehicle status message to indicate when the decklid/liftgate is ajar.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | TrunkClosed | 0x0 |  |
|  | TrunkOpen | 0x1 |  |

#### MD-REQ-014023/A-GearLvrPos_D_Actl (TcSE ROIN-266648-1)

Message Type: Status

Vehicle status signal for the Gear Lever Position on an automatic transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Park | 0x0 |  |
|  | Reverse | 0x1 |  |
|  | Neutral | 0x2 |  |
|  | Drive | 0x3 |  |
|  | Sport_DriveSport | 0x4 |  |
|  | Low | 0x5 |  |
|  | First | 0x6 |  |
|  | Second | 0x7 |  |
|  | Third | 0x8 |  |
|  | Fourth | 0x9 |  |
|  | Fifth | 0xA |  |
|  | Sixth | 0xB |  |
|  | Undefined_Treat_as_Fault | 0xC |  |
|  | Undefined_Treat_as_Fault1 | 0xD |  |
|  | Unknown_Position | 0xE |  |
|  | Fault | 0xF |  |

#### MD-REQ-014024/A-GearRvrseActv_D_Actl (TcSE ROIN-266649-1)

Message Type: Status

Vehicle status signal for notifying that Reverse Gear is engaged on a manual transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |
|  | Unknown | 0x2 |  |
|  | Fault | 0x3 |  |

#### MD-REQ-014025/A-VehicleSpeed_St (TcSE ROIN-223023-1)

Message Type: Status

Status used to indicate vehicle speed.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Indicates vehicle speed. Unit: kph Resolution:0.01 Offset:0 |
|  | kph | 0x0 to 0xFFFF |  |

#### MD-REQ-014084/B-ParkBrake_St (TcSE ROIN-287064-1)

Message Type: Status

Signal used to indicate the Parking Brake status.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |

#### MD-REQ-014085/B-ElectronicParkBrake_St (TcSE ROIN-287065-1)

Message Type: Status

Signal used to indicate the Electronic Parking Brake status.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | NotUsed | 0x0 |  |
|  | Rear_Caliper_Closed | 0x1 |  |
|  | Rear_Caliper_Transition | 0x2 |  |
|  | RWU_By_EPB_Active | 0x3 |  |
|  | Rear_Caliper_Open | 0x4 |  |
|  | EPB_Limphome_Active | 0x5 |  |
|  | ECD_by_Brake_ECU_Active | 0x6 |  |
|  | GeneralFault_MaintenceMode | 0x7 |  |

Electronic Park Brake is considered active when the signal has a value of 0x1 (Rear_Caliper_Closed)

#### MD-REQ-014071/B-CameraServerButton_St (TcSE ROIN-287063-1)

Message Type: Status

Signal used to indicate Front Camera Hard Button status. Real Can name: CamraFrntStat_D_Stat

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| ButtonState | - | - | - |
|  | Off | 0x0 |  |
|  | On | 0x1 |  |
|  | NotUsed | 0x2 |  |
|  | NoDataPresent | 0x3 |  |

#### RVC-MD-REQ-292389/B-GearRvrse_D_Actl

GearRvrse_D_Actl

The purpose of this signal is to notify that Reverse Gear is engaged on a manual transmission vehicle.

$0: Inactive_not_confirmed

$1: Inactive_confirmed

$2: Active_not_confirmed

$3: Active_confirmed

$4: NotUsed_1

$5: NotUsed_2

$6: NotUsed_3

$7: Fault

Reverse status is indicated by both $2 (Active_not_confirmed) and $3 (Active_confirmed)

#### RVC-REQ-292387/A-GearPos_D_Trg

GearPos_D_Trg

This signal is used to indicate Gear direction. Used with other gear signals to determine whether or not RVC is to be turned On or Off.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Neutral | 0x0 |  |
|  | First | 0x1 |  |
|  | Second | 0x2 |  |
|  | Third | 0x3 |  |
|  | Fourth | 0x4 |  |
|  | Fifth | 0x5 |  |
|  | Sixth | 0x6 |  |
|  | Seventh | 0x7 |  |
|  | Eighth | 0x8 |  |
|  | Ninth | 0x9 |  |
|  | Tenth | 0xA |  |
|  | Undefined_3 | 0xB |  |
|  | Undefined_4 | 0xC |  |
|  | Undefined_5 | 0xD |  |
|  | Reverse | 0xE |  |
|  | Unknown | 0xF |  |

#### RVC-REQ-292388/A-Veh_V_ActlEng

Veh_V_ActlEng

This signal is used to indicate vehicle speed. Refer to database for proper signal values.

## DriverAssistFrontViewCameraClient Interface

### DAFVCv1-IR-REQ-014066/F-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061)

The below table defines the Feature ID information related to the Feature Based Message Protocol command and control method used in the Driver Assist Front View Camera feature.

The below table defines the Feature ID information related to the Feature Based Message Protocol command and control method used in the Rear View Camera feature.

| Feature Name | Feature ID | Config Number | Config Value |
| --- | --- | --- | --- |
| Camera Showing | 0x081B | 0x00 | Off |
|  |  | 0x01 | Front |
|  |  | 0x02 | Rear |
|  |  | 0x03 | Remote |
|  |  | 0x04 | Rear 360 |
|  |  | 0x05 | Rear Normal |
|  |  | 0x06 | Rear Split |
|  |  | 0x07 | Front 360 |
|  |  | 0x08 | Front Normal |
|  |  | 0x09 | Front Split |
|  |  | 0x0A | Rear Zoom |
|  |  | 0x0B | CHMSL |
|  |  | 0x0C | CHMSL Zoom |
|  |  | 0x0D | AUX |
|  |  | 0x0E | TRG |
|  |  | 0x0F | TRG Rear Normal |
|  |  | 0x10 | Straight Back Up Mode |
|  |  | 0x11 | TBA2 Rear Split |
|  |  | 0x12 | TBA2 Rear 360 |
|  |  | 0x13 | TBA2 CHMSL |
|  |  | 0x14 | TBA2 Aux |
|  |  | 0x15 | TBA2 50/50 |
|  |  | 0x16 | TBA2 Rear Normal |
|  |  | 0x17 | Auto Hitch Rear Normal |
|  |  | 0x18 | Rear Frwd Offset |
|  |  | 0x19 | Rear Rrwd Offset |
|  |  | 0x1A | Front Frwd Offset |
|  |  | 0x1B | Front Rrwd Offset |
|  |  | 0x1C | Rear FL Corner |
|  |  | 0x1D | Rear FR Corner |
|  |  | 0x1E | Rear RL Corner |
|  |  | 0x1F | Rear RR Corner |
|  |  | 0x20 | Front FL Corner |
|  |  | 0x21 | Front FR Corner |
|  |  | 0x22 | Front RL Corner |
|  |  | 0x23 | Front RR Corner |
|  |  | 0x24 | Rock Crawl Front |
|  |  | 0x25 | Rock Crawl Rear |
|  |  | 0x26 | Hitch |
|  |  | 0x27 | Rear Camera on Demand (360) |

### DAFVCv1-IIR-REQ-014067/C-DriverAssistFrontViewCameraClient_Tx (TcSE ROIN-282846-1)

#### MD-REQ-014068/A-Feature_Rq (TcSE ROIN-282333-2)

Message Type: Request

Represents the request to command a feature change (select new feature, change feature setting, query features, etc.).

Included Parameters:

Operation

FeatureID

Configuration

PersIndex

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Operation | - | - | Type of operation being requested |
|  | Null | 0x0 |  |
|  | Query | 0x1 |  |
|  | Set | 0x2 |  |
|  | Upload | 0x3 |  |
|  | Restore | 0x4 |  |
|  | Copy | 0x5 |  |
|  | NotUsed | 0x6 – 0x7 |  |
| FeatureID | - | - | Feature number being requested |
|  |  | 0x0000 – 0xFFFF |  |
| Configuration | - | - | Configuration value being requested |
|  |  | 0x0000 – 0xFFFF |  |
| PersIndex | - | - | Indicates which personality profile is being accessed |
|  | PERS_1 | 0x0 |  |
|  | PERS_2 | 0x1 |  |
|  | PERS_3 | 0x2 |  |
|  | PERS_4 | 0X3 |  |
|  | VEHICLE | 0X4 |  |
|  | Not Used | 0x5 |  |
|  | Not Used | 0x6 |  |
|  | Not Used | 0x7 |  |

#### MD-REQ-331342/B-CamraFrntBttn_D_Stat3

Message Type: Command

CamraFrntBttn_D_Stat3 : Signal is sent by client to server to request camera view activation/deactivation.

The trigger of this signal can be through a soft button press, which is internally to client or through a hard button connected to client through LIN protocol.

When the trigger of this signal is due to hard button LIN switch press, map this signal according to the details in Req 331343 and Req 331344.

Included Parameters:

Not pressed

Pressed

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Operation | - | - |  |
|  | Not pressed | 0x0 | Switch is not pressed |
|  | Pressed | 0x1 | Switch is pressed |

### DAFVCv1-IIR-REQ-014069/C-DriverAssistFrontViewCameraClient_Rx (TcSE ROIN-282847-2)

#### MD-REQ-014070/A-Feature_St (TcSE ROIN-282399-2)

Message Type: Status

Represents the current status of a feature (feature selected, feature setting, etc.).

Included Parameters:

FeatureID

Configuration

PersIndex

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| FeatureID | - | - | Active feature number |
|  |  | 0x0000 – 0xFFFF |  |
| Configuration | - | - | Active configuration value |
|  |  | 0x0000 – 0xFFFF |  |
| PersIndex | - | - | Indicates which personality profile is active |
|  | PERS_1 | 0x0 |  |
|  | PERS_2 | 0x1 |  |
|  | PERS_3 | 0x2 |  |
|  | PERS_4 | 0X3 |  |
|  | VEHICLE | 0X4 |  |
|  | Not Used | 0x5 |  |
|  | Not Used | 0x6 |  |
|  | Not Used | 0x7 |  |

#### MD-REQ-014023/A-GearLvrPos_D_Actl (TcSE ROIN-266648-1)

Message Type: Status

Vehicle status signal for the Gear Lever Position on an automatic transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Park | 0x0 |  |
|  | Reverse | 0x1 |  |
|  | Neutral | 0x2 |  |
|  | Drive | 0x3 |  |
|  | Sport_DriveSport | 0x4 |  |
|  | Low | 0x5 |  |
|  | First | 0x6 |  |
|  | Second | 0x7 |  |
|  | Third | 0x8 |  |
|  | Fourth | 0x9 |  |
|  | Fifth | 0xA |  |
|  | Sixth | 0xB |  |
|  | Undefined_Treat_as_Fault | 0xC |  |
|  | Undefined_Treat_as_Fault1 | 0xD |  |
|  | Unknown_Position | 0xE |  |
|  | Fault | 0xF |  |

#### MD-REQ-014024/A-GearRvrseActv_D_Actl (TcSE ROIN-266649-1)

Message Type: Status

Vehicle status signal for notifying that Reverse Gear is engaged on a manual transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |
|  | Unknown | 0x2 |  |
|  | Fault | 0x3 |  |

#### MD-REQ-014025/A-VehicleSpeed_St (TcSE ROIN-223023-1)

Message Type: Status

Status used to indicate vehicle speed.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Indicates vehicle speed. Unit: kph Resolution:0.01 Offset:0 |
|  | kph | 0x0 to 0xFFFF |  |

#### MD-REQ-014071/B-CameraServerButton_St (TcSE ROIN-287063-1)

Message Type: Status

Signal used to indicate Front Camera Hard Button status. Real Can name: CamraFrntStat_D_Stat

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| ButtonState | - | - | - |
|  | Off | 0x0 |  |
|  | On | 0x1 |  |
|  | NotUsed | 0x2 |  |
|  | NoDataPresent | 0x3 |  |

#### MD-REQ-014020/A-Elocker_St (TcSE ROIN-266643-1)

(CAN Name:  RearDiffLckLamp_D_Rq)

Message Type: Status

Vehicle status signal for displaying the Electric Differential Locker (Elocker) engagement message to the user.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | OFF | 0x0 |  |
|  | ON | 0x1 |  |
|  | Flash | 0x2 |  |
|  | Notused | 0x3 |  |

#### MD-REQ-014021/A-AwdRange_St (TcSE ROIN-266644-1)

(CAN Name:  AwdRnge_D_Actl)

Message Type: Status

Vehicle status signal for the state of the transfer box in terms of range and coupling locking.

"locked" means the  AWD coupling / differential is locked.

"auto" means the AWD coupling / differential is under active control (could be locked, open or anywhere in between depending on system).

"2wd" means the AWD coupling is open.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | LowRangeLocked | 0x0 |  |
|  | LowRangeAuto | 0x1 |  |
|  | LowRange2wd | 0x2 |  |
|  | Neutral | 0x3 |  |
|  | HighRangeLocked | 0x4 |  |
|  | HighRangeAuto | 0x5 |  |
|  | HighRange2wd | 0x6 |  |
|  | Unknown | 0x7 |  |

#### MD-REQ-014022/A-OffRoadMode_St (TcSE ROIN-266645-1)

(CAN Name:  AwdOffRoadMode_D_Stats)

Message Type: Status

Vehicle status signal for the state of Off Road Mode.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | NormalMode | 0x0 |  |
|  | OffRoad | 0x1 |  |
|  | ExtemeOffRoad | 0x2 |  |
|  | Invalid | 0x3 |  |

#### MD-REQ-014024/A-GearRvrseActv_D_Actl (TcSE ROIN-266649-1)

Message Type: Status

Vehicle status signal for notifying that Reverse Gear is engaged on a manual transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |
|  | Unknown | 0x2 |  |
|  | Fault | 0x3 |  |

#### MD-REQ-331343/A-Camera Button LIN signal

Message Type: Command

Represents the state of the physical front camera switch. This signal is the instantaneous status of the LIN switch which is wired to an external ECU. LIN signal name used is ICPBtnID_Camera.

Included Parameters:

Inactive

Active

Short Event

Short Elapsed

Long Event

Stuck

Idle

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Operation | - | - |  |
|  | Inactive | 0 | Switch is not pressed |
|  | Active | 1 | Switch is pressed |
|  | Short Event | 2 | Switch is pressed |
|  | Short Elapsed | 3 | Switch is pressed |
|  | Long Event | 4 | Switch is pressed |
|  | Stuck | 5 | Switch is not pressed |
|  | Not used | 6-14 | Switch is not pressed |
|  | Idle | 15 | Switch is not pressed |

#### MD-REQ-331847/A-CamraFrntOffRd_B_Stat

Message Type: Status

Vehicle status signal as determined by the IPMB for the state of Off Road Mode.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |

## VehicleInformation Interface

### RVCv2-IIR-REQ-014086/C-VehicleInformation_Tx (TcSE ROIN-282400-2)

#### MD-REQ-014083/A-PJB_Bootlid_St (TcSE ROIN-282394-1)

Message Type: Status

Vehicle status message to indicate when the decklid/liftgate is ajar.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | TrunkClosed | 0x0 |  |
|  | TrunkOpen | 0x1 |  |

#### MD-REQ-014023/A-GearLvrPos_D_Actl (TcSE ROIN-266648-1)

Message Type: Status

Vehicle status signal for the Gear Lever Position on an automatic transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Park | 0x0 |  |
|  | Reverse | 0x1 |  |
|  | Neutral | 0x2 |  |
|  | Drive | 0x3 |  |
|  | Sport_DriveSport | 0x4 |  |
|  | Low | 0x5 |  |
|  | First | 0x6 |  |
|  | Second | 0x7 |  |
|  | Third | 0x8 |  |
|  | Fourth | 0x9 |  |
|  | Fifth | 0xA |  |
|  | Sixth | 0xB |  |
|  | Undefined_Treat_as_Fault | 0xC |  |
|  | Undefined_Treat_as_Fault1 | 0xD |  |
|  | Unknown_Position | 0xE |  |
|  | Fault | 0xF |  |

#### MD-REQ-014024/A-GearRvrseActv_D_Actl (TcSE ROIN-266649-1)

Message Type: Status

Vehicle status signal for notifying that Reverse Gear is engaged on a manual transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |
|  | Unknown | 0x2 |  |
|  | Fault | 0x3 |  |

#### MD-REQ-014025/A-VehicleSpeed_St (TcSE ROIN-223023-1)

Message Type: Status

Status used to indicate vehicle speed.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Indicates vehicle speed. Unit: kph Resolution:0.01 Offset:0 |
|  | kph | 0x0 to 0xFFFF |  |

#### MD-REQ-014084/B-ParkBrake_St (TcSE ROIN-287064-1)

Message Type: Status

Signal used to indicate the Parking Brake status.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |

#### MD-REQ-014085/B-ElectronicParkBrake_St (TcSE ROIN-287065-1)

Message Type: Status

Signal used to indicate the Electronic Parking Brake status.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | NotUsed | 0x0 |  |
|  | Rear_Caliper_Closed | 0x1 |  |
|  | Rear_Caliper_Transition | 0x2 |  |
|  | RWU_By_EPB_Active | 0x3 |  |
|  | Rear_Caliper_Open | 0x4 |  |
|  | EPB_Limphome_Active | 0x5 |  |
|  | ECD_by_Brake_ECU_Active | 0x6 |  |
|  | GeneralFault_MaintenceMode | 0x7 |  |

Electronic Park Brake is considered active when the signal has a value of 0x1 (Rear_Caliper_Closed)

#### RVC-REQ-292387/A-GearPos_D_Trg

GearPos_D_Trg

This signal is used to indicate Gear direction. Used with other gear signals to determine whether or not RVC is to be turned On or Off.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Neutral | 0x0 |  |
|  | First | 0x1 |  |
|  | Second | 0x2 |  |
|  | Third | 0x3 |  |
|  | Fourth | 0x4 |  |
|  | Fifth | 0x5 |  |
|  | Sixth | 0x6 |  |
|  | Seventh | 0x7 |  |
|  | Eighth | 0x8 |  |
|  | Ninth | 0x9 |  |
|  | Tenth | 0xA |  |
|  | Undefined_3 | 0xB |  |
|  | Undefined_4 | 0xC |  |
|  | Undefined_5 | 0xD |  |
|  | Reverse | 0xE |  |
|  | Unknown | 0xF |  |

#### MD-REQ-014024/A-GearRvrseActv_D_Actl (TcSE ROIN-266649-1)

Message Type: Status

Vehicle status signal for notifying that Reverse Gear is engaged on a manual transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |
|  | Unknown | 0x2 |  |
|  | Fault | 0x3 |  |

#### RVC-MD-REQ-292389/B-GearRvrse_D_Actl

GearRvrse_D_Actl

The purpose of this signal is to notify that Reverse Gear is engaged on a manual transmission vehicle.

$0: Inactive_not_confirmed

$1: Inactive_confirmed

$2: Active_not_confirmed

$3: Active_confirmed

$4: NotUsed_1

$5: NotUsed_2

$6: NotUsed_3

$7: Fault

Reverse status is indicated by both $2 (Active_not_confirmed) and $3 (Active_confirmed)

#### MD-REQ-347573/A-PowerMode

PowerMode

This signal is used in CGEA 1.2 only, not for later architectures.

$0: KeyOut

$1: KeyRecentlyOut

$2: KeyApproved_0

$3: PostAccessory_0

$4: Accessory_1

$5: PostIgnition_1

$6: IgnitionOn_2

$7: Running_2

$8: Not_Used

$9: Crank_3

$A: Not_Used1

# General Requirements

## REQ-331344/A-Button Press Logic+

When configured for LIN-based camera button logic, the CAN output shall be populated as follows:

| Switch Configuration (Method 2) | LIN Input | CamraFrntBttn_D_Stat3 |
| --- | --- | --- |
| LIN Camera Switch Not Supported | X | 0x0 (Not pressed) |
| LIN Camera Switch Supported | 0x0 (Inactive) | 0x0 (Not pressed) |
| LIN Camera Switch Supported | 0x1 (Active) | 0x1 (Pressed) |
| LIN Camera Switch Supported | 0x2 (Short Event) | 0x1 (Pressed) |
| LIN Camera Switch Supported | 0x3 (Short Elapsed) | 0x1 (Pressed) |
| LIN Camera Switch Supported | 0x4 (Long Event) | 0x1 (Pressed) |
| LIN Camera Switch Supported | 0x5 (Stuck) | 0x0 (Not pressed) |
| LIN Camera Switch Supported | 0x6-0xE (Not used) | 0x0 (Not pressed) |
| LIN Camera Switch Supported | 0xF (Idle) | 0x0 (Not pressed) |

## CAMERA-REQ-014077/C-Feature Maximum Speed (TcSE ROIN-290556)

The feature maximum speed when displaying a camera image in forward gear shall be as described for each operational scenario below:

- Any camera feature activation attempted by User

  - Rear Camera

Feature maximum speed = 10 kph

  - Off Road Front Camera configured:  Not Available

Feature maximum speed = 10 kph

  - Off Road Front Camera configured:  Available and does NOT meet conditions for Off Road Mode per Determine Off Road Mode requirement

Feature maximum speed = 10 kph

  - Off Road Front Camera configured:  Available and meets conditions for Off Road Mode per Determine Off Road Mode requirement

Feature maximum speed = 20 kph

- Rear Camera Active

Feature maximum speed = 10 kph

- Front Camera Active

  - Off Road Front Camera configured:  Not Available

Feature maximum speed = 10 kph

  - Off Road Front Camera configured:  Available and does NOT meet conditions for Off Road Mode per Determine Off Road Mode requirement

Feature maximum speed = 10 kph

  - Off Road Front Camera configured:  Available and meets conditions for Off Road Mode per Determine Off Road Mode requirement

Feature maximum speed = 24 kph

## DAFVCv1-FUR-REQ-166648/B-Determine Off Road Mode

Off Road Mode is considered active when the vehicle is configured for (Off Road Front Camera and hard camera button strategy) and one of the following conditions are true:

- 4x4 transfer case is locked in 4L (AwdRange_St = 0x0:LowRangeLocked)

or

- Off-Road Mode is on (OffRoadMode_St = 0x1:OffRoad or 0x2:ExtremeOffRoad)

and

E-Locker is locked (Elocker_St = 0x1:ON)

Off Road Mode is considered active when the vehicle is configured for (Off Road Front Camera and soft camera button strategy) and the following condition is true:

CamraFrntOffRd_B_Stat = Active

When transitioning out of Off Road Mode the Driver Assist Front View Camera Client shall check vehicle speed to determine if condition exists that results in Front View Camera turning off.

Example:  Driver Assist Front View Camera Client determines Off Road Mode is no longer active and vehicle speed is 20 kph.  The max speed to exit is now 10 kph so the Client shall exit Front View Camera based on Feature Maximum Speed requirement 014077.

## RVC-FUR-REQ-014087/B-RVC Malfunction (TcSE ROIN-146656-2)

When the RVC Client (RearViewCameraClient) does not detect video present in the signal from the camera it shall set a DTC and the RVC Client shall display camera overlays for T_cameraMalfunctionDelay before displaying an error message allowing the user to acknowledge the video error and revert to the previous screen.  At any time the video signal is detected RVC client should check for Gear position and show the camera.

## DAFVCv1-FUR-REQ-166722/A-DAFVC Malfunction

When the DAFVC Client (DriverAssistFrontViewCameraClient) does not detect video present in the signal from the camera it shall set a DTC and the DAFVC Client shall display camera overlays for T_cameraMalfunctionDelay before displaying an error message allowing the user to acknowledge the video error and revert to the previous screen.  At any time the video signal is detected DAFVC client should check for Gear position and show the camera.

## RVC-TMR-REQ-166649/A-T_cameraMalfunctionDelay

| Name | Description | Units | Range | Resolution | Default |
| --- | --- | --- | --- | --- | --- |
| T_cameraMalfunctionDelay | Time DAFVC or RVC Client should wait before displaying an error message to the user according to RVC-REQ-014087-RVC Malfunction or DAFVC-REQ-166649 DAFVC Malfunction. | sec | 0-30 | 1 | 10 |

## REQ-331863/A-Lin Button Client Operation+

When Client receives signal ICPBtnID_Camera  as pressed, it shall send the signal  CamraFrntBttn_D_Stat3 with the value Pressed.

# Functional Definition

## Rear View Camera

### Rear View Camera General Requirements

#### RVC-FUR-REQ-014088/E-Deactivate RVC (TcSE ROIN-293328)

The RVC Client (RearViewCameraClient) shall stop displaying RVC video when one of the following conditions is met:

1. 	Vehicle is shifted out of reverse (Camera Delay = OFF)

2. 	Vehicle is shifted out of reverse (GearLvrPos_D_Actl does not equal Reverse in automatic Transmission vehicle or GearRvrse_D_Actl or GearRvrseActv_D_Actl does not equal active in Manual Transmission vehicle) and vehicle speed > limit per CAMERA-REQ-014077-Feature Maximum Speed (Camera Delay = ON)

3. 	CGEA 1.2:

Power Mode does not equal IgnitionOn_2 or Running_2 or Crank_3

CGEA 1.3:

Ignition_Status does not equal Run

4. 	Vehicle is shifted into Park

- Automatic Transmission vehicle GearLvrPos_D_Actl == 0x0

- Manual Transmission Vehicle with Mechanical Park Brake

GearRvrse_D_Actl == Inactive or GearRvrseActv_D_Actl == Inactive AND PrkBrkActv_B_Actl == Active

- Manual Transmission Vehicle with Electronic Park Brake

GearRvrse_D_Actl == Inactive or GearRvrseActv_D_Actl == Inactive AND PrkBrkStatus == Active

#### RVC-FUR-REQ-014089/A-Decklid/Liftgate Ajar (TcSE ROIN-146658-2)

When the RVC Client (RearViewCameraClient) detects the decklid or liftgate is ajar, the RVC Client shall display a warning message that explains why no guidelines are available in the camera image.

#### RVC-FUR-REQ-014090/G-Display RVC Video (TcSE ROIN-194462-2)

There are two ways for Reverse Detection. What way to use is decided on configuration values.

Reverse Detection NEW:

Reverse_Gear is determined as mentioned in below table. Once GearLvrPos_D_Actl is reverse, System need to loop through signal GearPos_D_Trg to determine reverse gear until either GearLvrPos_D_Actl  is not reverse OR Camera turn ON.

| GearLvrPos_D_Actl = 0x1 (Reverse) (automatic transmission) | GearPos_D_Trg | Gear position and Camera Status |
| --- | --- | --- |
| Reverse | 0xE (Reverse) | Gear is Reverse, Turn Camera On |

Upon detecting the conditions for RVC to be ON, the RVC Client (RearViewCameraClient) shall start a timer (T_minImageDisp) and shall not display the RVC image until the expiration of this timer.  Upon expiration of the timer, the RVC Client shall start another timer (T_maxImageDisp). The RVC Client must display the RVC image prior to the expiration of T_maxImageDisp.

Once the conditions for displaying RVC are no longer applicable the RVC client shall:

1.            Cancel the timer

2.            Not display the RVC image

Reverse Detection LEGACY:

Reverse Detection is determined as mentioned in below table.

| GearLvrPos_D_Actl = 0x1 (Reverse) (automatic transmission) or GearRvrseActv_D_Actl = 0x1 (Active )(manual transmission vehicle and Legacy Message Set) or GearRvrse_D_Actl = 0x3 or 0x2 ( Active Confirmed or Active_not_confirmed) (manual transmission vehicle and New Message Set) | Gear position and Camera Status |
| --- | --- |
| Reverse | Gear is Reverse, Turn Camera On |

Upon detecting the conditions for RVC to be ON, the RVC Client (RearViewCameraClient) shall start a timer (T_minImageDisp) and shall not display the RVC image until the expiration of this timer.  Upon expiration of the timer, the RVC Client shall start another timer (T_maxImageDisp). The RVC Client must display the RVC image prior to the expiration of T_maxImageDisp.

Once the conditions for displaying RVC are no longer applicable the RVC client shall:

1.            Cancel the timer

2.            Not display the RVC image

#### RVC-TMR-REQ-014091/A-T_minImageDisp (TcSE ROIN-264661-1)

| Name | Description | Units | Range | Resolution | Default |
| --- | --- | --- | --- | --- | --- |
| T_minImageDisp | Minimum time RVC Client should wait before displaying the RVC video image to the user according to RVC-GREQ-194462-2-Display RVC Video. | msec | 225-275 | 5 | 250 |

#### RVC-TMR-REQ-014092/A-T_maxImageDisp (TcSE ROIN-264662-1)

| Name | Description | Units | Range | Resolution | Default |
| --- | --- | --- | --- | --- | --- |
| T_maxImageDisp | Maximum time RVC Client should wait before displaying the RVC video image to the user according to RVC-GREQ-194462-2-Display RVC Video. | msec | 450-550 | 5 | 500 |

#### CAMERA-FUR-REQ-014093/B-Camera Image Priority (TcSE ROIN-264652-1)

Once the camera image has been displayed to user, the image shall be maintained as long as the conditions required to be in the particular camera view are present and shall have highest priority:

•	No pop-up screens shall interrupt the video image.

•	Media functions (source change, volume control, etc.) shall be available, but shall not interrupt the image view to the user.

### RVCv2-FUN-REQ-014094/A-Activate/Deactivate Rear View Camera (TcSE ROIN-293352)

#### Use Cases

##### RVC-UC-REQ-014095/A-Activate Rear View Camera (TcSE ROIN-289794)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver activates the Rear View Camera (RVC) by placing the vehicle in Reverse Gear. |
| Post-conditions | The vehicle display shows the RVC image. |
| List of Exception Use Cases | E1 – Rear View Camera Malfunction E2 – Decklid/Liftgate is Ajar while Rear View Camera is ON |
| Interfaces | G-HMI Vehicle System Interface |

##### UC-REQ-238578/A-CHMSL Activation. Soft button press

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - APIM is configured (CHMSL) - Vehicle in Run/Start - Rear view (Aux, Rear Normal, Rear Split-View, Rear 360) is showing and not in CHSML view OR Front view (Front Normal, Front Split-View, Front 360) is showing and not in CHSML view |
| Scenario Description | User presses CHMSL view button Display sends “Camera Showing(81B): 0x0B” |
| Post-conditions | The CHMSL view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-238579/A-AUX Activation. Soft button press

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - APIM is configured (Aux) - Vehicle in Run/Start - Rear view (CHMSL, Rear Normal, Rear Split-View, Rear 360) is showing and not in Aux view OR Front view (Front Normal, Front Split-View, Front 360) is showing and not in Aux view |
| Scenario Description | User presses Aux view button Display sends “Camera Showing(81B): 0x0D” |
| Post-conditions | The Aux view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### RVC-UC-REQ-014096/A-Rear View Camera Malfunction (TcSE ROIN-289795)

Linked Elements

RVC-UC-REQ-014095/A-Activate Rear View Camera (TcSE ROIN-289794)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The HMI interface indicates that the Rear View Camera (RVC) image cannot be shown because of a malfunction. |
| Post-conditions | The vehicle display is NOT showing RVC image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014097/A-Decklid/Liftgate is Ajar while Rear View Camera is ON (TcSE ROIN-289796)

Linked Elements

RVC-UC-REQ-014095/A-Activate Rear View Camera (TcSE ROIN-289794)

RVC-UC-REQ-128278/A-Activate Rear View Camera

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The HMI interface indicates that the Decklid/Liftgate is Ajar. |
| Post-conditions | The vehicle display shows the Rear View Camera image. The video feed from the Rear View Camera contains an image without guideline overlays. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver deactivates the Rear View Camera (RVC) by shifting the vehicle out of Reverse Gear. |
| Post-conditions | The vehicle display is NOT showing RVC image. |
| List of Exception Use Cases | E1 – Rear Camera Delay Mode is On E2 – Active Park Assist is Active E3 – Trailer Backup Assist is Active (N/A for stand-alone RVC) |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014099/B-Rear Camera Delay Mode is On (TcSE ROIN-289798)

Linked Elements

RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

RVC-UC-REQ-128280/A-Deactivate Rear View Camera

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The driver shifts out of Reverse Gear and into any gear other than Park. The RVC image remains displayed to the driver until the vehicle reaches limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display stops showing Rear View Camera image when vehicle speed reaches limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014100/B-Active Park Assist is Active (TcSE ROIN-290554)

Linked Elements

RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

RVC-UC-REQ-128280/A-Deactivate Rear View Camera

DAFVCv1-UC-REQ-128313/A-Deactivate Driver Assist Front View Camera

DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The driver shifts out of Reverse Gear and into any other gear while Active Park Assist (APA) is active. The camera image feed remains displayed to the driver as long as APA is active and vehicle speed does not exceed limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display stops showing Rear View Camera image when APA is no longer active or vehicle speed exceeds limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014101/B-Trailer Backup Assist is Active (TcSE ROIN-290555)

Linked Elements

RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

RVC-UC-REQ-128280/A-Deactivate Rear View Camera

DAFVCv1-UC-REQ-128313/A-Deactivate Driver Assist Front View Camera

DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The driver shifts out of Reverse Gear and into any other gear while Trailer Backup Assist (TBA) is active. The camera image feed remains displayed to the driver as long as TBA is active and vehicle speed does not exceed limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display stops showing Rear View Camera image when TBA is no longer active or vehicle speed exceeds limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### RVCv2-ACT-REQ-014102/A-Activate Rear View Camera (TcSE ROIN-282388-2)

Linked Elements

RVCv2-SD-REQ-014104/B-Activate Rear View Camera (TcSE ROIN-282316-2)

Activity Diagram

###### RVCv2-ACT-REQ-014103/A-Deactivate Rear View Camera (TcSE ROIN-286992-1)

Linked Elements

RVCv2-SD-REQ-014105/B-Deactivate Rear View Camera (TcSE ROIN-282323-2)

Activity Diagram

##### Sequence Diagrams

###### RVCv2-SD-REQ-014104/B-Activate Rear View Camera (TcSE ROIN-282316-2)

Scenario

Normal Usage

The user activates the Rear View Camera (RVC) by placing the vehicle in Reverse Gear.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

HMI Display shows the RVC image

Sequence Diagram

###### RVCv2-SD-REQ-014105/B-Deactivate Rear View Camera (TcSE ROIN-282323-2)

Scenario

Normal Usage

The user deactivates the Rear View Camera (RVC) by taking the vehicle out of Reverse Gear.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

HMI Display stops showing the RVC image

Sequence Diagram

### RVCv3-FUN-REQ-128277/A-Activate/Deactivate Rear View Camera v3

#### Use Cases

##### RVCv3-UC-REQ-127872/A-Activate Rear Multicamera View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START |
| Scenario Description | The driver shifts gear to reverse for 250ms |
| Post-conditions | The Rear Multicamera view appears with overlays and review button |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014097/A-Decklid/Liftgate is Ajar while Rear View Camera is ON (TcSE ROIN-289796)

Linked Elements

RVC-UC-REQ-014095/A-Activate Rear View Camera (TcSE ROIN-289794)

RVC-UC-REQ-128278/A-Activate Rear View Camera

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The HMI interface indicates that the Decklid/Liftgate is Ajar. |
| Post-conditions | The vehicle display shows the Rear View Camera image. The video feed from the Rear View Camera contains an image without guideline overlays. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver deactivates the Rear View Camera (RVC) by shifting the vehicle out of Reverse Gear. |
| Post-conditions | The vehicle display is NOT showing RVC image. |
| List of Exception Use Cases | E1 – Rear Camera Delay Mode is On E2 – Active Park Assist is Active E3 – Trailer Backup Assist is Active (N/A for stand-alone RVC) |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014099/B-Rear Camera Delay Mode is On (TcSE ROIN-289798)

Linked Elements

RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

RVC-UC-REQ-128280/A-Deactivate Rear View Camera

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The driver shifts out of Reverse Gear and into any gear other than Park. The RVC image remains displayed to the driver until the vehicle reaches limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display stops showing Rear View Camera image when vehicle speed reaches limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128191/A-Enter CHMSL Delay Mode

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehcile is configured with CHMSL camera - The vehicle is in Run/Start - CHMSL or CHMSL Zoom shown - Camera Delay is set On |
| Scenario Description | The driver shifts from Reverse to Neutral or Drive |
| Post-conditions | The vehicle shows CHMSL without zoom button |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014100/B-Active Park Assist is Active (TcSE ROIN-290554)

Linked Elements

RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

RVC-UC-REQ-128280/A-Deactivate Rear View Camera

DAFVCv1-UC-REQ-128313/A-Deactivate Driver Assist Front View Camera

DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The driver shifts out of Reverse Gear and into any other gear while Active Park Assist (APA) is active. The camera image feed remains displayed to the driver as long as APA is active and vehicle speed does not exceed limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display stops showing Rear View Camera image when APA is no longer active or vehicle speed exceeds limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014101/B-Trailer Backup Assist is Active (TcSE ROIN-290555)

Linked Elements

RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

RVC-UC-REQ-128280/A-Deactivate Rear View Camera

DAFVCv1-UC-REQ-128313/A-Deactivate Driver Assist Front View Camera

DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The driver shifts out of Reverse Gear and into any other gear while Trailer Backup Assist (TBA) is active. The camera image feed remains displayed to the driver as long as TBA is active and vehicle speed does not exceed limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display stops showing Rear View Camera image when TBA is no longer active or vehicle speed exceeds limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### TRG-UC-REQ-102959/A-Vehicle Not in RUN/START

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as normal usage use case |
| Scenario Description | Driver moves the ignition out of Run/Start |
| Post-conditions | The vehicle returns to non-TRG behavior |
| List of Exception Use Cases | N/A |
| Interfaces | G-HMI Vehicle System Interface |

##### TRG-UC-REQ-102960/A-Loss of communication with IPMB Module

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as normal usage use case |
| Scenario Description | Driver attempts to do something in TRG and APIM has lost communication with IPMB |
| Post-conditions | The vehicle shows camera view, without any overlays |
| List of Exception Use Cases | N/A |
| Interfaces | G-HMI Vehicle System Interface |

##### TRG-UC-REQ-102961/A-Valid Camera Video Signal not present

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as normal usage use case |
| Scenario Description | Driver attempts to do something in TRG and APIM is not receiving valid video |
| Post-conditions | The vehicle returns to non-TRG behavior |
| List of Exception Use Cases | N/A |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### RVCv2-ACT-REQ-014102/A-Activate Rear View Camera (TcSE ROIN-282388-2)

Linked Elements

RVCv2-SD-REQ-014104/B-Activate Rear View Camera (TcSE ROIN-282316-2)

Activity Diagram

###### RVCv2-ACT-REQ-014103/A-Deactivate Rear View Camera (TcSE ROIN-286992-1)

Linked Elements

RVCv2-SD-REQ-014105/B-Deactivate Rear View Camera (TcSE ROIN-282323-2)

Activity Diagram

##### Sequence Diagrams

###### RVCv2-SD-REQ-014104/B-Activate Rear View Camera (TcSE ROIN-282316-2)

Scenario

Normal Usage

The user activates the Rear View Camera (RVC) by placing the vehicle in Reverse Gear.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

HMI Display shows the RVC image

Sequence Diagram

###### RVCv2-SD-REQ-014105/B-Deactivate Rear View Camera (TcSE ROIN-282323-2)

Scenario

Normal Usage

The user deactivates the Rear View Camera (RVC) by taking the vehicle out of Reverse Gear.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

HMI Display stops showing the RVC image

Sequence Diagram

### RVCv2-FUN-REQ-014106/A-Rear View Camera Zoom Setting (TcSE ROIN-293363)

#### Use Cases

##### RVC-UC-REQ-014107/A-Select Manual Zoom Level X (TcSE ROIN-289799)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle display is showing the Rear View Camera image. |
| Scenario Description | The driver activates Manual Zoom Mode Level X via the HMI interface. |
| Post-conditions | The vehicle display continues to show the Rear View Camera image. The vehicle display indicates that a zoom level is selected. The video feed from the Rear View Camera contains a zoomed-in image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |
| Notes | There are three defined zoom levels and "Level X" is used to generically designate that one of the three is selected as described in this use case. Refer to HMI documentation (requirements and/or screen-flow) for which level(s) of zoom will be utilized. |

##### RVC-UC-REQ-014108/A-Deactivate Manual Zoom (TcSE ROIN-289802)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle display is showing the Rear View Camera image with Zoom Level X selected. |
| Scenario Description | The user deactivates Manual Zoom Mode via HMI interface. |
| Post-conditions | The vehicle display continues to show the Rear View Camera image. The vehicle display indicates that no zoom level is selected. The video feed from the Rear View Camera contains a normal (no zoom applied) image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |
| Notes | There are three defined zoom levels and "Level X" is used to generically designate that one of the three is selected as described in this use case. Refer to HMI documentation (requirements and/or screen-flow) for which level(s) of zoom will be utilized. |

#### White Box View

##### Activity Diagrams

###### RVCv2-ACT-REQ-014109/A-Manual Zoom (TcSE ROIN-282390-1)

Linked Elements

RVCv2-SD-REQ-014110/A-Manual Zoom (TcSE ROIN-282356-1)

Activity Diagram

##### Sequence Diagrams

###### RVCv2-SD-REQ-014110/A-Manual Zoom (TcSE ROIN-282356-1)

Scenario

Normal Usage

The user changes the manual zoom level setting via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Pre-condition

HMI Display is showing RVC Image.

Post-condition

The HMI display is showing the Rear Video Camera image that reflects the updated zoom setting.

Sequence Diagram

### RVCv3-FUN-REQ-127106/A-Rear View Camera Zoom Setting v3

#### Use Cases

##### RVCv3-UC-REQ-128173/A-Press Zoom Button from Rear 360 View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - In reverse and in Multicamera RVC view |
| Scenario Description | The driver presses the Zoom button |
| Post-conditions | The RVC zoom view appears with Multicamera view still being highlighted |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128176/A-Press Zoom Button from Rear Normal View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with camera - The vehicle is in RUN/START - RVC view shown |
| Scenario Description | The Driver presses the Zoom button |
| Post-conditions | The RVC zoom view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128179/A-Press Zoom Button from CHMSL View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicke is configured with CHMSL camera - The vehicle is in Run/Start - The vehicle in Reverse - CHMSL View is shown |
| Scenario Description | The driver presses Zoom Button |
| Post-conditions | CHMSL Zoom view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128188/A-Press Rear 360 Unzoom

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in Run/Start - Rear Zoom Shown (from Rear Multicamera) |
| Scenario Description | The driver presses soft Button to unzoom |
| Post-conditions | Rear Multicamera shows |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128189/A-Press Rear Normal Unzoom

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with any camera - The vehicle is in Run/Start - Rear Zoom Shown (from Rear Normal) |
| Scenario Description | The driver presses soft Button to unzoom |
| Post-conditions | Rear Normal shows |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128190/A-Press CHMSL Unzoom

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with CHMSL camera - The vehicle is in Run/Start - CHMSL Zoom Shown |
| Scenario Description | The driver presses soft Button to unzoom |
| Post-conditions | Rear Normal shows |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### RVCv3-ACT-REQ-127095/A-Manual Zoom

Linked Elements

RVCv2-SD-REQ-014110/A-Manual Zoom (TcSE ROIN-282356-1)

Activity Diagram

##### Sequence Diagrams

###### RVCv3-SD-REQ-127097/A-Manual Zoom

Scenario

Normal Usage

The user changes the manual zoom setting via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Pre-condition

HMI Display is showing RVC Image.

Post-condition

The HMI display is showing the Rear Video Camera image that reflects the updated zoom setting.

Sequence Diagram

### RVCv2-FUN-REQ-014111/A-Rear View Camera Delay Mode Setting (TcSE ROIN-293368)

#### Use Cases

##### RVC-UC-REQ-014112/A-Activate/Deactivate Rear Camera Delay (TcSE ROIN-289803)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver activates/deactivates the Rear View Camera (RVC) Delay Mode via the HMI interface. |
| Post-conditions | The RVC Delay Mode is activated/deactivated. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

### RVCv2-FUN-REQ-014113/A-Camera Image View (TcSE ROIN-293370)

#### Use Cases

##### RVCv2-UC-REQ-014114/A-Select Rear Camera Split-View (TcSE ROIN-289805)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle display is showing the Rear View Camera image. |
| Scenario Description | The driver selects split-view image via hard switch interface. |
| Post-conditions | The vehicle display continues to show the Rear View Camera image. The video feed from the Rear View Camera contains a split-view image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### RVCv2-UC-REQ-014115/A-Select Rear Camera 360-View (TcSE ROIN-289806)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle display is showing the Rear View Camera image. |
| Scenario Description | The driver selects 360-view image via hard switch interface. |
| Post-conditions | The vehicle display continues to show the Rear View Camera image. The video feed from the Rear View Camera contains a 360-view image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### RVCv2-UC-REQ-014116/A-Select Rear Camera Normal View (TcSE ROIN-289807)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle display is showing the Rear View Camera image. |
| Scenario Description | The driver selects normal view image via hard switch interface. |
| Post-conditions | The vehicle display continues to show the Rear View Camera image. The video feed from the Rear View Camera contains a normal view image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### RVCv2-ACT-REQ-014117/A-Change Camera View Setting (TcSE ROIN-282392-2)

Linked Elements

RVCv2-SD-REQ-014118/A-Change Camera View Setting (TcSE ROIN-282363-3)

Activity Diagram

##### Sequence Diagrams

###### RVCv2-SD-REQ-014118/A-Change Camera View Setting (TcSE ROIN-282363-3)

Scenario

Normal Usage

The user selects alternate view image setting via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

The HMI display is showing the Rear Video Camera image that reflects the updated alternate view setting.

Sequence Diagram

### RVCv3-FUN-REQ-127111/A-Camera Image View v3

#### Use Cases

##### RVCv3-UC-REQ-127874/A-Press Rear 360 View Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Reverse camera shown and not in Rear Multicamera view. |
| Scenario Description | The driver presses Multicamera rear button |
| Post-conditions | Multicamera RVC view appears with overlays |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128175/A-Press RVC Normal View Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with non- Multicamera - The vehicle is in RUN/START - In reverse and not in RVC view |
| Scenario Description | The Driver presses the RVC button |
| Post-conditions | RVC view appears with overlays |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128177/A-Press RVC Split View Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured as Multicamera with Rear Split View - The Vehicle is in Run/Start - RVC View being shown, and not in RVC Split view |
| Scenario Description | The Driver presses RVC Split view button |
| Post-conditions | RVC Split view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128178/A-Press CHMSL Camera View Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicke is configured with CHMSL camera - Vehicle is in Run/Start - Rear Camera shown and Not in CHMSL |
| Scenario Description | The driver presses CHMSL view button |
| Post-conditions | CHMSL view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128180/B-Press Aux Camera view Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Vehicle in Run/Start - Vehicle is in Reverse, Park, Netural, or Drive - The CHMSL view is not showing - Vehicle below 10kph |
| Scenario Description | - User presses camera hard button IPMB sends “Camera Showing (81B): 0x0D” |
| Post-conditions | The Aux view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128181/A-Press Trailer Reverse Guidance Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with TRG - The vehicle is in Run/Start - The vehicle is in Reverse - TRG is not activated |
| Scenario Description | The driver presses TRG view button |
| Post-conditions | TRG scenarios enter; see TRG Use cases for more details |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128396/B-Press Hard Button to Enter CHMSL View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Vehicle in Run/Start - Vehicle is in Reverse, Park, Neutral, or Drive. - The CHMSL view is not showing. - Vehicle below 10kph. |
| Scenario Description | - User presses camera hard button - IPMB sends “Camera Showing (81B): 0x0D” |
| Post-conditions | CHMSL view is shown |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128397/A-Press Hard Button to Enter RVC View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with CHMSL, Rear Camera and Soft button strategy - The vehicle is in Run/Start - CHMSL or CHMSL Zoom is shown |
| Scenario Description | The driver presses Hard Camera Button to enter Rear Normal View |
| Post-conditions | Rear Normal shows |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### RVCv3-ACT-REQ-127096/A-Change Camera View Setting

Linked Elements

RVCv2-SD-REQ-014118/A-Change Camera View Setting (TcSE ROIN-282363-3)

Activity Diagram

##### Sequence Diagrams

###### RVCv3-SD-REQ-127098/A-Change Camera View Setting

Scenario

Normal Usage

The user selects alternate view image setting via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

The HMI display is showing the Rear Video Camera image that reflects the updated alternate view setting.

Sequence Diagram

### RVCv2-FUN-REQ-014119/A-Activate/Deactivate Enhanced Park Aids (TcSE ROIN-293375)

#### Requirements

##### RVCv2-REQ-014120/A-Visual Park Aid Alert Availability (TcSE ROIN-281600-2)

The Rear Video Camera and the HMI Display must both be configured with Park Aid Module present in order to enable the Visual Park Aid Alert setting options (HMI refers to this as Enhanced Park Aids and this refers to Visual Park Aid Alert or Visual Park Assist depending on vehicle configuration. See HMI specification for implementation details).

#### Use Cases

##### RVC-UC-REQ-014121/A-Activate/Deactivate Enhanced Park Aids (TcSE ROIN-289804)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver activates/deactivates the Enhanced Park Aids via the HMI interface. |
| Post-conditions | The Enhance Park Aids are activated/deactivated. The HMI indicates the setting change determined by vehicle system interface signal. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### RVCv2-ACT-REQ-014122/A-Activate/Deactivate Enhanced Park Aids (TcSE ROIN-286990-1)

Linked Elements

RVCv2-SD-REQ-014123/A-Activate/Deactivate Enhanced Park Aids (TcSE ROIN-286982-2)

Activity Diagram

##### Sequence Diagrams

###### RVCv2-SD-REQ-014123/A-Activate/Deactivate Enhanced Park Aids (TcSE ROIN-286982-2)

Scenario

Normal Usage

The user activates/deactivates Enhanced Park Aids via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

Enhanced Park Aids are enabled/disabled.

Sequence Diagram

### RVCv2-FUN-REQ-014124/A-Camera Overlay Setting (TcSE ROIN-293357)

#### Requirements

##### RVC-FUR-REQ-014125/A-Guideline Availability (TcSE ROIN-264660-2)

	In order for Active (Dynamic) Guidelines to be available, the vehicle must be equipped with an Absolute Steering Angle Sensor and both the Rear Video Camera and the HMI Display must be configured with Absolute Steering Angle Sensor present.

	When Active (Dynamic) Guidelines are active, Fixed (Static) Guidelines must also be active.

	When Fixed (Static) Guidelines are inactive, Active (Dynamic) Guidelines must also be inactive.

#### Use Cases

##### RVCv2-UC-REQ-014126/A-Select Global Overlay Setting (All Guidelines) (TcSE ROIN-289808)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver activates/deactivates all Rear View Camera overlays (guidelines) via the HMI interface. |
| Post-conditions | The vehicle display indicates the selected overlay setting. The video feed from the Rear View Camera contains an image with or without all guideline overlays as indicated by the selected setting. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |
| Notes | Use Case is optional. Current direction is to not apply guideline related use cases. |

##### RVCv2-UC-REQ-014127/A-Select Active Guideline Setting (TcSE ROIN-289809)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver activates/deactivates the Active (Dynamic) Guidelines Setting via the HMI interface. |
| Post-conditions | The vehicle display indicates the selected Active Guideline setting. The video feed from the Rear View Camera contains an image with or without Active guideline overlays as indicated by the selected setting. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |
| Notes | Use Case is optional. Current direction is to not apply guideline related use cases. |

##### RVCv2-UC-REQ-014128/A-Select Fixed Guideline Setting (TcSE ROIN-289810)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver activates/deactivates the Fixed (Static) Guidelines Setting via the HMI interface. |
| Post-conditions | The vehicle display indicates the selected Fixed Guideline setting. The video feed from the Rear View Camera contains an image with or without Fixed guideline overlays as indicated by the selected setting. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |
| Notes | Use Case is optional. Current direction is to not apply guideline related use cases. |

#### White Box View

##### Activity Diagrams

###### RVCv2-ACT-REQ-014129/A-Change Camera Overlay Setting (TcSE ROIN-282386-2)

Linked Elements

RVCv2-SD-REQ-014130/A-Activate Overlays (TcSE ROIN-282334-2)

RVCv2-SD-REQ-014131/A-Deactivate Overlays (TcSE ROIN-282341-2)

Activity Diagram

##### Sequence Diagrams

###### RVCv2-SD-REQ-014130/A-Activate Overlays (TcSE ROIN-282334-2)

Scenario

Normal Usage

The user activates Rear View Camera overlays via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

Rear View Camera overlays are enabled.

Sequence Diagram

###### RVCv2-SD-REQ-014131/A-Deactivate Overlays (TcSE ROIN-282341-2)

Scenario

Normal Usage

The user deactivates Rear View Camera overlays via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

Rear View Camera overlays are disabled.

Sequence Diagram

## Driver Assist Front View Camera

### DAFVCv1-FUN-REQ-014045/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-293385)

#### Requirements

##### DAFVCv1-FUR-REQ-166641/A-Deactivate FVC

The DriverAssistFrontViewCameraClient shall stop displaying Front View Camera (FVC) image when one of the following conditions is met:

•	Vehicle is shifted into 'Reverse' (Rear View Camera image will be shown)

•	Vehicle is shifted into 'Park' (No Image shown in 'Park')

•	Power Mode does not equal IgnitionOn_2 or Running_2 or Crank_3

• 	FVC is disabled (Feature_St equals 0x81B Camera Showing and 0x00 Off)

•	Vehicle speed exceeds Feature Maximum Speed requirement 014077

##### DAFVCv1-FUR-REQ-013997/B-E-Locker Deactivation Delay (TcSE ROIN-266607-1)

If the FrontViewCameraClient is displaying the Front View Camera (FVC) image (all conditions for front camera image display are true) and the E-Locker transistions from 'Locked' (Elocker_St = 0x1:ON) to 'Unlocked' (Elocker_St = 0x0:OFF), the FrontViewCameraClient shall start a timer (T_eLockerDelay) and shall continue to display the FVC image until the expiration of this timer.

If the FrontViewCameraClient detects that the E-Locker transistions back to 'Locked' prior to the expiration of T_eLockerDelay, and all other conditions for front camera image remain true, the FrontViewCameraClient shall cancel the timer and continue to display the FVC image.

##### DAFVCv1-TMR-REQ-013998/B-T_eLockerDelay (TcSE ROIN-266609-1)

| Name | Description | Units | Range | Resolution | Default |
| --- | --- | --- | --- | --- | --- |
| T_eLockerDelay | Maximum time FrontViewCameraClient should wait before stopping the display of the FVC video image to the user according to FAS-FVC-GREQ-266607-E-Locker Deactivation Delay. | msec | 4975-5025 | 5 | 5000 |

#### Use Cases

##### DAFVCv1-UC-REQ-014046/B-Activate Driver Assist Front View Camera (TcSE ROIN-290143)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The gear position status is NOT Reverse. |
| Scenario Description | The driver activates the Driver Assist Front View Camera (DAFVC) via hard button interface, or vehicle system indicates DAFVC activation for Trailer Backup Assist, Active Park Assist or Off Road Front Camera. |
| Post-conditions | The vehicle display shows the DAFVC image. |
| List of Exception Use Cases | E1 – Driver Assist Front View Camera Malfunction E2 – Activation Attempt During Overspeed Condition |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### DAFVCv1-UC-REQ-014047/A-Driver Assist Front View Camera Malfunction (TcSE ROIN-290144)

Linked Elements

DAFVCv1-UC-REQ-014046/B-Activate Driver Assist Front View Camera (TcSE ROIN-290143)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case |
| Scenario Description | The driver attempts to activate the Driver Assist Front View Camera (DAFVC) via hard button interface. The HMI interface indicates that the DAFVC image cannot be shown because of a malfunction. |
| Post-conditions | The vehicle display does NOT show the DAFVC image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### DAFVCv1-UC-REQ-014048/B-Activation Attempt During Overspeed Condition (TcSE ROIN-290145)

Linked Elements

DAFVCv1-UC-REQ-014046/B-Activate Driver Assist Front View Camera (TcSE ROIN-290143)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case |
| Scenario Description | The driver attempts to activate the Driver Assist Front View Camera (DAFVC) via hard button interface while traveling at a speed greater than the limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display does NOT show the DAFVC image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle system display is showing front view camera image |
| Scenario Description | The user deactivates the Driver Assist Front View Camera (DAFVC) by one of the following: - Disabling DAFVC via hard button interface - Exceeding the speed limit per CAMERA-REQ-014077-Feature Maximum Speed - Shifting the vehicle into Reverse (RVC image will be shown if conditions for RVC allow it) - Shifting the vehicle into Park (No image shown in ‘Park’) |
| Post-conditions | The vehicle display does NOT show the DAFVC image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### DAFVCv1-ACT-REQ-014043/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-282606-3)

Linked Elements

DAFVCv1-SD-REQ-014051/A-Activation Attempt During Overspeed (TcSE ROIN-287030-1)

DAFVCv1-SD-REQ-014050/C-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2)

Activity Diagram

##### Sequence Diagrams

###### REQ-331856/B-Lin Button Camera Request

###### DAFVCv1-SD-REQ-014050/C-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2)

Sequence Diagram

###### DAFVCv1-SD-REQ-014051/A-Activation Attempt During Overspeed (TcSE ROIN-287030-1)

Scenario

Normal Usage

The user attempts to activate the Driver Assist Front View Camera (DAFVC) by pressing the hard switch while exceeding the speed limit per CAMERA-REQ-014077-Feature Maximum Speed.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

HMI Display shows an over-speed notification.

Sequence Diagram

### DAFVCv2-FUN-REQ-128309/B-Activate/Deactivate Driver Assist Front View Camera v2

#### Requirements

##### DAFVCv1-FUR-REQ-166641/A-Deactivate FVC

The DriverAssistFrontViewCameraClient shall stop displaying Front View Camera (FVC) image when one of the following conditions is met:

•	Vehicle is shifted into 'Reverse' (Rear View Camera image will be shown)

•	Vehicle is shifted into 'Park' (No Image shown in 'Park')

•	Power Mode does not equal IgnitionOn_2 or Running_2 or Crank_3

• 	FVC is disabled (Feature_St equals 0x81B Camera Showing and 0x00 Off)

•	Vehicle speed exceeds Feature Maximum Speed requirement 014077

##### DAFVCv1-FUR-REQ-013997/B-E-Locker Deactivation Delay (TcSE ROIN-266607-1)

If the FrontViewCameraClient is displaying the Front View Camera (FVC) image (all conditions for front camera image display are true) and the E-Locker transistions from 'Locked' (Elocker_St = 0x1:ON) to 'Unlocked' (Elocker_St = 0x0:OFF), the FrontViewCameraClient shall start a timer (T_eLockerDelay) and shall continue to display the FVC image until the expiration of this timer.

If the FrontViewCameraClient detects that the E-Locker transistions back to 'Locked' prior to the expiration of T_eLockerDelay, and all other conditions for front camera image remain true, the FrontViewCameraClient shall cancel the timer and continue to display the FVC image.

##### DAFVCv1-TMR-REQ-013998/B-T_eLockerDelay (TcSE ROIN-266609-1)

| Name | Description | Units | Range | Resolution | Default |
| --- | --- | --- | --- | --- | --- |
| T_eLockerDelay | Maximum time FrontViewCameraClient should wait before stopping the display of the FVC video image to the user according to FAS-FVC-GREQ-266607-E-Locker Deactivation Delay. | msec | 4975-5025 | 5 | 5000 |

#### Use Cases

##### DAFVCv2-UC-REQ-128182/B-Entering Front Camera - 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera or FVC only - The vehicle is in Run/Start - The vehicle is NOT in reverse - The vehicle speed is below limit per CAMERA-REQ-014077-Feature Maximum Speed |
| Scenario Description | The driver presses Front Camera Hard Button |
| Post-conditions | Front Multicamera view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### DAFVCv2-UC-REQ-128184/B-Entering Front Camera – Non-360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - APIM is configured with FVC only (no Multicamera) - Vehicle in Run/Start - Vehicle Not in reverse - Vehicle speed per CAMERA-REQ-014077-Feature Maximum Speed |
| Scenario Description | - User presses Camera Hard Button |
| Post-conditions | Front Normal view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### DAFVCv1-UC-REQ-128312/B-Activation Attempt During Overspeed Condition

Linked Elements

DAFVCv1-UC-REQ-128310/A-Activate Driver Assist Front View Camera

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case |
| Scenario Description | The driver attempts to activate the Driver Assist Front View Camera (DAFVC) via hard button interface while traveling at a speed greater than the limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display does NOT show the DAFVC image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### TRG-UC-REQ-102959/A-Vehicle Not in RUN/START

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as normal usage use case |
| Scenario Description | Driver moves the ignition out of Run/Start |
| Post-conditions | The vehicle returns to non-TRG behavior |
| List of Exception Use Cases | N/A |
| Interfaces | G-HMI Vehicle System Interface |

##### TRG-UC-REQ-102960/A-Loss of communication with IPMB Module

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as normal usage use case |
| Scenario Description | Driver attempts to do something in TRG and APIM has lost communication with IPMB |
| Post-conditions | The vehicle shows camera view, without any overlays |
| List of Exception Use Cases | N/A |
| Interfaces | G-HMI Vehicle System Interface |

##### TRG-UC-REQ-102961/A-Valid Camera Video Signal not present

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as normal usage use case |
| Scenario Description | Driver attempts to do something in TRG and APIM is not receiving valid video |
| Post-conditions | The vehicle returns to non-TRG behavior |
| List of Exception Use Cases | N/A |
| Interfaces | G-HMI Vehicle System Interface |

##### DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle system display is showing front view camera image |
| Scenario Description | The user deactivates the Driver Assist Front View Camera (DAFVC) by one of the following: - Disabling DAFVC via hard button interface - Exceeding the speed limit per CAMERA-REQ-014077-Feature Maximum Speed - Shifting the vehicle into Reverse (RVC image will be shown if conditions for RVC allow it) - Shifting the vehicle into Park (No image shown in ‘Park’) |
| Post-conditions | The vehicle display does NOT show the DAFVC image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### DAFVCv1-ACT-REQ-014043/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-282606-3)

Linked Elements

DAFVCv1-SD-REQ-014051/A-Activation Attempt During Overspeed (TcSE ROIN-287030-1)

DAFVCv1-SD-REQ-014050/C-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2)

Activity Diagram

##### Sequence Diagrams

###### REQ-331856/B-Lin Button Camera Request

###### DAFVCv1-SD-REQ-014050/C-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2)

Sequence Diagram

###### DAFVCv1-SD-REQ-014051/A-Activation Attempt During Overspeed (TcSE ROIN-287030-1)

Scenario

Normal Usage

The user attempts to activate the Driver Assist Front View Camera (DAFVC) by pressing the hard switch while exceeding the speed limit per CAMERA-REQ-014077-Feature Maximum Speed.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

HMI Display shows an over-speed notification.

Sequence Diagram

### DAFVCv1-FUN-REQ-014052/A-Driver Assist Front Camera Image View (TcSE ROIN-293390)

#### Use Cases

##### DAFVCv1-UC-REQ-014053/A-Select Front Camera Split-View (TcSE ROIN-290147)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The gear position status is NOT Reverse. The vehicle display is showing the Driver Assist Front View Camera image. |
| Scenario Description | The driver selects split-view image setting via hard switch interface. |
| Post-conditions | The vehicle display continues to show the Driver Assist Front View Camera image. The video feed from the Driver Assist Front View Camera contains a split-view image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### DAFVCv1-UC-REQ-014054/A-Select Front Camera 360-View (TcSE ROIN-290148)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The gear position status is NOT Reverse. The vehicle display is showing the Driver Assist Front View Camera image. |
| Scenario Description | The driver selects 360-view image setting via hard switch interface. |
| Post-conditions | The vehicle display continues to show the Driver Assist Front View Camera image. The video feed from the Driver Assist Front View Camera contains a 360-view image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### DAFVCv1-UC-REQ-014055/A-Select Front Camera Normal View (TcSE ROIN-290149)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The gear position status is NOT Reverse. The vehicle display is showing the Driver Assist Front View Camera image. |
| Scenario Description | The driver selects normal view image setting via hard switch interface. |
| Post-conditions | The vehicle display continues to show the Driver Assist Front View Camera image. The video feed from the Driver Assist Front View Camera contains a normal view image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### DAFVCv1-UC-REQ-014056/A-Select Front Camera Image Off (TcSE ROIN-290150)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The gear position status is NOT Reverse. The vehicle display is showing the Driver Assist Front View Camera image. |
| Scenario Description | The driver selects image off setting via hard switch interface. |
| Post-conditions | The vehicle display returns to the state previous to entering Driver Assist Front View Camera. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### DAFVCv1-ACT-REQ-014044/A-Change Camera View Setting (TcSE ROIN-282608-2)

Linked Elements

DAFVCv1-SD-REQ-014057/A-Change Camera View Setting (TcSE ROIN-282599-3)

Activity Diagram

##### Sequence Diagrams

###### DAFVCv1-SD-REQ-014057/A-Change Camera View Setting (TcSE ROIN-282599-3)

Scenario

Normal Usage

The user selects alternate view image setting via hard switch interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

The HMI display is showing the Driver Assist Front Video Camera image that reflects the updated alternate view setting.

Sequence Diagram

### DAFVCv2-FUN-REQ-127102/A-Driver Assist Front Camera Image View v2

#### Use Cases

##### DAFVCv2-UC-REQ-128183/A-Press Front 360 View Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera or FVC only - The vehicle is in Run/Start - Front Camera is shown, and not in Front 360 view. |
| Scenario Description | The user presses Front 360 soft Button |
| Post-conditions | Front 360 view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### DAFVCv2-UC-REQ-128185/A-Press Front Normal View Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with front camera - The vehicle is in Run/Start - Front Camera View is shown but not in Front Normal View |
| Scenario Description | The driver presses Front Normal view soft Button |
| Post-conditions | Front Normal View shown. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### DAFVCv2-UC-REQ-128186/A-Press Front Split View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with front camera - The vehicle is in Run/Start - Front Camera view is shown, but not in Front Split View |
| Scenario Description | Thre driver presses Front Split soft Button |
| Post-conditions | Front Split View shown |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### DAFVCv2-UC-REQ-128187/A-Press Rear button From Front camera

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with front camera - The vehicle is in Run/Start - Front Camera view is shown |
| Scenario Description | The Driver presses rear soft Button |
| Post-conditions | Rear Camera View shows |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### DAFVCv2-ACT-REQ-127100/A-Change Camera View Setting

Linked Elements

DAFVCv1-SD-REQ-014057/A-Change Camera View Setting (TcSE ROIN-282599-3)

Activity Diagram

##### Sequence Diagrams

###### DAFVCv2-SD-REQ-127099/A-Change Camera View Setting

Scenario

Normal Usage

The user selects alternate view image setting via soft button interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

The HMI display is showing the Camera image that reflects the updated alternate view setting.

Sequence Diagram

## CAMERA-FUN-REQ-331348/A-Offset View

### CAMERA-UC-REQ-331349/A-Press Rear Offset View Button from Rear 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Rear 360 is shown |
| Scenario Description | The driver presses rear offset view button |
| Post-conditions | Rear RWD Offset View appears with overlays |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

### CAMERA-UC-REQ-331350/A-Press Rear Offset View Button from Front 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Front 360 view is shown |
| Scenario Description | The driver presses rear offset view button |
| Post-conditions | Front RWD Offset View appears with overlays |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

### CAMERA-UC-REQ-331351/A-Press Rear Left Corner View Button from Rear 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Rear 360 camera is shown |
| Scenario Description | The driver presses rear left corner view button |
| Post-conditions | Multicamera Rear_RL_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331352/A-Press Rear Left Corner View Button from Front 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Front 360 camera is shown |
| Scenario Description | The driver presses rear left corner view button |
| Post-conditions | Multicamera Front_RL_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331353/A-Press Rear Right Corner View Button from Rear 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Rear 360 camera is shown |
| Scenario Description | The driver presses rear right corner view button |
| Post-conditions | Multicamera Rear_RR_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331354/A-Press Rear Right Corner View Button from Front 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Front 360 camera is shown |
| Scenario Description | The driver presses rear right corner view button |
| Post-conditions | Multicamera Front_RR_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331356/A-Press Front Offset View Button from Rear 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Rear 360 is shown |
| Scenario Description | The driver presses front offset view button |
| Post-conditions | Rear FWD Offset View appears with overlays |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

### CAMERA-UC-REQ-331360/A-Press Front Offset View Button from Front 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Front 360 view is shown |
| Scenario Description | The driver presses front offset view button |
| Post-conditions | Front FWD Offset View appears with overlays |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

### CAMERA-UC-REQ-331365/A-Press Front Left Corner View Button from Rear 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Rear 360 camera is shown |
| Scenario Description | The driver presses rear left corner view button |
| Post-conditions | Multicamera Rear_FL_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331369/A-Press Front Left Corner View Button from Front 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Front 360 camera is shown |
| Scenario Description | The driver presses front left corner view button |
| Post-conditions | Multicamera Front_FL_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331373/A-Press Front Right Corner View Button from Rear 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Rear 360 camera is shown |
| Scenario Description | The driver presses front right corner view button |
| Post-conditions | Multicamera Rear_FR_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331374/A-Press Front Right Corner View Button from Front 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Front 360 camera is shown |
| Scenario Description | The driver presses front right corner view button |
| Post-conditions | Multicamera Front_FR_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

## Views At Speed

### CAMERA-FUN-REQ-354982/A-Views At Speed

#### Requirements

##### CAMERA-REQ-354992/A-Views At Speed Availability

Views At Speed are a set of views that are available even when vehicle reaches speeds above those mentioned in req 014077, while not in reverse drive.

For some implementations the views will deactivate under the speed limitations of req 014077, the user will be able to reactivate the applicable views again by button press.

The user is able to request a view change and Server will decide on what kind of views to display.

Available views are setup through Method II configuration.

##### CAMERA-REQ-354993/A-Views At Speed List

Views at Speed include the following views:

| View Name | Config Value |
| --- | --- |
| CHMSL | 0x0B |
| Aux | 0x0D |
| Hitch | 0x26 |
| RCOD* | 0x27 |

*This is the RCOD in multicamera SPSS with Config value 0x27, not to be confused with Rear Camera On Demand SPSS which also is commonly referred to RCOD.

##### CAMERA-REQ-358450/A-Off Road Mode Views At Speed

When the vehicle is configured for off road mode views at speed and off road mode is active, there is no max speed limit for showing Off Road Front Camera Image.

#### Use Cases

##### CAMERA-UC-REQ-355016/A-Views At Speed Trigger

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. Vehicle is in Drive above 10kph. Vehicle is equipped with 360 camera with any or all of the views at speed: Aux, CHMSL, HITCH, RCOD No camera view is active. |
| Scenario Description | Customer presses Camera button. |
| Post-conditions | Client displays available view at speed per camera request. |
| List of Exception Use Cases |  |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box Views

##### Activity Diagram

###### CAMERA-ACT-REQ-358468/A-Views At Speed

##### Sequence Diagram

###### CAMERA-SD-REQ-354994/A-Views At Speed

# Appendix: Reference Documents

| Reference # | Document Title |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |