Research & Vehicle Technology

“Infotainment Systems Product Development”

Feature – Smart Charging

Infotainment Subsystem Part Specific Specification (SPSS)

Version 1.4

UNCONTROLLED COPY IF PRINTED

Version Date: December 12, 2019

FORD CONFIDENTIALF

# Revision History

| Date | Version | Notes |  |
| --- | --- | --- | --- |
| November 9, 2018 | 0.1 | Draft |  |
| December 13, 2018 | 1.0 | Initial Release |  |
| February 12, 2019 | 1.1 |  |  |
|  | SCG-CLD-REQ-333261/B-SC Client |  | rpaquet2 - Added functional requirements |
|  | 619845/A-Functional Requirements |  | rpaquet2 - New |
|  | SCG-REQ-343119/A-Invalid Values Associated with Unfilled Smart Charging Locations |  | rpaquet2 - New |
|  | SCG-REQ-343050/A-Out of Range Values for Invalid Smart Charging Locations |  | rpaquet2 - New |
|  | SCG-REQ-343727/A-Out of Range Values for Unchanged Minimum SOC |  | rpaquet2 - New |
|  | SCG-STR-598559/B-Logical Signal Mapping |  | rpaquet2 - Updated CAn Signal names |
|  | SCG-IIR-REQ-333303/B-SC Client_Tx |  | rpaquet 2- Added 339246, 343026, 304071, 304072, 342970 removed 333213, 333222 |
|  | MD-REQ-333219/B-SCSettings_Rq |  | rpaquet2 - Update signal name |
|  | MD-REQ-339246/A-setSCLocationNameInfo |  | rpaquet2- Updated |
|  | MD-REQ-304071/A-getEcgDidData |  | MBORREL4: New req. |
|  | MD-REQ-304072/A-getEcgDidUpdateData |  | MBORREL4: New req. |
|  | MD-REQ-342970/A-ClearAllUserSettings_Rq |  | rpaquet2 - New |
|  | MD-REQ-343026/A-ChargeStationID_Rq |  | rpaquet2 - new |
|  | SCG-IIR-REQ-333304/B-SC Client_Rx |  | rpaquet 2- Added 304071, 326679, 343299 removed 333212, 333228 |
|  | MD-REQ-337016/B-ChargeStationSchedule_St |  | rpaquet2 - Update the Time interval size and restructured the vector |
|  | MD-REQ-333227/B-SCMinimumSOC_St |  | rpaquet2 - Corrected spelling |
|  | MD-REQ-333225/B-SCSettingsUpdate_St |  | rpaquet2 - changed signal names |
|  | MD-REQ-343299/A-CurntTargetSOC_St |  | rpaquet2 - New |
|  | SCG-IIR-REQ-333305/B-SC Onboard HMI Client_Tx |  | rpaquet2 - 333221 was in here two times removed one of them added 304071 |
|  | MD-REQ-333221/B-SCMinimumSOC_Rq |  | rpaquet2 - corrected spelling |
|  | SCG-IIR-REQ-333306/B-SC Onboard HMI Client_Rx |  | rpaquet2 - Added 339246, 304071, 304072, 343299 removed 333212, 333228 |
|  | SCG-STR-597770/B-Requirements |  | rpaquet2 - removed 335184, 335187, 335209, 335210, 335186, 335217 |
|  | SCG-REQ-328705/B-Gateway Module Receives Activation Command from the Cloud |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-328706/B-Gateway Module sends command response to the cloud (CVFMA) stating it received the updated DID to activate |  | rpaquet2 - Updated requirement name and text per feature owner |
|  | SCG-REQ-328707/B-Gateway Module alerts the Cloud (CVFMA) Smart Charging DID successfully updated for Activation |  | rpaquet2 - Updated requirement name and text per feature owner |
|  | SCG-REQ-335185/B-Vehicle HMI Displays SC On/Off Toggle after Activation |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335193/B-Cloud to Gateway Module Activation Command Time Out |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335194/B-Activation Failure Response to the Cloud |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335196/B-Smart Charging is Deactivated by Default |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335206/B-Gateway Module receives Deactivation Command from the Cloud |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335208/B-Gateway Module sends command response to the Cloud (CVFMA) stating it received the updated DID to Deactivate |  | rpaquet2 - Updated requirement name and text per feature owner |
|  | SCG-REQ-335212/B-Vehicle HMI hides the SC On/Off Toggle after Deactivation |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335218/B-Gateway Module returns Successful Alert of Deactivation |  | rpaquet2 - Updated requirement name and text per feature owner |
|  | SCG-REQ-335220/B-Cloud to Gateway Module DeactivationCommand Time Out |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335221/B-Deactivation Failure Response to the Cloud |  | rpaquet2 - Updated text per feature owner |
|  | SCG-UC-REQ-333253/B-Cloud Activates Smart Charging |  | rpaquet2 - Updated post condition and interfaces |
|  | SCG-UC-REQ-333254/B-Cloud Deactivates Smart Charging |  | rpaquet2 - Updated post condition and interfaces |
|  | SCG-UC-REQ-333255/B-Activation or Deactivation Failure |  | rpaquet2 - Updated post condition and interfaces |
|  | SCG-UC-REQ-333256/B-Activation or Deactivation Timeout |  | rpaquet2 - Updated post condition and interfaces |
|  | SCG-ACT-REQ-333307/B-Activate Deactivate Smart Charging |  | rpaquet2 - Updated diagram |
|  | SCG-STR-597774/B-Sequence Diagrams |  | rpaquet2 - Added 342972 |
|  | SCG-SD-REQ-334521/B-Activate or Deactivate Smart Charging |  | rpaquet2 - Updated diagram |
|  | SCG-SD-REQ-342972/A-Request DID Info on Wake Up |  | rpaquet2 - New |
|  | SCG-STR-584574/B-Requirements |  | rpaquet2 - Added 342466, 342467, 343290 removes 335253, 335255, 335270, 335273 |
|  | SCG-REQ-335227/B-Gateway Module requests Charging Module to turn On Smart Charging based On Cloud Command |  | rpaquet2 - Added content |
|  | SCG-REQ-335228/B-Charging Module turns On Smart Charging |  | rpaquet2 - Update to include VehicleHMI |
|  | SCG-REQ-335229/B-Gateway Module responds to Cloud Command that Smart Charging is On |  | rpaquet2 - Added Fresh data info |
|  | SCG-REQ-335230/B-Charging Module sends Smart Charging Status to Vehicle HMI |  | rpaquet2 - Changed requirement name and content |
|  | SCG-REQ-342466/A-UAllow bit updated to reflect Smart Charging state changed via Offboard |  | rpaquet2 - New |
|  | SCG-REQ-342467/A-UAllow bit updated to reflect Smart Charging state changed via Vehicle HMI |  | rpaquet2 - New |
|  | SCG-REQ-335246/B-Vehicle Turns Off Smart Charging from the Cloud |  | rpaquet2 - Added fresh data info |
|  | SCG-REQ-335248/B-Charging Module sends Smart Charging Off Status to Vehicle HMI |  | rpaquet2 - Updated content and change requirement name |
|  | SCG-REQ-335249/B-Gateway Module responds to Cloud Command that Smart Charging is Off |  | rpaquet2 - Added fresh data info |
|  | SCG-REQ-335252/B-Vehicle HMI sends Smart Charging On request to Charging Module |  | rpaquet2 - updated text |
|  | SCG-REQ-343290/A-Charging Module Turns On Smart Charging |  | rpaquet2 - New |
|  | SCG-REQ-335256/B-Vehicle HMI Displays SC On/Off Toggle as On |  | rpaquet2 - Changed Gateway to Charging |
|  | SCG-REQ-335257/B-Gateway Module Alerts the Cloud that Smart Charging was turned On |  | rpaquet2 - Update content |
|  | SCG-REQ-335258/B-Charging Module Reports Smart Charging On Failure to Vehicle HMI |  | rpaquet2 - Updated content |
|  | SCG-REQ-335259/B-Vehicle HMI sends Smart Charging Off request to Charging Module |  | rpaquet2 - Updated content |
|  | SCG-REQ-335274/B-Vehicle HMI displays SC On/Off toggle as Off |  | rpaquet2 - Updated content |
|  | SCG-REQ-335276/B-Gateway Module alerts the Cloud that Smart Charging was turned Off |  | rpaquet2 - Updated content |
|  | SCG-REQ-335277/B-Charging Module reports Smart Charging Off Failure to Vehicle HMI |  | rpaquet2 - Updated content |
|  | SCG-REQ-343291/A-Charging Module turns OFF Smart Charging |  | rpaquet2 - Updated content |
|  | SCG-STR-584575/B-Use Cases |  | rpaquet2 - removed 329026 |
|  | SCG-UC-REQ-329021/B-User Turns On Smart charging via FP/LW |  | rpaquet2 - Update post condition |
|  | SCG-UC-REQ-329022/B-User Turns On Smart charging via SYNC HMI |  | rpaquet2 - Updated post condition |
|  | SCG-ACT-REQ-329635/B-Turn On or Off Smart Charging From Offboard |  | rpaquet2 - Updated diagram |
|  | SCG-ACT-REQ-329636/B-Turn On or Off Smart Charging From In Vehicle |  | rpaquet2 - Updated diagram |
|  | SCG-SD-REQ-329950/B-Turn On or Off Smart Charge from Onboard |  | rpaquet 2- Updated diagram |
|  | SCG-STR-584594/B-Requirements |  | rpaquet2 - Added 339227, 342471 , 343292, 343878 remove 335297 |
|  | SCG-REQ-335279/B-Gateway Module receives Updated Smart Charging Settings from the Cloud |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335290/B-Gateway Module responds to Cloud command to notify that it received the Update SC Settings Command |  | rpaquet2- Updated text |
|  | SCG-REQ-335291/B-Gateway Module sends request to Charging Module with Updated Smart Charging Settings |  | rpaquet2 - updated text |
|  | SCG-REQ-335292/B-Charging Module updates Smart Charging Settings stored in the vehicle |  | rpaquet2 - Updated to state Save SC settings |
|  | SCG-REQ-335293/B-Gateway Module checks the Update Request against the received Settings Payload |  | rpaquet2- Updated content per feature owner |
|  | SCG-REQ-335294/B-Gateway Module sends alert to cloud that Smart Charging Setting were successfully updated |  | rpaquet2 - Updated text |
|  | SCG-REQ-335295/B-Charging Module sends Smart Charging Settings to Vehicle HMI |  | rpaquet2 - Updated requirement name and content |
|  | SCG-REQ-335298/B-Gateway Module sends failure if received Payload does not match the request sent originally |  | rpaquet2- Updated content |
|  | SCG-REQ-342471/A-Gateway Module Error Codes for Failure to Update |  | rpaquet2 - New |
|  | SCG-REQ-343292/A-Cloud sends Smart Charging Location Names to the Gateway Module |  | rpaquet2 - New |
|  | SCG-REQ-336982/B-Location Name is Optional to Display on Vehicle HMI |  | rpaquet2 - correct typo |
|  | SCG-REQ-339227/A-Smart Charging Location Label Names |  | rpaquet2 - New |
|  | SCG-REQ-343878/A-Static List Locations |  | rpaquet2 - New |
|  | SCG-UC-REQ-329032/B-Update Smart Charging Settings Successful |  | rpaquet2 - Updated Post Condition |
|  | SCG-UC-REQ-329033/B-Update Smart Charging Settings Failed |  | rpaquet2 - Updated post condition |
|  | SCG-ACT-REQ-329637/B-Smart Charging Learned Settings Update |  | rpaquet2 - Updated Diagram |
|  | SCG-SD-REQ-329957/B-Machine Learning |  | rpaquet2 - Updated diagram |
|  | SCG-STR-584605/B-Requirements |  | rpaquet2 - Added 342472, 342473 removed 339732, 335340 |
|  | SCG-REQ-335303/B-Vehicle is at Smart Charging Location |  | rpaquet2 - Update text |
|  | SCG-REQ-335309/B-Vehicle is at a location with duration setting ON, target SOC not reached by end of time |  | rpaquet2 - Update text per feature owner |
|  | SCG-REQ-335310/B-Vehicle is at a location with duration setting ON, target SOC is reached before end of time |  | rpaquet 2- Updated text per feature owner |
|  | SCG-REQ-335339/B-Digital Station Schedule |  | rpaquet2 - Updated requirement text |
|  | SCG-REQ-335344/B-Duration Timer during lost power incident while charging |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-342472/A-Charging Module updates Gateway Module the Vehicle has reached a Smart Charging Saved Location |  | rpaquet2 - New |
|  | SCG-REQ-342473/A-Gateway Module sends Alert when Vehicle has reached a Smart Charging Saved Location |  | rpaquet2 - New |
|  | SCG-ACT-REQ-330006/B-Plugin |  | rpaquet2 - Updated diagram |
|  | SCG-SD-REQ-330005/B-Plugin |  | rpaquet2 - Updated alert name |
|  | SCG-STR-584600/B-Requirements |  | rpaquet2 - removed 335353, 335348, 335349, 335346 per feature owner added 343293, 343294, 343295, 343296 |
|  | SCG-REQ-335345/B-Vehicle HMI sends new Minimum SOC to Charging Module |  | rpaquet2 - Updated text |
|  | SCG-REQ-335347/B-Charging Module updates Smart Charging Settings stored in the Vehicle based on the new Minimum SOC |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335350/B-Gateway Module alerts the Cloud that Smart Charging Settings were updated |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335351/B-Charging Module Reports Smart Charging Settings Update Failure back to User |  | rpaquet2 - no content change typo |
|  | SCG-REQ-343293/A-User Selects "Auto" Min SOC to allow Cloud to control the minimum SOC on Vehicle HMI |  | rpaquet2 - new |
|  | SCG-REQ-343294/A-Vehicle updates Minimum SOC to zero percent to allow cloud most flexibility |  | rpaquet2 - new |
|  | SCG-REQ-343295/A-Vehicle Displays minimum SOC as "AUTO" when selected |  | rpaquet2 - new |
|  | SCG-REQ-343296/A-Gateway Module sends minimum SOC to the Cloud |  | rpaquet2 - new |
|  | SCG-UC-REQ-329024/B-User wants to Set Minimum SOC in HMI |  | rpaquet2 - updated post condition |
|  | SCG-ACT-REQ-333381/B-Update Minimum SOC |  | rpaquet2 - Updated diagram |
|  | SCG-SD-REQ-334519/B-Update Minimum SOC |  | rpaquet2 - Updated diagram |
|  | SCG-STR-597800/B-Requirements |  | rpaquet2 - Removed 335367 added 343297 |
|  | SCG-REQ-335366/B-Vehicle HMI sends request to turn OFF Smart Charging when user turns OFF CCS |  | rpaquet2 - Changed Gateway to Charging |
|  | SCG-REQ-335368/B-Charging Module turns OFF Smart Charging and responds to Vehicle HMI request |  | rpaquet2 - Updated content |
|  | SCG-REQ-343297/A-Gateway Module Alerts the Cloud that Smart Charging was Turned OFF |  | rpaquet2 - New |
|  | SCG-REQ-335370/B-Vehicle HMI displays SC ON/OFF toggle as OFF |  | rpaquet2 - Updated content |
|  | SCG-REQ-335371/B-Charging Module reports failure to Turn OFF Smart Charging to the Vehicle HMI |  | rpaquet2 - Updated content |
|  | SCG-STR-584610/B-Requirements |  | rpaquet2 - removed 335376, 335379 added 343298 |
|  | SCG-REQ-335372/B-Gateway Module receives request to initiate Master Reset |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-343298/A-Vehicle HMI sends Master Reset to the Charging Module |  | rpaquet2 - new |
|  | SCG-REQ-335373/B-Charging Module turns OFF Smart Charging and RESETS all Smart Charging Settings from the Vehicle |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335374/B-Charging Module sends Status to the Vehicle HMI that Smart Charging has been turned OFF |  | rpaquet2- Update content per feature owner |
|  | SCG-REQ-335375/B-Gateway Module sends alert to the Cloud that Master Reset was successful |  | rpaquet2 - Updated content per feature owner |
|  | SCG-REQ-335377/B-Vehicle HMI requests to initiate Brand Reset |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335378/B-Charging Module turns OFF Smart Charging and SAVES all Smart Charging Settings on the Vehicle |  | rpaquet2 - Updated content |
|  | SCG-REQ-335380/B-Gateway Module alerts the Cloud that Brand Reset was successful |  | rpaquet2 - Updated content per feature owner |
|  | SCG-REQ-335381/B-Gateway Module reports failure to turn OFF Smart Charging to Vehicle HMI and the user |  | rpaquet2 - Updated content |
|  | SCG-ACT-REQ-333379/B-Reset Performed In Vehicle |  | rpaquet2 - Updated diagram |
|  | SCG-ACT-REQ-333380/B-Last User Removed Reset |  | rpaquet2 - Updated diagram |
|  | SCG-SD-REQ-334554/B-Master Reset Performed In Vehicle |  | rpaquet2 - Updated diagram |
|  | SCG-SD-REQ-334561/B-Brand Reset Performed In Vehicle |  | rpaquet2 - Updated diagram |
|  | SCG-SD-REQ-334553/B-Last User Master Reset |  | rpaquet2 - Updated diagram |
|  | SCG-STR-598332/B-Requirements |  | rpaquet2 - Updated |
|  | SCG-REQ-335386/B-Gateway Module receives command to delete SC location |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-343052/A-Gateway Module responds to cloud with in-progress Command Response during after receiving Delete Smart Charging Location Command |  | rpaquet2 - New |
|  | SCG-REQ-335387/B-Charging Module deletes SC location |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335388/B-Gateway Module sends alert to the cloud once successfully deleted location |  | rpaquet2 - Updated text per feature owner |
|  | SCG-REQ-335392/B-Gateway Module reports Delete SC Location failure to the Cloud |  | rpaquet2 - Updated text |
|  | SCG-REQ-337298/B-Cloud shall send TurnOFFSmartCharging Command after DeleteSCLocation Command is successful |  | rpaquet2 - Updated text |
|  | SCG-ACT-REQ-333378/B-Delete Smart Charge Location |  | rpaquet2 - Updated diagram |
|  | SCG-SD-REQ-334522/B-Delete Smart Charge Location |  | rpaquet2 - Updated diagram |
| May 28, 2019 | 1.2 |  |  |
|  | SCG-STR-598559/C-Logical Signal Mapping |  | rpaquet2 - removed and added a couple of signals |
|  | SCG-IIR-REQ-333303/C-SC Client_Tx |  | rpaquet2 - revised but no change |
|  | SCG-IIR-REQ-333304/C-SC Client_Rx |  | rpaquet2- removed 333225 and 333235 added 349015, 349014 |
|  | MD-REQ-349015/A-Payload_Update_Status |  | rpaquet2 - New |
|  | MD-REQ-349014/A-Notification_Conflict_Rq |  | rpaquet2 - New |
|  | SCG-IIR-REQ-333306/C-SC Onboard HMI Client_Rx |  | rpaquet2 - removed 333225 added 349014 |
|  | SCG-ACT-REQ-333307/C-Activate Deactivate Smart Charging |  | rpaquet2 - Updated diagram |
|  | SCG-SD-REQ-334521/C-Activate or Deactivate Smart Charging |  | rpaquet2 - Updated diagram |
|  | SCG-STR-584594/C-Requirements |  | rpaquet2 - removed 335293 |
|  | SCG-REQ-335291/C-Gateway Module sends request to Charging Module with Updated Smart Charging Settings |  | rpaquet 2- deleted SCUpdateSettings_St and replaced with PayloadUpdateStatus |
|  | SCG-REQ-335292/C-Charging Module updates Smart Charging Settings stored in the vehicle |  | rpaquet2 - added PayloadUpdateStatus |
|  | SCG-REQ-335294/C-Gateway Module sends alert to cloud that Smart Charging Setting were successfully updated |  | rpaquet2 - Updated with PayloadUpdateStatus |
|  | SCG-REQ-335295/C-Charging Module sends Smart Charging Settings to Vehicle HMI |  | rpaquet2 - removed ScLocIdCur and CurntTrgtSoc |
|  | SCG-REQ-335298/C-Gateway Module sends failure if received Payload does not match the request sent originally |  | rpaquet2 - Updated content |
|  | SCG-REQ-343292/B-Cloud sends Smart Charging Location Names to the Gateway Module |  | rpaquet2 - Added associated location ID refernce |
|  | SCG-REQ-336982/C-Location Name is Optional to Display on Vehicle HMI |  | rpaquet 2- Updated content |
|  | SCG-UC-REQ-329032/C-Update Smart Charging Settings Successful |  | rpaquet2 - Updated content |
|  | SCG-UC-REQ-329033/C-Update Smart Charging Settings Failed |  | rpaquet2 - update post condition |
|  | SCG-ACT-REQ-329637/C-Smart Charging Learned Settings Update |  | rpaquet2 - Updated diagram |
|  | SCG-SD-REQ-329957/C-Machine Learning |  | rpaquet2 - Updated Diagram |
|  | SCG-FUN-REQ-328712/B-Charging & Arriving at a Smart Charging Location |  | rpaquet2 - Updated function name |
|  | SCG-STR-584605/C-Requirements |  | rpaquet2 - removed 335307 added 349016, 349017, 349018, 349019,349020,349021,349022 |
|  | SCG-REQ-335303/C-Vehicle is at Smart Charging Location |  | rpaquet2 - Updated content |
|  | SCG-REQ-335304/B-Vehicle is not at a Smart Charging Location |  | rpaquet 2- Updated content |
|  | SCG-REQ-342472/B-Charging Module updates Gateway Module the Vehicle has reached a Smart Charging Saved Location |  | Added ScLocIdCurnt_No_Stat to definition of bundle |
|  | SCG-REQ-349016/A-Charging Module updates Vehicle HMI and Gateway Module when the Vehicle has reached a Smart Charging Location |  | rpaquet2 - new |
|  | SCG-REQ-342473/B-Gateway Module sends Alert when Vehicle has reached a Smart Charging Location |  | rpaquet2 - Updated Content |
|  | SCG-REQ-349017/A-Conflict Notifications - Charging Outside Time Windows |  | rpaquet2 - new |
|  | SCG-REQ-349018/A-Conflict Notifications - Charging Outside Time Windows Vehicle HMI |  | rpaquet2 - new |
|  | SCG-REQ-349019/A-Conflict Notifications - Charging Outside Time Windows Alert to Cloud |  | rpaquet2 - new |
|  | SCG-REQ-349020/A-Conflict Notifications - Cannot Reach Target SOC by Departure Time Charging Module Set Signal |  | rpaquet2 - new |
|  | SCG-REQ-349021/A-Conflict Notifications - Cannot Reach Target SOC by Departure Time Vehicle HMI |  | rpaquet2 - new |
|  | SCG-REQ-349022/A-Conflict Notifications - Cannot Reach Target SOC by Departure Time Alert to Cloud |  | rpaquet2 - new |
|  | SCG-STR-584606/B-Use Cases |  | rpaquet2 - removed 329037 |
|  | SCG-UC-REQ-329038/B-User plugs in, SC is Turned On, Location is not a Smart Charging Location |  | rpaquet2 - revised name and post condition |
|  | SCG-STR-584609/B-Sequence Diagrams |  | rpaquet2 - Added 349023 |
|  | SCG-SD-REQ-349023/A-Vehicle Arrives at Smart Charging Location |  | rpaquet2 - New |
|  | SCG-FUN-REQ-328711/B-Updating Minimum State Of Charge Settings |  | rpaquet2 - Updated function name |
|  | SCG-STR-584600/C-Requirements |  | rpaquet2 - Removed 343293, 343294, 343295 and 343296 |
|  | SCG-REQ-335373/C-Charging Module turns OFF Smart Charging and RESETS all Smart Charging Settings from the Vehicle |  | rpaquet2 - updated content |
|  | SCG-SD-REQ-334554/C-Master Reset Performed In Vehicle |  | rpaquet2 - Updated diagram |
|  | SCG-SD-REQ-334561/C-Brand Reset Performed In Vehicle |  | rpaquet2 - Updated diagram |
|  | SCG-REQ-343052/B-Gateway Module responds to cloud with in-progress Command Response during after receiving Delete Smart Charging Location Command |  | rpaquet2 - updated per feature owner |
|  | SCG-REQ-335387/C-Charging Module deletes SC location |  | rpaquet2- updated per feature owner |
|  | SCG-REQ-335388/C-Gateway Module sends alert to the cloud once successfully deleted location |  | rpaquet2 - updated per featuer owner |
|  | SCG-REQ-335392/C-Gateway Module reports Delete SC Location failure to the Cloud |  | rpaquet2 - updated per feature owner |
|  | SCG-ACT-REQ-333378/C-Delete Smart Charge Location |  | rpaquet2 - Updated diagram |
|  | SCG-SD-REQ-334522/C-Delete Smart Charge Location |  | rpaquet2 - Updated diagram |
| August 5, 2019 | 1.3 |  |  |
|  | SCG-FRD-REQ-333302/B-Smart Charging |  | rpaquet2 - Added a General requirements section |
|  | STR-673999/A-General Requirements |  | rpaquet2 - new |
|  | SCG-REQ-358501/A-HMI Updates to Departure Times and Comfort Settings Tile |  | rpaquet2 - new |
|  | SCG-REQ-358502/A-HMI Updates to Charge Settings Tile |  | rpaquet2 - new |
|  | SCG-STR-584594/D-Requirements |  | rpaquet2 - Added 355860 |
|  | SCG-REQ-355860/A-Smart Charging Location Label Synchronization |  | rpaquet2 - new |
| December 12, 2019 | 1.4 |  |  |
|  | SCG-REQ-335185/C-Vehicle HMI Displays SC On/Off Toggle after Activation |  | rpaquet2 - removed notification to inform user per feature owner |
|  | SCG-ACT-REQ-333307/D-Activate Deactivate Smart Charging |  | rpaquet2 - Update a self call from showactivationnotification to show toggle |
|  | SCG-SD-REQ-334521/D-Activate or Deactivate Smart Charging |  | rpaquet2 - Update a self call from showactivationnotification to show toggle |
|  | SCG-FUN-REQ-333070/B-Update CCS via HMI |  | rpaquet2 - removed the use case and white box view sections |
|  | SCG-STR-597800/C-Requirements |  | rpaquet2 - removed 335371 per feature owner |
|  | SCG-REQ-335366/C-User Turns Off CCS |  | rpaquet2 - Update name and content per feature owner |
|  | SCG-REQ-335368/C-Gateway Module informs Charging Module Smart Charging is OFF |  | rpaquet2 - Update name and content per feature owner |
|  | SCG-REQ-343297/B-Charging Module turns OFF Smart Charging and responds to Gateway Module and Vehicle HMI |  | rpaquet2 - Update name and content per feature owner |
|  | SCG-REQ-343052/C-Gateway Module responds to cloud with in-progress Command Response during after receiving Delete Smart Charging Location Command |  | rpaquet2- changed In progress to Success |
|  | SCG-REQ-335387/D-Charging Module deletes SC location |  | rpaquet2 - change requests to request |

Table of Contents

Revision History	2

1	Architectural Design	11

1.1	Overview	11

1.2	Physical Mapping of Classes	12

1.3	SCG-CLD-REQ-333257/A-SC Off Board Gateway	12

1.4	SCG-CLD-REQ-333258/A-SC Off Board HMI Client	12

1.5	SCG-CLD-REQ-333259/A-SC Off Board Server	12

1.6	SCG-CLD-REQ-333260/A-SC On Board HMI Client	13

1.7	SCG-CLD-REQ-333261/B-SC Client	13

1.7.1	Functional Requirements	13

1.8	SCG-CLD-REQ-333262/A-SC Server	13

1.9	Logical Signal Mapping	13

1.10	SC Client Interface	14

1.10.1	SCG-IIR-REQ-333303/C-SC Client_Tx	14

1.10.2	SCG-IIR-REQ-333304/C-SC Client_Rx	20

1.11	SC Onboard HMI Client Interface	23

1.11.1	SCG-IIR-REQ-333305/B-SC Onboard HMI Client_Tx	23

1.11.2	SCG-IIR-REQ-333306/C-SC Onboard HMI Client_Rx	25

2	General Requirements	29

2.1	SCG-REQ-358501/A-HMI Updates to Departure Times and Comfort Settings Tile	29

2.2	SCG-REQ-358502/A-HMI Updates to Charge Settings Tile	29

3	Functional Definition	30

3.1	SCG-FUN-REQ-333036/A-Activate or Deactivate Smart Charging	30

3.1.1	Requirements	30

3.1.2	Use Cases	31

3.1.3	White Box Views	33

3.2	SCG-FUN-REQ-328704/A-Turning On or Off Smart Charging	35

3.2.1	Requirements	35

3.2.2	Use Cases	38

3.2.3	White Box Views	41

3.3	SCG-FUN-REQ-328710/A-Update Smart Charge Settings	43

3.3.1	Requirements	43

3.3.2	Use Cases	45

3.3.3	White Box Views	46

3.4	SCG-FUN-REQ-328712/B-Charging & Arriving at a Smart Charging Location	47

3.4.1	Requirements	47

3.4.2	Use Cases	49

3.4.3	White Box Views	51

3.5	SCG-FUN-REQ-328711/B-Updating Minimum State Of Charge Settings	51

3.5.1	Requirements	51

3.5.2	Use Cases	52

3.5.3	White Box Views	53

3.6	SCG-FUN-REQ-333070/B-Update CCS via HMI	53

3.6.1	Requirements	53

3.7	SCG-FUN-REQ-328713/A-Master or Brand Reset	54

3.7.1	Requirements	54

3.7.2	Use Cases	56

3.7.3	White Box Views	56

3.8	SCG-FUN-REQ-333209/A-Delete Smart Charge Location	60

3.8.1	Requirements	60

3.8.2	Use Cases	62

3.8.3	White Box Views	62

4	Appendix A: Definitions / Acronyms	64

5	Appendix B: Reference Documents	65

# Architectural Design

## Overview

The high-level data flow between the vehicle, cloud, and mobile app is shown below. The smart charging algorithm is hosted in the cloud – optimal charging settings will be sent from the cloud to the vehicle. Trip and charge information from the vehicle feeds to the cloud and serve as inputs into the algorithm; the algorithm continually refines smart charge settings to evolve with routine and preference changes over time. The user will have the ability to turn on and turn off Smart Charging via FordPass/Lincolnway or the HMI.

Below is a view of the Smart Charging architecture; the main pieces are the mobile app, the cloud, and the vehicle.

## Physical Mapping of Classes

The table below shows an example of how the logical classes may be mapped into physical modules. This mapping example is specific to FNV2.

| Logical Class | Physical Module (ECU) |
| --- | --- |
| SC Offboard HMI Client | Ford Pass/Lincoln Way |
| SC Offboard Server | SDN(Cloud) |
| SC Offboard Gateway | TCU |
| SC Server | HPCM(Charging Module)/OBCC |
| SC Onboard HMI Client | APIM(Vehicle HMI) |
| SC Client | ECG(Gateway Module) |

## SCG-CLD-REQ-333257/A-SC Off Board Gateway

Responsible for providing a modem to transmit data into and out of the vehicle.

## SCG-CLD-REQ-333258/A-SC Off Board HMI Client

Responsible for providing a user interface outside of the Vehicle.

## SCG-CLD-REQ-333259/A-SC Off Board Server

Responsible for handling cloud content and computing smart charging settings to download to the vehicle.

## SCG-CLD-REQ-333260/A-SC On Board HMI Client

Responsible for providing an interface to the in vehicle user.

## SCG-CLD-REQ-333261/B-SC Client

Responsible for providing logical computations within the vehicle and to provide information to the In Vehicle and Off board HMI.

### Functional Requirements

#### SCG-REQ-343119/A-Invalid Values Associated with Unfilled Smart Charging Locations

The Gateway Module shall transmit Smart Charging Location Coordinates associated with an invalid Smart Charging Location ID with the values below when the number of Smart Charging Locations is less than the allocated maximum:

| CAN Signal | HEX | DEC |
| --- | --- | --- |
| ScOfbLocLattDeg_An_Rq | 7F | 127 |
| ScOfbLocLattFrct_An_Rq | FFFFF | 1048575 |
| ScOfbLocLattPostv_B_Rq | 0 | 0 |
| ScOfbLocLongDeg_An_Rq | FF | 255 |
| ScOfbLocLongFrct_An_Rq | FFFFF | 1048575 |

#### SCG-REQ-343050/A-Out of Range Values for Invalid Smart Charging Locations

The Gateway Module shall transmit Smart Charging Location Coordinates associated with an invalid Smart Charging Location ID with the values below when the number of Smart Charging Locations is less than the allocated maximum:

| CAN Signal | HEX | DEC |
| --- | --- | --- |
| ScOfbLocLattDeg_An_Rq | 7F | 127 |
| ScOfbLocLattFrct_An_Rq | FFFFF | 1048575 |
| ScOfbLocLattPostv_B_Rq | 0 | 0 |
| ScOfbLocLongDeg_An_Rq | FF | 255 |
| ScOfbLocLongFrct_An_Rq | FFFFF | 1048575 |

#### SCG-REQ-343727/A-Out of Range Values for Unchanged Minimum SOC

The Gateway Module shall transmit out of range values associated with Minimum SOC when there is no change in the value received from the cloud.

## SCG-CLD-REQ-333262/A-SC Server

Responsible for providing charge information from within the vehicle.

## Logical Signal Mapping

Each logical name used in this document is mapped to its corresponding CAN signal. Please refer to the following mapping:

| Logical name | CAN signal name |
| --- | --- |
| SmartCharging_St | ScEnbl_B_Stat |
| SmartCharging_Rq | ScEnbl_D_RqCld |
| SmartChargingInVeh_Rq | ScEnbl_D_RqMnu |
| SCSettings_Rq | ScLocId_No_RqCld |
|  | ScLocLattDeg_An_RqCld |
|  | ScLocLattPostv_B_RqCld |
|  | ScLocLattFrct_An_RqCld |
|  | ScLocLongDeg_An_RqCld |
|  | ScLocLongPostv_B_RqCld |
|  | ScLocLongFrct_An_RqCld |
|  | ScLocDelete_B_RqCld |
|  | ScLocSetID_No_RqCld |
|  | ScChrgToPc_Pc_RqCld |
|  | ScChrgDurSet_D_RqCld |
|  | ScChrgDur_T_RqCld |
|  | ScLocRdius_L_RqCld |
|  | ScDayOfWeekId_D_RqCld |
|  | ScChrgPrfl_No_RqCld |
|  | ScMnSoc_Pc_RqCld |
|  | ScLocPwID_No_RqCld |
|  | ScChrgrPwMax_Pw_RqCld |
| Payload_Update_Status | ScPayloadUpdate_B_Stat |
| SCMinimumSOC_Rq | ScMnSoc_Pc_RqCld ScMnSoc_Pc_RqMnu |
| SCMinimumSOC_St | ScMnSoc_Pc_Actl |
| CurntTargetSOC_St | CurntTrgtSoc_Pc_Dsply |
| SCDeleteLocation_Rq | ScLocDelete_B_RqCld |
| ScLocIdCurnt_No_Stat | ScLocIdCurnt_No_Stat |
| ClearAllUserSettings_Rq | OfbChrgClearAll_B_Rq |
| ChargeStationID_St | TP Method |
| ChargeStationSchedule_St | TP Method |
| FactoryReset_Rq | FactoryReset_Rq |
| FactoryReset_St | FactoryReset_Stat |
| BrandReset_Rq | EmbeddedModemReset_Rq |
| BrandReset_St | EmbeddedModemReset_Stat |
| FreshData_Rq | ScFreshDataEnbl_B_Rq |
| ChargeInPowerMode_St | ChrgrInPwMde_D_Actl |
| ChargeStationID_Rq | TP Method |
| setSCLocationNameInfo | SOA |
| getEcgDidData | SOA |
| getEcgDidUpdateData | SOA |
| Notification_Conflict_Rq | NtfctnConflict1_D_Rq |

## SC Client Interface

### SCG-IIR-REQ-333303/C-SC Client_Tx

#### MD-REQ-333214/A-SmartCharging_Rq

Message Type: request

Smart Charging enabled via the cloud

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - |  |
|  | Null | 0x0 |  |
|  | Disable | 0x1 |  |
|  | Enable | 0x2 |  |
|  | Not Used | 0x3 |  |

#### MD-REQ-333234/A-SCDeleteLocation_Rq

Message Type: request

Signal used to delete a Smart Charge Location.  Signal will coordinate with the Smart Charge Location ID signal to identify whether a Smart Charge Location is to be deleted.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - |  |
|  | No request | 0x0 |  |
|  | Request | 0x1 |  |

#### MD-REQ-333219/B-SCSettings_Rq

Message Type: request

Request to update Charging Setting and Location Attributes

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| ScLocId_No_RqCld | - | - | Smart Charge Location ID |
|  | ID 0 | 0x0 |  |
|  | ID 1 | 0x1 |  |
|  | ID 2 | 0x2 |  |
|  | ID 3 | 0x3 |  |
|  | ID 4 | 0x4 |  |
|  | ID 5 | 0x5 |  |
| ScLocLattDeg_An_RqCld | - |  | GPS Degrees Latitude |
|  |  | DEC 0 - 89 |  |
| ScLocLattPostv_B_RqCld | - | - | GPS Latitude Degree Sign |
|  | No | 0x0 |  |
|  | Yes | 0x1 |  |
| ScLocLattFrct_An_RqCld | - | - | GPS Degrees Decimal Latitude |
|  |  | DEC 0.000001 - 0.999999 |  |
| ScLocLongDeg_An_RqCld | - | - | GPS Degrees Longitude |
|  |  | DEC 0-179 |  |
| ScLocLongPostv_B_RqCld | - | - | GPS Longitude Degree Sign |
|  | No | 0x0 |  |
|  | Yes | 0x1 |  |
| ScLocLongFrct_An_RqCld | - | - | GPS Degrees Decimal Longitude |
|  |  | DEC 0.000001 - 0.999999 |  |
| ScLocDelete_B_RqCld | - | - | Signal used to delete a Smart Charge Location |
|  | No Request | 0x0 |  |
|  | Request | 0x1 |  |
| ScLocSetID_No_RqCld |  |  | Location Settings ID |
|  | ID 0 | 0x0 |  |
|  | ID 1 | 0x1 |  |
|  | ID 2 | 0x2 |  |
|  | ID 3 | 0x3 |  |
|  | ID 4 | 0x4 |  |
|  | ID 5 | 0x5 |  |
| ScChrgToPc_Pc_RqCld | - | - | Charge to Percent value stored for a Smart Charge location coordinated by the Smart Charge Location Settings ID signal |
|  |  | Percent 0-100 |  |
| ScChrgDurSet_D_RqCld | - | - | Duration-based Charging setting for a Smart Charge Location |
|  | Null | 0x0 |  |
|  | Off | 0x1 |  |
|  | On | 0x2 |  |
| ScChrgDur_T_RqCld | - | - | Time value of Duration-based Charging for a Smart Charge location |
|  |  | 0-1440 Min |  |
| ScLocRdius_L_RqCld | - | - | Radial distance from the GPS Coordinates of a Smart Charge Location within which the location is valid. 100 m resolution |
|  |  | 0-12700m |  |
| ScDayOfWeekId_D_RqCld | - | - | Day of the Week ID. This is a cycling (1-7) signal that will be used to update the Charge Profile settings for a Smart Charge Location for a given day of the week |
|  | Null | 0x0 |  |
|  | Monday | 0x1 |  |
|  | Tuesday | 0x2 |  |
|  | Wednesday | 0x3 |  |
|  | Thursday | 0x4 |  |
|  | Friday | 0x5 |  |
|  | Saturday | 0x6 |  |
|  | Sunday | 0x7 |  |
| ScChrgPrfl_No_RqCld | - | - | The Charge Profile for a Smart Charge Location coordinated by the Smart Charging location settings ID signal |
|  |  | Charge/Value Charge Bits (24 hours) 0 = Charge 1 = Value Charge Bit 23 == 00:00 Bit 22 == 01:00…. Bit 11 = 12:00… Bit 0 == 23:00 |  |
| ScMnSoc_Pc_RqCld | - | - | Minimum Charge to Percent "comfort level" value stored for a Smart Charge location |
|  |  | Percent 0-100 |  |
| ScChrgrPwMax_Pw_RqCld | - | - | Power available to the charger from the Electric Vehicle Supply Equipment - the maximum power the charger is allowed to draw from the mains. |
|  |  | 0 – 204700 watts |  |
| ScLocPwID_No_RqCld | - | - | Location Settings ID |
|  | ID_0 | 0x0 |  |
|  | ID_1 | 0x1 |  |
|  | ID_2 | 0x2 |  |
|  | ID_3 | 0x3 |  |
|  | ID_4 | 0x4 |  |
|  | ID_5 | 0x5 |  |

#### MD-REQ-213361/C-FactoryReset_Rq

Message Type: Request

Signal sent by the Master Reset Client to initiate a Master Reset

| Logical Signal Name | Literals | Value | Description |
| --- | --- | --- | --- |
| FactoryReset_Rq | Inactive | 0x0 |  |
|  | ResetFactoryDefaults | 0x1 |  |

#### MD-REQ-339246/A-setSCLocationNameInfo

This message is used to transmit the Smart Charging Location Name.

| Method Type |  | On Change |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Level |  | Default |  |  |  |  |
| Retained |  | Yes |  |  |  |  |
| R/O | Name |  | Type | Literals | Value | Description |
| Request (_St) |  |  |  |  |  |  |
| R | SCLocID |  | Enum | - | - | LocID tied to the Location Name below |
|  |  |  |  | ID 0 | 0x0 |  |
|  |  |  |  | ID 1 | 0x1 |  |
|  |  |  |  | ID 2 | 0x2 |  |
|  |  |  |  | ID 3 | 0x3 |  |
|  |  |  |  | ID 4 | 0x4 |  |
|  |  |  |  | ID 5 | 0x5 |  |
| R | SCLocationName |  | String | - | 20 chars. |  |

#### MD-REQ-304071/A-getEcgDidData

This SOA API is used to request DID information from the ECG. The ECG also uses this SOA API for its response.

| Method Type |  | One-Shot (Synch) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Level |  | Default |  |  |  |  |
| Retained |  | No |  |  |  |  |
| R/O | Name |  | Type | Literals | Value | Description |
| Request (_Rq) |  |  |  |  |  |  |
| R | RequestEcgDid Address |  | Int32 | - | 0x00000000 - 0xFFFFFFFF | Requested DID address |
| Response (_Rsp) |  |  |  |  |  |  |
| O | EcgResponseStatus |  | Enum | - | - | Response to initial request |
|  |  |  |  | Success | 0x00 |  |
|  |  |  |  | Error-Internal | 0x01 |  |
|  |  |  |  | Error-Access Permissions | 0x02 |  |
|  |  |  |  | Error-Invalid Parameter | 0x03 |  |
| R | ecgDidAddress |  | Int32 | - | 0x00000000 - 0xFFFFFFFF | DID address |
| R | ecgDidConfigData |  | Bytes | - | - | DID Data |

#### MD-REQ-304072/A-getEcgDidUpdateData

This SOA API is used to receive updated DID information from the ECG. The ECG publishes all updated DID information (OnChange) via this SOA API.

| Method Type |  | OnChange |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Level |  | Default |  |  |  |  |
| Retained |  | Yes |  |  |  |  |
| R/O | Name |  | Type | Literals | Value | Description |
| Request (_Rq) |  |  |  |  |  |  |
| - | - |  | - | - | - | N/A |
| Response (_Rsp) |  |  |  |  |  |  |
| R | ecgDidAddress |  | Int32 | - | 0x0000000 - 0xFFFFFFFF | DID address that was updated |
| R | ecgDidConfigData |  | Bytes | - | - | Corresponding updated DID Data |

#### MD-REQ-342970/A-ClearAllUserSettings_Rq

Message Type: Request

Signal sent to request to Clear User Settings

| Logical Signal Name | Literals | Value | Description |
| --- | --- | --- | --- |
|  | No Request | 0x0 |  |
|  | Request | 0x1 |  |

#### MD-REQ-343026/A-ChargeStationID_Rq

Message Type: request

This signal is used to request the Charge Station information.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| RequestData | - | - |  |
|  | No Request | 0x00 |  |
|  | Request Charge Station ID | 0x01 |  |

### SCG-IIR-REQ-333304/C-SC Client_Rx

#### MD-REQ-333215/A-SmartCharging_St

Message Type: status

Smart Charging Enablement Status

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - |  |
|  | Disabled | 0x0 |  |
|  | Enabled | 0x1 |  |

#### MD-REQ-326502/B-ChargeStationID_St

Message Type: status

This signal is used to provide the Charge Station information.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| TLS Status | - | - |  |
|  | No TLS | 0x00 |  |
|  | TLS | 0x01 |  |
| Charge Station ID | - | - | Max. 37 bytes Variable Raw Data |

#### MD-REQ-337016/B-ChargeStationSchedule_St

Message Type: status

This signal is used to provide the schedule for the connected Charge Station

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Schedule ID | - | - |  |
|  | ID 1 | 0x1 |  |
|  | ID 254 | 0xFF |  |
| Schedule Index | - | - | This parameter shall increment for every messsage payload sent until all data is transferred. When sending data for new Schedule ID the index shall start at 1 again |
|  | ID 1 | 0x1 |  |
|  | ID 254 | 0xFF |  |
| Number Of Items Transmittied | - | - | Indicates the number of schedule inputs sent in current payload. Max number of items to be sent in a gvien Schedule Index is 800 |
|  | 1 | 0x0001 |  |
|  | … |  |  |
|  | 800 | 0x0320 |  |
|  | Reserved | 0x0321-0xFFFF |  |
| Power Value | - | - | 15 bits Fixed Units: Watts Resolution: 50 Watts |
| Time Interval | - | - | 25 bits Fixed Units: Time Resolution: Seconds |

#### MD-REQ-333227/B-SCMinimumSOC_St

Message Type: status

Minimum Charge to Percent "comfort level" value status for a Smart Charge location

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Used to indicate minimum charge level status. Unit:Percent |
|  | 0 to 100 Percent |  |  |

#### MD-REQ-222036/B-FactoryReset.St

Message Type: Status

Signal sent by the Master Reset Server indicating that the master reset default settings were restored for a master reset event

| Logical Signal Name | Literals | Value | Description |
| --- | --- | --- | --- |
| FactoryReset.St | Inactive | 0x0 |  |
|  | FactoryDefaultsRestored | 0x1 |  |
|  | Reserved | 0x2 |  |
|  | Reserved | 0x3 |  |

#### MD-REQ-246274/B-EmbeddedModemReset_St

Message Type: Status

This signal is used to indicate the status of the factory reset performed for the specified Embedded Modem feature.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Embedded Modem feature factory reset status. |
|  | Null | 0x0 |  |
|  | Reset_NotComplete | 0x1 |  |
|  | PaaKReset_Complete | 0x2 |  |
|  | OnlineTrafficReset_Complete | 0x3 |  |
|  | CCSReset_Complete | 0x4 |  |
|  | WifiHotspotReset_Complete | 0x5 |  |
|  | Reserved | 0x6-0xF |  |

#### MD-REQ-304071/A-getEcgDidData

This SOA API is used to request DID information from the ECG. The ECG also uses this SOA API for its response.

| Method Type |  | One-Shot (Synch) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Level |  | Default |  |  |  |  |
| Retained |  | No |  |  |  |  |
| R/O | Name |  | Type | Literals | Value | Description |
| Request (_Rq) |  |  |  |  |  |  |
| R | RequestEcgDid Address |  | Int32 | - | 0x00000000 - 0xFFFFFFFF | Requested DID address |
| Response (_Rsp) |  |  |  |  |  |  |
| O | EcgResponseStatus |  | Enum | - | - | Response to initial request |
|  |  |  |  | Success | 0x00 |  |
|  |  |  |  | Error-Internal | 0x01 |  |
|  |  |  |  | Error-Access Permissions | 0x02 |  |
|  |  |  |  | Error-Invalid Parameter | 0x03 |  |
| R | ecgDidAddress |  | Int32 | - | 0x00000000 - 0xFFFFFFFF | DID address |
| R | ecgDidConfigData |  | Bytes | - | - | DID Data |

#### MD-REQ-326679/A-ChargeInPowerMode_St

Message Type: status

This signal is used to indicate the operational mode of EVSE(Electrical Vehicle Supply Equipment).

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - |  |
|  | EvseNotDetected | 0x0 |  |
|  | EvsePaused | 0x1 |  |
|  | DigitalCommDetected | 0x2 |  |
|  | AcBasic | 0x3 |  |
|  | AcDigital | 0x4 |  |
|  | DcCharging | 0x5 |  |
|  | IcCharging | 0x6 |  |
|  | EvseNotCompatible | 0x7 |  |
|  | EvseFault | 0x8 |  |

#### MD-REQ-343299/A-CurntTargetSOC_St

Message Type: Status

Current Location Target SOC for display to the user.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Unit:Percent |
|  | 0 to 100 Percent |  |  |

#### MD-REQ-349015/A-Payload_Update_Status

Message Type: Status

This signal communicates the HPCM's status in saving a Smart Charging Payload.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - |  |
|  | Null | 0x0 |  |
|  | In-Process | 0x1 |  |
|  | Complete | 0x2 |  |

#### MD-REQ-349014/A-Notification_Conflict_Rq

Message Type: Request

Signal to track if vehicle must charge outside windows to reach target SOC or if vehicle is unable to reach target SOC by next departure Time

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Indicates if vehicle detected a charge window conflict |
|  | Null | 0x0 |  |
|  | No_Conflict_Exists | 0x1 |  |
|  | Departure_Time | 0x2 |  |
|  | Charge_Window | 0x3 |  |
|  | Not_Used | 0x4 |  |
|  | Not_Used | 0x5 |  |
|  | Not_Used | 0x6 |  |
|  | Not_Used | 0x7 |  |

## SC Onboard HMI Client Interface

### SCG-IIR-REQ-333305/B-SC Onboard HMI Client_Tx

#### MD-REQ-337015/A-SmartChargingInVeh_Rq

Message Type: request

Smart Charging enablement via vehicle

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - |  |
|  | Null | 0x0 |  |
|  | Disable | 0x1 |  |
|  | Enable | 0x2 |  |
|  | Not Used | 0x3 |  |

#### MD-REQ-333221/B-SCMinimumSOC_Rq

Message Type: request

Minimum Charge to Percent "comfort level" value stored for a Smart Charge location.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Used to set desired charge level. Unit:Percent |
|  | 0 to 100 Percent |  |  |

#### MD-REQ-213361/C-FactoryReset_Rq

Message Type: Request

Signal sent by the Master Reset Client to initiate a Master Reset

| Logical Signal Name | Literals | Value | Description |
| --- | --- | --- | --- |
| FactoryReset_Rq | Inactive | 0x0 |  |
|  | ResetFactoryDefaults | 0x1 |  |

#### MD-REQ-246273/C-EmbeddedModemReset_Rq

Message Type: Request

This signal is used to perform a factory reset for the specified Embedded Modem feature.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Embedded Modem feature to be reset to factory defaults. |
|  | Null | 0x0 |  |
|  | WifiHotspot_Reset | 0x1 |  |
|  | PaaK_Reset | 0x2 |  |
|  | OnlineTraffic_Reset | 0x3 |  |
|  | CCS_Reset | 0x4 |  |
|  | BrandConnect_Reset1 | 0x5 |  |
|  | BrandConnect_Reset2 | 0x6 |  |
|  | Reserved | 0x7 – 0xF |  |

#### MD-REQ-304071/A-getEcgDidData

This SOA API is used to request DID information from the ECG. The ECG also uses this SOA API for its response.

| Method Type |  | One-Shot (Synch) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Level |  | Default |  |  |  |  |
| Retained |  | No |  |  |  |  |
| R/O | Name |  | Type | Literals | Value | Description |
| Request (_Rq) |  |  |  |  |  |  |
| R | RequestEcgDid Address |  | Int32 | - | 0x00000000 - 0xFFFFFFFF | Requested DID address |
| Response (_Rsp) |  |  |  |  |  |  |
| O | EcgResponseStatus |  | Enum | - | - | Response to initial request |
|  |  |  |  | Success | 0x00 |  |
|  |  |  |  | Error-Internal | 0x01 |  |
|  |  |  |  | Error-Access Permissions | 0x02 |  |
|  |  |  |  | Error-Invalid Parameter | 0x03 |  |
| R | ecgDidAddress |  | Int32 | - | 0x00000000 - 0xFFFFFFFF | DID address |
| R | ecgDidConfigData |  | Bytes | - | - | DID Data |

### SCG-IIR-REQ-333306/C-SC Onboard HMI Client_Rx

#### MD-REQ-333215/A-SmartCharging_St

Message Type: status

Smart Charging Enablement Status

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - |  |
|  | Disabled | 0x0 |  |
|  | Enabled | 0x1 |  |

#### MD-REQ-222036/B-FactoryReset.St

Message Type: Status

Signal sent by the Master Reset Server indicating that the master reset default settings were restored for a master reset event

| Logical Signal Name | Literals | Value | Description |
| --- | --- | --- | --- |
| FactoryReset.St | Inactive | 0x0 |  |
|  | FactoryDefaultsRestored | 0x1 |  |
|  | Reserved | 0x2 |  |
|  | Reserved | 0x3 |  |

#### MD-REQ-246274/B-EmbeddedModemReset_St

Message Type: Status

This signal is used to indicate the status of the factory reset performed for the specified Embedded Modem feature.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Embedded Modem feature factory reset status. |
|  | Null | 0x0 |  |
|  | Reset_NotComplete | 0x1 |  |
|  | PaaKReset_Complete | 0x2 |  |
|  | OnlineTrafficReset_Complete | 0x3 |  |
|  | CCSReset_Complete | 0x4 |  |
|  | WifiHotspotReset_Complete | 0x5 |  |
|  | Reserved | 0x6-0xF |  |

#### MD-REQ-339246/A-setSCLocationNameInfo

This message is used to transmit the Smart Charging Location Name.

| Method Type |  | On Change |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Level |  | Default |  |  |  |  |
| Retained |  | Yes |  |  |  |  |
| R/O | Name |  | Type | Literals | Value | Description |
| Request (_St) |  |  |  |  |  |  |
| R | SCLocID |  | Enum | - | - | LocID tied to the Location Name below |
|  |  |  |  | ID 0 | 0x0 |  |
|  |  |  |  | ID 1 | 0x1 |  |
|  |  |  |  | ID 2 | 0x2 |  |
|  |  |  |  | ID 3 | 0x3 |  |
|  |  |  |  | ID 4 | 0x4 |  |
|  |  |  |  | ID 5 | 0x5 |  |
| R | SCLocationName |  | String | - | 20 chars. |  |

#### MD-REQ-304071/A-getEcgDidData

This SOA API is used to request DID information from the ECG. The ECG also uses this SOA API for its response.

| Method Type |  | One-Shot (Synch) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Level |  | Default |  |  |  |  |
| Retained |  | No |  |  |  |  |
| R/O | Name |  | Type | Literals | Value | Description |
| Request (_Rq) |  |  |  |  |  |  |
| R | RequestEcgDid Address |  | Int32 | - | 0x00000000 - 0xFFFFFFFF | Requested DID address |
| Response (_Rsp) |  |  |  |  |  |  |
| O | EcgResponseStatus |  | Enum | - | - | Response to initial request |
|  |  |  |  | Success | 0x00 |  |
|  |  |  |  | Error-Internal | 0x01 |  |
|  |  |  |  | Error-Access Permissions | 0x02 |  |
|  |  |  |  | Error-Invalid Parameter | 0x03 |  |
| R | ecgDidAddress |  | Int32 | - | 0x00000000 - 0xFFFFFFFF | DID address |
| R | ecgDidConfigData |  | Bytes | - | - | DID Data |

#### MD-REQ-304072/A-getEcgDidUpdateData

This SOA API is used to receive updated DID information from the ECG. The ECG publishes all updated DID information (OnChange) via this SOA API.

| Method Type |  | OnChange |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Level |  | Default |  |  |  |  |
| Retained |  | Yes |  |  |  |  |
| R/O | Name |  | Type | Literals | Value | Description |
| Request (_Rq) |  |  |  |  |  |  |
| - | - |  | - | - | - | N/A |
| Response (_Rsp) |  |  |  |  |  |  |
| R | ecgDidAddress |  | Int32 | - | 0x0000000 - 0xFFFFFFFF | DID address that was updated |
| R | ecgDidConfigData |  | Bytes | - | - | Corresponding updated DID Data |

#### MD-REQ-343299/A-CurntTargetSOC_St

Message Type: Status

Current Location Target SOC for display to the user.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Unit:Percent |
|  | 0 to 100 Percent |  |  |

#### MD-REQ-349014/A-Notification_Conflict_Rq

Message Type: Request

Signal to track if vehicle must charge outside windows to reach target SOC or if vehicle is unable to reach target SOC by next departure Time

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Indicates if vehicle detected a charge window conflict |
|  | Null | 0x0 |  |
|  | No_Conflict_Exists | 0x1 |  |
|  | Departure_Time | 0x2 |  |
|  | Charge_Window | 0x3 |  |
|  | Not_Used | 0x4 |  |
|  | Not_Used | 0x5 |  |
|  | Not_Used | 0x6 |  |
|  | Not_Used | 0x7 |  |

# General Requirements

## SCG-REQ-358501/A-HMI Updates to Departure Times and Comfort Settings Tile

When Smart Charging is turned ON, the HMI screen for the new Departure Times and Comfort Settings Tile shall make the following changes.

- The edit buttons for departure times shall be removed

- The clear all button for departure times shall be removed

- The screen shall explain to the user that Departure Times have been automated by the Smart Charging feature.

## SCG-REQ-358502/A-HMI Updates to Charge Settings Tile

When Smart Charging is turned ON, the HMI screen for Charge Settings shall make the following changes.

- The global ON/OFF and edit line item for Departure Times in the Charge Settings tile shall be removed.

# Functional Definition

## SCG-FUN-REQ-333036/A-Activate or Deactivate Smart Charging

### Requirements

#### SCG-REQ-328705/B-Gateway Module Receives Activation Command from the Cloud

Smart Charging will leverage the existing flow for feature activation by utilizing DID flows.  The Gateway Module shall receive the updated DID from the Cloud (CVFMA) and save the value in the Gateway Module.

#### SCG-REQ-328706/B-Gateway Module sends command response to the cloud (CVFMA) stating it received the updated DID to activate

After the Gateway Module receives  updated DID for Activation of Smart Charging, it shall send a command response back to the Cloud (CVFMA) to signify that it received the command and is in progress.

#### SCG-REQ-328707/B-Gateway Module alerts the Cloud (CVFMA) Smart Charging DID successfully updated for Activation

After the Gateway Module successfully saves the updated DID for activating Smart Charging, it shall send an alert to the Cloud (CVFMA).

#### SCG-REQ-335185/C-Vehicle HMI Displays SC On/Off Toggle after Activation

After the DID is updated in the Gateway Module, the Vehicle HMI shall receive the status via SOA API and the vehicle HMI shall display the ON/OFF Smart Charging toggle on the Vehicle HMI.

#### SCG-REQ-335193/B-Cloud to Gateway Module Activation Command Time Out

After receiving, the command from the Cloud (CVFMA) to update the DID for Smart Charging, if the Gateway Module does not send a response in 90 seconds then the command shall time out.

#### SCG-REQ-335194/B-Activation Failure Response to the Cloud

In the event that Smart Charging Activation fails, the Gateway Module shall report failure back to the cloud.  Gateway Module sends an alert with value of failure and description of the error to the cloud (CVFMA).

#### SCG-REQ-335196/B-Smart Charging is Deactivated by Default

Smart Charging DID = DEACTIVATED by default.

#### SCG-REQ-335206/B-Gateway Module receives Deactivation Command from the Cloud

Smart Charging will leverage the existing flow for feature activation by utilizing DID flows.  The Gateway Module shall receive the updated DID from the Cloud (CVFMA) and save the value in the Gateway Module

#### SCG-REQ-335208/B-Gateway Module sends command response to the Cloud (CVFMA) stating it received the updated DID to Deactivate

After the Gateway Module receives updated DID for Deactivation of Smart Charging, it shall send a command response back to the Cloud (CVFMA) to signify that it received the command and is in progress.

#### SCG-REQ-335212/B-Vehicle HMI hides the SC On/Off Toggle after Deactivation

After the DID is updated in the Gateway Module, the Vehicle HMI shall receive the updated status via SoA API’s and the Vehicle HMI shall hide the ON/OFF Smart Charging toggle on the Vehicle HMI.

#### SCG-REQ-335218/B-Gateway Module returns Successful Alert of Deactivation

After the Gateway Module updates the Smart Charging DID to deactivate the feature, it shall send an alert to the cloud (CVFMA).

#### SCG-REQ-335220/B-Cloud to Gateway Module DeactivationCommand Time Out

After receiving command to deactivate Smart Charging DID from the Cloud (CVFMA), if the Gateway Module does not send a response in 90 seconds then the command shall time out.

#### SCG-REQ-335221/B-Deactivation Failure Response to the Cloud

In the event that Smart Charging Deactivation fails, the Gateway Module shall report failure back to the cloud (CVFMA).  The Gateway Module sends an alert to the cloud (CVFMA) with a description of the error.

#### SCG-REQ-335222/A-Smart Charging Activation requires Vehicle Lifecycle to be set to Normal mode

Smart Charging shall only be available when the Vehicle Life cycle is set to NORMAL mode. In other modes (TRANSPORT OR MANUFACTURING), Smart Charging shall be deactivated.  If Vehicle Life cycle is changed from NORMAL mode to either TRANSPORT or MANUFACTURING Smart Charging shall be deactivated.

### Use Cases

#### SCG-UC-REQ-333253/B-Cloud Activates Smart Charging

| Actors | Cloud, Vehicle |
| --- | --- |
| Pre-conditions | CCS settings are turned ON Smart Charging is DEACTIVATED |
| Scenario | Cloud needs to activate smart charging for specific VIN Cloud switches DEACTIVATED to ACTIVATED for Smart Charging feature |
| Post-conditions | CVFMA sends DID to the ECG to activate Smart Charging ECG reads the status of the DID ECG sends activation status to APIM via SoA API’s Smart Charging appears on SYNC HMI ECG sends alert to CVFMA CVFMA marks Smart Charging as activated |
| Interfaces | CVFMA, ECG, APIM |
| Notes |  |

#### SCG-UC-REQ-333254/B-Cloud Deactivates Smart Charging

| Actors | Cloud, Vehicle |
| --- | --- |
| Pre-conditions | CCS settings are turned ON Smart Charging is ACTIVATED |
| Scenario | Cloud needs to deactivate smart charging for specific VIN Cloud switches ACTIVATED to DEACTIVATED for Smart Charging feature |
| Post-conditions | CVFMA sends a DID to the ECG to deactivate Smart Charging ECG reads the status of the DID ECG sends deactivation status to APIM via SoA API’s Smart Charging is hidden from SYNC HMI ECG sends alert to CVFMA CVFMA marks Smart Charging as deactivated |
| Interfaces | CVFMA, ECG, APIM, |
| Notes |  |

#### SCG-UC-REQ-333255/B-Activation or Deactivation Failure

| Actors | Cloud, Vehicle |
| --- | --- |
| Pre-conditions | CCS settings are turned ON Smart Charging is ACTIVATED or DEACTIVATED |
| Scenario | Cloud needs to activate or deactivate smart charging for specific VIN |
| Post-conditions | CVFMA sends a DID to the ECG to activate or deactivate Smart Charging ECG fails to read the status of the DID ECG sends an alert to CVFMA as failed CVFMA marks the command as failed |
| Interfaces | CVFMA ECG, |
| Notes |  |

#### SCG-UC-REQ-333256/B-Activation or Deactivation Timeout

| Actors | Cloud, Vehicle |
| --- | --- |
| Pre-conditions | CCS settings are turned ON Smart Charging is ACTIVATED or DEACTIVATED |
| Scenario | Cloud needs to activate or deactivate smart charging for specific VIN |
| Post-conditions | CVFMA sends a DID to the ECG to activate or deactivate Smart Charging ECG does not send response to SDN within 90 seconds CVFMA marks the command as failed |
| Interfaces | CVFMA, ECG |
| Notes |  |

### White Box Views

#### Activity Diagrams

##### SCG-ACT-REQ-333307/D-Activate Deactivate Smart Charging

#### Sequence Diagrams

##### SCG-SD-REQ-334521/D-Activate or Deactivate Smart Charging

##### SCG-SD-REQ-342972/A-Request DID Info on Wake Up

## SCG-FUN-REQ-328704/A-Turning On or Off Smart Charging

### Requirements

#### SCG-REQ-335226/A-Gateway Module receives On Command from the Cloud

When the Gateway Module receives the smartChargingONCommand from the Cloud, the Gateway Module shall begin the process to turn ON Smart Charging.

#### SCG-REQ-335227/B-Gateway Module requests Charging Module to turn On Smart Charging based On Cloud Command

After the Gateway Module receives the command to turn ON Smart Charging, the Gateway Module shall send smartChargingONRequest = ON to the Charging Module.  The Gateway Module shall raise FreshData_Rq and hold until Gateway Module receives smartChargingONStatus = ON.

#### SCG-REQ-335228/B-Charging Module turns On Smart Charging

After the Charging Module receives smartChargingONRequest to turn Smart Charging ON from the Gateway Module, the Charging Module shall turn ON Smart Charging and return smartChargingONStatus = ON to the Gateway Module and to the Vehicle HMI.

#### SCG-REQ-335229/B-Gateway Module responds to Cloud Command that Smart Charging is On

Once the Gateway Module receives the smartChargingONStatus = ON from the Charging Module, the Gateway Module shall send smartChargingONCommandResponse with value of successful status and smartChargingONStatus = ON to the cloud to recognize Smart Charging feature was successfully turned ON for the Vehicle. Gateway Module return the FreshData_Rq to 0 once SmartChargingONStatus = ON is received.

#### SCG-REQ-335230/B-Charging Module sends Smart Charging Status to Vehicle HMI

After Charging Module sends smartChargingONStatus = ON to the Vehicle HMI, the Vehicle HMI then shall toggle Smart Charging ON.

#### SCG-REQ-335231/A-Cloud to Gateway Module Turn On Command Time Out

After receiving smartChargingONCommand from the Cloud, if the Gateway Module does not send a response in 90 seconds then the command shall time out.

#### SCG-REQ-335233/A-Turn On Smart Charging Failure response to the Cloud

In the event that Smart Charging Status fails, the Gateway Module shall report failure back to the cloud.  Gateway Module sends smartChargingONCommandResponse with value of failure and description of the error.

#### SCG-REQ-342466/A-UAllow bit updated to reflect Smart Charging state changed via Offboard

If user turns on/off Smart Charging via offboard user interface, then uAllow bit shall be updated – entity type 1, ID 28

#### SCG-REQ-335235/A-Smart Charging is turned Off by Default

SmartChargingONStatus = OFF by default.

#### SCG-REQ-342467/A-UAllow bit updated to reflect Smart Charging state changed via Vehicle HMI

If user turns on/off Smart Charging via vehicle HMI, then uAllow bit shall be updated – entity type 1, ID 28

#### SCG-REQ-335245/A-Gateway Module receives Smart Charging Off Command from the Cloud

When the Gateway Module receives the smartChargingOFFCommand from the Cloud, the Gateway Module shall begin the process to turn OFF Smart Charging

#### SCG-REQ-335246/B-Vehicle Turns Off Smart Charging from the Cloud

After receiving the OFF command from the cloud, the Gateway Module shall send smartChargingONRequest = OFF to the Charging Module.  The Gateway Module shall raise FreshData_Rq and hold until Gateway Module receives smartChargingONStatus = OFF.

#### SCG-REQ-335247/A-Charging Module turns Off Smart Charging

Once the Charging Module receives the request from the Gateway Module, the Charging Module shall turn OFF Smart Charging and return smartChargingONStatus = OFF to the Gateway Module.

#### SCG-REQ-335248/B-Charging Module sends Smart Charging Off Status to Vehicle HMI

Charging Module shall send smartChargingONStatus = OFF to the Vehicle HMI.  The Vehicle HMI shall toggle OFF Smart Charging.

#### SCG-REQ-335249/B-Gateway Module responds to Cloud Command that Smart Charging is Off

After the Charging Module has successfully turned OFF Smart Charging the Gateway Module shall send smartChargingOFFCommandResponse with successful status and smartChargingONStatus = OFF to the cloud to recognize Smart Charging feature was successfully turned OFF for the Vehicle. Gateway Module return the FreshData_Rq to 0 once SmartChargingONStatus = OFF is received.

#### SCG-REQ-335250/A-Cloud to Gateway Module Turn Off Command Time Out

After receiving smartChargingOFFCommand from the Cloud, if the Gateway Module does not send a response in 90 seconds then the command shall time out.

#### SCG-REQ-335251/A-Gateway Module reports Smart Charging Off Failure to the Cloud

In the event that Smart Charging Status fails, the Gateway Module shall report failure back to the cloud.  Gateway Module sends smartChargingOFFCommandResponse with a failure value and description of the error

#### SCG-REQ-335252/B-Vehicle HMI sends Smart Charging On request to Charging Module

In order to turn ON Smart Charging through the Vehicle HMI the Vehicle HMI shall send smartChargingONRequest = ON to the Charging Module.

#### SCG-REQ-343290/A-Charging Module Turns On Smart Charging

After the Charging Module receives smartChargingONRequest to turn Smart Charging ON from the Vehicle HMI, the Charging Module shall turn ON Smart Charging and return smartChargingONStatus = ON to the Gateway Module and Vehicle HMI to signify the request was successful.

#### SCG-REQ-335256/B-Vehicle HMI Displays SC On/Off Toggle as On

After the Vehicle HMI receives smartChargingONStatus = ON from the Charging Module, the Vehicle HMI shall display the SC ON/OFF toggled to the ON position.

#### SCG-REQ-335257/B-Gateway Module Alerts the Cloud that Smart Charging was turned On

Upon receiving SmartChargingONStatus from the Charging Module,the Gateway Module shall send smartChargingONAlert = ON to the cloud to recognize Smart Charging was successfully turned ON in the vehicle.

#### SCG-REQ-335258/B-Charging Module Reports Smart Charging On Failure to Vehicle HMI

In the event that Smart Charging ON Request fails, the Charging Module shall report failure back to the Vehicle HMI screen.

#### SCG-REQ-335259/B-Vehicle HMI sends Smart Charging Off request to Charging Module

To turn OFF Smart Charging through the Vehicle HMI the Vehicle HMI shall send smartChargingONRequest = OFF to the Charging Module.

#### SCG-REQ-335274/B-Vehicle HMI displays SC On/Off toggle as Off

After the Vehicle HMI receives smartChargingONStatus = OFF from the Charging Module, the Vehicle HMI shall display the SC ON/OFF toggled to the OFF position.

#### SCG-REQ-335276/B-Gateway Module alerts the Cloud that Smart Charging was turned Off

After the OFF Status is returned to the Vehicle HMI the Gateway Module shall send smartChargingONAlert with value of OFF to the cloud to recognize Smart Charging was successfully turned OFF in the vehicle.

#### SCG-REQ-335277/B-Charging Module reports Smart Charging Off Failure to Vehicle HMI

In the event that Smart Charging OFF fails, the Gateway Module shall report failure back to the HMI screen.

#### SCG-REQ-343291/A-Charging Module turns OFF Smart Charging

After the Vehicle HMI sends the request to turn OFF Smart Charging the Charging Module shall turn OFF Smart Charging and return smartChargingONStatus = OFF to the Gateway Module and Vehicle HMI once successful.

### Use Cases

#### SCG-UC-REQ-329021/B-User Turns On Smart charging via FP/LW

| Actors | FP/LW User,Cloud, Vehicle |
| --- | --- |
| Pre-conditions | User registers for FP/LW app User authorizes vehicle CCS settings are turned ON Smart Charging is ACTIVATED Smart Charging is OFF |
| Scenario | The user wants to turn on smart charging from FP/LW The user toggles the smart charging feature from OFF > ON |
| Post-conditions | SDN receives command from FP/LW to turn on smart charging SDN sends a command to the ECG to turn on smart charging ECG sends request to HPCM to turn on Smart Charging HPCM turns on Smart Charging HPCM does NOT delete existing preferred charge times and location if some already exist HPCM sends status to ECG and APIM APIM shows Smart Charging as ON ECG sends command response to SDN that Smart Charging is on SDN notifies FP/LW that Smart Charging is ON |
| Interfaces | FP/LW, SDN, ECG |
| Notes |  |

#### SCG-UC-REQ-329022/B-User Turns On Smart charging via SYNC HMI

| Actors | Vehicle |
| --- | --- |
| Pre-conditions | User registers to FP/LW app User authorizes vehicle CCS settings are turned ON Smart Charging is ACTIVATED Smart charging is OFF |
| Scenario | The user wants to turn on smart charging from SYNC HMI The user toggles the smart charging feature from OFF > ON |
| Post-conditions | APIM sends request to HPCM to turn on smart charging HPCM turns on Smart Charging HPCM does NOT delete existing preferred charge times and location if some already exist HPCM sends status to ECG and APIM APIM shows Smart Charging as ON ECG sends alert to SDN that Smart Charging is on SDN notifies FP/LW that Smart Charging is ON |
| Interfaces | SYNC HMI, APIM, ECG |
| Notes |  |

#### SCG-UC-REQ-329027/A-User turns On Smart charging when CCS settings are OFF (FP/LW request)

| Actors | FP/LW user |
| --- | --- |
| Pre-conditions | User registers for FP/LW app User authorizes vehicle CCS settings are turned OFF (location sharing or vehicle data or driving characteristics or remote controls or vehicle connectivity) Smart Charging is ACTIVATED Smart charging is OFF |
| Scenario | The user wants to turn on smart charging from FP/LW |
| Post-conditions | The user toggles the smart charging feature from OFF > ON SDN informs FP/LW that CCS settings are OFF The user gets an error message informing him/her that CCS settings must be turned on in the vehicle User has the option to see CCS settings user guide to learn about it and know how to turn it ON |
| Interfaces | FP/LW, SDN |
| Notes |  |

#### SCG-UC-REQ-329028/A-User Turns On Smart charging when CCS settings are OFF (SYNC HMI request)

| Actors | User, Vehicle |
| --- | --- |
| Pre-conditions | User registers for FP/LW app User authorizes vehicle CCS settings are turned OFF (location sharing or vehicle data or driving characteristics or remote controls or vehicle connectivity) Smart Charging is ACTIVATED Smart charging is OFF |
| Scenario | The user wants to turn on smart charging from SYNC HMI |
| Post-conditions | The user toggles the smart charging feature from OFF > ON on the SYNC HMI APIM checks CCS settings ECG informs APIM that CCS Settings are OFF The user gets an error message on the SYNC HMI screen informing him/her they must turn on CCS settings User can click on the notification to be redirected to the CCS settings screen to turn on CCS |
| Interfaces | SYNC HMI, APIM, ECG |
| Notes |  |

#### SCG-UC-REQ-329029/A-User Turns On Smart charging via FP/LW - Command Failed

| Actors | FP/LW User |
| --- | --- |
| Pre-conditions | User registers for FP/LW app User authorizes vehicle CCS settings are turned ON Smart Charging is ACTIVATED Smart Charging is OFF |
| Scenario | The user wants to turn on smart charging from FP/LW The user toggle the smart charging feature from OFF > ON |
| Post-conditions | SDN receives request from FP/LW to turn on smart charging SDN sends a command to the ECG to turn on smart charging ECG fails to processes the command ECG sends a command response to the SDN with a failure state SDN marks the command as "FAILED" SDN informs FP/LW command has failed FP/LW user is notified command has failed |
| Interfaces | FP/LW, SDN, ECG |
| Notes |  |

#### SCG-UC-REQ-329030/A-User Turns On Smart charging via FP/LW - Command timeout

| Actors | FP/LW User |
| --- | --- |
| Pre-conditions | User registers for FP/LW app User authorizes vehicle CCS settings are turned ON Smart Charging is ACTIVATED Smart Charging is OFF |
| Scenario | The user wants to turn on smart charging from FP/LW The user toggle the smart charging feature from OFF > ON |
| Post-conditions | SDN receives request from FP/LW to turn on smart charging SDN sends a command to the ECG to turn on smart charging SDN does not get command response within 90sec SDN marks the command as "TIMEOUT" SDN notifies FP/LW command has timed out User is notified command has timed out |
| Interfaces | FP/LW, SDN |
| Notes |  |

### White Box Views

#### Activity Diagrams

##### SCG-ACT-REQ-329635/B-Turn On or Off Smart Charging From Offboard

##### SCG-ACT-REQ-329636/B-Turn On or Off Smart Charging From In Vehicle

#### Sequence Diagrams

##### SCG-SD-REQ-329949/A-Turn On or Off Smart Charge from the Offboard

##### SCG-SD-REQ-329950/B-Turn On or Off Smart Charge from Onboard

## SCG-FUN-REQ-328710/A-Update Smart Charge Settings

### Requirements

#### SCG-REQ-335279/B-Gateway Module receives Updated Smart Charging Settings from the Cloud

When the Gateway Module receives SCSettingsUpdateCommand with the updated Smart Charging Settings from the Cloud, the Gateway Module shall begin the process to update the Smart Charging Settings. Cloud shall send all Smart Charging Settings for all Smart Charging Locations whenever an update is required.

#### SCG-REQ-335290/B-Gateway Module responds to Cloud command to notify that it received the Update SC Settings Command

After the command is received from the Cloud, the Gateway Module shall return SCSettingsUpdateCommandRepsonse with “in-progress” status to the Cloud to acknowledge that it received the command.

#### SCG-REQ-335291/C-Gateway Module sends request to Charging Module with Updated Smart Charging Settings

The Gateway Module sends SCUpdateSettings_Rq to the Charging Module with the SC Settings Payload for the Charging Module to update Smart Charge Settings. The Gateway Module shall raise FreshData_Rq and hold until Gateway Module receives  PayloadUpdateStatus = “Complete”.

#### SCG-REQ-335292/C-Charging Module updates Smart Charging Settings stored in the vehicle

The Charging Module shall update and save SC Settings with the updated SC Settings Payload which contain new values for Target SOC, Charge Windows, Charge Duration, Charge Duration State, and Max Power. The Charging Module then sends the PayloadUpdateStatus = Complete to the Gateway Module.

#### SCG-REQ-335294/C-Gateway Module sends alert to cloud that Smart Charging Setting were successfully updated

After the Gateway Module receives the PayloadUpdateStatus = Complete from the Charging Module and confirms, the Gateway Module shall send SCSettingsUpdateAlert with the  successful status to the Cloud to acknowledge the vehicle successfully updated the Smart Charging Settings.Gateway Module return the FreshData_Rq to 0 once it has received PayloadUpdateStatus = “Complete”.

#### SCG-REQ-335295/C-Charging Module sends Smart Charging Settings to Vehicle HMI

After the Charging Module is done updating the Smart Charging Settings Payload, it shall send SC_Minimum_SOC_Actual to the Vehicle HMI to display

#### SCG-REQ-335296/A-Cloud to Gateway Module Update SC Settings command time out

After receiving SCSettingsUpdateCommand from the Cloud, if the Gateway Module does not send a response in 90 seconds then the command shall time out.

#### SCG-REQ-335298/C-Gateway Module sends failure if received Payload does not match the request sent originally

If the PayloadUpdateStatus changes from “In-progress” to “Null” the Charging Module failed to save the new Smart Charging payload. The Gateway Modules shall send SCSettingsUpdateAlert with the failure status to the cloud.

#### SCG-REQ-342471/A-Gateway Module Error Codes for Failure to Update

Gateway Module shall have the following error codes for issues with updating the Charging Module and Smart Charging Settings in the vehicle.  Failures shall be sent to the cloud.

- EVErrorENUM{

  - //Charging Module did not respond

    - NO_RESPONSE_FROM_HPCM = 0;

  - //Incomplete charge settings data provided

    - INCOMPLETE_CHARGE_SETTINGS_DATA = 1;

  - //Charging Module is busy with another update

    - ANOTHER_UPDATE_IN_PROGRES = 2;

#### SCG-REQ-343292/B-Cloud sends Smart Charging Location Names to the Gateway Module

The Cloud shall send Smart Charging Location Names with associated location ID to the Gateway Module within the SC Settings Payload.

#### SCG-REQ-336982/C-Location Name is Optional to Display on Vehicle HMI

Location Name is optional to display on the Vehicle HMI. If location name contains a null value the Vehicle HMI shall not display a value/name.

#### SCG-REQ-339227/A-Smart Charging Location Label Names

SmartChargingOnBoardHMIClient shall store labels for Smart Charging Location via Service Oriented Architecture (SOA) as sent from SmartChargingClient.

Example:

Smart Charging Location ID 01 == Home

Smart Charging Location ID 02 == Work

Smart Charging Location ID 03 == Gym

#### SCG-REQ-343878/A-Static List Locations

The Charging Module, Vehicle HMI, and Gateway Module shall maintain Smart Charging locations and associated settings in a static position with respect to the other Saved Charge Location IDs.

#### SCG-REQ-355860/A-Smart Charging Location Label Synchronization

Gateway Module (ECG) shall synchronize Smart Charge Location x with the Vehicle HMI (APIM) via Service Oriented Architecture(SOA) when there is a mobile account authenticated with the vehicle. Synchronization shall occur when the following changes are made to Smart Charge Location Settings:

  - Creation of a Smart Charge Location

  - Deletion of a Smart Charge Location

  - Modification of the name of a label for a Smart Charge Location

The following steps apply for Label Synchronization when Vehicle is OFF

- After an Sync Smart Charging Profiles Command is received from SDN, the ECG will store the Smart Charging Location Name in the following cases:

  - APIM does not respond.  ECG will retry 4 times with 15 internal

  - Vehicle is OFF (APIM is OFF)

- ECG will keep a cached list of all Smart Charging location names and will add it as part of the correlated Alert payload  along with new location name as received in the SDN command.

- When APIM is awake in the next ignition cycle, the ECG will transmit the updated Smart Charging Location Names to APIM for synchronization

- ECG will store the updated Smart Charging location name until APIM responds back with the appropriate location names list (per ECG comparison). Once confirmed by APIM, the ECG will retain the Smart Charging location names in the cache and will not need to transmit any names to APIM.

### Use Cases

#### SCG-UC-REQ-329031/A-Smart Charging Settings Update Requests (Cloud Initiated)

| Actors | Cloud, Vehicle |
| --- | --- |
| Pre-conditions | SDN has updated the smart charging settings |
| Scenario | SDN sends a command to the ECG to update the smart charging settings |
| Post-conditions | ECG sends an acknowledgment (command response) to the SDN that it was able to process the command SDN processes the command response SDN marks the command as "IN-PROGRESS" |
| Interfaces | SDN, ECG |
| Notes |  |

#### SCG-UC-REQ-329032/C-Update Smart Charging Settings Successful

| Actors | Vehicle, Cloud |
| --- | --- |
| Pre-conditions | SCG-UC-REQ-329031ECG has received request to update smart charging settings |
| Scenario | ECG process the SC charge settings payload sent from the SDN ECG sends SC charge settings payload and update request to HPCM |
| Post-conditions | HPCM overrides previous SC settings HPCM responds to ECG with update status HPCM sends appropriate SC settings to APIM ECG sends an alert to the SDN that SC settings were successfully updated SDN marks the current command from "IN-PROGRESS" to "COMPLETED" |
| Interfaces | HPCM, ECG, APIM |
| Notes |  |

#### SCG-UC-REQ-329033/C-Update Smart Charging Settings Failed

| Actors | Vehicle, Cloud |
| --- | --- |
| Pre-conditions | SCG-UC-REQ-329031 ECG has received request to update charge settings |
| Scenario | ECG process the SC charge settings payload sent from the SDN ECG sends SC charge settings payload update to HPCM |
| Post-conditions | HPCM fails to override previous SC settings HPCM responds to ECG with update status as failure ECG sends an alert to the SDN that SC settings failed to update SDN marks the current command from "IN-PROGRESS" to "FAILED” |
| Interfaces | HPCM, ECG, SDN |
| Notes |  |

#### SCG-UC-REQ-329034/A-Smart Charging Settings Update Requests Command Failed

| Actors | Cloud, Vehicle |
| --- | --- |
| Pre-conditions | SDN has updated the Smart charging settings |
| Scenario | SDN sends a command to the ECG to update the Smart Charging settings |
| Post-conditions | ECG processes the command ECG sends an acknowledgment (command response) to the SDN that it was NOT able to process the command SDN processes the command response SDN marks the command as "FAILED" |
| Interfaces | SDN, ECG |
| Notes |  |

#### SCG-UC-REQ-329035/A-Smart Charging Settings Update Requests Command Timeout

| Actors | Cloud, Vehicle |
| --- | --- |
| Pre-conditions | SDN has updated the Smart Charging settings |
| Scenario | SDN sends a command to the ECG to update the Smart Charging settings |
| Post-conditions | SDN does not get a command response within 90sec SDN marks the command as timeout |
| Interfaces | SDN |
| Notes |  |

### White Box Views

#### Activity Diagrams

##### SCG-ACT-REQ-329637/C-Smart Charging Learned Settings Update

#### Sequence Diagrams

##### SCG-SD-REQ-329957/C-Machine Learning

## SCG-FUN-REQ-328712/B-Charging & Arriving at a Smart Charging Location

### Requirements

#### SCG-REQ-335302/A-Vehicle charges to higher charge between Target and Minimum SOC

The vehicle shall charge according the higher value for state of charge between the target SOC and user set minimum SOC.

#### SCG-REQ-335303/C-Vehicle is at Smart Charging Location

If the vehicle is plugged-in at a Smart Charging location, the Charging Module shall charge according to the SC settings Payload, only if Smart Charging is ON.

#### SCG-REQ-335304/B-Vehicle is not at a Smart Charging Location

If Smart Charging is ON, but vehicle is not at a Smart Charging location, the charging module shall charge immediately upon plugging in.

#### SCG-REQ-335308/A-Vehicle is at location with duration setting ON

If vehicle is at a Smart Charging location with duration state as ON, charging module will charge immediately upon plugging in and duration timer countdown will begin

#### SCG-REQ-335309/B-Vehicle is at a location with duration setting ON, target SOC not reached by end of time

If vehicle does not reach the higher value between target SOC and minimum SOC by the end of the duration timer, the vehicle shall return to previous setting.  The vehicle shall charge to the higher value between target SOC and minimum SOC based on Smart Charging settings.  If there is a future window before departure time Smart Charging shall charge later to reach the higher value between target SOC and minimum SOC.  If no window exists, the vehicle shall charge outside of any window to reach the higher value between target SOC and minimum SOC.

#### SCG-REQ-335310/B-Vehicle is at a location with duration setting ON, target SOC is reached before end of time

If vehicle reaches the higher value between target SOC and minimum before the end of duration timer, the vehicle will continue to charge until duration timer runs out

#### SCG-REQ-335311/A-Vehicle is at a location with duration setting ON, vehicle is unplugged during charging

If duration setting is ON for a location and the vehicle is unplugged during charging, then the duration timer shall reset and the timer will default to its original value upon plug-in

#### SCG-REQ-335338/A-Vehicle is at a location with duration setting ON, cloud smart charging settings updated during charging

If charging setting is ON for a location and the duration timer is running, any new duration times sent as part of the Smart Charging settings payload shall update the time remaining from old duration value.  The vehicle shall use the amount of time passed on the Duration_Countdown_Timer and subtract time elapsed from the new duration timer to come up with the updated Duration_Countdown_Timer tunable parameter that is equal to the total duration timer sent from the cloud minus the amount of time the vehicle has already charged.  This will insure that the vehicle only charges for a total amount of time equal to the updated duration that was sent from the cloud.

#### SCG-REQ-335339/B-Digital Station Schedule

The OBCC module shall receive a charging schedule - SAScheduleTuple – to the vehicle via ChargeParameterDiscoveryRes message, which shall include EVSE ID, Time interval points, and max power points per time interval points. This requirement is captured in the PnCChargingStationInfoAlert.

#### SCG-REQ-335341/A-Vehicle HMI displays the higher charge percentage for target SOC

When the vehicle is located at a SC location it shall display the location target SOC as the higher charge percentage value between the Location Target SOC and the Global Minimum SOC set by the user.

#### SCG-REQ-335342/A-Smart Charging Duration State is OFF by default

SC_Duration_ON_OFF_Request = OFF by default.

#### SCG-REQ-335343/A-Duration Timer begins immediately upon plug in

Duration timer shall begin immediately upon plug in.

#### SCG-REQ-335344/B-Duration Timer during lost power incident while charging

If the charging station loses power (ChrgrInPwMde_D_Actl = 0x0 (EVSE Not detected) is signal used to indicate loss in power) while charging the vehicle and a duration timer is active, the timer shall be paused until the charge is resumed.

#### SCG-REQ-342472/B-Charging Module updates Gateway Module the Vehicle has reached a Smart Charging Saved Location

When the vehicle arrives at a Smart Charging Location the Charging Module shall update ChrgLocIdCurnt_D_Sav = 0 and send to the Gateway Module along with the ScLocIdCurnt_No_Stat = the current Smart Charging  location of the vehicle.  ScLocIdCurnt_No_Stat shall also be part of the VSTAT bundle.

#### SCG-REQ-349016/A-Charging Module updates Vehicle HMI and Gateway Module when the Vehicle has reached a Smart Charging Location

When vehicle arrives at a Smart Charging Location, the Charging Module shall send ScLocIdCurnt_No_Stat and CurntTargetSOC_St to vehicle HMI and the Gateway Module.

#### SCG-REQ-342473/B-Gateway Module sends Alert when Vehicle has reached a Smart Charging Location

When the vehicle is keyed off at a Smart Charging Location the Gateway Module shall send xEVSmartChargingLocationReachedAlert to the cloud to alert it that the vehicle has arrived at a Smart Charging Location and at which location ID.  The alert shall contain the most recent values for ChrgLocIDCurnt_D_Sav =0 and ScLocIdCurnt_No_Stat = the current Smart Charging Location ID the Gateway Module received from the Charging Module.

#### SCG-REQ-349017/A-Conflict Notifications - Charging Outside Time Windows

The vehicle may need to start charging outside the smart charging windows in order to achieve the desired charge level if a Departure Time is set before the charge end time occurs.In such instances, the charging Module shall set NtfctnConflict1_D_Rq = “Charging outside time window” (state 0x3) and send signal to the Gateway Module and to the Vehicle HMI.

#### SCG-REQ-349018/A-Conflict Notifications - Charging Outside Time Windows Vehicle HMI

Vehicle HMI shall ignore NtfctnConflict1_D_Rq = “Charging outside time window” (state 0x3) when sent by the Charging Module and Smart Charging is turned ON.

#### SCG-REQ-349019/A-Conflict Notifications - Charging Outside Time Windows Alert to Cloud

When Gateway Module receives NtfctnConflict1_D_Rq = “Charging outside time window” (state 0x3) from the Charging Module it shall populate the signal and send XEVChargeScheduleChangeNotificationAlert to the Cloud.

#### SCG-REQ-349020/A-Conflict Notifications - Cannot Reach Target SOC by Departure Time Charging Module Set Signal

The vehicle may not reached the desired charge level if a Departure Time is set before the charge end time occurs. In such instances, cabin conditioning may be limited and charging Module shall populate NtfctnConflict1_D_Rq = “Not reaching desired Level” (state 0x2) and send to the Gateway Module and to the Vehicle HMI

#### SCG-REQ-349021/A-Conflict Notifications - Cannot Reach Target SOC by Departure Time Vehicle HMI

Vehicle HMI shall ignore NtfctnConflict1_D_Rq = “Not reaching desired level” (state 0x2) when it is sent by the Charging Module while Smart Charging is turned ON.

#### SCG-REQ-349022/A-Conflict Notifications - Cannot Reach Target SOC by Departure Time Alert to Cloud

Upon receiving NtfctnConflict1_D_Rq = “Not reaching desired level” (state 0x2) from the Charging Module, the Gateway Module shall send XEVChargeScheduleChangeNotificationAlert to the Cloud with the signal populated.

### Use Cases

#### SCG-UC-REQ-329036/A-User plugs in, Smart Charging (SC) is Turned On, SC location exists

| Actors | User, Vehicle |
| --- | --- |
| Pre-conditions | Smart charging is activated Smart charging is turned on |
| Scenario | User plugs a charger into their vehicle |
| Post-conditions | HPCM checks if Smart Charging is turned on Smart Charging is turned on, HPCM checks if location has SC settings SC settings location exists, HPCM charges based on SC settings for location |
| Interfaces | HPCM, ECG |
| Notes | Charge processing is the same as existing, no changes to it |

#### SCG-UC-REQ-329038/B-User plugs in, SC is Turned On, Location is not a Smart Charging Location

| Actors | User, Vehicle |
| --- | --- |
| Pre-conditions | Smart Charging is ACTIVATED Smart charging is turned on |
| Scenario | User plugs a charger into their vehicle |
| Post-conditions | HPCM checks if Smart Charging is on Smart Charging is ON, HPCM checks if location has SC settings SC settings for location DOES NOT exist, HPCM “charges immediately” – even if it was a previously saved location with Preferred Charge Times |
| Interfaces | HPCM |
| Notes | Charge processing is the same as existing, no changes to it |

#### SCG-UC-REQ-329039/A-User plugs in, Smart Charging is Off

| Actors | User, Vehicle |
| --- | --- |
| Pre-conditions | Smart charging is turned off |
| Scenario | User plugs a charger into their vehicle |
| Post-conditions | HPCM checks if Smart Charging is ON Smart Charging is OFF, so HPCM uses standard preferred charge logic for the current location |
| Interfaces | HPCM |
| Notes |  |

#### SCG-UC-REQ-329040/A-EVSE Schedule Sent To Cloud

| Actors | EVSE, Vehicle |
| --- | --- |
| Pre-conditions | Smart Charging is ACTIVATED Smart Charging is ON |
| Scenario | User plugs vehicle into EVSE with a digital schedule |
| Post-conditions | OBCC picks default schedule – SAScheduleTuple - provided by EVSE. Schedule includes: - EVSE ID - Time interval points (displayed as seconds from NOW or 0) - Max power points per time interval points OBCC sends default schedule to ECG using TP protocol ECG sends protocol to SDN with default schedule |
| Interfaces | OBCC, ECG, SDN |
| Notes |  |

### White Box Views

#### Activity Diagrams

##### SCG-ACT-REQ-330006/B-Plugin

See PNC-ACT-REQ-330594-Plugged In and Charging Has Not Begun Transmit EVSEid

#### Sequence Diagrams

##### SCG-SD-REQ-330005/B-Plugin

See PNC-ACT-REQ-330599-Plugged In and Charging Has Not Started Transmit EVSEid

##### SCG-SD-REQ-349023/A-Vehicle Arrives at Smart Charging Location

## SCG-FUN-REQ-328711/B-Updating Minimum State Of Charge Settings

### Requirements

#### SCG-REQ-335345/B-Vehicle HMI sends new Minimum SOC to Charging Module

After the user updates the minimum SOC through the Vehicle HMI, the Vehicle HMI sends the new value for SC_Minimum_SOC_Request to the Charging Module.

#### SCG-REQ-335347/B-Charging Module updates Smart Charging Settings stored in the Vehicle based on the new Minimum SOC

The Charging Module processes the new SC Minimum SOC Request from the Vehicle HMI and successfully updates minimum SOC. The Charging Module shall then send back the updated minimum SOC to the Gateway Module and to the Vehicle HMI. The Vehicle HMI shall display the updated Minimum SOC.

#### SCG-REQ-335350/B-Gateway Module alerts the Cloud that Smart Charging Settings were updated

After receivin the updated minimum SOC from the Charging Module, the Gateway Module shall send SCSettingsUpdateAlert with the Updated minimum SOC to the cloud to notify the cloud of the new Smart Charging Settings.

#### SCG-REQ-335351/B-Charging Module Reports Smart Charging Settings Update Failure back to User

In the event that SC_Minimum_SOC_Request fails, the Charging Module shall report failure back to the Vehicle HMI.

### Use Cases

#### SCG-UC-REQ-329024/B-User wants to Set Minimum SOC in HMI

| Actors | Cloud, Vehicle |
| --- | --- |
| Pre-conditions | Smart Charging is ACTIVATED Smart Charging is ON |
| Scenario | User wants to set/update minimum SOC for SC |
| Post-conditions | User selects minimum SOC APIM sends minimum SOC to HPCM HPCM will charge to higher percentage between minimum SOC and target SOC for location HPCM sends status back to APIM and ECG ECG sends an alert to SDN with updated SC settings |
| Interfaces | SDN, APIM, ECG, HPCM |
| Notes |  |

### White Box Views

#### Activity Diagrams

##### SCG-ACT-REQ-333381/B-Update Minimum SOC

#### Sequence Diagrams

##### SCG-SD-REQ-334519/B-Update Minimum SOC

## SCG-FUN-REQ-333070/B-Update CCS via HMI

### Requirements

#### SCG-REQ-335366/C-User Turns Off CCS

If the user turns OFF any single or combination of the following CCS – Vehicle Data, Vehicle Connectivity, Location Sharing, Driving Characteristics - while Smart Charging is ON the Gateway Module shall shutdown the Smart Charging app.

#### SCG-REQ-335368/C-Gateway Module informs Charging Module Smart Charging is OFF

Before the Gateway Module shuts down, it sends smartChargingONRequest = OFF to the Charging Module.

#### SCG-REQ-343297/B-Charging Module turns OFF Smart Charging and responds to Gateway Module and Vehicle HMI

The Charging Module shall process the smartChargingONRequest = OFF from the Gateway Module and turn OFF Smart Charging.  The Charging Module then returns smartChargingONStatus = OFF to the Gateway Module and to the Vehicle HMI.

#### SCG-REQ-335370/B-Vehicle HMI displays SC ON/OFF toggle as OFF

After receiving smartChargingONStatus = OFF from the Charging Module, the Vehicle HMI shall display Smart Charging toggle as OFF.

## SCG-FUN-REQ-328713/A-Master or Brand Reset

### Requirements

#### SCG-REQ-335372/B-Gateway Module receives request to initiate Master Reset

Cloud Initiated:

After receiving the ClearAllUserSettingsCommand from the Cloud, the Gateway Module shall send ClearAllSettings_Rq to the Charging Module to reset all Smart Charging Settings and turn OFF Smart Charging.  If a user does not authorize within 48 hours, the Smart Chargign DID shall reset, as well, per current functionality.

#### SCG-REQ-343298/A-Vehicle HMI sends Master Reset to the Charging Module

Vehicle HMI Initiated:

After receiving FactoryReset_Rq from the Vehicle HMI, the Charging Module to reset all Smart Charging Settings and turn OFF Smart Charging.  If a user does not authorize within 48 hours, the Smart Chargign DID shall reset, as well, per current functionality.

#### SCG-REQ-335373/C-Charging Module turns OFF Smart Charging and RESETS all Smart Charging Settings from the Vehicle

Cloud Initiated:

After receiving the ClearAllSettings_Rq from the Gateway Module, the Charging Module shall turn OFF Smart Charging and RESET all Smart Charging settings in the vehicle before resetting base PEPC charge settings

Vehicle HMI Initiated:

After receiving the FactoryReset_Rq from the Vehicle HMI, the Charging Module shall turn OFF Smart Charging and RESET all Smart Charging settings in the vehicle before resetting base PEPC charge settings.

#### SCG-REQ-335374/B-Charging Module sends Status to the Vehicle HMI that Smart Charging has been turned OFF

Cloud Initiated:

After resetting Smart Charging Signals, the Charging Module shall send smartChargingONStatus = OFF to the Vehicle HMI

Vehicle HMI Initiated:

After resetting Smart Charging Signals, the Charging Module shall send smartChargingONStatus = OFF to the Vehicle HMI

#### SCG-REQ-335375/B-Gateway Module sends alert to the Cloud that Master Reset was successful

Cloud Initiated:

After Master Reset is complete, the Gateway Module shall send ClearAllUserSettingsCommandResponse to the Cloud to signify that Smart Charging has been turned OFF and Smart Charging Settings were RESET in the Charging Module.

Vehicle HMI Initiated:

After Master Reset is complete the Gateway Module shall send MasterResetAlert = Master Reset to the Cloud to signify that Smart Charging has been turned OFF and Smart Charging Settings were RESET in the Charging Module.

#### SCG-REQ-335377/B-Vehicle HMI requests to initiate Brand Reset

Vehicle HMI triggers a Brand Reset and shall send EmbeddedModemReset _Rq = Save to the Charging Module to SAVE Smart Charging Settings and turn OFF Smart Charging.  If EmbeddedModemReset_Rq = Reset than the Charging Module shall Reset Smart Charging Settings and turn OFF Smart Charging.

#### SCG-REQ-335378/B-Charging Module turns OFF Smart Charging and SAVES all Smart Charging Settings on the Vehicle

After receiving EmbeddedModemReset_Rq = Save, the Charging Module shall SAVE Smart Charging Settings and turn OFF Smart Charging.  The Charging Module shall respond to the Vehicle HMI with smartChargingONStatus = OFF and EmbeddedModemReset_Status = Saved once complete.

If the Charging Module receives EmbeddedModemReset_Rq = Reset, the Charging Module shall Reset Smart Charging Settings and turn OFF Smart Charging. The Charging Module shall respond to the Gateway Module and Vehicle HMI with smartChargingONStatus = OFF and EmbeddedModemReset_Status = Reset once complete

#### SCG-REQ-335380/B-Gateway Module alerts the Cloud that Brand Reset was successful

Once the Brand Reset is complete the Gateway Module shall send MasterResetAlert = Brand Reset to update the Cloud that Smart Charging was turned OFF and Smart Charging Settings were saved in the Charging Module. During a Brand Reset, If a user does not authorize within 48 hours, the Smart Chargign DID shall reset, as well, per current functionality.

#### SCG-REQ-335381/B-Gateway Module reports failure to turn OFF Smart Charging to Vehicle HMI and the user

In the event that the Brand Reset fails, the Charging Module shall report failure back to the Vehicle HMI.

### Use Cases

### White Box Views

#### Activity Diagrams

##### SCG-ACT-REQ-333379/B-Reset Performed In Vehicle

##### SCG-ACT-REQ-333380/B-Last User Removed Reset

#### Sequence Diagrams

##### SCG-SD-REQ-334554/C-Master Reset Performed In Vehicle

##### SCG-SD-REQ-334561/C-Brand Reset Performed In Vehicle

##### SCG-SD-REQ-334553/B-Last User Master Reset

## SCG-FUN-REQ-333209/A-Delete Smart Charge Location

### Requirements

#### SCG-REQ-335386/B-Gateway Module receives command to delete SC location

Gateway Module receives command from the Cloud to delete a Smart Charging Location. The Cloud shall send all Smart Charging Settings for all Smart Charging Locations to the Gateway Module.  Within the payload, The DeleteSCLocationRequest shall be flagged as Yes for whatever SC Location needs to be deleted. The Gateway Module shall send the updated Smart Charging Settings payload to the Charging Module.

#### SCG-REQ-343052/C-Gateway Module responds to cloud with in-progress Command Response during after receiving Delete Smart Charging Location Command

After receiving the Delete Smart Charging Location Command from the Cloud, the Gateway Module shall respond with a command response with the status of “Success”. The Gateway Module shall raise the FreshData_Rq and hold until Gateway Module receives PayloadUpdateStatus = “Complete”.

#### SCG-REQ-335387/D-Charging Module deletes SC location

After receiving the request to delete a smart charging location from the Gateway Module, the Charging Module shall delete the Smart Charging Location and all the settings tied to it; the other Smart Charging locations and settings shall be saved with the values in the payload. The Charging module shall then send PayloadUpdateStatus = “Complete” to the Gateway Module.

#### SCG-REQ-335388/C-Gateway Module sends  alert to the cloud once successfully deleted location

After receiving the PayloadUpdateStatus = “Complete” from the Charging Module, the Gateway Module shall send SCSettingsUpdateAlert to the cloud with a successful status for Delete SC Location. Gateway Module shall return FreshData_Rq to 0 once it has received PayloadUpdateStatus = “Complete”.

#### SCG-REQ-335390/A-Cloud to Gateway Module Delete SC Location Command Time Out

After receiving DeleteSCLocationCommand from the Cloud, if the Gateway Module does not send a response in 90 seconds then the command shall time out.

#### SCG-REQ-335392/C-Gateway Module reports Delete SC Location failure to the Cloud

In the event that Delete SC Location fails, (PayloadUpdateStatus changes from “In-progress” to “Null” the Charging Module failed to save the new Smart Charging Payload and delete the selected SC Location. The Gateway Module shall send SCSettingsUpdateAlert with a failure value and description of the error.

#### SCG-REQ-337298/B-Cloud shall send TurnOFFSmartCharging Command after DeleteSCLocation Command is successful

If the user deletes a Smart Charging Location from the cloud, the cloud shall send the Delete Smart Charging Location command to the vehicle.  After successful completion, then he cloud shall send another command to turn OFF Smart Charging.

### Use Cases

### White Box Views

#### Activity Diagrams

##### SCG-ACT-REQ-333378/C-Delete Smart Charge Location

#### Sequence Diagrams

##### SCG-SD-REQ-334522/C-Delete Smart Charge Location

# Appendix A: Definitions / Acronyms

| Abbr. | Stands for | Description |
| --- | --- | --- |

# Appendix B: Reference Documents

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

The requirements of the documents listed in the reference table above, of the latest revision level, form a part of this Engineering Specification