|  | Ambient Lighting <<Feature>> (F000063 - Model Rev. 196) |  |  |  |
| --- | --- | --- | --- | --- |
| Document Type | Feature Document (FD) |  |  |  |
| Template Version | 6.1b |  |  |  |
| SysML Report Template Version | 6.1b.5 |  |  |  |
| Document ID | FD2 - Ambient Lighting - F000063 196 |  |  |  |
| Document Location | https://www.vsemweb.ford.com/tc/launchapp?-attach=true&-s=226TCSession&-o=SfvJeEL9x3NrTDAAAAAAAAAAAAA&servername=Production_Server |  |  |  |
| Document Owner | Matheus Clavel |  |  |  |
| Document Revision | FD2 |  |  |  |
| Document Status | Released |  |  |  |
| Date Issued | 2022/08/16 |  |  |  |
| Date Revised | 2022/06/27 |  |  |  |
| Model Name and Version | F000063 - Ambient Lighting - RCRUZ92 – [#196] |  |  |  |
| Document Classification | GIS1 Item Number: | 27.60/35 |  |  |
|  | GIS2 Classification: | Confidential |  |  |
| Document Approval |  |  |  |  |
| Person | Role |  | Email Confirmation | Date |
| mclavel | Core Feature Systems Engineer |  | mclavel@ford.com |  |
| bfreita4 | Feature Systems Supervisor |  | bfreita4@ford.com |  |

Auto-Generated by MagicDraw

Printed Copies Are Uncontrolled

# Disclaimer

This document contains Ford Motor Company Confidential information. Disclosure of the information contained in any portion of this document is not permitted without the expressed, written consent of a duly authorized representative of Ford Motor Company, Dearborn, Michigan, U.S.A.

Copyright, Ó 2021  Ford Motor Company

This document contains information developed and accumulated by and for FORD MOTOR COMPANY. As such, it is a proprietary document, which, if disseminated to unauthorized persons, would provide others with restricted information, data, or procedures not otherwise available, exposing the FORD MOTOR COMPANY to potential harm.

Employees and suppliers having custody of this specification or authorized to use it must be cognizant of its proprietary nature and ensure that the information herein is not made available to unauthorized persons.

FORD MOTOR COMPANY reserves the right to protect this work as an unpublished copyrighted work in the event of an inadvertent or deliberate unauthorized publication. FORD MOTOR COMPANY also reserves its rights under copyright laws to protect this work as a published work.

This document or portions thereof shall not be distributed outside FORD MOTOR COMPANY without prior written consent. Refer all questions concerning disclosure to the author(s) or to any duly authorized representative of Ford Motor Company.

Copyright © 2021 Ford Motor Company

# Contents

Disclaimer	2

Contents	3

1	Introduction	5

1.1	Document Purpose	5

1.2	Document Scope	5

1.3	Document Audience	5

1.3.1	Stakeholder List	5

1.4	Document Organization	5

1.4.1	Document Context	5

1.4.2	Document Structure	6

1.5	Document Conventions	6

1.5.1	Classification of Chapters	6

1.5.2	Requirements Templates	6

1.6	References	7

1.6.1	Ford Documents	7

1.6.2	External Documents and Publications	8

1.7	Glossary	8

1.7.1	Definitions	8

1.7.2	Abbreviations	8

1.7.3	Parameters / Values	8

2	Feature Overview	9

2.1	Purpose and Description of Feature	9

2.2	Feature Variants	9

2.2.1	Regions & Markets	10

2.3	Input Requirements/Documents	10

2.4	Lessons Learned	11

2.5	Assumptions	11

3	Feature Context	12

3.1	Feature Context Diagram	12

3.2	List of Influences	12

4	Feature Modeling	14

4.1	Operation Modes and States	14

4.2	Use Cases	18

4.2.1	Use Case Diagram	18

4.2.2	Actors	19

4.2.3	Use Case Descriptions	19

4.3	Driving and Operation Scenarios	Error! Bookmark not defined.

4.4	Decision Tables	Error! Bookmark not defined.

5	Feature Requirements	23

5.1	Functional Requirements	23

5.2	Non-Functional Requirements	32

5.2.1	Security	32

5.2.2	Reliability	32

5.2.3	Performance	32

5.3	HMI Requirements	32

5.4	Other Requirements	34

5.4.1	Design Requirements	34

5.4.2	Manufacturing Requirements	34

5.4.3	Service Requirements	34

5.4.4	After Sales Requirements	34

5.4.5	Process Requirements	34

6	Functional Safety	35

6.1	System Behaviors for HARA	35

6.2	Functional Safety Assumptions	35

6.3	Safety Goals	35

6.4	Functional Safety Requirements	35

6.4.1	Derivation of Functional Safety Requirements on Assumptions	Error! Bookmark not defined.

6.4.2	ASIL Decomposition of Functional Safety Requirements	Error! Bookmark not defined.

7	CyberSecurity	36

7.1	Security Goals	36

7.2	Cybersecurity Requirements	Error! Bookmark not defined.

8	Architecture	37

8.1	Functional Decomposition	37

8.1.1	Functions	38

8.2	Logical Architecture	38

8.2.1	Logical Elements	39

8.2.2	Logical Interfaces	42

9	Traceability Matrix	65

10	Open Concerns	67

11	Revision History	68

12	Appendix	69

12.1	Definitions	69

12.2	Abbreviations	69

List of Figures

Figure 2: Feature Image Here	9

Figure 4: Ambient Lighting Context	12

Figure 5: Operational Modes and States	14

Figure 6: Feature Use Cases & System Behaviors	19

Figure 10: Functional Boundary Behavior	37

Figure 11: Logical	39

List of Tables

Table 1: Features described in this FD	5

Table 3: Ford internal Documents	8

Table 4: External documents and publications	8

Table 6: Feature Variants	10

Table 7: Regions & Markets	10

Table 8: Input Requirements/Documents	11

Table 9: List of Influences	13

Table 10: Operation Modes and States on Operational Modes and States	15

Table 11: Transitions between Operation Modes and States on Operational Modes and States	18

Table 12: List of Actors	19

Table 14: System Behaviors for HARA	35

Table 15: Functional Safety Assumptions	35

Table 16: Functional Safety Goals	Error! Bookmark not defined.

Table 17: Cybersecurity Goals	36

Table 18: List of Functions	38

Table 20: Logical Elements	42

Table 21: Feature Interactions	64

Table 23: Open Concerns (Not supported by MagicDraw report generation)	67

Table 24: Revision History	68

Table 25: Definitions used in this document	69

Table 26: Abbreviations used in this document	69

# Introduction

## Document Purpose

A Feature Document (FD) document defines a Feature on Concept Level. It specifies what the feature shall do and how it shall behave from customer perspective. It should also provide reasoning and background why we have the feature in the vehicle.

The FD also serves as an Item Definition as defined by ISO26262 for those features, which follow the Ford Functional Safety process. Refer FFSG01.10 Feature Document Guideline for how to apply the Feature Doc template for Functional Safety.

## Document Scope

This Feature Document (FD) specifies the following features:

| Feature ID | Feature Name | Owner | Reference |
| --- | --- | --- | --- |
| F000063 - Model Rev. 196 | Ambient Lighting | Matheus Clavel | https://www.vsemweb.ford.com/tc/launchapp?-attach=true&-s=226TCSession&-o=SfvJeEL9x3NrTDAAAAAAAAAAAAA&servername=Production_Server |

Table 1: Features described in this FD

## Document Audience

The FD is written by the feature owner of Ambient Lighting. All Stakeholders, i.e., all people who have a valid interest in the feature should read and, if possible, review the FD. It needs to be guaranteed, that all stakeholders have access to the currently valid version of the FD.

### Stakeholder List

For the latest list of stakeholder of the feature and their influence refer to: Click here to open the latest Stakeholders List.

## Document Organization

### Document Context

Refer to the Specification Structure page in the Ford RE Wiki to understand how the FD relates to other Ford Requirements Documents and Specifications.

### Document Structure

The structure of this document is explained below:

Introduction		– 	Explains how to use this document including responsibilities and requisite documents. Explains the terminology. Gives a clarification of the definitions, concepts and abbreviations used in the document.

Feature Overview		– 	States briefly the background and the purpose of the feature, feature variants and corresponding regions and markets. Also includes input requirements, assumptions, and constraints.

Feature Context 		– 	Describes all external entities, which have an influence on the feature.

Feature Modeling		– 	Contains Use Case, Driving Scenarios, State Charts to describe the functional behavior of the feature.

Feature Requirements 	– 	Lists functional and non-functional requirements of the feature.

Functional Safety		– 	Lists System Behaviors, Safety Goals and Safety Requirements of the feature.

CyberSecurity	 	– 	Lists Security Goals and Security Requirements of the feature.

Architecture 		– 	Shows the coarse architecture, which the feature requirements are deployed to. Describes the elements and the boundary of the feature as well as the decomposition and distribution of associated functions.

Traceability Matrix	– 	Traceability Matrix.

Open Concerns		– 	List of Open Concerns

Revision History 		– 	Document Change History including a list of new or modified requirements. The requirements in this document are tagged, and this section contains different types of tables listing all, new, or changed requirements by their title and page no.

Appendix		– 	Appendix

## Document Conventions

### Classification of Chapters

A chapter is considered mandatory, unless the chapter or its parent chapter(s) are categorized by using the tag:

#Classification: Some Condition

If no requirement/other content is known for a mandatory chapter, leave a statement “Not Applicable”

Some chapters have a follow certain rules in context of specific Ford processes, e.g. Functional Safety. This is indicated at the beginning of the corresponding chapter by the tags:

#Functional Safety: Some process specific explanation

#Cybersecurity: Some process specific explanation

### Requirements Templates

Refer to “How to use the Specification Templates” on how to use the specification templates and the VBA macros to create/edit the requirements in the specifications.

#### Requirements Attributes

The templates provided by Specification_Macros.dotm define a list of attributes for each requirement. This helps to classify the requirement. The attributes are explained at RE Wiki - Requirements Attributes.

## References

### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Document Location | Revision |
| --- | --- | --- | --- | --- |
| Cybersecurity Assessment | Cybersecurity Relevancy Assessment Template V1.2 - Ambient Lighting 09-02-2021 | GIS1 Item Number: 27.60/35 | https://www.vsemweb.ford.com/tc/launchapp?-attach=true&-s=226TCSession&-o=0VuFivKLx3NrTD | v1.2 |
| FD | Feature Document | FD - Ambient Lighting - F000063 | https://www.vsemweb.ford.com/tc/launchapp?-attach=true&-s=226TCSession&-o=SfvJeEL9x3NrTDAAAAAAAAAAAAA&servername=Production_Server |  |
| FIS | Feature Implementation Specification | FIS – Ambient Lighting F000063 | https://www.vsemweb.ford.com/tc/launchapp?-attach=true&-s=226TCSession&-o=zZsJ$ihEx3NrTDAAAAAAAAAAAAA&servername=Production_Server |  |
| Ford GIS Standard | Ford GIS Standard |  |  |  |
| FS | Functional Specification | FGS - Ambient Lighting - F000063 | https://www.vsemweb.ford.com/tc/launchapp?-attach=true&-s=226TCSession&-o=jhqJ$ihEx3NrTDAAAAAAAAAAAAA&servername=Production_Server |  |
| Functional Specification Body Control Module | BCM Functional Specification | FS-RU5T-14B476-AGB001 | https://www.vsemweb.ford.com/tc/launchapp?-attach=true&-s=226TCSession&-o=CzkF7IK0x3NrTD | 16.01 |
| FUSA documentation guideline for FD | FFSG01.10 Feature Document (Item Definition) Guideline |  |  | 2022.1 |
| HMI Spec - APIM | VEHICLE SETTINGS APIM SPSS V1.23 | VSv2-FUN-REQ-025223 |  | 1.23 |

Table 2: Ford internal Documents

### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication | Document Location |
| --- | --- | --- |
| CIE_1938 Standard | Light Color Standard Pattern - Cx, Cy | https://en.wikipedia.org/wiki/CIE_1931_color_space |
| IEEE Std 1012-2004 IEEE Standard for Software Verification and Validation |  |  |
| ISO/IEC 19500-2:2003 | Information technology -- Open Distributed Processing -- Part 2 |  |
| Lumen definition | Lumen Definition | https://pt.wikipedia.org/wiki/L%C3%BAmen |
| UML Testing Profile (UTP), v1.2 |  |  |

Table 3: External documents and publications

## Glossary

See Appendix for Definitions and Abbreviations.

### Definitions

### Abbreviations

### Parameters / Values

No Parameters / Values specified.

# Feature Overview

## Purpose and Description of Feature

With the ambient lighting feature, a user can set the mood in the vehicle’s cabin by selecting one of several different accent colors.

When activated ambient lighting illuminates foot wells, cup holders, and door release handles, map pockets, etc., depending on the vehicle model. A user can also adjust brightness settings to further personalize the interior.

On multicolor variants equipped with door handle lights, when Ambient Lighting feature is available to the user experience it will also be capable of indicating a cabin door being ajar by activating the door’s lights in a pre-configured color (Red).

On vehicles equipped with a Contextual Ambient Lighting (CAL) system, the Ambient Lighting will also provide support the Selectable Drive Mode feature. Which means that when the CAL is in Auto mode, the user will see Ambient Lighting lamps being orchestrated by the Selectable Drive Mode feature (colors shift) through traditional drive modes.

Some features such as Welcome & Farewell, Rejuvenate and Selectable Drive Mode use Ambient Lighting lamps as a resource to perform their requirements.

Figure 1: Feature Image Here

## Feature Variants

| Variant Name | Variant Description | Remarks |
| --- | --- | --- |
| Contextual Ambient Lighting | Variant for vehicles equipped with the SDM feature. |  |
| Ford Multi Color | Variant for Ford brand vehicles (except S650 Mustang and prior) with multi-color ambient lighting capability. |  |
| Ford Single Color | Variant for Ford brand vehicles (except S650 Mustang and prior) with single color ambient lighting capability. |  |
| Lincoln | Variant for Lincoln brand vehicles with multicolor ambient lighting capability. |  |

Table 4: Feature Variants

### Regions & Markets

| Market / Region Variant Name | North America | South America | Europe | Middle East/Africa | Asia / Pacific | China |
| --- | --- | --- | --- | --- | --- | --- |
| Contextual Ambient Lighting | Optional | Optional | Optional | Optional | Optional | Optional |
| Ford Multi Color | Optional | Optional | Optional | Optional | Optional | Optional |
| Ford Single Color | Optional | Optional | Optional | Optional | Optional | Optional |
| Lincoln | Optional | Optional | Optional | Optional | Optional | Optional |

Table 5: Regions & Markets

## Input Requirements/Documents

| Reference (Reference as listed in ch. “References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Feature Requirements”) |
| --- | --- | --- | --- |
| Attribute Requirements |  |  |  |
|  | Function Execution Cycle Time | The Ambient Lighting Control function shall have an execution cycle time of no more than 21ms from receipt of all necessary input signals. |  |
|  | Light Output Color Range | This function shall be able to emit light in the color coordinates based on CIE 1931 Color Space. |  |
|  | Execution of Logical Elements | The ambient lighting control function shall evaluate state charts and decision tables a maximum of one (1) time each execution cycle. |  |
|  | Light Output Intensity Range | This function shall be able to emit light with intensity measurable in lumens [lm] unit at . Its intensity ranges from 0 up to 2.5 lm. |  |
|  | Example AR |  |  |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system should be developed according to Ford's implementation of Functional Safety. |  |
|  | Ambient Lighting Strategy (UX / CIED) | Ambient Lighting shall comply with to RQT-002004-022593 the Ambient Lighting Strategy. |  |
| Other Sources |  |  |  |

Table 6: Input Requirements/Documents

## Lessons Learned

No lessons learned specified.

## Assumptions

No Assumptions specified.

# Feature Context

## Feature Context Diagram

F000063

Figure 2: Ambient Lighting Context

## List of Influences

| ID | External Entity | Influence Description |
| --- | --- | --- |
| Ambient Light | Ambient Lighting To User | Ambient lighting illumination output to vehicle interior. |
| AmbientLightingConfiguration | Diagnostics and Configuration To Ambient Lighting | Configuration data provided through off vehicle diagnostics software. |
| ApproachLightShow | Vehicle Status To Ambient Lighting | Light show feature status. |
| AutoAmbientLightRequest | Rejuvenate To Ambient Lighting | The Rejuvenate feature request the HMI to put Ambient Lighting in automatic mode. |
| BatteryState | Vehicle Status To Ambient Lighting | Battery state of charge. |
| ContextualAmbientLightFeedback | Ambient Lighting To SDM | Current Contextual Ambient Lighting status to inform whether it is in Auto or Manual Mode. |
| ContextualAmbientLightRequest | SDM To Ambient Lighting | User request to Ambient Lighting to let Automatic Drive Mode feature drive Ambient Lighting color. |
| CourtesyDelay | Vehicle Status To Ambient Lighting | Request from Courtesy Delay feature to hold lights On for a preset amount of time. |
| DelayedAccessory | Vehicle Status To Ambient Lighting | Delayed Accessory status. |
| DoorAjar | Vehicle Status To Ambient Lighting | Doors ajar status. |
| DoorLocked | Vehicle Status To Ambient Lighting | Doors locked status. |
| DriveMode | SDM To Ambient Lighting | Current drive mode selection. |
| HeadLampsState | Vehicle Status To Ambient Lighting | Head lamp status. |
| IgnitionState | Vehicle Status To Ambient Lighting | Ignition status. |
| IluminatedEntry | Vehicle Status To Ambient Lighting | Status of feature Illuminated Entry. |
| IluminatedExit | Vehicle Status To Ambient Lighting | Status of feature Illuminated Exit. |
| LightColor | Front HMI To Ambient Lighting | Command to choose Ambient Lighting color. |
|  | Rear HMI To Ambient Lighting | Command to choose Ambient Lighting color. |
|  | Rejuvenate To Ambient Lighting | Command to choose Ambient Lighting color. |
| LightColorFeedback | Ambient Lighting To Front HMI | Current Ambient Lighting color. |
|  | Ambient Lighting To Rear HMI | Current Ambient Lighting color. |
|  | Front HMI To User | Current Ambient Lighting color. |
|  | Rear HMI To User | Current Ambient Lighting color. |
| LightIntensity | Front HMI To Ambient Lighting | Command to choose Ambient Lighting intensity. |
|  | Rear HMI To Ambient Lighting | Command to choose Ambient Lighting intensity. |
|  | Rejuvenate To Ambient Lighting | Command to choose Ambient Lighting intensity. |
| LightIntensityFeedback | Ambient Lighting To Front HMI | Current Ambient Lighting intensity. |
|  | Ambient Lighting To Rear HMI | Current Ambient Lighting intensity. |
|  | Front HMI To User | Current Ambient Lighting intensity. |
|  | Rear HMI To User | Current Ambient Lighting intensity. |
| PersonalityRequest | Enhanced Memory To Ambient Lighting | User request for personalization: Copy of current Ambient Lighting settings to a selected profile. User can recall personalized profile later on. |
| Touch | User To Front HMI | User touches HMI screen. |
|  | User To Rear HMI | User touches HMI screen. |
| User Profile Number | Enhanced Memory To Ambient Lighting | User profile number to store current Ambient Lighting color and intensity. |
| User Profile NumberFeedback | Ambient Lighting To Enhanced Memory | Current profile number number in use. |
| W/F State/Substate | WelcomeFarewell To Ambient Lighting | Current Welcome Farewell status. |

Table 7: List of Influences

# Feature Modeling

## Operation Modes and States

Figure 3: Operational Modes and States

| State | Description | Requirements Reference (optional) |
| --- | --- | --- |
| Ajar | Doors are illuminated in a configurable color when they are ajar. Do behavior: outputLightAjar | - ID=R_F_AMBL_000037 : Door Ajar Behavior |
| Ambient Lighting Disabled | Ambient Lighting not enabled. User can' t change neither lighting color / intensity nor use any other ambient lighting function. Do behavior: outputLightDisabled | - ID=R_F_AMBL_000045 : Enable / Disable Ambient Lighting feature |
| Ambient Lighting Enabled | Ambient Lighting is enabled. User can change either lighting color / intensity and also use any other ambient lighting function. | - ID=R_F_AMBL_000045 : Enable / Disable Ambient Lighting feature |
| Ambient Lighting Running | When in this mode ambient lighting can be driven by manual user input or by SEL feature according to user 's choice through HMI or even by Rejuvenate feature also from user input from HMI. |  |
| Automatic HMI | Ambient Lighting is driven by Rejuvenate feature. Do behavior: outputLightRejuvenate |  |
| Eco | Ambient Lighting is driven by feature SEL drive mode Eco. Do behavior: outputLightEco | - ID=R_F_AMBL_000022 : Contextual Ambient Light behavior when in AUTO Mode - ID=R_F_AMBL_000023 : Drive Mode Selection on the HMI in AUTO Mode - ID=R_F_AMBL_000021 : Contextual Ambient Light behavior when the slider transitions from Manual to AUTO Mode on the HMI |
| Manual | Ambient Lighting is driven by user manual input through HMI. Do behavior: outputLightColorManual | - ID=R_F_AMBL_00003 : Manually Selectable Intensity - ID=R_F_AMBL_000026 : Drive Mode Selection on the HMI in Manual Mode |
| Normal | Ambient Lighting is driven by feature SEL drive mode Normal. Do behavior: outputLightNormal | - ID=R_F_AMBL_000022 : Contextual Ambient Light behavior when in AUTO Mode - ID=R_F_AMBL_000023 : Drive Mode Selection on the HMI in AUTO Mode - ID=R_F_AMBL_000021 : Contextual Ambient Light behavior when the slider transitions from Manual to AUTO Mode on the HMI |
| Operational | Represents the phase after EoL programming / configuration. Feature is already configured and ready. | - ID=R_F_AMBL_00001 : Host Vehicle State (Power Mode) for Feature Operation |
| Selective Drive Mode (CAL) | Ambient Lighting is driven by one of the SEL drive modes: Normal, Eco and Sport. | - ID=R_F_AMBL_000022 : Contextual Ambient Light behavior when in AUTO Mode |
| Sport | Ambient Lighting is driven by feature SEL drive mode Normal. Do behavior: outputLightSport | - ID=R_F_AMBL_000022 : Contextual Ambient Light behavior when in AUTO Mode - ID=R_F_AMBL_000023 : Drive Mode Selection on the HMI in AUTO Mode - ID=R_F_AMBL_000021 : Contextual Ambient Light behavior when the slider transitions from Manual to AUTO Mode on the HMI |
| Welcome / Farewell | Ambient lighting will have its behavior driven by Welcome / Farewell strategy. Do behavior: outputLightWF | - ID=R_F_AMBL_00002 : Conformance to Welcome Farewell Feature Specification |

Table 8: Operation Modes and States on Operational Modes and States

| Transition ID | Source | Destination | Description | Requirements Reference (optional) |
| --- | --- | --- | --- | --- |
| T1 | Automatic HMI | Selective Drive Mode (CAL) | ChangeEvent when ((Automatic_HMI == 'ON') && (AutomaticDriveMode == 'ON')) |  |
| T2 | Manual | Manual | Trigger signal: evTurnAmbientLightingOnOff Effect: turnAmbLightOnOff SignalEvent evTurnAmbientLightingOnOff |  |
| T3 | Initial | Ambient Lighting Disabled | Name: Default Transition Documentation: No Condition. No Action. |  |
| T4 | Manual | Manual | Trigger signal: evIncIntensity Effect: IncrementIntensity SignalEvent evIncIntensity |  |
| T5 | Initial | Operational | Name: Default Transition. Documentation: No Condition. No Action. | Requirements: Host Vehicle State (Power Mode) for Feature Operation |
| T6 | Ambient Lighting Disabled | Welcome / Farewell | Documentation: Event captured as a requirement of Welcome / Farewell feature. ChangeEvent when ((WelcomeFarewellStatus != 'OFF') && (IgnitionStatus == 'OFF')) | Requirements: Host Vehicle State (Power Mode) for Feature Operation, Conformance to Welcome Farewell Feature Specification |
| T7 | Manual | Manual | Trigger signal: evDecColor Effect: decrementColor SignalEvent evDecColor | Requirements: Host Vehicle State (Power Mode) for Feature Operation, Conformance to Welcome Farewell Feature Specification, Ramp ON (To Welcome/Farewell Mode) |
| T8 | Sport | Eco | Trigger signal: evChangeDriveMode Effect: setLightColorEco SignalEvent evChangeDriveMode |  |
| T9 | Ambient Lighting Disabled | Ambient Lighting Enabled | ChangeEvent when ((WelcomeFarewellStatus == 'OFF') && ((IgnitionStatus != 'OFF') && (HeadLampSwitchStatus != 'OFF'))) |  |
| T10 | Ambient Lighting Enabled | Welcome / Farewell | ChangeEvent when ((WelcomeFarewellStatus != 'OFF') && (IgnitionStatus == 'OFF')) |  |
| T11 | Selective Drive Mode (CAL) | Manual | Trigger signal: evIncColor Effect: retreaveLastColor SignalEvent evIncColor | Requirements: Conformance to Welcome Farewell Feature Specification |
| T12 | Initial | Manual | Name: Default Documentation: No Condition. No Action. |  |
| T13 | Eco | Normal | Trigger signal: evChangeDriveMode Effect: setLightColorNormal SignalEvent evChangeDriveMode |  |
| T14 | Ambient Lighting Enabled | Ambient Lighting Disabled | ChangeEvent when ((WelcomeFarewellStatus == 'OFF') && ((IgnitionStatus == 'OFF') ¦¦ (HeadLampSwitchStatus == 'OFF'))) |  |
| T15 | Ambient Lighting Running | Ajar | ChangeEvent when ((DoorStatus == 'AJAR') && (multiColorCfg == true)) |  |
| T16 | Selective Drive Mode (CAL) | Automatic HMI | ChangeEvent when (Automatic_HMI == 'ON') | Requirements: Door Ajar Behavior |
| T17 | Welcome / Farewell | Ambient Lighting Enabled | ChangeEvent when ((IgnitionStatus != 'OFF') && (HeadLampSwitchStatus == true)) |  |
| T18 | Normal | Sport | Documentation: Represents an event of user changing driver mode. Trigger signal: evChangeDriveMode Effect: setLightColorSport SignalEvent evChangeDriveMode | Requirements: Conformance to Welcome Farewell Feature Specification |
| T19 | Ajar | Deep History | ChangeEvent when (DoorStatus == 'CLOSED') |  |
| T20 | Selective Drive Mode (CAL) | Manual | Trigger signal: evDecColor Effect: retreaveLastColor SignalEvent evDecColor |  |
| T21 | Manual | Manual | Trigger signal: evDecIntensity Effect: decrementIntensity SignalEvent evDecIntensity |  |
| T22 | Initial | Ambient Lighting Running | Name: Default Transition. Documentation: No Condition. No Action. |  |
| T23 | Manual | Manual | Trigger signal: evIncColor Effect: incrementColor SignalEvent evIncColor |  |
| T24 | Manual | Automatic HMI | ChangeEvent when ((Automatic_HMI == 'ON') && (ContextAL_Config == 'ENABLE')) |  |
| T25 | Manual | Selective Drive Mode (CAL) | Effect: storeLastColor ChangeEvent when ((AutomaticDriveMode == 'ON') && (Automatic_HMI == 'OFF') && (ContextAL_Config == 'ENABLE')) |  |
| T26 |  |  | Effect: setLightColorNormal | Requirements: a, Soft button to toggle AAL (Auto Ambient Light) between AUTO or MANUAL. |
| T27 | Welcome / Farewell | Ambient Lighting Disabled | Name: s ChangeEvent when ((WelcomeFarewellStatus == 'OFF') && (IgnitionStatus == 'OFF')) | Requirements: a, Soft button to toggle AAL (Auto Ambient Light) between AUTO or MANUAL., Contextual Ambient Light behavior when the slider transitions from Manual to AUTO Mode on the HMI |
| T28 | Automatic HMI | Manual | ChangeEvent when (Automatic_HMI == 'OFF') |  |

Table 9: Transitions between Operation Modes and States on Operational Modes and States

## Use Cases

### Use Case Diagram

Figure 4: Feature Use Cases & System Behaviors

### Actors

| Actor | Description |
| --- | --- |
| Enhanced Memory | Represents Enhanced Memory feature. |
| Rejuvenate | Represents Rejuvenate feature. |
| SDM | Represents Selectable Drive Mode feature. |
| User | Person requesting ambient lighting changes. |
| WelcomeFarewell | Represents Welcome / Farewell feature. |

Table 10: List of Actors

### Use Case Descriptions

UC_F_AMBL_00001 Enable Ambient Operation

| Actors | Primary | User |
| --- | --- | --- |
|  | Secondary |  |
| Subject |  | Ambient Lighting |
| Description |  |  |
| Preconditions | PreC1 | Vehicle Ignition is in RUN/START. |
| Main Flow Description |  | To describe the scenario of enabling “Ambient” operation mode of the feature. |
| Main Flow | M1 | The User turns the headlamp switch to ON. |
| Alternative Flow Steps | A1 | The user turns the headlamp switch to AUTO. |
|  | A2 | Nighttime visibility conditions have been detected. |
| Postconditions | PostC1 | The Ambient lighting turns ON in Ambient Mode. |

UC_F_AMBL_00002 Disable Ambient Operation

| Actors | Primary | User |
| --- | --- | --- |
|  | Secondary |  |
| Subject |  | Ambient Lighting |
| Description |  |  |
| Preconditions | PreC1 | Vehicle Ignition is in RUN/START. |
| Main Flow Description |  | To describe the scenario of disabling “Ambient” operation mode of the feature. |
| Main Flow | M1 | The User turns the headlamp switch to OFF. |
| Alternative Flow Steps | A1 | A1 User turns headlamp switch to AUTO. |
|  | A2 | A2 Daytime visibility conditions is detected. |
|  | A3 | B1 User turns ignition off. |
| Postconditions | PostC1 | The Ambient lighting turns OFF. |

UC_F_AMBL_00003 Perform Welcome Farewell Behavior (F000064)

| Actors | Primary | User |
| --- | --- | --- |
|  | Secondary | WelcomeFarewell |
| Subject |  | Ambient Lighting |
| Description |  |  |
| Preconditions | PreC1 | Vehicle Ignition is OFF. |
| Main Flow Description |  | To describe a scenario of the feature responding to a Welcome Farewell Trigger. For a full list of triggers and behaviors refer to the Welcome Farewell / Lincoln Embrace Strategy RQT-002004-704098. |
| Main Flow | M1 | The User interacts with the Welcome Farewell feature. |
|  | M2 | Welcome Farewell instructs ambient lighting to turn on/off in a specific color and intensity. |
| Postconditions | PostC1 | Based on instructions received the Ambient Lights are ON in a specific color and intensity or OFF. |

UC_F_AMBL_00004 Change Ambient Lighting Color

| Actors | Primary | User |
| --- | --- | --- |
|  | Secondary |  |
| Subject |  | Ambient Lighting |
| Description |  |  |
| Preconditions | PreC1 | Color X is active. |
|  | PreC2 | Intensity X is active. |
|  | PreC3 | Vehicle ignition is in RUN/START. |
| Main Flow Description |  | To describe the scenario of a user changing the ambient lighting color setting. |
| Main Flow | M1 | The user selects color Y through the feature HMI. |
| Postconditions | PostC1 | The ambient lighting is ON in color Y and intensity X The HMI screen displays updated settings. |

UC_F_AMBL_00005 Change Ambient Lighting Intensity

| Actors | Primary | User |
| --- | --- | --- |
|  | Secondary |  |
| Subject |  | Ambient Lighting |
| Description |  |  |
| Preconditions | PreC1 | Vehicle ignition is in RUN/START. |
| Main Flow Description |  | To describe the scenario of a user changing the ambient lighting intensity setting. |
| Main Flow | M1 | The user selects a new intensity through the feature HMI. |
| Postconditions | PostC1 | The ambient lighting is ON in a new color and intensity The HMI screen displays updated settings. |

UC_F_AMBL_00006 Turn OFF Ambient Lighting via HMI

| Actors | Primary | User |
| --- | --- | --- |
|  | Secondary |  |
| Subject |  | Ambient Lighting |
| Description |  |  |
| Preconditions | PreC1 | Ambient Lighting is ON in the vehicle. |
|  | PreC2 | Vehicle ignition is in RUN/START. |
| Main Flow Description |  | To describe the scenario of a user turning OFF ambient lighting via the HMI. |
| Main Flow | M1 | The user selects Ambient Lighting OFF via the HMI. |
| Postconditions | PostC1 | The Ambient Lighting is turned OFF in the vehicle The HMI shows Ambient Lighting turned OFF . |

UC_F_AMBL_00007 Personalize Ambient Lighting Profile

| Actors | Primary | User |
| --- | --- | --- |
|  | Secondary | Enhanced Memory |
| Subject |  | Ambient Lighting |
| Description |  |  |
| Preconditions | PreC1 | Profile X exists with associated ambient lighting settings but is not active. |
|  | PreC2 | Vehicle equipped with Enhanced Memory. |
| Main Flow Description |  | Describes the scenario of changing ambient lighting settings through interaction with the Enhanced Memory Feature. Reference the Enhanced Memory feature specification (F000172) for more details on methods by which a driver profile can be changed. |
| Main Flow | M1 | The user changes to driver profile X via Enhanced Memory. |
|  | M2 | Enhanced Memory provides new profile to ambient lighting. |
| Postconditions | PostC1 | The ambient lights transition to the color and intensity associated with profile X The HMI screen displays updated settings. |

UC_F_AMBL_00008 Indicate Door Ajar

| Actors | Primary | User |
| --- | --- | --- |
|  | Secondary |  |
| Subject |  | Ambient Lighting |
| Description |  |  |
| Preconditions | PreC1 | All doors are closed. |
|  | PreC2 | Ambient Lighting is enabled. |
|  | PreC3 | Lincoln/Ford Multicolor variant ONLY. |
|  | PreC4 | Vehicle ignition is in RUN/START. |
| Main Flow Description |  | To describe the scenario of a user opening a door or leaving it ajar in ambient operation mode. |
| Main Flow | M1 | A user opens any vehicle door. |
| Postconditions | PostC1 | The door ambient lighting LEDs for any open door are ON in the Ajar color (Red) at 100% intensity. |

UC_F_AMBL_00009 Perform Contextual Ambient Lighting in AUTO Mode

| Actors | Primary | User |
| --- | --- | --- |
|  | Secondary | SDM |
| Subject |  | Ambient Lighting |
| Description |  |  |
| Preconditions | PreC1 | AAL is set to ON. |
|  | PreC2 | Ambient Lighting is enabled. |
|  | PreC3 | CAL variant ONLY. |
|  | PreC4 | Vehicle ignition is in RUN/START. |
| Main Flow Description |  | To describe the scenario of a user changing the drive mode in Auto Mode for Contextual Ambient Lighting. |
| Main Flow | M1 | The active drive mode changes from Drive Mode X to Drive Mode Y. |
|  | M2 | The Ambient Lighting changes color from that associated with Drive Mode X to that associated with Drive Mode Y. |
| Alternative Flow Steps | A1 | The Ambient Lighting is ON in Drive Mode Y associated color and the intensity is unchanged. |

UC_F_AMBL_000010 Perform Rejuvenate

| Actors | Primary | User |
| --- | --- | --- |
|  | Secondary | Rejuvenate |
| Subject |  | Ambient Lighting |
| Description |  |  |
| Preconditions | PreC1 | All doors are closed. |
|  | PreC2 | Ambient Lighting is enabled. |
|  | PreC3 | Headlamps are NOT OFF. |
|  | PreC4 | Lincoln variant ONLY. |
|  | PreC5 | Vehicle ignition is in RUN/START. |
| Main Flow Description |  | Describe the scenario of the Rejuvenate feature taking control of the ambient lighting. |
| Main Flow | M1 | The User activates Rejuvenate Mode. |
|  | M2 | All manual input methods for Ambient Lighting are made inaccessible. |
|  | M3 | Rejuvenate requests ambient lighting activation in specific color and intensity. |
|  | M4 | Repeat M2 for each phase of Rejuvenate Mode. |
|  | M5 | Rejuvenate Mode ends and restores Ambient Lighting to the original state prior to experience. |
| Postconditions | PostC1 | Ambient Lighting is in original state. |

# Feature Requirements

## Functional Requirements

R_F_AMBL_00001 Host Vehicle State (Power Mode) for Feature Operation

While the Ignition is NOT OFF and HeadLamp Switch is ON or AUTO ON, the feature Ambient Lighting shall available.

| Requirement ID: R_F_AMBL_00001 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_00002 Conformance to Welcome Farewell Feature Specification

While the Ignition is OFF the ambient lights shall available to perform the Welcome & Farewell feature behaviors.

| Requirement ID: R_F_AMBL_00002 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - DR_FNC_45 Generate Internal Welcome Farewell Status Signal via Decision Table 4.2-1 - DR_FNC_46 WelcomeFarewell_Status Value at Reset - R_FNC_00001 Receive Welcome Farewell State & Substate Signals |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_00003 Manually Selectable Intensity

While the feature operation state, the feature Ambient Lighting shall allow a user to manually select the ambient lighting intensity.

| Requirement ID: R_F_AMBL_00003 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_00004 Light Output Color

While the feature operation state, and when more than one Ambient Lighting color is available, the feature Ambient Lighting shall allow a user to manually select the ambient lighting color.

| Requirement ID: R_F_AMBL_00004 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_00005 Ambient Lighting User Feedback

While the feature is in Operational state, the feature Ambient Lighting shall provide feedback of active settings.

| Requirement ID: R_F_AMBL_00005 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_00006 Data Analytics

Ambient Lighting shall record:

o    How frequently users change the color of Ambient Lighting (Multi Color Variants)

o    How often users manually change the intensity of Ambient Lighting

o    How often NJOD color is overridden by customer (Contextual Ambient Lighting)

| Requirement ID: R_F_AMBL_00006 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_00007 Lighting Transition Interruption

While a ramp ON or ramp OFF illumination behavior is running, if a new request is received, then the feature Ambient Lighting shall execute the new request instantly.

| Requirement ID: R_F_AMBL_00007 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_00008 Ramp ON (Ambient Mode)

While the feature operation state, and when transitioning to ON, the feature Ambient Lighting shall ramp ON to the desired intensity at a configurable rate.

| Requirement ID: R_F_AMBL_00008 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_00009 Ramp OFF (Ambient Mode)

While the feature operation state, and when transitioning to OFF, the feature Ambient Lighting shall ramp OFF at a configurable rate.

| Requirement ID: R_F_AMBL_00009 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000010 Ramp ON (To Welcome/Farewell Mode)

While the Welcome & Farewell state, and when transitioning to ON, the ambient lights shall ramp ON at a configured rate of 3 seconds.

| Requirement ID: R_F_AMBL_000010 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000011 Ramp OFF (From Welcome/Farewell Mode)

While the Welcome & Farewell state, and when transitioning to OFF, the ambient lights shall ramp OFF at a configured rate of 4 seconds.

| Requirement ID: R_F_AMBL_000011 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000012 Enhanced Memory – Store Profile Settings

While the feature operation state, and when the feature Enhanced Memory operations are supported, the feature Ambient Lighting shall support Enhanced Memory by storing settings for each Personalization Profile as they are updated.

| Requirement ID: R_F_AMBL_000012 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000013 Enhanced Memory - Operations - Copy

While the feature operation state, and when the feature Enhanced Memory operations are supported, the feature Ambient Lighting shall provide resources to copy Ambient Lighting settings from the active personalization profile onto the selected personalization profile.

| Requirement ID: R_F_AMBL_000013 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000015 Contextual Ambient Lighting at reset

At reset Contextual Ambient Lighting shall be in Auto Mode.

| Requirement ID: R_F_AMBL_000015 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000017 Contextual Ambient Lighting Behavior on transition from OFF/ACCESSORY to RUN/START in Manual Mode

When manual mode is active, the transition from OFF/ACCESSORY to RUN/START shall cause the ambient lighting to transition from last manually chosen color and 100 percent intensity to last manually chosen color and last saved intensity.

| Requirement ID: R_F_AMBL_000017 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - DR_FNC_4 Determination of Ambient Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000018 Contextual Ambient Lighting Behavior on transition from RUN/START to OFF/ACCESSORY in Manual Mode

When occur the transition from feature operation enabled to feature operation disabled (Ignition OFF), if MANUAL mode is active, then the feature ambient lighting shall illuminate in 100 percent intensity in the last manually choosen color for 25 seconds.

| Requirement ID: R_F_AMBL_000018 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - DR_FNC_4 Determination of Ambient Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000019 Manual color change through HMI while in AUTO Mode

While auto mode is active, when a manual ambient lighting color change request occurs via the HMI, Contextual Ambient Lighting shall transition from auto to manual and go to the selected color at the same intensity.

| Requirement ID: R_F_AMBL_000019 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000020 Contextual Ambient Lighting behavior while in Manual Mode

When in MANUAL Mode the Contextual Ambient Lighting shall use the last manually selected color and intensity for illumination.

| Requirement ID: R_F_AMBL_000020 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000021 Contextual Ambient Light behavior when the slider transitions from Manual to AUTO Mode on the HMI

When a transitions from MANUAL to AUTO Mode occurs, Contextual Ambient Lighting shall use the color that corresponds to the active drive mode from the SDM for illumination.

| Requirement ID: R_F_AMBL_000021 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000022 Contextual Ambient Light behavior when in AUTO Mode

When Contextual Ambient Lighting is in AUTO Mode, the Ambient Lighting color used shall correspond with the active drive mode.

| Requirement ID: R_F_AMBL_000022 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000023 Drive Mode Selection on the HMI in AUTO Mode

While Contextual Ambient Lighting is in AUTO mode, when a new drive mode is selected, the feature Ambient Lighting shall be in the color associated with the new drive mode when illuminated.

| Requirement ID: R_F_AMBL_000023 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000024 Contextual Ambient Lighting behavior on transition from RUN/START to OFF/ACCESSORY in AUTO Mode

When a transition from RUN/START to OFF/ACCESSORY occurs while Auto mode is active, Contextual Ambient Lighting shall cause the ambient lights to illuminate in the last manually chosen color and 100 percent intensity for 25 seconds.

| Requirement ID: R_F_AMBL_000024 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - DR_FNC_4 Determination of Ambient Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000025 Contextual Ambient Lighting behavior on transition from OFF/ACCESSORY to RUN/START in AUTO Mode

When occur the transition from feature operation disabled (ignition OFF) to feature operation enabled, if AUTO mode is active, then the feature shall use the color associated with the active drive mode reveived from SDM.

| Requirement ID: R_F_AMBL_000025 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - DR_FNC_4 Determination of Ambient Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000026 Drive Mode Selection on the HMI in Manual Mode

When occur the transition from feature operation enabled to feature operation disabled (Ignition OFF), if AUTO mode is active, then the feature ambient lighting shall illuminate in 100 percent intensity in the last manually choosen color for 25 seconds.

| Requirement ID: R_F_AMBL_000026 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000027 Intensity change on HMI in AUTO Mode

When occur the transition from feature operation disabled (ignition OFF) to feature operation enabled, if AUTO mode is active, then the feature shall use the color associated with the active drive mode reveived from SDM.

| Requirement ID: R_F_AMBL_000027 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000028 Profile change from Profile X to Profile Y (No change to AAL or Drive Mode, AAL Mode is in AUTO)

When the profile is changed from profile X to profile Y with no change to AAL and drive mode, while AAL is in Auto Mode, there will be no color change, and the intensity will change to profile Y intensity setting.

| Requirement ID: R_F_AMBL_000028 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000029 Profile change from Profile X to Profile Y (No change to AAL or Drive Mode, AAL Mode is in Manual)

When the profile is changed from profile X to profile Y with no change to AAL and drive mode, while AAL is in Auto Mode, there will be a color and intensity change if stored values for profile Y are different from profile X.

| Requirement ID: R_F_AMBL_000029 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000030 Profile change from Profile X to Profile Y (Change to AAL Mode, No change to Drive Mode, AAL Mode changes from Manual to AUTO)

When the profile is changed from profile X to profile Y with a change to AAL Mode, no change to Drive Mode, when AAL Mode changes from manual to auto mode, there will be a color change if defined SDM color values for profile Y are different than profile X drive mode. Ambient Lighting intensity will be set to profile Y’s saved intensity.

| Requirement ID: R_F_AMBL_000030 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000031 Profile change from Profile X to Profile Y (Change to AAL Mode, No change to Drive Mode, AAL Mode changes from AUTO to Manual)

When the profile is changed from profile X to profile Y with a change to AAL Mode, no change to Drive Mode, when AAL Mode changes from auto to manual mode, there will be a color and intensity change if stored values for profile Y are different than profile X.

| Requirement ID: R_F_AMBL_000031 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000032 Profile change from Profile X to Profile Y (No change to AAL Mode, Change to Drive Mode, AAL Mode is in Manual)

When the profile is changed from profile X to profile Y with no change to AAL Mode, a change to Drive Mode, when AAL Mode is in manual mode, there will be a color and intensity change if stored values for profile Y are different than profile X.

| Requirement ID: R_F_AMBL_000032 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000033 Profile change from Profile X to Profile Y (No change to AAL Mode, Change to Drive Mode, AAL Mode is in AUTO)

When the profile is changed from profile X to profile Y with a change to AAL Mode, no change to Drive Mode, when AAL Mode changes from manual to auto mode, there will be a color change if defined SDM color values for profile Y are different than profile X drive mode. Ambient Lighting intensity will be set to profile Y’s saved intensity.

| Requirement ID: R_F_AMBL_000033 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000034 Profile change from Profile X to Profile Y (Change to AAL Mode, Change to Drive Mode, AAL Mode changes from Manual to AUTO)

When the profile is changed from profile X to profile Y with a change to AAL Mode, change to Drive Mode, when AAL Mode changes from manual to auto mode, there will be a color change if defined SDM color values for profile Y are different than profile X stored color. Ambient Lighting intensity will be set to profile Y’s saved intensity.

| Requirement ID: R_F_AMBL_000034 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000035 Profile change from Profile x to Profile Y (Change to AAL Mode, Change to Drive Mode, AAL changes from AUTO to Manual)

When the profile is changed from profile X to profile Y with a change to AAL Mode, a change to Drive Mode, when AAL Mode changes from auto to manual mode, there will be a color change if stored values for profile Y are different than SDM color values for profile X drive mode. Ambient Lighting intensity will be set to profile Y’s saved intensity.

| Requirement ID: R_F_AMBL_000035 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000036 AAL Mode Slider Behavior

While the feature operation state, the feature Ambient Lighting shall provide HMI control elements for the following commands regarding AAL (Auto Ambient Lighting): ON and OFF.

| Requirement ID: R_F_AMBL_000036 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000037 Door Ajar Behavior

When any door is ajar, while the Ignition is ON, the ajar door 's light shall turn on in the red color (or the ajar color config. in data dictionary) and 100 percent intensity.

| Requirement ID: R_F_AMBL_000037 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000038 Rejuvenate Ambient Lighting Settings Retention

While the feature operation state, the feature Ambient Lighting shall provide HMI control elements for the following commands regarding AAL (Auto Ambient Lighting): ON and OFF.

| Requirement ID: R_F_AMBL_000038 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000045 Enable / Disable Ambient Lighting feature

When Welcome / Farewell status is OFF and ((ignition status is not OFF) or (headlamp status is not OFF))

ambient lighting Shall enable its functions to the user (changing lighting colors, intensity, using SEL, etc.)

| Requirement ID: R_F_AMBL_000045 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Functional Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Non-Functional Requirements

### Security

No Security Requirements specified.

### Reliability

No Reliability Requirements specified.

### Performance

R_F_AMBL_000044 Flickering avoidance

The ambient lighting feature shall not flicker.

| Requirement ID: R_F_AMBL_000044 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | Non-Functional Requirement - Performance |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## HMI Requirements

R_F_AMBL_000016 Soft button to toggle AAL (Auto Ambient Light) between AUTO or MANUAL.

On the HMI, there shall be a soft button to toggle between Auto and Manual modes that is always accessible in RUN/START under normal operation.

| Requirement ID: R_F_AMBL_000016 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | HMI Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000039 Variant HMI display

Each Feature Variant Shall have a distinct HMI display.

| Requirement ID: R_F_AMBL_000039 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | HMI Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000040 Lighting intensity indication

The HMI shall have a means of indicating intensity for each ambient lighting color.

| Requirement ID: R_F_AMBL_000040 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | HMI Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000041 HMI Lockout when in Rejuvenate Mode.

When Rejuvenate Mode is active, the HMI for Ambient Lighting shall be inaccessible or unresponsive to user requests.

| Requirement ID: R_F_AMBL_000041 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | HMI Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000042 Rear HMI lockout /disabling from front HMI

The front HMI shall provide the option to lockout/disable the rear HMI for this feature.

| Requirement ID: R_F_AMBL_000042 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | HMI Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

R_F_AMBL_000043 CAL Manual vs Automatic setup option

The front HMI shall provide the option to select Contextual Ambient Lighting between MANUAL and AUTOMATIC.

| Requirement ID: R_F_AMBL_000043 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes | HMI Requirement |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Other Requirements

### Design Requirements

Not supported by MagicDraw report generation.

### Manufacturing Requirements

No Manufacturing Requirements specified.

### Service Requirements

No Service Requirements specified.

#### Cloud Connectivity Data Analytics Requirements

### After Sales Requirements

No After Sales Requirements specified.

### Process Requirements

No Process Requirements specified.

# Functional Safety

## System Behaviors for HARA

| ID | Name | Description |
| --- | --- | --- |
| 1 | Provide Illumination | System function that contains all feature functions that are related to provide the actual light. |
| 2 | Provide User input and Feedback to HMI | Receive/Translate/Transmit User Input to change ambient lighting settings and the feedback sent to the primary and secondary (second-row display) HMI. This System Behavior won't be considered in HARA (as mentioned in FFSG01.10 Feature Document at item 2.3.6.1 - Item Definition Guideline). |

Table 11: System Behaviors for HARA

## Functional Safety Assumptions

| ID | Assumption |  |
| --- | --- | --- |
| ALA1 | Name | Unintended On when initial state is on |
|  | Description | Assumed initial state is On and User/System requested intentionally to be on and does not turns off. |
|  | Purpose | Severity rationale |
|  | Category | Behavioral |
|  | Related Requirement IDs |  |

Table 12: Functional Safety Assumptions

## Safety Goals

Feature is QM. Not applicable.

## Functional Safety Requirements

Feature is QM. Not applicable.

# CyberSecurity

## Security Goals

| ID | Goal |  |
| --- | --- | --- |
| 1 | Goal Name | Cyber Security Not Relevant / Not Applicable |
|  | Description |  |
|  | CAL |  |
|  | Related CSR IDs |  |

Table 13: Cybersecurity Goals

# Architecture

## Functional Decomposition

This functional boundary diagram pictures logical function allocation in terms of logical function units.

Figure 5: Functional Boundary Behavior

### Functions

| Function Name | Description | Comments |
| --- | --- | --- |
| (activity) ProvideIllumination |  |  |
| (activity) ProcessUserInputAndFeedback | (activity) Updates HMI 1 with user with current user settings. |  |
| (activity) ProcessUserFeedback | (activity) Updates HMI 2 with user with current user settings. |  |
| (activity) ProcessAmbientLightingOutput | (activity) Determines which light / group of light is to be turned ON / OFF. Determines which behavior (Welcome, Farewell, Ajar, etc) is to be performed by a single / group of lights. |  |
| (activity) PerformLedPowerControl | (activity) Command Ambient Light Module power supply ON / OFF according to Table 2.2.17-25 LIN-LED Power Control. |  |
| (activity) PerformALMCircuitOutput | (activity) Manages ambient lighting by providing LED light module with pwm duty cycle necessary to light dimming, turn ON/OFF command, diagnostic reading, etc. |  |
| (activity) PerformAmbientLightingInputProcessing | (activity) Process ambient lighting color and intensity requested by user and updates HMI. |  |
| (activity) PerformWelcomeFarewellFeatureProcessing | (activity) Determines which Welcome Farewell behavior is to be used when Ambient Lighting Feature is not operating in Ambient mode. |  |
| (activity) PerformCoreAmbientLightingProcessing | (activity) Manages Ambient Lighting features: - HMI Comm Variant Decoding - Memory and Personalization - Contextual Ambient Lighting (Selectable Drive Modes). |  |
| (activity) PerformAmbientProcessing | (activity) Command ambient lighting ON / OFF according to decision table 2.2.17-3 Ambient Lighting Decision Table. |  |
| (activity) ProcessAmbientLightOutput | (activity) Process ambient lighting requests and send them out to ALM through LIN. |  |

Table 14: List of Functions

## Logical Architecture

Ambient Lighting logical blocks and data shared among them.

Figure 6: Logical

Architecture

### Logical Elements

| Element Name | Description | Allocated Functions | Comments |
| --- | --- | --- | --- |
| ALMCircuitOutput | Logic that process command to drive ambient lighting ON / Off when ambient lighting is configured as single color. | - PerformALMCircuitOutput - DetermineALMConditionedPwrCmd - DetermineALMPowerStatus - DetermineALMCktOutput |  |
| AmbientLighting | Logical block responsible to do all of the ambient lighting processing /management. | - PerformCoreAmbientLightingProcessing - PerformWelcomeFarewellFeatureProcessing - PerformAmbientLightingInputProcessing - PerformAmbientProcessing - PerformLedPowerControl - ProcessAmbientLightingOutput - ProcessAmbientLightOutput - ProcessUserInputAndFeedback - ProcessUserFeedback - PerformALMCircuitOutput - DeterminePassengerRearDoorLockLightBehavior - DeterminePassengerFrontDoorLockLightBehavior - DetermineDriverRearDoorLockLightBehavior - DetermineDriverFrontDoorLockLightBehavior - PerformSelectiveDriveModeRequestDecision - PerformSelectiveDriveModeRangeSelectionDecision - PerformHMICommVariantDecoding - PerformDriveModeBasedLightAmbientColorDecision - PerformContextualAmbientLightingColorIntensityValueDecision - PerformContextualAmbientLightingHMIStatusUpdate - PerformContextualAmbientLighting - PerformAmbientLightingMemoryAndPersonalization - PerformAmbientColorMappingDecision - PerformAmbientIntensityMappingDecision - DeterminePersonalizedAmbientLightingStatus - DeterminePersonalizedAmbientLightingIntensity - DeterminePersonalizedAmbientLightingColor - DeterminePassengerRearDoorLightBehavior - DeterminePassengerRearDoorLockIndication - DeterminePassengerFrontDoorLockIndication - DeterminePassengerFrontDoorLightBehavior - DetermineNonDoorLIN Master_L1_P00LedState - DetermineLedParams - DetermineLedLocationID - DetermineDriverRearDoorLockIndication - DetermineDriverFrontDoorLockIndication - DetermineDriverFrontDoorLightBehavior - DetermineDriverRearDoorLightBehavior - DetermineALMPowerStatus - DetermineALMConditionedPwrCmd - CheckLINWakeUp - CalculateAmbientLightingActiveProfileIndex - GenerateMessage0x37AmbientLightingData - ProvideIllumination |  |
| AmbientLightingOutput | Logic that provides ambient lighting with behavior according to each light location inside the vehicle. | - ProcessAmbientLightingOutput - DetermineDriverRearDoorLightBehavior - DetermineDriverFrontDoorLightBehavior - DeterminePassengerRearDoorLightBehavior - DeterminePassengerFrontDoorLightBehavior - DetermineNonDoorLIN Master_L1_P00LedState - DetermineDriverFrontDoorLockIndication - DeterminePassengerFrontDoorLockIndication - DetermineDriverRearDoorLockIndication - DeterminePassengerRearDoorLockIndication - DetermineDriverFrontDoorLockLightBehavior - DeterminePassengerFrontDoorLockLightBehavior - DetermineDriverRearDoorLockLightBehavior - DeterminePassengerRearDoorLockLightBehavior |  |
| AmbientLightOutput | Logic responsible to pack ambient lighting information (color, intensity, update, etc) and send it over the network. | - ProcessAmbientLightOutput - GenerateMessage0x37AmbientLightingData - CheckLINWakeUp - DetermineLedLocationID - DetermineLedParams |  |
| AmbientProcessing | Logic responsible to decide when ambient lighting feature should be enabled / disabled. | - PerformAmbientProcessing |  |
| Core | Logical module responsible to compute / manage variant configurations, user profile personalization and user driver mode selection. | - PerformCoreAmbientLightingProcessing - PerformHMICommVariantDecoding - PerformAmbientLightingMemoryAndPersonalization - PerformContextualAmbientLighting - PerformContextualAmbientLightingHMIStatusUpdate - PerformSelectiveDriveModeRequestDecision - PerformSelectiveDriveModeRangeSelectionDecision - PerformContextualAmbientLightingColorIntensityValueDecision - CalculateAmbientLightingActiveProfileIndex - DeterminePersonalizedAmbientLightingStatus - DeterminePersonalizedAmbientLightingColor - DeterminePersonalizedAmbientLightingIntensity |  |
| DiagnosticsAndConfig | Logic responsible to provide ambient lighting configuration data to ambient lighting. |  |  |
| FrontHMILogic | Logic responsible to provide lighting color, intensity, drive mode and rejuvenate option from HMI to ambient lighting as well as update HMI with current user choice. | - ProcessUserInputAndFeedback |  |
| InputProcessing | Logic responsible to process user input command and update HMI with user choices. | - PerformAmbientLightingInputProcessing - PerformAmbientColorMappingDecision - PerformDriveModeBasedLightAmbientColorDecision - PerformAmbientIntensityMappingDecision |  |
| LightingSystem | Logic that receives ambient lighting commands (update, ramp, etc) and data (color, intensity, location, etc) and drives ambient lighting output. | - ProvideIllumination |  |
| linLEDPowerControl | Logic that arbitrates / manages light module in the network with respect to network sleep mode, electric power delayed accessory and welcome / farewell trigger. | - PerformLedPowerControl |  |
| Personalization | Personalization feature responsible to send user personalization profile number to ambient lighting. |  |  |
| RearHMILogic | Logic responsible to provide lighting color, intensity, drive mode and rejuvenate option from HMI to ambient lighting as well as update HMI with current user choice. | - ProcessUserFeedback |  |
| SDM | Logic responsible to provide current drive mode selected by user to ambient lighting. |  |  |
| SwSleepAwaking |  |  |  |
| User | Logical representation of user. | - Ideal Subfunction 2 - Ideal Subfunction 5 |  |
| VehicleStatus | Logic responsible to send vehicle state data to ambient lighting. |  |  |
| WFInterpreter | Logic responsible to provide ambient lighting with Welcome / Farewell behavior like by interpreting Welcome / Farewell command. | - PerformWelcomeFarewellFeatureProcessing |  |

Table 15: Logical Elements

### Logical Interfaces

| Interface | Direction | Description | Value Range |
| --- | --- | --- | --- |
| AjarColor_Cfg | pI_07 (DiagnosticsAndConfig) To pI_17 (AmbientLighting) | Name: AjarColor_Cfg Description: Ajar color level when vehicle door is ajar for ambient lighting. Min Value: 0 Max Value: 15 | AjarColor_Cfg as byte: |
|  | pI_17 (AmbientLighting) To p1 (AmbientLightOutput) | Name: AjarColor_Cfg Description: Ajar color level when vehicle door is ajar for ambient lighting. Min Value: 0 Max Value: 15 | AjarColor_Cfg as byte: |
| All_DoorLock_Status | p1 (AmbientLighting) To p2 (linLEDPowerControl) |  | All_DoorLock_Status as LockKind: - 0x0-LOCK - 0x1-NULL - 0x2-UNLOCK as LockKind: - LOCK - NULL - UNLOCK |
|  | p1 (VehicleStatus) To p1 (AmbientLighting) |  | All_DoorLock_Status as LockKind: - 0x0-LOCK - 0x1-NULL - 0x2-UNLOCK as LockKind: - LOCK - NULL - UNLOCK |
| ALMPowerMode_Cfg | pI_06 (DiagnosticsAndConfig) To pI_10 (AmbientLighting) | Name: ALMPowerMode_Cfg Description: The method 2 signal will decide one of two approaches, One approach is to keep ALM conditioned power dependent on headlamp status, other approach is to keep ALM conditioned power independent of headlamp status, while making it independent of headlamp status it is controlled on the basis of BCM | ALMPowerMode_Cfg as ALMPowerMode_CfgKind: - 0x0-BCM_IO_ACTIVE - 0x1-HEADLAMP_CONTROLLED as ALMPowerMode_CfgKind: - BCM_IO_ACTIVE - HEADLAMP_CONTROLLED |
|  | pI_10 (AmbientLighting) To p1 (linLEDPowerControl) | Name: ALMPowerMode_Cfg Description: The method 2 signal will decide one of two approaches, One approach is to keep ALM conditioned power dependent on headlamp status, other approach is to keep ALM conditioned power independent of headlamp status, while making it independent of headlamp status it is controlled on the basis of BCM | ALMPowerMode_Cfg as ALMPowerMode_CfgKind: - 0x0-BCM_IO_ACTIVE - 0x1-HEADLAMP_CONTROLLED as ALMPowerMode_CfgKind: - BCM_IO_ACTIVE - HEADLAMP_CONTROLLED |
| ALM_CommUp_Time_Cfg | pI_07 (DiagnosticsAndConfig) To pI_17 (AmbientLighting) | Name: ALM_CommUp_Time_Cfg Description: Configuration parameter specifying the time requried for ambient light node to wake up. | ALM_CommUp_Time_Cfg as byte: |
|  | pI_17 (AmbientLighting) To p1 (AmbientLightOutput) | Name: ALM_CommUp_Time_Cfg Description: Configuration parameter specifying the time requried for ambient light node to wake up. | ALM_CommUp_Time_Cfg as byte: |
| ALM_ConditionedPower | pI_02 (ALMCircuitOutput) To pI_01 (LightingSystem) |  | ALM_ConditionedPower as byte: |
| ALM_Conditioned_Pwr_Arb | pI_02 (linLEDPowerControl) To pI_01 (ALMCircuitOutput) | Name: ALM_Conditioned_Pwr_Arb Description: Ambient Light Moudule conditioned power supply. | ALM_Conditioned_Pwr_Arb as ALM_Conditioned_Pwr_ArbKind: - 0x0-INACTIVE - 0x1-ACTIVE as ALM_Conditioned_Pwr_ArbKind: - INACTIVE - ACTIVE |
| ALM_Location_ID | pI_02 (AmbientLightOutput) To pI_02 (LightingSystem) | Name: ALM_Location_ID Description: Indicates the ALM module NODE ADDRESS placed at various locations.Generally, lighting elements will be located in the cupholders, center console, footwells, doors, scuffplates. | ALM_Location_ID as ALM_Location_IDKind: - ALL - CUPHOLDERS_ALL - DF_DOOR - DF_DOOR_HANDLE - DF_DOOR_LIGHTBAR - DR_DOOR - DR_DOOR_HANDLE - DR_DOOR_LIGHTBAR - FOOTWELLS - IDLE - PF_DOOR - PF_DOOR_HANDLE - PF_DOOR_LIGHTBAR - PR_DOOR - PR_DOOR_HANDLE - PR_DOOR_LIGHTBAR - STORAGEBIN_LIGHTBAR |
| ALM_Power_Status | pI_03 (ALMCircuitOutput) To pI_07 (AmbientLightingOutput) | Name: ALM_Power_Status Description: This dataflow is used to convert the numeric output of ALM_Conditioned_Pwr_Ckt to discrete to send it over LIN | ALM_Power_Status as ALM_Power_StatusKind: - INACTIVE - ACTIVE as ALM_Power_StatusKind: - 0x0-INACTIVE - 0x1-ACTIVE |
|  | pI_04 (ALMCircuitOutput) To pI_07 (AmbientLightOutput) | Name: ALM_Power_Status Description: This dataflow is used to convert the numeric output of ALM_Conditioned_Pwr_Ckt to discrete to send it over LIN | ALM_Power_Status as ALM_Power_StatusKind: - INACTIVE - ACTIVE as ALM_Power_StatusKind: - 0x0-INACTIVE - 0x1-ACTIVE |
| AmbDrvMdeColor_Cfg | pI_01 (DiagnosticsAndConfig) To pI_01 (Core) | Description: This configuration parameter will define the color table lookup for the drive mode colors. The color defined in each entry is a HMI color index value in the range 1 to 7. The color index 0 will present a fault that will result in the contextual ambient to operate in Manual behavior. | AmbDrvMdeColor_Cfg as AmbientLighting_CfgKind: - AMB_ONLY - DISABLED - WF_AND_AMB - WF_ONLY - SINGLE_COL as AmbientLighting_CfgKind: - 0x0-AMB_ONLY - 0x1-DISABLED - 0x2-WF_AND_A MB - 0x3-WF_ONLY - 0x4-SINGLE_COLOR |
|  | pI_01 (DiagnosticsAndConfig) To pI_13 (AmbientLighting) | Description: This configuration parameter will define the color table lookup for the drive mode colors. The color defined in each entry is a HMI color index value in the range 1 to 7. The color index 0 will present a fault that will result in the contextual ambient to operate in Manual behavior. | AmbDrvMdeColor_Cfg as AmbientLighting_CfgKind: - AMB_ONLY - DISABLED - WF_AND_AMB - WF_ONLY - SINGLE_COL as AmbientLighting_CfgKind: - 0x0-AMB_ONLY - 0x1-DISABLED - 0x2-WF_AND_A MB - 0x3-WF_ONLY - 0x4-SINGLE_COLOR |
|  | pI_13 (AmbientLighting) To pI_01 (Core) | Description: This configuration parameter will define the color table lookup for the drive mode colors. The color defined in each entry is a HMI color index value in the range 1 to 7. The color index 0 will present a fault that will result in the contextual ambient to operate in Manual behavior. | AmbDrvMdeColor_Cfg as AmbientLighting_CfgKind: - AMB_ONLY - DISABLED - WF_AND_AMB - WF_ONLY - SINGLE_COL as AmbientLighting_CfgKind: - 0x0-AMB_ONLY - 0x1-DISABLED - 0x2-WF_AND_A MB - 0x3-WF_ONLY - 0x4-SINGLE_COLOR |
| AmbientColor_Cfg | pI_06 (InputProcessing) To pI_04 (AmbientLightOutput) | Name: AmbientColor_Cfg Description: Currently selected ambient color DomainElement : 0:15 DomainDescription : Color level for ambient lighting | AmbientColor_Cfg as byte: |
| AmbientDelay_Cfg | pI_03 (AmbientLighting) To pI_05 (AmbientLightingOutput) | Name: AmbientDelay_Cfg Description: Configuration Parameter which indicates the minimum time delay before entering to the ambient state from welcome state, to avoid the ambient light turning OFF due to momentary low voltage during crank when battery voltage is very low. | AmbientDelay_Cfg as DoorLock_Indication_CfgKind: - COMMON - INDIVIDUAL - NULL as DoorLock_Indication_CfgKind: - 0x0-COMMON - 0x1-INDIVIDUAL - 0x2-NULL |
|  | pI_03 (DiagnosticsAndConfig) To pI_03 (AmbientLighting) | Name: AmbientDelay_Cfg Description: Configuration Parameter which indicates the minimum time delay before entering to the ambient state from welcome state, to avoid the ambient light turning OFF due to momentary low voltage during crank when battery voltage is very low. | AmbientDelay_Cfg as DoorLock_Indication_CfgKind: - COMMON - INDIVIDUAL - NULL as DoorLock_Indication_CfgKind: - 0x0-COMMON - 0x1-INDIVIDUAL - 0x2-NULL |
| AmbientDim_Cfg | pI_06 (InputProcessing) To pI_04 (AmbientLightOutput) | Name: AmbientDim_Cfg Description: Currently selected ambient dimming level DomainElement : 0:15 DomainDescription : Ambient ligiting dimming level | AmbientDim_Cfg as byte: |
| AmbientLightingColor | User To FrontHMILogic |  |  |
|  | User To RearHMILogic |  |  |
| AmbientLightingIntensity | User To FrontHMILogic |  |  |
|  | User To RearHMILogic |  |  |
| AmbientLightingPersonalizationRequest | User To FrontHMILogic | The description of the signal in the Documentation field. |  |
| AmbientLightingSettings | FrontHMILogic To User |  |  |
|  | RearHMILogic To User |  |  |
| AmbientLighting_Cfg | p1 (DiagnosticsAndConfig) To pI_09 (AmbientLighting) | Name: AmbientLighting_Cfg Description: Configuration used to enable/disable the ambient and welcome & farewell behavior | AmbientLighting_Cfg as AmbientLighting_CfgKind: - AMB_ONLY - DISABLED - WF_AND_AMB - WF_ONLY - SINGLE_COL as AmbientLighting_CfgKind: - 0x0-AMB_ONLY - 0x1-DISABLED - 0x2-WF_AND_A MB - 0x3-WF_ONLY - 0x4-SINGLE_COLOR |
|  | pI_04 (AmbientLighting) To pI_02 (WFInterpreter) | Name: AmbientLighting_Cfg Description: Configuration used to enable/disable the ambient and welcome & farewell behavior | AmbientLighting_Cfg as AmbientLighting_CfgKind: - AMB_ONLY - DISABLED - WF_AND_AMB - WF_ONLY - SINGLE_COL as AmbientLighting_CfgKind: - 0x0-AMB_ONLY - 0x1-DISABLED - 0x2-WF_AND_A MB - 0x3-WF_ONLY - 0x4-SINGLE_COLOR |
|  | pI_04 (DiagnosticsAndConfig) To pI_04 (AmbientLighting) | Name: AmbientLighting_Cfg Description: Configuration used to enable/disable the ambient and welcome & farewell behavior | AmbientLighting_Cfg as AmbientLighting_CfgKind: - AMB_ONLY - DISABLED - WF_AND_AMB - WF_ONLY - SINGLE_COL as AmbientLighting_CfgKind: - 0x0-AMB_ONLY - 0x1-DISABLED - 0x2-WF_AND_A MB - 0x3-WF_ONLY - 0x4-SINGLE_COLOR |
|  | pI_09 (AmbientLighting) To pI_03 (AmbientProcessing) | Name: AmbientLighting_Cfg Description: Configuration used to enable/disable the ambient and welcome & farewell behavior | AmbientLighting_Cfg as AmbientLighting_CfgKind: - AMB_ONLY - DISABLED - WF_AND_AMB - WF_ONLY - SINGLE_COL as AmbientLighting_CfgKind: - 0x0-AMB_ONLY - 0x1-DISABLED - 0x2-WF_AND_A MB - 0x3-WF_ONLY - 0x4-SINGLE_COLOR |
| AmbientMsg_Rqst | pI_01 (AmbientLightingOutput) To pI_01 (AmbientLightOutput) | Name: AmbientMsg_Rqst Description: This array contains the ambient message request for LIN module. The index 'n' has following domain values: ALL FOOTWELLS DF_DOOR PF_DOOR DR_DOOR PR_DOOR DF_DOOR_HANDLE PF_DOOR_HANDLE DR_DOOR_HANDLE PR_DOOR_HANDLE | LED as LightLocationKind: - AJAR - AMBIENT - DF_DOOR - NULL - OFF - RAMP_OFF - WELCOME - WELCOME_LE |
| Ambient_Status | pI_01 (AmbientProcessing) To pI_02 (AmbientLightingOutput) | Name: Ambient_Status Description: Dataflow indicating the output of the ambient processing | Ambient_Status as Ambient_StatusKind: - 0x0-OFF - 0x1-AMBIENT as Ambient_StatusKind: - OFF - AMBIENT |
|  | pI_04 (AmbientProcessing) To pI_01 (linLEDPowerControl) | Name: Ambient_Status Description: Dataflow indicating the output of the ambient processing | Ambient_Status as Ambient_StatusKind: - 0x0-OFF - 0x1-AMBIENT as Ambient_StatusKind: - OFF - AMBIENT |
| Approach_LightShow | pI_03 (VehicleStatus) To pI_05 (AmbientLighting) | Name: Approach_LightShow Description: Indicates when to start approach light show. Note: It will take care of the maximum time limit of 25 seconds. | Approach_LightShow as Approach_LightShowKind: - SHOW_OFF - SHOW_ON as Approach_LightShowKind: - 0x0-SHOW_OFF - 0x1-SHOW_ON |
|  | pI_05 (AmbientLighting) To pI_03 (WFInterpreter) | Name: Approach_LightShow Description: Indicates when to start approach light show. Note: It will take care of the maximum time limit of 25 seconds. | Approach_LightShow as Approach_LightShowKind: - SHOW_OFF - SHOW_ON as Approach_LightShowKind: - 0x0-SHOW_OFF - 0x1-SHOW_ON |
| Color | pI_02 (AmbientLightOutput) To pI_02 (LightingSystem) | Name: Color Description: Indicates Color need to set for ambient lighting. OFF domain value for Color is 0. | Update_Color as byte: |
| ContextAL_Cfg | pI_01 (DiagnosticsAndConfig) To pI_01 (Core) | Name: ContextAL_Cfg Description: This configuration parameter will define if the Contextual Ambient Lighting is enabled or disabled. | ContextAL_Cfg as ContextAL_CfgKind: - 0x0-DISABLE - 0x1-ENABLE as ContextAL_CfgKind: - DISABLE - ENABLE |
|  | pI_01 (DiagnosticsAndConfig) To pI_13 (AmbientLighting) | Name: ContextAL_Cfg Description: This configuration parameter will define if the Contextual Ambient Lighting is enabled or disabled. | ContextAL_Cfg as ContextAL_CfgKind: - 0x0-DISABLE - 0x1-ENABLE as ContextAL_CfgKind: - DISABLE - ENABLE |
|  | pI_13 (AmbientLighting) To pI_01 (Core) | Name: ContextAL_Cfg Description: This configuration parameter will define if the Contextual Ambient Lighting is enabled or disabled. | ContextAL_Cfg as ContextAL_CfgKind: - 0x0-DISABLE - 0x1-ENABLE as ContextAL_CfgKind: - DISABLE - ENABLE |
| ContextualAmbientLightingRequest | User To FrontHMILogic |  |  |
| CourtesyDelay_Rqst | pI_03 (VehicleStatus) To pI_05 (AmbientLighting) | Name: CourtesyDelay_Rqst Description: Request to illuminate the interior courtesy lights. | CourtesyDelay_Rqst as CourtesyDelay_RqstKind: - OFF - ON as CourtesyDelay_RqstKind: - 0x0-OFF - 0x1-ON |
|  | pI_05 (AmbientLighting) To pI_03 (WFInterpreter) | Name: CourtesyDelay_Rqst Description: Request to illuminate the interior courtesy lights. | CourtesyDelay_Rqst as CourtesyDelay_RqstKind: - OFF - ON as CourtesyDelay_RqstKind: - 0x0-OFF - 0x1-ON |
| CourtesyDemand_BSave_Rqst | p1 (AmbientLighting) To p2 (linLEDPowerControl) | Name: CourtesyDemand_BSave_Rqst Description: Request from Battery Saver to turn off the courtesy lights and the demand lights. | CourtesyDemand_BSave_Rqst as CourtesyDemand_BSave_RqstKind: - NO_EFFECT - OFF as CourtesyDemand_BSave_RqstKind: - 0x0-NO_EFFECT - 0x1-OFF |
|  | p1 (VehicleStatus) To p1 (AmbientLighting) | Name: CourtesyDemand_BSave_Rqst Description: Request from Battery Saver to turn off the courtesy lights and the demand lights. | CourtesyDemand_BSave_Rqst as CourtesyDemand_BSave_RqstKind: - NO_EFFECT - OFF as CourtesyDemand_BSave_RqstKind: - 0x0-NO_EFFECT - 0x1-OFF |
| Delayed_Accessory_Cmd | pI_04 (VehicleStatus) To pI_07 (AmbientLighting) | Name: Delayed_Accessory_Cmd Description: Input to load shedding - delayed accesory status | Delayed_Accessory_Cmd as Delayed_Accessory_CmdKind: - OFF - ON as Delayed_Accessory_CmdKind: - 0x0-OFF - 0x1-ON |
|  | pI_07 (AmbientLighting) To pI_02 (AmbientProcessing) | Name: Delayed_Accessory_Cmd Description: Input to load shedding - delayed accesory status | Delayed_Accessory_Cmd as Delayed_Accessory_CmdKind: - OFF - ON as Delayed_Accessory_CmdKind: - 0x0-OFF - 0x1-ON |
| DiagSession | pI_06 (DiagnosticsAndConfig) To pI_10 (AmbientLighting) | Name: DiagSession Description: Indicates the active session of Diagnostics | DiagSession as DiagSessionKind: - 0x0-DEFAULT - 0x1-EXTEND - 0x2-PROGRAM - 0x3-NKNOWN as DiagSessionKind: - DEFAULT - EXTEND - PROGRAM - UNKNOWN |
|  | pI_10 (AmbientLighting) To p1 (linLEDPowerControl) | Name: DiagSession Description: Indicates the active session of Diagnostics | DiagSession as DiagSessionKind: - 0x0-DEFAULT - 0x1-EXTEND - 0x2-PROGRAM - 0x3-NKNOWN as DiagSessionKind: - DEFAULT - EXTEND - PROGRAM - UNKNOWN |
| DispOpr_CS | pI_03 (FrontHMILogic) To pI_03 (Core) | Name: DispOpr_CS Description: Operation request command for personalization features from Center Stack. | DispOpr_CS as DispOpr_CSKind: - 0x0-COPY - 0x1-NULL - 0x2-RESTORE - 0x3-SET - 0x4-UPLOAD as DispOpr_CSKind: - COPY - NULL - RESTORE - SET - UPLOAD |
| DoorAjar | pI_01 (AmbientLighting) To pI_03 (AmbientLightingOutput) |  | state as AjarKind: - 0x0-CLOSED - 0x1-AJAR as AjarKind: - CLOSED - AJAR |
|  | pI_01 (VehicleStatus) To pI_01 (AmbientLighting) |  | state as AjarKind: - 0x0-CLOSED - 0x1-AJAR as AjarKind: - CLOSED - AJAR |
| DoorCourtesyLight_Rqst | pI_03 (VehicleStatus) To pI_05 (AmbientLighting) | Name: DoorCourtesyLight_Rqst Description: Request to illuminate the interior courtesy lights. | DoorCourtesyLight_Rqst as DoorCourtesyLight_RqstKind: - OFF - ON as DoorCourtesyLight_RqstKind: - 0x0-OFF - 0x1-ON |
|  | pI_05 (AmbientLighting) To pI_03 (WFInterpreter) | Name: DoorCourtesyLight_Rqst Description: Request to illuminate the interior courtesy lights. | DoorCourtesyLight_Rqst as DoorCourtesyLight_RqstKind: - OFF - ON as DoorCourtesyLight_RqstKind: - 0x0-OFF - 0x1-ON |
| DoorLocked | pI_02 (AmbientLighting) To pI_04 (AmbientLightingOutput) |  | state as LockKind: - 0x0-LOCK - 0x1-NULL - 0x2-UNLOCK as LockKind: - LOCK - NULL - UNLOCK |
|  | pI_02 (VehicleStatus) To pI_02 (AmbientLighting) |  | state as LockKind: - 0x0-LOCK - 0x1-NULL - 0x2-UNLOCK as LockKind: - LOCK - NULL - UNLOCK |
| DoorLock_Indication_Cfg | pI_03 (AmbientLighting) To pI_05 (AmbientLightingOutput) | Name: DoorLock_Indication_Cfg Description: Leave at initial value of NULL. This is for Ambient Lighting to indicate lock status via illumination of the interior door pull handle. This feature is no longer supported. It is replaced by the trim panel lock indicator LED feature (Section 2.2.18). | DoorLock_Indication_Cfg |
|  | pI_03 (DiagnosticsAndConfig) To pI_03 (AmbientLighting) | Name: DoorLock_Indication_Cfg Description: Leave at initial value of NULL. This is for Ambient Lighting to indicate lock status via illumination of the interior door pull handle. This feature is no longer supported. It is replaced by the trim panel lock indicator LED feature (Section 2.2.18). | DoorLock_Indication_Cfg |
| EM_Exists_Cfg | pI_01 (DiagnosticsAndConfig) To pI_01 (Core) | Name: EM_Exists_Cfg Description: The method 2 configuration parameter to decide whether Enhanced Memory configuration exists for Ambient Lighting, If this is NOT_PRESENT then no personalization of ambient lighting will be provided. | EM_Exists_Cfg as EM_Exists_CfgKind: - NOT_PRESENT - PRESENT as EM_Exists_CfgKind: - 0x0-NOT_PRESENT - 0x1-PRESENT |
| EntryColor_Cfg | pI_02 (DiagnosticsAndConfig) To pI_01 (InputProcessing) | Name: EntryColor_Cfg Description: Entry color level for ambient lighting | EntryColor_Cfg as byte: |
|  | pI_02 (DiagnosticsAndConfig) To pI_12 (AmbientLighting) | Name: EntryColor_Cfg Description: Entry color level for ambient lighting | EntryColor_Cfg as byte: |
|  | pI_07 (DiagnosticsAndConfig) To pI_17 (AmbientLighting) | Name: EntryColor_Cfg Description: Entry color level for ambient lighting | EntryColor_Cfg as byte: |
|  | pI_12 (AmbientLighting) To pI_01 (InputProcessing) | Name: EntryColor_Cfg Description: Entry color level for ambient lighting | EntryColor_Cfg as byte: |
|  | pI_17 (AmbientLighting) To p1 (AmbientLightOutput) | Name: EntryColor_Cfg Description: Entry color level for ambient lighting | EntryColor_Cfg as byte: |
| EntryInt_Cfg | pI_07 (DiagnosticsAndConfig) To pI_17 (AmbientLighting) | Name: EntryInt_Cfg Description: Entry level intensity for ambient lighting | EntryInt_Cfg as byte: |
|  | pI_17 (AmbientLighting) To p1 (AmbientLightOutput) | Name: EntryInt_Cfg Description: Entry level intensity for ambient lighting | EntryInt_Cfg as byte: |
| HMI_Comm_Var_Cfg | pI_01 (DiagnosticsAndConfig) To pI_01 (Core) | This configuration parameter will select the Communication variant used in HMI to BCM Ambient Lighting Interface. Variant1 is the original carryover protocol. Variant2 was added to support enhanced memory and supported for contextual ambient lighting. This configuration can also be used to support standard ambient lighting. The module providing the customer interface (currently SYNC) and the BCM must use the same variant settings for proper operation of any of Ambient Lighting features. | HMI_Comm_Var_Cfg as HMI_Comm_Var_CfgKind: - 0x1-VARIANT1 - 0x2-VARIANT2 as HMI_Comm_Var_CfgKind: - VARIANT1 - VARIANT2 |
|  | pI_01 (DiagnosticsAndConfig) To pI_13 (AmbientLighting) | This configuration parameter will select the Communication variant used in HMI to BCM Ambient Lighting Interface. Variant1 is the original carryover protocol. Variant2 was added to support enhanced memory and supported for contextual ambient lighting. This configuration can also be used to support standard ambient lighting. The module providing the customer interface (currently SYNC) and the BCM must use the same variant settings for proper operation of any of Ambient Lighting features. | HMI_Comm_Var_Cfg as HMI_Comm_Var_CfgKind: - 0x1-VARIANT1 - 0x2-VARIANT2 as HMI_Comm_Var_CfgKind: - VARIANT1 - VARIANT2 |
|  | pI_13 (AmbientLighting) To pI_01 (Core) | This configuration parameter will select the Communication variant used in HMI to BCM Ambient Lighting Interface. Variant1 is the original carryover protocol. Variant2 was added to support enhanced memory and supported for contextual ambient lighting. This configuration can also be used to support standard ambient lighting. The module providing the customer interface (currently SYNC) and the BCM must use the same variant settings for proper operation of any of Ambient Lighting features. | HMI_Comm_Var_Cfg as HMI_Comm_Var_CfgKind: - 0x1-VARIANT1 - 0x2-VARIANT2 as HMI_Comm_Var_CfgKind: - VARIANT1 - VARIANT2 |
| IlluminatedEntry_Rqst | pI_03 (VehicleStatus) To pI_05 (AmbientLighting) | Name: IlluminatedEntry_Rqst Description: Request to illuminate the interior courtesy lights. | IlluminatedEntry_Rqst as IlluminatedEntry_RqstKind: - OFF - ON as IlluminatedEntry_RqstKind: - 0x0-OFF - 0x1-ON |
|  | pI_05 (AmbientLighting) To pI_03 (WFInterpreter) | Name: IlluminatedEntry_Rqst Description: Request to illuminate the interior courtesy lights. | IlluminatedEntry_Rqst as IlluminatedEntry_RqstKind: - OFF - ON as IlluminatedEntry_RqstKind: - 0x0-OFF - 0x1-ON |
| IlluminatedExit_Rqst | pI_03 (VehicleStatus) To pI_05 (AmbientLighting) | Name: IlluminatedExit_Rqst Description: Request to illuminate the interior courtesy lights. | IlluminatedExit_Rqst as IlluminatedExit_RqstKind: - OFF - ON as IlluminatedExit_RqstKind: - 0x0-OFF - 0x1-ON |
|  | pI_05 (AmbientLighting) To pI_03 (WFInterpreter) | Name: IlluminatedExit_Rqst Description: Request to illuminate the interior courtesy lights. | IlluminatedExit_Rqst as IlluminatedExit_RqstKind: - OFF - ON as IlluminatedExit_RqstKind: - 0x0-OFF - 0x1-ON |
| Intensity | pI_02 (AmbientLightOutput) To pI_02 (LightingSystem) | Name: Intensity Description: Indicates the intesity required to set for ambient lighting. | Intensity as byte: |
| LghtAmbColr_No_Rq | p1 (RearHMILogic) To p1 (Core) | Ambient Lighting Color Selection Request. | Encoding as byte: |
| LghtAmbDrvMde_B_Stat | pI_02 (Core) To pI_02 (InputProcessing) | Name: LghtAmbDrvMde_B_Stat Description: CAN signal for Contextual Ambient Lighting MANUAL and AUTOMATIC BCM f eedback. | LghtAmbDrvMde_B_Stat as LghtAmbDrvMde_B_StatKind: - MANUAL - AUTOMATIC as LghtAmbDrvMde_B_StatKind: - 0x0-MANUAL - 0x1-AUTOMATIC |
|  | pI_06 (Core) To pI_05 (FrontHMILogic) | Name: LghtAmbDrvMde_B_Stat Description: CAN signal for Contextual Ambient Lighting MANUAL and AUTOMATIC BCM f eedback. | LghtAmbDrvMde_B_Stat as LghtAmbDrvMde_B_StatKind: - MANUAL - AUTOMATIC as LghtAmbDrvMde_B_StatKind: - 0x0-MANUAL - 0x1-AUTOMATIC |
| LghtAmbDrvMde_D_Rq | pI_03 (FrontHMILogic) To pI_03 (Core) | Name: LghtAmbDrvMde_D_Rq Description: CAN signal for setting Contextual Ambient Lighting MANUAL and AUTOMATIC HMI request. | LghtAmbDrvMde_D_Rq as LghtAmbDrvMde_D_RqKind: - NULL - MANUAL - AUTOMATIC as LghtAmbDrvMde_D_RqKind: - 0x0-NULL - 0x1-MANUAL - 0x2-AUTOMATIC |
| LghtAmbIntns_No_Rq | p1 (RearHMILogic) To p1 (Core) | User request to set ambient intensity level. | Encoding as byte: |
| LghtAmbMode_Stat | pI_02 (Core) To pI_02 (InputProcessing) | Name: LghtAmbDrvMde_B_Stat Description: CAN signal for Contextual Ambient Lighting MANUAL and AUTOMATIC BCM f eedback. | LghtAmbMode_Stat as LghtAmbMode_StatKind: - MANUAL - AUTOMATIC |
| LghtAmbRqSrc_B_Stat | pI_03 (FrontHMILogic) To pI_03 (Core) |  |  |
| LghtAmbRqSrc_Stat | pI_03 (FrontHMILogic) To pI_03 (Core) |  |  |
| Light | LightingSystem To User |  |  |
| LightAmbColor_No | pI_02 (Core) To pI_02 (InputProcessing) | Name: LightAmbColor_No Description: This number will be derived from CAN inputs from ambient color number depending on whether Enhanced memory exists. | LightAmbColor_No as byte: |
| LightAmbColor_No_Actl | pI_03 (InputProcessing) To pI_01 (RearHMILogic) | Name: LightAmbColor_No_Actl Description: CAN signal for ambient color level status DomainElement : 0:15 DomainDescription : ambient color level | Encoding as byte: |
|  | pI_05 (InputProcessing) To pI_04 (FrontHMILogic) | Name: LightAmbColor_No_Actl Description: CAN signal for ambient color level status DomainElement : 0:15 DomainDescription : ambient color level | Encoding as byte: |
| LightAmbColor_No_Rq | pI_03 (FrontHMILogic) To pI_03 (Core) | Name: LightAmbColor_No_Rq Description: CAN signal for setting ambient color level DomainElement : 0:15 DomainDescription : ambient color level | LightAmbColor_No_Rq as byte: |
| LightAmbColor_Temp | pI_06 (InputProcessing) To pI_04 (AmbientLightOutput) | Name: LightAmbColor_Temp Description: Ambient color for Lincoln and Ford. 0-ICE_BLUE; 1- ORANGE; 2-SOFT_BLUE; 3-RED; 4-GREEN; 5-BLUE; 6-PURPLE; 7-LICOLN_WHITE; 8-AMBER; 9-TEAL; 10-BURNT_ORANGE; 11-LINCOLN_BLUE; 12-LINCOLN_GREEN; 13-LILAC | LightAmbColor_Temp as byte: |
| LightAmbIntsty_No | pI_02 (Core) To pI_02 (InputProcessing) | Name: LightAmbIntsty_No Description: This number will be derived from CAN inputs from ambient intensity number depending on whether Enhanced memory exists | LightAmbIntsty_No as byte: |
| LightAmbIntsty_No_Actl | pI_03 (InputProcessing) To pI_01 (RearHMILogic) | Name: LightAmbIntsty_No_Actl Description: CAN signal for ambient color level status DomainElement : 0:100 DomainDescription : ambient dimming level | Encoding as byte: |
|  | pI_05 (InputProcessing) To pI_04 (FrontHMILogic) | Name: LightAmbIntsty_No_Actl Description: CAN signal for ambient color level status DomainElement : 0:100 DomainDescription : ambient dimming level | Encoding as byte: |
| LightAmbIntsty_No_Rq | pI_03 (FrontHMILogic) To pI_03 (Core) | Name: LightAmbIntsty_No_Rq Description: CAN signal for setting ambient dim level DomainElement : 0:255 DomainDescription : ambient dimming level | LightAmbIntsty_No_Rq as byte: |
| Lincoln_Experience_Cfg | pI_03 (AmbientLighting) To pI_05 (AmbientLightingOutput) | Name: Lincoln_Experience_Cfg Description: The configuration parameter is to differentiate the Welcome/Farewell Lighting (Interior and Exterior) behaviour between Lincoln Experience vehicles and Non Lincoln vehicles. When Lincoln_Experience_Cfg is set to LINCOLN_EXPERIENCE1 then method 2 configuration parameters LR_Stop_Pos_Lamp_Ckt_Usage_Cfg and RR_Stop_Pos_Lamp_Ckt_Usage_Cfg shall be always set to STOP_TURN, similarly method 3 configuration parameter Freq_Cfg[Freq_Sel_Cfg][Park_Lamps_Body_Ckt] shall be always set to DC. | Lincoln_Experience_Cfg as Lincoln_Experience_CfgKind: - 0x0-LINCOLN_EXPERIENCE1 - 0x1-NULL as Lincoln_Experience_CfgKind: - LINCOLN_EXPERIENCE1 - NULL |
|  | pI_03 (DiagnosticsAndConfig) To pI_03 (AmbientLighting) | Name: Lincoln_Experience_Cfg Description: The configuration parameter is to differentiate the Welcome/Farewell Lighting (Interior and Exterior) behaviour between Lincoln Experience vehicles and Non Lincoln vehicles. When Lincoln_Experience_Cfg is set to LINCOLN_EXPERIENCE1 then method 2 configuration parameters LR_Stop_Pos_Lamp_Ckt_Usage_Cfg and RR_Stop_Pos_Lamp_Ckt_Usage_Cfg shall be always set to STOP_TURN, similarly method 3 configuration parameter Freq_Cfg[Freq_Sel_Cfg][Park_Lamps_Body_Ckt] shall be always set to DC. | Lincoln_Experience_Cfg as Lincoln_Experience_CfgKind: - 0x0-LINCOLN_EXPERIENCE1 - 0x1-NULL as Lincoln_Experience_CfgKind: - LINCOLN_EXPERIENCE1 - NULL |
|  | pI_04 (AmbientLighting) To pI_02 (WFInterpreter) | Name: Lincoln_Experience_Cfg Description: The configuration parameter is to differentiate the Welcome/Farewell Lighting (Interior and Exterior) behaviour between Lincoln Experience vehicles and Non Lincoln vehicles. When Lincoln_Experience_Cfg is set to LINCOLN_EXPERIENCE1 then method 2 configuration parameters LR_Stop_Pos_Lamp_Ckt_Usage_Cfg and RR_Stop_Pos_Lamp_Ckt_Usage_Cfg shall be always set to STOP_TURN, similarly method 3 configuration parameter Freq_Cfg[Freq_Sel_Cfg][Park_Lamps_Body_Ckt] shall be always set to DC. | Lincoln_Experience_Cfg as Lincoln_Experience_CfgKind: - 0x0-LINCOLN_EXPERIENCE1 - 0x1-NULL as Lincoln_Experience_CfgKind: - LINCOLN_EXPERIENCE1 - NULL |
|  | pI_04 (DiagnosticsAndConfig) To pI_04 (AmbientLighting) | Name: Lincoln_Experience_Cfg Description: The configuration parameter is to differentiate the Welcome/Farewell Lighting (Interior and Exterior) behaviour between Lincoln Experience vehicles and Non Lincoln vehicles. When Lincoln_Experience_Cfg is set to LINCOLN_EXPERIENCE1 then method 2 configuration parameters LR_Stop_Pos_Lamp_Ckt_Usage_Cfg and RR_Stop_Pos_Lamp_Ckt_Usage_Cfg shall be always set to STOP_TURN, similarly method 3 configuration parameter Freq_Cfg[Freq_Sel_Cfg][Park_Lamps_Body_Ckt] shall be always set to DC. | Lincoln_Experience_Cfg as Lincoln_Experience_CfgKind: - 0x0-LINCOLN_EXPERIENCE1 - 0x1-NULL as Lincoln_Experience_CfgKind: - LINCOLN_EXPERIENCE1 - NULL |
| Local_OpMode | pI_01 (SwSleepAwaking) To pI_03 (linLEDPowerControl) | Name: Local_OpMode Description: initial value changed Awake -> Sleep(Conti.m) This Dataflow indicates the current state of the ECU. | Local_OpMode as Local_OpModeKind: - 0x0-AWAKE - 0x1-CAN_SLEEP - 0x2-PERIPS_ACTIVE - 0x3-SLEEP as Local_OpModeKind: - 0x0-AWAKE - 0x1-CAN_SLEEP - 0x2-LOW_MHZ - 0x3-PERIPS_ACTIVE - 0x4-SLEEP |
|  | pI_01 (SwSleepAwaking) To pI_11 (AmbientLighting) | Name: Local_OpMode Description: initial value changed Awake -> Sleep(Conti.m) This Dataflow indicates the current state of the ECU. | Local_OpMode as Local_OpModeKind: - 0x0-AWAKE - 0x1-CAN_SLEEP - 0x2-PERIPS_ACTIVE - 0x3-SLEEP as Local_OpModeKind: - 0x0-AWAKE - 0x1-CAN_SLEEP - 0x2-LOW_MHZ - 0x3-PERIPS_ACTIVE - 0x4-SLEEP |
|  | pI_11 (AmbientLighting) To pI_03 (linLEDPowerControl) | Name: Local_OpMode Description: initial value changed Awake -> Sleep(Conti.m) This Dataflow indicates the current state of the ECU. | Local_OpMode as Local_OpModeKind: - 0x0-AWAKE - 0x1-CAN_SLEEP - 0x2-PERIPS_ACTIVE - 0x3-SLEEP as Local_OpModeKind: - 0x0-AWAKE - 0x1-CAN_SLEEP - 0x2-LOW_MHZ - 0x3-PERIPS_ACTIVE - 0x4-SLEEP |
| Parklamps_Command | pI_04 (VehicleStatus) To pI_07 (AmbientLighting) | Name: Parklamps_Command Description: Command to control the position/parklamps. | Parklamps_Command as Parklamps_CommandKind: - 0x0-OFF - 0x1-ON as Parklamps_CommandKind: - OFF - ON |
|  | pI_07 (AmbientLighting) To pI_02 (AmbientProcessing) | Name: Parklamps_Command Description: Command to control the position/parklamps. | Parklamps_Command as Parklamps_CommandKind: - 0x0-OFF - 0x1-ON as Parklamps_CommandKind: - OFF - ON |
| PersIndex_CS | pI_03 (FrontHMILogic) To pI_03 (Core) | Name: PersIndex_CS Description: Personality index to use with operation from Center Stack. | PersIndex_CS as PersIndex_CSKind: - PERS_1 - PERS_2 - PERS_3 - PERS_4 - VEHICLE as PersIndex_CSKind: - 0x0-PERS_1 - 0x1-PERS_2 - 0x2-PERS_3 - 0x3-PERS_4 - 0x4-VEHICLE |
| PersNum | pI_01 (Personalization) To pI_14 (AmbientLighting) | Name: PersNum Description: Indicates which personality is currently selected. | PersNum as PersNumKind: - 0x0-PERS_1 - 0x1-PERS_2 - 0x2-PERS_3 - 0x3-PERS_4 - 0x4-VEHICLE as PersNumKind: - PERS_1 - PERS_2 - PERS_3 - PERS_4 - VEHICLE |
|  | pI_14 (AmbientLighting) To p2 (Core) | Name: PersNum Description: Indicates which personality is currently selected. | PersNum as PersNumKind: - 0x0-PERS_1 - 0x1-PERS_2 - 0x2-PERS_3 - 0x3-PERS_4 - 0x4-VEHICLE as PersNumKind: - PERS_1 - PERS_2 - PERS_3 - PERS_4 - VEHICLE |
| PersonalizationProfileNumber | User To FrontHMILogic |  |  |
| Ramp | pI_02 (AmbientLightOutput) To pI_02 (LightingSystem) | Name: Ramp Description: Indicates ramping is required/not required for ambient lighting. | Ramp as RampKind: - 0x0-NO - 0x1-YES as RampKind: - NO - YES |
| RejuvenateRequest | User To FrontHMILogic |  |  |
| Run_Active_Count_Max_Cfg | pI_07 (DiagnosticsAndConfig) To pI_17 (AmbientLighting) | Name: Run_Active_Count_Max_Cfg Description: The dataflow decides the max range of a count which is used in Static Sequential by Light Engines. | Run_Active_Count_Max_Cfg as byte: |
|  | pI_17 (AmbientLighting) To p1 (AmbientLightOutput) | Name: Run_Active_Count_Max_Cfg Description: The dataflow decides the max range of a count which is used in Static Sequential by Light Engines. | Run_Active_Count_Max_Cfg as byte: |
| Run_Active_Count_Min_Cfg | pI_07 (DiagnosticsAndConfig) To pI_17 (AmbientLighting) | Description: The dataflow decides the starting range of a count which is used for Static Sequential in Lighting Engines | Run_Active_Count_Min_Cfg as byte: |
|  | pI_17 (AmbientLighting) To p1 (AmbientLightOutput) | Description: The dataflow decides the starting range of a count which is used for Static Sequential in Lighting Engines | Run_Active_Count_Min_Cfg as byte: |
| SelDrvMdeHmi03_Rq | pI_01 (SDM) To pI_15 (AmbientLighting) | Name: SelDrvMdeHmi03_Rq Description: CAN signal for the selected drive mode. | currDriveMode as byte: |
|  | pI_15 (AmbientLighting) To pI_04 (Core) | Name: SelDrvMdeHmi03_Rq Description: CAN signal for the selected drive mode. | currDriveMode as byte: |
| Stat_Seq_Time_Cfg | pI_07 (DiagnosticsAndConfig) To pI_17 (AmbientLighting) | Name: Stat_Seq_Time_Cfg Description: This method 2 data decides the increment frequency of counter,The counter is transmitted over LIN | Stat_Seq_Time_Cfg as byte: |
|  | pI_17 (AmbientLighting) To p1 (AmbientLightOutput) | Name: Stat_Seq_Time_Cfg Description: This method 2 data decides the increment frequency of counter,The counter is transmitted over LIN | Stat_Seq_Time_Cfg as byte: |
| Stat_Seq_Wait_Time_Cfg | pI_07 (DiagnosticsAndConfig) To pI_17 (AmbientLighting) | Name: Stat_Seq_Wait_Time_Cfg Description: The dataflow is used to decide the rate at which the value of signal Run_Active_Count is written in protected RAM once the value of is stabilized to either min or max | Stat_Seq_Wait_Time_Cfg as byte: |
|  | pI_17 (AmbientLighting) To p1 (AmbientLightOutput) | Name: Stat_Seq_Wait_Time_Cfg Description: The dataflow is used to decide the rate at which the value of signal Run_Active_Count is written in protected RAM once the value of is stabilized to either min or max | Stat_Seq_Wait_Time_Cfg as byte: |
| Update_Color | pI_02 (AmbientLightOutput) To pI_02 (LightingSystem) | Name: Update_Color Description: Indicates whether the color set for ambient lighting needs update or not. | Update_Color as Update_ColorKind: - NO - YES as Update_ColorKind: - 0x0-NO - 0x0-YES |
| Update_Intensity | pI_02 (AmbientLightOutput) To pI_02 (LightingSystem) | Name: Update_Intensity Description: Indicates the intesity for ambient lighting needs to be updated or not. | Update_Intensity as Update_IntensityKind: - 0x0-NO - 0x1YES as Update_IntensityKind: - NO - YES |
| WelcomeFarewell_Status | pI_01 (WFInterpreter) To pI_06 (AmbientLightingOutput) | Name: WelcomeFarewell_Status Description: Indicates welcome or farewell status for ambient lighting. | WelcomeFarewell_Status as WelcomeFarewell_StatusKind: - OFF - WELCOME1 - WELCOME1_EXIT - WELCOME2 - WELCOME3 - WELCOME_LE as WelcomeFarewell_StatusKind: - 0x0-OFF - 0x1-WELCOME1 - 0x2-WELCOME1_EXIT - 0x3-WELCOME2 - 0x4-WELCOME3 as WelcomeFarewell_StatusKind: - OFF - WELCOME1 - WELCOME1_EXIT - WELCOME2 - WELCOME3 |
|  | pI_04 (WFInterpreter) To pI_05 (AmbientLightOutput) | Name: WelcomeFarewell_Status Description: Indicates welcome or farewell status for ambient lighting. | WelcomeFarewell_Status as WelcomeFarewell_StatusKind: - OFF - WELCOME1 - WELCOME1_EXIT - WELCOME2 - WELCOME3 - WELCOME_LE as WelcomeFarewell_StatusKind: - 0x0-OFF - 0x1-WELCOME1 - 0x2-WELCOME1_EXIT - 0x3-WELCOME2 - 0x4-WELCOME3 as WelcomeFarewell_StatusKind: - OFF - WELCOME1 - WELCOME1_EXIT - WELCOME2 - WELCOME3 |

Table 16: Feature Interactions

# Traceability Matrix

# Open Concerns

| ID | Concern Description | e-Tracker / Reference | Responsible | Status | Solution |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |

Table 17: Open Concerns (Not supported by MagicDraw report generation)

# Revision History

| Rev. (revision) | Date | Description | Approved by | Responsible |
| --- | --- | --- | --- | --- |
| FD1 | 2022-06-27 | V 1.0 June 22, 2022 Auto generated from Magicdraw model - to be submitted for review. |  | rcruz92 |
| FD2 | 2022-08-16 | V 2.0 August 16h, 2022 Auto generated from Magicdraw model - ready to be archived in VSEM. |  | rcruz92 |

Table 18: Revision History

## Template Revisions

| Version | Rev. | Date | Description | Responsible |
| --- | --- | --- | --- | --- |
| 0 | 6 | 2015-05-26 | - Chapter “Feature Overview” and made a 2nd level heading. - Chapter “Feature Modeling” divided into 3 subchapter (“Scenarios”, “Use Cases”, “State Machines”) for different modeling methods | Jbaden1 |
| 0 | 7 | 2015-05-27 | - Table of Content updated - Template Revision History chapter added | Jbaden1 |
| 0 | 8 | 2015-07-02 | - Section “Unsettled Issues” added | Alevin7 |
| 0 | 9 | 2015-08-04 | - Section “Feature Variants” added - Section “Feature Boundary Diagram” renamed to “Feature Context Diagram” - Document Properties adapted to match needs of VBA macros | Jbaden1, Awegman1 |
| 1 | 0 | 2015-09-11 | - Section “Feature Variants” reworked - Feature Goals removed. Only “Safety Goals“ chapter remains. - Heading 2 formatting issues corrected. - Requirements / Use Cases Listing removed from traceability chapter. - Formatting of attribute table in Notation chapter corrected - Open Topics / Known Issues chapter moved to the end | Jbaden1 |
| 1 | 1 | 2015-11-16 | - Table-Styles removed (for smooth VSEM import) - Some clean-up of sections “Purpose” and “Audience” | Awegman1, jbaden1 |
| 1 | 2 | 2016-02-26 | - Minor corrections based on lessons learned from CC and PCL pilot (e.g. section market/regions) and discussion with Functional Safety Team (purpose of feature) - Footer corrected - Boundary diagram interface chapter renamed to influences. | Jbaden1 |
| 1 | 3 | 2016-02-26 | - Minor corrections after review with Whitney Keith from Functional Safety team | Jbaden1 |
| 1 | 4 | 2016-03-10 | - Some cleanup of meta-data in Word Properties | Jbaden1 |
| 1 | 5 | 2016-03-10 | - Footer formatting corrected (Issue 19) - Results from review with Functional Safety Team incorporated (Issue 20). | jbaden1 |
| 1 | 6 | 2016-04-18 | - Scenario Template added | Jbaden1 |
| 1 | 7 | 2016-04-18 | - Chapter “Operation Modes and States” moved before “Use Case” section. | Jbaden1 |
| 1 | 8 | 2016-04-18 | - Broken Wiki links repaired. | Jbaden1 |
| 2 | 0 | 2016-05-19 | - Adapted to Specification_Macros.dotm V2.0 - Requirements Templates chapter (ch. 1.7.1) no longer has an attribute table, but refers directly to the Wiki.. | Jbaden1 |
| 2 | 1 | 2016-06-10 | - Table for Context Diagram modified (lists external entities and Influence Description only) | Jbaden1 |
| 2 | 2 | 2016-07-08 | - Template version added to footer - Several hints added to the various sections - Findings from Functional Safety Team incorporated. - RE_SafetyRequirement style added | Jbaden1 |
| 2 | 3 | 2016-09-21 | - Update from Functional Safety Team incorporated (“Lessons Learned”, “System Behaviors for HARA”) | Jbaden1 |
| 2 | 4 | 2016-11-15 | - Update from Functional Safety Team incorporated (“Lessons Learned”, “System Behaviors for HARA”) - Explanatory notes made more formal | Jbaden1 |
| 3 |  |  | Skipped to synchronize with Specification_Macros.dotm |  |
| 4 |  |  |  |  |
| 5 | 0 | 2017-01-13 | - Meta data updated for specification macros, version 3.1 - SW Unit chapter removed for the time being - Green boxes added for user hints | Jbaden1 |
| 5 | 1 | 2017-01-18 | - Minor editorial changes | Jbaden1 |
| 6 | 0 | 2017-02-03 | - CR48: Chapter 6 renamed from “Safety” to “Functional Safety”. New sub-chapter “Safety” introduced in Non-Functional Requirements section | Jbaden1 |
| 6 | 0 | 2017-04-28 | - CR7: “RequirementsTraceability” chapter removed | Jbaden1 |
| 6 | 0 | 2017-11-15 | - CR32/53: New Cover Sheet + Disclaimer replaces FAP-150 like ones. - CR75: Some rewording -> Terminology to Glossary, Notation -> Document Conventions - CR49: Rename “Assumptions & Constraints” to “Assumptions” - CR74: Safety Assumptions added to chapter 6. - CR58: Add function allocation column to Logical Architecture chapter | Jbaden1 |
| 6 | 0 | 2018-01-31 | - CR63: Updated links to Functional Safety Sharepoint | Jbaden1 |
| 6 | 0 | 2018-07-24 | - CR69: Add FSR to FeatureDoc - CR64: Add new section "Design Requirements" to Function Spec and Feature Spec | Jbaden1 |
| 6 | 0 | 2018-08-06 | - CR53: some corrections for metada and formatting | Jbaden1 |
| 6 | 0 | 2018-09-28 | - Broken links to RE Wiki repaired | Jbaden1 |
| 6 | 0 | 2018-10-31 | - Cover sheet and footer more GIS like. Functional Safety team feedback incorporated: - New subsections “Functional Safety Requirements, (Decomposed) FSRs and Parameters / Values - Removal of “Logical Architecture” | Jbaden1 |
| 6 | 0 | 2018-12-12 | - FSR template removed, now as a macro in the Specification_Macros.dotm | Jbaden1 |
| 6 | 0a | 2019-05-23 | - Re-introduce “Logical Architecture” (for Functional Safety) | Jbaden1 |
| 6 | 0b | 2019-06-26 | - Chapter “Logical Elements” in “Logical Architecture” section added (FuSa CR 15136240) | Jbaden1 |
| 6 | 0c | 2019-03-22 | - Chapter “Decomposed FSRs” renamed to “ASIL Decomposition of Functional Safety Requirements” and moved beneath Chapter “Functional Safety Requirements”. Explanatory text improved. | Jbaden1 |
| 6 | 0c | 2019-04-05 | - Some wording in ASIL decomposition table modified. Description of fields in that table improved. | Jbaden1 |
| 6 | 0c | 2019-06-24 | - “Input Requirements” section modified (table approach as for the other RE templates). - “References” and “Glossary” chapter moved to the “Introduction” chapter. | Jbaden1 |
| 6 | 0c | 2019-07-02 | - "Important" box added on cover sheet which points to the macros | Jbaden1 |
| 6 | 0c | 2019-07-02 | - Subsection “Error Handling” removed form chapter “Feature Requirements”->”Functional Requirements” (teams are free to create their own substructure of that section). Note tells author not to forget about error handling. - Hint for chapter “Feature Variants” improved reworded upon request from Functional Safety Team. | Jbaden1 |
| 6 | 0c | 2019-05-11 | - Copyright notice shortened and moved to cover sheet and added to footer (to be compliant with Ford copyright guidelines) - Term “Disclaimer” no longer used for what is actually only a copyright notice | Jbaden1 |
| 6 | 0c | 2019-22-11 | - Chapter “Input Requirements/Documentst: minor modifications (examples added), Word comment removed” | Jbaden1 |
| 6 | 0c | 2019-12-05 | - Upstream Documents section added to “Input Requirements/Documents” table - Custom style table formatting removed - Hint on system behaviors modified as requested from FuSa team | Jbaden1 |
| 6 | 0c | 2019-12-09 | - Term “Upstream Documents” replaced by “Attribute Requirements” in “Input Requirements/Documents” table - ASIL Decomposition table replaced by a version, which get not corrupted during VSEM import. | Jbaden1 |
| 6 | 0c | 2019-12-10 | - In ch. “Functional Safety Requirements” Word reference Id by Word reference text replaced.. | Jbaden1 |
| 6 | 1a | 2020-02-12 | - New chapter “Cybersecurity” added. | Jbaden1 |
| 6 | 1a | 2020-03-03 | - All User Hints formatted using style “RE_UserHint” to enable automatic removal by a macro. | Jbaden1 |
| 6 | 1a | 2020-03-04 | - Chapter “Cloud Connectivity Data Analytics Requirements” added upon request by D. Crockett/J. Rawlings | Jbaden1 |
| 6 | 1a | 2020-03-09 | - Missing doc property “LatestSigMappingID” and “LatestAisInterfaceID” added - doc property “CopyrightDate” re-formatted to text and copyright date field in footer corrected - Version numbering re-initialized as 0.1 - Init value of version/revision date set to “yyyy/mm/dd” instead of “yyyy-mm-dd” to be in line with the “Edit Document Property” dialog - type of document property for latest IDs changed to number instead of text | Jbaden1 |
| 6 | 1b | 2020-03-17 | - Chapter “Functional Architecture” renamed to “Functional Decomposition” - New MBSE terminology introduced: “Feature Level”, “Function Level” and “Component Level” renamed to “Concept Level”, “Logical Level” and “Technology Level” | Jbaden1 |
| 6 | 1b | 2020-07-03 | - CR31: Chapter “Traceability Matrix” added. | Jbaden1 |
| 6 | 1b | 2020-23-09 | - CR28: Alignment to FFSG01.10 Feature Document Guideline for how to apply the Feature Doc template for Functional Safety. New section “Classification of Chapters” added. “Active Tilt Control” Example in section “Logical Architecture” updated based on input from HARA training. | Jbaden1 |
| 6 | 1b | 2020-25-11 | - Reference to process definition in Stages added to “How to Use” section on cover sheet. User hints removed from “Document Purpose” chapter. - RE-Wiki links mostly replaced by Stages links, links to Functional Safety Sharepoint updated | Jbaden1 |

# Appendix

## Definitions

| Definition | Description |
| --- | --- |
| Ajar | Any door position other than closed. |
| AUTO | Contextual Ambient Lighting Automatic Mode: Lighting color and intensity driven by SDM. |
| Auto Ambient Lighting | Auto Ambient Lighting is a trade name displayed on the HMI for Contextual Ambient Lighting. |
| Contextual Ambient Lighting | Operation mode in which Ambient Lighting can be driven either by user input (in this case it is called MANUAL mode) or by SDM (in this case it is called AUTOMATIC). |
| Diagnostics and Configuration | Feature / System that deals with vehicle's configuration and diagnostics parameters and services such as lighting ramping up and down times, default colors, diagnostics services, etc. |
| Drive Mode | Current drive mode selected by user through SDM. |
| Front HMI | HMI available to driver and passenger at vehicle interior 1st row. |
| MANUAL | Contextual Ambient Lighting Manual Mode: Lighting color and intensity driven by user selection. |
| Rear HMI | HMI available to passengers at vehicle 2nd / 3rd interior rows. |
| Selectable Drive Mode | Feature that allows user to select a drive mode. |
| Vehicle Status | Any data that exposes vehicle system state variables such as ignition state, battery state, door ajar status, etc. |

Table 19: Definitions used in this document

## Abbreviations

| Abbr. | Stands for |
| --- | --- |
| AAL | Auto Ambient Lighting |
| Acc | Accessory |
| BCM | Body Control Module |
| CAL | Contextual Ambient Lighting |
| ECU | Electronic Control Unit |
| HMI | Human Machine Interface |
| LIN | Local Interconnected Network |
| SDM | Selectable Drive Mode |
| WF | Welcome / Farewell |

Table 20: Abbreviations used in this document

Document ends here.