Ambient Lighting

()

(F000063)

| Document Type | Feature Implementation Specification (FIS) |  |  |
| --- | --- | --- | --- |
| Template Version | 6.1a |  |  |
| SysML Report Template Version | 6.1a.11 |  |  |
| Document ID | FIS0 - Ambient Lighting - F000063 212 |  |  |
| Document Location | https://www.vsemweb.ford.com/tc/launchapp?-attach=true&-s=226TCSession&-o=zZsJ$ihEx3NrTDAAAAAAAAAAAAA&servername=Production_Server |  |  |
| Document Owner | Matheus Clavel |  |  |
| Document Revision | FIS0 |  |  |
| Document Status | Released |  |  |
| Date Issued | 2022/09/22 |  |  |
| Date Revised | 2022/09/22 |  |  |
| Model Name and Version | F000063-Ambient Lighting-RCRUZ92 - [#212] |  |  |
| Document Classification | GIS1 Item Number: | 27.60/35 |  |
|  | GIS2 Classification: | Confidential |  |

| Document Approval |  |  |  |
| --- | --- | --- | --- |
| Person | Role | Email Confirmation | Date |
| mclavel | Core Feature Systems Engineer | mclavel@ford.com | 2022/09/22 |
| bfreita4 | Feature Systems Supervisor | bfreita4@ford.com | 2022/09/22 |

This document contains Ford Motor Company Confidential information. Disclosure of the information contained in any portion of this document is not permitted without the expressed, written consent of a duly authorized representative of Ford Motor Company, Dearborn, Michigan, U.S.A.

Copyright ©2021, Ford Motor Company

Auto-Generated by MagicDraw

Printed Copies Are Uncontrolled

Disclaimer

This document contains Ford Motor Company Confidential information. Disclosure of the information contained in any portion of this document is not permitted without the expressed, written consent of a duly authorized representative of Ford Motor Company, Dearborn, Michigan, U.S.A.

This document contains information developed and accumulated by and for FORD MOTOR COMPANY. As such, it is a proprietary document, which, if disseminated to unauthorized persons, would provide others with restricted information, data, or procedures not otherwise available, exposing the FORD MOTOR COMPANY to potential harm.

Employees and suppliers having custody of this specification or authorized to use it must be cognizant of its proprietary nature and ensure that the information herein is not made available to unauthorized persons.

FORD MOTOR COMPANY reserves the right to protect this work as an unpublished copyrighted work in the event of an inadvertent or deliberate unauthorized publication. FORD MOTOR COMPANY also reserves its rights under copyright laws to protect this work as a published work.

This document or portions thereof shall not be distributed outside FORD MOTOR COMPANY without prior written consent. Refer all questions concerning disclosure to the author(s) or any duly authorized representative of Ford Motor Company.

Copyright © 2021 Ford Motor Company

Contents

1	Introduction	6

1.1	Document Purpose	6

1.2	Document Scope	6

1.3	Document Audience	6

1.3.1	Stakeholder List	6

1.4	Document Organization	6

1.4.1	Document Context	6

1.4.2	Document Structure	6

1.5	Document Conventions	7

1.5.1	Requirements Templates	7

1.5.1.1	Identification of requirements	7

1.5.1.2	Requirements Attributes	7

1.6	References	7

1.6.1	Ford Documents	8

1.6.2	External Documents and Publications	8

1.7	Glossary	9

1.7.1	Definitions	9

1.7.2	Abbreviations	10

2	Feature Implementation Overview	11

2.1	Description	11

2.2	Input Requirements/Documents	11

2.3	Lessons Learned	12

2.4	Assumptions	12

3	Feature Implementation Architecture	13

3.1	Functional Architecture	13

3.1.1	Description	13

3.1.2	Function List	13

3.1.3	Signal List	17

3.2	Physical Architecture	20

3.2.1	E/E Architecture	20

3.2.1.1	E/E Architecture Variants	20

3.2.1.2	E/E Components	21

3.2.1.3	E/E Connections	22

3.2.1.4	Signal List	23

3.2.2	Software Component Architecture	26

3.3	Function Deployment	26

3.3.1	Deployment Variants	26

3.3.1.1	Deployment Vehicle System Behavior	26

3.3.2	Function Allocation	26

4	Feature Implementation Modeling	33

4.1	Component Interaction Diagrams	33

4.2	Component Interface Behavior Diagrams	33

5	Feature Implementation Requirements	34

5.1	Functional Safety	34

5.1.1	ASIL Decomposition of Technical Safety Requirements	34

5.2	Requirements on Components	34

5.2.1	APIM	34

5.2.1.1	Technology Function  APIM_ProcessUserInputAndFeedback	34

5.2.2	BCM	54

5.2.2.1	Technology Function  BCM_PerformALMCircuitOutput	54

5.2.2.2	Technology Function  BCM_PerformAmbientLightingInputProcessing	56

5.2.2.3	Technology Function  BCM_PerformWelcomeFarewellFeatureProcessing	57

5.2.2.4	Technology Function  BCM_ProcessAmbientLightOutput	58

5.2.2.5	Technology Function  BCM_PerformAmbientProcessing	60

5.2.2.6	Technology Function  BCM_PerformCoreAmbientLightingProcessing	61

5.2.2.7	Technology Function  BCM_ProcessAmbientLightingOutput	63

5.2.2.8	Technology Function  BCM_PerformLINLedPowerControl	65

5.2.3	PCBA	66

5.2.3.1	Technology Function  ALCM_ProvideIllumination	66

5.2.4	RACM	67

5.2.4.1	Technology Function  RACM_ProcessUserFeedback	67

5.2.5	Vehicle	68

5.3	Requirements on Connections	68

5.3.1	Networks	68

5.3.1.1	“CAN Bus xxx”	68

5.3.1.2	“LIN Bus xxx”	69

5.3.2	HW I/Os	69

5.3.2.1	“HW I/O xxx”	69

6	Open Concerns	69

7	Revision History	70

8	Appendix	70

8.1	Data Dictionary	70

8.1.1	Logical Signals	70

8.1.2	Logical Parameters	82

8.1.3	Technical Signals	82

8.1.3.1	GSDB Signals	92

8.1.3.2	HW I/Os	92

8.1.3.3	Diagnostic Interfaces	92

8.1.4	Technical Parameters	100

8.1.5	Mappings	105

8.1.6	Technical Interfaces	110

8.1.6.1	AIS Interfaces	110

8.1.6.2	AUTOSAR Ports	110

8.1.7	Messages/APIs	110

8.1.7.1	CAN Bus “<Bus Name>”	110

8.1.7.2	LIN Bus “<Bus Name>”	128

8.1.7.3	AUTOSAR Interfaces	130

8.1.7.4	SOA Service Contracts	130

8.1.8	Encoding Types	130

8.1.8.1	Logical Encoding Types	130

8.1.8.2	Technology Encoding Types	138

List of Figures

Figure 3-1: Functional Boundary Behavior	13

Figure 3-2-1: FNV3 Variant	21

Figure 3-3-2: FNV2 Variant	21

Figure 3-4: Vehicle System Behavior	26

List of Tables

Table 1-1: Electrical Architecture(s) referenced in this document	6

Table 1-2: Ford internal Documents	8

Table 1-3: External documents and publications	9

Table 1-4: Definitions used in this document	10

Table 1-5: Abbreviations used in this document.	10

Table 2-1: Input Requirements/Documents	12

Table 3-1: List of Functions	17

Table 3-2: List of Logical Signals	20

Table 3-3: List of E/E Architecture Variants	20

Table 3-4: Electrical Components	22

Table 3-5: E/E Connections	23

Table 3-6: List of Technical Signals	26

Table 3-7: Function Allocation Table (Basic)	32

Table 5-1: Input Signal mappings of Function	34

Table 5-2: Output Signal mappings of Function	35

Table 5-3: Input Signal mappings of Function	55

Table 5-4: Output Signal mappings of Function	55

Table 5-5: Parameter mappings of Function	55

Table 5-6: Input Signal mappings of Function	56

Table 5-7: Output Signal mappings of Function	56

Table 5-8: Parameter mappings of Function	57

Table 5-9: Input Signal mappings of Function	58

Table 5-10: Output Signal mappings of Function	58

Table 5-11: Parameter mappings of Function	58

Table 5-12: Input Signal mappings of Function	59

Table 5-13: Output Signal mappings of Function	59

Table 5-14: Parameter mappings of Function	60

Table 5-15: Input Signal mappings of Function	60

Table 5-16: Output Signal mappings of Function	61

Table 5-17: Parameter mappings of Function	61

Table 5-18: Input Signal mappings of Function	62

Table 5-19: Output Signal mappings of Function	62

Table 5-20: Parameter mappings of Function	63

Table 5-21: Input Signal mappings of Function	64

Table 5-22: Output Signal mappings of Function	64

Table 5-23: Parameter mappings of Function	64

Table 5-24: Input Signal mappings of Function	65

Table 5-25: Output Signal mappings of Function	65

Table 5-26: Parameter mappings of Function	66

Table 5-27: Input Signal mappings of Function	67

Table 5-28: Input Signal mappings of Function	67

Table 5-29: Output Signal mappings of Function	67

Table 6-1: Open Concerns	69

# Introduction

## Document Purpose

The Feature Implementation Specification (FIS) specifies the deployment of the logical functions of a feature to an electrical architecture. The FIS specifies all interactions between the ECUs of the electrical architecture required for the feature including the technical signals and the interfaces. It also gives interface and integration requirements, which are specific to the feature for the electrical architecture.

To get more information about the concept of feature, function and component level abstraction refer to the Ford RE Wiki.

## Document Scope

This FIS describes the deployment of the feature  Ambient Lighting  to the following electrical architecture(s):

| Electrical Architecture Name | Owner | Reference |
| --- | --- | --- |
| FNV3 |  |  |
| FNV2 |  |  |

Table 1-1: Electrical Architecture(s) referenced in this document

## Document Audience

The FIS is authored by  Matheus Clavel . All Stakeholders, i.e., all people who have a valid interest in the feature implementation should read and, if possible, review the FIS. It needs to be guaranteed, that all stakeholders have access to the currently valid version of the FIS.

### Stakeholder List

For the latest list of stakeholder of the feature and their influence refer to:

Click here to open the latest Stakeholders List.

## Document Organization

### Document Context

Refer to the Specification Structure page in the Ford RE Wiki to understand how the FIS relates to other Ford Requirements Documents and Specifications.

### Document Structure

The structure of this document is explained below:

Section 1 – 	Introduction – Giving an explanation how to use this document including responsibilities and the scope of the document. Additionally, it contains the revision history and a list of unsettled but known issues that have to be consolidated in future versions. It explains the terminology and gives a clarification of the definitions, concepts and abbreviations used in the document.

Section 2 – 	Feature Implementation Description – Giving an overview of the platform and listing assumptions, constraints or dependencies

Section 3 – 	Feature Implementation Architecture – Describing 3 Architecture Views:

- Functional Architecture – Showing the logical architecture of functions

- Physical Architecture – Showing the physical architecture (first of all the E/E Architecture), which the Logical Functions get allocated to.

- Software Architecture – Showing the software architecture relevant for the feature (for features with in-house development only)

- Function Deployment – Presenting the allocation of logical functions and signals to the electrical and other components

Section 4 – 	Deployment Specific Modeling –Modeling techniques providing additional detail on e.g. interface behavior

Section 5 – 	Deployment Specific Requirements – Deployment specific requirements for ECUs, Network Communication, and Process

Section 6 – 	List of Open Concerns

Section 7 – 	Revision History

Section 8 – 	Appendix - Presenting additional data mainly in a tabular form, e.g., a data dictionary

## Document Conventions

### Requirements Templates

Refer to “How to use the Specification Templates” on how to use the specification templates and the VBA macros to create/edit the requirements in the specifications.

The VBA macro enable the import of the specification to VSEM (refer to "How to import specifications into VSEM as separate requirements").

#### Identification of requirements

The unique requirement ID given in the headline of any requirement follows the requirement throughout the development process. The requirement ID format follows a well-defined syntax.

All identifiers in an FIS shall be composed of 4 parts:

- A leading prefix, which indicates the type of requirement (R=Requirement, UC=Use Case, SC=Scenario, …)

- A prefix, which indicates the abstraction level (F=Feature, FNC=Function, CMP = component).

- Followed by a name, indicating the scope, which the requirement belongs to (e.g. feature or function name )

- Ending with the actual requirement number

Example:

R_CMP_LockArbitrator_00004	This is the fourth requirement on component level for the function Lock Arbitrator.

#### Requirements Attributes

Additionally attributes can be added to each requirement. This helps to classify requirements. A list of available attributes is given in the RE Wiki.

## References

### Ford Documents

The list of all Ford internal documents, which are directly related.

| Reference | Title | Doc. ID | Document Location | Revision |
| --- | --- | --- | --- | --- |
| Cybersecurity Assessment | Cybersecurity Relevancy Assessment Template V1.2 - Ambient Lighting 09-02-2021 | GIS1 Item Number: 27.60/35 | https://www.vsemweb.ford.com/tc/launchapp?-attach=true&-s=226TCSession&-o=0VuFivKLx3NrTD | v1.2 |
| FD | FD - Ambient Lighting - F000063 5-28-2021 (Working) | VDOC084911 | https://www.vsemweb.ford.com/tc/launchapp?-attach=true&-s=226TCSession&-o=xKe9M8cLx3NrTDAAAAAAAAAAAAA&servername=Production_Server | Draft |
| FIS | FIS - Ambient Lighting F000063 07-20-2021 (Working) | VDOC165786/A | https://www.vsemweb.ford.com/tc/launchapp?-attach=true&-s=226TCSession&-o=GthJJV8ax3NrTD | Draft |
| Ford GIS Standard | Ford GIS Standard |  |  |  |
| FS | FGS - Ambient Lighting 11-30-2020 | VDOC179646/A | https://www.vsemweb.ford.com/tc/launchapp?-attach=true&-s=226TCSession&-o=C3qJsAbGx3NrTD | Draft |
| Functional Specification Body Control Module | BCM Functional Specification | FS-RU5T-14B476-AGB001 | https://www.vsemweb.ford.com/tc/launchapp?-attach=true&-s=226TCSession&-o=CzkF7IK0x3NrTD | 16.01 |
| FUSA documentation guideline for FD | FFSG01.10 Feature Document (Item Definition) Guideline |  |  | 2022.1 |
| HMI Spec - APIM | VEHICLE SETTINGS APIM SPSS V1.23 | VSv2-FUN-REQ-025223 |  | 1.23 |

Table 1-2: Ford internal Documents

### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication | Document Location |
| --- | --- | --- |
| CIE_1938 Standard | Light Color Standard Pattern - Cx, Cy | https://en.wikipedia.org/wiki/CIE_1931_color_space |
| IEEE Std 1012-2004 IEEE Standard for Software Verification and Validation |  |  |
| ISO/IEC 19500-2:2003 | Information technology -- Open Distributed Processing -- Part 2 |  |
| Lumen definition | Lumen Definition | https://pt.wikipedia.org/wiki/L%C3%BAmen |
| UML Testing Profile (UTP), v1.2 |  |  |

Table 1-3: External documents and publications

## Glossary

### Definitions

| Definition | Description |
| --- | --- |
| Ajar | Any door position other than closed. |
| Another Three Letter Acronym | self explanatory |
| Arbitrator | Strategy that chooses an output based on several inputs according to well defined rules. |
| AUTO | Contextual Ambient Lighting Automatic Mode: Lighting color and intensity driven by SDM. |
| Auto Ambient Lighting | Auto Ambient Lighting is a trade name displayed on the HMI for Contextual Ambient Lighting. |
| concept | An abstract idea representing fundamental characteristics of its representation. Concepts are perceptions of an object, its characteristics (i.e., structural and behavioral), and its relationship(s) to another object(s). |
| Contextual Ambient Lighting | Operation mode in which Ambient Lighting can be driven either by user input (in this case it is called MANUAL mode) or by SDM (in this case it is called AUTOMATIC). |
| controlled vocabulary | an organized arrangement of words and phrases used to index content and/or to retrieve content through browsing or searching. It typically includes preferred and variant terms and has a defined scope or describes a specific domain. |
| controlled vocabulary term | An element of a controlled vocabulary |
| Diagnostics and Configuration | Feature / System that deals with vehicle's configuration and diagnostics parameters and services such as lighting ramping up and down times, default colors, diagnostics services, etc. |
| Drive Mode | Current drive mode selected by user through SDM. |
| End of Line | Production line phase in which electronic modules get programmed and configured for a specific product line variant / model. |
| Front HMI | HMI available to driver and passenger at vehicle interior 1st row. |
| Human Machine Interface | Any device that allows for interaction between humans and machines (touch screens, knobs, buttons, voice decoders, microphones, etc). |
| MANUAL | Contextual Ambient Lighting Manual Mode: Lighting color and intensity driven by user selection. |
| Master | Master node in a network. |
| Rear HMI | HMI available to passengers at vehicle 2nd / 3rd interior rows. |
| Selectable Drive Mode | Feature that allows user to select a drive mode. |
| Slave | Slave node in a network. |
| Traceability Matrix | Matrix that establishes relations between model elements. |
| Vehicle Status | Any data that exposes vehicle system state variables such as ignition state, battery state, door ajar status, etc. |
| vocabulary | A set of terms, each representing a single concept in a Domain of Discourse. A fundamental tool for communication and creating a common understanding of a Domain. |
| vocabulary Term | An abstraction, in the form of a natural language expression, representing a concept existing in the Ontology of a Domain of Discourse |

Table 1-4: Definitions used in this document

### Abbreviations

| Abbr. | Stands for |
| --- | --- |
| AAL | Auto Ambient Lighting |
| Acc | Accessory |
| AMBL | The Ambient Lighting feature itself. |
| BCM | Body Control Module |
| CAL | Contextual Ambient Lighting |
| ECU | Electronic Control Unit |
| EOL | End of Line |
| HMI | Human Machine Interface |
| LIN | Local Interconnected Network |
| SDM | Selectable Drive Mode |
| WF | Welcome / Farewell |

Table 1-5: Abbreviations used in this document.

# Feature Implementation Overview

## Description

Ambient Lighting Physical

Representation of the Ambient Lighting feature.

Description of Ambient Lighting:

With the ambient lighting feature, a user can set the mood in the vehicle’s cabin by selecting one of several different accent colors.

When activated ambient lighting illuminates foot wells, cup holders, and door release handles, map pockets, etc., depending on the vehicle model. A user can also adjust brightness settings to further personalize the interior.

On multicolor variants equipped with door handle lights, when Ambient Lighting feature is available to the user experience it will also be capable of indicating a cabin door being ajar by activating the door’s lights in a pre-configured color (Red).

On vehicles equipped with a Contextual Ambient Lighting (CAL) system, the Ambient Lighting will also provide support the Selectable Drive Mode feature. Which means that when the CAL is in Auto mode, the user will see Ambient Lighting lamps being orchestrated by the Selectable Drive Mode feature (colors shift) through traditional drive modes.

Some features such as Welcome & Farewell, Rejuvenate and Selectable Drive Mode use Ambient Lighting lamps as a resource to perform their requirements.

## Input Requirements/Documents

| Reference (Reference as listed in ch. “References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Feature Implementation Requirements”) |
| --- | --- | --- | --- |
| Feature/Function Requirements |  |  |  |
| Cybersecurity Assessment | Cybersecurity Relevancy Assessment Template V1.2 - Ambient Lighting 09-02-2021 | v1.2 |  |
| FD | FD - Ambient Lighting - F000063 5-28-2021 (Working) | Draft |  |
| FIS | FIS - Ambient Lighting F000063 07-20-2021 (Working) | Draft |  |
| Ford GIS Standard | Ford GIS Standard |  |  |
| FS | FGS - Ambient Lighting 11-30-2020 | Draft |  |
| Functional Specification Body Control Module | BCM Functional Specification | 16.01 |  |
| FUSA documentation guideline for FD | FFSG01.10 Feature Document (Item Definition) Guideline | 2022.1 |  |
| HMI Spec - APIM | VEHICLE SETTINGS APIM SPSS V1.23 | 1.23 |  |
| Ford Engineering Standards |  |  |  |
| Legal Regulations |  |  |  |
| Industry Standards |  |  |  |
|  | Ambient Lighting Strategy (UX / CIED) | Ambient Lighting shall comply with to RQT-002004-022593 the Ambient Lighting Strategy. |  |
|  | ISO 26262 | The system should be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Example AR |  |  |
|  | Execution of Logical Elements | The ambient lighting control function shall evaluate state charts and decision tables a maximum of one (1) time each execution cycle. |  |
|  | Function Execution Cycle Time | The Ambient Lighting Control function shall have an execution cycle time of no more than 21ms from receipt of all necessary input signals. |  |
|  | Light Output Color Range | This function shall be able to emit light in the color coordinates based on CIE 1931 Color Space. |  |
|  | Light Output Intensity Range | This function shall be able to emit light with intensity measurable in lumens [lm] unit at . Its intensity ranges from 0 up to 2.5 lm. |  |

Table 2-1: Input Requirements/Documents

## Lessons Learned

No lessons learned specified.

## Assumptions

No Assumptions specified.

# Feature Implementation Architecture

## Functional Architecture

### Description

This functional boundary diagram depicts logical function allocation in terms of logical function units.

Figure 3-1: Functional Boundary Behavior

### Function List

The following functions from the Global Feature & Function List are referenced in this Feature Implementation Specification:

| Function ID | Function Name | Function Description |
| --- | --- | --- |
|  | DetermineDriverRearDoorLockLightBehavior | Determine driver rear door lock light behavior when door is ajar according to Figure 2.2.17-14 Set Driver Rear Door Lock Indication FSM. |
|  | DeterminePassengerRearDoorLockLightBehavior | Determine passenger rear door lock light behavior when door is ajar according to Figure 2.2.17-15 Set Passenger Rear Door Lock Indication FSM. |
|  | PerformContextualAmbientLightingColorIntensityValueDecision | Returns ambient lighting color and intensity when contextual ambient lighting is active according to Table 2.2.17-26g Contextual Ambient Lighting Color and Intensity Value Decision Table. |
|  | PerformAmbientColorMappingDecision | Remaps user color input by user according to Table 2.2.17-11a Ambient Color mapping Decision Table. |
|  | DetermineNonDoorLIN Master_L1_P00LedState | Determines light color, intensity and behavior of all ambient lights based on state machine Figure 2.2.17-7 Determining non-Door LIN Master_L1_P00 FSM. |
|  | ProcessAmbientLightingOutput | Determines which light / group of light is to be turned ON / OFF. Determines which behavior (Welcome, Farewell, Ajar, etc) is to be performed by a single / group of lights. |
|  | DeterminePassengerRearDoorLockIndication | Turns passenger rear door lock indication ON / OFF according to Table 2.2.17-17 Determining Passenger Rear Door Lock Indication. |
|  | PerformAmbientLightingMemoryAndPersonalization | Manages user ambient lighting personalization / profile changes. |
|  | DeterminePassengerFrontDoorLockIndication | Turns passenger front door lock indication ON / OFF according to Table 2.2.17-19 Determining Passenger Front Door Lock Indication. |
|  | ProcessAmbientLightOutput | Process ambient lighting requests and send them out to ALM through LIN. |
|  | PerformCoreAmbientLightingProcessing | Manages Ambient Lighting features: - HMI Comm Variant Decoding - Memory and Personalization - Contextual Ambient Lighting (Selectable Drive Modes). |
|  | ProcessUserFeedback | Updates HMI 2 with user with current user settings. |
|  | DetermineALMPowerStatus | Determines ALM power status according to Table 2.2.36-3 ALM Power Status Decision Table. |
|  | DetermineLedLocationID | Determines which individual / group LED light is to be driven in terms of its location ID. |
|  | DetermineDriverFrontDoorLockIndication | Turns driver front door lock indication ON / OFF according to Table 2.2.17-17 Determining Driver Front Door Lock Indication. |
|  | PerformLedPowerControl | Command Ambient Light Module power supply ON / OFF according to Table 2.2.17-25 LIN-LED Power Control. |
|  | PerformSelectiveDriveModeRangeSelectionDecision | Determine which HMI Index will be used by HMI logic when ambient lighting is in contextual ambient lighting mode according to Table 2.2.17-26f Selective Drive Mode Range Selection Decision Table. |
|  | DeterminePassengerFrontDoorLockLightBehavior | Determine passenger front door lock light behavior when door is ajar according to Figure 2.2.17-13 Set Passenger Front Door Lock Indication FSM. |
|  | PerformContextualAmbientLighting | Manages ambient lighting when user switches to automatic mode. In automatic mode ambient lighting changes as drive mode changes. |
|  | DetermineDriverFrontDoorLightBehavior | Determine driver front door light behavior when door is Ajar according to Figure 2.2.17-8 Determining Driver Front Door FSM. |
|  | CalculateAmbientLightingActiveProfileIndex | Determines current ambient lighting active profile index according to Table 2.2.17-26 Ambient Lighting Active Profile Decision Table. |
|  | GenerateMessage0x37AmbientLightingData | Calculate ambient lighting color and intensity for LIN message 0x37 according to Table 5.3.14-3b Process Ambient Color and Intensity Request for LIN Message 0x37. |
|  | PerformAmbientProcessing | Command ambient lighting ON / OFF according to decision table 2.2.17-3 Ambient Lighting Decision Table. |
|  | ProcessUserInputAndFeedback | Updates HMI 1 with user with current user settings. |
|  | DeterminePersonalizedAmbientLightingColor | Determines current ambient lighting color according to Table 2.2.17-26b Personalized Ambient Lighting Color Selection Decision Table. |
|  | PerformAmbientLightingInputProcessing | Process ambient lighting color and intensity requested by user and updates HMI. |
|  | PerformSelectiveDriveModeRequestDecision | Evaluate selective drive mode enable conditions according to Table 2.2.17-26f Selective Drive Mode Range Selection Decision Table. |
|  | CheckLINWakeUp | Enable / disable communication with ALM when LIN wakes up according to Figure 5.3.14-2 ALM LIN Module Wake Up Status FSM. |
|  | PerformWelcomeFarewellFeatureProcessing | Determines which Welcome Farewell behavior is to be used when Ambient Lighting Feature is not operating in Ambient mode. |
|  | PerformDriveModeBasedLightAmbientColorDecision | Updates internal user chosen ambient color as user put ambient lighting feature in automatic mode according to Table 2.2.17-11ab Drive Mode Based Light Ambient Color Decision Table. |
|  | DeterminePassengerRearDoorLightBehavior | Determine passenger rear door light behavior when door is Ajar according to Figure 2.2.17-11 Determining Passenger Rear Door FSM. |
|  | PerformHMICommVariantDecoding | Determine ambient lighting color and intensity to be sent to HMI logic according to Table 2.2.17-25a HMI Comm Variant Color and Intensity Update Decision Table. |
|  | DetermineDriverRearDoorLockIndication | Turns driver rear door lock indication ON / OFF according to Table 2.2.17-21 Determining Driver Rear Door Lock Indication. |
|  | PerformALMCircuitOutput | Manages ambient lighting by providing LED light module with pwm duty cycle necessary to light dimming, turn ON/OFF command, diagnostic reading, etc. |
|  | DetermineLedParams | Determines Led light commands given by BCM to ALM according to Table 5.3.14-3 Process Ambient Message Request. |
|  | DetermineDriverRearDoorLightBehavior | Determine driver rear door light behavior when door is Ajar according to Figure 2.2.17-10 Determining Driver Rear Door FSM. |
|  | DetermineALMConditionedPwrCmd | Determines the command to be sent to ALM according to Table 2.2.36-1 ALM Circuit command Decision Table. |
|  | PerformContextualAmbientLightingHMIStatusUpdate | Updates HMI Logic with ambient lighting color associated with current drive mode picked by user according to Table 2.2.17-26d Contextual Ambient Lighting HMI Status Update Decision Table. |
|  | DeterminePassengerFrontDoorLightBehavior | Determine passenger front door light behavior when door is Ajar according to Figure 2.2.17-9 Determining Passenger Front Door FSM. |
|  | DetermineALMCktOutput | Determines output type according to ambient lighting type (single color vs multiple colors) according to Table 2.2.36-2 ALM Circuit Output Decision Table. |
|  | PerformAmbientIntensityMappingDecision | Rescale user chosen ambient lighting intensity from 0-100 to 0-15 according to Table 2.2.17-11b Ambient Intensity mapping Decision Table. |
|  | DeterminePersonalizedAmbientLightingIntensity | Determines current ambient lighting intensity according to Table 2.2.17-26c Personalized Ambient Lighting Intensity Selection Decision Table. |
|  | DeterminePersonalizedAmbientLightingStatus | Returns current ambient lighting status according to Table 2.2.17-26a Personalized Ambient Lighting Status Decision Table. |
|  | DetermineDriverFrontDoorLockLightBehavior | Determine driver front door lock light behavior when door is ajar according to Figure 2.2.17-12 Set Driver Front Door Lock Indication FSM. |

Table 3-1: List of Functions

### Signal List

| Signal Name | Description |
| --- | --- |
| All_DoorLock_Status |  |
| ALM_Conditioned_Pwr_Arb | Name: ALM_Conditioned_Pwr_Arb Description: Ambient Light Moudule conditioned power supply. |
| ALM_ConditionedPower |  |
| ALM_Location_ID | Name: ALM_Location_ID Description: Indicates the ALM module NODE ADDRESS placed at various locations.Generally, lighting elements will be located in the cupholders, center console, footwells, doors, scuffplates. |
| ALM_Power_Status | Name: ALM_Power_Status Description: This dataflow is used to convert the numeric output of ALM_Conditioned_Pwr_Ckt to discrete to send it over LIN |
| Ambient_Status | Name: Ambient_Status Description: Dataflow indicating the output of the ambient processing |
| AmbientColor_Cfg | Name: AmbientColor_Cfg Description: Currently selected ambient color DomainElement : 0:15 DomainDescription : Color level for ambient lighting |
| AmbientDim_Cfg | Name: AmbientDim_Cfg Description: Currently selected ambient dimming level DomainElement : 0:15 DomainDescription : Ambient ligiting dimming level |
| AmbientMsg_Rqst | Name: AmbientMsg_Rqst Description: This array contains the ambient message request for LIN module. The index 'n' has following domain values: ALL FOOTWELLS DF_DOOR PF_DOOR DR_DOOR PR_DOOR DF_DOOR_HANDLE PF_DOOR_HANDLE DR_DOOR_HANDLE PR_DOOR_HANDLE |
| Approach_LightShow | Name: Approach_LightShow Description: Indicates when to start approach light show. Note: It will take care of the maximum time limit of 25 seconds. |
| Color | Name: Color Description: Indicates Color need to set for ambient lighting. OFF domain value for Color is 0. |
| CourtesyDelay_Rqst | Name: CourtesyDelay_Rqst Description: Request to illuminate the interior courtesy lights. |
| CourtesyDemand_BSave_Rqst | Name: CourtesyDemand_BSave_Rqst Description: Request from Battery Saver to turn off the courtesy lights and the demand lights. |
| Customer_Color | Name: Customer_Color Description: Indicates Color need to set for ambient lighting. |
| Customer_Intensity | Name: Customer_Intensity Description: Indicates the intesity required to set for ambient lighting. |
| DBR_Reset_Cmd | Name: DBR_Reset_Cmd Description: Reset command for latching dual battery relay. This opens the latching relay |
| Delayed_Accessory_Cmd | Name: Delayed_Accessory_Cmd Description: Input to load shedding - delayed accesory status |
| DF_Door_Ajar_Status | Name: DF_Door_Ajar_Status Description: Indicates if the driver's front door is ajar. |
| DF_DoorLock_Status | Name: DF_DoorLock_Status Description: Driver Front door lock status |
| DiagSession | Name: DiagSession Description: Indicates the active session of Diagnostics |
| DispOpr_CS | Name: DispOpr_CS Description: Operation request command for personalization features from Center Stack. |
| DoorAjar |  |
| DoorCourtesyLight_Rqst | Name: DoorCourtesyLight_Rqst Description: Request to illuminate the interior courtesy lights. |
| DoorLocked |  |
| DR_Door_Ajar_Status | Name: DR_Door_Ajar_Status Description: Indicates if the driver's rear door is ajar. |
| DR_DoorLock_Status | Name: DR_DoorLock_Status Description: Driver rear door lock status |
| FET_Ctrl | Name: FET_Ctrl Description: Overrides FET output controller when SHORT. OK allows FET output process to drive the FET. |
| Ignition_Status | Name: Ignition_Status Description: The processed value for current Ignition state. |
| IlluminatedEntry_Rqst | Name: IlluminatedEntry_Rqst Description: Request to illuminate the interior courtesy lights. |
| IlluminatedExit_Rqst | Name: IlluminatedExit_Rqst Description: Request to illuminate the interior courtesy lights. |
| Intensity | Name: Intensity Description: Indicates the intesity required to set for ambient lighting. |
| LG_Other_Door_Ajar_Status | Name: LG_Other_Door_Ajar_Status Description: Indicates whether only LG is open or LG along with some other Door is Ajar. |
| LghtAmbColr_No_Rq | Ambient Lighting Color Selection Request. |
| LghtAmbDrvMde_B_Stat | Name: LghtAmbDrvMde_B_Stat Description: CAN signal for Contextual Ambient Lighting MANUAL and AUTOMATIC BCM f eedback. |
| LghtAmbDrvMde_D_Rq | Name: LghtAmbDrvMde_D_Rq Description: CAN signal for setting Contextual Ambient Lighting MANUAL and AUTOMATIC HMI request. |
| LghtAmbIntns_No_Rq | User request to set ambient intensity level. |
| LightAmbColor_No | Name: LightAmbColor_No Description: This number will be derived from CAN inputs from ambient color number depending on whether Enhanced memory exists. |
| LightAmbColor_No_Actl | Name: LightAmbColor_No_Actl Description: CAN signal for ambient color level status DomainElement : 0:15 DomainDescription : ambient color level |
| LightAmbColor_No_Rq | Name: LightAmbColor_No_Rq Description: CAN signal for setting ambient color level DomainElement : 0:15 DomainDescription : ambient color level |
| LightAmbColor_Temp | Name: LightAmbColor_Temp Description: Ambient color for Lincoln and Ford. 0-ICE_BLUE; 1- ORANGE; 2-SOFT_BLUE; 3-RED; 4-GREEN; 5-BLUE; 6-PURPLE; 7-LICOLN_WHITE; 8-AMBER; 9-TEAL; 10-BURNT_ORANGE; 11-LINCOLN_BLUE; 12-LINCOLN_GREEN; 13-LILAC |
| LightAmbIntsty_No | Name: LightAmbIntsty_No Description: This number will be derived from CAN inputs from ambient intensity number depending on whether Enhanced memory exists |
| LightAmbIntsty_No_Actl | Name: LightAmbIntsty_No_Actl Description: CAN signal for ambient color level status DomainElement : 0:100 DomainDescription : ambient dimming level |
| LightAmbIntsty_No_Rq | Name: LightAmbIntsty_No_Rq Description: CAN signal for setting ambient dim level DomainElement : 0:255 DomainDescription : ambient dimming level |
| Local_OpMode | Name: Local_OpMode Description: initial value changed Awake -> Sleep(Conti.m) This Dataflow indicates the current state of the ECU. |
| LowVoltMode | Name: LowVoltMode Description: This signal determines if active load will be affected or unaffected by Stop/Start event voltage drop. |
| Parklamps_Command | Name: Parklamps_Command Description: Command to control the position/parklamps. |
| PersIndex_CS | Name: PersIndex_CS Description: Personality index to use with operation from Center Stack. |
| PersNum | Name: PersNum Description: Indicates which personality is currently selected. |
| PF_Door_Ajar_Status | Name: PF_Door_Ajar_Status Description: Indicates if the passenger's front door is ajar. |
| PF_DoorLock_Status | Name: PF_DoorLock_Status Description: Passenger Front Door lock status. |
| PR_Door_Ajar_Status | Name: PR_Door_Ajar_Status Description: Indicates if the passenger's rear door is ajar. |
| PR_DoorLock_Status | Name: PR_DoorLock_Status Description: Passanger rear door lock status. |
| Ramp | Name: Ramp Description: Indicates ramping is required/not required for ambient lighting. |
| Reset | Name: Reset Description: Reset based on Tire Index |
| SelDrvMdeHmi03_D_Rq_ComStat | Name: SelDrvMdeHmi03_D_Rq_ComStat Description: Communication Status Received over CAN for SelDrvMdeHmi03_D_Rq. |
| SelDrvMdeHmi03_Rq | Name: SelDrvMdeHmi03_Rq Description: CAN signal for the selected drive mode. |
| Update_Color | Name: Update_Color Description: Indicates whether the color set for ambient lighting needs update or not. |
| Update_Intensity | Name: Update_Intensity Description: Indicates the intesity for ambient lighting needs to be updated or not. |
| VBattState | Name: VBattState Description: Array indicating Current State of a specific Voltage Range. |
| WelcomeFarewell_Status | Name: WelcomeFarewell_Status Description: Indicates welcome or farewell status for ambient lighting. |

Table 3-2: List of Logical Signals

## Physical Architecture

### E/E Architecture

#### E/E Architecture Variants

| E/E Architecture Variant Name | Variant Description | Variant Condition (Optional) |
| --- | --- | --- |
| FNV2 | BCM interface through HS1-CAN. |  |
| FNV3 | BCM interface through FD1-CAN. |  |

Table 3-3: List of E/E Architecture Variants

##### E/E Architecture “Architecture Variant: FNV3 Variant “

Physical FNV3 implementation of Ambient Lighting.

Figure 3-2-1: FNV3 Variant

##### E/E Architecture “Architecture Variant: FNV2 Variant “

Physical FNV2 implementation of Ambient Lighting.

Figure 3-3-2: FNV2 Variant

#### E/E Components

| Component Name | Description |
| --- | --- |
| ABS | Anti-lock breaking system. |
| AL (Ambient Lighting Physical) | Representation of the Ambient Lighting feature. |
| als (Ambient Light Sensor) | Ambient light sensor to indicate either a dark or light vehicle interior. |
| APIM | Accessory Protocol Interface Module - Main HMI. |
| bcm (BCM) | Body control module. |
| das (Door Ajar Switch) | Door ajar switch module. |
| ecg (ECG) | Enhanced Central Gateway. |
| environment (Environment) | Representation of the environment. |
| FD1-CAN | Flexible-data CAN Bus. |
| hs (Light Switch Module) | Head lamp / light switch module. |
| HS3-CAN | HS CAN bus for infotainment. |
| is (Ignition Switch) | Ignition switch module. |
| LIN-G9 (LIN-G(9)) | LIN Bus - H. For sending commands to PCB. |
| LIN-H5 (LIN-H(5)) | LIN Bus - L. For sending commands to PCB. |
| pcba (PCBA) | Printed Circuit Board – Ambient Lighting Module containing all LED drivers. |
| racm (RACM) | Rear Audio Control Module. |
| user (User) | Representation of the user. |

Table 3-4: Electrical Components

#### E/E Connections

| Connection Name | Connection Type | Protocol Only if ‘Connection Type’ is “Network”/”RF-Digital” | Description | Allocated Messages Only if ‘Connection Type’ is “Network”/”RF-Digital” | Connected Nodes |
| --- | --- | --- | --- | --- | --- |
| LIN | Network | N/A | LIN G - Please refer to the HLSM functional spec. LIN H and L - Please refer to the LDF described on 5.3.1.2 chapter | BCM_H_P00 (0x00) BCM_H_P04 (0x37) BCM_L_P00 (0x00) BCM_L_P04 (0x37) |  |
| Analog Hardwired | Analog | N/A | Ambient Light Sensor (ALS - Environment sensor) | AmbientLightLevelStatus |  |
| CAN HS1 | Network | CAN (High Speed) | BCM core processing and status sending | FNV2: APIM_Send_Signals_2_HS1 Body_Info_10 Personality_APIM_Data3_HS1 Personality_APIM_Data_HS1 Personality_BCM_Data Personality_BCM2_Data GatewayData8_HS1 |  |
| CAN FD1 | Network | CAN FD | BCM core processing and status sending | FNV2 Messages: APIM_Send_Signals_2_FD1 Body_Info_10_FD1 Personality_APIM_Data3_FD1 Personality_APIM_Data_FD1 Personality_BCM2_Data_FD1 SelectDriveModeData FNV3 Messages: APIM_Send_Signals_2_FD1 Rear_Climate_Data3_FD1 Body_Info_10 Personality_APIM_Data3_FD1 Personality_APIM_Data_FD1 Personality_BCM_Data Personality_BCM2_Data SelectDriveModeData |  |
| PWM Hardwired | PMW | N/A | ALM power supply circuit | ALM_Conditioned_Pwr_Ckt |  |
| CAN HS3 | Network | CAN (High Speed) | HMI sent and received signals | FNV2 Messages: APIM_Send_Signals_2 Body_Info_10_HS3 Personality_APIM_Data3 Personality_APIM_Data Personality_BCM_Data_HS3 Personality_BCM2_Data_HS3 SelectDriveModeData_HS3 FNV3 Messages: APIM_Send_Signals_2 Rear_Climate_Data3 Body_Info_10_HS3 Personality_APIM_Data3 Personality_APIM_Data Personality_BCM_Data_HS3 Personality_BCM2_Data_HS3 SelectDriveModeData_HS3 |  |
| Digital Hardwired | Digital | N/A | Door sensors Ignition switch | DoorAjarStatus IgnitionSwitchState |  |

Table 3-5: E/E Connections

#### Signal List

| Signal Name | Description |
| --- | --- |
| All_DoorLock_Status | This dataflow is used to convert the numeric output of ALM_Conditioned_Pwr_Ckt to discrete to send it over LIN. |
| ALM_Conditioned_Pwr_Arb | Ambient Light Moudule conditioned power supply. |
| ALM_Conditioned_Pwr_Ckt | Voltage supply to modules from body control module (BCM). |
| ALM_Power_Status | This dataflow is used to convert the numeric output of ALM_Conditioned_Pwr_Ckt to discrete. |
| Ambient_Status | Dataflow indicating the output of the ambient processing. |
| AmbientColor_Cfg | Currently selected ambient color |
| AmbientDim_Cfg | Currently selected ambient dimming level. |
| AmbientMsg_Rqst | This array contains the ambient message request for LIN module. The index 'n' has following domain values: ALL FOOTWELLS DF_DOOR PF_DOOR DR_DOOR PR_DOOR DF_DOOR_HANDLE PF_DOOR_HANDLE DR_DOOR_HANDLE PR_DOOR_HANDLE |
| Approach_LightShow | Indicates when to start approach light show. |
| Color |  |
| CourtesyDelay_Rqst | Request to illuminate the interior courtesy lights. |
| CourtesyDemand_BSave_Rqst | Request from Battery Saver to turn off the courtesy lights and the demand lights. |
| CtrStkDsplyOp_D_Rq | Operation request command for personalization features from Center Stack. |
| CtrStkPersIndex_D_Actl | Personality index to use with operation from Center Stack. |
| Customer_Color | Indicates Color need to set for ambient lighting. |
| Customer_Intensity | Indicates the intesity required to set for ambient lighting. |
| DBR_Reset_Cmd | Reset command for latching dual battery relay. This opens the latching relay. |
| Delayed_Accessory_Cmd | Input to load shedding - delayed accesory status. |
| DF_Door_Ajar_Status | Driver front door ajar status. |
| DF_DoorLock_Status | Driver Front door lock status. |
| DiagSession | DiagSession. |
| DoorCourtesyLight_Rqst | Request to illuminate the interior courtesy lights. |
| DR_Door_Ajar_Status | Driver rear door ajar status. |
| DR_DoorLock_Status | Driver rear door lock status. |
| FET_Ctrl | Overrides FET output controller when SHORT. OK allows FET output process to drive the FET. |
| Ignition_Status | Provide vehicle ignition status (OFF/ACC/RUN). |
| IlluminatedEntry_Rqst | Request to illuminate the interior courtesy lights. |
| IlluminatedExit_Rqst | Request to illuminate the interior courtesy lights. |
| Intensity |  |
| LG_Other_Door_Ajar_Status | Indicates whether only LG is open or LG along with some other Door is Ajared. |
| LghtAmbColr_No_Rq | Ambient Lighting Color Selection Request. |
| LghtAmbDrvMde_B_Stat | Feedback signal for Contextual Ambient Lighting status to HMI ECUs. |
| LghtAmbDrvMde_D_Rq |  |
| LghtAmbIntns_No_Rq | Ambient Lighting Intensity Selection Request. |
| LghtAmbRqSrc_B_Stat |  |
| LightAmbColor_No | This number will be derived from CAN inputs from ambient color number depending on whether Enhanced memory exists. |
| LightAmbColor_No_Actl | Ambient Lighting Color Selection Status. |
| LightAmbColor_No_Rq | Ambient Lighting Color Selection Request. |
| LightAmbColor_Temp | Ambient color for Lincoln and Ford. 0-ICE_BLUE; 1- ORANGE; 2-SOFT_BLUE; 3-RED; 4-GREEN; 5-BLUE; 6-PURPLE; 7- LICOLN_WHITE; 8-AMBER; 9-TEAL; 10-BURNT_ORANGE; 11-LINCOLN_BLUE; 12- LINCOLN_GREEN; 13-LILAC. |
| LightAmbIntsty_No | This number will be derived from CAN inputs from ambient intensity number depending on whether Enhanced memory exists. |
| LightAmbIntsty_No_Actl | Ambient Lighting Intensity Selection Status. |
| LightAmbIntsty_No_Rq | Ambient Lighting Intensity Selection Request. |
| Local_OpMode | initial value changed Awake -> Sleep(Conti.m) This Dataflow indicates the current state of the ECU. |
| Location_ID | Indicates the ALM module NODE ADDRESS placed at various locations.Generally, lighting elements will be located in the cupholders, center console, footwells, doors, scuffplates. |
| LowVoltMode | This signal determines if active load will be affected or unaffected by Stop/Start event voltage drop. |
| Parklamps_Command | Command to control the position/parklamps. |
| PersNo_D_Actl | Personalization profile number feedback to HMI. |
| PF_Door_Ajar_Status | Passenger Front Door lock status |
| PF_DoorLock_Status | Passember front door lock status. |
| PR_Door_Ajar_Status | Passenger rear door ajar status. |
| PR_DoorLock_Status | Passenger rear door lock status. |
| Ramp | Indicates ramping is required/not required for ambient lighting. |
| Reset | BCM reset. |
| SelDrvMdeHmi03_D_Rq | Drive mode request communication for the SDM interface HMI -Ambient Lighting. |
| SelDrvMdeHmi03_D_Rq_ComStat | Communication Status Received over CAN for SelDrvMdeHmi03_D_Rq. |
| Update_Color | Indicates whether the color set for ambient lighting needs update or not. |
| Update_Intensity | Indicates the intesity for ambient lighting needs to be updated or not. |
| VBattState | Array indicating Current State of a specific Voltage Range. |
| WelcomeFarewell_Status | Indicates welcome or farewell status for ambient lighting. |

Table 3-6: List of Technical Signals

### Software Component Architecture

No Software Component Architecture Diagrams identified.

## Function Deployment

### Deployment Variants

#### Deployment Vehicle System Behavior

FNV3/FNV2 Deployment Diagram.

Figure 3-4: Vehicle System Behavior

### Function Allocation

| Component | Technology Function Name | Logical Function Name |
| --- | --- | --- |
| APIM | APIM_ProcessUserInputAndFeedback | ProcessUserInputAndFeedback |
| BCM | BCM_PerformALMCircuitOutput | DetermineALMConditionedPwrCmd DetermineALMPowerStatus DetermineALMCktOutput PerformALMCircuitOutput |
|  | BCM_PerformAmbientLightingInputProcessing | PerformAmbientLightingInputProcessing PerformAmbientColorMappingDecision PerformAmbientIntensityMappingDecision PerformDriveModeBasedLightAmbientColorDecision |
|  | BCM_PerformWelcomeFarewellFeatureProcessing | PerformWelcomeFarewellFeatureProcessing |
|  | BCM_ProcessAmbientLightOutput | DetermineLedParams ProcessAmbientLightOutput DetermineLedLocationID GenerateMessage0x37AmbientLightingData CheckLINWakeUp |
|  | BCM_PerformAmbientProcessing | PerformAmbientProcessing |
|  | BCM_PerformCoreAmbientLightingProcessing | PerformContextualAmbientLightingHMIStatusUpdate PerformSelectiveDriveModeRangeSelectionDecision PerformContextualAmbientLightingColorIntensityValueDecision PerformSelectiveDriveModeRequestDecision CalculateAmbientLightingActiveProfileIndex PerformHMICommVariantDecoding PerformContextualAmbientLighting DeterminePersonalizedAmbientLightingIntensity DeterminePersonalizedAmbientLightingColor PerformCoreAmbientLightingProcessing PerformAmbientLightingMemoryAndPersonalization DeterminePersonalizedAmbientLightingStatus |
|  | BCM_ProcessAmbientLightingOutput | DetermineDriverRearDoorLockLightBehavior DeterminePassengerRearDoorLightBehavior DeterminePassengerFrontDoorLightBehavior DetermineDriverFrontDoorLockIndication DeterminePassengerFrontDoorLockLightBehavior DetermineNonDoorLIN Master_L1_P00LedState DetermineDriverRearDoorLightBehavior DeterminePassengerRearDoorLockLightBehavior DetermineDriverFrontDoorLockLightBehavior DeterminePassengerFrontDoorLockIndication DetermineDriverFrontDoorLightBehavior ProcessAmbientLightingOutput DetermineDriverRearDoorLockIndication DeterminePassengerRearDoorLockIndication |
|  | BCM_PerformLINLedPowerControl | PerformLedPowerControl |
| PCBA | ALCM_ProvideIllumination | No logical function Realized by this Technology function. |
| RACM | RACM_ProcessUserFeedback | ProcessUserFeedback |

Table 3-7: Function Allocation Table (Basic)

# Feature Implementation Modeling

## Component Interaction Diagrams

No Component Interaction Diagrams identified.

## Component Interface Behavior Diagrams

No Component Interface Diagrams identified.

# Feature Implementation Requirements

## Functional Safety

### ASIL Decomposition of Technical Safety Requirements

ASIL Decompositions not specified.

## Requirements on Components

### APIM

APIM

#### Technology Function  APIM_ProcessUserInputAndFeedback

Takes user input from HMI APIM module and updates it with user with current user input and current feature settings comming from BCM module.

##### Function Interfaces

###### Inputs

| Input Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Subscriber Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| LightAmbColor_No_Actl | LightAmbColor_No_Actl | LightAmbColor_No_Actl | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LightAmbIntsty_No_Actl | LightAmbIntsty_No_Actl | LightAmbIntsty_No_Actl | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LghtAmbDrvMde_B_Stat | LghtAmbDrvMde_B_Stat | LghtAmbDrvMde_B_Stat | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| PersNum | PersNo_D_Actl | PersNum | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-1: Input Signal mappings of Function

###### Outputs

| Output Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Publisher Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| LghtAmbDrvMde_D_Rq | LghtAmbDrvMde_D_Rq | LghtAmbDrvMde_D_Rq | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LightAmbIntsty_No_Rq | LightAmbIntsty_No_Rq | LightAmbIntsty_No_Rq | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LightAmbColor_No_Rq | LightAmbColor_No_Rq | LightAmbColor_No_Rq | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| DispOpr_CS | CtrStkDsplyOp_D_Rq | DispOpr_CS | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| PersIndex_CS | CtrStkPersIndex_D_Actl | PersIndex_CS | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LghtAmbRqSrc_B_Stat | LghtAmbRqSrc_B_Stat | No logical signals Realized by this Technology signal. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-2: Output Signal mappings of Function

###### Parameters

| Config Block | Byte | Bit(s) | Description | Default | Operation | Configuration DTC (E10100) |
| --- | --- | --- | --- | --- | --- | --- |
| DE00 | 8 | 2 | Ambient Lighting | 0 | 0 – Not Available 1 – Available | N/A |
| DE00 | 10 | 4 | Ambient Lighting Type | 0 | 0 – Pantone 1 – Single Color | N/A |
| DE00 | 11 | 4 | Ambient Lighting System Strategy | 0 | 0 – Variant 1 1 – Variant 2 | N/A |
| DE08 | 21 | 6 | Ambient Light Auto/Manual Setting | 0 | 0 – Disabled 1 – Enabled | N/A |
| DE08 | 9 | 1 | Select Mode | 0 | 0 – Disabled 1 - Enabled | N/A |

###### Interface Requirements

No Interface Requirements identified for Function APIM_ProcessUserInputAndFeedback

##### Function Requirements

###### Component Specific Requirements

IR2 - Ambient Lighting / Vehicle Settings Client/Server

The Vehicle Settings Client’s for Ambient Lighting feature responsibility shall be defined as: The Ambient Lighting Settings Client makes requests to the external vehicle settings function to change Ambient lighting color or intensity as requested by the user.

The Vehicle Settings Server for Ambient Lighting feature provides status of color or intensity settings client.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR3 - Ambient Lighting Drive Mode Client/Server

The Ambient Lighting Drive Mode Client interfaces with the user via HMI and shall be responsible for sending the Ambient Lighting Drive Mode setting request to the Ambient Lighting Drive Mode Server.

The Ambient Lighting Drive Mode Server shall be responsible for the ambient lighting drive mode function and interfaces with the Ambient Lighting Drive Mode Client.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR4 - Ambient Lighting / Sending of Request and Response

The feature shall follow the general rule for sending out the signals stated in APIM SPSS (IFS-MMCAN-FUR-REQ-015114/E-Sending of Request and Response (TcSE ROIN-66252-1)), i.e., request and response signals shall be sent out at the requested value and not put back to inactive/null until 100 msec +/- 10% has elapsed since the requested value was first put on the bus.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR5 - Ambient Lighting - Variant 2 Specific

This Ambient Lighting feature implementation shall be according to the Variant 2 specified in the latest infotainment Vehicle Settings SPSS.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR5 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR5.1 - Ambient Lighting Settings Client_Tx - Variant 2

The Ambient Lighting Client signals for transmitting the settings and requests to the server shall be the ones stated below:

- Color selection: LightAmbColor_No_Rq – Variant 2

- Intensity selection: LightAmbIntsty_No_Rq – Variant 2

- Manual or automatic mode status: LghtAmbRqSrc_B_Stat

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR5.1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR5.2 - Ambient Lighting Settings Client_Rx - Variant 2

The Ambient Lighting Client signals for reading feature’s status from the server shall be the ones stated below:

- Color selection: LightAmbColor_No_Actl – Variant 2

- Intensity selection: LightAmbIntsty_No_Actl – Variant 2

The signals are described in the Data Dictionary of this implementation specification.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR5.2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR6 - Ambient Lighting Server handling of "Inactive" in the Request signals

The Ambient Lighting Server shall treat LightAmbColor_No_Rq = Inactive and LightAmbIntsty_No_Rq = Inactive as don’t cares and shall not update the LightAmbColor_No_Actl and LightAmbIntsty_No_Actl status signals based on the request signals set to Inactive.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR7 - Ambient Lighting Bus Start-up

At network bus start-up the Ambient Lighting Server shall only publish the actual ambient lighting values of LightAmbColor_No_Actl and LightAmbIntsty_No_Actl and shall not publish the network init values.

At network bus start-up the Ambient Lighting Client shall set the request signals to Inactive.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR8 - Ambient Lighting HMI

The Ambient Lighting Client shall only display, on the ambient lighting HMI, the values indicated in the LightAmbColor_No_Actl and LightAmbIntsty_No_Actl status signals from the Ambient Lighting Server.

If the Ambient Lighting HMI is being displayed, a change in the ambient lighting status signal shall update the HMI.

If the Ambient Lighting Server sends LightAmbIntsty_No_Actl = “0% Intensity / Ambient Lighting OFF” then the Ambient Lighting Client HMI shall set Ambient Lighting HMI OFF.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR8 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR9 - Ambient Lighting Request and Response signals

The Ambient Lighting Client, when requesting an Ambient Lighting Color or Ambient Lighting Intensity value, shall set the color or intensity being requested and then set the request signal back to inactive.

When setting the request signal back to inactive the Ambient Lighting Client shall set to Inactive within 50 msec of making the request.

When setting the request signal back to inactive the Ambient Lighting Client shall set to Inactive no sooner than 20 msec after making the request.

The Ambient Lighting Server shall respond back to the LightAmbColor_No_Rq and LightAmbIntsty_No_Rq request signals within 150 msec of receiving the ambient lighting request.

Ex.

1. User selects a new ambient lighting color from the HMI

2. Ambient Lighting Client sets LightAmbIntsty_No_Rq = Color X and then 35 msec later sets LightAmbIntsty_No_Rq = Inactive.

3. The Ambient Lighting Server responds back within 150 msec of receiving LightAmbIntsty_No_Rq = Color X with LightAmbColor_No_Actl = Color X.

4. The Ambient Lighting Client updates the Ambient Lighting HMI based on the LightAmbColor_No_Actl status signal.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR9 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR10 - Turning ON and OFF Ambient Lighting

The Ambient Lighting Client can request the Ambient Lighting is turned ON using LightAmbIntsty_No_Rq = “Ambient Lighting Turn ON” or Ambient Lighting is turned OFF using “0% Intensity / Ambient Lighting Turn OFF”.

The Ambient Lighting Server is responsible for remembering the Color and Intensity values between Power Mode / Ignition cycles, network bus wake-ups, and B+ resets.

If Ambient Lighting is turned OFF the Ambient Lighting Server shall remember the color and intensity values before ambient lighting was turned OFF.

If Ambient Lighting is turned off (ie LightAmbIntsty_No_Actl = 0% Intensity / Ambient Lighting OFF) and if the Ambient Lighting Server receives LightAmbIntsty_No_Rq = “Ambient Lighting Turn ON” then the Ambient Lighting Server shall be responsible for publishing the Color and Intensity values to be used when turned ON.

- The Ambient Lighting Client could request Ambient Lighting ON with a particular Color set, OR

- The Ambient Lighting Client could request Ambient Lighting ON with the Color and Intensity set to Inactive

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR11 - Enhanced Memory - Ambient Lighting

If Enhanced Memory is configured ON in the Ambient Lighting Client than this “Ambient Lighting – Variant 2” strategy shall be used.

If Enhanced Memory is configured ON in the Ambient Lighting Server than this “Ambient Lighting – Variant 2” strategy shall be used.

The Ambient Lighting Server shall update the LightAmbColor_No_Actl and LightAmbIntsty_No_Actl status signals when changing to new enhanced memory profiles (ie when the active personality profile changes).

- If the Ambient Lighting Server is turned OFF the Ambient Lighting Server shall remember what all the personality profiles where before they were turned off (in case turned back on).

If the Ambient Lighting Client HMI is active the Ambient Lighting Client HMI will automatically update to whatever the new Color and Intensity values are when there is a new active personality since the Ambient Lighting Client will use the LightAmbColor_No_Actl and LightAmbIntsty_No_Actl status signals when they are updated.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR11 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR12 - Ambient Lighting Insensity - Variant 2 Specific

For FNV2 and FVN3 the Ambient Lighting Server only supports the intensity values called out in the table below.

The Ambient Lighting Client is only allowed to request the intensity values called out in the table below.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR12 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR13 - Ambient Lighting Sequence Diagrams

The implementation of the feature in the Client and its interaction with the server shall be per the following sequence diagrams:

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR13.1 - Changing Ambient Lighting Color

Pre-condition: Color Y is not the active color.

Event: User selects color Y.

Post-condition: Color Y is active on the HMI and the vehicle.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR13.1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR13.2 - Changing Ambient Lighting Intensity

Pre-condition: Intensity X is not the active intensity.

Event: User selects intensity X.

Post-Condition: Intensity X is shown on the HMI and Intensity X is active in the vehicle.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR13.2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR13.3 - Turning ON Ambient Lighting by selecting a Color

Pre-Condition:

Ambient Lighting was previously turned OFF via the HMI

Ambient Lighting in the vehicle is OFF

Event:

User selects colorX to turn ON ambient lighting

Post-Condition:

Ambient Lighting HMI is shown with ColorX active

Ambient Lighting Intensity is shown with last Intensity before turned back ON

Ambient Lighting is turned ON in the vehicle

Note: if enhanced memory is turned on then in the sequence diagram for network signal LightAmbIntsty_No_Actl = ‘Last Used’ is referring to the last used Intensity for the personality profile being turned on.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR13.3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR13.4 - Turning ON Ambient Lighting via ON/OFF HMI Selection

Pre-Condition:

Ambient Lighting turned OFF with the previous Color when last ON set to ColorY.

Ambient Lighting turned OFF with the previous Intensity when last ON set to IntensityX.

Event:

The user selects Ambient Lighting ON via the HMI.

Post-Condition:

Ambient Lighting HMI shows Ambient Lighting ON with ColorY and IntensityX.

Ambient Lighting is turned ON in the vehicle with ColorY and IntensityX.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR13.4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR13.5 - Turning OFF Ambient Lighting

Pre-Condition:

Ambient Lighting HMI is active showing Ambient Lighting is ON.

Event:

The user selects Ambient Lighting OFF via the HMI.

Post-Condition:

The HMI shows Ambient Lighting turned OFF.

Ambient Lighting is OFF in the vehicle.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR13.5 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR13.6 - Enhanced Memory - Recall new personality profile with Ambient Lighting active

Pre-Condition: Ambient Lighting HMI is active for PersonX.

Event: User changes from PersonX to PersonY.

Post-Condition: Ambient Lighting HMI is active for PersonY.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR13.6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR13.7 - Enhanced Memory - New Profile at Network Wake-up

Pre-Condition:

Network bus is asleep.

When Network bus last awake Person Z was the active profile.

Event:

1. Network bus wakes up.

2. The Active Profile is Person X.

3. The Ambient Lighting HMI screen is selected.

Post-Condition:

Ambient Lighting HMI is active for Person X.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR13.7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR14 - Ambient Lighting Drive Mode Client_Tx

The Ambient Lighting Drive Mode Client signals for transmitting the settings and requests to the server shall be the ones stated below:

Selection if Ambient Lighting is tied to Drive Mode or not: LghtAmbDrvMde_D_Rq.

Color selection: LightAmbColor_No_Rq – Variant 2 Specific.

The signals are described in the Data Dictionary of this implementation specification.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR14 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR15 - Ambient Lighting Drive Mode Client_Rx

The Ambient Lighting Drive Mode Client signals for reading feature’s status from the server shall be the ones stated below:

Color selection: LightAmbColor_No_Actl – Variant 2 Specific.

Status of whether Ambient Lighting is tied to Drive Mode or not: LghtAmbDrvMde_B_Stat

The signals are described in the Data Dictionary of this implementation specification.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR15 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR16 - Ambient Lighting Strategy required to be used when supporting Automatic/Manual Ambient Lighting Drive Mode

In order to support Manual and Auto Mode (color tied to drive mode in auto) both the Ambient Lighting Drive Mode Client and Server shall support the function requirements “VSv2-FUN-192195-Ambient Lighting – Variant 2” described in the latest infotainment Vehicle Setting SPSS.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR16 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR17 - Ambient Lighting Drive Mode Server functional requirement

The Ambient Lighting Drive Mode Server shall publish the Auto/Manual mode status via the LghtAmbDrvMde_B_Stat signal.

When in Auto mode, only the ambient lighting color is tied to Drive Mode.  The Ambient Lighting Drive Mode Server shall update the ambient lighting color based on drive mode.

Ambient Lighting Intensity is not tied to auto mode (ie not tied to drive mode).

If enhanced memory is supported the Ambient Lighting Drive Mode Server shall update the LghtAmbDrvMde_B_Stat signal to reflect the Auto/Manual status for the new personality profile.  See Ambient Lighting Drive Mode Server enhanced memory specification for details.

If the user selects a color during auto mode (ie receives LightAmbColor_No_Rq) then the Ambient Lighting Drive Mode Server shall change to manual mode and update LghtAmbDrvMde_B_Stat to manual mode to reflect the update.

See Ambient Lighting Drive Mode Server specification for additional details and requirements.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR17 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR18 - Ambient Lighting Drive Mode Client functional requirement

The Ambient Lighting Drive Mode Client shall use the LghtAmbDrvMde_B_Stat status signal to update the settings HMI to show whether the Ambient Lighting is in Auto or Manual mode.

The Ambient Lighting Drive Mode Client shall use the LghtAmbDrvMde_D_Rq signal to request Auto or Manual mode.

HMI Setting ID: 1026

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR18 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR19 - Enabling/Disabling Ambient Lighting Auto/Manual setting via the HMI

When the Ambient Lighting Automatic / Manual Drive Mode setting is selected via the HMI:

1. The Ambient Lighting Drive Mode Client shall set LghtAmbDrvMde_D_Rq to select Automatic or Manual based on what the user selected.

2. The Ambient Lighting Drive Mode Server shall respond with T_LghtAmbDrvMde_Rsp to the LghtAmbDrvMde_D_Rq Manual or Automatic request with the response via the LghtAmbDrvMde_B_Stat signal.

3. The Ambient Lighting Drive Mode Client shall update its HMI (if there is an update) with the Ambient Lighting Auto/Manual mode status after receiving the LightAmbDrvMde_B_Stat response to the request

Note:  See sequence diagrams with examples

The Auto/Manual setting on the HMI should only be available for selection when the ignition_status = Run.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR19 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR20 - T_LghtAmbDrvMde_Rsp

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR20 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR21 - Ambient Lighting Drive Mode Sequence Diagrams

The implementation of the Ambient Lighting Drive Mode settings in the Client and its interaction with the server shall be per the following sequence diagrams:

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR21 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR21.1 - Ambient Lighting Drive Mode set to Automatic via the HMI

Pre-Condition: Ambient Lighting Drive Mode set to manual.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR21.1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR21.2 - Ambient Lighting Drive Mode set to Manual via the HMI

Pre-Condition: Ambient Lighting Drive Mode in Auto Mode.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR21.2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

IR21.3 - User changes ambient lighting color while in auto mode

Pre-Condition: Ambient Lighting Drive Mode in Auto mode.

Satisfied by:

- APIM_ProcessUserInputAndFeedback

| Requirement ID: IR21.3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | Approved |
| Req. Template Version |  | 6.0c | End of Requirement |  |  |  |  |

### BCM

BCM

#### Technology Function  BCM_PerformALMCircuitOutput

Provides ALM with physical pwm / duty cycle necessary to Power Up/Down ALM.

##### Function Interfaces

###### Inputs

| Input Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Subscriber Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| ALM_Conditioned_Pwr_Arb | ALM_Conditioned_Pwr_Arb | ALM_Conditioned_Pwr_Arb | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| DBR_Reset_Cmd | DBR_Reset_Cmd | DBR_Reset_Cmd | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| FET_Ctrl | FET_Ctrl | FET_Ctrl | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| VBattState | VBattState | VBattState | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LowVoltMode | LowVoltMode | LowVoltMode | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-3: Input Signal mappings of Function

###### Outputs

| Output Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Publisher Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| ALM_Power_Status | ALM_Power_Status | ALM_Power_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| PIDReadAlmConditioned_Pwr |  | No logical signals Realized by this Technology signal. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| ALM_Conditioned_Pwr_Ckt | ALM_Conditioned_Pwr_Ckt | ALM_ConditionedPower | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-4: Output Signal mappings of Function

###### Parameters

| Logical Signal Name | Technical Parameter Name | Mapping Details (Conditional) | Method | Method Details |
| --- | --- | --- | --- | --- |
| No logical parameters Realized by this Technology parameter. | (AmbientLighting_CfgKind) AmbientLighting_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (DBRInCar_CfgKind) DBRInCar_Cfg |  | Choose an item. |  |

Table 5-5: Parameter mappings of Function

###### Interface Requirements

No Interface Requirements identified for Function BCM_PerformALMCircuitOutput

##### Function Requirements

###### Component Specific Requirements

No “Approved” or “Ready for Review” requirements identified for this function.

No removed/modified/added requirements from the logical specification (100% reuse/carry over of the Logical Function requirements – please refer to the latest Functional Specification).

#### Technology Function  BCM_PerformAmbientLightingInputProcessing

The Ambient Lighting Input processing will process ambient light color and intensity as well as user profile number requested by user from either APIM or RACM and keep them updated with the current feature settings.

##### Function Interfaces

###### Inputs

| Input Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Subscriber Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| LightAmbIntsty_No | LightAmbIntsty_No | LightAmbIntsty_No | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LightAmbColor_No | LightAmbColor_No | LightAmbColor_No | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LghtAmbDrvMde_B_Stat | LghtAmbDrvMde_B_Stat | LghtAmbDrvMde_B_Stat | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Reset | Reset | Reset | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-6: Input Signal mappings of Function

###### Outputs

| Output Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Publisher Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| AmbientDim_Cfg | AmbientDim_Cfg | AmbientDim_Cfg | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| AmbientColor_Cfg | AmbientColor_Cfg | AmbientColor_Cfg | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LightAmbIntsty_No_Actl | LightAmbIntsty_No_Actl | LightAmbIntsty_No_Actl | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LightAmbColor_No_Actl | LightAmbColor_No_Actl | LightAmbColor_No_Actl | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LightAmbColor_Temp | LightAmbColor_Temp | LightAmbColor_Temp | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-7: Output Signal mappings of Function

###### Parameters

| Logical Signal Name | Technical Parameter Name | Mapping Details (Conditional) | Method | Method Details |
| --- | --- | --- | --- | --- |
| No logical parameters Realized by this Technology parameter. | (byte) EntryColor_Cfg |  | Choose an item. |  |

Table 5-8: Parameter mappings of Function

###### Interface Requirements

No Interface Requirements identified for Function BCM_PerformAmbientLightingInputProcessing

##### Function Requirements

###### Component Specific Requirements

No “Approved” or “Ready for Review” requirements identified for this function.

No removed/modified/added requirements from the logical specification (100% reuse/carry over of the Logical Function requirements – please refer to the latest Functional Specification).

#### Technology Function  BCM_PerformWelcomeFarewellFeatureProcessing

Determines which Welcome Farewell behavior is to be used when Ambient Lighting Feature is not operating in Ambient mode.

##### Function Interfaces

###### Inputs

| Input Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Subscriber Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| Ignition_Status | Ignition_Status | Ignition_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| DoorCourtesyLight_Rqst | DoorCourtesyLight_Rqst | DoorCourtesyLight_Rqst | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| IlluminatedEntry_Rqst | IlluminatedEntry_Rqst | IlluminatedEntry_Rqst | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| IlluminatedExit_Rqst | IlluminatedExit_Rqst | IlluminatedExit_Rqst | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| CourtesyDelay_Rqst | CourtesyDelay_Rqst | CourtesyDelay_Rqst | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Approach_LightShow | Approach_LightShow | Approach_LightShow | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LG_Other_Door_Ajar_Status | LG_Other_Door_Ajar_Status | LG_Other_Door_Ajar_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| WelcomeFarewell_Status | WelcomeFarewell_Status | WelcomeFarewell_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Reset | Reset | Reset | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-9: Input Signal mappings of Function

###### Outputs

| Output Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Publisher Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| WelcomeFarewell_Status | WelcomeFarewell_Status | WelcomeFarewell_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-10: Output Signal mappings of Function

###### Parameters

| Logical Signal Name | Technical Parameter Name | Mapping Details (Conditional) | Method | Method Details |
| --- | --- | --- | --- | --- |
| No logical parameters Realized by this Technology parameter. | (AmbientLighting_CfgKind) AmbientLighting_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (Lincoln_Experience_CfgKind) Lincoln_Experience_Cfg |  | Choose an item. |  |

Table 5-11: Parameter mappings of Function

###### Interface Requirements

No Interface Requirements identified for Function BCM_PerformWelcomeFarewellFeatureProcessing

##### Function Requirements

###### Component Specific Requirements

No “Approved” or “Ready for Review” requirements identified for this function.

No removed/modified/added requirements from the logical specification (100% reuse/carry over of the Logical Function requirements – please refer to the latest Functional Specification).

#### Technology Function  BCM_ProcessAmbientLightOutput

Send out feature commands regarding LED dimming behavior, LED color / intensity updates, LED turning ON/OFF to ALM through LIN.

##### Function Interfaces

###### Inputs

| Input Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Subscriber Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| AmbientMsg_Rqst | AmbientMsg_Rqst | AmbientMsg_Rqst | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| ALM_Power_Status | ALM_Power_Status | ALM_Power_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LghtAmbDrvMde _B_Stat | LghtAmbDrvMde_B_Stat | LghtAmbDrvMde_B_Stat | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| AmbientColor_Cfg | AmbientColor_Cfg | AmbientColor_Cfg | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| AmbientDim_Cfg | AmbientDim_Cfg | AmbientDim_Cfg | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| WelcomeFarewell_Status | WelcomeFarewell_Status | WelcomeFarewell_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LightAmbColor_Temp | LightAmbColor_Temp | LightAmbColor_Temp | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-12: Input Signal mappings of Function

###### Outputs

| Output Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Publisher Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| Ramp | Ramp | Ramp | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Color | Color | Color | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Update_Color | Update_Color | Update_Color | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Intensity | Intensity | Intensity | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Update_Intensity | Update_Intensity | Update_Intensity | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| ALM_Location_ID | Location_ID | ALM_Location_ID | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Customer_Color | Customer_Color | Customer_Color | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Customer_Intensity | Customer_Intensity | Customer_Intensity | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-13: Output Signal mappings of Function

###### Parameters

| Logical Signal Name | Technical Parameter Name | Mapping Details (Conditional) | Method | Method Details |
| --- | --- | --- | --- | --- |
| No logical parameters Realized by this Technology parameter. | (byte) Run_Active_Count_Min_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (byte) Run_Active_Count_Max_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (byte) ALM_CommUp_Time_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (byte) Stat_Seq_Time_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (byte) EntryInt_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (byte) AjarColor_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (Integer) Stat_Seq_Wait_Time_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (byte) EntryColor_Cfg |  | Choose an item. |  |

Table 5-14: Parameter mappings of Function

###### Interface Requirements

No Interface Requirements identified for Function BCM_ProcessAmbientLightOutput

##### Function Requirements

###### Component Specific Requirements

No Requirements with Status “Approved” or “Ready for Review” satisfied by this function.

No removed/modified/added requirements from the logical specification (100% reuse/carry over of the Logical Function requirements – please refer to the latest Functional Specification).

#### Technology Function  BCM_PerformAmbientProcessing

Checks feature enabling conditions.

##### Function Interfaces

###### Inputs

| Input Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Subscriber Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| Parklamps_Command | Parklamps_Command | Parklamps_Command | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Ignition_Status | Ignition_Status | Ignition_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Delayed_Accessory_Cmd | Delayed_Accessory_Cmd | Delayed_Accessory_Cmd | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Reset | Reset | Reset | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-15: Input Signal mappings of Function

###### Outputs

| Output Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Publisher Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| Ambient_Status | Ambient_Status | Ambient_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-16: Output Signal mappings of Function

###### Parameters

| Logical Signal Name | Technical Parameter Name | Mapping Details (Conditional) | Method | Method Details |
| --- | --- | --- | --- | --- |
| No logical parameters Realized by this Technology parameter. | (AmbientLighting_CfgKind) AmbientLighting_Cfg |  | Choose an item. |  |

Table 5-17: Parameter mappings of Function

###### Interface Requirements

No Interface Requirements identified for Function BCM_PerformAmbientProcessing

##### Function Requirements

###### Component Specific Requirements

No “Approved” or “Ready for Review” requirements identified for this function.

No removed/modified/added requirements from the logical specification (100% reuse/carry over of the Logical Function requirements – please refer to the latest Functional Specification).

#### Technology Function  BCM_PerformCoreAmbientLightingProcessing

Manages Ambient Lighting features:

- HMI Comm Variant Decoding

- Memory and Personalization

- Contextual Ambient Lighting (Selectable Drive Modes).

##### Function Interfaces

###### Inputs

| Input Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Subscriber Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| DispOpr_CS | CtrStkDsplyOp_D_Rq | DispOpr_CS | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| PersIndex_CS | CtrStkPersIndex_D_Actl | PersIndex_CS | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LightAmbColor_No_Rq | LightAmbColor_No_Rq | LightAmbColor_No_Rq | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LightAmbIntsty_No_Rq | LightAmbIntsty_No_Rq | LightAmbIntsty_No_Rq | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LghtAmbDrvMde_D_Rq | LghtAmbDrvMde_D_Rq | LghtAmbDrvMde_D_Rq | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| SelDrvMdeHmi03_D_Rq | SelDrvMdeHmi03_D_Rq | SelDrvMdeHmi03_Rq | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Ignition_Status | Ignition_Status | Ignition_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Reset | Reset | Reset | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LghtAmbDrvMde_B_Stat | SelDrvMdeHmi03_D_Rq_ComStat | SelDrvMdeHmi03_D_Rq_ComStat | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| PersNum | PersNo_D_Actl | PersNum | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LghtAmbIntns_No_Rq | LghtAmbIntns_No_Rq | LghtAmbIntns_No_Rq | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LghtAmbColr_No_Rq | LghtAmbColr_No_Rq | LghtAmbColr_No_Rq | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LghtAmbRqSrc_B_Stat | LghtAmbRqSrc_B_Stat | No logical signals Realized by this Technology signal. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-18: Input Signal mappings of Function

###### Outputs

| Output Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Publisher Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| LightAmbIntsty_No | LightAmbIntsty_No | LightAmbIntsty_No | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LightAmbColor_No | LightAmbColor_No | LightAmbColor_No | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LghtAmbDrvMde_B_Stat | LghtAmbDrvMde_B_Stat | LghtAmbDrvMde_B_Stat | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-19: Output Signal mappings of Function

###### Parameters

| Logical Signal Name | Technical Parameter Name | Mapping Details (Conditional) | Method | Method Details |
| --- | --- | --- | --- | --- |
| No logical parameters Realized by this Technology parameter. | (AmbDrvMdeColor_CfgKind) AmbDrvMdeColor_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (ContextAL_CfgKind) ContextAL_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (HMI_Comm_Var_CfgKind) HMI_Comm_Var_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (EM_Exists_CfgKind) EM_Exists_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (byte) Ambient_Lighting_ON_Cfg |  | Choose an item. |  |

Table 5-20: Parameter mappings of Function

###### Interface Requirements

No Interface Requirements identified for Function BCM_PerformCoreAmbientLightingProcessing

##### Function Requirements

###### Component Specific Requirements

No “Approved” or “Ready for Review” requirements identified for this function.

No removed/modified/added requirements from the logical specification (100% reuse/carry over of the Logical Function requirements – please refer to the latest Functional Specification).

#### Technology Function  BCM_ProcessAmbientLightingOutput

Determines wich LED / group of LED is to be turned ON / OFF.

Determines which behavior (Welcome, Farewell, Ajar, etc) is to be used by a single / group of LEDs.

##### Function Interfaces

###### Inputs

| Input Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Subscriber Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| Ambient_Status | Ambient_Status | Ambient_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| WelcomeFarewell_Status | WelcomeFarewell_Status | WelcomeFarewell_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| All_DoorLock_Status | All_DoorLock_Status | All_DoorLock_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| DF_DoorLock_Status | DF_DoorLock_Status | DoorLocked DF_DoorLock_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| PF_DoorLock_Status | PF_DoorLock_Status | DoorLocked PF_DoorLock_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| DR_DoorLock_Status | DR_DoorLock_Status | DR_DoorLock_Status DoorLocked | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| PR_DoorLock_Status | PR_DoorLock_Status | PR_DoorLock_Status DoorLocked | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| DF_Door_Ajar_Status | DF_Door_Ajar_Status | DoorAjar DF_Door_Ajar_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| DR_Door_Ajar_Status | DR_Door_Ajar_Status | DoorAjar DR_Door_Ajar_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| PR_Door_Ajar_Status | PR_Door_Ajar_Status | PR_Door_Ajar_Status DoorAjar | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| PF_Door_Ajar_Status | PF_Door_Ajar_Status | PF_Door_Ajar_Status DoorAjar | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| AmbientColor_Cfg | AmbientColor_Cfg | AmbientColor_Cfg | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| AmbientDim_Cfg | AmbientDim_Cfg | AmbientDim_Cfg | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| ALM_Power_Status | ALM_Power_Status | ALM_Power_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Reset | Reset | Reset | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-21: Input Signal mappings of Function

###### Outputs

| Output Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Publisher Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| AmbientMsg_Rqst | AmbientMsg_Rqst | AmbientMsg_Rqst | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-22: Output Signal mappings of Function

###### Parameters

| Logical Signal Name | Technical Parameter Name | Mapping Details (Conditional) | Method | Method Details |
| --- | --- | --- | --- | --- |
| No logical parameters Realized by this Technology parameter. | (Integer) AmbientDelay_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (Lincoln_Experience_CfgKind) Lincoln_Experience_Cfg |  | Choose an item. |  |
| No logical parameters Realized by this Technology parameter. | (DoorLock_Indication_CfgKind) DoorLock_Indication_Cfg |  | Choose an item. |  |

Table 5-23: Parameter mappings of Function

###### Interface Requirements

No Interface Requirements identified for Function BCM_ProcessAmbientLightingOutput

##### Function Requirements

###### Component Specific Requirements

No “Approved” or “Ready for Review” requirements identified for this function.

No removed/modified/added requirements from the logical specification (100% reuse/carry over of the Logical Function requirements – please refer to the latest Functional Specification).

#### Technology Function  BCM_PerformLINLedPowerControl

Outputs a command to turn ON/OFF ALM based on several feature inputs.

##### Function Interfaces

###### Inputs

| Input Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Subscriber Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| DiagSession | DiagSession | DiagSession | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| CourtesyDemand_BSave_Rqst | CourtesyDemand_BSave_Rqst | CourtesyDemand_BSave_Rqst | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Local_OpMode | Local_OpMode | Local_OpMode | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Ambient_Status | Ambient_Status | Ambient_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| All_DoorLock_St atus | All_DoorLock_Status | All_DoorLock_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| WelcomeFarewell_Status | WelcomeFarewell_Status | WelcomeFarewell_Status | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-24: Input Signal mappings of Function

###### Outputs

| Output Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Publisher Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| ALM_Conditioned_Pwr_Arb | ALM_Conditioned_Pwr_Arb | ALM_Conditioned_Pwr_Arb | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-25: Output Signal mappings of Function

###### Parameters

| Logical Signal Name | Technical Parameter Name | Mapping Details (Conditional) | Method | Method Details |
| --- | --- | --- | --- | --- |
| No logical parameters Realized by this Technology parameter. | (ALMPowerMode_CfgKind) ALMPowerMode_Cfg |  | Choose an item. |  |

Table 5-26: Parameter mappings of Function

###### Interface Requirements

No Interface Requirements identified for Function BCM_PerformLINLedPowerControl

##### Function Requirements

###### Component Specific Requirements

No “Approved” or “Ready for Review” requirements identified for this function.

No removed/modified/added requirements from the logical specification (100% reuse/carry over of the Logical Function requirements – please refer to the latest Functional Specification).

### PCBA

PCBA

#### Technology Function  ALCM_ProvideIllumination

Interprets commands sent from BCM through LIN and drive LEDs.

##### Function Interfaces

###### Inputs

| Input Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Subscriber Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| ALM_Conditioned_Pwr_Ckt | ALM_Conditioned_Pwr_Ckt | ALM_ConditionedPower | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Ramp | Ramp | Ramp | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Color | Color | Color | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Update_Color | Update_Color | Update_Color | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Intensity | Intensity | Intensity | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Update_Intensity | Update_Intensity | Update_Intensity | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Location_ID | Location_ID | ALM_Location_ID | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Customer_Color | Customer_Color | Customer_Color | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| Customer_Intensity | Customer_Intensity | Customer_Intensity | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-27: Input Signal mappings of Function

###### Outputs

(No outputs have been defined)

###### Parameters

(No parameters have been defined)

###### Interface Requirements

No Interface Requirements identified for Function ALCM_ProvideIllumination

##### Function Requirements

###### Component Specific Requirements

No Requirements with Status “Approved” or “Ready for Review” satisfied by this function.

No removed/modified/added requirements from the logical specification (100% reuse/carry over of the Logical Function requirements – please refer to the latest Functional Specification).

### RACM

RACM

#### Technology Function  RACM_ProcessUserFeedback

Updates RACM module with current feature settings comming from BCM module.

##### Function Interfaces

###### Inputs

| Input Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Subscriber Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| LightAmbColor_No_Actl | LightAmbColor_No_Actl | LightAmbColor_No_Actl | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LightAmbIntsty_No_Actl | LightAmbIntsty_No_Actl | LightAmbIntsty_No_Actl | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-28: Input Signal mappings of Function

###### Outputs

| Output Name | Technical Signal Name | Logical Signal Name | Mapping Details (Conditional) | Publisher Interface | Connection (Optional) |
| --- | --- | --- | --- | --- | --- |
| LghtAmbIntns_No_Rq | LghtAmbIntns_No_Rq | LghtAmbIntns_No_Rq | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |
| LghtAmbColr_No_Rq | LghtAmbColr_No_Rq | LghtAmbColr_No_Rq | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. | Not supported by Magicdraw generation. |

Table 5-29: Output Signal mappings of Function

###### Parameters

(No parameters have been defined)

###### Interface Requirements

No Interface Requirements identified for Function RACM_ProcessUserFeedback

##### Function Requirements

###### Component Specific Requirements

No “Approved” or “Ready for Review” requirements identified for this function.

No removed/modified/added requirements from the logical specification (100% reuse/carry over of the Logical Function requirements – please refer to the latest Functional Specification).

### Vehicle

Vehicle abstraction – physical representation of the vehicle. Feature’s architecture allocation across all the vehicle components.

List of technological functions allocated:

BCM_PerformAmbientLightingInputProcessing

BCM_PerformLINLedPowerControl

BCM_ProcessAmbientLightOutput

RACM_ProcessUserFeedback

BCM_PerformAmbientProcessing

BCM_PerformWelcomeFarewellFeatureProcessing

ALCM_ProvideIllumination

BCM_PerformALMCircuitOutput

BCM_ProcessAmbientLightingOutput

APIM_ProcessUserInputAndFeedback

BCM_PerformCoreAmbientLightingProcessing

## Requirements on Connections

### Networks

#### “CAN Bus xxx”

No requirements deviated from the standard Netcom speficifacation (please refer to VSEM “Multiplexing Specifications” section).

The CAN messages relevant for this feature are listed in the section “CAN bus message/APIs” of the Data Dictionary.

###### Protocol Requirements

No specific requirements on FNOS.

##### Electrical Requirements

No deviation from the SDS CAN.

#### “LIN Bus xxx”

##### Protocol Requirements

LIN L BUS LDF:

LIN H BUS LDF:

###### Schedule Table

Please refer to the LDF attached above.

##### Electrical Requirements

Please refer to the LDF attached above.

### HW I/Os

#### “HW I/O xxx”

No specific protocol is used to send/receive signal information or multiplex/demultiplex signals on the HW circuit.

# Open Concerns

| ID | Concern Description | e-Tracker Reference | Status | Solution |
| --- | --- | --- | --- | --- |
|  | No open concerns |  |  |  |

Table 6-1: Open Concerns

# Revision History

| Rev. (revision) | Date | Description | Approved by | Responsible |
| --- | --- | --- | --- | --- |
| FIS0 | 2022-09-22 | V 0.0 2022 Auto generated from Magicdraw model - ready to be archived in VSEM. |  | rcruz92 |

## Template Revisions

| Version | Rev. | Date | Description | Responsible |
| --- | --- | --- | --- | --- |
| 0 | 2 | 2015-08-05 | - TOC corrected - Document Properties adapted to match needs of VBA macros | Awegman1 |
| 1 | 0 | 2015-11-16 | - Revision History moved to chapter 7 - Table-Styles removed | Awegman1 |
| 1 | 1 | 2016-03-02 | - Rework according to PCL example | Jbaden1 |
| 1 | 2 | 2016-03-22 | - V1.3: Footer formating corrected (Issue 19) - “Constraints” chapter renamed to “Input Requirements” (Issue 20) | Jbaden1 |
| 1 | 3 | 2016-04-20 | - Broken Wiki links repaired | Jbaden1 |
| 2 | 0 | 2016-05-23 | - Prepared for Specification_Macros.dotm v2.0 - Additional explanations added to ch. 2.2 “Input Requirements” (ARL and SDS requirements often go here) | Jbaden1 |
| 2 | 1 | 2016-07-08 | - Template version added to footer | Jbaden1 |
| 2 | 2 | 2016-07-15 | - Sample SysML diagrams added - Data Dictionary reworked - Alignment with relevant sections in SRD templated | Jbaden1 |
| 3 | 0 | 2016-09-05 | - Lessons learned from IPRB incorporated | Jbaden1 |
| 4 | 0 | 2016-09-27 | - Alignment with QPIP Feature Function Ownership workstream. Platform Spec renamed to Feature Implementation Spec | Jbaden1 |
| 4 | 1 | 2016-11-04 | - Chapters “Purpose” and “Scope” reworked. | Jbaden1 |
| 4 | 1 | 2016-11-10 | - Subsection for “Logical Service Interfaces” added. | Jbaden1 |
| 5 | 0 | 2017-01-13 | - Meta data updated for specification macros, version 3.1 - SW Unit chapter removed for the time being - Green boxes added for user hints | Jbaden1 |
| 5 | 1 | 2017-01-18 | - Minor editorial changes (e.g. hyperlinks highlighted in comments) | Jbaden1 |
| 5 | 1b | 2017-01-20 | - Some editorial corrections - Substructure of old Network Communication (now Connections) moved to Requirements on Connections | Jbaden1 |
| 6 | 0 | 2018-07-24 | - CR53: - Add new cover sheet - Add disclaimer section - Add the following meta-data to the doc properties for the the new cover sheet - DocGis1ItemNumber - DocGis2Classification - DocType - DocStatus - DocIssueDate - DocReleaseDate - CR63: Update FuSa sharepoint references in templates | Jbaden1 |
| 6 | 0 | 2018-08-06 | - CR81: Incorporate lessons learned from System Service Spec pilot (Vehicle Speed) into AFS and FIS | Jbaden1 |
| 6 | 0 | 2018-09-28 | - Broken links to RE Wiki repaired | Jbaden1 |
| 6 | 0 | 2018-10-31 | - Minor corrections on cover sheet and in footer to be more GIS compliant and VSEM aligned - “Overview” and “Description” exchanged in headings (following common sense) | Jbaden1 |
| 6 | 0 | 2018-11-30 | - Update of Functional Safety sections after review by Functional Safety Team - Initial support for variant handling | Jbaden1 |
| 6 | 0 | 2018-12-01 | - Variant condition fields added consistently - Links updated | Jbaden1 |
| 6 | 0 | 2018-12-11 | - Variant condition fields removed from mapping/allocation tables - Mapping tables simplified - Explanatory text for “Variants” sections revised | Jbaden1 |
| 6 | 0a | 2019-01-04 | - Chapter heading “Inherited Function Requirements” removed. Corresponding table renamed to “Requirements not cascaded”. - E/E Connection table got another column for allocated messages - Naming conventions for Implemented Functions corrected (FncName_CmpName instead of FncName_on_CmpName) - Editorial corrections on the cover sheet - Explanatory text added to “Ethernet” section in chapter “Requirements on Connections” - AIS templates updated. Linked to Wiki page | Jbaden1 |
| 6 | 0a | 2019-01-04 | - Minor restructuring in FuSa chapter – after aligning with ECU Functional Spec - Bugfix: table 13 renamed from FTTI table to FHT table, includes a bug fix: each FSR is allocated to only one ECU/component | Jbaden1 |
| 6 | 0b | 2019-02-04 | - Change: Chapter “Interface Requirements” added to “Implemented Function xxx” section (to have a single chapter for to collect subscriber/publisher interface and mapping requirements which to not conform to the corresponding Data Dictionary objects) - Change: “CAN Interface” subsection renamed to “AIS Interfaces” again. Although several Subscriber/Publisher interface attributes are probably CAN bus specific, other attributes seem to be well suited for other networks than CAN. - Change: Chapter “ECU Specific Requirements” renamed to “Component Specific Requirements” in chapter “Implemented Function xxx”. Table “Requirements not cascaded” renamed to “Component Specific Requirements” and refined to describe changes from Logical Function requirements set more formally. This is also to help during VSEM import to identify those requirements of the Logical Function which cannot be simply carried over to the ECU. - Change: Explanatory text in section “Implemented Function xxx” improved. | Jbaden1 |
| 6 | 0c | 2019-02-05 | - Change: Layout of AIS Interfaces in Data Dictionary reworked to enable Excel Import | Jbaden1 |
| 6 | 0c | 2019-02-20 | - Bugfix: In AIS Interfaces none-picklist fields formatted as invisible | Jbaden1 |
| 6 | 1a | 2019-02-05 | Functional Safety related changes: - Table “Architectural Redundancy Summary” updated - Section “Functional Flows for FTTI ‘xyz’” added to chapter “Component Interaction Diagrams” - Fault Tolerant Time Summary section added to Functional Safety chapter - Chapter “HW Metrics” added | Jbaden1 |
| 6 | 1a | 2019-04-02 | Headings of “Architectural Redundancy Summary” table clarified | Jbaden1 |
| 6 | 1a | 2019-04-10 | - ASIL Decomposition table moved from Function Spec into the Feature Implementation Spec (ASIL Decomposition of Technical Safety Requirements) - 2 alternative versions of the Function Allocation Table (Standard variant vs. Functional Safety variant) placed next to each other. | Jbaden1 |
| 6 | 1a | 2019-05-31 | - Function Allocation Table split into a base (non FuSa) part and a FuSa part to allow a more flexible mapping of MBSE functions (Logical and Technology) to RE functions (Atomic Logical and Implemented). | Jbaden1 |
| 6 | 1a | 2019-05-31 | - “Input Requirement” section reworked (symmetrically to all other templates). - Sections “Functional Flows for FTTI xyz” and “Fault Tolerant Time Summary” removed, because guidance is not available yet. - “Reference” and “Glossary” section moved back to introduction, i.e., to the very beginning of the document (such that also section 2 can already rely on it). - Some mostly editorial changes per request from FuSa team. | Jbaden1 |
| 6 | 1a | 2019-07-02 | - "Important" box added on cover sheet which points to the macros - “Input Requirements” section renamed to Input Information (after discussion with FuSa team) | Jbaden1 |
| 6 | 1a | 2019-07-17 | - Chapter “Message List” removed from CAN and LIN specific chapters of section “Requirements on Connections” | Jbaden1 |
| 6 | 1a | 2019-10-08 | - Chapter “ASIL Decomposition of Technical Safety Requirements”: Input TSRs are specified in the chapter right above the decomposition table. | Jbaden1 |
| 6 | 1a | 2019-10-09 | - Chapter “Service Oriented Communication” moved to section “Messages” in the Data Dictionary. Details from Central SW Wiki about FNV2 SOA added | Jbaden1 |
| 6 | 1a | 2019-10-25 | - Minor updates for HW IOs/Signals - Subsection “Functional Safety” removed from chapter “Feature Implementation Modeling”. Per requrest from FuSa team since no guidance is available how to model e.g. FHT timing diagram. | Jbaden1 |
| 6 | 1a | 2019-05-11 | - Copyright notice shortened and moved to cover sheet and added to footer (to be compliant with Ford copyright guidelines) - Term “Disclaimer” no longer used for what is actually only a copyright notice | Jbaden1 |
| 6 | 1a | 2019-22-11 | - Some minor modifications for the SOA APIs/MQTT Messages in the section “Messages” of the Data Dictionary (section references Service Contracts via the API name) - Some minor updates of the Input/Output mapping tables in section “Requirements on Components” for mappings to SOA APIs and EDAS signals. | Jbaden1 |
| 6 | 1a | 2019-12-05 | - Upstream Documents section added to “Input Requirements/Documents” table - Custom style table formatting removed | Jbaden1 |
| 6 | 1a | 2020-01-07 | - Some fine tuning for naming conventions of E/E components and connections. - List of HW I/O signal types reduced to RF-A, RF-D, D, A, Networked and PWM. - Protocol column added to the E/E connection table | Jbaden1 |
| 6 | 1a | 2020-01-07 | - “HW Metric” and “Architecture Redundancy Summary” sections removed per request from the Functional Architecture Team (based on Governance Board decision FSTGB-97) - “Functional Safety” chapter moved to “Feature Implementation Requirements” section. “Function Allocation” chapter seemed no longer appropriate. | Jbaden1 |
| 6 | 1a | 2020-01-07 | - Ordering of fields in AIS interfaces tables modified to conform with the Macro Template and the Importer Sheet - Page Header: no longer in bold letters | Jbaden1 |
| 6 | 1a | 2020-03-09 | - Missing doc property “LatestSigMappingID” and “LatestAisInterfaceID” added - doc property “CopyrightDate” re-formatted to text and copyright date field in footer corrected - Version numbering re-initialized as 0.1 - Init value of version/revision date set to “yyyy/mm/dd” instead of “yyyy-mm-dd” to be in line with the “Edit Document Property” dialog - Type of “Latest….ID” doc properties changed from Text to Number | Jbaden1 |
| 6 | 1a | 2020-03-11 | - “Mapping” table removed from template. Has been migrated to macro. | Jbaden1 |
| 6 | 1a | 2020-03-13 | - Separate chapter “Technical Safety Requirements” removed. Content already covered by Allocation Table in chapter Function Allocation. - “Implemented Function” replaced by term “Technology Function” | Jbaden1 |

# Appendix

## Data Dictionary

### Logical Signals

All_DoorLock_Status

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | LockKind |

Table: Signal Details of All_DoorLock_Status

ALM_Conditioned_Pwr_Arb

Name: ALM_Conditioned_Pwr_Arb

Description: Ambient Light Moudule conditioned power supply.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | ALM_Conditioned_Pwr_ArbKind |

Table: Signal Details of ALM_Conditioned_Pwr_Arb

ALM_ConditionedPower

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of ALM_ConditionedPower

ALM_Location_ID

Name: ALM_Location_ID

Description: Indicates the ALM module NODE ADDRESS placed at various locations.Generally, lighting elements

will be located in the cupholders, center console, footwells, doors, scuffplates.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | ALM_Location_IDKind |

Table: Signal Details of ALM_Location_ID

ALM_Power_Status

Name: ALM_Power_Status

Description: This dataflow is used to convert the numeric output of ALM_Conditioned_Pwr_Ckt to discrete to send

it over LIN

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | ALM_Power_StatusKind |

Table: Signal Details of ALM_Power_Status

Ambient_Status

Name: Ambient_Status

Description: Dataflow indicating the output of the ambient processing

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Ambient_StatusKind |

Table: Signal Details of Ambient_Status

AmbientColor_Cfg

Name: AmbientColor_Cfg

Description: Currently selected ambient color

DomainElement : 0:15

DomainDescription : Color level for ambient lighting

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of AmbientColor_Cfg

AmbientDim_Cfg

Name: AmbientDim_Cfg

Description: Currently selected ambient dimming level

DomainElement : 0:15

DomainDescription : Ambient ligiting dimming level

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of AmbientDim_Cfg

AmbientMsg_Rqst

Name: AmbientMsg_Rqst

Description: This array contains the ambient message request for LIN module.

The index 'n' has following domain values:

ALL

FOOTWELLS

DF_DOOR

PF_DOOR

DR_DOOR

PR_DOOR

DF_DOOR_HANDLE

PF_DOOR_HANDLE

DR_DOOR_HANDLE

PR_DOOR_HANDLE

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | LightLocationKind |

Table: Signal Details of AmbientMsg_Rqst

Approach_LightShow

Name: Approach_LightShow

Description: Indicates when to start approach light show.

Note: It will take care of the maximum time limit of 25 seconds.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Approach_LightShowKind |

Table: Signal Details of Approach_LightShow

Color

Name: Color

Description: Indicates Color need to set for ambient lighting.

OFF domain value for Color is 0.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of Color

CourtesyDelay_Rqst

Name: CourtesyDelay_Rqst

Description: Request to illuminate the interior courtesy lights.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | CourtesyDelay_RqstKind |

Table: Signal Details of CourtesyDelay_Rqst

CourtesyDemand_BSave_Rqst

Name: CourtesyDemand_BSave_Rqst

Description: Request from Battery Saver to turn off the courtesy lights and the demand lights.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | CourtesyDemand_BSave_RqstKind |

Table: Signal Details of CourtesyDemand_BSave_Rqst

Customer_Color

Name: Customer_Color

Description: Indicates Color need to set for ambient lighting.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of Customer_Color

Customer_Intensity

Name: Customer_Intensity

Description: Indicates the intesity required to set for ambient lighting.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of Customer_Intensity

DBR_Reset_Cmd

Name: DBR_Reset_Cmd

Description: Reset command for latching dual battery relay. This opens the latching relay

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DBR_Reset_CmdKind |

Table: Signal Details of DBR_Reset_Cmd

Delayed_Accessory_Cmd

Name: Delayed_Accessory_Cmd

Description: Input to load shedding - delayed accesory status

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Delayed_Accessory_CmdKind |

Table: Signal Details of Delayed_Accessory_Cmd

DF_Door_Ajar_Status

Name: DF_Door_Ajar_Status

Description: Indicates if the driver's front door is ajar.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | AjarKind |

Table: Signal Details of DF_Door_Ajar_Status

DF_DoorLock_Status

Name: DF_DoorLock_Status

Description: Driver Front door lock status

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DoorLock_StatusKind |

Table: Signal Details of DF_DoorLock_Status

DiagSession

Name: DiagSession

Description: Indicates the active session of Diagnostics

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DiagSessionKind |

Table: Signal Details of DiagSession

DispOpr_CS

Name: DispOpr_CS

Description: Operation request command for personalization features from Center Stack.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DispOpr_CSKind |

Table: Signal Details of DispOpr_CS

DoorAjar

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | AjarKind |

Table: Signal Details of DoorAjar

DoorCourtesyLight_Rqst

Name: DoorCourtesyLight_Rqst

Description: Request to illuminate the interior courtesy lights.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DoorCourtesyLight_RqstKind |

Table: Signal Details of DoorCourtesyLight_Rqst

DoorLocked

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | LockKind |

Table: Signal Details of DoorLocked

DR_Door_Ajar_Status

Name: DR_Door_Ajar_Status

Description: Indicates if the driver's rear door is ajar.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | AjarKind |

Table: Signal Details of DR_Door_Ajar_Status

DR_DoorLock_Status

Name: DR_DoorLock_Status

Description: Driver rear door lock status

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | LockKind |

Table: Signal Details of DR_DoorLock_Status

FET_Ctrl

Name: FET_Ctrl

Description: Overrides FET output controller when SHORT. OK allows FET output process to drive the FET.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | FET_CtrlKind |

Table: Signal Details of FET_Ctrl

Ignition_Status

Name: Ignition_Status

Description: The processed value for current Ignition state.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | -- |
|  | Value Range | [ - ] |
|  | Resolution |  |
|  | Unit |  |

Table: Signal Details of Ignition_Status

IlluminatedEntry_Rqst

Name: IlluminatedEntry_Rqst

Description: Request to illuminate the interior courtesy lights.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | IlluminatedEntry_RqstKind |

Table: Signal Details of IlluminatedEntry_Rqst

IlluminatedExit_Rqst

Name: IlluminatedExit_Rqst

Description: Request to illuminate the interior courtesy lights.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | IlluminatedExit_RqstKind |

Table: Signal Details of IlluminatedExit_Rqst

Intensity

Name: Intensity

Description: Indicates the intesity required to set for ambient lighting.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of Intensity

LG_Other_Door_Ajar_Status

Name: LG_Other_Door_Ajar_Status

Description: Indicates whether only LG is open or LG along with some other Door is Ajar.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | LG_Other_Door_Ajar_StatusKind |

Table: Signal Details of LG_Other_Door_Ajar_Status

LghtAmbColr_No_Rq

Ambient Lighting Color Selection Request.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LghtAmbColr_No_Rq

LghtAmbDrvMde_B_Stat

Name: LghtAmbDrvMde_B_Stat

Description: CAN signal for Contextual Ambient Lighting MANUAL and AUTOMATIC BCM f eedback.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | LghtAmbDrvMde_B_StatKind |

Table: Signal Details of LghtAmbDrvMde_B_Stat

LghtAmbDrvMde_D_Rq

Name: LghtAmbDrvMde_D_Rq

Description: CAN signal for setting Contextual Ambient Lighting MANUAL and AUTOMATIC HMI request.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | LghtAmbDrvMde_D_RqKind |

Table: Signal Details of LghtAmbDrvMde_D_Rq

LghtAmbIntns_No_Rq

User request to set ambient intensity level.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LghtAmbIntns_No_Rq

LightAmbColor_No

Name: LightAmbColor_No

Description: This number will be derived from CAN inputs from ambient color number depending on whether

Enhanced memory exists.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LightAmbColor_No

LightAmbColor_No_Actl

Name: LightAmbColor_No_Actl

Description: CAN signal for ambient color level status

DomainElement : 0:15

DomainDescription : ambient color level

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LightAmbColor_No_Actl

LightAmbColor_No_Rq

Name: LightAmbColor_No_Rq

Description: CAN signal for setting ambient color level

DomainElement : 0:15

DomainDescription : ambient color level

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LightAmbColor_No_Rq

LightAmbColor_Temp

Name: LightAmbColor_Temp

Description: Ambient color for Lincoln and Ford.

0-ICE_BLUE; 1- ORANGE; 2-SOFT_BLUE; 3-RED; 4-GREEN; 5-BLUE; 6-PURPLE; 7-LICOLN_WHITE; 8-AMBER; 9-TEAL; 10-BURNT_ORANGE; 11-LINCOLN_BLUE; 12-LINCOLN_GREEN; 13-LILAC

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LightAmbColor_Temp

LightAmbIntsty_No

Name: LightAmbIntsty_No

Description: This number will be derived from CAN inputs from ambient intensity number depending on whether

Enhanced memory exists

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LightAmbIntsty_No

LightAmbIntsty_No_Actl

Name: LightAmbIntsty_No_Actl

Description: CAN signal for ambient color level status

DomainElement : 0:100

DomainDescription : ambient dimming level

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LightAmbIntsty_No_Actl

LightAmbIntsty_No_Rq

Name: LightAmbIntsty_No_Rq

Description: CAN signal for setting ambient dim level

DomainElement : 0:255

DomainDescription : ambient dimming level

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LightAmbIntsty_No_Rq

Local_OpMode

Name: Local_OpMode

Description: initial value changed Awake -> Sleep(Conti.m)

This Dataflow indicates the current state of the ECU.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Local_OpModeKind |

Table: Signal Details of Local_OpMode

LowVoltMode

Name: LowVoltMode

Description: This signal determines if active load will be affected or unaffected by Stop/Start event voltage drop.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | LowVoltModeKind |

Table: Signal Details of LowVoltMode

Parklamps_Command

Name: Parklamps_Command

Description: Command to control the position/parklamps.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Parklamps_CommandKind |

Table: Signal Details of Parklamps_Command

PersIndex_CS

Name: PersIndex_CS

Description: Personality index to use with operation from Center Stack.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | PersIndex_CSKind |

Table: Signal Details of PersIndex_CS

PersNum

Name: PersNum

Description: Indicates which personality is currently selected.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | PersNumKind |

Table: Signal Details of PersNum

PF_Door_Ajar_Status

Name: PF_Door_Ajar_Status

Description: Indicates if the passenger's front door is ajar.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | AjarKind |

Table: Signal Details of PF_Door_Ajar_Status

PF_DoorLock_Status

Name: PF_DoorLock_Status

Description: Passenger Front Door lock status.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | -- |
|  | Value Range | [ - ] |
|  | Resolution |  |
|  | Unit |  |

Table: Signal Details of PF_DoorLock_Status

PR_Door_Ajar_Status

Name: PR_Door_Ajar_Status

Description: Indicates if the passenger's rear door is ajar.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | AjarKind |

Table: Signal Details of PR_Door_Ajar_Status

PR_DoorLock_Status

Name: PR_DoorLock_Status

Description: Passanger rear door lock status.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | LockKind |

Table: Signal Details of PR_DoorLock_Status

Ramp

Name: Ramp

Description: Indicates ramping is required/not required for ambient lighting.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | RampKind |

Table: Signal Details of Ramp

Reset

Name: Reset

Description: Reset based on Tire Index

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | ResetKind |

Table: Signal Details of Reset

SelDrvMdeHmi03_D_Rq_ComStat

Name: SelDrvMdeHmi03_D_Rq_ComStat

Description: Communication Status Received over CAN for SelDrvMdeHmi03_D_Rq.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | SelDrvMdeHmi03_D_Rq_ComStatKind |

Table: Signal Details of SelDrvMdeHmi03_D_Rq_ComStat

SelDrvMdeHmi03_Rq

Name: SelDrvMdeHmi03_Rq

Description: CAN signal for the selected drive mode.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of SelDrvMdeHmi03_Rq

Update_Color

Name: Update_Color

Description: Indicates whether the color set for ambient lighting needs update or not.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Update_ColorKind |

Table: Signal Details of Update_Color

Update_Intensity

Name: Update_Intensity

Description: Indicates the intesity for ambient lighting needs to be updated or not.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Update_IntensityKind |

Table: Signal Details of Update_Intensity

VBattState

Name: VBattState

Description: Array indicating Current State of a specific Voltage Range.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | VBattStateKind |

Table: Signal Details of VBattState

WelcomeFarewell_Status

Name: WelcomeFarewell_Status

Description: Indicates welcome or farewell status for ambient lighting.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | WelcomeFarewell_StatusKind |

Table: Signal Details of WelcomeFarewell_Status

### Logical Parameters

Please check the latest Functional Group Specification for the logical parameters and the 8.1.4 section (link) where they are reflected to technical parameters.

### Technical Signals

All_DoorLock_Status

This dataflow is used to convert the numeric output of ALM_Conditioned_Pwr_Ckt to discrete to send

it over LIN.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DoorLock_StatusKind |

Table: Signal Details of All_DoorLock_Status

ALM_Conditioned_Pwr_Arb

Ambient Light Moudule conditioned power supply.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | ALM_Conditioned_Pwr_ArbKind |

Table: Signal Details of ALM_Conditioned_Pwr_Arb

ALM_Conditioned_Pwr_Ckt

Voltage supply to modules from body control module (BCM).

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | voltage |

Table: Signal Details of ALM_Conditioned_Pwr_Ckt

ALM_Power_Status

This dataflow is used to convert the numeric output of ALM_Conditioned_Pwr_Ckt to discrete.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | ALM_Power_StatusKind |

Table: Signal Details of ALM_Power_Status

Ambient_Status

Dataflow indicating the output of the ambient processing.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Ambient_StatusKind |

Table: Signal Details of Ambient_Status

AmbientColor_Cfg

Currently selected ambient color

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of AmbientColor_Cfg

AmbientDim_Cfg

Currently selected ambient dimming level.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of AmbientDim_Cfg

AmbientMsg_Rqst

This array contains the ambient message request for LIN module.

The index 'n' has following domain values:

ALL

FOOTWELLS

DF_DOOR

PF_DOOR

DR_DOOR

PR_DOOR

DF_DOOR_HANDLE

PF_DOOR_HANDLE

DR_DOOR_HANDLE

PR_DOOR_HANDLE

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | AmbientMsg_RqstKind |

Table: Signal Details of AmbientMsg_Rqst

Approach_LightShow

Indicates when to start approach light show.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Approach_LightShowKind |

Table: Signal Details of Approach_LightShow

Color

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | ColorKind |

Table: Signal Details of Color

CourtesyDelay_Rqst

Request to illuminate the interior courtesy lights.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | CourtesyDelay_RqstKind |

Table: Signal Details of CourtesyDelay_Rqst

CourtesyDemand_BSave_Rqst

Request from Battery Saver to turn off the courtesy lights and the demand lights.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | CourtesyDemand_BSave_RqstKind |

Table: Signal Details of CourtesyDemand_BSave_Rqst

CtrStkDsplyOp_D_Rq

Operation request command for personalization features from Center Stack.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DispOpr_CSKind |

Table: Signal Details of CtrStkDsplyOp_D_Rq

CtrStkPersIndex_D_Actl

Personality index to use with operation from Center Stack.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | PersIndex_CSKind |

Table: Signal Details of CtrStkPersIndex_D_Actl

Customer_Color

Indicates Color need to set for ambient lighting.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | ColorKind |

Table: Signal Details of Customer_Color

Customer_Intensity

Indicates the intesity required to set for ambient lighting.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | -- |
|  | Value Range | [ - ] |
|  | Resolution |  |
|  | Unit |  |

Table: Signal Details of Customer_Intensity

DBR_Reset_Cmd

Reset command for latching dual battery relay. This opens the latching relay.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DBR_Reset_CmdKind |

Table: Signal Details of DBR_Reset_Cmd

Delayed_Accessory_Cmd

Input to load shedding - delayed accesory status.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Delayed_Accessory_CmdKind |

Table: Signal Details of Delayed_Accessory_Cmd

DF_Door_Ajar_Status

Driver front door ajar status.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | AjarKind |

Table: Signal Details of DF_Door_Ajar_Status

DF_DoorLock_Status

Driver Front door lock status.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DoorLock_StatusKind |

Table: Signal Details of DF_DoorLock_Status

DiagSession

DiagSession.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DiagSessionKind |

Table: Signal Details of DiagSession

DoorCourtesyLight_Rqst

Request to illuminate the interior courtesy lights.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DoorCourtesyLight_RqstKind |

Table: Signal Details of DoorCourtesyLight_Rqst

DR_Door_Ajar_Status

Driver rear door ajar status.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | AjarKind |

Table: Signal Details of DR_Door_Ajar_Status

DR_DoorLock_Status

Driver rear door lock status.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DoorLock_StatusKind |

Table: Signal Details of DR_DoorLock_Status

FET_Ctrl

Overrides FET output controller when SHORT. OK allows FET output process to drive the FET.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | FET_CtrlKind |

Table: Signal Details of FET_Ctrl

Ignition_Status

Provide vehicle ignition status (OFF/ACC/RUN).

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | IgnitionStatusKind |

Table: Signal Details of Ignition_Status

IlluminatedEntry_Rqst

Request to illuminate the interior courtesy lights.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | IlluminatedEntry_RqstKind |

Table: Signal Details of IlluminatedEntry_Rqst

IlluminatedExit_Rqst

Request to illuminate the interior courtesy lights.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | IlluminatedExit_RqstKind |

Table: Signal Details of IlluminatedExit_Rqst

Intensity

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of Intensity

LG_Other_Door_Ajar_Status

Indicates whether only LG is open or LG along with some other Door is Ajared.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | LG_Other_Door_Ajar_StatusKind |

Table: Signal Details of LG_Other_Door_Ajar_Status

LghtAmbColr_No_Rq

Ambient Lighting Color Selection Request.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LghtAmbColr_No_Rq

LghtAmbDrvMde_B_Stat

Feedback signal for Contextual Ambient Lighting status to HMI ECUs.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LghtAmbDrvMde_B_Stat

LghtAmbDrvMde_D_Rq

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | -- |
|  | Value Range | [ - ] |
|  | Resolution |  |
|  | Unit |  |

Table: Signal Details of LghtAmbDrvMde_D_Rq

LghtAmbIntns_No_Rq

Ambient Lighting Intensity Selection Request.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LghtAmbIntns_No_Rq

LghtAmbRqSrc_B_Stat

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | LghtAmbRqSrc_B_StatKind |

Table: Signal Details of LghtAmbRqSrc_B_Stat

LightAmbColor_No

This number will be derived from CAN inputs from ambient color number depending on whether Enhanced memory exists.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LightAmbColor_No

LightAmbColor_No_Actl

Ambient Lighting Color Selection Status.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LightAmbColor_No_Actl

LightAmbColor_No_Rq

Ambient Lighting Color Selection Request.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LightAmbColor_No_Rq

LightAmbColor_Temp

Ambient color for Lincoln and Ford.

0-ICE_BLUE; 1- ORANGE; 2-SOFT_BLUE; 3-RED; 4-GREEN; 5-BLUE; 6-PURPLE; 7-

LICOLN_WHITE; 8-AMBER; 9-TEAL; 10-BURNT_ORANGE; 11-LINCOLN_BLUE; 12-

LINCOLN_GREEN; 13-LILAC.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | LightAmbColor_TempKind |

Table: Signal Details of LightAmbColor_Temp

LightAmbIntsty_No

This number will be derived from CAN inputs from ambient intensity number depending on whether

Enhanced memory exists.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LightAmbIntsty_No

LightAmbIntsty_No_Actl

Ambient Lighting Intensity  Selection Status.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LightAmbIntsty_No_Actl

LightAmbIntsty_No_Rq

Ambient Lighting Intensity Selection Request.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of LightAmbIntsty_No_Rq

Local_OpMode

initial value changed Awake -> Sleep(Conti.m)

This Dataflow indicates the current state of the ECU.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Local_OpModeKind |

Table: Signal Details of Local_OpMode

Location_ID

Indicates the ALM module NODE ADDRESS placed at various locations.Generally, lighting elements

will be located in the cupholders, center console, footwells, doors, scuffplates.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Location_IDKind |

Table: Signal Details of Location_ID

LowVoltMode

This signal determines if active load will be affected or unaffected by Stop/Start event voltage drop.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | LowVoltModeKind |

Table: Signal Details of LowVoltMode

Parklamps_Command

Command to control the position/parklamps.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Parklamps_CommandKind |

Table: Signal Details of Parklamps_Command

PersNo_D_Actl

Personalization profile number feedback to HMI.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | PersNo_D_ActlKind |

Table: Signal Details of PersNo_D_Actl

PF_Door_Ajar_Status

Passenger Front Door lock status

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | AjarKind |

Table: Signal Details of PF_Door_Ajar_Status

PF_DoorLock_Status

Passember front door lock status.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DoorLock_StatusKind |

Table: Signal Details of PF_DoorLock_Status

PR_Door_Ajar_Status

Passenger rear door ajar status.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | AjarKind |

Table: Signal Details of PR_Door_Ajar_Status

PR_DoorLock_Status

Passenger rear door lock status.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | DoorLock_StatusKind |

Table: Signal Details of PR_DoorLock_Status

Ramp

Indicates ramping is required/not required for ambient lighting.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | RampKind |

Table: Signal Details of Ramp

Reset

BCM reset.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Boolean |

Table: Signal Details of Reset

SelDrvMdeHmi03_D_Rq

Drive mode request communication for the SDM interface HMI -Ambient Lighting.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | byte |

Table: Signal Details of SelDrvMdeHmi03_D_Rq

SelDrvMdeHmi03_D_Rq_ComStat

Communication Status Received over CAN for SelDrvMdeHmi03_D_Rq.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | SelDrvMdeHmi03_D_Rq_ComStatKind |

Table: Signal Details of SelDrvMdeHmi03_D_Rq_ComStat

Update_Color

Indicates whether the color set for ambient lighting needs update or not.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Update_ColorKind |

Table: Signal Details of Update_Color

Update_Intensity

Indicates the intesity for ambient lighting needs to be updated or not.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | Update_IntensityKind |

Table: Signal Details of Update_Intensity

VBattState

Array indicating Current State of a specific Voltage Range.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | VBattStateKind |

Table: Signal Details of VBattState

WelcomeFarewell_Status

Indicates welcome or farewell status for ambient lighting.

| ASIL |  |  |
| --- | --- | --- |
| Value | Encoding Name | WelcomeFarewell_StatusKind |

Table: Signal Details of WelcomeFarewell_Status

#### GSDB Signals

No new signals to be requested for the GSDB.

#### HW I/Os

-ALM_Conditioned_Pwr_Ckt (power circuit).

Name: ALM_Conditioned_Pwr_Ckt

Description: Depending on Configuration of AmbientLighting_Cfg, this ckt will either power the Ambient Light

module, or power the Single color ambient lighting

Type: Numeric

Category: HW Output

Initial Value: 0

DataType: uint8

Storage Class: Volatile

Structure of Data: Scalar

Units: Percent

Resolution: 1

Min Value: 0

Max Value: 100

#### Diagnostic Interfaces

##### DTCs

###### BCM DTCs

Functional DTCs

| DTC | DTC Name | Config_Reqts | Rate (msec) | SelfTest | VBatt Guard | IgnitionStable_5sec | DTC_Ctrl | Inc_Criteria | Inc_Val | Max Action | Dec_Criteria | Dec_Val | Min Action |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 946500 | (Subnet Node count mismatch for ALM LIN_H) Ambient Lighting Bus 1 | AmbientLighting_Cfg <> DISABLED & (DTCSuppression_Cfg[DTCSuppression_Cfg_Index], bit0 = 1) | 1000 | NULL | n/a | n/a | WATCH | ALMCensusStat = DONE & ExpectedALM_NodesCount_Cfg[0] <> Count_Present(ALM_LIN_H_Stat[]) | 64 | DTCMaxAction() | Else | 2 | DTCMinAction() |
| 946700 | (Subnet Node count mismatch for ALM LIN_L) Ambient Lighting Bus 3 | AmbientLighting_Cfg <> DISABLED & (DTCSuppression_Cfg[DTCSuppression_Cfg_Index], bit0 = 1) | 1000 | NULL | n/a | n/a | WATCH | ALMCensusStat = DONE & ExpectedALM_NodesCount_Cfg[2] <> Count_Present(ALM_LIN_L_Stat[]) | 64 | DTCMaxAction() | Else | 2 | DTCMinAction() |
| 945C15 | Ambient Light Module Conditioned Power Supply Output (Circuit Open or Short to Battery ) | BodyHwareExists(ALM_Conditioned_Pwr_Ckt) & ALMConditionedPwrCkt_Cfg = USED & AmbientLighting_Cfg <> SINGLE_COL & DBRInCar_Cfg = FALSE & (DTCSuppression_Cfg[DTCSuppression_Cfg_Index], bit0 = 1) | 1000 | NULL | Note 1 | STABLE | WATCH | Note 2 | 33 | DTCMaxAction() | Note 2 | 13 | DTCMinAction() |
| 945C15 | Ambient Light Module Conditioned Power Supply Output (Circuit Open or Short to Battery ) | BodyHwareExists(ALM_Conditioned_Pwr_Ckt) & ALMConditionedPwrCkt_Cfg = USED & AmbientLighting_Cfg <> SINGLE_COL & DBRInCar_Cfg = FALSE & (DTCSuppression_Cfg[DTCSuppression_Cfg_Index], bit1 = 1) | 200 | TEST | Note 1 | n/a | n/a | Note 2 | 33 | DTCSTest[].MonitorMAX=MAX | Note 2 | 33 | DTCSTest[].MonitorMIN=MIN |
| 945C11 | Ambient Light Module Conditioned Power Supply Output (Circuit Short to Ground) | ActlHwareExists(ALM_Conditioned_Pwr_Ckt) & AmbientLighting_Cfg <> SINGLE_COL & DBRInCar_Cfg = FALSE & (DTCSuppression_Cfg[DTCSuppression_Cfg_Index], bit0 = 1) | 1000 | NULL | n/a | n/a | WATCH | FET_Ctrl[ALM_Conditioned_Pwr_Ckt] = SHORT ¦ FETMode[ALM_Conditioned_Pwr_Ckt] = NO_RETRY | 127 | DTCMaxAction() | FET_Ctrl[ALM_Conditioned_Pwr_Ckt] <> SHORT & FETMode[ALM_Conditioned_Pwr_Ckt] <> NO_RETRY & FETGood[ALM_Conditioned_Pwr_Ckt] = GOOD | 33 | DTCMinAction() |
| 945C11 | Ambient Light Module Conditioned Power Supply Output (Circuit Short to Ground) | ActlHwareExists(ALM_Conditioned_Pwr_Ckt) & AmbientLighting_Cfg <> SINGLE_COL & DBRInCar_Cfg = FALSE & (DTCSuppression_Cfg[DTCSuppression_Cfg_Index], bit1 = 1) | 200 | TEST | n/a | n/a | n/a | FET_Ctrl[ALM_Conditioned_Pwr_Ckt] = SHORT ¦ FETMode[ALM_Conditioned_Pwr_Ckt] = NO_RETRY | 127 | DTCSTest[].MonitorMAX=MAX | FET_Ctrl[ALM_Conditioned_Pwr_Ckt] <> SHORT & FETMode[ALM_Conditioned_Pwr_Ckt] <> NO_RETRY & FETGood[ALM_Conditioned_Pwr_Ckt] = GOOD | 33 | DTCSTest[].MonitorMIN=MIN |
| 953C15 | Color Ambient Light Output (Circuit Open or Short to Battery) | BodyHwareExists(ALM_Conditioned_Pwr_Ckt) & ALMConditionedPwrCkt_Cfg = USED & AmbientLighting_Cfg = SINGLE_COL & DBRInCar_Cfg = FALSE & (DTCSuppression_Cfg[DTCSuppression_Cfg_Index], bit0 = 1) | 1000 | NULL | Note 1 | STABLE | WATCH | Note 2 | 33 | DTCMaxAction() | Note 2 | 13 | DTCMinAction() |
| 953C15 | Color Ambient Light Output (Circuit Open or Short to Battery) | BodyHwareExists(ALM_Conditioned_Pwr_Ckt) & ALMConditionedPwrCkt_Cfg = USED & AmbientLighting_Cfg = SINGLE_COL & DBRInCar_Cfg = FALSE & (DTCSuppression_Cfg[DTCSuppression_Cfg_Index], bit1 = 1) | 200 | TEST | Note 1 | n/a | n/a | Note 2 | 33 | DTCSTest[].MonitorMAX=MAX | Note 2 | 33 | DTCSTest[].MonitorMIN=MIN |
| 953C11 | Color Ambient Light Output (Circuit Short to Ground) | ActlHwareExists(ALM_Conditioned_Pwr_Ckt) & AmbientLighting_Cfg = SINGLE_COL & DBRInCar_Cfg = FALSE & (DTCSuppression_Cfg[DTCSuppression_Cfg_Index], bit0 = 1) | 1000 | NULL | n/a | n/a | WATCH | FET_Ctrl[ALM_Conditioned_Pwr_Ckt] = SHORT ¦ FETMode[ALM_Conditioned_Pwr_Ckt] = NO_RETRY | 127 | DTCMaxAction() | FET_Ctrl[ALM_Conditioned_Pwr_Ckt] <> SHORT & FETMode[ALM_Conditioned_Pwr_Ckt] <> NO_RETRY & FETGood[ALM_Conditioned_Pwr_Ckt] = GOOD | 33 | DTCMinAction() |
| 953C11 | Color Ambient Light Output (Circuit Short to Ground) | ActlHwareExists(ALM_Conditioned_Pwr_Ckt) & AmbientLighting_Cfg = SINGLE_COL & DBRInCar_Cfg = FALSE & (DTCSuppression_Cfg[DTCSuppression_Cfg_Index], bit1 = 1) | 200 | TEST | n/a | n/a | n/a | FET_Ctrl[ALM_Conditioned_Pwr_Ckt] = SHORT ¦ FETMode[ALM_Conditioned_Pwr_Ckt] = NO_RETRY | 127 | DTCSTest[].MonitorMAX=MAX | FET_Ctrl[ALM_Conditioned_Pwr_Ckt] <> SHORT & FETMode[ALM_Conditioned_Pwr_Ckt] <> NO_RETRY & FETGood[ALM_Conditioned_Pwr_Ckt] = GOOD | 33 | DTCSTest[].MonitorMIN=MIN |

Note 2: Fault detection must use the appropriate value of FETOpenThreshold_Cfg[FET_Ctrl_Index] or FETShortThreshold_Cfg[FET_Ctrl_Index] determine the presence of a fault. Also, only increment or decrement the counter if the specific fault is detectable.

##### DIDs

###### BCM DIDs

Functional Feature DIDs List

| DID | DID Name / Description | Config_Reqts | Dataflow | Circuit_Name |
| --- | --- | --- | --- | --- |
| $41DA | Ambient Light LED Power Status | ALMConditionedPwrCkt_Cfg = USED & BodyHwareExists(ALM_Conditioned_Pwr_Ckt) & DBRInCar_Cfg = FALSE & AmbientLighting_Cfg <> SINGLE_COL | PIDReadAlmConditioned_Pwr | ALM_Conditioned_Pwr_Ckt |
| $4226 | Node Count On Ambient Lighting Bus | ALMCensusStat = DONE | Report_LIN_node_Cnt_DID() Dataflow affected is LIN_Node_Count_Subnet[] Refer Error! Reference source not found. Error! Reference source not found. | n/a |
| $42AE | Color Ambient Light Output Status | ALMConditionedPwrCkt_Cfg = USED & BodyHwareExists(ALM_Conditioned_Pwr_Ckt) & DBRInCar_Cfg = FALSE & AmbientLighting_Cfg = SINGLE_COL | PIDReadAlmConditioned_Pwr | ALM_Conditioned_Pwr_Ckt |

Development and General DIDs List

| DID | DID Name / Description | Config_Reqts | Dataflow | DID Type |
| --- | --- | --- | --- | --- |
| $EE75 | ALM Light Engine Present Status Bus H | n/a | Byte 1, Bit 0 - Light Engine 01 Status: ALM_LIN_H_Stat[1] Byte 1, Bit 1 - Light Engine 02 Status : ALM_LIN_H_Stat[2] Byte 1, Bit 2 - Light Engine 03 Status : ALM_LIN_H_Stat[3] Byte 1, Bit 3 - Light Engine 04 Status : ALM_LIN_H_Stat[4] Byte 1, Bit 4 - Light Engine 05 Status : ALM_LIN_H_Stat[5] Byte 1, Bit 5 - Light Engine 06 Status : ALM_LIN_H_Stat[6] Byte 1, Bit 6 - Light Engine 07 Status : ALM_LIN_H_Stat[7] Byte 1, Bit 7 - Light Engine 08 Status : ALM_LIN_H_Stat[8] Byte 2, Bit 0 - Light Engine 09 Status : ALM_LIN_H_Stat[9] Byte 2, Bit 1 - Light Engine 10 Status : ALM_LIN_H_Stat[10] Byte 2, Bit 2 - Light Engine 11 Status : ALM_LIN_H_Stat[11] Byte 2, Bit 3 - Light Engine 12 Status : ALM_LIN_H_Stat[12] Byte 2, Bit 4 - Light Engine 13 Status : ALM_LIN_H_Stat[13] | Bit Mapped |
| $EE76 | ALM Light Engine Present Status Bus L | n/a | Byte 1, Bit 0 - Light Engine 01 Status ALM_LIN_L_Stat[1] Byte 1, Bit 1 - Light Engine 02 Status ALM_LIN_L_Stat[2] Byte 1, Bit 2 - Light Engine 03 Status ALM_LIN_L_Stat[3] Byte 1, Bit 3 - Light Engine 04 Status ALM_LIN_L_Stat[4] Byte 1, Bit 4 - Light Engine 05 Status ALM_LIN_L_Stat[5] Byte 1, Bit 5 - Light Engine 06 Status ALM_LIN_L_Stat[6] Byte 1, Bit 6 - Light Engine 07 Status ALM_LIN_L_Stat[7] Byte 1, Bit 7 - Light Engine 08 Status ALM_LIN_L_Stat[8] Byte 2, Bit 0 - Light Engine 09 Status ALM_LIN_L_Stat[9] Byte 2, Bit 1 - Light Engine 10 Status ALM_LIN_L_Stat[10] Byte 2, Bit 2 - Light Engine 11 Status ALM_LIN_L_Stat[11] Byte 2, Bit 3 - Light Engine 12 Status ALM_LIN_L_Stat[12] Byte 2, Bit 4 - Light Engine 13 Status ALM_LIN_L_Stat[13] | Bit Mapped |
| $EE9D | ALM Bus H Light Engine Fault Status | ALMCensusStat = DONE | Byte 1, Bit 0 - Light Engine 01 Fault Status : ALM_LIN_H_Fault[1] Byte 1, Bit 1 - Light Engine 02 Fault Status : ALM_LIN_H_Fault[2] Byte 1, Bit 2 - Light Engine 03 Fault Status : ALM_LIN_H_Fault[3] Byte 1, Bit 3 - Light Engine 04 Fault Status : ALM_LIN_H_Fault[4] Byte 1, Bit 4 - Light Engine 05 Fault Status : ALM_LIN_H_Fault[5] Byte 1, Bit 5 - Light Engine 06 Fault Status : ALM_LIN_H_Fault[6] Byte 1, Bit 6 - Light Engine 07 Fault Status : ALM_LIN_H_Fault[7] Byte 1, Bit 7 - Light Engine 08 Fault Status : ALM_LIN_H_Fault[8] Byte 2, Bit 0 - Light Engine 09 Fault Status : ALM_LIN_H_Fault[9] Byte 2, Bit 1 - Light Engine 10 Fault Status : ALM_LIN_H_Fault[10] Byte 2, Bit 2 - Light Engine 11 Fault Status : ALM_LIN_H_Fault[11] Byte 2, Bit 3 - Light Engine 12 Fault Status : ALM_LIN_H_Fault[12] Byte 2, Bit 4 - Light Engine 13 Fault Status : ALM_LIN_H_Fault[13] | Bit Mapped |
| $EE9E | ALM Bus L Light Engine Fault Status | ALMCensusStat = DONE | Byte 1, Bit 0 - Light Engine 01 Fault Status : ALM_LIN_L_Fault[1] Byte 1, Bit 1 - Light Engine 02 Fault Status : ALM_LIN_L_Fault[2] Byte 1, Bit 2 - Light Engine 03 Fault Status : ALM_LIN_L_Fault[3] Byte 1, Bit 3 - Light Engine 04 Fault Status : ALM_LIN_L_Fault[4] Byte 1, Bit 4 - Light Engine 05 Fault Status : ALM_LIN_L_Fault[5] Byte 1, Bit 5 - Light Engine 06 Fault Status : ALM_LIN_L_Fault[6] Byte 1, Bit 6 - Light Engine 07 Fault Status : ALM_LIN_L_Fault[7] Byte 1, Bit 7 - Light Engine 08 Fault Status : ALM_LIN_L_Fault[8] Byte 2, Bit 0 - Light Engine 09 Fault Status : ALM_LIN_L_Fault[9] Byte 2, Bit 1 - Light Engine 10 Fault Status : ALM_LIN_L_Fault[10] Byte 2, Bit 2 - Light Engine 11 Fault Status : ALM_LIN_L_Fault[11] Byte 2, Bit 3 - Light Engine 12 Fault Status : ALM_LIN_L_Fault[12] Byte 2, Bit 4 - Light Engine 13 Fault Status : ALM_LIN_L_Fault[13] | Bit Mapped |
| $EE9F | ALM Bus H Light Engine Node Location IDs | ALMCensusStat = DONE | Byte 1 - Light Engine 01 Location ID : ALM_LIN_H_LocID[1] Byte 2 - Light Engine 02 Location ID : ALM_LIN_H_LocID[2] Byte 3 - Light Engine 03 Location ID : ALM_LIN_H_LocID[3] Byte 4 - Light Engine 04 Location ID : ALM_LIN_H_LocID[4] Byte 5 - Light Engine 05 Location ID : ALM_LIN_H_LocID[5] Byte 6 - Light Engine 06 Location ID : ALM_LIN_H_LocID[6] Byte 7 - Light Engine 07 Location ID : ALM_LIN_H_LocID[7] Byte 8 - Light Engine 08 Location ID : ALM_LIN_H_LocID[8] Byte 9 - Light Engine 09 Location ID : ALM_LIN_H_LocID[9] Byte 10 - Light Engine 10 Location ID : ALM_LIN_H_LocID[10] Byte 11 - Light Engine 11 Location ID : ALM_LIN_H_LocID[11] Byte 12 - Light Engine 12 Location ID : ALM_LIN_H_LocID[12] Byte 13 - Light Engine 13 Location ID : ALM_LIN_H_LocID[13] | Bit Mapped |
| $EEA0 | ALM Bus L Light Engine Node Location IDs | ALMCensusStat = DONE | Byte 1 - Light Engine 01 Location ID : ALM_LIN_L_LocID[1] Byte 2 - Light Engine 02 Location ID : ALM_LIN_L_LocID[2] Byte 3 - Light Engine 03 Location ID : ALM_LIN_L_LocID[3] Byte 4 - Light Engine 04 Location ID : ALM_LIN_L_LocID[4] Byte 5 - Light Engine 05 Location ID : ALM_LIN_L_LocID[5] Byte 6 - Light Engine 06 Location ID : ALM_LIN_L_LocID[6] Byte 7 - Light Engine 07 Location ID : ALM_LIN_L_LocID[7] Byte 8 - Light Engine 08 Location ID : ALM_LIN_L_LocID[8] Byte 9 - Light Engine 09 Location ID : ALM_LIN_L_LocID[9] Byte 10 - Light Engine 10 Location ID : ALM_LIN_L_LocID[10] Byte 11 - Light Engine 11 Location ID : ALM_LIN_L_LocID[11] Byte 12 - Light Engine 12 Location ID : ALM_LIN_L_LocID[12] Byte 13 - Light Engine 13 Location ID : ALM_LIN_L_LocID[13] | Bit Mapped |
| $EEA1 | ALM Bus H Light Engine Node Status Response | ALMCensusStat = DONE | Byte 1 - Light Engine 01 Status Response : ALM_LIN_H_StatResp[1] Byte 2 - Light Engine 02 Status Response : ALM_LIN_H_StatResp[2] Byte 3 - Light Engine 03 Status Response : ALM_LIN_H_StatResp[3] Byte 4 - Light Engine 04 Status Response : ALM_LIN_H_StatResp[4] Byte 5 - Light Engine 05 Status Response : ALM_LIN_H_StatResp[5] Byte 6 - Light Engine 06 Status Response : ALM_LIN_H_StatResp[6] Byte 7 - Light Engine 07 Status Response : ALM_LIN_H_StatResp[7] Byte 8 - Light Engine 08 Status Response : ALM_LIN_H_StatResp[8] Byte 9 - Light Engine 09 Status Response : ALM_LIN_H_StatResp[9] Byte 10 - Light Engine 10 Status Response : ALM_LIN_H_StatResp[10] Byte 11 - Light Engine 11 Status Response : ALM_LIN_H_StatResp[11] Byte 12 - Light Engine 12 Status Response : ALM_LIN_H_StatResp[12] Byte 13 - Light Engine 13 Status Response : ALM_LIN_H_StatResp[13] | Bit Mapped |
| $EEA2 | ALM Bus L Light Engine Node Status Response | ALMCensusStat = DONE | Byte 1 - Light Engine 01 Status Response : ALM_LIN_L_StatResp[1] Byte 2 - Light Engine 02 Status Response : ALM_LIN_L_StatResp[2] Byte 3 - Light Engine 03 Status Response : ALM_LIN_L_StatResp[3] Byte 4 - Light Engine 04 Status Response : ALM_LIN_L_StatResp[4] Byte 5 - Light Engine 05 Status Response : ALM_LIN_L_StatResp[5] Byte 6 - Light Engine 06 Status Response : ALM_LIN_L_StatResp[6] Byte 7 - Light Engine 07 Status Response : ALM_LIN_L_StatResp[7] Byte 8 - Light Engine 08 Status Response : ALM_LIN_L_StatResp[8] Byte 9 - Light Engine 09 Status Response : ALM_LIN_L_StatResp[9] Byte 10 - Light Engine 10 Status Response : ALM_LIN_L_StatResp[10] Byte 11 - Light Engine 11 Status Response : ALM_LIN_L_StatResp[11] Byte 12 - Light Engine 12 Status Response : ALM_LIN_L_StatResp[12] Byte 13 - Light Engine 13 Status Response : ALM_LIN_L_StatResp[13] | Bit Mapped |

### Technical Parameters

AjarColor_Cfg

Ajar color level when vehicle door is ajar for ambient lighting.

Type: Numeric

Category: ECU Internal

Initial Value: 1

DataType: uint8

Storage Class: Non-Volatile -- Factory Set Method 2 Estimated NVRAM Write 1

Config Param is: Application Dependent

Structure of Data: Scalar

Units: None

Resolution: 1

Min Value: 0

Max Value: 15

ALM_CommUp_Time_Cfg

Configuration parameter specifying the time requried for ambient light node to wake up.

Type: Numeric

Category: ECU Internal

Initial Value: 120

DataType: uint8

Storage Class: Non-Volatile -- Factory Set Method 3

Config Param is: Core Strategy Dependent (BCM Software Team)

Structure of Data: Scalar

Units: Milliseconds

Resolution: 1

Min Value: 0

Max Value: 200

ALMPowerMode_Cfg

The method 2 signal will decide one of two approaches, One approach is to keep ALM conditioned

power dependent on headlamp status, other approach is to keep ALM conditioned power independent

of headlamp status, while making it independent of headlamp status it is controlled on the basis of BCM.

Type: Discrete

Category: ECU Internal

Initial Value: HEADLAMP_CONTROLLED

DataType: uint8

Storage Class: Non-Volatile -- Factory Set Method 2

Estimated NVRAM Write 1

Config Param is: Application Dependent

Structure of Data: Scalar

BCM_IO_ACTIVE

HEADLAMP_CONTROLLED

AmbDrvMdeColor_Cfg

This configuration parameter will define the color table lookup for the drive mode colors.

The color defined in each entry is a HMI color index value in the range 1 to 7.

The color index 0 will present a fault that will result in the contextual ambient to operate in Manual

behavior.

Type: Numeric

Category: ECU Internal

Initial Value: {1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1}

DataType: uint8

Storage Class: Non-Volatile -- Factory Set Method 3

Config Param is: Application Dependent

Structure of Data: 1D array

Index1D: SelDrvMdeHmi03_Index

Units: Counts

Resolution: 1

Min Value: 0

Max Value: 7

Ambient_Lighting_ON_Cfg

Method 3 configuration to decide reserved value of ambient lighting intensity,received over CAN, which

will say that Ambient Lighting should be turned ON to its previous intensity. It's value should not be

changed unless the SYNC module changes this value for this purpose.

Initial Value: 0x66

Units: N/A

Resolution: 1

Min Value: 0x00

Max Value: 0xFF

AmbientDelay_Cfg

Configuration Parameter which indicates the minimum time delay before entering to the ambient state

from welcome state, to avoid the ambient light turning OFF due to momentary low voltage during crank

when battery voltage is very low.

Type: Numeric

Category: ECU Internal

Initial Value: 500

DataType: uint16

Storage Class: Non-Volatile -- Factory Set Method 2

Estimated NVRAM Write 1

Config Param is: Application Dependent

Structure of Data: Scalar

Units: Milliseconds

Resolution: 1

Min Value: 0

Max Value: 2500

AmbientLighting_Cfg

Configuration used to enable/disable the ambient and welcome & farewell behavior.

Type: Discrete

Category: ECU Internal

Initial Value: WF_AND_AMB

DataType: uint8

Storage Class: Non-Volatile -- Factory Set Method 2

Estimated NVRAM Write 1

Config Param is: Application Dependent

Structure of Data: Scalar

AMB_ONLY            Only Ambient is enabled

DISABLED              Both Ambient and Welcome/farewell is disabled

SINGLE_COL          Single Color Ambient Lighting feature is enabled

WF_AND_AMB      Both Welcome/farewell and Ambient is enabled

WF_ONLY Only     Welcome/farewell is enabled

ContextAL_Cfg

This configuration parameter will define if the Contextual Ambient Lighting is enabled or disabled.

Type: Discrete

Category: ECU Internal

Initial Value: DISABLE

DataType: uint8

Storage Class: Non-Volatile -- Factory Set Method 2

Estimated NVRAM Write 1

Config Param is: Application Dependent

Structure of Data: Scalar

DISABLE    Disabled contextual ambient lighting

ENABLE      Enabled contextual ambient lighting

DBRInCar_Cfg

"Master Switch" EOL Config to be set True if CCP2-relay is present and to be used.

Type: Discrete

Category: ECU Internal

Initial Value: FALSE

DataType: uint8

Storage Class: Non-Volatile -- Factory Set Method 2

Estimated NVRAM Write 1

Config Param is: Application Dependent

Structure of Data: Scalar

FALSE

TRUE

DoorLock_Indication_Cfg

Leave at initial value of NULL. This is for Ambient Lighting to indicate lock status via illumination of

the interior door pull handle. This feature is no longer supported. It is replaced by the trim panel lock

indicator LED feature (Section 2.2.18).

Type: Discrete

Category: ECU Internal

Initial Value: NULL

DataType: uint8

Storage Class: Non-Volatile -- Factory Set Method 2 Estimated NVRAM Write 1

Config Param is: Core Strategy Dependent (BCM Software Team)

Structure of Data: Scalar

COMMON           Indicates vehicle lock status on all doors

INDIVIDUAL      Indicates individual door lock status

NULL                   No indication of vehicle door lock status

EM_Exists_Cfg

The method 2 configuration parameter to decide whether Enhanced Memory configuration exists for

Ambient Lighting,

If this is NOT_PRESENT then no personalization of ambient lighting will be provided.

Type: Discrete

Category: ECU Internal

Initial Value: PRESENT

DataType: uint8

Storage Class: Non-Volatile -- Factory Set Method 2

Estimated NVRAM Write 10000

Config Param is: Application Dependent

Structure of Data: Scalar

NOT_PRESENT

PRESENT

EntryColor_Cfg

Entry color level for ambient lighting.

Type: Numeric

Category: ECU Internal

Initial Value: 1

DataType: uint8

Storage Class: Non-Volatile -- Factory Set Method 2

Estimated NVRAM Write 1

Config Param is: Application Dependent

Structure of Data: Scalar

Units: None

Resolution: 1

Min Value: 0

Max Value: 15

EntryInt_Cfg

Entry level intensity for ambient lighting.

Type: Numeric

Category: ECU Internal

Initial Value: 15

DataType: uint8

Storage Class: Non-Volatile -- Factory Set Method 2 Estimated NVRAM Write 1

Config Param is: Application Dependent

Structure of Data: Scalar

Units: None

Resolution: 1

Min Value: 0

Max Value: 15

HMI_Comm_Var_Cfg

This configuration parameter will select the Communication variant used in HMI to BCM Ambient

Lighting Interface. Variant1 is the original carryover protocol.

Variant2 was added to support enhanced memory and supported for contextual ambient lighting; this

configuration can also be used to support standard ambient lighting.

The module providing the customer interface (currently SYNC) and the BCM must use the same variant

settings for proper operation of any of Ambient Lighting features.

Type: Discrete

Category: ECU Internal

Initial Value: VARIANT1

Storage Class: Non-Volatile -- Factory Set Method 2

Estimated NVRAM Write 1

Config Param is: Application Dependent

Structure of Data: Scalar

VARIANT1     Variant1 HMI communication

VARIANT2     Variant2 HMI communication

Lincoln_Experience_Cfg

The configuration parameter is to differentiate the Welcome/Farewell Lighting (Interior and Exterior)

behaviour between Lincoln Experience vehicles and Non Lincoln vehicles.

When Lincoln_Experience_Cfg is set to LINCOLN_EXPERIENCE1 then method 2 configuration

parameters LR_Stop_Pos_Lamp_Ckt_Usage_Cfg and RR_Stop_Pos_Lamp_Ckt_Usage_Cfg shall be

always set to STOP_TURN, similarly method 3 configuration parameter

Freq_Cfg[Freq_Sel_Cfg][Park_Lamps_Body_Ckt] shall be always set to DC.

Type: Discrete

Category: ECU Internal

Initial Value: NULL

DataType: uint8

Storage Class: Non-Volatile -- Factory Set Method 2 Estimated NVRAM Write 1

Config Param is: Application Dependent

Structure of Data: Scalar

LINCOLN_EXPERIENCE1 Lincoln Experience Behaviour1

NULL                                  Non Lincoln Vehicle

Run_Active_Count_Max_Cfg

The dataflow decides the max range of a count which is used in Static Sequential by Light Engines.

Type: Numeric

Category: ECU Internal

Initial Value: 255

DataType: uint8

Storage Class: Constant

Structure of Data: Scalar

Units: Counts

Resolution: 1

Min Value: 0

Max Value: 255

Run_Active_Count_Min_Cfg

The dataflow decides the starting range of a count which is used for Static Sequential in Lighting Engines.

Type: Numeric

Category: ECU Internal

Initial Value: 0

DataType: uint8

Storage Class: Constant

Structure of Data: Scalar

Units: Counts

Resolution: 1

Min Value: 0

Max Value: 50

Stat_Seq_Time_Cfg

This method 2 data decides the increment frequency of counter,The counter is transmitted over LIN.

Type: Numeric

Category: ECU Internal

Initial Value: 100

DataType: uint8

Storage Class: Non-Volatile -- Factory Set Method 2 Estimated NVRAM Write 1

Config Param is: Application Dependent

Structure of Data: Scalar

Units: Milliseconds

Resolution: 1

Min Value: 0

Max Value: 250

Stat_Seq_Wait_Time_Cfg

The dataflow is used to decide the rate at which the value of signal Run_Active_Count is written in

protected RAM once the value of is stabilized to either min or max.

Type: Numeric

Category: ECU Internal

Initial Value: 5000

DataType: uint16

Storage Class: Constant

Structure of Data: Scalar

Units: Milliseconds

Resolution: 1

Min Value: 100

Max Value: 7000

### Mappings

ALM_Power_Status - ALM_Power_Status

LightAmbColor_Temp - LightAmbColor_Temp

LghtAmbColr_No_Rq - LghtAmbColr_No_Rq

LghtAmbDrvMde_B_Stat - LghtAmbDrvMde_B_Stat

PR_DoorLock_Status - PR_DoorLock_Status

IlluminatedExit_Rqst - IlluminatedExit_Rqst

Ambient_Status - Ambient_Status

PF_DoorLock_Status - PF_DoorLock_Status

Local_OpMode - Local_OpMode

AmbientColor_Cfg - AmbientColor_Cfg

PR_Door_Ajar_Status - PR_Door_Ajar_Status

LghtAmbIntns_No_Rq - LghtAmbIntns_No_Rq

ALM_ConditionedPower - ALM_Conditioned_Pwr_Ckt

DispOpr_CS - CtrStkDsplyOp_D_Rq

DF_DoorLock_Status - DF_DoorLock_Status

LghtAmbDrvMde_D_Rq - LghtAmbDrvMde_D_Rq

Update_Intensity - Update_Intensity

ALM_Conditioned_Pwr_Arb - ALM_Conditioned_Pwr_Arb

SelDrvMdeHmi03_Rq - SelDrvMdeHmi03_D_Rq

DBR_Reset_Cmd - DBR_Reset_Cmd

PersIndex_CS - CtrStkPersIndex_D_Actl

Customer_Color - Customer_Color

DoorAjar - PF_Door_Ajar_Status

DoorAjar - DR_Door_Ajar_Status

DoorAjar - PR_Door_Ajar_Status

DoorAjar - DF_Door_Ajar_Status

PF_Door_Ajar_Status - PF_Door_Ajar_Status

AmbientDim_Cfg - AmbientDim_Cfg

WelcomeFarewell_Status - WelcomeFarewell_Status

Delayed_Accessory_Cmd - Delayed_Accessory_Cmd

DR_Door_Ajar_Status - DR_Door_Ajar_Status

PersNum - PersNo_D_Actl

Update_Color - Update_Color

LightAmbColor_No - LightAmbColor_No

LowVoltMode - LowVoltMode

LightAmbIntsty_No_Rq - LightAmbIntsty_No_Rq

AmbientMsg_Rqst - AmbientMsg_Rqst

CourtesyDemand_BSave_Rqst - CourtesyDemand_BSave_Rqst

DoorCourtesyLight_Rqst - DoorCourtesyLight_Rqst

DF_Door_Ajar_Status - DF_Door_Ajar_Status

LightAmbColor_No_Actl - LightAmbColor_No_Actl

VBattState - VBattState

Color - Color

DoorLocked - DR_DoorLock_Status

DoorLocked - DF_DoorLock_Status

DoorLocked - PR_DoorLock_Status

DoorLocked - PF_DoorLock_Status

Approach_LightShow - Approach_LightShow

DiagSession - DiagSession

LightAmbIntsty_No - LightAmbIntsty_No

Parklamps_Command - Parklamps_Command

DR_DoorLock_Status - DR_DoorLock_Status

ALM_Location_ID - Location_ID

Reset - Reset

Intensity - Intensity

LG_Other_Door_Ajar_Status - LG_Other_Door_Ajar_Status

LightAmbColor_No_Rq - LightAmbColor_No_Rq

CourtesyDelay_Rqst - CourtesyDelay_Rqst

All_DoorLock_Status - All_DoorLock_Status

SelDrvMdeHmi03_D_Rq_ComStat - SelDrvMdeHmi03_D_Rq_ComStat

FET_Ctrl - FET_Ctrl

IlluminatedEntry_Rqst - IlluminatedEntry_Rqst

Ramp - Ramp

Customer_Intensity - Customer_Intensity

LightAmbIntsty_No_Actl - LightAmbIntsty_No_Actl

Ignition_Status - Ignition_Status

### Technical Interfaces

Not supported by MagicDraw report generation.

#### AIS Interfaces

##### Publisher Interfaces

##### Subscriber Interfaces

#### AUTOSAR Ports

Not supported by MagicDraw report generation.

### Messages/APIs

#### CAN Bus “<Bus Name>”

1.FNV3

FD1 CAN:

| Msg Id | Description | Detailed Meaning | State Encoded | Transmitters | Signal Receivers | Send Type |
| --- | --- | --- | --- | --- | --- | --- |
| 0x215 | APIM_Send_Signals_2_FD1 |  |  | GWM | ABS BCM ECM ECM_DIESEL ECM_HEV GENERIC IPMA_ADAS_FD1 PCM PCM_HEV SOBDMC_HPCM VECTOR__XXX | NoMsgSend Type |
|  | LghtAmbDrvMde_D_Rq |  |  | GWM | BCM | No Send Type |
|  |  | Null | 0x0 |  |  |  |
|  |  | Manual | 0x1 |  |  |  |
|  |  | Automatic | 0x2 |  |  |  |
|  |  | NotUsed | 0x3 |  |  |  |
| 0x2A5 | Rear_Climate_Data3_FD1 |  |  | GWM | BCM VECTOR__XXX | NoMsgSend Type |
|  | LghtAmbColr_No_Rq |  |  | GWM | BCM | No Send Type |
|  | LghtAmbIntns_No_Rq |  |  | GWM | BCM | No Send Type |
| 0x3BA | Body_Info_10 |  |  | BCM | ABS BCMD_IPDB GENERIC GWM HCM_FD1 IPMA_ADAS_FD1 ODMCA PCM PCM_HEV RCM_FD1 TCM_DSL | Event Periodic |
|  | LghtAmbDrvMde_B_Stat |  |  | BCM | GWM | OnChange |
|  |  | Manual | 0x0 |  |  |  |
|  |  | Automatic | 0x1 |  |  |  |
| 0x3DA | Personality_APIM_Data3_FD1 |  |  | GWM | BCM VDM VECTOR__XXX | NoMsgSend Type |
|  | LightAmbColor_No_Rq |  |  | GWM | BCM | No Send Type |
|  | LightAmbIntsty_No_Rq |  |  | GWM | BCM | No Send Type |
|  |  | Off | 0x0 |  |  |  |
|  | LghtAmbRqSrc_B_Stat |  |  | GWM | BCM | No Send Type |
|  |  | Manual | 0x0 |  |  |  |
|  |  | Automatic | 0x1 |  |  |  |
| 0x3E2 | Personality_APIM_Data_FD1 |  |  | GWM | ABS BCM ECM_HEV GENERIC HCM_FD1 IPMA_ADAS_FD1 PCM SOBDMC_HPCM VDM | NoMsgSend Type |
|  | CtrStkPersIndex_D_Actl |  |  | GWM | ABS BCM ECM_HEV HCM_FD1 IPMA_ADAS_FD1 SOBDMC_HPCM VDM | No Send Type |
|  |  | PERS_1 | 0x0 |  |  |  |
|  |  | PERS_2 | 0x1 |  |  |  |
|  |  | PERS_3 | 0x2 |  |  |  |
|  |  | PERS_4 | 0x3 |  |  |  |
|  |  | Vehicle | 0x4 |  |  |  |
|  |  | Unused_1 | 0x5 |  |  |  |
|  |  | Unused_2 | 0x6 |  |  |  |
|  |  | Unused_3 | 0x7 |  |  |  |
|  | CtrStkDsplyOp_D_Rq |  |  | GWM | ABS BCM ECM_HEV HCM_FD1 IPMA_ADAS_FD1 PCM SOBDMC_HPCM VDM | No Send Type |
|  |  | Null | 0x0 |  |  |  |
|  |  | Query | 0x1 |  |  |  |
|  |  | Set | 0x2 |  |  |  |
|  |  | Upload | 0x3 |  |  |  |
|  |  | Restore | 0x4 |  |  |  |
|  |  | Copy | 0x5 |  |  |  |
|  |  | Unused_1 | 0x6 |  |  |  |
|  |  | Unused_2 | 0x7 |  |  |  |
| 0x3E3 | Personality_BCM_Data |  |  | BCM | GWM | Event Periodic |
|  | LightAmbColor_No_Actl |  |  | BCM | GWM | OnChange |
|  | LightAmbIntsty_No_Actl |  |  | BCM | GWM | OnChange |
|  |  | Off | 0x0 |  |  |  |
| 0x3EB | Personality_BCM2_Data |  |  | BCM | ABS ECM_HEV GWM HCM_FD1 IPMA_ADAS_FD1 SOBDMC_HPCM VDM | Event Periodic |
|  | PersNo_D_Actl |  |  | BCM | ABS ECM_HEV GWM HCM_FD1 IPMA_ADAS_FD1 SOBDMC_HPCM VDM | OnChange |
|  |  | PERS_1 | 0x0 |  |  |  |
|  |  | PERS_2 | 0x1 |  |  |  |
|  |  | PERS_3 | 0x2 |  |  |  |
|  |  | PERS_4 | 0x3 |  |  |  |
|  |  | Vehicle | 0x4 |  |  |  |
|  |  | NotDetermined | 0x5 |  |  |  |
|  |  | Unused_2 | 0x6 |  |  |  |
|  |  | Unused_3 | 0x7 |  |  |  |
| 0x420 | SelectDriveModeData |  |  | ABS | AWD_DLCM BCM ECM ECM_DIESEL ECM_HEV GWM IPMA_ADAS_FD1 PCM PCM_HEV SOBDMC_HPCM TCCM | Event Periodic |
|  | SelDrvMdeHmi03_D_Rq |  |  | ABS | BCM GWM | OnChange |
|  |  | SelDrvMde01 | 0x0 |  |  |  |
|  |  | SelDrvMde02 | 0x1 |  |  |  |
|  |  | SelDrvMde03 | 0x2 |  |  |  |
|  |  | SelDrvMde04 | 0x3 |  |  |  |
|  |  | SelDrvMde05 | 0x4 |  |  |  |
|  |  | SelDrvMde06 | 0x5 |  |  |  |
|  |  | SelDrvMde07 | 0x6 |  |  |  |
|  |  | SelDrvMde08 | 0x7 |  |  |  |
|  |  | SelDrvMde09 | 0x8 |  |  |  |
|  |  | SelDrvMde10 | 0x9 |  |  |  |
|  |  | SelDrvMde11 | 0xA |  |  |  |
|  |  | SelDrvMde12 | 0xB |  |  |  |
|  |  | SelDrvMde13 | 0xC |  |  |  |
|  |  | SelDrvMde14 | 0xD |  |  |  |
|  |  | SelDrvMde15 | 0xE |  |  |  |
|  |  | SelDrvMde16 | 0xF |  |  |  |
|  |  | SelDrvMde17 | 0x10 |  |  |  |
|  |  | SelDrvMde18 | 0x11 |  |  |  |
|  |  | SelDrvMde19 | 0x12 |  |  |  |
|  |  | SelDrvMde20 | 0x13 |  |  |  |
|  |  | SelDrvMde21 | 0x14 |  |  |  |
|  |  | SelDrvMde22 | 0x15 |  |  |  |
|  |  | SelDrvMde23 | 0x16 |  |  |  |
|  |  | SelDrvMde24 | 0x17 |  |  |  |
|  |  | SelDrvMde25 | 0x18 |  |  |  |
|  |  | SelDrvMde26 | 0x19 |  |  |  |
|  |  | SelDrvMde27 | 0x1A |  |  |  |
|  |  | SelDrvMde28 | 0x1B |  |  |  |
|  |  | SelDrvMde29 | 0x1C |  |  |  |
|  |  | SelDrvMde30 | 0x1D |  |  |  |
|  |  | SelDrvMde31 | 0x1E |  |  |  |
|  |  | Faulty | 0x1F |  |  |  |

HS3 CAN:

| Msg Id | Description | Detailed Meaning | State Encoded | Transmitters | Signal Receivers | Send Type |
| --- | --- | --- | --- | --- | --- | --- |
| 0x215 | APIM_Send_Signals_2 |  |  | APIM APIM_CDC APIM_CIM APIM_CISM | GWM TRM_ITRM | Event Periodic |
|  | LghtAmbDrvMde_D_Rq |  |  | APIM APIM_CDC APIM_CIM APIM_CISM | GWM | OnChange |
|  |  | Null | 0x0 |  |  |  |
|  |  | Manual | 0x1 |  |  |  |
|  |  | Automatic | 0x2 |  |  |  |
|  |  | NotUsed | 0x3 |  |  |  |
| 0x2A5 | Rear_Climate_Data3 |  |  | RACM_RCIM | GWM | Event Periodic |
|  | LghtAmbColr_No_Rq |  |  | RACM_RCIM | GWM | OnChange |
|  | LghtAmbIntns_No_Rq |  |  | RACM_RCIM | GWM | OnChange |
| 0x3BA | Body_Info_10_HS3 |  |  | GWM | APIM APIM_CDC APIM_CIM APIM_CISM GENERIC IPC RACM_RCIM TRM_ITRM VECTOR__XXX | NoMsgSend Type |
|  | LghtAmbDrvMde_B_Stat |  |  | GWM | APIM APIM_CDC APIM_CIM APIM_CISM | No Send Type |
|  |  | Manual | 0x0 |  |  |  |
|  |  | Automatic | 0x1 |  |  |  |
| 0x3DA | Personality_APIM_Data3 |  |  | APIM APIM_CDC APIM_CIM APIM_CISM | GWM RACM_RCIM | Event Periodic |
|  | LightAmbColor_No_Rq |  |  | APIM APIM_CDC APIM_CIM APIM_CISM | GWM | OnChange |
|  | LightAmbIntsty_No_Rq |  |  | APIM APIM_CDC APIM_CIM APIM_CISM | GWM | OnChange |
|  |  | Off | 0x0 |  |  |  |
|  | LghtAmbRqSrc_B_Stat |  |  | APIM APIM_CDC APIM_CIM APIM_CISM | GWM RACM_RCIM | OnChange |
|  |  | Manual | 0x0 |  |  |  |
|  |  | Automatic | 0x1 |  |  |  |
| 0x3E2 | Personality_APIM_Data |  |  | APIM APIM_CDC APIM_CIM APIM_CISM | ACM DSP GWM IPC | Event Periodic |
|  | CtrStkPersIndex_D_Actl |  |  | APIM APIM_CDC APIM_CIM APIM_CISM | ACM DSP GWM IPC | OnChange |
|  |  | PERS_1 | 0x0 |  |  |  |
|  |  | PERS_2 | 0x1 |  |  |  |
|  |  | PERS_3 | 0x2 |  |  |  |
|  |  | PERS_4 | 0x3 |  |  |  |
|  |  | Vehicle | 0x4 |  |  |  |
|  |  | Unused_1 | 0x5 |  |  |  |
|  |  | Unused_2 | 0x6 |  |  |  |
|  |  | Unused_3 | 0x7 |  |  |  |
|  | CtrStkDsplyOp_D_Rq |  |  | APIM APIM_CDC APIM_CIM APIM_CISM | ACM DSP GWM IPC | OnChange |
|  |  | Null | 0x0 |  |  |  |
|  |  | Query | 0x1 |  |  |  |
|  |  | Set | 0x2 |  |  |  |
|  |  | Upload | 0x3 |  |  |  |
|  |  | Restore | 0x4 |  |  |  |
|  |  | Copy | 0x5 |  |  |  |
|  |  | Unused_1 | 0x6 |  |  |  |
|  |  | Unused_2 | 0x7 |  |  |  |
| 0x3E3 | Personality_BCM_Data_HS3 |  |  | GWM | APIM APIM_CDC APIM_CIM APIM_CISM IPC | NoMsgSend Type |
|  | LightAmbColor_No_Actl |  |  | GWM | APIM APIM_CDC APIM_CIM APIM_CISM | No Send Type |
|  | LightAmbIntsty_No_Actl |  |  | GWM | APIM APIM_CDC APIM_CIM APIM_CISM | No Send Type |
|  |  | Off | 0x0 |  |  |  |
| 0x3EC | Personality_BCM2_Data_HS3 |  |  | GWM | ACM APIM APIM_CDC APIM_CIM APIM_CISM DSP HUD_ADV IPC VECTOR__XXX | NoMsgSend Type |
|  | PersNo_D_Actl |  |  | GWM | ACM APIM APIM_CDC APIM_CIM APIM_CISM DSP IPC | No Send Type |
|  |  | PERS_1 | 0x0 |  |  |  |
|  |  | PERS_2 | 0x1 |  |  |  |
|  |  | PERS_3 | 0x2 |  |  |  |
|  |  | PERS_4 | 0x3 |  |  |  |
|  |  | Vehicle | 0x4 |  |  |  |
|  |  | NotDetermined | 0x5 |  |  |  |
|  |  | Unused_2 | 0x6 |  |  |  |
|  |  | Unused_3 | 0x7 |  |  |  |
| 0x420 | SelectDriveModeData_HS3 |  |  | GWM | ACM APIM APIM_CDC APIM_CIM APIM_CISM GENERIC HUD_ADV IPC VECTOR__XXX | NoMsgSend Type |
|  | SelDrvMdeHmi03_D_Rq |  |  | GWM | VECTOR__XXX | No Send Type |
|  |  | SelDrvMde01 | 0x0 |  |  |  |
|  |  | SelDrvMde02 | 0x1 |  |  |  |
|  |  | SelDrvMde03 | 0x2 |  |  |  |
|  |  | SelDrvMde04 | 0x3 |  |  |  |
|  |  | SelDrvMde05 | 0x4 |  |  |  |
|  |  | SelDrvMde06 | 0x5 |  |  |  |
|  |  | SelDrvMde07 | 0x6 |  |  |  |
|  |  | SelDrvMde08 | 0x7 |  |  |  |
|  |  | SelDrvMde09 | 0x8 |  |  |  |
|  |  | SelDrvMde10 | 0x9 |  |  |  |
|  |  | SelDrvMde11 | 0xA |  |  |  |
|  |  | SelDrvMde12 | 0xB |  |  |  |
|  |  | SelDrvMde13 | 0xC |  |  |  |
|  |  | SelDrvMde14 | 0xD |  |  |  |
|  |  | SelDrvMde15 | 0xE |  |  |  |
|  |  | SelDrvMde16 | 0xF |  |  |  |
|  |  | SelDrvMde17 | 0x10 |  |  |  |
|  |  | SelDrvMde18 | 0x11 |  |  |  |
|  |  | SelDrvMde19 | 0x12 |  |  |  |
|  |  | SelDrvMde20 | 0x13 |  |  |  |
|  |  | SelDrvMde21 | 0x14 |  |  |  |
|  |  | SelDrvMde22 | 0x15 |  |  |  |
|  |  | SelDrvMde23 | 0x16 |  |  |  |
|  |  | SelDrvMde24 | 0x17 |  |  |  |
|  |  | SelDrvMde25 | 0x18 |  |  |  |
|  |  | SelDrvMde26 | 0x19 |  |  |  |
|  |  | SelDrvMde27 | 0x1A |  |  |  |
|  |  | SelDrvMde28 | 0x1B |  |  |  |
|  |  | SelDrvMde29 | 0x1C |  |  |  |
|  |  | SelDrvMde30 | 0x1D |  |  |  |
|  |  | SelDrvMde31 | 0x1E |  |  |  |
|  |  | Faulty | 0x1F |  |  |  |

2.FNV2

HS1 CAN:

| Msg Id | Description | Detailed Meaning | State Encoded | Transmitters | Signal Receivers | Send Type |
| --- | --- | --- | --- | --- | --- | --- |
| 0x215 | APIM_Send_Signals_2_HS1 |  |  | GWM | BCM GENERIC VECTOR__XXX | NoMsgSend Type |
|  | LghtAmbDrvMde_D_Rq |  |  | GWM | BCM | No Send Type |
|  |  | Null | 0x0 |  |  |  |
|  |  | Manual | 0x1 |  |  |  |
|  |  | Automatic | 0x2 |  |  |  |
|  |  | NotUsed | 0x3 |  |  |  |
| 0x3BA | Body_Info_10 |  |  | BCM | GWM | Event Periodic |
|  | LghtAmbDrvMde_B_Stat |  |  | BCM | GWM | OnChange |
|  |  | Manual | 0x0 |  |  |  |
|  |  | Automatic | 0x1 |  |  |  |
| 0x3DA | Personality_APIM_Data3_HS1 |  |  | GWM | BCM GENERIC PACM | NoMsgSend Type |
|  | LightAmbColor_No_Rq |  |  | GWM | BCM | No Send Type |
|  | LightAmbIntsty_No_Rq |  |  | GWM | BCM | No Send Type |
|  |  | Off | 0x0 |  |  |  |
| 0x3E2 | Personality_APIM_Data_HS1 |  |  | GWM | BCM GENERIC | NoMsgSend Type |
|  | CtrStkPersIndex_D_Actl |  |  | GWM | BCM | No Send Type |
|  |  | PERS_1 | 0x0 |  |  |  |
|  |  | PERS_2 | 0x1 |  |  |  |
|  |  | PERS_3 | 0x2 |  |  |  |
|  |  | PERS_4 | 0x3 |  |  |  |
|  |  | Vehicle | 0x4 |  |  |  |
|  |  | Unused_1 | 0x5 |  |  |  |
|  |  | Unused_2 | 0x6 |  |  |  |
|  |  | Unused_3 | 0x7 |  |  |  |
|  | CtrStkDsplyOp_D_Rq |  |  | GWM | BCM | No Send Type |
|  |  | Null | 0x0 |  |  |  |
|  |  | Query | 0x1 |  |  |  |
|  |  | Set | 0x2 |  |  |  |
|  |  | Upload | 0x3 |  |  |  |
|  |  | Restore | 0x4 |  |  |  |
|  |  | Copy | 0x5 |  |  |  |
|  |  | Unused_1 | 0x6 |  |  |  |
|  |  | Unused_2 | 0x7 |  |  |  |
| 0x3E3 | Personality_BCM_Data |  |  | BCM | GWM | Event Periodic |
|  | LightAmbColor_No_Actl |  |  | BCM | GWM | OnChange |
|  | LightAmbIntsty_No_Actl |  |  | BCM | GWM | OnChange |
|  |  | Off | 0x0 |  |  |  |
| 0x3EB | Personality_BCM2_Data |  |  | BCM | GWM | Event Periodic |
|  | PersNo_D_Actl |  |  | BCM | GWM | OnChange |
|  |  | PERS_1 | 0x0 |  |  |  |
|  |  | PERS_2 | 0x1 |  |  |  |
|  |  | PERS_3 | 0x2 |  |  |  |
|  |  | PERS_4 | 0x3 |  |  |  |
|  |  | Vehicle | 0x4 |  |  |  |
|  |  | NotDetermined | 0x5 |  |  |  |
|  |  | Unused_2 | 0x6 |  |  |  |
|  |  | Unused_3 | 0x7 |  |  |  |
| 0x41A | GatewayData8_HS1 |  |  | GWM | BCM GENERIC VECTOR__XXX | Event Periodic |
|  | SelDrvMdeHmi03_D_Rq |  |  | GWM | BCM | OnChange |
|  |  | SelDrvMde01 | 0x0 |  |  |  |
|  |  | SelDrvMde02 | 0x1 |  |  |  |
|  |  | SelDrvMde03 | 0x2 |  |  |  |
|  |  | SelDrvMde04 | 0x3 |  |  |  |
|  |  | SelDrvMde05 | 0x4 |  |  |  |
|  |  | SelDrvMde06 | 0x5 |  |  |  |
|  |  | SelDrvMde07 | 0x6 |  |  |  |
|  |  | SelDrvMde08 | 0x7 |  |  |  |
|  |  | SelDrvMde09 | 0x8 |  |  |  |
|  |  | SelDrvMde10 | 0x9 |  |  |  |
|  |  | SelDrvMde11 | 0xA |  |  |  |
|  |  | SelDrvMde12 | 0xB |  |  |  |
|  |  | SelDrvMde13 | 0xC |  |  |  |
|  |  | SelDrvMde14 | 0xD |  |  |  |
|  |  | SelDrvMde15 | 0xE |  |  |  |
|  |  | SelDrvMde16 | 0xF |  |  |  |
|  |  | SelDrvMde17 | 0x10 |  |  |  |
|  |  | SelDrvMde18 | 0x11 |  |  |  |
|  |  | SelDrvMde19 | 0x12 |  |  |  |
|  |  | SelDrvMde20 | 0x13 |  |  |  |
|  |  | SelDrvMde21 | 0x14 |  |  |  |
|  |  | SelDrvMde22 | 0x15 |  |  |  |
|  |  | SelDrvMde23 | 0x16 |  |  |  |
|  |  | SelDrvMde24 | 0x17 |  |  |  |
|  |  | SelDrvMde25 | 0x18 |  |  |  |
|  |  | SelDrvMde26 | 0x19 |  |  |  |
|  |  | SelDrvMde27 | 0x1A |  |  |  |
|  |  | SelDrvMde28 | 0x1B |  |  |  |
|  |  | SelDrvMde29 | 0x1C |  |  |  |
|  |  | SelDrvMde30 | 0x1D |  |  |  |
|  |  | SelDrvMde31 | 0x1E |  |  |  |
|  |  | Faulty | 0x1F |  |  |  |

HS3 CAN:

| Msg Id | Description | Detailed Meaning | State Encoded | Transmitters | Signal Receivers | Send Type |
| --- | --- | --- | --- | --- | --- | --- |
| 0x215 | APIM_Send_Signals_2 |  |  | APIM APIM_CIM APIM_CISM | GWM TRM_ITRM | Event Periodic |
|  | LghtAmbDrvMde_D_Rq |  |  | APIM APIM_CIM APIM_CISM | GWM | OnChange |
|  |  | Null | 0x0 |  |  |  |
|  |  | Manual | 0x1 |  |  |  |
|  |  | Automatic | 0x2 |  |  |  |
|  |  | NotUsed | 0x3 |  |  |  |
| 0x3BA | Body_Info_10_HS3 |  |  | GWM | APIM APIM_CIM APIM_CISM GENERIC GWM IPC RACM_RCIM | NoMsgSend Type |
|  | LghtAmbDrvMde_B_Stat |  |  | GWM | APIM APIM_CIM APIM_CISM | No Send Type |
|  |  | Manual | 0x0 |  |  |  |
|  |  | Automatic | 0x1 |  |  |  |
| 0x3DA | Personality_APIM_Data3 |  |  | APIM APIM_CIM APIM_CISM | GWM | Event Periodic |
|  | LightAmbColor_No_Rq |  |  | APIM APIM_CIM APIM_CISM | GWM | OnChange |
|  | LightAmbIntsty_No_Rq |  |  | APIM APIM_CIM APIM_CISM | GWM | OnChange |
|  |  | Off | 0x0 |  |  |  |
| 0x3E2 | Personality_APIM_Data |  |  | APIM APIM_CIM APIM_CISM | ACM DSP GWM IPC | Event Periodic |
|  | CtrStkPersIndex_D_Actl |  |  | APIM APIM_CIM APIM_CISM | ACM DSP GWM IPC | OnChange |
|  |  | PERS_1 | 0x0 |  |  |  |
|  |  | PERS_2 | 0x1 |  |  |  |
|  |  | PERS_3 | 0x2 |  |  |  |
|  |  | PERS_4 | 0x3 |  |  |  |
|  |  | Vehicle | 0x4 |  |  |  |
|  |  | Unused_1 | 0x5 |  |  |  |
|  |  | Unused_2 | 0x6 |  |  |  |
|  |  | Unused_3 | 0x7 |  |  |  |
|  | CtrStkDsplyOp_D_Rq |  |  | APIM APIM_CIM APIM_CISM | ACM DSP GWM IPC | OnChange |
|  |  | Null | 0x0 |  |  |  |
|  |  | Query | 0x1 |  |  |  |
|  |  | Set | 0x2 |  |  |  |
|  |  | Upload | 0x3 |  |  |  |
|  |  | Restore | 0x4 |  |  |  |
|  |  | Copy | 0x5 |  |  |  |
|  |  | Unused_1 | 0x6 |  |  |  |
|  |  | Unused_2 | 0x7 |  |  |  |
| 0x3E3 | Personality_BCM_Data_HS3 |  |  | GWM | APIM APIM_CIM APIM_CISM IPC | NoMsgSend Type |
|  | LightAmbColor_No_Actl |  |  | GWM | APIM APIM_CIM APIM_CISM | No Send Type |
|  | LightAmbIntsty_No_Actl |  |  | GWM | APIM APIM_CIM APIM_CISM | No Send Type |
|  |  | Off | 0x0 |  |  |  |
| 0x3EC | Personality_BCM2_Data_HS3 |  |  | GWM | ACM APIM APIM_CIM APIM_CISM DSP GENERIC HUD_ADV IPC | NoMsgSend Type |
|  | PersNo_D_Actl |  |  | GWM | ACM APIM APIM_CIM APIM_CISM DSP IPC | No Send Type |
|  |  | PERS_1 | 0x0 |  |  |  |
|  |  | PERS_2 | 0x1 |  |  |  |
|  |  | PERS_3 | 0x2 |  |  |  |
|  |  | PERS_4 | 0x3 |  |  |  |
|  |  | Vehicle | 0x4 |  |  |  |
|  |  | NotDetermined | 0x5 |  |  |  |
|  |  | Unused_2 | 0x6 |  |  |  |
|  |  | Unused_3 | 0x7 |  |  |  |
| 0x420 | SelectDriveModeData_HS3 |  |  | GWM | ACM APIM APIM_CIM APIM_CISM DACMC GENERIC HUD_ADV IPC | NoMsgSend Type |
|  | SelDrvMdeHmi03_D_Rq |  |  | GWM | GENERIC | No Send Type |
|  |  | SelDrvMde01 | 0x0 |  |  |  |
|  |  | SelDrvMde02 | 0x1 |  |  |  |
|  |  | SelDrvMde03 | 0x2 |  |  |  |
|  |  | SelDrvMde04 | 0x3 |  |  |  |
|  |  | SelDrvMde05 | 0x4 |  |  |  |
|  |  | SelDrvMde06 | 0x5 |  |  |  |
|  |  | SelDrvMde07 | 0x6 |  |  |  |
|  |  | SelDrvMde08 | 0x7 |  |  |  |
|  |  | SelDrvMde09 | 0x8 |  |  |  |
|  |  | SelDrvMde10 | 0x9 |  |  |  |
|  |  | SelDrvMde11 | 0xA |  |  |  |
|  |  | SelDrvMde12 | 0xB |  |  |  |
|  |  | SelDrvMde13 | 0xC |  |  |  |
|  |  | SelDrvMde14 | 0xD |  |  |  |
|  |  | SelDrvMde15 | 0xE |  |  |  |
|  |  | SelDrvMde16 | 0xF |  |  |  |
|  |  | SelDrvMde17 | 0x10 |  |  |  |
|  |  | SelDrvMde18 | 0x11 |  |  |  |
|  |  | SelDrvMde19 | 0x12 |  |  |  |
|  |  | SelDrvMde20 | 0x13 |  |  |  |
|  |  | SelDrvMde21 | 0x14 |  |  |  |
|  |  | SelDrvMde22 | 0x15 |  |  |  |
|  |  | SelDrvMde23 | 0x16 |  |  |  |
|  |  | SelDrvMde24 | 0x17 |  |  |  |
|  |  | SelDrvMde25 | 0x18 |  |  |  |
|  |  | SelDrvMde26 | 0x19 |  |  |  |
|  |  | SelDrvMde27 | 0x1A |  |  |  |
|  |  | SelDrvMde28 | 0x1B |  |  |  |
|  |  | SelDrvMde29 | 0x1C |  |  |  |
|  |  | SelDrvMde30 | 0x1D |  |  |  |
|  |  | SelDrvMde31 | 0x1E |  |  |  |
|  |  | Faulty | 0x1F |  |  |  |

FD1 CAN:

| Msg Id | Description | Detailed Meaning | State Encoded | Transmitters | Signal Receivers | Send Type |
| --- | --- | --- | --- | --- | --- | --- |
| 0x215 | APIM_Send_Signals_2_FD1 |  |  | GWM | ABS_ESC ECM ECM_DIESEL ECM_HEV ECM_HEVDSL GENERIC IPMA_ADAS PCM PCM_HEV PSCM SOBDMC_HPCM | NoMsgSend Type |
|  | LghtAmbDrvMde_D_Rq |  |  | GWM | GENERIC | No Send Type |
|  |  | Null | 0x0 |  |  |  |
|  |  | Manual | 0x1 |  |  |  |
|  |  | Automatic | 0x2 |  |  |  |
|  |  | NotUsed | 0x3 |  |  |  |
| 0x3BA | Body_Info_10_FD1 |  |  | GWM | ABS_ESC GENERIC HCM IPMA_ADAS PCM PCM_HEV SIMA_DLCS TCM_DSL VECTOR__XXX | NoMsgSend Type |
|  | LghtAmbDrvMde_B_Stat |  |  | GWM | GENERIC | No Send Type |
|  |  | Manual | 0x0 |  |  |  |
|  |  | Automatic | 0x1 |  |  |  |
| 0x3DA | Personality_APIM_Data3_FD1 |  |  | GWM | GENERIC VDM | NoMsgSend Type |
|  | LightAmbColor_No_Rq |  |  | GWM | GENERIC | No Send Type |
|  | LightAmbIntsty_No_Rq |  |  | GWM | GENERIC | No Send Type |
|  |  | Off | 0x0 |  |  |  |
| 0x3E2 | Personality_APIM_Data_FD1 |  |  | GWM | ABS_ESC ECM_HEV GENERIC HCM IPMA_ADAS PCM SOBDMC_HPCM VDM | NoMsgSend Type |
|  | CtrStkPersIndex_D_Actl |  |  | GWM | ABS_ESC ECM_HEV HCM IPMA_ADAS SOBDMC_HPCM VDM | No Send Type |
|  |  | PERS_1 | 0x0 |  |  |  |
|  |  | PERS_2 | 0x1 |  |  |  |
|  |  | PERS_3 | 0x2 |  |  |  |
|  |  | PERS_4 | 0x3 |  |  |  |
|  |  | Vehicle | 0x4 |  |  |  |
|  |  | Unused_1 | 0x5 |  |  |  |
|  |  | Unused_2 | 0x6 |  |  |  |
|  |  | Unused_3 | 0x7 |  |  |  |
|  | CtrStkDsplyOp_D_Rq |  |  | GWM | ABS_ESC ECM_HEV HCM IPMA_ADAS PCM SOBDMC_HPCM VDM | No Send Type |
|  |  | Null | 0x0 |  |  |  |
|  |  | Query | 0x1 |  |  |  |
|  |  | Set | 0x2 |  |  |  |
|  |  | Upload | 0x3 |  |  |  |
|  |  | Restore | 0x4 |  |  |  |
|  |  | Copy | 0x5 |  |  |  |
|  |  | Unused_1 | 0x6 |  |  |  |
|  |  | Unused_2 | 0x7 |  |  |  |
| 0x3EB | Personality_BCM2_Data_FD1 |  |  | GWM | ABS_ESC ECM_HEV GENERIC HCM IPMA_ADAS SOBDMC_HPCM VDM | NoMsgSend Type |
|  | PersNo_D_Actl |  |  | GWM | ABS_ESC ECM_HEV HCM IPMA_ADAS SOBDMC_HPCM VDM | No Send Type |
|  |  | PERS_1 | 0x0 |  |  |  |
|  |  | PERS_2 | 0x1 |  |  |  |
|  |  | PERS_3 | 0x2 |  |  |  |
|  |  | PERS_4 | 0x3 |  |  |  |
|  |  | Vehicle | 0x4 |  |  |  |
|  |  | NotDetermined | 0x5 |  |  |  |
|  |  | Unused_2 | 0x6 |  |  |  |
|  |  | Unused_3 | 0x7 |  |  |  |
| 0x420 | SelectDriveModeData |  |  | ABS_ESC | AWD_DLCM ECM ECM_DIESEL ECM_HEV ECM_HEVDSL GWM IPMA_ADAS PCM PCM_HEV SOBDMC_HPCM TCCM | Event Periodic |
|  | SelDrvMdeHmi03_D_Rq |  |  | ABS_ESC | GWM | OnChange |
|  |  | SelDrvMde01 | 0x0 |  |  |  |
|  |  | SelDrvMde02 | 0x1 |  |  |  |
|  |  | SelDrvMde03 | 0x2 |  |  |  |
|  |  | SelDrvMde04 | 0x3 |  |  |  |
|  |  | SelDrvMde05 | 0x4 |  |  |  |
|  |  | SelDrvMde06 | 0x5 |  |  |  |
|  |  | SelDrvMde07 | 0x6 |  |  |  |
|  |  | SelDrvMde08 | 0x7 |  |  |  |
|  |  | SelDrvMde09 | 0x8 |  |  |  |
|  |  | SelDrvMde10 | 0x9 |  |  |  |
|  |  | SelDrvMde11 | 0xA |  |  |  |
|  |  | SelDrvMde12 | 0xB |  |  |  |
|  |  | SelDrvMde13 | 0xC |  |  |  |
|  |  | SelDrvMde14 | 0xD |  |  |  |
|  |  | SelDrvMde15 | 0xE |  |  |  |
|  |  | SelDrvMde16 | 0xF |  |  |  |
|  |  | SelDrvMde17 | 0x10 |  |  |  |
|  |  | SelDrvMde18 | 0x11 |  |  |  |
|  |  | SelDrvMde19 | 0x12 |  |  |  |
|  |  | SelDrvMde20 | 0x13 |  |  |  |
|  |  | SelDrvMde21 | 0x14 |  |  |  |
|  |  | SelDrvMde22 | 0x15 |  |  |  |
|  |  | SelDrvMde23 | 0x16 |  |  |  |
|  |  | SelDrvMde24 | 0x17 |  |  |  |
|  |  | SelDrvMde25 | 0x18 |  |  |  |
|  |  | SelDrvMde26 | 0x19 |  |  |  |
|  |  | SelDrvMde27 | 0x1A |  |  |  |
|  |  | SelDrvMde28 | 0x1B |  |  |  |
|  |  | SelDrvMde29 | 0x1C |  |  |  |
|  |  | SelDrvMde30 | 0x1D |  |  |  |
|  |  | SelDrvMde31 | 0x1E |  |  |  |
|  |  | Faulty | 0x1F |  |  |  |

#### LIN Bus “<Bus Name>”

Please refer to the LDF attached on 5.3.1.2 for further details.

LIN_H:

Frames {

BCM_H_P00 : 0x00, BCM, 3     {

Location_ID,              	0;

Ramp,                     	8;

Color,                    	10;

Update_Color,             	14;

Intensity,                	16;

Update_Intensity,         	22;

}

BCM_H_P04 : 0x37, BCM, 8    {

Customer_Color,                	0;

Customer_Intensity,           	4;

Ignition_Status,               		8;

DF_Door_Ajar_Status,           	12;

PF_Door_Ajar_Status,           	13;

DR_Door_Ajar_Status,           	14;

PR_Door_Ajar_Status,           	15;

LG_Ajar_Status,                		16;

IllumEntry_LockOccurred,       	17;

Day_Night_Status,              	19;

Approach_LightShow,            	21;

ALM_WelcomeFarewell_State,     	22;

ALM_WelcomeFarewell_Substate, 24;

Run_Active_Count,              	32;

Static_Seq_Cfg,                		40;

Headlight_Status,              		41;

ALM_Conditioned_Pwr_Ckt,       	44;

AmbientLighting_Cfg,           	45;

Park_Status,                   		48;

Veh_Speed_Avail,               	49;

NotConfigured_Cmd,             	50;

Alarm_Lights_Courtesy_Cmd,     	52;

Crash_Courtesy_Rqst,           	54;

SilentMode_Status,             	55;

Veh_Speed,                     		56;

}

LIN_L:

Frames {

BCM_L_P00 : 0x00, BCM, 3     {

Location_ID,              	0;

Ramp,                     	8;

Color,                    	10;

Update_Color,             	14;

Intensity,                	16;

Update_Intensity,        	22;

}

BCM_L_P04 : 0x37, BCM, 8     {

Customer_Color,               	 	0;

Customer_Intensity,            	4;

Ignition_Status,               		8;

DF_Door_Ajar_Status,           	12;

PF_Door_Ajar_Status,           	13;

DR_Door_Ajar_Status,           	14;

PR_Door_Ajar_Status,           	15;

LG_Ajar_Status,                		16;

IllumEntry_LockOccurred,       	17;

Day_Night_Status,              	19;

Approach_LightShow,            	21;

ALM_WelcomeFarewell_State,     	22;

ALM_WelcomeFarewell_Substate, 24;

Run_Active_Count,              	32;

Static_Seq_Cfg,                		40;

Headlight_Status,              		41;

ALM_Conditioned_Pwr_Ckt,       	44;

AmbientLighting_Cfg,          	45;

Park_Status,                   		48;

Veh_Speed_Avail,               	49;

NotConfigured_Cmd,             	50;

Alarm_Lights_Courtesy_Cmd,     	52;

Crash_Courtesy_Rqst,           	54;

SilentMode_Status,             	55;

Veh_Speed,                     		56;

}

#### AUTOSAR Interfaces

Not supported by MagicDraw report generation.

#### SOA Service Contracts

Not supported by MagicDraw report generation.

### Encoding Types

#### Logical Encoding Types

AjarKind

Indicates individual door ajar status.

CLOSED Door is closed

AJAR   Door is ajar

| Value Range | [CLOSED, AJAR, ] |
| --- | --- |
| Resolution |  |
| Units |  |

ALM_Conditioned_Pwr_ArbKind

Ambient Lighting Module conditioned power supply.

ACTIVE

INACTIVE

| Value Range | [INACTIVE, ACTIVE, ] |
| --- | --- |
| Resolution |  |
| Units |  |

ALM_Location_IDKind

Indicates light location.

ALL                                        All ambient lights

CUPHOLDERS_ALL             Cupholders location

DF_DOOR                             Front Door location

DF_DOOR_HANDLE            Front Door handle location

DF_DOOR_LIGHTBAR        Front driver door Light bar

DR_DOOR Rear                   Door location

DR_DOOR_HANDLE           Rear Door handle location

DR_DOOR_LIGHTBAR       Rear driver door Light bar

FOOTWELLS                         Footwell location

IDLE                                       Address out of specified nodes

PF_DOOR                              Front pessanger door location

PF_DOOR_HANDLE             Front pessanger door handle location

PF_DOOR_LIGHTBAR         Front pessanger door Light bar

PR_DOOR                              Rear pessanger door location

PR_DOOR_HANDLE            Rear pessanger door handle location

PR_DOOR_LIGHTBAR        Rear pessanger door Light bar

STORAGEBIN_LIGHTBAR Storage bin Light bar

| Value Range | [ALL, CUPHOLDERS_ALL, DF_DOOR, DF_DOOR_HANDLE, DF_DOOR_LIGHTBAR, DR_DOOR, DR_DOOR_HANDLE, DR_DOOR_LIGHTBAR, FOOTWELLS, IDLE, PF_DOOR, PF_DOOR_HANDLE, PF_DOOR_LIGHTBAR, PR_DOOR, PR_DOOR_HANDLE, PR_DOOR_LIGHTBAR, STORAGEBIN_LIGHTBAR, ] |
| --- | --- |
| Resolution |  |
| Units |  |

ALM_Power_StatusKind

Converts numeric output of ALM_Conditioned_Pwr_Ckt to discrete to send it over LIN.

ACTIVE      ALM_Conditioned_Pwr_Ckt is greater than zero

INACTIVE ALM_Conditioned_Pwr_Ckt is equal to zero

| Value Range | [INACTIVE, ACTIVE, ] |
| --- | --- |
| Resolution |  |
| Units |  |

Ambient_StatusKind

Indicates Ambient Lighting status .

AMBIENT Ambient Lighting is running

OFF            Ambient Lighting is not running

| Value Range | [OFF, AMBIENT, ] |
| --- | --- |
| Resolution |  |
| Units |  |

Approach_LightShowKind

Indicates when to start approach light show.

SHOW_OFF        Approach Light Show should be OFF

SHOW_ON         Approach Light Show should be ON

| Value Range | [SHOW_OFF, SHOW_ON, ] |
| --- | --- |
| Resolution |  |
| Units |  |

byte

| Value Range | [ ] |
| --- | --- |
| Resolution |  |
| Units |  |

CourtesyDelay_RqstKind

| Value Range | [OFF, ON, ] |
| --- | --- |
| Resolution |  |
| Units |  |

CourtesyDemand_BSave_RqstKind

Indicates request from Battery Saver to turn OFF courtesy / demand lights

NO_EFFECT Normal operation

OFF               Turn off

| Value Range | [NO_EFFECT, OFF, ] |
| --- | --- |
| Resolution |  |
| Units |  |

DBR_Reset_CmdKind

Reset command for latching dual battery relay (opens latching relay).

OFF

ON

| Value Range | [OFF, ON, ] |
| --- | --- |
| Resolution |  |
| Units |  |

Delayed_Accessory_CmdKind

Input to load shedding - delayed accessory status.

OFF                   the delayed accessory power is unavailable

ON                    the delayed accessory power is available

| Value Range | [OFF, ON, ] |
| --- | --- |
| Resolution |  |
| Units |  |

DiagSessionKind

Indicates active diagnostic session.

DEFAULT

EXTEND

PROGRAM

UNKNOWN

| Value Range | [DEFAULT, EXTEND, PROGRAM, UNKNOWN, ] |
| --- | --- |
| Resolution |  |
| Units |  |

DispOpr_CSKind

Request from Center Stack for personalization.

COPY              Copies the active configuration for all features for requested personality

NULL              No command requested

QUERY           Query and return value for requested feature and personality

RESTORE       Restore initial default value from FeatLst_to_SPDJB_Map[.] array to requested personality

SET                  Set feature to requested value for requested personality

UPLOAD         Copy setting to user profile indicated, but do not make active.

| Value Range | [COPY, NULL, RESTORE, SET, UPLOAD, ] |
| --- | --- |
| Resolution |  |
| Units |  |

DoorCourtesyLight_RqstKind

Request to illuminate interior with courtesy lights.

OFF   Turn off the courtesy lights

ON    Turn on the courtesy lights

| Value Range | [OFF, ON, ] |
| --- | --- |
| Resolution |  |
| Units |  |

DoorLock_StatusKind

| Value Range | [0x0-LOCK, 0x1-NULL, 0x2-UNLOCK, ] |
| --- | --- |
| Resolution |  |
| Units |  |

FET_CtrlKind

Indicates FET output.

OK NORM        allows FET output process to drive the FET

SHORT             Overrides FET output controller

| Value Range | [OK, SHORT, ] |
| --- | --- |
| Resolution |  |
| Units |  |

IlluminatedEntry_RqstKind

Request to illuminate interior with courtesy lights.

OFF Turn off the courtesy lights

ON  Turn on the courtesy lights

| Value Range | [OFF, ON, ] |
| --- | --- |
| Resolution |  |
| Units |  |

IlluminatedExit_RqstKind

Request to illuminate the interior courtesy lights.

OFF Turn off the courtesy lights

ON  Turn on the courtesy lights

| Value Range | [OFF, ON, ] |
| --- | --- |
| Resolution |  |
| Units |  |

LG_Other_Door_Ajar_StatusKind

Indicates whether only LG is open or LG along with some other door is Ajar.

CLOSED                 Lift gate and all doors closed

LG_AJAR               Lift gate is ajar and all doors closed

OTHER_AJAR       At least one door open and lift gate is either open or closed

| Value Range | [CLOSED, LG_AJAR, OTHER_AJAR, ] |
| --- | --- |
| Resolution |  |
| Units |  |

LghtAmbDrvMde_B_StatKind

Feedback signal to update HMI.

AUTOMATIC  Auto Ambient Lighting

MANUAL        Auto Ambient Lighting

| Value Range | [MANUAL, AUTOMATIC, ] |
| --- | --- |
| Resolution |  |
| Units |  |

LghtAmbDrvMde_D_RqKind

Sets CAL request from HMI is Manual or Automatic.

AUTOMATIC      Auto Ambient Lighting ON

MANUAL            Auto Ambient Lighting OFF

NULL                   No Change

| Value Range | [NULL, MANUAL, AUTOMATIC, ] |
| --- | --- |
| Resolution |  |
| Units |  |

LightLocationKind

| Value Range | [AJAR, AMBIENT, DF_DOOR, NULL, OFF, RAMP_OFF, WELCOME, WELCOME_LE, ] |
| --- | --- |
| Resolution |  |
| Units |  |

Local_OpModeKind

Indicates ECU current state.

CAN_SLEEP        Can sleep

LO_MHZ             Microcontroller in a low frequency clock mode

PERIPS_ACTIVE Peripherals active

SLEEP                  Microprocessor and network is asleep (low power mode). Some of

the features are still active

| Value Range | [0x0-AWAKE, 0x1-CAN_SLEEP, 0x2-LOW_MHZ, 0x3-PERIPS_ACTIVE, 0x4-SLEEP, ] |
| --- | --- |
| Resolution |  |
| Units |  |

LockKind

Indicates individual door lock status.

LOCK           Door is locked

NULL           No feedback from door Latch

UNLOCK     Door is unlocked

| Value Range | [LOCK, NULL, UNLOCK, ] |
| --- | --- |
| Resolution |  |
| Units |  |

LowVoltModeKind

Determines if active load will be affected or unaffected by Stop / Start event voltage drop.

NORM           The active load will be affected by Stop/Start event voltage drop.

SUPPRESS    The active load will be unaffected by Stop/Start event voltage drop.

| Value Range | [NORM, SUPRESS, ] |
| --- | --- |
| Resolution |  |
| Units |  |

Parklamps_CommandKind

Park lamps command.

OFF                 turn off all position/parklamps

ON                  turn on all position/parklamps

| Value Range | [OFF, ON, ] |
| --- | --- |
| Resolution |  |
| Units |  |

PersIndex_CSKind

Personality index to use sent from Center Statck.

PERS_1       Personality 1

PERS_2       Personality 2

PERS_3       Personality 3

PERS_4       Personality 4

VEHICLE     Vehicle level

| Value Range | [PERS_1, PERS_2, PERS_3, PERS_4, VEHICLE, ] |
| --- | --- |
| Resolution |  |
| Units |  |

PersNumKind

Indicates which personality number is currently selected.

PERS_1   Reporting user 1 settings

PERS_2   Reporting user 1 settings

PERS_3   Reporting user 1 settings

PERS_4   Reporting user 1 settings

VEHICLE Reporting vehicle level settings

| Value Range | [PERS_1, PERS_2, PERS_3, PERS_4, VEHICLE, ] |
| --- | --- |
| Resolution |  |
| Units |  |

RampKind

Request for ramping behavior.

NO                Ramping is not required

YES               Ramping is required

| Value Range | [NO, YES, ] |
| --- | --- |
| Resolution |  |
| Units |  |

ResetKind

| Value Range | [FALSE, TRUE, ] |
| --- | --- |
| Resolution |  |
| Units |  |

SelDrvMdeHmi03_D_Rq_ComStatKind

Indicates if CAN signal SelDrvMdeHmi03_D_Rq is received.

MISSING Signal not received

PRESENT Signal received

| Value Range | [MISSING, PRESENT, ] |
| --- | --- |
| Resolution |  |
| Units |  |

Update_ColorKind

Indicates whether color set of Ambient Lighting needs update.

NO                Do not update color

YES               Update color

| Value Range | [NO, YES, ] |
| --- | --- |
| Resolution |  |
| Units |  |

Update_IntensityKind

Indicates whether intensity of Ambient Lighting needs update.

NO                 Do not udate Intensity

YES                Update Intensity

| Value Range | [NO, YES, ] |
| --- | --- |
| Resolution |  |
| Units |  |

VBattStateKind

| Value Range | [0x0-HI_V, 0x1-LO_V, 0x2-NORM_V, 0x3-OVER_V, 0x4-UNDER_V, ] |
| --- | --- |
| Resolution |  |
| Units |  |

WelcomeFarewell_StatusKind

Indicates Welcome Farewell status calculated by Ambient Lighting.

OFF                                     welcome/farewell status is off

WELCOME1        welcome/farewell status is welcome

WELCOME1_EXIT   welcome/farewell status during exit

WELCOME2        welcome/farewell status is farewell

WELCOME3        welcome/farewell status during pproach

| Value Range | [OFF, WELCOME1, WELCOME1_EXIT, WELCOME2, WELCOME3, WELCOME_LE, ] |
| --- | --- |
| Resolution |  |
| Units |  |

#### Technology Encoding Types

valuetype

DoorLock_StatusKind

| Value Range | 0x0-LOCK 0x1-NULL 0x2-UNLOCK |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

IgnitionStatusKind

| Value Range | 0x0-UNKNOWN 0x1-OFF 0x2-ACCESSORY 0x4-RUN 0x8-START 0x15-INVALID |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

Update_IntensityKind

| Value Range | 0x0-NO 0x1YES |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

Delayed_Accessory_CmdKind

| Value Range | 0x0-OFF 0x1-ON |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

CourtesyDelay_RqstKind

| Value Range | 0x0-OFF 0x1-ON |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

LG_Other_Door_Ajar_StatusKind

| Value Range | 0x0-CLOSED 0x1-LG_AJAR 0x2-OTHER_AJAR |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

Local_OpModeKind

| Value Range | 0x0-AWAKE 0x1-CAN_SLEEP 0x2-PERIPS_ACTIVE 0x3-SLEEP |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

WelcomeFarewell_StatusKind

| Value Range | 0x0-OFF 0x1-WELCOME1 0x2-WELCOME1_EXIT 0x3-WELCOME2 0x4-WELCOME3 |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

ALM_Conditioned_Pwr_ArbKind

| Value Range | 0x0-INACTIVE 0x1-ACTIVE |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

PersNo_D_ActlKind

| Value Range | 0x0-PERS_1 0x1-PERS_2 0x2-PERS_3 0x3-PERS_4 0x4-VEHICLE |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

VBattStateKind

| Value Range | 0x0-HI_V 0x1-LO_V 0x2-NORM_V 0x3-OVER_V 0x4-UNDER_V |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

DoorCourtesyLight_RqstKind

| Value Range | 0x0-OFF 0x1-ON |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

AmbientMsg_RqstKind

| Value Range | 0x0-AJAR 0x1-AMBIENT 0x2-NULL 0x3-OFF 0x4-RAMP_OFF 0x5-WELCOME 0x6-WELCOME_LE |
| --- | --- |
| Resolution |  |
| Units |  |

primitivetype

Boolean

Boolean is an instance of PrimitiveType. In the metamodel, Boolean defines an enumeration that denotes a logical condition.

Its enumeration literals are:

• true - The Boolean condition is satisfied.

• false - The Boolean condition is not satisfied.

It is used for Boolean attribute and Boolean expressions in the metamodel, such as OCL expression.

| Value Range |  |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

voltage

| Value Range |  |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

FET_CtrlKind

| Value Range | 0x0-OK 0x1-SHORT |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

ALM_Power_StatusKind

| Value Range | 0x0-INACTIVE 0x1-ACTIVE |
| --- | --- |
| Resolution |  |
| Units |  |

datatype

byte

| Value Range |  |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

IlluminatedExit_RqstKind

| Value Range | 0x0-OFF 0x1-ON |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

Approach_LightShowKind

| Value Range | 0x0-SHOW_OFF 0x1-SHOW_ON |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

DispOpr_CSKind

| Value Range | 0x0-COPY 0x1-NULL 0x2-RESTORE 0x3-SET 0x4-UPLOAD |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

DBR_Reset_CmdKind

| Value Range | 0x0-OFF 0x1-ON |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

Location_IDKind

| Value Range | 0x00-All 0x10-Cupholder_All 0x11-Cupholder_Loc1 0x12-Cupholder_Loc2 0x13-Cupholder_Loc3 0x14-Cupholder_Loc4 0x20-Footwells_All 0x21-LeftFrontFooltWells 0x22-RightFrontFootWells 0x23-LeftRearFoolWells 0x24-RightRearFootWells 0x30-StorageBinTrinketTray_All 0x31-StorageBinTrinketTray_StorageBin 0x32-StorageBinTrinketTray_TrinketTray 0x33-PassThrough 0x40-Doors_All 0x50-LeftFrontDoor_All 0x51-LeftFrontDoor_ReleaseHandle 0x52-LeftFrontDoor_MapPocket 0x53-LeftFrontDoor_Loc3 0x54-LeftFrontDoor_Loc4 0x60-RightFrontDoor_All 0x61-RightFrontDoor_ReleaseHandle 0x62-RightFrontDoor_MapPocket 0x63-RightFrontDoor_Loc3 0x64-RightFrontDoor_Loc4 0x70-LeftRearDoor_All 0x71-LeftRearDoor_ReleaseHandle 0x72-LeftRearDoor_MapPocket 0x73-LeftRearDoor_Loc3 0x74-LeftRearDoor_Loc4 0x80-RightRearDoor_All 0x81-RightRearDoor_ReleaseHandle 0x82-RightRearDoor_MapPocket 0x83-RightRearDoor_Loc3 0x84-RightRearDoor_Loc4 0x90-OverHead_All 0x91-OverHead_Loc1 0x92-OverHead_Loc2 0x93-OverHead_Loc3 0x94-OverHead_Loc4 0x95-OverHead_Loc5 0x96-OverHead_Loc6 0x97-OverHead_Loc7 0x98-OverHead_Loc8 0x99-OverHead_Loc9 0xA0-Scuff_All 0xA1-Scuff_Loc1 0xA2-Scuff_Loc2 0xA3-Scuff_Loc3 0xA4-Scuff_Loc4 0xA5-Scuff_Loc5 0xB0-IPLightBar_All 0xB1-IPLightBar_Loc1 0xB2-IPLightBar_Loc2 0xB3-IPLightBar_Loc3 0xB4-IPLightBar_Loc4 0xC0-Spare_Loc2_All 0xD0-Spare_Loc3_All 0xE0-Spare_Loc4_All 0xF0-Spare_Loc5_All |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

Update_ColorKind

| Value Range | 0x0-NO 0x0-YES |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

IlluminatedEntry_RqstKind

| Value Range | 0x0-OFF 0x1-ON |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

LightAmbColor_TempKind

| Value Range | 0x0-ICE_BLUE 0x1-ORANGE 0x2-SOFT_BLUE 0x3-RED 0x4-GREEN 0x5-BLUE 0x6-PURPLE 0x7-LINCOLN_WHITE 0x8-AMBER 0x9-TEAL 0xA-BURNT_ORANGE 0xB-LINCOLN_BLUE 0xC-LINCOLN_GREEN 0xD-LILAC |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

Parklamps_CommandKind

| Value Range | 0x0-OFF 0x1-ON |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

Ambient_StatusKind

| Value Range | 0x0-OFF 0x1-AMBIENT |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

LghtAmbRqSrc_B_StatKind

| Value Range | PRESENT MISSING |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

RampKind

| Value Range | 0x0-NO 0x1-YES |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

CourtesyDemand_BSave_RqstKind

| Value Range | 0x0-NO_EFFECT 0x1-OFF |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

LowVoltModeKind

| Value Range | 0x0-NORM 0x1-SUPPRESS |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

ColorKind

| Value Range | 0x0-color 0 0x1-color 1 0x2-color 2 0x3-color 3 0x4-color 4 0x5-color 5 0x6-color 6 0x7-color 7 0x8-color 8 0x9-color 9 0xA-color 10 0xB-color 11 0xC-color 12 0xD-color 13 0xE-color 14 0xF-color 15 |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

PersIndex_CSKind

| Value Range | 0x0-PERS_1 0x1-PERS_2 0x2-PERS_3 0x3-PERS_4 0x4-VEHICLE |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

AjarKind

| Value Range | 0x0-CLOSED 0x1-AJAR |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

DiagSessionKind

| Value Range | 0x0-DEFAULT 0x1-EXTEND 0x2-PROGRAM 0x3-NKNOWN |
| --- | --- |
| Resolution |  |
| Units |  |

valuetype

SelDrvMdeHmi03_D_Rq_ComStatKind

| Value Range | 0x0-MISSING 0x1-PRESENT |
| --- | --- |
| Resolution |  |
| Units |  |

Document ends here.