Research & Vehicle Technology

“Infotainment Systems Product Development”

Feature – Multi-Camera Client

Infotainment Subsystem Part Specific Specification (SPSS)

Version 1.16

UNCONTROLLED COPY IF PRINTED

Version Date: April 5, 2022

FORD CONFIDENTIALF

# Revision History

| Date | Version | Notes |  |
| --- | --- | --- | --- |
| May 31, 2013 | 1.0 | Initial Release |  |
| January 16, 2015 | 1.1 | SPSS Updated to add soft button view implementation |  |
|  | RVCv2-IR-REQ-014080/B-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3) |  | rpaquet2 - Updated encodings in table |
|  | MD-REQ-014070/A-Feature_St (TcSE ROIN-282399-2) |  | sorris1: Updated the Personalization Index parameter encoding to match the CAN database. |
|  | DAFVCv1-IR-REQ-014066/B-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061) |  | rpaquet2 - Updated encodings |
|  | MD-REQ-014070/A-Feature_St (TcSE ROIN-282399-2) |  | sorris1: Updated the Personalization Index parameter encoding to match the CAN database. |
|  | STR-052389/B-Rear View Camera (TcSE ROIN-293312-1) |  | rpaquet2 - Added two new variants Camera image View v3 and Rear View Camera Zoom Settings v3 to cover the new way of zooming and soft button inputs. |
|  | RVC-FUR-REQ-014088/B-Deactivate RVC (TcSE ROIN-293328) |  | rpaquet2 - Added new text to clarify Forward gear and Park no prak for Manual transmission applications. |
|  | CAMERA-FUR-REQ-014093/B-Camera Image Priority (TcSE ROIN-264652-1) |  | rpaquet2 - Updated requirement to work for all camera views. no change to requirement intent. |
|  | RVCv3-UC-REQ-127872/A-Activate Rear Multicamera View |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128191/A-Enter CHMSL Delay Mode |  | rpaquet2 - New use case |
|  | TRG-UC-REQ-102959/A-Vehicle Not in RUN/START |  | rpaquet2 - Added use case to multi-camera APIM SPSS |
|  | TRG-UC-REQ-102960/A-Loss of communication with IPMB Module |  | rpaquet2 - Added use case to multi-camera APIM SPSS |
|  | TRG-UC-REQ-102961/A-Valid Camera Video Signal not present |  | rpaquet2 - Added use case to multi-camera APIM SPSS |
|  | RVCv3-UC-REQ-128173/A-Press Zoom Button from Rear 360 View |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128176/A-Press Zoom Button from Rear Normal View |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128179/A-Press Zoom Button from CHMSL View |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128188/A-Press Rear 360 Unzoom |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128189/A-Press Rear Normal Unzoom |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128190/A-Press CHMSL Unzoom |  | rpaquet2 - New use case |
|  | RVCv3-ACT-REQ-127095/A-Manual Zoom |  | rpaquet2: New Requirement |
|  | RVCv3-UC-REQ-127874/A-Press Rear 360 View Button |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128175/A-Press RVC Normal View Button |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128177/A-Press RVC Split View Button |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128178/A-Press CHMSL Camera View Button |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128180/A-Press Aux Camera view Button |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128181/A-Press Trailer Reverse Guidance Button |  | rpaquet2 - New use case |
|  | RVCv3-UC-REQ-128396/A-Press Hard Button to Enter CHMSL View |  | rpaquet2 - New Use Case for CHMSL |
|  | RVCv3-UC-REQ-128397/A-Press Hard Button to Enter RVC View |  | rpaquet2 - New use case |
|  | RVCv3-ACT-REQ-127096/A-Change Camera View Setting |  | rpaquet2: Diagram updated to reflect the change in HMI to go back to soft buttons |
|  | STR-052463/B-Driver Assist Front View Camera (TcSE ROIN-293325) |  | rpaquet2 - Added New function variant 2 for Driver Assist Front Camera Image View to capture the soft button implementation. |
|  | DAFVCv2-UC-REQ-128182/A-Entering Front Camera - 360 |  | rpaquet2 - New use case |
|  | DAFVCv2-UC-REQ-128184/A-Entering Front Camera – Non-360 |  | rpaquet2 - New use case |
|  | TRG-UC-REQ-102959/A-Vehicle Not in RUN/START |  | rpaquet2 - Added use case to multi-camera APIM SPSS |
|  | TRG-UC-REQ-102960/A-Loss of communication with IPMB Module |  | rpaquet2 - Added use case to multi-camera APIM SPSS |
|  | TRG-UC-REQ-102961/A-Valid Camera Video Signal not present |  | rpaquet2 - Added use case to multi-camera APIM SPSS |
|  | DAFVCv2-UC-REQ-128183/A-Press Front 360 View Button |  | rpaquet2 - New use case |
|  | DAFVCv2-UC-REQ-128185/A-Press Front Normal View Button |  | rpaquet2 - New use case |
|  | DAFVCv2-UC-REQ-128186/A-Press Front Split View |  | rpaquet2 - New use case |
|  | DAFVCv2-UC-REQ-128187/A-Press Rear button From Front camera |  | rpaquet2 - New use case |
|  | DAFVCv2-ACT-REQ-127100/A-Change Camera View Setting |  | rpaquet2: Diagram updated to reflect use of soft button implementation. |
| June 15, 2015 | 1.2 | Updates to Merge Off Road FVC into Multi-Camera and update Max Speed requirements |  |
|  | DAFVCv1-IIR-REQ-014069/B-DriverAssistFrontViewCameraClient_Rx (TcSE ROIN-282847-2) |  | rpaquet2 - Added Elocker_St, AwdRange_St and OffRoadMode_St in order to merge FVC Offroad Camera with MultiCamera. |
|  | STR-052387/B-General Requirements (TcSE ROIN-293327-1) |  | rpaquet2 - Added Determine Off Road Mode and DAFVC Malfunction and T_cameraMalfunctionDelay. And moved RVC Malfunction to this folder. |
|  | CAMERA-REQ-014077/B-Feature Maximum Speed (TcSE ROIN-290556) |  | wstephe1: Updated requirement for Max Speed per feature scenario types: camera activation (any feature), RVC active, and DAFVC active by configuration of front (off road) camera. Scenarios to continue application across camera features. |
|  | DAFVCv1-FUR-REQ-166648/A-Determine Off Road Mode |  | rpaquet2 - New requirement created during merge of Offroad and multi camera specs. |
|  | RVC-FUR-REQ-014087/B-RVC Malfunction (TcSE ROIN-146656-2) |  | rpaquet2 - Updated requirement per APIM team. |
|  | DAFVCv1-FUR-REQ-166722/A-DAFVC Malfunction |  | rpaquet2 - Added new requirement per APIM team. |
|  | RVC-TMR-REQ-166649/A-T_cameraMalfunctionDelay |  | rpaquet2 - Added new timer requirement for delay. |
|  | STR-052390/B-Rear View Camera General Requirements (TcSE ROIN-293310-1) |  | rpaquet2 - Moved RVC Malfunction from Rear View Camera general requirements folder to the general requirements folder. |
|  | RVC-FUR-REQ-014088/C-Deactivate RVC (TcSE ROIN-293328) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | RVC-UC-REQ-014099/B-Rear Camera Delay Mode is On (TcSE ROIN-289798) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | RVC-UC-REQ-014100/B-Active Park Assist is Active (TcSE ROIN-290554) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | RVC-UC-REQ-014101/B-Trailer Backup Assist is Active (TcSE ROIN-290555) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | RVC-UC-REQ-014099/B-Rear Camera Delay Mode is On (TcSE ROIN-289798) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | RVC-UC-REQ-014100/B-Active Park Assist is Active (TcSE ROIN-290554) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | RVC-UC-REQ-014101/B-Trailer Backup Assist is Active (TcSE ROIN-290555) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | DAFVCv1-FUN-REQ-014045/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-293385) |  | rpaquet2 - Added requirement folder with the off road requirement due to merging Offroad and Multicamera SPSS. |
|  | DAFVCv1-FUR-REQ-166641/A-Deactivate FVC |  | rpaquet2 - New requirement number created during merge of Offroad and multi camera specs due to signal change. |
|  | DAFVCv1-FUR-REQ-013997/B-E-Locker Deactivation Delay (TcSE ROIN-266607-1) |  | rpaquet2 - Changed FID to DAFVC due to merge of Offroad and Multicamera spec. |
|  | DAFVCv1-TMR-REQ-013998/B-T_eLockerDelay (TcSE ROIN-266609-1) |  | rpaquet2 - Changed FID to DAFVC due to merge of Offroad and Multicamera spec. |
|  | DAFVCv1-UC-REQ-014048/B-Activation Attempt During Overspeed Condition (TcSE ROIN-290145) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 rpaquet2 - Updated the precondition and Scenario description to combine the Offroad use case for deactivate with that from Multi Camera. |
|  | DAFVCv1-ACT-REQ-014043/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-282606-3) |  | rpaquet2 - Added note to diagram to state check Feature Maximum Speed requirement for exit speed value. |
|  | DAFVCv1-SD-REQ-014050/B-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2) |  | rpaquet2 - Update diagram to add note to check feature Maximum Speed requirement for exit speed value. |
|  | DAFVCv2-FUN-REQ-128309/B-Activate/Deactivate Driver Assist Front View Camera v2 |  | rpaquet2 - Added requirement folder with the off road requirement due to merging Offroad and Multicamera SPSS. |
|  | STR-246672/A-Requirements |  |  |
|  | DAFVCv1-FUR-REQ-166641/A-Deactivate FVC |  | rpaquet2 - New requirement number created during merge of Offroad and multi camera specs due to signal change. |
|  | DAFVCv1-FUR-REQ-013997/B-E-Locker Deactivation Delay (TcSE ROIN-266607-1) |  | rpaquet2 - Changed FID to DAFVC due to merge of Offroad and Multicamera spec. |
|  | DAFVCv1-TMR-REQ-013998/B-T_eLockerDelay (TcSE ROIN-266609-1) |  | rpaquet2 - Changed FID to DAFVC due to merge of Offroad and Multicamera spec. |
|  | DAFVCv2-UC-REQ-128182/B-Entering Front Camera - 360 |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | DAFVCv2-UC-REQ-128184/B-Entering Front Camera – Non-360 |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 |
|  | DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146) |  | wstephe1: Revised to align with Max Speed requirement CAMERA-REQ-014077 rpaquet2 - Updated the precondition and Scenario description to combine the Offroad use case for deactivate with that from Multi Camera. |
|  | DAFVCv1-ACT-REQ-014043/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-282606-3) |  | rpaquet2 - Added note to diagram to state check Feature Maximum Speed requirement for exit speed value. |
|  | DAFVCv1-SD-REQ-014050/B-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2) |  | rpaquet2 - Update diagram to add note to check feature Maximum Speed requirement for exit speed value. |
| September 16, 2015 | 1.3 | Updates for Off Road FVC Activation and Max Speed |  |
|  | CAMERA-REQ-014077/C-Feature Maximum Speed (TcSE ROIN-290556) |  | tmertiri-Updated requirement to account for off road changes. |
|  | DAFVCv1-UC-REQ-014046/B-Activate Driver Assist Front View Camera (TcSE ROIN-290143) |  | tmertiri-Updated uses case to add Off Road Front Camera to the vehicle system indicated indications (via IPMB sending hard button press) |
|  | DAFVCv1-SD-REQ-014050/C-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2) |  | tmertiri-removed scenario of pre and post condition from sequence diagram requirement. Already defined in the use cases. |
|  | DAFVCv1-SD-REQ-014050/C-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2) |  | tmertiri-removed scenario of pre and post condition from sequence diagram requirement. Already defined in the use cases. |
| October 19, 2016 | 1.4 |  |  |
|  | RVC-FUR-REQ-014088/D-Deactivate RVC (TcSE ROIN-293328)+ |  | tmertiri: updated Reverse can signal name |
|  | RVC-FUR-REQ-014090/B-Display RVC Video (TcSE ROIN-194462-2)+ |  | tmertiri: replaces old signal name to new one. GearRvrse_D_Actl. |
|  | STR-052391/B-Use Cases (TcSE ROIN-293353) |  | tmertiri: updated use case |
|  | UC-REQ-238578/A-CHMSL Activation. Soft button press |  | tmertiri: updated use case |
|  | UC-REQ-238579/A-AUX Activation. Soft button press |  | tmertiri: updated use case |
|  | RVCv3-UC-REQ-128180/B-Press Aux Camera view Button |  | tmertiri: updated use case |
|  | RVCv3-UC-REQ-128396/B-Press Hard Button to Enter CHMSL View |  | tmertiri: updated use case |
| January 19, 2018 | 1.5 |  |  |
|  | RVC-REQ-292389/A-GearRvrse_D_Actl |  | tmertiri: Added new signal name |
|  | RVC-REQ-292387/A-GearPos_D_Trg |  | tmertiri: Added new signal name |
|  | RVC-REQ-292388/A-Veh_V_ActlEng |  | tmertiri: Added new signal name |
|  | RVC-FUR-REQ-014090/D-Display RVC Video (TcSE ROIN-194462-2) |  | tmertiri: updated with new signal names |
|  | RVCv2-SD-REQ-014104/B-Activate Rear View Camera (TcSE ROIN-282316-2) |  | Updated SD with new signals |
|  | RVCv2-SD-REQ-014105/B-Deactivate Rear View Camera (TcSE ROIN-282323-2) |  | Updated SD with new signals |
| February 1, 2018 | 1.6 |  |  |
|  | RVC-FUR-REQ-014090/E-Display RVC Video (TcSE ROIN-194462-2) |  | tmertiri: Update wording |
| July 27, 2018 | 1.7 |  |  |
|  | RVC-FUR-REQ-014090/F-Display RVC Video (TcSE ROIN-194462-2) |  | tm: Remove DE values details. |
| November 7, 2018 | 1.8 |  |  |
|  | RVCv2-IR-REQ-014080/C-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3) |  | tmertiri: added additonal camera views |
|  | MD-REQ-014071/B-CameraServerButton_St (TcSE ROIN-287063-1) |  | tmertiri: added signal name |
|  | DAFVCv1-IR-REQ-014066/C-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061) |  | tmertiri:Update encoding table |
|  | DAFVCv1-IIR-REQ-014067/B-DriverAsssitFrontViewCameraClient_Tx (TcSE ROIN-282846-1) |  | tmertiri: Added new signals |
|  | MD-REQ-331342/A-CamraFrntBttn_D_Stat3 |  | tmertiri: New signal added |
|  | DAFVCv1-IIR-REQ-014069/C-DriverAssistFrontViewCameraClient_Rx (TcSE ROIN-282847-2) |  | tmertiri: added LIN details |
|  | MD-REQ-331343/A-Camera Button LIN signal |  | tmertiri: new requirement added |
|  | MD-REQ-331847/A-CamraFrntOffRd_B_Stat |  | tmertiri: New signal |
|  | STR-052387/C-General Requirements (TcSE ROIN-293327-1) |  | tmertiri: Added new requirement. |
|  | REQ-331344/A-Button Press Logic |  | tmertiri: new requirement |
|  | DAFVCv1-FUR-REQ-166648/B-Determine Off Road Mode |  | tmertiri:Update requirement |
|  | REQ-331863/A-Lin Button Client Operation |  | tmertiri: New diagram |
|  | STR-052463/C-Driver Assist Front View Camera (TcSE ROIN-293325) |  | tmertiri:Modified diagrams for LIN button interface |
|  | STR-052307/B-Sequence Diagrams (TcSE ROIN-293389) |  | tmertiri: added new diagram |
|  | REQ-331856/A-Lin Button Camera Request |  | tmertiri: new diagram |
| December 18, 2018 | 1.9 |  |  |
|  | RVCv2-IR-REQ-014080/D-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3) |  | tmertiri: added additional camera views |
|  | MD-REQ-014084/B-ParkBrake_St (TcSE ROIN-287064-1) |  | tmertiri:no content change |
|  | MD-REQ-014085/B-ElectronicParkBrake_St (TcSE ROIN-287065-1) |  | tmertiri: add clarification verbiage |
|  | DAFVCv1-IR-REQ-014066/D-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061) |  | tmertiri: Update table |
|  | DAFVCv1-IIR-REQ-014067/C-DriverAssistFrontViewCameraClient_Tx (TcSE ROIN-282846-1) |  | tmertiri: fix typo in title |
|  | STR-052388/B-Functional Definition (TcSE ROIN-293326-1) |  | tmertiri: added offset view |
|  | REQ-331856/B-Lin Button Camera Request |  | tmertiri: verbiage correction in the boxlines |
|  | CAMERA-FUN-REQ-331348/A-Offset View |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331349/A-Press Rear Offset View Button from Rear 360 |  | tmertiri: new usecases Dec 14 |
|  | CAMERA-UC-REQ-331350/A-Press Rear Offset View Button from Front 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331351/A-Press Rear Left Corner View Button from Rear 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331352/A-Press Rear Left Corner View Button from Front 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331353/A-Press Rear Right Corner View Button from Rear 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331354/A-Press Rear Right Corner View Button from Front 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331356/A-Press Front Offset View Button from Rear 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331360/A-Press Front Offset View Button from Front 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331365/A-Press Front Left Corner View Button from Rear 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331369/A-Press Front Left Corner View Button from Front 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331373/A-Press Front Right Corner View Button from Rear 360 |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-331374/A-Press Front Right Corner View Button from Front 360 |  | tmertiri: new usecases |
| April 26, 2019 | 1.10 |  |  |
|  | RVCv2-IR-REQ-014080/E-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3) |  | tmertiri: added rock crawl views |
|  | RVC-MD-REQ-292389/B-GearRvrse_D_Actl |  | tmertiri: clarified what to consider as reverse |
|  | DAFVCv1-IR-REQ-014066/E-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061) |  | tmertiri: added rock crawl views |
|  | RVCv2-IIR-REQ-014086/C-VehicleInformation_Tx (TcSE ROIN-282400-2) |  | tmertiri: Structural change. Added to conent in this structure. |
|  | MD-REQ-347573/A-PowerMode |  | tmertiri: added for clarification. needed only in CGEA 1.2 |
|  | RVC-FUR-REQ-014090/G-Display RVC Video (TcSE ROIN-194462-2) |  | tmertiri: update the GeaRvrse_D_Actl New Strategy |
| October 2, 2019 | 1.11 |  |  |
|  | RVCv2-IR-REQ-014080/F-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3) |  | tmertiri: added Views at Speed Configs |
|  | DAFVCv1-IR-REQ-014066/F-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061) |  | tmertiri: adding Views At Speed |
|  | MD-REQ-331342/B-CamraFrntBttn_D_Stat3 |  | tmertiri: updating requirement. Adding Soft Press origins |
|  | CAMERA-REQ-331344/B-Button Press Logic |  | tmertiri: no content changes |
|  | CAMERA-REQ-331863/B-Lin Button Client Operation |  | tmertiri: no content change |
|  | STR-052388/C-Functional Definition (TcSE ROIN-293326-1) |  | tmertiri:revised to allow views at speed function |
|  | STR-669257/A-Views At Speed |  | tmertiri: new section |
|  | CAMERA-FUN-REQ-354982/A-Views At Speed |  | tmertiri: new section |
|  | STR-669258/A-Requirements |  | tmertiri: new section |
|  | CAMERA-REQ-354992/A-Views At Speed Availability |  | tmertiri: new req |
|  | CAMERA-REQ-354993/A-Views At Speed List |  | tmertiri: new req |
|  | CAMERA-REQ-358450/A-Off Road Mode Views At Speed |  | tmertiri: new req |
|  | STR-669259/A-Use Cases |  | tmertiri: new section |
|  | CAMERA-UC-REQ-355016/A-Views At Speed Trigger |  | tmertiri: new usecase |
|  | STR-669260/A-White Box Views |  | tmertiri: new section |
|  | STR-673941/A-Activity Diagram |  | tmertiri: new diagram |
|  | CAMERA-ACT-REQ-358468/A-Views At Speed |  | tmertiri: new diagram |
|  | STR-669262/A-Sequence Diagram |  | tmertiri: new section |
|  | CAMERA-SD-REQ-354994/A-Views At Speed |  | tmertiri: new SD |
| March 10, 2020 | 1.12 |  |  |
|  | STR-052388/D-Functional Definition (TcSE ROIN-293326-1) |  | tmertiri: add views at speed v2. |
|  | STR-669257/B-Views At Speed v1 |  | tmertiri: rename the content. V1 was added to the feature for Gen 3.2 |
|  | CAMERA-FUN-REQ-354982/B-Views At Speed v1 |  | tmertiri: rename requirement |
|  | CAMERA-REQ-354993/B-Views At Speed v1 List |  | tmertiri: rename and remove update table |
|  | CAMERA-REQ-358450/B-Off Road Mode Views At Speed |  | tmertiri: new requirement |
|  | CAMERA-UC-REQ-355016/B-Views At Speed Trigger |  | tmertiri: update usecase |
|  | CAMERA-ACT-REQ-358468/B-Views At Speed v1 |  | tmertiri: update diagram |
|  | CAMERA-SD-REQ-354994/B-Views At Speed v1 |  | tmertiri: update the parameters |
|  | 735207/A-Views At Speed V2 |  | tmertiri: new Views at Speed V2 are applicable for FNV2 |
|  | CAMERA-FUN-REQ-381834/A-Views At Speed V2 |  | tmertiri: new funcion |
|  | CAMERA-REQ-381833/A-Views At Speed V2 Enumerations |  | tmertiri: new requirement |
|  | CAMERA-REQ-382183/A-Views at Speed Deactivation |  | tmertiri: new requirement |
|  | 734944/A-UseCases |  | tmertiri: new usecase |
|  | CAMERA-UC-REQ-381763/A-RCOD Transition from Below Speed to Above Speed on 360 Variants |  | tmertiri: new usecase |
|  | CAMERA-UC-REQ-381770/A-Hitch Transition from Below Speed to Above Speed on 360 Variants |  | tmertiri: new usecase |
|  | CAMERA-UC-REQ-381808/A-Aux Transition from Below Speed to Above Speed on 360 Variants |  | tmertiri: new usecase |
|  | CAMERA-UC-REQ-381809/A-CHMSL Transition from Below Speed to Above Speed on 360 Variants |  | tmertiri: new usecase |
|  | CAMERA-UC-REQ-381810/A-Views at Speed above Speed Menu Options on 360 Variants |  | tmertiri: new usecase |
|  | CAMERA-UC-REQ-381811/A-RCOD Activation Above Speed on 360 Variants |  | tmertiri: new usecase |
|  | CAMERA-UC-REQ-381812/A-Hitch Activation Above Speed on 360 Variants |  | tmertiri: new usecase |
|  | CAMERA-UC-REQ-381813/A-Aux Activation Above Speed on 360 Variants |  | tmertiri: new usecase |
|  | CAMERA-UC-REQ-381814/A-CHMSL Activation Above Speed on 360 Variants |  | tmertiri: new usecase |
|  | CAMERA-UC-REQ-381815/A-RCOD Transition from Above Speed to Below Speed |  | tmertiri: new usecase |
|  | CAMERA-UC-REQ-381816/A-Hitch Transition from Above Speed to Below Speed |  | tmertiri: new usecase |
|  | CAMERA-UC-REQ-381818/A-Aux Transition from Above Speed to Below Speed |  | tmertiri: new usecase |
|  | CAMERA-UC-REQ-381817/A-CHMSL Transition from Above Speed to Below Speed |  | tmertiri: new usecase |
|  | 734946/A-White Box Views |  | tmertiri: new diagram |
|  | 734947/A-Activity Diagram |  | tmertiri: new diagram |
|  | CAMERA-ACT-REQ-381772/A-Views At Speed Gen 4 |  | tmertiri: new diagram |
|  | 734948/A-Sequence Diagram |  | tmertiri: new diagram |
|  | CAMERA-SD-REQ-381773/A-Views At Speed Gen 4 |  | tmertiri: new diagram |
|  | 738691/A-CHMSL IOD |  | tmertiri: new section |
|  | FUN-REQ-382683/A-CHMSL IOD |  | tmertiri: new function |
|  | CAMERA-REQ-382684/A-CHMSL IOD Config Values |  | tmertiri: new requirement |
|  | 738641/A-UseCases |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-382686/A-CHMSL IOD Activation |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-382687/A-CHMSL IOD Deactivation Via Client |  | tmertiri: new usecases |
|  | CAMERA-UC-REQ-382688/A-Returning to CHMSL IOD from Parking Feature |  | tmertiri: new usecases |
|  | 738655/A-White Box View |  | tmertiri: new section |
|  | 738652/A-Activity Diagram |  | tmertiri: new section |
|  | CAMERA-ACT-REQ-382730/A-CHMSL IOD Operation |  | tmertiri: new diagram |
|  | 738654/A-Sequence Diagrams |  | tmertiri: new section |
|  | CAMERA-SD-REQ-382731/A-CHMSL IOD Operation |  | tmertiri: new diagram |
| August 19, 2020 | 1.13 |  |  |
|  | RVCv2-IIR-REQ-014082/C-RearViewCameraClient_Rx (TcSE ROIN-282396-3) |  | tmertiri: adding new signals for RVC ON check |
|  | MD-REQ-395472/A-GearLvrPos_D_Actl_UB |  | tmertiri: new signal |
|  | MD-REQ-395473/A-VehVActlEng_D_Qf |  | tmertiri: new signal |
|  | MD-REQ-395474/A-GearPos_D_Trg_UB |  | tmertiri: new signal |
|  | DAFVCv1-IIR-REQ-014069/D-DriverAssistFrontViewCameraClient_Rx (TcSE ROIN-282847-2) |  | tmertiri: added new md for FNVSS table |
|  | RVCv2-IIR-REQ-014086/D-VehicleInformation_Tx (TcSE ROIN-282400-2) |  | tmertiri: added new md for FNVSS table |
|  | STR-052388/E-Functional Definition (TcSE ROIN-293326-1) |  | tmertiri: Added usecases for Reverse Gear Strategy |
|  | RVC-FUR-REQ-014090/H-Display RVC Video (TcSE ROIN-194462-2) |  | tmertiri: update table for new automatic transmission RVC state |
|  | RVC-FUN-REQ-395521/A-Reverse Gear Strategy |  | tmertiri: new structure |
|  | 788109/A-Use Cases |  | tmertiri: new structure |
|  | RVC-UC-REQ-395465/A-Reverse Gear Strategy on Automatic Transmission Vehicles - Moving Below 16.09 KPH |  | tmertiri: new usecase |
|  | RVC-UC-REQ-395468/A-Reverse Gear Strategy on Automatic Transmission Vehicles - Moving Above 16.09 KPH |  | tmertiri: new usecase |
|  | RVC-UC-REQ-395466/A-Reverse Gear Strategy on Automatic Transmission Vehicles - Shift Above 16.09 KPH and Deceleration Below 16.09 KPH |  | tmertiri: new usecase |
|  | RVC-UC-REQ-395467/A-Reverse Gear Strategy on Automatic Transmission Vehicles - Vehicle Speed Unavailable or is Not OK |  | tmertiri: new usecase |
|  | RVC-UC-REQ-395469/A-Reverse Gear Strategy on Automatic Transmission Vehicles - Exit Criteria when Gear Shifter Position is not Reverse |  | tmertiri: new usecase |
|  | RVC-UC-REQ-395470/A-Exit Criteria when RVC is active, Gear Shifter is in Reverse, and the vehicle is moving forward above 16.09 KPH |  | tmertiri: new usecase |
| March 31, 2021 | 1.14 |  |  |
|  | STR-052384/B-Architectural Design (TcSE ROIN-293311-1) |  | tmertiri: added new signals and elements |
|  | CAMERA-CLD-REQ-412756/A-Camera Client |  | tmertiri: added generic class |
|  | CAMERA-CLD-REQ-412757/A-Camera Server |  | tmertiri: added generic class |
|  | 879418/A-Turn Signal View Logical Mapping Table |  | tmertiri: Trailer Signal View function related changes |
|  | 879419/A-CTA Split View Logical Mapping Table |  | tmertiri: CTA 180 related changes |
|  | CAMERA-IIR-REQ-412758/A-Camera Client Tx |  | tmertiri: added generic simplified iir |
|  | MD-REQ-410094/A-LMnuRq |  | tmertiri: Trailer Signal View function related changes |
|  | MD-REQ-411949/A-CTASVRq |  | tmertiri: CTA 180 related changes |
|  | CAMERA-IIR-REQ-412759/A-Camera Client Rx |  | tmertiri: added generic simplified iir |
|  | MD-REQ-410095/A-LTviewSt |  | tmertiri: Trailer Signal View function related changes |
|  | MD-REQ-410096/A-LSodLSt |  | tmertiri: Trailer Signal View function related changes |
|  | MD-REQ-410097/A-LSodRSt |  | tmertiri: Trailer Signal View function related changes |
|  | MD-REQ-411950/A-CTASVSt |  | tmertiri: CTA 180 related changes |
|  | STR-052388/F-Functional Definition (TcSE ROIN-293326-1) |  | tmertiri: added many new functions |
|  | 882163/A-Turn Signal View |  | tmertiri: New structure with new content |
|  | FUN-REQ-410168/A-Turn Signal View |  | tmertiri: Trailer Signal View function related changes |
|  | 881818/A-Functional Definition |  | tmertiri: Trailer Signal View function related changes |
|  | 882149/A-Requirements |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410098/A-Trailer Turn Signal View - Menu Setting Updates |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410099/A-Trailer Turn Signal View - Menu Grayed Out |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410101/A-Trailer Turn Signal View - Grayed Out Menu Displays Off |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410102/A-Trailer Turn Signal View - No User Change to Menu |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410103/A-Menu Setting Change - Missing Message |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410104/A-Menu Setting Change - Message Agreement Timeout |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410105/A-Menu Setting based on Server Feature Memory |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410106/A-Trailer Turn Signal View - Menu Setting Change - ON to OFF |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410107/A-Trailer Turn Signal View - Menu Setting Change - OFF to ON |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410108/A-Menu Setting Change - Parity THEN Revert to NULL |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410109/A-Trailer Turn Signal View LEFT Activation - NO Blind Spot Threat Reported |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410110/A-Trailer Turn Signal View LEFT Activation - Blind Spot Threat Reported |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410111/A-Trailer Turn Signal View RIGHT Activation - NO Blind Spot Threat Reported |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-REQ-410112/A-- Trailer Turn Signal View RIGHT Activation - Blind Spot Threat Reported |  | tmertiri: Trailer Signal View function related changestmertiri: Trailer Signal View function related changes |
|  | 881910/A-Use Cases |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-UC-REQ-409518/A-Trailer Turn Signal View Missing Signal |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-UC-REQ-409519/A-Trailer Turn Signal View Setting Change |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-UC-REQ-409520/A-Trailer Turn Signal View because of Cancel "X" Button Press |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-UC-REQ-409521/A-Trailer Turn Signal View due to Shift Reverse |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-UC-REQ-410090/A-Trailer Turn Signal View Menu Setting based on Server Feature Memory |  | tmertiri: Trailer Signal View function related changes |
|  | 881918/A-White Box Views |  | tmertiri: Trailer Signal View function related changes |
|  | 881919/A-Activity Diagrams |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-ACT-REQ-411945/A-Operate TSV |  | tmertiri: Trailer Signal View function related changestmertiri: Trailer Signal View function related changes |
|  | CAMERA-ACT-REQ-411947/A-Disable TSV |  | tmertiri: Trailer Signal View function related changes |
|  | 881923/A-Sequence Diagrams |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-SD-REQ-411946/A-Operate TSV |  | tmertiri: Trailer Signal View function related changes |
|  | CAMERA-SD-REQ-411948/A-Disable TSV |  | tmertiri: Trailer Signal View function related changes |
|  | 882164/A-CTA 180 Multicamera |  | tmertiri: New structure with new content |
|  | FUN-REQ-410169/A-CTA 180 Multicamera |  | tmertiri: CTA 180 related changes |
|  | 881819/A-Functional Definition |  | tmertiri: CTA 180 related changes |
|  | 881911/A-Use Cases |  | tmertiri: CTA 180 related changes |
|  | CAMERA-UC-REQ-410120/A-Menu to be not available |  | tmertiri: CTA 180 related changes |
|  | CAMERA-UC-REQ-410121/A-Menu to be available in Settings |  | tmertiri: CTA 180 related changes |
|  | CAMERA-UC-REQ-410122/A-Activating Feature via Settings menu |  | tmertiri: CTA 180 related changestmertiri: CTA 180 related changes |
|  | CAMERA-UC-REQ-410123/A-De-activating Feature via Settings menu |  | tmertiri: CTA 180 related changes |
|  | CAMERA-UC-REQ-410124/A-No Switching when not in Reverse Gear |  | tmertiri: CTA 180 related changes |
|  | CAMERA-UC-REQ-410125/A-View switching due to a CTA Event |  | tmertiri: CTA 180 related changes |
|  | CAMERA-UC-REQ-410126/A-Switching to last known view after a CTA Event |  | tmertiri: CTA 180 related changes |
|  | CAMERA-UC-REQ-410127/A-Missing Signal |  | tmertiri: CTA 180 related changes |
|  | 881915/A-White Box Views |  | tmertiri: CTA 180 related changes |
|  | 881920/A-Activity Diagrams |  | tmertiri: CTA 180 related changes |
|  | CAMERA-ACT-REQ-411951/A-Enable Disable CTASV |  | tmertiri: CTA 180 related changes |
|  | CAMERA-ACT-REQ-411953/A-Switching to SV |  | tmertiri: CTA 180 related changes |
|  | 881924/A-Sequence Diagrams |  | tmertiri: CTA 180 related changes |
|  | CAMERA-SD-REQ-411952/A-Enable Disable CTASV |  | tmertiri: CTA 180 related changes |
|  | CAMERA-SD-REQ-411954/A-Switching to SV |  | tmertiri: CTA 180 related changes |
|  | 882165/A-Invisible Van View |  | tmertiri: New structure with new content |
|  | FUN-REQ-410171/A-Invisible Van View |  | tmertiri: invisible van view function related changes |
|  | CAMERA-REQ-411963/A-IVV Config Value |  | tmertiri: invisible van view function related changes |
|  | 881821/A-Functional Definition |  | tmertiri: invisible van view function related changes |
|  | 881913/A-Use Cases |  | tmertiri: invisible van view function related changes |
|  | CAMERA-UC-REQ-410139/A-Requesting Invisible Van View |  | tmertiri: invisible van view function related changes |
|  | CAMERA-UC-REQ-410140/A-Deactivating Invisible Van View (1) |  | tmertiri: invisible van view function related changes |
|  | CAMERA-UC-REQ-411961/A-Deactivating Invisible Van View (2) |  | tmertiri: invisible van view function related changes |
|  | CAMERA-UC-REQ-411962/A-Deactivating Invisible Van View (3) |  | tmertiri: invisible van view function related changes |
|  | 881917/A-White Box Views |  | tmertiri: invisible van view function related changes |
|  | 881922/A-Activity Diagrams |  | tmertiri: invisible van view function related changes |
|  | CAMERA-ACT-REQ-411959/A-Operate Invisible Van View |  | tmertiri: invisible van view function related changes |
|  | 881926/A-Sequence Diagrams |  | tmertiri: invisible van view function related changes |
|  | CAMERA-SD-REQ-411960/A-Operate Invisible Van View |  | tmertiri: invisible van view function related changes |
|  | 882166/A-50/50 Views |  | tmertiri: New structure with new content |
|  | FUN-REQ-410167/A-50/50 Views |  | tmertiri: changes related to 50 50 view function |
|  | CAMERA-REQ-411964/A-50 50 Split View Configurations |  | tmertiri: changes related to 50 50 view function |
|  | 881909/A-Functional Definition |  | tmertiri: changes related to 50 50 view function |
|  | 881914/A-Use Cases |  | tmertiri: changes related to 50 50 view function |
|  | CAMERA-UC-REQ-410164/A-50/50 SPLIT VIEW Activation Below Speed on 360 Variants |  | tmertiri: changes related to 50 50 view function |
|  | CAMERA-UC-REQ-410165/A-50/50 SPLIT VIEW Transition from Below Speed to Above Speed on 360 Variants |  | tmertiri: changes related to 50 50 view function |
|  | CAMERA-UC-REQ-410166/A-50/50 Split View at Speed Activation Above Speed on 360 Variants |  | tmertiri: changes related to 50 50 view function |
|  | CAMERA-UC-REQ-410163/A-50/50 Split View Transition from Above Speed to Below Speed |  | tmertiri: changes related to 50 50 view function |
|  | 882147/A-White Box Views |  | tmertiri: changes related to 50 50 view function |
|  | 882148/A-Activity Diagrams |  | tmertiri: changes related to 50 50 view function |
|  | CAMERA-ACT-REQ-411965/A-Operate 50 50 Split View |  | tmertiri: changes related to 50 50 view function |
|  | 881927/A-Sequence Diagrams |  | tmertiri: changes related to 50 50 view function |
|  | CAMERA-SD-REQ-411966/A-Operate 50 50 Split View |  | tmertiri: changes related to 50 50 view function |
| June 29, 2021 | 1.15 |  |  |
|  | STR-052384/C-Architectural Design (TcSE ROIN-293311-1) |  | tmertiri: new signals added in the structure |
|  | 902993/A-Logical Mapping Table |  | tmertiri: New signals added |
|  | CAMERA-IIR-REQ-412759/B-Camera Client Rx |  | tmertiri: new signals added to structure |
|  | MD-REQ-419851/A-LTrlrConnect |  | tmertiri: new signal added in structure |
|  | MD-REQ-421155/A-LTgateSt |  | tmertiri: new signal added in structure |
|  | STR-052388/G-Functional Definition (TcSE ROIN-293326-1) |  | tmertiri: new functions added |
|  | RVC-FUR-REQ-014090/I-Display RVC Video (TcSE ROIN-194462-2) |  | tmertiri: updated req for FNV3 |
|  | 902987/A-Trailer 360 Views |  | tmertiri: new structure with new content |
|  | CAMERA-FUN-REQ-419878/A-Trailer 360 Views |  | tmertiri: new structure with new content |
|  | 915759/A-Requirements |  | tmertiri: new structure with new content |
|  | CAMERA-REQ-419865/A-Trailer 360 View Configuration Values |  | tmertiri: new structure with new content |
|  | CAMERA-REQ-419879/A-Trailer 360 Views Availability |  | tmertiri: new structure with new content |
|  | 902992/A-Use Cases |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-419705/A-Trailer 360 Soft Button Activation |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-419706/A-Trailer 360 view selection below 10 KPH or in Reverse |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-419707/A-Trailer Rear Normal view selection below 10 KPH or in Reverse |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-419708/A-Trailer Interior View 1 selection below 10 KPH or in Reverse |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-419709/A-Trailer Interior View 2 selection below 10 KPH or in Reverse |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-419710/A-Trailer Left View selection below 10 KPH or in Reverse |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-419711/A-Trailer Right View selection below 10 KPH or in Reverse |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-419712/A-Trailer 360 Soft Button Deactivation |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-419713/A-Trailer Rear Normal View transition from below 10 KPH to above 10 KPH |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-419714/A-Trailer Rear Normal View at Speed selection above 10 KPH |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-419715/A-Trailer Rear Normal View transition from above 10 KPH to below 10 KPH |  | tmertiri: new structure with new content |
|  | 915760/A-White Box Views |  | tmertiri: new structure with new content |
|  | 915761/A-Activity Diagrams |  | tmertiri: new structure with new content |
|  | CAMERA-ACT-REQ-423119/A-Trailer 360 Views |  | tmertiri: new structure with new content |
|  | 915762/A-Sequence Diagrams |  | tmertiri: new structure with new content |
|  | CAMERA-SD-REQ-423120/A-Trailer 360 Views |  | tmertiri: new structure with new content |
|  | 915763/A-PiP At Speed |  | tmertiri: new structure with new content |
|  | FUN-REQ-423142/A-PiP At Speed |  | tmertiri: new structure with new content |
|  | 915751/A-Requirements |  | tmertiri: new structure with new content |
|  | CAMERA-REQ-422921/A-Views Parameter Values |  | tmertiri: new structure with new content |
|  | 915752/A-Use Cases |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420591/A-Requesting PiP AUX from AUX below 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420592/A-Requesting AUX from PiP AUX with PiP button at below 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420593/A-Requesting Swap from PiP AUX at below 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420594/A-Requesting PiP 50/50 from 50/50 view , below 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420595/A-Requesting 50/50 from PiP 50/50 with PiP button at below 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420596/A-Requesting Swap from PiP 50/50 at below 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420597/A-Requesting Pip Trailer Rear Normal from Trailer Rear Normal below 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420598/A-Requesting Trailer Rear Normal from PiP Trailer Rear Normal with PiP button at below 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420599/A-Requesting Swap from Pip Trailer Rear Normal below 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420600/A-Transitioning from PiP AUX to PiP AUX at Speed |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420601/A-Transitioning from PiP AUX at Speed to PiP AUX |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420602/A-Transitioning from PiP 50/50 to PiP 50/50 at speed |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420603/A-Transitioning from PiP 50/50 at speed to PiP 50/50 |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420604/A-Transitioning from PiP Trailer Rear Normal to PiP Trailer Rear Normal at speed |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420605/A-Transitioning from PiP Trailer Rear Normal at speed to PiP Trailer Rear Normal |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420606/A-Requesting PiP AUX at Speed from AUX at Speed |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420607/A-Requesting PiP 50/50 at speed from 50/50 at speed with PiP button |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420608/A-Requesting PiP Trailer Rear Normal at Speed from Trailer Rear Normal at Speed with PiP button |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420609/A-Requesting AUX at speed view from PiP AUX at Speed view with PiP button |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420610/A-Requesting 50/50 at speed view from PiP 50/50 at Speed view with PiP button |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420611/A-Requesting Trailer Rear Normal at speed from PiP Trailer Rear Normal at speed view with PiP button |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420612/A-Exit PiP Aux at Speed View with Camera Hard button |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420613/A-Exit PiP 50/50 at Speed View with Camera Hard button |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420614/A-Exit PiP Trailer Rear Normal at Speed View with Camera Hard button |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420615/A-Requesting AUX from PiP AUX view with Aux button below 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420616/A-Requesting 50/50 from PiP 50/50 view with 50/50 button below 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420617/A-Requesting Trailer Normal View from PiP Trailer Rear Normal view with Trailer Rear Normal View button below 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420618/A-Requesting Swap from PiP AUX at speed above 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420619/A-Requesting Swap from PiP 50/50 at speed above 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420620/A-Requesting Swap from PiP Trailer Rear Normal at speed above 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420621/A-Last Remembered view for PiP AUX at Speed |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420622/A-Last Remembered view for PiP 50/50 at Speed |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420623/A-Last Remembered view for PiP Trailer Rear Normal at Speed |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420624/A-Requesting AUX at speed view from PiP 50/50 at Speed view above 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-420625/A-Requesting 50/50 at speed view from PiP AUX at Speed view above 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-422845/A-Requesting Trailer Rear Normal at speed view from PiP 50/50 at Speed view above 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-UC-REQ-422844/A-Requesting 50/50 at speed from PiP Trailer Rear Normal at speed view above 10kph |  | tmertiri: new structure with new content |
|  | 915753/A-White Box Views |  | tmertiri: new structure with new content |
|  | 915754/A-Activity Diagrams |  | tmertiri: new structure with new content |
|  | CAMERA-ACT-REQ-422922/A-PiP Views |  | tmertiri: new structure with new content |
|  | 915755/A-Sequence Diagrams |  | tmertiri: new structure with new content |
|  | CAMERA-SD-REQ-422923/A-PiP Views |  | tmertiri: new structure with new content |
|  | 915764/A-Tailgate RVC |  | tmertiri: new structure with new content |
|  | CAMERA-FUN-REQ-421101/A-Tailgate RVC |  | tmertiri: new structure with new content |
|  | 907725/A-Overview |  | tmertiri: new structure with new content |
|  | 907726/A-Requirements |  | tmertiri: new structure with new content |
|  | CAMERA-REQ-421102/A-Tailgate Down In Reverse |  | tmertiri: new structure with new content |
|  | CAMERA-REQ-421103/A-Tailgate Down Not In Reverse |  | tmertiri: new structure with new content |
|  | CAMERA-REQ-421104/A-Tailgate Down And Vehicle Speed Above 10kph |  | tmertiri: new structure with new content |
|  | CAMERA-REQ-421105/A-Tailgate States |  | tmertiri: new structure with new content |
|  | 907727/A-Use Cases |  | tmertiri: new structure with new content |
|  | UC-REQ-421106/A-Suppressed Views with Tailgate Down RVC in Reverse |  | tmertiri: new structure with new content |
|  | UC-REQ-421107/A-Suppressed Views with Tailgate Down RVC not in Reverse |  | tmertiri: new structure with new content |
|  | UC-REQ-421108/A-Suppressed Views-at-speed with Tailgate Down RVC |  | tmertiri: new structure with new content |
|  | 915758/A-White Box Views |  | tmertiri: new structure with new content |
|  | 915756/A-Activity Diagrams |  | tmertiri: new structure with new content |
|  | ACT-REQ-422407/A-Tailgate Camera Limiting |  | tmertiri: new structure with new content |
|  | 915757/A-Sequence Diagrams |  | tmertiri: new structure with new content |
|  | SD-REQ-422408/A-Tailgate Camera Limiting |  | tmertiri: new structure with new content |
| April 5, 2022 | 1.16 |  |  |
|  | STR-052388/H-Functional Definition (TcSE ROIN-293326-1) |  | tmertiri: add offset view function |
|  | STR-052390/C-Rear View Camera General Requirements (TcSE ROIN-293310-1) |  | tmertiri: removed 014089 |
|  | STR-052391/C-Use Cases (TcSE ROIN-293353) |  | tmertiri:removed 014100 . Usecase not totally applicable to RVC |
|  | 1039274/A-AUX IOD |  | tmertiri: new requirement |
|  | FUN-REQ-483789/A-AUX IOD |  | tmertiri: new requirement |
|  | 1039550/A-Use Cases |  | tmertiri: new requirement |
|  | UC-REQ-483740/A-AUX IOD Activation |  | tmertiri: new requirement |
|  | UC-REQ-483741/A-AUX IOD Deactivation via Camera Button |  | tmertiri: new requirement |
|  | UC-REQ-483743/A-AUX IOD Deactivation via Shift to Reverse |  | tmertiri: new requirement |
|  | UC-REQ-483744/A-AUX IOD Deactivation via Pro Trailer Assist Activation |  | tmertiri: new requirement |
|  | UC-REQ-483745/A-AUX IOD Deactivation via APA Activation |  | tmertiri: new requirement |
|  | UC-REQ-483747/A-AUX IOD Deactivation via Ignition |  | tmertiri: new requirement |
|  | UC-REQ-483748/A-AUX IOD while Shifting to Park |  | tmertiri: new requirement |
|  | UC-REQ-483749/A-AUX IOD Deactivation via Sync |  | tmertiri: new requirement |
|  | UC-REQ-483750/A-AUX IOD Deactivation via Boundary Alert |  | tmertiri: new requirement |
|  | UC-REQ-483781/A-AUX IOD Deactivation via Off-road Mode |  | tmertiri: new requirement |
|  | UC-REQ-483751/A-Returning to AUX IOD from Parking Feature |  | tmertiri: new requirement |
|  | UC-REQ-483752/A-No Response from ADAS/IPMB Handling |  | tmertiri: new requirement |
|  | UC-REQ-483753/A-No Video from ADAS/IPMB Handling |  | tmertiri: new requirement |
|  | 1039490/A-RCOD |  | tmertiri: new requirement |
|  | FUN-REQ-483788/A-RCOD |  | tmertiri: new requirement |
|  | 1039551/A-Use Cases |  | tmertiri: new requirement |
|  | UC-REQ-483754/A-RCOD Black Box Not Configured ADAS |  | tmertiri: new requirement |
|  | UC-REQ-483755/A-RCOD Black Box Is Configured ADAS Variant |  | tmertiri: new requirement |
|  | UC-REQ-483757/A-RCOD Black Box Is Configured Non-ADAS Variant |  | tmertiri: new requirement |
|  | UC-REQ-483756/A-RCOD Black Box Not Configured Non-ADAS variant |  | tmertiri: new requirement |

Table of Contents

Revision History	2

1	Architectural Design	16

1.1	RVC-CLD-REQ-014079/A-Rear View Camera Client (TcSE ROIN-282382-1)	16

1.2	DAFVCv1-CLD-REQ-014064/A-Driver Assist Front View Camera Client (TcSE ROIN-282843-1)	16

1.3	RearViewCameraClient Interface	16

1.3.1	RVCv2-IR-REQ-014080/F-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3)	16

1.3.2	RVCv2-IIR-REQ-014081/A-RearViewCameraClient_Tx (TcSE ROIN-282395-3)	17

1.3.3	RVCv2-IIR-REQ-014082/C-RearViewCameraClient_Rx (TcSE ROIN-282396-3)	18

1.4	DriverAssistFrontViewCameraClient Interface	22

1.4.1	DAFVCv1-IR-REQ-014066/F-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061)	22

1.4.2	DAFVCv1-IIR-REQ-014067/C-DriverAssistFrontViewCameraClient_Tx (TcSE ROIN-282846-1)	23

1.4.3	DAFVCv1-IIR-REQ-014069/D-DriverAssistFrontViewCameraClient_Rx (TcSE ROIN-282847-2)	24

1.5	VehicleInformation Interface	28

1.5.1	RVCv2-IIR-REQ-014086/D-VehicleInformation_Tx (TcSE ROIN-282400-2)	28

1.6	CAMERA-CLD-REQ-412756/A-Camera Client	32

1.7	CAMERA-CLD-REQ-412757/A-Camera Server	32

1.8	Turn Signal View Logical Mapping Table	32

1.9	CTA Split View Logical Mapping Table	32

1.10	Logical Mapping Table	33

1.11	CAMERA-IIR-REQ-412758/A-Camera Client Tx	33

1.11.1	MD-REQ-410094/A-LMnuRq	33

1.11.2	MD-REQ-411949/A-CTASVRq	33

1.12	CAMERA-IIR-REQ-412759/B-Camera Client Rx	34

1.12.1	MD-REQ-410095/A-LTviewSt	34

1.12.2	MD-REQ-410096/A-LSodLSt	34

1.12.3	MD-REQ-410097/A-LSodRSt	34

1.12.4	MD-REQ-411950/A-CTASVSt	34

1.12.5	MD-REQ-419851/A-LTrlrConnect	34

1.12.6	MD-REQ-421155/A-LTgateSt	35

2	General Requirements	36

2.1	CAMERA-REQ-331344/B-Button Press Logic	36

2.2	CAMERA-REQ-014077/C-Feature Maximum Speed (TcSE ROIN-290556)	36

2.3	DAFVCv1-FUR-REQ-166648/B-Determine Off Road Mode	37

2.4	RVC-FUR-REQ-014087/B-RVC Malfunction (TcSE ROIN-146656-2)	37

2.5	DAFVCv1-FUR-REQ-166722/A-DAFVC Malfunction	37

2.6	RVC-TMR-REQ-166649/A-T_cameraMalfunctionDelay	37

2.7	CAMERA-REQ-331863/B-Lin Button Client Operation	38

3	Functional Definition	39

3.1	Rear View Camera	39

3.1.1	Rear View Camera General Requirements	39

3.1.2	RVCv2-FUN-REQ-014094/A-Activate/Deactivate Rear View Camera (TcSE ROIN-293352)	41

3.1.3	RVCv3-FUN-REQ-128277/A-Activate/Deactivate Rear View Camera v3	49

3.1.4	RVCv2-FUN-REQ-014106/A-Rear View Camera Zoom Setting (TcSE ROIN-293363)	58

3.1.5	RVCv3-FUN-REQ-127106/A-Rear View Camera Zoom Setting v3	61

3.1.6	RVCv2-FUN-REQ-014111/A-Rear View Camera Delay Mode Setting (TcSE ROIN-293368)	65

3.1.7	RVCv2-FUN-REQ-014113/A-Camera Image View (TcSE ROIN-293370)	65

3.1.8	RVCv3-FUN-REQ-127111/A-Camera Image View v3	68

3.1.9	RVCv2-FUN-REQ-014119/A-Activate/Deactivate Enhanced Park Aids (TcSE ROIN-293375)	74

3.1.10	RVCv2-FUN-REQ-014124/A-Camera Overlay Setting (TcSE ROIN-293357)	75

3.2	Driver Assist Front View Camera	80

3.2.1	DAFVCv1-FUN-REQ-014045/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-293385)	80

3.2.2	DAFVCv2-FUN-REQ-128309/B-Activate/Deactivate Driver Assist Front View Camera v2	86

3.2.3	DAFVCv1-FUN-REQ-014052/A-Driver Assist Front Camera Image View (TcSE ROIN-293390)	93

3.2.4	DAFVCv2-FUN-REQ-127102/A-Driver Assist Front Camera Image View v2	96

3.3	CAMERA-FUN-REQ-331348/A-Offset View	102

3.3.1	CAMERA-UC-REQ-331349/A-Press Rear Offset View Button from Rear 360	102

3.3.2	CAMERA-UC-REQ-331350/A-Press Rear Offset View Button from Front 360	102

3.3.3	CAMERA-UC-REQ-331351/A-Press Rear Left Corner View Button from Rear 360	102

3.3.4	CAMERA-UC-REQ-331352/A-Press Rear Left Corner View Button from Front 360	102

3.3.5	CAMERA-UC-REQ-331353/A-Press Rear Right Corner View Button from Rear 360	103

3.3.6	CAMERA-UC-REQ-331354/A-Press Rear Right Corner View Button from Front 360	103

3.3.7	CAMERA-UC-REQ-331356/A-Press Front Offset View Button from Rear 360	103

3.3.8	CAMERA-UC-REQ-331360/A-Press Front Offset View Button from Front 360	104

3.3.9	CAMERA-UC-REQ-331365/A-Press Front Left Corner View Button from Rear 360	104

3.3.10	CAMERA-UC-REQ-331369/A-Press Front Left Corner View Button from Front 360	104

3.3.11	CAMERA-UC-REQ-331373/A-Press Front Right Corner View Button from Rear 360	104

3.3.12	CAMERA-UC-REQ-331374/A-Press Front Right Corner View Button from Front 360	105

3.4	Views At Speed v1	105

3.4.1	CAMERA-FUN-REQ-354982/B-Views At Speed v1	105

3.5	Views At Speed V2	107

3.5.1	CAMERA-FUN-REQ-381834/A-Views At Speed V2	107

3.6	CHMSL IOD	115

3.6.1	FUN-REQ-382683/A-CHMSL IOD	115

3.7	RVC-FUN-REQ-395521/A-Reverse Gear Strategy	117

3.7.1	Use Cases	117

3.8	Turn Signal View	120

3.8.1	FUN-REQ-410168/A-Turn Signal View	120

3.9	CTA 180 Multicamera	126

3.9.1	FUN-REQ-410169/A-CTA 180 Multicamera	126

3.10	Invisible Van View	131

3.10.1	FUN-REQ-410171/A-Invisible Van View	131

3.11	50/50 Views	135

3.11.1	FUN-REQ-410167/A-50/50 Views	135

3.12	Trailer 360 Views	139

3.12.1	CAMERA-FUN-REQ-419878/A-Trailer 360 Views	139

3.13	PiP At Speed	146

3.13.1	FUN-REQ-423142/A-PiP At Speed	146

3.14	Tailgate RVC	159

3.14.1	CAMERA-FUN-REQ-421101/A-Tailgate RVC	159

3.15	AUX IOD	161

3.15.1	FUN-REQ-483789/A-AUX IOD	161

3.16	RCOD	166

3.16.1	FUN-REQ-483788/A-RCOD	166

4	Appendix: Reference Documents	169

# Architectural Design

## RVC-CLD-REQ-014079/A-Rear View Camera Client (TcSE ROIN-282382-1)

Responsibility: The Rear View Camera Client is the interface of the Rear View Camera function. It acts with other system parts that control the Rear View Camera or need data from it.

## DAFVCv1-CLD-REQ-014064/A-Driver Assist Front View Camera Client (TcSE ROIN-282843-1)

Responsibility: The Driver Assist Front View Camera Client is the interface of the Driver Assist Front View Camera function. It acts with other system parts that control the Driver Assist Front View Camera or need data from it.

## RearViewCameraClient Interface

### RVCv2-IR-REQ-014080/F-Rear View Camera Feature ID Definition (TcSE ROIN-287010-3)

The below table defines the Feature ID information related to the Feature Based Message Protocol command and control method used in the Rear View Camera feature.

| Feature Name | Feature ID | Config Number | Config Value |
| --- | --- | --- | --- |
| Rear Camera Visual Park Aid Overlays | 0x080A | 0x01 | On |
|  |  | 0x00 | Off |
| All Overlays | 0x0812 | 0x01 | On |
|  |  | 0x00 | Off |
| Rear Camera Static Overlays | 0x0818 | 0x01 | On |
|  |  | 0x00 | Off |
| Rear Camera Dynamic Overlays | 0x0819 | 0x01 | On |
|  |  | 0x00 | Off |
| Manual Zoom | 0x081A | 0x00 | Off |
|  |  | 0x01 | Level 1 |
|  |  | 0x02 | Level2 |
|  |  | 0x03 | Level3 |

| Feature Name | Feature ID | Config Number | Config Value |
| --- | --- | --- | --- |
| Camera Showing | 0x081B | 0x00 | Off |
|  |  | 0x01 | Front |
|  |  | 0x02 | Rear |
|  |  | 0x03 | Remote |
|  |  | 0x04 | Rear 360 |
|  |  | 0x05 | Rear Normal |
|  |  | 0x06 | Rear Split |
|  |  | 0x07 | Front 360 |
|  |  | 0x08 | Front Normal |
|  |  | 0x09 | Front Split |
|  |  | 0x0A | Rear Zoom |
|  |  | 0x0B | CHMSL |
|  |  | 0x0C | CHMSL Zoom |
|  |  | 0x0D | AUX |
|  |  | 0x0E | TRG |
|  |  | 0x0F | TRG Rear Normal |
|  |  | 0x10 | Straight Back Up Mode |
|  |  | 0x11 | TBA2 Rear Split |
|  |  | 0x12 | TBA2 Rear 360 |
|  |  | 0x13 | TBA2 CHMSL |
|  |  | 0x14 | TBA2 Aux |
|  |  | 0x15 | TBA2 50/50 |
|  |  | 0x16 | TBA2 Rear Normal |
|  |  | 0x17 | Auto Hitch Rear Normal |
|  |  | 0x18 | Rear Frwd Offset |
|  |  | 0x19 | Rear Rrwd Offset |
|  |  | 0x1A | Front Frwd Offset |
|  |  | 0x1B | Front Rrwd Offset |
|  |  | 0x1C | Rear FL Corner |
|  |  | 0x1D | Rear FR Corner |
|  |  | 0x1E | Rear RL Corner |
|  |  | 0x1F | Rear RR Corner |
|  |  | 0x20 | Front FL Corner |
|  |  | 0x21 | Front FR Corner |
|  |  | 0x22 | Front RL Corner |
|  |  | 0x23 | Front RR Corner |
|  |  | 0x24 | Rock Crawl Front |
|  |  | 0x25 | Rock Crawl Rear |
|  |  | 0x26 | Hitch |
|  |  | 0x27 | Rear Camera On Demand (360) |
|  |  | 0x51 | Left side |
|  |  | 0x52 | Right side |
|  |  | 0x53 | 2D |
|  |  | 0x54 | 3D |
|  |  | 0x55 | 3D front |
|  |  | 0x56 | 3D rear |
|  |  | 0x57 | 3D left side |
|  |  | 0x58 | 3D right side |

The Main Image consists of the following views：

P/N/D/R to offer same view to customer:

2D:

- Rear split 06

- rear normal 04(default in reverse)

- front split (09)

- front normal (07 default in P/N/D)

- left side view (51)

- right side view (52)

- front rock crawl (24)

- rear rock crawl (25)

3D:

- rear 3D (56)

- front 3D (55)

- left 3D (57)

- Right 3D (58)

### RVCv2-IIR-REQ-014081/A-RearViewCameraClient_Tx (TcSE ROIN-282395-3)

#### MD-REQ-014068/A-Feature_Rq (TcSE ROIN-282333-2)

Message Type: Request

Represents the request to command a feature change (select new feature, change feature setting, query features, etc.).

Included Parameters:

Operation

FeatureID

Configuration

PersIndex

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Operation | - | - | Type of operation being requested |
|  | Null | 0x0 |  |
|  | Query | 0x1 |  |
|  | Set | 0x2 |  |
|  | Upload | 0x3 |  |
|  | Restore | 0x4 |  |
|  | Copy | 0x5 |  |
|  | NotUsed | 0x6 – 0x7 |  |
| FeatureID | - | - | Feature number being requested |
|  |  | 0x0000 – 0xFFFF |  |
| Configuration | - | - | Configuration value being requested |
|  |  | 0x0000 – 0xFFFF |  |
| PersIndex | - | - | Indicates which personality profile is being accessed |
|  | PERS_1 | 0x0 |  |
|  | PERS_2 | 0x1 |  |
|  | PERS_3 | 0x2 |  |
|  | PERS_4 | 0X3 |  |
|  | VEHICLE | 0X4 |  |
|  | Not Used | 0x5 |  |
|  | Not Used | 0x6 |  |
|  | Not Used | 0x7 |  |

### RVCv2-IIR-REQ-014082/C-RearViewCameraClient_Rx (TcSE ROIN-282396-3)

#### MD-REQ-014070/A-Feature_St (TcSE ROIN-282399-2)

Message Type: Status

Represents the current status of a feature (feature selected, feature setting, etc.).

Included Parameters:

FeatureID

Configuration

PersIndex

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| FeatureID | - | - | Active feature number |
|  |  | 0x0000 – 0xFFFF |  |
| Configuration | - | - | Active configuration value |
|  |  | 0x0000 – 0xFFFF |  |
| PersIndex | - | - | Indicates which personality profile is active |
|  | PERS_1 | 0x0 |  |
|  | PERS_2 | 0x1 |  |
|  | PERS_3 | 0x2 |  |
|  | PERS_4 | 0X3 |  |
|  | VEHICLE | 0X4 |  |
|  | Not Used | 0x5 |  |
|  | Not Used | 0x6 |  |
|  | Not Used | 0x7 |  |

#### MD-REQ-014083/A-PJB_Bootlid_St (TcSE ROIN-282394-1)

Message Type: Status

Vehicle status message to indicate when the decklid/liftgate is ajar.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | TrunkClosed | 0x0 |  |
|  | TrunkOpen | 0x1 |  |

#### MD-REQ-014023/A-GearLvrPos_D_Actl (TcSE ROIN-266648-1)

Message Type: Status

Vehicle status signal for the Gear Lever Position on an automatic transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Park | 0x0 |  |
|  | Reverse | 0x1 |  |
|  | Neutral | 0x2 |  |
|  | Drive | 0x3 |  |
|  | Sport_DriveSport | 0x4 |  |
|  | Low | 0x5 |  |
|  | First | 0x6 |  |
|  | Second | 0x7 |  |
|  | Third | 0x8 |  |
|  | Fourth | 0x9 |  |
|  | Fifth | 0xA |  |
|  | Sixth | 0xB |  |
|  | Undefined_Treat_as_Fault | 0xC |  |
|  | Undefined_Treat_as_Fault1 | 0xD |  |
|  | Unknown_Position | 0xE |  |
|  | Fault | 0xF |  |

#### MD-REQ-014024/A-GearRvrseActv_D_Actl (TcSE ROIN-266649-1)

Message Type: Status

Vehicle status signal for notifying that Reverse Gear is engaged on a manual transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |
|  | Unknown | 0x2 |  |
|  | Fault | 0x3 |  |

#### MD-REQ-014025/A-VehicleSpeed_St (TcSE ROIN-223023-1)

Message Type: Status

Status used to indicate vehicle speed.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Indicates vehicle speed. Unit: kph Resolution:0.01 Offset:0 |
|  | kph | 0x0 to 0xFFFF |  |

#### MD-REQ-014084/B-ParkBrake_St (TcSE ROIN-287064-1)

Message Type: Status

Signal used to indicate the Parking Brake status.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |

#### MD-REQ-014085/B-ElectronicParkBrake_St (TcSE ROIN-287065-1)

Message Type: Status

Signal used to indicate the Electronic Parking Brake status.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | NotUsed | 0x0 |  |
|  | Rear_Caliper_Closed | 0x1 |  |
|  | Rear_Caliper_Transition | 0x2 |  |
|  | RWU_By_EPB_Active | 0x3 |  |
|  | Rear_Caliper_Open | 0x4 |  |
|  | EPB_Limphome_Active | 0x5 |  |
|  | ECD_by_Brake_ECU_Active | 0x6 |  |
|  | GeneralFault_MaintenceMode | 0x7 |  |

Electronic Park Brake is considered active when the signal has a value of 0x1 (Rear_Caliper_Closed)

#### MD-REQ-014071/B-CameraServerButton_St (TcSE ROIN-287063-1)

Message Type: Status

Signal used to indicate Front Camera Hard Button status. Real Can name: CamraFrntStat_D_Stat

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| ButtonState | - | - | - |
|  | Off | 0x0 |  |
|  | On | 0x1 |  |
|  | NotUsed | 0x2 |  |
|  | NoDataPresent | 0x3 |  |

#### RVC-MD-REQ-292389/B-GearRvrse_D_Actl

GearRvrse_D_Actl

The purpose of this signal is to notify that Reverse Gear is engaged on a manual transmission vehicle.

$0: Inactive_not_confirmed

$1: Inactive_confirmed

$2: Active_not_confirmed

$3: Active_confirmed

$4: NotUsed_1

$5: NotUsed_2

$6: NotUsed_3

$7: Fault

Reverse status is indicated by both $2 (Active_not_confirmed) and $3 (Active_confirmed)

#### RVC-REQ-292387/A-GearPos_D_Trg

GearPos_D_Trg

This signal is used to indicate Gear direction. Used with other gear signals to determine whether or not RVC is to be turned On or Off.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Neutral | 0x0 |  |
|  | First | 0x1 |  |
|  | Second | 0x2 |  |
|  | Third | 0x3 |  |
|  | Fourth | 0x4 |  |
|  | Fifth | 0x5 |  |
|  | Sixth | 0x6 |  |
|  | Seventh | 0x7 |  |
|  | Eighth | 0x8 |  |
|  | Ninth | 0x9 |  |
|  | Tenth | 0xA |  |
|  | Undefined_3 | 0xB |  |
|  | Undefined_4 | 0xC |  |
|  | Undefined_5 | 0xD |  |
|  | Reverse | 0xE |  |
|  | Unknown | 0xF |  |

#### RVC-REQ-292388/A-Veh_V_ActlEng

Veh_V_ActlEng

This signal is used to indicate vehicle speed. Refer to database for proper signal values.

#### MD-REQ-395472/A-GearLvrPos_D_Actl_UB

GearLvrPos_D_Actl_UB : This signal tells to its recipients if GearLvrPos_D_Actl signal has fresh data or unchanged data.

| Parameter | Description |
| --- | --- |
| 0x0 | Unchanged_data |
| 0x1 | Fresh_data |

#### MD-REQ-395473/A-VehVActlEng_D_Qf

VehVActlEng_D_Qf : This signal is used to indicate the quality factor of the signal Veh_V_ActlEng.

| Parameter | Descritpion |
| --- | --- |
| 0x0 | Faulty |
| 0x1 | No_Data_Exists |
| 0x2 | Not_Within_Specifications |
| 0x3 | OK |

#### MD-REQ-395474/A-GearPos_D_Trg_UB

GearPos_D_Trg_UB: This signal tells to its recipients if GearPos_D_Trg signal has fresh data or unchanged data.

| Parameter | Description |
| --- | --- |
| 0x0 | Unchanged_data |
| 0x1 | Fresh_data |

## DriverAssistFrontViewCameraClient Interface

### DAFVCv1-IR-REQ-014066/F-Driver Assist Front View Camera Feature ID Definition (TcSE ROIN-287061)

The below table defines the Feature ID information related to the Feature Based Message Protocol command and control method used in the Driver Assist Front View Camera feature.

The below table defines the Feature ID information related to the Feature Based Message Protocol command and control method used in the Rear View Camera feature.

| Feature Name | Feature ID | Config Number | Config Value |
| --- | --- | --- | --- |
| Camera Showing | 0x081B | 0x00 | Off |
|  |  | 0x01 | Front |
|  |  | 0x02 | Rear |
|  |  | 0x03 | Remote |
|  |  | 0x04 | Rear 360 |
|  |  | 0x05 | Rear Normal |
|  |  | 0x06 | Rear Split |
|  |  | 0x07 | Front 360 |
|  |  | 0x08 | Front Normal |
|  |  | 0x09 | Front Split |
|  |  | 0x0A | Rear Zoom |
|  |  | 0x0B | CHMSL |
|  |  | 0x0C | CHMSL Zoom |
|  |  | 0x0D | AUX |
|  |  | 0x0E | TRG |
|  |  | 0x0F | TRG Rear Normal |
|  |  | 0x10 | Straight Back Up Mode |
|  |  | 0x11 | TBA2 Rear Split |
|  |  | 0x12 | TBA2 Rear 360 |
|  |  | 0x13 | TBA2 CHMSL |
|  |  | 0x14 | TBA2 Aux |
|  |  | 0x15 | TBA2 50/50 |
|  |  | 0x16 | TBA2 Rear Normal |
|  |  | 0x17 | Auto Hitch Rear Normal |
|  |  | 0x18 | Rear Frwd Offset |
|  |  | 0x19 | Rear Rrwd Offset |
|  |  | 0x1A | Front Frwd Offset |
|  |  | 0x1B | Front Rrwd Offset |
|  |  | 0x1C | Rear FL Corner |
|  |  | 0x1D | Rear FR Corner |
|  |  | 0x1E | Rear RL Corner |
|  |  | 0x1F | Rear RR Corner |
|  |  | 0x20 | Front FL Corner |
|  |  | 0x21 | Front FR Corner |
|  |  | 0x22 | Front RL Corner |
|  |  | 0x23 | Front RR Corner |
|  |  | 0x24 | Rock Crawl Front |
|  |  | 0x25 | Rock Crawl Rear |
|  |  | 0x26 | Hitch |
|  |  | 0x27 | Rear Camera on Demand (360) |

In P/N/D, front 360(2D), 2D+3D, rear, left side, right side, front rock crawl, front split

In R: Rear 360(2D), 2D+3D, rear rock crawl, rear split view

### DAFVCv1-IIR-REQ-014067/C-DriverAssistFrontViewCameraClient_Tx (TcSE ROIN-282846-1)

#### MD-REQ-014068/A-Feature_Rq (TcSE ROIN-282333-2)

Message Type: Request

Represents the request to command a feature change (select new feature, change feature setting, query features, etc.).

Included Parameters:

Operation

FeatureID

Configuration

PersIndex

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Operation | - | - | Type of operation being requested |
|  | Null | 0x0 |  |
|  | Query | 0x1 |  |
|  | Set | 0x2 |  |
|  | Upload | 0x3 |  |
|  | Restore | 0x4 |  |
|  | Copy | 0x5 |  |
|  | NotUsed | 0x6 – 0x7 |  |
| FeatureID | - | - | Feature number being requested |
|  |  | 0x0000 – 0xFFFF |  |
| Configuration | - | - | Configuration value being requested |
|  |  | 0x0000 – 0xFFFF |  |
| PersIndex | - | - | Indicates which personality profile is being accessed |
|  | PERS_1 | 0x0 |  |
|  | PERS_2 | 0x1 |  |
|  | PERS_3 | 0x2 |  |
|  | PERS_4 | 0X3 |  |
|  | VEHICLE | 0X4 |  |
|  | Not Used | 0x5 |  |
|  | Not Used | 0x6 |  |
|  | Not Used | 0x7 |  |

#### MD-REQ-331342/B-CamraFrntBttn_D_Stat3

Message Type: Command

CamraFrntBttn_D_Stat3 : Signal is sent by client to server to request camera view activation/deactivation.

The trigger of this signal can be through a soft button press, which is internally to client or through a hard button connected to client through LIN protocol.

When the trigger of this signal is due to hard button LIN switch press, map this signal according to the details in Req 331343 and Req 331344.

Included Parameters:

Not pressed

Pressed

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Operation | - | - |  |
|  | Not pressed | 0x0 | Switch is not pressed |
|  | Pressed | 0x1 | Switch is pressed |

### DAFVCv1-IIR-REQ-014069/D-DriverAssistFrontViewCameraClient_Rx (TcSE ROIN-282847-2)

#### MD-REQ-014070/A-Feature_St (TcSE ROIN-282399-2)

Message Type: Status

Represents the current status of a feature (feature selected, feature setting, etc.).

Included Parameters:

FeatureID

Configuration

PersIndex

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| FeatureID | - | - | Active feature number |
|  |  | 0x0000 – 0xFFFF |  |
| Configuration | - | - | Active configuration value |
|  |  | 0x0000 – 0xFFFF |  |
| PersIndex | - | - | Indicates which personality profile is active |
|  | PERS_1 | 0x0 |  |
|  | PERS_2 | 0x1 |  |
|  | PERS_3 | 0x2 |  |
|  | PERS_4 | 0X3 |  |
|  | VEHICLE | 0X4 |  |
|  | Not Used | 0x5 |  |
|  | Not Used | 0x6 |  |
|  | Not Used | 0x7 |  |

#### MD-REQ-014023/A-GearLvrPos_D_Actl (TcSE ROIN-266648-1)

Message Type: Status

Vehicle status signal for the Gear Lever Position on an automatic transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Park | 0x0 |  |
|  | Reverse | 0x1 |  |
|  | Neutral | 0x2 |  |
|  | Drive | 0x3 |  |
|  | Sport_DriveSport | 0x4 |  |
|  | Low | 0x5 |  |
|  | First | 0x6 |  |
|  | Second | 0x7 |  |
|  | Third | 0x8 |  |
|  | Fourth | 0x9 |  |
|  | Fifth | 0xA |  |
|  | Sixth | 0xB |  |
|  | Undefined_Treat_as_Fault | 0xC |  |
|  | Undefined_Treat_as_Fault1 | 0xD |  |
|  | Unknown_Position | 0xE |  |
|  | Fault | 0xF |  |

#### MD-REQ-014024/A-GearRvrseActv_D_Actl (TcSE ROIN-266649-1)

Message Type: Status

Vehicle status signal for notifying that Reverse Gear is engaged on a manual transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |
|  | Unknown | 0x2 |  |
|  | Fault | 0x3 |  |

#### MD-REQ-014025/A-VehicleSpeed_St (TcSE ROIN-223023-1)

Message Type: Status

Status used to indicate vehicle speed.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Indicates vehicle speed. Unit: kph Resolution:0.01 Offset:0 |
|  | kph | 0x0 to 0xFFFF |  |

#### MD-REQ-014071/B-CameraServerButton_St (TcSE ROIN-287063-1)

Message Type: Status

Signal used to indicate Front Camera Hard Button status. Real Can name: CamraFrntStat_D_Stat

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| ButtonState | - | - | - |
|  | Off | 0x0 |  |
|  | On | 0x1 |  |
|  | NotUsed | 0x2 |  |
|  | NoDataPresent | 0x3 |  |

#### MD-REQ-014020/A-Elocker_St (TcSE ROIN-266643-1)

(CAN Name:  RearDiffLckLamp_D_Rq)

Message Type: Status

Vehicle status signal for displaying the Electric Differential Locker (Elocker) engagement message to the user.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | OFF | 0x0 |  |
|  | ON | 0x1 |  |
|  | Flash | 0x2 |  |
|  | Notused | 0x3 |  |

#### MD-REQ-014021/A-AwdRange_St (TcSE ROIN-266644-1)

(CAN Name:  AwdRnge_D_Actl)

Message Type: Status

Vehicle status signal for the state of the transfer box in terms of range and coupling locking.

"locked" means the  AWD coupling / differential is locked.

"auto" means the AWD coupling / differential is under active control (could be locked, open or anywhere in between depending on system).

"2wd" means the AWD coupling is open.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | LowRangeLocked | 0x0 |  |
|  | LowRangeAuto | 0x1 |  |
|  | LowRange2wd | 0x2 |  |
|  | Neutral | 0x3 |  |
|  | HighRangeLocked | 0x4 |  |
|  | HighRangeAuto | 0x5 |  |
|  | HighRange2wd | 0x6 |  |
|  | Unknown | 0x7 |  |

#### MD-REQ-014022/A-OffRoadMode_St (TcSE ROIN-266645-1)

(CAN Name:  AwdOffRoadMode_D_Stats)

Message Type: Status

Vehicle status signal for the state of Off Road Mode.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | NormalMode | 0x0 |  |
|  | OffRoad | 0x1 |  |
|  | ExtemeOffRoad | 0x2 |  |
|  | Invalid | 0x3 |  |

#### MD-REQ-014024/A-GearRvrseActv_D_Actl (TcSE ROIN-266649-1)

Message Type: Status

Vehicle status signal for notifying that Reverse Gear is engaged on a manual transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |
|  | Unknown | 0x2 |  |
|  | Fault | 0x3 |  |

#### MD-REQ-331343/A-Camera Button LIN signal

Message Type: Command

Represents the state of the physical front camera switch. This signal is the instantaneous status of the LIN switch which is wired to an external ECU. LIN signal name used is ICPBtnID_Camera.

Included Parameters:

Inactive

Active

Short Event

Short Elapsed

Long Event

Stuck

Idle

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Operation | - | - |  |
|  | Inactive | 0 | Switch is not pressed |
|  | Active | 1 | Switch is pressed |
|  | Short Event | 2 | Switch is pressed |
|  | Short Elapsed | 3 | Switch is pressed |
|  | Long Event | 4 | Switch is pressed |
|  | Stuck | 5 | Switch is not pressed |
|  | Not used | 6-14 | Switch is not pressed |
|  | Idle | 15 | Switch is not pressed |

#### MD-REQ-331847/A-CamraFrntOffRd_B_Stat

Message Type: Status

Vehicle status signal as determined by the IPMB for the state of Off Road Mode.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |

#### MD-REQ-395472/A-GearLvrPos_D_Actl_UB

GearLvrPos_D_Actl_UB : This signal tells to its recipients if GearLvrPos_D_Actl signal has fresh data or unchanged data.

| Parameter | Description |
| --- | --- |
| 0x0 | Unchanged_data |
| 0x1 | Fresh_data |

#### MD-REQ-395473/A-VehVActlEng_D_Qf

VehVActlEng_D_Qf : This signal is used to indicate the quality factor of the signal Veh_V_ActlEng.

| Parameter | Descritpion |
| --- | --- |
| 0x0 | Faulty |
| 0x1 | No_Data_Exists |
| 0x2 | Not_Within_Specifications |
| 0x3 | OK |

#### MD-REQ-395474/A-GearPos_D_Trg_UB

GearPos_D_Trg_UB: This signal tells to its recipients if GearPos_D_Trg signal has fresh data or unchanged data.

| Parameter | Description |
| --- | --- |
| 0x0 | Unchanged_data |
| 0x1 | Fresh_data |

## VehicleInformation Interface

### RVCv2-IIR-REQ-014086/D-VehicleInformation_Tx (TcSE ROIN-282400-2)

#### MD-REQ-014083/A-PJB_Bootlid_St (TcSE ROIN-282394-1)

Message Type: Status

Vehicle status message to indicate when the decklid/liftgate is ajar.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | TrunkClosed | 0x0 |  |
|  | TrunkOpen | 0x1 |  |

#### MD-REQ-014023/A-GearLvrPos_D_Actl (TcSE ROIN-266648-1)

Message Type: Status

Vehicle status signal for the Gear Lever Position on an automatic transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Park | 0x0 |  |
|  | Reverse | 0x1 |  |
|  | Neutral | 0x2 |  |
|  | Drive | 0x3 |  |
|  | Sport_DriveSport | 0x4 |  |
|  | Low | 0x5 |  |
|  | First | 0x6 |  |
|  | Second | 0x7 |  |
|  | Third | 0x8 |  |
|  | Fourth | 0x9 |  |
|  | Fifth | 0xA |  |
|  | Sixth | 0xB |  |
|  | Undefined_Treat_as_Fault | 0xC |  |
|  | Undefined_Treat_as_Fault1 | 0xD |  |
|  | Unknown_Position | 0xE |  |
|  | Fault | 0xF |  |

#### MD-REQ-014024/A-GearRvrseActv_D_Actl (TcSE ROIN-266649-1)

Message Type: Status

Vehicle status signal for notifying that Reverse Gear is engaged on a manual transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |
|  | Unknown | 0x2 |  |
|  | Fault | 0x3 |  |

#### MD-REQ-014025/A-VehicleSpeed_St (TcSE ROIN-223023-1)

Message Type: Status

Status used to indicate vehicle speed.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | Indicates vehicle speed. Unit: kph Resolution:0.01 Offset:0 |
|  | kph | 0x0 to 0xFFFF |  |

#### MD-REQ-014084/B-ParkBrake_St (TcSE ROIN-287064-1)

Message Type: Status

Signal used to indicate the Parking Brake status.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |

#### MD-REQ-014085/B-ElectronicParkBrake_St (TcSE ROIN-287065-1)

Message Type: Status

Signal used to indicate the Electronic Parking Brake status.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | NotUsed | 0x0 |  |
|  | Rear_Caliper_Closed | 0x1 |  |
|  | Rear_Caliper_Transition | 0x2 |  |
|  | RWU_By_EPB_Active | 0x3 |  |
|  | Rear_Caliper_Open | 0x4 |  |
|  | EPB_Limphome_Active | 0x5 |  |
|  | ECD_by_Brake_ECU_Active | 0x6 |  |
|  | GeneralFault_MaintenceMode | 0x7 |  |

Electronic Park Brake is considered active when the signal has a value of 0x1 (Rear_Caliper_Closed)

#### RVC-REQ-292387/A-GearPos_D_Trg

GearPos_D_Trg

This signal is used to indicate Gear direction. Used with other gear signals to determine whether or not RVC is to be turned On or Off.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Neutral | 0x0 |  |
|  | First | 0x1 |  |
|  | Second | 0x2 |  |
|  | Third | 0x3 |  |
|  | Fourth | 0x4 |  |
|  | Fifth | 0x5 |  |
|  | Sixth | 0x6 |  |
|  | Seventh | 0x7 |  |
|  | Eighth | 0x8 |  |
|  | Ninth | 0x9 |  |
|  | Tenth | 0xA |  |
|  | Undefined_3 | 0xB |  |
|  | Undefined_4 | 0xC |  |
|  | Undefined_5 | 0xD |  |
|  | Reverse | 0xE |  |
|  | Unknown | 0xF |  |

#### MD-REQ-014024/A-GearRvrseActv_D_Actl (TcSE ROIN-266649-1)

Message Type: Status

Vehicle status signal for notifying that Reverse Gear is engaged on a manual transmission vehicle.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| Type | - | - | - |
|  | Inactive | 0x0 |  |
|  | Active | 0x1 |  |
|  | Unknown | 0x2 |  |
|  | Fault | 0x3 |  |

#### RVC-MD-REQ-292389/B-GearRvrse_D_Actl

GearRvrse_D_Actl

The purpose of this signal is to notify that Reverse Gear is engaged on a manual transmission vehicle.

$0: Inactive_not_confirmed

$1: Inactive_confirmed

$2: Active_not_confirmed

$3: Active_confirmed

$4: NotUsed_1

$5: NotUsed_2

$6: NotUsed_3

$7: Fault

Reverse status is indicated by both $2 (Active_not_confirmed) and $3 (Active_confirmed)

#### MD-REQ-347573/A-PowerMode

PowerMode

This signal is used in CGEA 1.2 only, not for later architectures.

$0: KeyOut

$1: KeyRecentlyOut

$2: KeyApproved_0

$3: PostAccessory_0

$4: Accessory_1

$5: PostIgnition_1

$6: IgnitionOn_2

$7: Running_2

$8: Not_Used

$9: Crank_3

$A: Not_Used1

#### MD-REQ-395472/A-GearLvrPos_D_Actl_UB

GearLvrPos_D_Actl_UB : This signal tells to its recipients if GearLvrPos_D_Actl signal has fresh data or unchanged data.

| Parameter | Description |
| --- | --- |
| 0x0 | Unchanged_data |
| 0x1 | Fresh_data |

#### MD-REQ-395473/A-VehVActlEng_D_Qf

VehVActlEng_D_Qf : This signal is used to indicate the quality factor of the signal Veh_V_ActlEng.

| Parameter | Descritpion |
| --- | --- |
| 0x0 | Faulty |
| 0x1 | No_Data_Exists |
| 0x2 | Not_Within_Specifications |
| 0x3 | OK |

#### MD-REQ-395474/A-GearPos_D_Trg_UB

GearPos_D_Trg_UB: This signal tells to its recipients if GearPos_D_Trg signal has fresh data or unchanged data.

| Parameter | Description |
| --- | --- |
| 0x0 | Unchanged_data |
| 0x1 | Fresh_data |

## CAMERA-CLD-REQ-412756/A-Camera Client

Camera client is the device where the user can interface with, and request particular views and sees the camera images at.

## CAMERA-CLD-REQ-412757/A-Camera Server

Camera Server the module or modules that collaborate with the client to generate the necessary views. Unlike other servers mentioned in this document, such as Rear View Server or Driver Assist Front View Camera, this is generic, an all-encompassing name which refers to any image type.

## Turn Signal View Logical Mapping Table

The CAN signals mentioned throughout this document shall refer to the CAN signal’s logical name. The logical names shall be mapped to their actual CAN signal names. Please use the table below to perform the mapping. The InfoCAN database file is the master file for the actual CAN signal names. Note: There may be cases where the actual CAN signal name is used in this documentation.

| Logical Name | CAN Signal Name |
| --- | --- |
| LTviewSt | PersTurnSglView_D_Stat |
| LMnuRq | PersTurnSglView_D_RqMnu |
| LSodLSt | SodAlrtLeft_D_Stat |
| LSodRSt | SodAlrtRight_D_Stat |

Table: Logical name/CAN signal mapping

TurnSglView_Left  		FBMP Feature ID 0x081B Config Number 0x2F

TurnSglView_Right		FBMP Feature ID 0x081B Config Number 0x2E

## CTA Split View Logical Mapping Table

The CAN signals mentioned throughout this document shall refer to the CAN signal’s logical name. The logical names shall be mapped to their actual CAN signal names. Please use the table below to perform the mapping. The InfoCAN database file is the master file for the actual CAN signal names. Note: There may be cases where the actual CAN signal name is used in this documentation.

| Logical Name | CAN Signal Name |
| --- | --- |
| CTASVRq | PersCtaSplitView_D_Rq |
| CTASVSt | PersCtaSplitView_D_Stat |
| CTAAlert_St(Left = Off, Right = On) | CtaAlrtRight_D_Stat |
| CTAAlert_St(Left = On, Right = Off) | CtaAlrtLeft_D_Stat |
| LCtaBrkLeft | CtaBrkLeftMsgTxt_B_Rq |
| LCtaBrkRight | CtaBrkRightMsgTxt_B_Rq |

Table: Logical name/CAN signal mapping

## Logical Mapping Table

The CAN signals mentioned in this table are used with a logical name through this documentation. This table contains the real physical name.

| Logical Name | CAN Signal Name |
| --- | --- |
| LTrlrConnect | TrlrISPCnnct_D_Stat |
| LTgateSt | DrTgateMde_D_Stat |

## CAMERA-IIR-REQ-412758/A-Camera Client Tx

### MD-REQ-410094/A-LMnuRq

LMnuRq: This signal is sent from the client to the server to request turning Signal View on, depending on user input.

| Name | Literals | Value | Description |
| --- | --- | --- | --- |
| LMnuRq |  |  |  |
|  | NULL | 0x0 | Do nothing. |
|  |  | 0x1 | Disable. |
|  |  | 0x2 | Enable. |
|  |  | 0x3 | Not Used. |

### MD-REQ-411949/A-CTASVRq

CTASVRq: This signal is sent from the client to the server to request CTA Split View to become enabled or disabled.

| Signal Parameter | Parameter Comment |
| --- | --- |
| 0x0 | Null |
| 0x1 | Disabled |
| 0x2 | Ignore this value |
| 0x3 | Enabled Full |

## CAMERA-IIR-REQ-412759/B-Camera Client Rx

### MD-REQ-410095/A-LTviewSt

LTviewSt: This signal is sent form the server to the client to indicate the state of Turn Signal view function.

| LTviewSt | Literals | Parameters | Description |
| --- | --- | --- | --- |
|  | Null | 0x0 | Do nothing |
|  | Disabled | 0x1 | TSV is disabled |
|  | Enabled | 0x2 | TSV is enabled |
|  | Not Configured | 0x3 | TSV is not configured |

### MD-REQ-410096/A-LSodLSt

LSodLSt: This signal is sent form the server to the client to indicate alert status of left side Blis.

| LSodLSt | Literals | Parameters | Description |
| --- | --- | --- | --- |
|  | Lamp Off | 0x0 |  |
|  | Lamp On | 0x1 |  |
|  | Flash | 0x2 |  |
|  | Bulb proveout | 0x3 |  |

### MD-REQ-410097/A-LSodRSt

LSodRSt: This signal is sent form the server to the client to indicate alert status of right side Blis.

| LSodRSt | Literals | Parameters | Description |
| --- | --- | --- | --- |
|  | Lamp Off | 0x0 |  |
|  | Lamp On | 0x1 |  |
|  | Flash | 0x2 |  |
|  | Bulb proveout | 0x3 |  |

### MD-REQ-411950/A-CTASVSt

CTASVSt: This signal is sent from the server to the client to indicate CTA Split View status.

| Signal Parameter | Parameter Comment |
| --- | --- |
| 0x0 | Null |
| 0x1 | Disabled |
| 0x2 | Ignore this value |
| 0x3 | Enabled Full |

### MD-REQ-419851/A-LTrlrConnect

LTrlrConnect: This signal is sent to client. It indicates trailer connection status.

| Parameter | Parameter Description |
| --- | --- |
| 0x0 | Not Connected: There is no trailer connected to vehicle. |
| 0x1 | Connected: There is a trailer connected to vehicle. |
| 0x2 | NotUsed_1 : Parameter is not currently being used |
| 0x3 | NotUsed_2 : Parameter is not currently being used |

### MD-REQ-421155/A-LTgateSt

LTgateSt: This signal is sent to the client to indicate the status of tailgate.

| Signal Parameter | Parameter Descritpion |
| --- | --- |
| 0x0 | NotAvailable |
| 0x1 | Down |
| 0x2 | Up |
| 0x3 | NotUsed_1 |

# General Requirements

## CAMERA-REQ-331344/B-Button Press Logic

When configured for LIN-based camera button logic, the CAN output shall be populated as follows:

| Switch Configuration (Method 2) | LIN Input | CamraFrntBttn_D_Stat3 |
| --- | --- | --- |
| LIN Camera Switch Not Supported | X | 0x0 (Not pressed) |
| LIN Camera Switch Supported | 0x0 (Inactive) | 0x0 (Not pressed) |
| LIN Camera Switch Supported | 0x1 (Active) | 0x1 (Pressed) |
| LIN Camera Switch Supported | 0x2 (Short Event) | 0x1 (Pressed) |
| LIN Camera Switch Supported | 0x3 (Short Elapsed) | 0x1 (Pressed) |
| LIN Camera Switch Supported | 0x4 (Long Event) | 0x1 (Pressed) |
| LIN Camera Switch Supported | 0x5 (Stuck) | 0x0 (Not pressed) |
| LIN Camera Switch Supported | 0x6-0xE (Not used) | 0x0 (Not pressed) |
| LIN Camera Switch Supported | 0xF (Idle) | 0x0 (Not pressed) |

## CAMERA-REQ-014077/C-Feature Maximum Speed (TcSE ROIN-290556)

The feature maximum speed when displaying a camera image in forward gear shall be as described for each operational scenario below:

- Any camera feature activation attempted by User

  - Rear Camera

Feature maximum speed = 10 kph

  - Off Road Front Camera configured:  Not Available

Feature maximum speed = 10 kph

  - Off Road Front Camera configured:  Available and does NOT meet conditions for Off Road Mode per Determine Off Road Mode requirement

Feature maximum speed = 10 kph

  - Off Road Front Camera configured:  Available and meets conditions for Off Road Mode per Determine Off Road Mode requirement

Feature maximum speed = 20 kph

- Rear Camera Active

Feature maximum speed = 10 kph

- Front Camera Active

  - Off Road Front Camera configured:  Not Available

Feature maximum speed = 10 kph

  - Off Road Front Camera configured:  Available and does NOT meet conditions for Off Road Mode per Determine Off Road Mode requirement

Feature maximum speed = 10 kph

  - Off Road Front Camera configured:  Available and meets conditions for Off Road Mode per Determine Off Road Mode requirement

Feature maximum speed = 24 kph

## DAFVCv1-FUR-REQ-166648/B-Determine Off Road Mode

Off Road Mode is considered active when the vehicle is configured for (Off Road Front Camera and hard camera button strategy) and one of the following conditions are true:

- 4x4 transfer case is locked in 4L (AwdRange_St = 0x0:LowRangeLocked)

or

- Off-Road Mode is on (OffRoadMode_St = 0x1:OffRoad or 0x2:ExtremeOffRoad)

and

E-Locker is locked (Elocker_St = 0x1:ON)

Off Road Mode is considered active when the vehicle is configured for (Off Road Front Camera and soft camera button strategy) and the following condition is true:

CamraFrntOffRd_B_Stat = Active

When transitioning out of Off Road Mode the Driver Assist Front View Camera Client shall check vehicle speed to determine if condition exists that results in Front View Camera turning off.

Example:  Driver Assist Front View Camera Client determines Off Road Mode is no longer active and vehicle speed is 20 kph.  The max speed to exit is now 10 kph so the Client shall exit Front View Camera based on Feature Maximum Speed requirement 014077.

## RVC-FUR-REQ-014087/B-RVC Malfunction (TcSE ROIN-146656-2)

When the RVC Client (RearViewCameraClient) does not detect video present in the signal from the camera it shall set a DTC and the RVC Client shall display camera overlays for T_cameraMalfunctionDelay before displaying an error message allowing the user to acknowledge the video error and revert to the previous screen.  At any time the video signal is detected RVC client should check for Gear position and show the camera.

## DAFVCv1-FUR-REQ-166722/A-DAFVC Malfunction

When the DAFVC Client (DriverAssistFrontViewCameraClient) does not detect video present in the signal from the camera it shall set a DTC and the DAFVC Client shall display camera overlays for T_cameraMalfunctionDelay before displaying an error message allowing the user to acknowledge the video error and revert to the previous screen.  At any time the video signal is detected DAFVC client should check for Gear position and show the camera.

## RVC-TMR-REQ-166649/A-T_cameraMalfunctionDelay

| Name | Description | Units | Range | Resolution | Default |
| --- | --- | --- | --- | --- | --- |
| T_cameraMalfunctionDelay | Time DAFVC or RVC Client should wait before displaying an error message to the user according to RVC-REQ-014087-RVC Malfunction or DAFVC-REQ-166649 DAFVC Malfunction. | sec | 0-30 | 1 | 10 |

## CAMERA-REQ-331863/B-Lin Button Client Operation

When Client receives signal ICPBtnID_Camera  as pressed, it shall send the signal  CamraFrntBttn_D_Stat3 with the value Pressed.

# Functional Definition

## Rear View Camera

### Rear View Camera General Requirements

#### RVC-FUR-REQ-014088/E-Deactivate RVC (TcSE ROIN-293328)

The RVC Client (RearViewCameraClient) shall stop displaying RVC video when one of the following conditions is met:

1. 	Vehicle is shifted out of reverse (Camera Delay = OFF)

2. 	Vehicle is shifted out of reverse (GearLvrPos_D_Actl does not equal Reverse in automatic Transmission vehicle or GearRvrse_D_Actl or GearRvrseActv_D_Actl does not equal active in Manual Transmission vehicle) and vehicle speed > limit per CAMERA-REQ-014077-Feature Maximum Speed (Camera Delay = ON)

3. 	CGEA 1.2:

Power Mode does not equal IgnitionOn_2 or Running_2 or Crank_3

CGEA 1.3:

Ignition_Status does not equal Run

4. 	Vehicle is shifted into Park

- Automatic Transmission vehicle GearLvrPos_D_Actl == 0x0

- Manual Transmission Vehicle with Mechanical Park Brake

GearRvrse_D_Actl == Inactive or GearRvrseActv_D_Actl == Inactive AND PrkBrkActv_B_Actl == Active

- Manual Transmission Vehicle with Electronic Park Brake

GearRvrse_D_Actl == Inactive or GearRvrseActv_D_Actl == Inactive AND PrkBrkStatus == Active

#### RVC-FUR-REQ-014090/I-Display RVC Video (TcSE ROIN-194462-2)

There are two ways for Reverse Detection. What way to use is decided on configuration values.

Reverse Detection NEW:

| GearLvrPos_D_Actl | GearLvrPos_D_Actl_UB | Veh_V_ActlEng | VehVActlEng_D_Qf | GearPos_D_Trg | GearPos_D_Trg_UB | Reverse Engaged |
| --- | --- | --- | --- | --- | --- | --- |
| Missing | Don't Care | Don't Care | Don't Care | Don't Care | Don't Care | OFF |
| !=0x01 (Reverse) | Don't Care | Don't Care | Don't Care | Don't Care | Don't Care | OFF |
| 0x01 (Reverse) | 1 | Missing | Don't Care | Don't Care | Don't Care | ON |
| 0x01 (Reverse) | 1 | Don't Care | Missing | Don't Care | Don't Care | ON |
| 0x01 (Reverse) | 1 | Don't Care | != 0x03 (OK) | Don't Care | Don't Care | ON |
| 0x01 (Reverse) | 1 | <=16.09 | 0x03 (OK) | Don't Care | Don't Care | ON |
| 0x01 (Reverse) | 1 | >16.09 | 0x03 (OK) | 0x0E (Reverse) | 1 | ON |
| 0x01 (Reverse) | 1 | >16.09 | 0x03 (OK) | !=0x0E (Reverse) | Don't Care | OFF |

If none of the conditions above are met, set Reverse Engaged to OFF.

If GearLvrPos_D_Actl_UB or GearPos_D_Trg_UB are not available, Client shall ignore the logic for the update bit signals and only rely on the GearLvrPos_D_Actl and GearPos_D_Trg signals in the logic.

For FNV3:

Update bits are not used so Client shall ignore the logic for the update bit signals and only rely on the GearLvrPos_D_Actl and GearPos_D_Trg signals in the logic.

Note: Existing delay mode behavior should still apply.

Upon detecting the conditions for RVC to be ON, the RVC Client (RearViewCameraClient) shall start a timer (T_minImageDisp) and shall not display the RVC image until the expiration of this timer.  Upon expiration of the timer, the RVC Client shall start another timer (T_maxImageDisp). The RVC Client must display the RVC image prior to the expiration of T_maxImageDisp.

Once the conditions for displaying RVC are no longer applicable the RVC client shall:

1.            Cancel the timer

2.            Not display the RVC image

Reverse Detection LEGACY:

Reverse Detection is determined as mentioned in below table.

| GearLvrPos_D_Actl = 0x1 (Reverse) (automatic transmission) or GearRvrseActv_D_Actl = 0x1 (Active )(manual transmission vehicle and Legacy Message Set) or GearRvrse_D_Actl = 0x3 or 0x2 ( Active Confirmed or Active_not_confirmed) (manual transmission vehicle and New Message Set) | Gear position and Camera Status |
| --- | --- |
| Reverse | Gear is Reverse, Turn Camera On |

Upon detecting the conditions for RVC to be ON, the RVC Client (RearViewCameraClient) shall start a timer (T_minImageDisp) and shall not display the RVC image until the expiration of this timer.  Upon expiration of the timer, the RVC Client shall start another timer (T_maxImageDisp). The RVC Client must display the RVC image prior to the expiration of T_maxImageDisp.

Once the conditions for displaying RVC are no longer applicable the RVC client shall:

1.            Cancel the timer

2.            Not display the RVC image

#### RVC-TMR-REQ-014091/A-T_minImageDisp (TcSE ROIN-264661-1)

| Name | Description | Units | Range | Resolution | Default |
| --- | --- | --- | --- | --- | --- |
| T_minImageDisp | Minimum time RVC Client should wait before displaying the RVC video image to the user according to RVC-GREQ-194462-2-Display RVC Video. | msec | 225-275 | 5 | 250 |

#### RVC-TMR-REQ-014092/A-T_maxImageDisp (TcSE ROIN-264662-1)

| Name | Description | Units | Range | Resolution | Default |
| --- | --- | --- | --- | --- | --- |
| T_maxImageDisp | Maximum time RVC Client should wait before displaying the RVC video image to the user according to RVC-GREQ-194462-2-Display RVC Video. | msec | 450-550 | 5 | 500 |

#### CAMERA-FUR-REQ-014093/B-Camera Image Priority (TcSE ROIN-264652-1)

Once the camera image has been displayed to user, the image shall be maintained as long as the conditions required to be in the particular camera view are present and shall have highest priority:

•	No pop-up screens shall interrupt the video image.

•	Media functions (source change, volume control, etc.) shall be available, but shall not interrupt the image view to the user.

### RVCv2-FUN-REQ-014094/A-Activate/Deactivate Rear View Camera (TcSE ROIN-293352)

#### Use Cases

##### RVC-UC-REQ-014095/A-Activate Rear View Camera (TcSE ROIN-289794)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver activates the Rear View Camera (RVC) by placing the vehicle in Reverse Gear. |
| Post-conditions | The vehicle display shows the RVC image. |
| List of Exception Use Cases | E1 – Rear View Camera Malfunction E2 – Decklid/Liftgate is Ajar while Rear View Camera is ON |
| Interfaces | G-HMI Vehicle System Interface |

##### UC-REQ-238578/A-CHMSL Activation. Soft button press

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - APIM is configured (CHMSL) - Vehicle in Run/Start - Rear view (Aux, Rear Normal, Rear Split-View, Rear 360) is showing and not in CHSML view OR Front view (Front Normal, Front Split-View, Front 360) is showing and not in CHSML view |
| Scenario Description | User presses CHMSL view button Display sends “Camera Showing(81B): 0x0B” |
| Post-conditions | The CHMSL view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-238579/A-AUX Activation. Soft button press

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - APIM is configured (Aux) - Vehicle in Run/Start - Rear view (CHMSL, Rear Normal, Rear Split-View, Rear 360) is showing and not in Aux view OR Front view (Front Normal, Front Split-View, Front 360) is showing and not in Aux view |
| Scenario Description | User presses Aux view button Display sends “Camera Showing(81B): 0x0D” |
| Post-conditions | The Aux view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### RVC-UC-REQ-014096/A-Rear View Camera Malfunction (TcSE ROIN-289795)

Linked Elements

RVC-UC-REQ-014095/A-Activate Rear View Camera (TcSE ROIN-289794)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The HMI interface indicates that the Rear View Camera (RVC) image cannot be shown because of a malfunction. |
| Post-conditions | The vehicle display is NOT showing RVC image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver deactivates the Rear View Camera (RVC) by shifting the vehicle out of Reverse Gear. |
| Post-conditions | The vehicle display is NOT showing RVC image. |
| List of Exception Use Cases | E1 – Rear Camera Delay Mode is On E2 – Active Park Assist is Active E3 – Trailer Backup Assist is Active (N/A for stand-alone RVC) |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014099/B-Rear Camera Delay Mode is On (TcSE ROIN-289798)

Linked Elements

RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

RVC-UC-REQ-128280/A-Deactivate Rear View Camera

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The driver shifts out of Reverse Gear and into any gear other than Park. The RVC image remains displayed to the driver until the vehicle reaches limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display stops showing Rear View Camera image when vehicle speed reaches limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014101/B-Trailer Backup Assist is Active (TcSE ROIN-290555)

Linked Elements

RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

RVC-UC-REQ-128280/A-Deactivate Rear View Camera

DAFVCv1-UC-REQ-128313/A-Deactivate Driver Assist Front View Camera

DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The driver shifts out of Reverse Gear and into any other gear while Trailer Backup Assist (TBA) is active. The camera image feed remains displayed to the driver as long as TBA is active and vehicle speed does not exceed limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display stops showing Rear View Camera image when TBA is no longer active or vehicle speed exceeds limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### RVCv2-ACT-REQ-014102/A-Activate Rear View Camera (TcSE ROIN-282388-2)

Linked Elements

RVCv2-SD-REQ-014104/B-Activate Rear View Camera (TcSE ROIN-282316-2)

Activity Diagram

###### RVCv2-ACT-REQ-014103/A-Deactivate Rear View Camera (TcSE ROIN-286992-1)

Linked Elements

RVCv2-SD-REQ-014105/B-Deactivate Rear View Camera (TcSE ROIN-282323-2)

Activity Diagram

##### Sequence Diagrams

###### RVCv2-SD-REQ-014104/B-Activate Rear View Camera (TcSE ROIN-282316-2)

Scenario

Normal Usage

The user activates the Rear View Camera (RVC) by placing the vehicle in Reverse Gear.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

HMI Display shows the RVC image

Sequence Diagram

###### RVCv2-SD-REQ-014105/B-Deactivate Rear View Camera (TcSE ROIN-282323-2)

Scenario

Normal Usage

The user deactivates the Rear View Camera (RVC) by taking the vehicle out of Reverse Gear.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

HMI Display stops showing the RVC image

Sequence Diagram

### RVCv3-FUN-REQ-128277/A-Activate/Deactivate Rear View Camera v3

#### Use Cases

##### RVCv3-UC-REQ-127872/A-Activate Rear Multicamera View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START |
| Scenario Description | The driver shifts gear to reverse for 250ms |
| Post-conditions | The Rear Multicamera view appears with overlays and review button |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014097/A-Decklid/Liftgate is Ajar while Rear View Camera is ON (TcSE ROIN-289796)

Linked Elements

RVC-UC-REQ-014095/A-Activate Rear View Camera (TcSE ROIN-289794)

RVC-UC-REQ-128278/A-Activate Rear View Camera

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The HMI interface indicates that the Decklid/Liftgate is Ajar. |
| Post-conditions | The vehicle display shows the Rear View Camera image. The video feed from the Rear View Camera contains an image without guideline overlays. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver deactivates the Rear View Camera (RVC) by shifting the vehicle out of Reverse Gear. |
| Post-conditions | The vehicle display is NOT showing RVC image. |
| List of Exception Use Cases | E1 – Rear Camera Delay Mode is On E2 – Active Park Assist is Active E3 – Trailer Backup Assist is Active (N/A for stand-alone RVC) |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014099/B-Rear Camera Delay Mode is On (TcSE ROIN-289798)

Linked Elements

RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

RVC-UC-REQ-128280/A-Deactivate Rear View Camera

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The driver shifts out of Reverse Gear and into any gear other than Park. The RVC image remains displayed to the driver until the vehicle reaches limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display stops showing Rear View Camera image when vehicle speed reaches limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128191/A-Enter CHMSL Delay Mode

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehcile is configured with CHMSL camera - The vehicle is in Run/Start - CHMSL or CHMSL Zoom shown - Camera Delay is set On |
| Scenario Description | The driver shifts from Reverse to Neutral or Drive |
| Post-conditions | The vehicle shows CHMSL without zoom button |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014100/B-Active Park Assist is Active (TcSE ROIN-290554)

Linked Elements

RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

RVC-UC-REQ-128280/A-Deactivate Rear View Camera

DAFVCv1-UC-REQ-128313/A-Deactivate Driver Assist Front View Camera

DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The driver shifts out of Reverse Gear and into any other gear while Active Park Assist (APA) is active. The camera image feed remains displayed to the driver as long as APA is active and vehicle speed does not exceed limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display stops showing Rear View Camera image when APA is no longer active or vehicle speed exceeds limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### RVC-UC-REQ-014101/B-Trailer Backup Assist is Active (TcSE ROIN-290555)

Linked Elements

RVC-UC-REQ-014098/A-Deactivate Rear View Camera (TcSE ROIN-289797)

RVC-UC-REQ-128280/A-Deactivate Rear View Camera

DAFVCv1-UC-REQ-128313/A-Deactivate Driver Assist Front View Camera

DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case. |
| Scenario Description | The driver shifts out of Reverse Gear and into any other gear while Trailer Backup Assist (TBA) is active. The camera image feed remains displayed to the driver as long as TBA is active and vehicle speed does not exceed limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display stops showing Rear View Camera image when TBA is no longer active or vehicle speed exceeds limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

##### TRG-UC-REQ-102959/A-Vehicle Not in RUN/START

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as normal usage use case |
| Scenario Description | Driver moves the ignition out of Run/Start |
| Post-conditions | The vehicle returns to non-TRG behavior |
| List of Exception Use Cases | N/A |
| Interfaces | G-HMI Vehicle System Interface |

##### TRG-UC-REQ-102960/A-Loss of communication with IPMB Module

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as normal usage use case |
| Scenario Description | Driver attempts to do something in TRG and APIM has lost communication with IPMB |
| Post-conditions | The vehicle shows camera view, without any overlays |
| List of Exception Use Cases | N/A |
| Interfaces | G-HMI Vehicle System Interface |

##### TRG-UC-REQ-102961/A-Valid Camera Video Signal not present

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as normal usage use case |
| Scenario Description | Driver attempts to do something in TRG and APIM is not receiving valid video |
| Post-conditions | The vehicle returns to non-TRG behavior |
| List of Exception Use Cases | N/A |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### RVCv2-ACT-REQ-014102/A-Activate Rear View Camera (TcSE ROIN-282388-2)

Linked Elements

RVCv2-SD-REQ-014104/B-Activate Rear View Camera (TcSE ROIN-282316-2)

Activity Diagram

###### RVCv2-ACT-REQ-014103/A-Deactivate Rear View Camera (TcSE ROIN-286992-1)

Linked Elements

RVCv2-SD-REQ-014105/B-Deactivate Rear View Camera (TcSE ROIN-282323-2)

Activity Diagram

##### Sequence Diagrams

###### RVCv2-SD-REQ-014104/B-Activate Rear View Camera (TcSE ROIN-282316-2)

Scenario

Normal Usage

The user activates the Rear View Camera (RVC) by placing the vehicle in Reverse Gear.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

HMI Display shows the RVC image

Sequence Diagram

###### RVCv2-SD-REQ-014105/B-Deactivate Rear View Camera (TcSE ROIN-282323-2)

Scenario

Normal Usage

The user deactivates the Rear View Camera (RVC) by taking the vehicle out of Reverse Gear.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

HMI Display stops showing the RVC image

Sequence Diagram

### RVCv2-FUN-REQ-014106/A-Rear View Camera Zoom Setting (TcSE ROIN-293363)

#### Use Cases

##### RVC-UC-REQ-014107/A-Select Manual Zoom Level X (TcSE ROIN-289799)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle display is showing the Rear View Camera image. |
| Scenario Description | The driver activates Manual Zoom Mode Level X via the HMI interface. |
| Post-conditions | The vehicle display continues to show the Rear View Camera image. The vehicle display indicates that a zoom level is selected. The video feed from the Rear View Camera contains a zoomed-in image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |
| Notes | There are three defined zoom levels and "Level X" is used to generically designate that one of the three is selected as described in this use case. Refer to HMI documentation (requirements and/or screen-flow) for which level(s) of zoom will be utilized. |

##### RVC-UC-REQ-014108/A-Deactivate Manual Zoom (TcSE ROIN-289802)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle display is showing the Rear View Camera image with Zoom Level X selected. |
| Scenario Description | The user deactivates Manual Zoom Mode via HMI interface. |
| Post-conditions | The vehicle display continues to show the Rear View Camera image. The vehicle display indicates that no zoom level is selected. The video feed from the Rear View Camera contains a normal (no zoom applied) image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |
| Notes | There are three defined zoom levels and "Level X" is used to generically designate that one of the three is selected as described in this use case. Refer to HMI documentation (requirements and/or screen-flow) for which level(s) of zoom will be utilized. |

#### White Box View

##### Activity Diagrams

###### RVCv2-ACT-REQ-014109/A-Manual Zoom (TcSE ROIN-282390-1)

Linked Elements

RVCv2-SD-REQ-014110/A-Manual Zoom (TcSE ROIN-282356-1)

Activity Diagram

##### Sequence Diagrams

###### RVCv2-SD-REQ-014110/A-Manual Zoom (TcSE ROIN-282356-1)

Scenario

Normal Usage

The user changes the manual zoom level setting via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Pre-condition

HMI Display is showing RVC Image.

Post-condition

The HMI display is showing the Rear Video Camera image that reflects the updated zoom setting.

Sequence Diagram

### RVCv3-FUN-REQ-127106/A-Rear View Camera Zoom Setting v3

#### Use Cases

##### RVCv3-UC-REQ-128173/A-Press Zoom Button from Rear 360 View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - In reverse and in Multicamera RVC view |
| Scenario Description | The driver presses the Zoom button |
| Post-conditions | The RVC zoom view appears with Multicamera view still being highlighted |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128176/A-Press Zoom Button from Rear Normal View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with camera - The vehicle is in RUN/START - RVC view shown |
| Scenario Description | The Driver presses the Zoom button |
| Post-conditions | The RVC zoom view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128179/A-Press Zoom Button from CHMSL View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicke is configured with CHMSL camera - The vehicle is in Run/Start - The vehicle in Reverse - CHMSL View is shown |
| Scenario Description | The driver presses Zoom Button |
| Post-conditions | CHMSL Zoom view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128188/A-Press Rear 360 Unzoom

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in Run/Start - Rear Zoom Shown (from Rear Multicamera) |
| Scenario Description | The driver presses soft Button to unzoom |
| Post-conditions | Rear Multicamera shows |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128189/A-Press Rear Normal Unzoom

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with any camera - The vehicle is in Run/Start - Rear Zoom Shown (from Rear Normal) |
| Scenario Description | The driver presses soft Button to unzoom |
| Post-conditions | Rear Normal shows |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128190/A-Press CHMSL Unzoom

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with CHMSL camera - The vehicle is in Run/Start - CHMSL Zoom Shown |
| Scenario Description | The driver presses soft Button to unzoom |
| Post-conditions | Rear Normal shows |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### RVCv3-ACT-REQ-127095/A-Manual Zoom

Linked Elements

RVCv2-SD-REQ-014110/A-Manual Zoom (TcSE ROIN-282356-1)

Activity Diagram

##### Sequence Diagrams

###### RVCv3-SD-REQ-127097/A-Manual Zoom

Scenario

Normal Usage

The user changes the manual zoom setting via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Pre-condition

HMI Display is showing RVC Image.

Post-condition

The HMI display is showing the Rear Video Camera image that reflects the updated zoom setting.

Sequence Diagram

### RVCv2-FUN-REQ-014111/A-Rear View Camera Delay Mode Setting (TcSE ROIN-293368)

#### Use Cases

##### RVC-UC-REQ-014112/A-Activate/Deactivate Rear Camera Delay (TcSE ROIN-289803)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver activates/deactivates the Rear View Camera (RVC) Delay Mode via the HMI interface. |
| Post-conditions | The RVC Delay Mode is activated/deactivated. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

### RVCv2-FUN-REQ-014113/A-Camera Image View (TcSE ROIN-293370)

#### Use Cases

##### RVCv2-UC-REQ-014114/A-Select Rear Camera Split-View (TcSE ROIN-289805)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle display is showing the Rear View Camera image. |
| Scenario Description | The driver selects split-view image via hard switch interface. |
| Post-conditions | The vehicle display continues to show the Rear View Camera image. The video feed from the Rear View Camera contains a split-view image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### RVCv2-UC-REQ-014115/A-Select Rear Camera 360-View (TcSE ROIN-289806)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle display is showing the Rear View Camera image. |
| Scenario Description | The driver selects 360-view image via hard switch interface. |
| Post-conditions | The vehicle display continues to show the Rear View Camera image. The video feed from the Rear View Camera contains a 360-view image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### RVCv2-UC-REQ-014116/A-Select Rear Camera Normal View (TcSE ROIN-289807)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle display is showing the Rear View Camera image. |
| Scenario Description | The driver selects normal view image via hard switch interface. |
| Post-conditions | The vehicle display continues to show the Rear View Camera image. The video feed from the Rear View Camera contains a normal view image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### RVCv2-ACT-REQ-014117/A-Change Camera View Setting (TcSE ROIN-282392-2)

Linked Elements

RVCv2-SD-REQ-014118/A-Change Camera View Setting (TcSE ROIN-282363-3)

Activity Diagram

##### Sequence Diagrams

###### RVCv2-SD-REQ-014118/A-Change Camera View Setting (TcSE ROIN-282363-3)

Scenario

Normal Usage

The user selects alternate view image setting via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

The HMI display is showing the Rear Video Camera image that reflects the updated alternate view setting.

Sequence Diagram

### RVCv3-FUN-REQ-127111/A-Camera Image View v3

#### Use Cases

##### RVCv3-UC-REQ-127874/A-Press Rear 360 View Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Reverse camera shown and not in Rear Multicamera view. |
| Scenario Description | The driver presses Multicamera rear button |
| Post-conditions | Multicamera RVC view appears with overlays |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128175/A-Press RVC Normal View Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with non- Multicamera - The vehicle is in RUN/START - In reverse and not in RVC view |
| Scenario Description | The Driver presses the RVC button |
| Post-conditions | RVC view appears with overlays |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128177/A-Press RVC Split View Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured as Multicamera with Rear Split View - The Vehicle is in Run/Start - RVC View being shown, and not in RVC Split view |
| Scenario Description | The Driver presses RVC Split view button |
| Post-conditions | RVC Split view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128178/A-Press CHMSL Camera View Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicke is configured with CHMSL camera - Vehicle is in Run/Start - Rear Camera shown and Not in CHMSL |
| Scenario Description | The driver presses CHMSL view button |
| Post-conditions | CHMSL view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128180/B-Press Aux Camera view Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Vehicle in Run/Start - Vehicle is in Reverse, Park, Netural, or Drive - The CHMSL view is not showing - Vehicle below 10kph |
| Scenario Description | - User presses camera hard button IPMB sends “Camera Showing (81B): 0x0D” |
| Post-conditions | The Aux view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128181/A-Press Trailer Reverse Guidance Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with TRG - The vehicle is in Run/Start - The vehicle is in Reverse - TRG is not activated |
| Scenario Description | The driver presses TRG view button |
| Post-conditions | TRG scenarios enter; see TRG Use cases for more details |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128396/B-Press Hard Button to Enter CHMSL View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Vehicle in Run/Start - Vehicle is in Reverse, Park, Neutral, or Drive. - The CHMSL view is not showing. - Vehicle below 10kph. |
| Scenario Description | - User presses camera hard button - IPMB sends “Camera Showing (81B): 0x0D” |
| Post-conditions | CHMSL view is shown |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### RVCv3-UC-REQ-128397/A-Press Hard Button to Enter RVC View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with CHMSL, Rear Camera and Soft button strategy - The vehicle is in Run/Start - CHMSL or CHMSL Zoom is shown |
| Scenario Description | The driver presses Hard Camera Button to enter Rear Normal View |
| Post-conditions | Rear Normal shows |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### RVCv3-ACT-REQ-127096/A-Change Camera View Setting

Linked Elements

RVCv2-SD-REQ-014118/A-Change Camera View Setting (TcSE ROIN-282363-3)

Activity Diagram

##### Sequence Diagrams

###### RVCv3-SD-REQ-127098/A-Change Camera View Setting

Scenario

Normal Usage

The user selects alternate view image setting via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

The HMI display is showing the Rear Video Camera image that reflects the updated alternate view setting.

Sequence Diagram

### RVCv2-FUN-REQ-014119/A-Activate/Deactivate Enhanced Park Aids (TcSE ROIN-293375)

#### Requirements

##### RVCv2-REQ-014120/A-Visual Park Aid Alert Availability (TcSE ROIN-281600-2)

The Rear Video Camera and the HMI Display must both be configured with Park Aid Module present in order to enable the Visual Park Aid Alert setting options (HMI refers to this as Enhanced Park Aids and this refers to Visual Park Aid Alert or Visual Park Assist depending on vehicle configuration. See HMI specification for implementation details).

#### Use Cases

##### RVC-UC-REQ-014121/A-Activate/Deactivate Enhanced Park Aids (TcSE ROIN-289804)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver activates/deactivates the Enhanced Park Aids via the HMI interface. |
| Post-conditions | The Enhance Park Aids are activated/deactivated. The HMI indicates the setting change determined by vehicle system interface signal. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### RVCv2-ACT-REQ-014122/A-Activate/Deactivate Enhanced Park Aids (TcSE ROIN-286990-1)

Linked Elements

RVCv2-SD-REQ-014123/A-Activate/Deactivate Enhanced Park Aids (TcSE ROIN-286982-2)

Activity Diagram

##### Sequence Diagrams

###### RVCv2-SD-REQ-014123/A-Activate/Deactivate Enhanced Park Aids (TcSE ROIN-286982-2)

Scenario

Normal Usage

The user activates/deactivates Enhanced Park Aids via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

Enhanced Park Aids are enabled/disabled.

Sequence Diagram

### RVCv2-FUN-REQ-014124/A-Camera Overlay Setting (TcSE ROIN-293357)

#### Requirements

##### RVC-FUR-REQ-014125/A-Guideline Availability (TcSE ROIN-264660-2)

	In order for Active (Dynamic) Guidelines to be available, the vehicle must be equipped with an Absolute Steering Angle Sensor and both the Rear Video Camera and the HMI Display must be configured with Absolute Steering Angle Sensor present.

	When Active (Dynamic) Guidelines are active, Fixed (Static) Guidelines must also be active.

	When Fixed (Static) Guidelines are inactive, Active (Dynamic) Guidelines must also be inactive.

#### Use Cases

##### RVCv2-UC-REQ-014126/A-Select Global Overlay Setting (All Guidelines) (TcSE ROIN-289808)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver activates/deactivates all Rear View Camera overlays (guidelines) via the HMI interface. |
| Post-conditions | The vehicle display indicates the selected overlay setting. The video feed from the Rear View Camera contains an image with or without all guideline overlays as indicated by the selected setting. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |
| Notes | Use Case is optional. Current direction is to not apply guideline related use cases. |

##### RVCv2-UC-REQ-014127/A-Select Active Guideline Setting (TcSE ROIN-289809)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver activates/deactivates the Active (Dynamic) Guidelines Setting via the HMI interface. |
| Post-conditions | The vehicle display indicates the selected Active Guideline setting. The video feed from the Rear View Camera contains an image with or without Active guideline overlays as indicated by the selected setting. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |
| Notes | Use Case is optional. Current direction is to not apply guideline related use cases. |

##### RVCv2-UC-REQ-014128/A-Select Fixed Guideline Setting (TcSE ROIN-289810)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. |
| Scenario Description | The driver activates/deactivates the Fixed (Static) Guidelines Setting via the HMI interface. |
| Post-conditions | The vehicle display indicates the selected Fixed Guideline setting. The video feed from the Rear View Camera contains an image with or without Fixed guideline overlays as indicated by the selected setting. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Vehicle System Interface |
| Notes | Use Case is optional. Current direction is to not apply guideline related use cases. |

#### White Box View

##### Activity Diagrams

###### RVCv2-ACT-REQ-014129/A-Change Camera Overlay Setting (TcSE ROIN-282386-2)

Linked Elements

RVCv2-SD-REQ-014130/A-Activate Overlays (TcSE ROIN-282334-2)

RVCv2-SD-REQ-014131/A-Deactivate Overlays (TcSE ROIN-282341-2)

Activity Diagram

##### Sequence Diagrams

###### RVCv2-SD-REQ-014130/A-Activate Overlays (TcSE ROIN-282334-2)

Scenario

Normal Usage

The user activates Rear View Camera overlays via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

Rear View Camera overlays are enabled.

Sequence Diagram

###### RVCv2-SD-REQ-014131/A-Deactivate Overlays (TcSE ROIN-282341-2)

Scenario

Normal Usage

The user deactivates Rear View Camera overlays via the HMI interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

Rear View Camera overlays are disabled.

Sequence Diagram

## Driver Assist Front View Camera

### DAFVCv1-FUN-REQ-014045/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-293385)

#### Requirements

##### DAFVCv1-FUR-REQ-166641/A-Deactivate FVC

The DriverAssistFrontViewCameraClient shall stop displaying Front View Camera (FVC) image when one of the following conditions is met:

•	Vehicle is shifted into 'Reverse' (Rear View Camera image will be shown)

•	Vehicle is shifted into 'Park' (No Image shown in 'Park')

•	Power Mode does not equal IgnitionOn_2 or Running_2 or Crank_3

• 	FVC is disabled (Feature_St equals 0x81B Camera Showing and 0x00 Off)

•	Vehicle speed exceeds Feature Maximum Speed requirement 014077

##### DAFVCv1-FUR-REQ-013997/B-E-Locker Deactivation Delay (TcSE ROIN-266607-1)

If the FrontViewCameraClient is displaying the Front View Camera (FVC) image (all conditions for front camera image display are true) and the E-Locker transistions from 'Locked' (Elocker_St = 0x1:ON) to 'Unlocked' (Elocker_St = 0x0:OFF), the FrontViewCameraClient shall start a timer (T_eLockerDelay) and shall continue to display the FVC image until the expiration of this timer.

If the FrontViewCameraClient detects that the E-Locker transistions back to 'Locked' prior to the expiration of T_eLockerDelay, and all other conditions for front camera image remain true, the FrontViewCameraClient shall cancel the timer and continue to display the FVC image.

##### DAFVCv1-TMR-REQ-013998/B-T_eLockerDelay (TcSE ROIN-266609-1)

| Name | Description | Units | Range | Resolution | Default |
| --- | --- | --- | --- | --- | --- |
| T_eLockerDelay | Maximum time FrontViewCameraClient should wait before stopping the display of the FVC video image to the user according to FAS-FVC-GREQ-266607-E-Locker Deactivation Delay. | msec | 4975-5025 | 5 | 5000 |

#### Use Cases

##### DAFVCv1-UC-REQ-014046/B-Activate Driver Assist Front View Camera (TcSE ROIN-290143)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The gear position status is NOT Reverse. |
| Scenario Description | The driver activates the Driver Assist Front View Camera (DAFVC) via hard button interface, or vehicle system indicates DAFVC activation for Trailer Backup Assist, Active Park Assist or Off Road Front Camera. |
| Post-conditions | The vehicle display shows the DAFVC image. |
| List of Exception Use Cases | E1 – Driver Assist Front View Camera Malfunction E2 – Activation Attempt During Overspeed Condition |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### DAFVCv1-UC-REQ-014047/A-Driver Assist Front View Camera Malfunction (TcSE ROIN-290144)

Linked Elements

DAFVCv1-UC-REQ-014046/B-Activate Driver Assist Front View Camera (TcSE ROIN-290143)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case |
| Scenario Description | The driver attempts to activate the Driver Assist Front View Camera (DAFVC) via hard button interface. The HMI interface indicates that the DAFVC image cannot be shown because of a malfunction. |
| Post-conditions | The vehicle display does NOT show the DAFVC image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### DAFVCv1-UC-REQ-014048/B-Activation Attempt During Overspeed Condition (TcSE ROIN-290145)

Linked Elements

DAFVCv1-UC-REQ-014046/B-Activate Driver Assist Front View Camera (TcSE ROIN-290143)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case |
| Scenario Description | The driver attempts to activate the Driver Assist Front View Camera (DAFVC) via hard button interface while traveling at a speed greater than the limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display does NOT show the DAFVC image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle system display is showing front view camera image |
| Scenario Description | The user deactivates the Driver Assist Front View Camera (DAFVC) by one of the following: - Disabling DAFVC via hard button interface - Exceeding the speed limit per CAMERA-REQ-014077-Feature Maximum Speed - Shifting the vehicle into Reverse (RVC image will be shown if conditions for RVC allow it) - Shifting the vehicle into Park (No image shown in ‘Park’) |
| Post-conditions | The vehicle display does NOT show the DAFVC image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### DAFVCv1-ACT-REQ-014043/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-282606-3)

Linked Elements

DAFVCv1-SD-REQ-014051/A-Activation Attempt During Overspeed (TcSE ROIN-287030-1)

DAFVCv1-SD-REQ-014050/C-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2)

Activity Diagram

##### Sequence Diagrams

###### REQ-331856/B-Lin Button Camera Request

###### DAFVCv1-SD-REQ-014050/C-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2)

Sequence Diagram

###### DAFVCv1-SD-REQ-014051/A-Activation Attempt During Overspeed (TcSE ROIN-287030-1)

Scenario

Normal Usage

The user attempts to activate the Driver Assist Front View Camera (DAFVC) by pressing the hard switch while exceeding the speed limit per CAMERA-REQ-014077-Feature Maximum Speed.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

HMI Display shows an over-speed notification.

Sequence Diagram

### DAFVCv2-FUN-REQ-128309/B-Activate/Deactivate Driver Assist Front View Camera v2

#### Requirements

##### DAFVCv1-FUR-REQ-166641/A-Deactivate FVC

The DriverAssistFrontViewCameraClient shall stop displaying Front View Camera (FVC) image when one of the following conditions is met:

•	Vehicle is shifted into 'Reverse' (Rear View Camera image will be shown)

•	Vehicle is shifted into 'Park' (No Image shown in 'Park')

•	Power Mode does not equal IgnitionOn_2 or Running_2 or Crank_3

• 	FVC is disabled (Feature_St equals 0x81B Camera Showing and 0x00 Off)

•	Vehicle speed exceeds Feature Maximum Speed requirement 014077

##### DAFVCv1-FUR-REQ-013997/B-E-Locker Deactivation Delay (TcSE ROIN-266607-1)

If the FrontViewCameraClient is displaying the Front View Camera (FVC) image (all conditions for front camera image display are true) and the E-Locker transistions from 'Locked' (Elocker_St = 0x1:ON) to 'Unlocked' (Elocker_St = 0x0:OFF), the FrontViewCameraClient shall start a timer (T_eLockerDelay) and shall continue to display the FVC image until the expiration of this timer.

If the FrontViewCameraClient detects that the E-Locker transistions back to 'Locked' prior to the expiration of T_eLockerDelay, and all other conditions for front camera image remain true, the FrontViewCameraClient shall cancel the timer and continue to display the FVC image.

##### DAFVCv1-TMR-REQ-013998/B-T_eLockerDelay (TcSE ROIN-266609-1)

| Name | Description | Units | Range | Resolution | Default |
| --- | --- | --- | --- | --- | --- |
| T_eLockerDelay | Maximum time FrontViewCameraClient should wait before stopping the display of the FVC video image to the user according to FAS-FVC-GREQ-266607-E-Locker Deactivation Delay. | msec | 4975-5025 | 5 | 5000 |

#### Use Cases

##### DAFVCv2-UC-REQ-128182/B-Entering Front Camera - 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera or FVC only - The vehicle is in Run/Start - The vehicle is NOT in reverse - The vehicle speed is below limit per CAMERA-REQ-014077-Feature Maximum Speed |
| Scenario Description | The driver presses Front Camera Hard Button |
| Post-conditions | Front Multicamera view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### DAFVCv2-UC-REQ-128184/B-Entering Front Camera – Non-360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - APIM is configured with FVC only (no Multicamera) - Vehicle in Run/Start - Vehicle Not in reverse - Vehicle speed per CAMERA-REQ-014077-Feature Maximum Speed |
| Scenario Description | - User presses Camera Hard Button |
| Post-conditions | Front Normal view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### DAFVCv1-UC-REQ-128312/B-Activation Attempt During Overspeed Condition

Linked Elements

DAFVCv1-UC-REQ-128310/A-Activate Driver Assist Front View Camera

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as Normal Usage Use Case |
| Scenario Description | The driver attempts to activate the Driver Assist Front View Camera (DAFVC) via hard button interface while traveling at a speed greater than the limit per CAMERA-REQ-014077-Feature Maximum Speed. |
| Post-conditions | The vehicle display does NOT show the DAFVC image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### TRG-UC-REQ-102959/A-Vehicle Not in RUN/START

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as normal usage use case |
| Scenario Description | Driver moves the ignition out of Run/Start |
| Post-conditions | The vehicle returns to non-TRG behavior |
| List of Exception Use Cases | N/A |
| Interfaces | G-HMI Vehicle System Interface |

##### TRG-UC-REQ-102960/A-Loss of communication with IPMB Module

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as normal usage use case |
| Scenario Description | Driver attempts to do something in TRG and APIM has lost communication with IPMB |
| Post-conditions | The vehicle shows camera view, without any overlays |
| List of Exception Use Cases | N/A |
| Interfaces | G-HMI Vehicle System Interface |

##### TRG-UC-REQ-102961/A-Valid Camera Video Signal not present

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Same as normal usage use case |
| Scenario Description | Driver attempts to do something in TRG and APIM is not receiving valid video |
| Post-conditions | The vehicle returns to non-TRG behavior |
| List of Exception Use Cases | N/A |
| Interfaces | G-HMI Vehicle System Interface |

##### DAFVCv1-UC-REQ-014049/B-Deactivate Driver Assist Front View Camera (TcSE ROIN-290146)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The vehicle system display is showing front view camera image |
| Scenario Description | The user deactivates the Driver Assist Front View Camera (DAFVC) by one of the following: - Disabling DAFVC via hard button interface - Exceeding the speed limit per CAMERA-REQ-014077-Feature Maximum Speed - Shifting the vehicle into Reverse (RVC image will be shown if conditions for RVC allow it) - Shifting the vehicle into Park (No image shown in ‘Park’) |
| Post-conditions | The vehicle display does NOT show the DAFVC image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### DAFVCv1-ACT-REQ-014043/B-Activate/Deactivate Driver Assist Front View Camera (TcSE ROIN-282606-3)

Linked Elements

DAFVCv1-SD-REQ-014051/A-Activation Attempt During Overspeed (TcSE ROIN-287030-1)

DAFVCv1-SD-REQ-014050/C-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2)

Activity Diagram

##### Sequence Diagrams

###### REQ-331856/B-Lin Button Camera Request

###### DAFVCv1-SD-REQ-014050/C-Activate_Deactivate Driver Asistance Front View Camera (TcSE ROIN-282617-2)

Sequence Diagram

###### DAFVCv1-SD-REQ-014051/A-Activation Attempt During Overspeed (TcSE ROIN-287030-1)

Scenario

Normal Usage

The user attempts to activate the Driver Assist Front View Camera (DAFVC) by pressing the hard switch while exceeding the speed limit per CAMERA-REQ-014077-Feature Maximum Speed.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

HMI Display shows an over-speed notification.

Sequence Diagram

### DAFVCv1-FUN-REQ-014052/A-Driver Assist Front Camera Image View (TcSE ROIN-293390)

#### Use Cases

##### DAFVCv1-UC-REQ-014053/A-Select Front Camera Split-View (TcSE ROIN-290147)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The gear position status is NOT Reverse. The vehicle display is showing the Driver Assist Front View Camera image. |
| Scenario Description | The driver selects split-view image setting via hard switch interface. |
| Post-conditions | The vehicle display continues to show the Driver Assist Front View Camera image. The video feed from the Driver Assist Front View Camera contains a split-view image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### DAFVCv1-UC-REQ-014054/A-Select Front Camera 360-View (TcSE ROIN-290148)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The gear position status is NOT Reverse. The vehicle display is showing the Driver Assist Front View Camera image. |
| Scenario Description | The driver selects 360-view image setting via hard switch interface. |
| Post-conditions | The vehicle display continues to show the Driver Assist Front View Camera image. The video feed from the Driver Assist Front View Camera contains a 360-view image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### DAFVCv1-UC-REQ-014055/A-Select Front Camera Normal View (TcSE ROIN-290149)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The gear position status is NOT Reverse. The vehicle display is showing the Driver Assist Front View Camera image. |
| Scenario Description | The driver selects normal view image setting via hard switch interface. |
| Post-conditions | The vehicle display continues to show the Driver Assist Front View Camera image. The video feed from the Driver Assist Front View Camera contains a normal view image. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

##### DAFVCv1-UC-REQ-014056/A-Select Front Camera Image Off (TcSE ROIN-290150)

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. The gear position status is NOT Reverse. The vehicle display is showing the Driver Assist Front View Camera image. |
| Scenario Description | The driver selects image off setting via hard switch interface. |
| Post-conditions | The vehicle display returns to the state previous to entering Driver Assist Front View Camera. |
| List of Exception Use Cases | NA |
| Interfaces | G-HMI Dedicated Hard Button Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### DAFVCv1-ACT-REQ-014044/A-Change Camera View Setting (TcSE ROIN-282608-2)

Linked Elements

DAFVCv1-SD-REQ-014057/A-Change Camera View Setting (TcSE ROIN-282599-3)

Activity Diagram

##### Sequence Diagrams

###### DAFVCv1-SD-REQ-014057/A-Change Camera View Setting (TcSE ROIN-282599-3)

Scenario

Normal Usage

The user selects alternate view image setting via hard switch interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

The HMI display is showing the Driver Assist Front Video Camera image that reflects the updated alternate view setting.

Sequence Diagram

### DAFVCv2-FUN-REQ-127102/A-Driver Assist Front Camera Image View v2

#### Use Cases

##### DAFVCv2-UC-REQ-128183/A-Press Front 360 View Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera or FVC only - The vehicle is in Run/Start - Front Camera is shown, and not in Front 360 view. |
| Scenario Description | The user presses Front 360 soft Button |
| Post-conditions | Front 360 view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### DAFVCv2-UC-REQ-128185/A-Press Front Normal View Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with front camera - The vehicle is in Run/Start - Front Camera View is shown but not in Front Normal View |
| Scenario Description | The driver presses Front Normal view soft Button |
| Post-conditions | Front Normal View shown. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### DAFVCv2-UC-REQ-128186/A-Press Front Split View

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with front camera - The vehicle is in Run/Start - Front Camera view is shown, but not in Front Split View |
| Scenario Description | Thre driver presses Front Split soft Button |
| Post-conditions | Front Split View shown |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

##### DAFVCv2-UC-REQ-128187/A-Press Rear button From Front camera

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with front camera - The vehicle is in Run/Start - Front Camera view is shown |
| Scenario Description | The Driver presses rear soft Button |
| Post-conditions | Rear Camera View shows |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box View

##### Activity Diagrams

###### DAFVCv2-ACT-REQ-127100/A-Change Camera View Setting

Linked Elements

DAFVCv1-SD-REQ-014057/A-Change Camera View Setting (TcSE ROIN-282599-3)

Activity Diagram

##### Sequence Diagrams

###### DAFVCv2-SD-REQ-127099/A-Change Camera View Setting

Scenario

Normal Usage

The user selects alternate view image setting via soft button interface.

Constraints

Pre-condition

Ignition_Status = Run

Post-condition

The HMI display is showing the Camera image that reflects the updated alternate view setting.

Sequence Diagram

## CAMERA-FUN-REQ-331348/A-Offset View

### CAMERA-UC-REQ-331349/A-Press Rear Offset View Button from Rear 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Rear 360 is shown |
| Scenario Description | The driver presses rear offset view button |
| Post-conditions | Rear RWD Offset View appears with overlays |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

### CAMERA-UC-REQ-331350/A-Press Rear Offset View Button from Front 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Front 360 view is shown |
| Scenario Description | The driver presses rear offset view button |
| Post-conditions | Front RWD Offset View appears with overlays |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

### CAMERA-UC-REQ-331351/A-Press Rear Left Corner View Button from Rear 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Rear 360 camera is shown |
| Scenario Description | The driver presses rear left corner view button |
| Post-conditions | Multicamera Rear_RL_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331352/A-Press Rear Left Corner View Button from Front 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Front 360 camera is shown |
| Scenario Description | The driver presses rear left corner view button |
| Post-conditions | Multicamera Front_RL_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331353/A-Press Rear Right Corner View Button from Rear 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Rear 360 camera is shown |
| Scenario Description | The driver presses rear right corner view button |
| Post-conditions | Multicamera Rear_RR_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331354/A-Press Rear Right Corner View Button from Front 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Front 360 camera is shown |
| Scenario Description | The driver presses rear right corner view button |
| Post-conditions | Multicamera Front_RR_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331356/A-Press Front Offset View Button from Rear 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Rear 360 is shown |
| Scenario Description | The driver presses front offset view button |
| Post-conditions | Rear FWD Offset View appears with overlays |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

### CAMERA-UC-REQ-331360/A-Press Front Offset View Button from Front 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Front 360 view is shown |
| Scenario Description | The driver presses front offset view button |
| Post-conditions | Front FWD Offset View appears with overlays |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | G-HMI Vehicle System Interface |

### CAMERA-UC-REQ-331365/A-Press Front Left Corner View Button from Rear 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Rear 360 camera is shown |
| Scenario Description | The driver presses rear left corner view button |
| Post-conditions | Multicamera Rear_FL_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331369/A-Press Front Left Corner View Button from Front 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Front 360 camera is shown |
| Scenario Description | The driver presses front left corner view button |
| Post-conditions | Multicamera Front_FL_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331373/A-Press Front Right Corner View Button from Rear 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Rear 360 camera is shown |
| Scenario Description | The driver presses front right corner view button |
| Post-conditions | Multicamera Rear_FR_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

### CAMERA-UC-REQ-331374/A-Press Front Right Corner View Button from Front 360

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is configured with Multicamera - The vehicle is in RUN/START - Front 360 camera is shown |
| Scenario Description | The driver presses front right corner view button |
| Post-conditions | Multicamera Front_FR_Corner view appears |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Camera Server E3 – Valid camera video signal not present |
| Interfaces | Vehicle System Interface |

## Views At Speed v1

### CAMERA-FUN-REQ-354982/B-Views At Speed v1

#### Requirements

##### CAMERA-REQ-354992/A-Views At Speed Availability

Views At Speed are a set of views that are available even when vehicle reaches speeds above those mentioned in req 014077, while not in reverse drive.

For some implementations the views will deactivate under the speed limitations of req 014077, the user will be able to reactivate the applicable views again by button press.

The user is able to request a view change and Server will decide on what kind of views to display.

Available views are setup through Method II configuration.

##### CAMERA-REQ-354993/B-Views At Speed v1 List

Views at Speed include the following views:

| View Name | Config Value |
| --- | --- |
| CHMSL | 0x0B |
| Aux | 0x0D |

##### CAMERA-REQ-358450/B-Off Road Mode Views At Speed

When the vehicle is configured for off road front camera and off road mode views at speed and off road mode is active, there is no max speed limit for showing Off Road Front Camera Image.

#### Use Cases

##### CAMERA-UC-REQ-355016/B-Views At Speed Trigger

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | The infotainment system is powered on. The ignition status is Run/Start. Vehicle is in Drive above 10kph. Vehicle is equipped with 360 camera with any or all of the views at speed: Aux, CHMSL. No camera view is active. |
| Scenario Description | Customer presses Camera button. |
| Post-conditions | Client displays available view at speed per camera request. |
| List of Exception Use Cases |  |
| Interfaces | G-HMI Vehicle System Interface |

#### White Box Views

##### Activity Diagram

###### CAMERA-ACT-REQ-358468/B-Views At Speed v1

##### Sequence Diagram

###### CAMERA-SD-REQ-354994/B-Views At Speed v1

## Views At Speed V2

### CAMERA-FUN-REQ-381834/A-Views At Speed V2

This function is about Views at Speed that were implemented for Gen 4 Client. These Views at Speed should not be confused with Views At Speed Function with ID 354982. While their purpose is similar, they have different use cases and different interfaces. Read the rest of the function for further details.

Views at speed are initiated by the server or client when conditions are applicable. Upon receiving the proper signaling, for any of the views, the client shall display the correct view.

#### CAMERA-REQ-381833/A-Views At Speed V2 Enumerations

The below are the configuration numbers associated with Feature ID 0x081B that communicates the different views at speed states and requests between client and server.

| Config Number | View Type |
| --- | --- |
| 0x28 | Hitch at speed |
| 0x29 | RCOD at speed |
| 0x2A | Aux at speed |
| 0x2B | CHMSL at speed |

#### CAMERA-REQ-382183/A-Views at Speed Deactivation

Deactivate views at speed when vehicle shift to park.

Whenever there is an active front camera view and the vehicle shifts to Park, the client must request OFF to the server.

Deactivate views at speed when ignition status goes from RUN to not Run

When the ignition status goes from RUN to something other than RUN, then the client shall turn off the view (front or rear view) and no request for OFF from/to client shall take place.

The views at speed shall deactivate when requested by server.

#### UseCases

##### CAMERA-UC-REQ-381763/A-RCOD Transition from Below Speed to Above Speed on 360 Variants

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for RCOD View, Views at Speed, and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is below 10 KPH - RCOD view is active - Front Camera Menu below speed is displayed. |
| Scenario Description | User accelerates from below 10 KPH to above 10 KPH Server requests RCOD at speed view. |
| Post-conditions | Client shall update view menu. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with server module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-381770/A-Hitch Transition from Below Speed to Above Speed on 360 Variants

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for Hitch View, Views at Speed, and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is below 10 KPH - Hitch view is the active view - Front Camera Menu below speed is displayed. |
| Scenario Description | User accelerates from below 10 KPH to above 10 KPH. Server requests Hitch at speed view. |
| Post-conditions | Client shall update view menu. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with server module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-381808/A-Aux Transition from Below Speed to Above Speed on 360 Variants

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for Aux view, Views at Speed and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is below 10 KPH - Aux is the active view - Front Camera Menu below speed is displayed. |
| Scenario Description | User accelerates from below 10 KPH to above 10 KPH. Server requests Aux at speed view. |
| Post-conditions | Client shall update view menu. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with server module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-381809/A-CHMSL Transition from Below Speed to Above Speed on 360 Variants

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for CHMSL view, Views at Speed and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is below 10 KPH - CHMSL is the active view - Front Camera Menu below speed is displayed. |
| Scenario Description | User accelerates from below 10 KPH to above 10 KPH. Server requests CHMSL at speed view. |
| Post-conditions | Client shall update view menu. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with server module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-381810/A-Views at Speed above Speed Menu Options on 360 Variants

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for RCOD and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is driving below 10 KPH - A front camera view other than RCOD is showing |
| Scenario Description | User presses the RCOD soft button. |
| Post-conditions | Client sends request to server to activate RCOD view. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with server module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-381811/A-RCOD Activation Above Speed on 360 Variants

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for Rear View Camera on Demand, Views at Speed and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is above 10 KPH - A view at speed other than RCOD is active - Views at Speed menu is displayed. |
| Scenario Description | User presses the Rear Camera on Demand at Speed button |
| Post-conditions | Client sends request to server to activate RCOD View at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Server module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-381812/A-Hitch Activation Above Speed on 360 Variants

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for Hitch View, Views at Speed, and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is above 10 KPH - A view at speed other than Hitch is active - Views at Speed menu is displayed. |
| Scenario Description | User presses the Hitch View at Speed menu icon |
| Post-conditions | Client sends request to server to activate Hitch View at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with server module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-381813/A-Aux Activation Above Speed on 360 Variants

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for Aux View, Views at Speed, and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is above 10 KPH - A view at speed other than AUX is active - Views at Speed menu is displayed. |
| Scenario Description | User presses the Aux View at Speed menu icon |
| Post-conditions | Client sends request to server to activate Aux View at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with Server module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-381814/A-CHMSL Activation Above Speed on 360 Variants

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for CHMSL View, Views at Speed, and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is above 10 KPH - A view at speed other than CHMSL is active - Views at Speed menu is displayed. |
| Scenario Description | User presses the CHMSL View at Speed menu icon |
| Post-conditions | Client sends request to server to activate CHMSL Views at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with server module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-381815/A-RCOD Transition from Above Speed to Below Speed

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for RCOD View, Views at Speed, and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is above 10 KPH - RCOD at speed view is active - Views at Speed menu is displayed. |
| Scenario Description | User decelerates from above 10 KPH to below 10 KPH. Server requests RCOD view. |
| Post-conditions | Client shall update view menu. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with server module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-381816/A-Hitch Transition from Above Speed to Below Speed

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for Hitch View, Views at Speed, and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is above 10 KPH - Hitch at speed view is active - Views at Speed menu is displayed. |
| Scenario Description | User decelerates from above 10 KPH to below 10 KPH. Server requests Hitch View. |
| Post-conditions | Client shall update view menu. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with server module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-381818/A-Aux Transition from Above Speed to Below Speed

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for Aux view, Views at Speed and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is below 10 KPH - Aux at speed is active - Views at Speed menu is displayed |
| Scenario Description | User decelerates from above 10 KPH to below 10 KPH. Server requests Aux view. |
| Post-conditions | Client shall update view menu. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with server module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-381817/A-CHMSL Transition from Above Speed to Below Speed

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for CHMSL view, Views at Speed and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is above 10 KPH - CHMSL at speed is active. - Views at Speed menu is displayed |
| Scenario Description | User decelerates from above 10 KPH to below 10 KPH. Server requests CHMSL view. |
| Post-conditions | Client shall update view menu. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with server module E3 – Valid camera video signal not present |
| Interfaces |  |

#### White Box Views

##### Activity Diagram

###### CAMERA-ACT-REQ-381772/A-Views At Speed Gen 4

##### Sequence Diagram

###### CAMERA-SD-REQ-381773/A-Views At Speed Gen 4

## CHMSL IOD

### FUN-REQ-382683/A-CHMSL IOD

#### CAMERA-REQ-382684/A-CHMSL IOD Config Values

CHMSL IOD is a new view available to the user. It is available in Feature ID 0x081B and Config Value 0X2E.

| Config Number | View Type |
| --- | --- |
| 0x2E | CHMSL IOD |

#### UseCases

##### CAMERA-UC-REQ-382686/A-CHMSL IOD Activation

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for CHMSL IOD - Vehicle in Run/Start - Vehicle is not in reverse - Client HMI is not active (Front Views, Front Views at Speed, Rear Views, APA, TBA/TRG) |
| Scenario Description | User Selects CHMSL IOD |
| Post-conditions | Client requests CHMSL IOD. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-382687/A-CHMSL IOD Deactivation Via Client

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for CHMSL IOD - Vehicle in Run/Start - Vehicle is not in reverse - CHMSL IOD is active |
| Scenario Description | IOD changes away from CHMSL to a non-camera state (i.e. navigation, IOD changes) |
| Post-conditions | Client requests OF |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-382688/A-Returning to CHMSL IOD from Parking Feature

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for CHMSL IOD - Vehicle in Run/Start - Vehicle is in a Camera Feature (Front Camera, Rear Camera, TBA, Autohitch, APA, Boundary Alert) - Previous client display state is CHMSL IOD |
| Scenario Description | Camera view feature becomes inactive |
| Post-conditions | Client request CHMSL IOD. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

#### White Box View

##### Activity Diagram

###### CAMERA-ACT-REQ-382730/A-CHMSL IOD Operation

##### Sequence Diagrams

###### CAMERA-SD-REQ-382731/A-CHMSL IOD Operation

## RVC-FUN-REQ-395521/A-Reverse Gear Strategy

### Use Cases

#### RVC-UC-REQ-395465/A-Reverse Gear Strategy on Automatic Transmission Vehicles - Moving Below 16.09 KPH

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is equipped with an automatic transmission. - The gear shifter (GearLvrPos_D_Actl) is in a non-reverse gear (P,N,D,L,M). - The gear shifter update bit (GearLvrPos_D_Actl_UB) is available. - Vehicle speed is available (Veh_V_ActlEng) - Vehicle speed quality Factor (VehVActlEng_QF) is available. The vehicle is moving below 16.09 KPH (10 miles per hour). |
| Scenario Description | The gear shifter is placed in reverse and the following signals are sent to the CLIENT: GearLvrPos_D_Actl = 0x01 (Reverse) GearLvrPos_D_Actl_UB = 0x1 Veh_V_ActlEng <= 16.09 KPH VehVActlEng_D_Qf = 0x03 (OK) |
| Post-conditions | Client shall display the reverse camera within 2 seconds of placing the shifter in reverse per FMVSS 111. Client shall ignore the reverse gear engagement signal. (GearPos_D_Trg) |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with CAMERA SERVER module E3 – Valid camera video signal not present |
| Interfaces |  |

#### RVC-UC-REQ-395468/A-Reverse Gear Strategy on Automatic Transmission Vehicles - Moving Above 16.09 KPH

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is equipped with an automatic transmission. - The gear shifter (GearLvrPos_D_Actl) is in a non-reverse gear (P,N,D,L,M). - The gear shifter update bit (GearLvrPos_D_Actl_UB) is available - Vehicle speed is available (Veh_V_ActlEng) - Vehicle speed quality Factor (VehVActlEng_QF) is available. - The gear engagement status (GearPos_D_Trg) is available. The vehicle is moving above 16.09 KPH (10 miles per hour). |
| Scenario Description | The gear shifter is placed in reverse and the following signals are sent to the CLIENT: GearLvrPos_D_Actl = 0x01 (Reverse) GearLvrPos_D_Actl_UB = 0x1 GearPos_D_Trg = 0x0E (Reverse) GearPos_D_Trg_UB = 0x1 Veh_V_ActlEng > 16.09 KPH VehVActlEng_D_Qf = 0x03 (OK) |
| Post-conditions | Client shall display the reverse camera within 2 seconds of reverse gear engagement being confirmed and the gear lever position is in reverse while the vehicle is moving above 16.09 KPH. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with CAMERA SERVER module E3 – Valid camera video signal not present |
| Interfaces |  |

#### RVC-UC-REQ-395466/A-Reverse Gear Strategy on Automatic Transmission Vehicles - Shift Above 16.09 KPH and Deceleration Below 16.09 KPH

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The gear shifter is in the reverse position (GearLvrPos_D_Actl = 0x01) - The gear shifter update bit is on (GearLvrPos_D_Actl_UB = 0x1) - The reverse gear is not engaged (GearPos_D_Trg != 0x0E) - The vehicle is moving above 10 MPH (Veh_V_ActlEng > 16.09 KPH) - The vehicle speed quality factor is OK (VehVActlEng_D_Qf = 0x03) |
| Scenario Description | The vehicle slows down to at or below 10 MPH (Veh_V_ActlEng <= 16.09 KPH) |
| Post-conditions | Client shall display the rear camera. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with CAMERA SERVER module E3 – Valid camera video signal not present |
| Interfaces |  |

#### RVC-UC-REQ-395467/A-Reverse Gear Strategy on Automatic Transmission Vehicles - Vehicle Speed Unavailable or is Not OK

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The vehicle is equipped with an automatic transmission. - The gear shifter (GearLvrPos_D_Actl) is in a non-reverse gear (P,N,D,L,M). - The gear shifter update bit (GearLvrPos_D_Actl_UB) is available. |
| Scenario Description | The vehicle speed (Veh_V_ActlEng) is missing, or the vehicle speed quality factor (VehVActlEng_D_Qf) is missing, or the vehicle speed quality factor is not okay (VehVActlEng_D_Qf != 0x03) The gear shifter is placed in reverse and the following signals are sent to the CLIENT: GearLvrPos_D_Actl = 0x01 (Reverse) GearLvrPos_D_Actl_UB = 0x1 |
| Post-conditions | Client shall display the reverse camera within 2 seconds of placing the shifter in reverse per FMVSS 111. Client shall ignore the reverse gear engagement signal. (GearPos_D_Trg) |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with CAMERA SERVER module E3 – Valid camera video signal not present |
| Interfaces |  |

#### RVC-UC-REQ-395469/A-Reverse Gear Strategy on Automatic Transmission Vehicles - Exit Criteria when Gear Shifter Position is not Reverse

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The RVC is showing - The gear shifter is in reverse (GearLvrPos_D_Actl = 0x01) - The gear shift update bit (GearLvrPos_D_Actl_UB) is available |
| Scenario Description | The gear shifter is placed out of reverse and the following signals are sent to CLIENT: - GearLvrPos_D_Actl != 0x01 (Reverse) GearLvrPos_D_Actl_UB = 0x1 |
| Post-conditions | Client shall turn off the rear camera view or follow the rear camera delay behavior (whichever is applicable). Client shall only look at gear lever position to shut-off the rear camera state. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with CAMERA SERVER module E3 – Valid camera video signal not present |
| Interfaces |  |

#### RVC-UC-REQ-395470/A-Exit Criteria when RVC is active, Gear Shifter is in Reverse, and the vehicle is moving forward above 16.09 KPH

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - The RVC is showing - The gear shifter is in reverse (GearLvrPos_D_Actl = 0x01) - The gear shift update bit (GearLvrPos_D_Actl_UB) is available - The gear engagement signal is available (GearPos_D_Trg) - The gear engagement update bit is available (GearPos_D_Trg_UB) - Vehicle speed is available and is moving less than 16.09 KPH (Veh_V_ActlEng < 16.09) - Vehicle speed quality factor is available (VehVActlEng_QF) |
| Scenario Description | The gear shifter is moving forward, the vehicle is moving above 16.09 KPH, and the reverse gear is not engaged. The following signals are sent to Client - GearLvrPos_D_Actl = 0x01 (Reverse) - GearLvrPos_D_Actl_UB = 0x1 - GearPos_D_Trg != 0x0E (Reverse) - GearPos_D_Trg_UB = 0x1 - Veh_V_ActlEng > 16.09 KPH - VehVActlEng_QF = 0x03 (OK) |
| Post-conditions | Client shall turn off the rear camera view or follow the rear camera delay behavior (whichever is applicable). Client shall only look at gear lever position to shut-off the rear camera state. If vehicle speed is missing (Veh_V_ActlEng), vehicle quality factor is missing (VehVActlEng_QF), or gear engagement is unavailable (GearPos_D_Trg), then the following use case does not apply |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with CAMERA SERVER module E3 – Valid camera video signal not present |
| Interfaces |  |

## Turn Signal View

### FUN-REQ-410168/A-Turn Signal View

#### Functional Definition

##### Requirements(no setting in APIM, 706 上面固定是enable)

###### CAMERA-REQ-410098/A-Trailer Turn Signal View - Menu Setting Updates

The current Trailer Turn Signal View Menu setting shall be updated from server status method parameters of LTviewSt.

###### CAMERA-REQ-410099/A-Trailer Turn Signal View - Menu Grayed Out

When client does not receive LTviewSt = Enabled or  Disabled for 5 seconds, the client shall gray out the Trailer Turn Signal View Menu.

###### CAMERA-REQ-410101/A-Trailer Turn Signal View - Grayed Out Menu Displays Off

When the Trailer Turn Signal View menu is grayed out, the Trailer Turn Signal View menu setting shall display OFF.

###### CAMERA-REQ-410102/A-Trailer Turn Signal View - No User Change to Menu

When a vehicle occupant makes no changes to the Trailer Turn Signal View menu setting, client shall send LMnuRq = Null.

###### CAMERA-REQ-410103/A-Menu Setting Change - Missing Message

When client requests Trailer Turn Signal View menu setting change (LMnuRq = Enable | Disable) and server status (LTviewSt) signal for Trailer Turn Signal View menu setting do not agree, client shall continue requesting the setting change until agreement occurs OR the end of the key cycle.

###### CAMERA-REQ-410104/A-Menu Setting Change - Message Agreement Timeout

When client requests Trailer Turn Signal View menu setting change LMnuRq = Enable | Disable) and server status (LTviewSt) signal for Trailer Turn Signal View menu setting do not agree for more than 5 seconds, client shall gray-out the Trailer Turn Signal View menu and display OFF.

###### CAMERA-REQ-410105/A-Menu Setting based on Server Feature Memory

Upon vehicle startup, Trailer Turn Signal View menu shall initialize to the Trailer Turn Signal View feature setting value that is stored by server LTviewSt = Enabled | Disabled).

###### CAMERA-REQ-410106/A-Trailer Turn Signal View - Menu Setting Change - ON to OFF

When vehicle occupant changes the Trailer Turn Signal View menu setting from ON to OFF, Client shall send a request for Trailer Turn Signal View feature to be turned OFF (LMnuRq = Disable) until client receives a status signal confirming Trailer Turn Signal View has been DISABLED (LTviewSt = Disabled).

###### CAMERA-REQ-410107/A-Trailer Turn Signal View - Menu Setting Change - OFF to ON

When vehicle occupant changes the Trailer Turn Signal View menu setting from OFF to ON, client shall send a request for Trailer Turn Signal View feature to be turned ON (LMnuRq = Enable) until client receives a status signal confirming Trailer Turn Signal View has been ENABLED (LTviewSt = Enabled).

###### CAMERA-REQ-410108/A-Menu Setting Change - Parity THEN Revert to NULL

When Trailer Turn Signal View menu change request (LMnuRq) sent by client is matched by the status (LTviewSt ) returned by Server, Client shall revert to sending a NULL request (LMnuRq = NULL) to ADAS.

###### CAMERA-REQ-410109/A-Trailer Turn Signal View LEFT Activation - NO Blind Spot Threat Reported

When Side Object Detection System does not detect a Left side threat (LSodLSt = OFF | Bulb_Proveout) during Left Turn Signal View active, client shall display no Left BLIS icon overlay.

###### CAMERA-REQ-410110/A-Trailer Turn Signal View LEFT Activation -  Blind Spot Threat Reported

When Side Object Detection System detects a Left side threat (LSodLSt = ON | FLASH) during Left Turn Signal View active, Client shall display Left BLIS icon overlay.

###### CAMERA-REQ-410111/A-Trailer Turn Signal View RIGHT Activation - NO Blind Spot Threat Reported

When Side Object Detection System does not detect a Right side threat ( LSodRSt = OFF | Bulb_Proveout) during Right Turn Signal View active, client shall display no Right BLIS icon overlay.

###### CAMERA-REQ-410112/A-- Trailer Turn Signal View RIGHT Activation -  Blind Spot Threat Reported

When Side Object Detection System detects a Right side threat (LSodRSt = ON | FLASH) during Right Turn Signal View active, client shall display Right BLIS icon overlay.

##### Use Cases

###### CAMERA-UC-REQ-409518/A-Trailer Turn Signal View Missing Signal

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Vehicle is Accessory/RUN/START |
| Scenario Description | Missing Message occurs (5 seconds) |
| Post-conditions | Client uses last known status until missing message timeout of 5 seconds After 5 seconds Gray-out menu and display OFF |
| List of Exception Use Cases | E1 – Vehicle is not Accessory/RUN/START E2 – Loss of communication with Server module |
| Interfaces | HMI |

###### CAMERA-UC-REQ-409519/A-Trailer Turn Signal View Setting Change

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Vehicle is Accessory/RUN/START Trailer Turn Signal View Menu not grayed-out |
| Scenario Description | Vehicle Occupant changes Trailer Turn Signal View Menu setting. |
| Post-conditions | Feature changes according to user input. |
| List of Exception Use Cases | E1 – Vehicle is not Accessory/RUN/START E2 – Loss of communication with ADAS module |
| Interfaces | HMI interface |

###### CAMERA-UC-REQ-409520/A-Trailer Turn Signal View because of Cancel "X" Button Press

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Trailer is connected Turn Signal Views = Present Vehicle is in Run/Start Trailer Turn Signal View menu setting is set to ON Trailer Turn Signal View actively displaying TurnSglView_Left or TurnSglView_Right |
| Scenario Description | Vehicle Occupant presses Cancel “X” Button. (Client Requests view OFF) |
| Post-conditions | Client displays the previous Client screen without confirmation from Server. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with server module E3 – Valid camera video signal not present |
| Interfaces | HMI |

###### CAMERA-UC-REQ-409521/A-Trailer Turn Signal View due to Shift Reverse

| Actors | Driver |
| --- | --- |
| Pre-conditions | Trailer is connected Turn Signal Views = Present Vehicle is in Run/Start Trailer Turn Signal View menu setting is set to ON Trailer Turn Signal View actively displaying TurnSglView_Left or TurnSglView_Right |
| Scenario Description | Driver shifts to REVERSE. |
| Post-conditions | Server sends default rear camera image to Client. Client displays default rear camera image. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces | HMI interface |

###### CAMERA-UC-REQ-410090/A-Trailer Turn Signal View Menu Setting based on Server Feature Memory

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Turn Signal Views = Present Vehicle is Accessory/RUN/START |
| Scenario Description | Trailer Turn Signal View Menu is selected. |
| Post-conditions | Trailer Turn Signal View Menu displays according to status signal. |
| List of Exception Use Cases | E1 – Vehicle is not Accessory/RUN/START E2 – Loss of communication with server module |
| Interfaces | HMI interface |

###### CAMERA-UC-REQ-410090/A- display Left/Right BLIS icon overlay

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Vehicle is in RUN/START. APIM is configured for 360 AND Turn signal view. ADAS is configured for 360 AND Turn signal view. PersTurnSglView_D_Stat=Enabled. Applicable for Half and Full-Pano display only. |
| Scenario Description | The customer is selecting Turn signal switch to Left (TurnLghtSwtch_D_Stat=Left/right) AND FeatConfigIpmbActl=0x2F/30 Left side threat (LSodLSt = ON ¦ FLASH) or Right side threat(LSodRSt = ON ¦ FLASH) |
| Post-conditions | Trailer Turn Signal View Menu displays according to status signal by displaying left/Right BLIS icon overlay. |
| List of Exception Use Cases | E1 – Vehicle is not Accessory/RUN/START E2 – Loss of communication with server module |
| Interfaces | HMI interface |

###### CAMERA-UC-REQ-410090/A-Left side turn signal view placement

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Vehicle is in RUN/START. APIM is configured for 360 AND Turn signal view. ADAS is configured for 360 AND Turn signal view. PersTurnSglView_D_Stat=Enabled. Applicable for Half and Full-Pano display only. |
| Scenario Description | The customer is selecting Turn signal switch to Left (TurnLghtSwtch_D_Stat=Left) AND FeatConfigIpmbActl=0x2F |
| Post-conditions | AVM shall output the Enum for Left side Turn signal view through CAN signal FeatConfigIpmbActl=0x2F, Then APIM shall display the Left side Turn signal view in FPD link on location near Left side of on below co-ordinates: X=13Pixel Y=126Pixel Reference picture with 0,0 co-ordinate for illustration: X=13,Y=126 X=0,Y=0 |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |

###### CAMERA-UC-REQ-410090/A-Left side turn signal view output

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Vehicle is in RUN/START. APIM is configured for 360 AND Turn signal view. ADAS is configured for 360 AND Turn signal view. PersTurnSglView_D_Stat=Enabled. Applicable for Half and Full-Pano display only. |
| Scenario Description | The customer is selecting Turn signal switch to Left (TurnLghtSwtch_D_Stat=Left) AND FeatConfigIpmbActl=0x2F |
| Post-conditions | ADAS shall output the Enum for Left side Turn signal view through CAN signal FeatConfigIpmbActl=0x2F, Then APIM shall display the Left side Turn signal view in FPD link based of placement defined in previous requirement. . |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces | CAN, FPD link |

###### CAMERA-UC-REQ-410090/A-Right side turn signal view placement

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | 1 Vehicle is in RUN/START. 2 APIM is configured for 360 AND Turn signal view. 3 ADAS is configured for 360 AND Turn signal view. 4 PersTurnSglView_D_Stat=Enabled. 5 Applicable for Half and Full-Pano display only. |
| Scenario Description | The customer is selecting Turn signal switch to Right (TurnLghtSwtch_D_Stat=Right) AND FeatConfigIpmbActl=0x30 |
| Post-conditions | ADAS shall output the Enum for Right side Turn signal view through CAN signal FeatConfigIpmbActl=0x30, Then APIM shall display the Right side Turn signal view in FPD link on location near Right side of on below co-ordinates: X=1360Pixel Y=126Pixel Reference picture with 0,0 co-ordinate for illustration: Left side Turn signal view Right side Turn signal view X=1360,Y=126 X=0,Y=0 . |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces | CAN, FPD link |

###### CAMERA-UC-REQ-410090/A-Right side turn signal view output

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | 1 Vehicle is in RUN/START. 2 APIM is configured for 360 AND Turn signal view. 3 ADAS is configured for 360 AND Turn signal view. 4 PersTurnSglView_D_Stat=Enabled. 5 Applicable for Half and Full-Pano display only. |
| Scenario Description | The customer is selecting Turn signal switch to Right (TurnLghtSwtch_D_Stat=Right) AND FeatConfigIpmbActl=0x30. |
| Post-conditions | ADAS shall output the Enum for Left side Turn signal view through CAN signal FeatConfigIpmbActl=0x30, Then APIM shall display the Left side Turn signal view in FPD link based of placement defined in previous requirement. . |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces | CAN, FPD link |

###### CAMERA-UC-REQ-410090/A-Entry conditions into left/right side turn signal view with other camera view on

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | 1 Vehicle is in RUN/START. 2 APIM is configured for 360 AND Turn signal view. 3 ADAS is configured for 360 AND Turn signal view. 4 PersTurnSglView_D_Stat=Enabled. 5 Applicable for Half and Full-Pano display only. |
| Scenario Description | Other camera view is active and user activating Left side Turn light switch(TurnLghtSwtch_D_Stat=Right/right) |
| Post-conditions | When APIM is showing the camera view in center stack and shall transition to show Left/right side Turn signal camera views in cluster on conditions when FeatConfigIpmbActl=0x2F/30 |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces | CAN, FPD link |

###### CAMERA-UC-REQ-410090/A-Exit conditions with right/left side Turn signal view active

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | 1 Vehicle is in RUN/START. 2 APIM is configured for 360 AND Turn signal view. 3 ADAS is configured for 360 AND Turn signal view. 4 PersTurnSglView_D_Stat=Enabled. 5 Applicable for Half and Full-Pano display only. |
| Scenario Description | Right side Turn signal view is active(FeatConfigIpmbActl=0x30/2F) AND user activating other camera views |
| Post-conditions | When FeatConfigIpmbActl!=0x30/2F, the APIM shall stop showing the camera view in cluster and shall transition to show camera views in center stack on conditions like: Shifting in to Reverse. Pressing camera button to activate Front 360 view. Object detected camera view activation. Off-road condition. Fully Automated parking. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces | CAN, FPD link |

###### CAMERA-UC-REQ-410090/A-Cropping and rescaling of left/right side Turn signal view image

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | 1 Vehicle is in RUN/START. 2 APIM is configured for 360 AND Turn signal view. 3 ADAS is configured for 360 AND Turn signal view. 4 PersTurnSglView_D_Stat=Enabled. 5 Applicable for Half and Full pano only. |
| Scenario Description | The customer is selecting Turn signal switch to Right (TurnLghtSwtch_D_Stat=Right/Right) AND FeatConfigIpmbActl=0x2F/30 AND Cropping the Left side Turn signal view from ADAS. |
| Post-conditions | ADAS shall output the Enum for Left side Turn signal view through CAN signal FeatConfigIpmbActl=0x2F/30, Then APIM shall Crop and rescale the image and display. Cropping and rescaling details: ADAS Shall output 1120*664 active pixels with resolution 1536*664 pixels APIM shall crop 448 pixels(i.e) cropping from 1120 to 672 pixels in the width of Image. APIM shall rescale the 664 pixels in Height of image to 581 pixels. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces | CAN, FPD link |

##### White Box Views

###### Activity Diagrams

CAMERA-ACT-REQ-411945/A-Operate TSV

CAMERA-ACT-REQ-411947/A-Disable TSV

###### Sequence Diagrams

CAMERA-SD-REQ-411946/A-Operate TSV

CAMERA-SD-REQ-411948/A-Disable TSV

## CTA 180 Multicamera

### FUN-REQ-410169/A-CTA 180 Multicamera

#### Functional Definition

##### Use Cases

###### CAMERA-UC-REQ-410120/A-Menu to be not available

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT has Config for CtaSV Configuration set to Disabled - Vehicle is in Run/Start |
| Scenario Description | User accesses the settings menu. |
| Post-conditions | CTA menu is not available. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with server module E3 – Valid camera video signal not present E4 – User switches to Rear Split View manually by selecting the soft button |
| Interfaces |  |

###### CAMERA-UC-REQ-410121/A-Menu to be available in Settings

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT has Config for CtaSV Configuration set to “Enabled” - Vehicle is in Run/Start |
| Scenario Description | User accesses the settings menu |
| Post-conditions | CTA menu is available |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with SERVER module E3 – Valid camera video signal not present |
| Interfaces |  |

###### CAMERA-UC-REQ-410122/A-Activating Feature via Settings menu

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT has Config for CtaSV Configuration set to “Enabled” - Vehicle is in Run/Start |
| Scenario Description | The feature is enabled by the driver from the settings menu |
| Post-conditions | The feature stays enabled, and performs as designed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with SERVER module E3 – Valid camera video signal not present |
| Interfaces |  |

###### CAMERA-UC-REQ-410123/A-De-activating Feature via Settings menu

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT has Config for CtaSV Configuration set to “Enabled” - Vehicle is in Run/Start |
| Scenario Description | The feature is disabled by the driver from the settings menu |
| Post-conditions | The feature stays disabled, and performs as designed. |
| List of Exception Use Cases |  |
| Interfaces | HMI |

###### CAMERA-UC-REQ-410124/A-No Switching when not in Reverse Gear

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT has Config for CtaSV Configuration set to “Enabled” - Gear is not in REVERSE |
| Scenario Description | CLIENT receives a CTA Trigger (Alert or braking) |
| Post-conditions | CLIENT does not request/perform split view. |
| List of Exception Use Cases |  |
| Interfaces | HMI |

###### CAMERA-UC-REQ-410125/A-View switching due to a CTA Event

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT has Config for CtaSV Configuration set to “Enabled” - Gear is in REVERSE |
| Scenario Description | CTA trigger becomes active (alert or braking) |
| Post-conditions | CLIENT shall switch the view based on SERVER request |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with SERVER module E3 – Valid camera video signal not present |
| Interfaces |  |

###### CAMERA-UC-REQ-410126/A-Switching to last known view after a CTA Event

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT has Config for CtaSV Configuration set to “Enabled” - Gear is in REVERSE |
| Scenario Description | CTA trigger (alert or braking) is no longer active |
| Post-conditions | Client shall switch the view based on SERVER instruction. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with SERVER module E3 – Valid camera video signal not present |
| Interfaces |  |

###### CAMERA-UC-REQ-410127/A-Missing Signal

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client has Config for CtaSV Configuration set to “Enabled” - Gear is in REVERSE - CTA trigger (alert or braking) is active |
| Scenario Description | Loss of communication or missing signal from Server module |
| Post-conditions | Client shall set display a “grayed out” feature. Feature display will be disabled. |
| List of Exception Use Cases |  |
| Interfaces |  |

##### White Box Views

###### Activity Diagrams

CAMERA-ACT-REQ-411951/A-Enable Disable CTASV

CAMERA-ACT-REQ-411953/A-Switching to SV

###### Sequence Diagrams

CAMERA-SD-REQ-411952/A-Enable Disable CTASV

CAMERA-SD-REQ-411954/A-Switching to SV

## Invisible Van View

### FUN-REQ-410171/A-Invisible Van View

#### CAMERA-REQ-411963/A-IVV Config Value

IVV Feature Config = 0x40

#### Functional Definition

##### Use Cases

###### CAMERA-UC-REQ-410139/A-Requesting Invisible Van View

| Actors | Driver |
| --- | --- |
| Pre-conditions | Vehicle is equipped with Invisible Van View and vehicle is in Reverse Gear. IVV is configured to enabled. |
| Scenario Description | Driver requests Invisible Van View from the Client menu |
| Post-conditions | Server activated IVV. |
| List of Exception Use Cases | Client displays IVV in HMI screen. |
| Interfaces |  |

###### CAMERA-UC-REQ-410140/A-Deactivating Invisible Van View (1)

| Actors | Driver |
| --- | --- |
| Pre-conditions | - Vehicle is equipped with Invisible Van View - Vehicle is in Reverse Gear - Invisible Van view is active |
| Scenario Description | Driver requests a different view while in Reverse |
| Post-conditions | Switch out (Stop Displaying) of Invisible Van View |
| List of Exception Use Cases |  |
| Interfaces |  |

###### CAMERA-UC-REQ-411961/A-Deactivating Invisible Van View (2)

| Actors | Driver |
| --- | --- |
| Pre-conditions | - Vehicle is equipped with Invisible Van View - Vehicle is in Reverse Gear - Invisible Van view is active - Delay mode is not active |
| Scenario Description | Driver shifts out of Reverse Gear |
| Post-conditions | Client stops displaying the Invisible van camera view. Client reverts back to home screen. |
| List of Exception Use Cases |  |
| Interfaces |  |

###### CAMERA-UC-REQ-411962/A-Deactivating Invisible Van View (3)

| Actors | Driver |
| --- | --- |
| Pre-conditions | - Vehicle is equipped with Invisible Van View - Vehicle is in Reverse Gear - Invisible Van view is active - Delay mode is active |
| Scenario Description | Driver shifts out of Reverse Gear |
| Post-conditions | Client would continue to show the invisible van view until speed limit is crossed, view is canceled or park gear is entered. |
| List of Exception Use Cases |  |
| Interfaces |  |

##### White Box Views

###### Activity Diagrams

CAMERA-ACT-REQ-411959/A-Operate Invisible Van View

###### Sequence Diagrams

CAMERA-SD-REQ-411960/A-Operate Invisible Van View

## 50/50 Views

### FUN-REQ-410167/A-50/50 Views

#### CAMERA-REQ-411964/A-50 50 Split View Configurations

50/50 Split View has a feature configuration 0x2C.

50/50 Split View At Speed has a feature configuration 0x2D.

#### Functional Definition

##### Use Cases

###### CAMERA-UC-REQ-410164/A-50/50 SPLIT VIEW Activation Below Speed on 360 Variants

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for 50/50 Split View, TRG, and 360 camera - Server is configured for 50/50 Split View, TRG, and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is below 10 KPH - Front Camera Menu below speed is displayed. |
| Scenario Description | User presses the 50/50 Split View menu icon |
| Post-conditions | Client sends request to Camera to activate 50/50 Split View. Server activates 50/50 Split View |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with camera server module E3 – Valid camera video signal not present |
| Interfaces |  |

###### CAMERA-UC-REQ-410165/A-50/50 SPLIT VIEW Transition from Below Speed to Above Speed on 360 Variants

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | • Client is configured for 50/50 Split View, TRG view, 50/50 Split Views at Speed and 360 camera • Server is configured for 50/50 Split View, TRG view, Views at Speed and 360 camera • Vehicle in Run/Start • Vehicle is not in reverse • Vehicle is below 10 KPH • 50/50 Split View is the active view • Front Camera Menu below speed is displayed. |
| Scenario Description | User accelerates from below 10 KPH to above 10 KPH |
| Post-conditions | FeatConfigIPMBActl will change from 0x2C (50/50 SPLIT VIEW) to 0x2D (50/50 SPLIT VIEW at Speed). The front camera views at speed menu shall be displayed (CHMSL, AUX, Hitch, RCOD, 50/50 split view buttons as configured on) |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with camera server module E3 – Valid camera video signal not present |
| Interfaces |  |

###### CAMERA-UC-REQ-410166/A-50/50 Split View at Speed Activation Above Speed on 360 Variants

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for 50/50 Split View at Speed, TRG View, and 360 camera - Server is configured for 50/50 Split View at Speed, TRG View, and 360 camera - Vehicle in Run/Start - Vehicle is not in reverse - Vehicle is above 10 KPH - A view at speed other than 50/50 is active Views at Speed menu is displayed. |
| Scenario Description | User presses the 50/50 Split View at Speed menu icon |
| Post-conditions | Client sends request to Camera to activate 50/50 Split Views at Speed. Server activates 50/50 Split Views at Speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with camera server module E3 – Valid camera video signal not present |
| Interfaces |  |

###### CAMERA-UC-REQ-410163/A-50/50 Split View Transition from Above Speed to Below Speed

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | • Client is configured for 50/50 Split View at Speed, TRG view, Views at Speed and 360 camera • Server is configured for 50/50 Split View at Speed, TRG view, Views at Speed and 360 camera • Vehicle in Run/Start • Vehicle is not in reverse • Vehicle is above 10 KPH • 50/50 Split View at speed is active. • Views at Speed menu is displayed |
| Scenario Description | User decelerates from above 10 KPH to below 10 KPH |
| Post-conditions | FeatConfigIPMBActl will change from 0x2D (50/50 SPLIT VIEW at Speed) to 0x2C (50/50 SPLIT VIEW). The front camera menu is active. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with camera server module E3 – Valid camera video signal not present |
| Interfaces |  |

##### White Box Views

###### Activity Diagrams

CAMERA-ACT-REQ-411965/A-Operate 50 50 Split View

###### Sequence Diagrams

CAMERA-SD-REQ-411966/A-Operate 50 50 Split View

## Trailer 360 Views

### CAMERA-FUN-REQ-419878/A-Trailer 360 Views

#### Requirements

##### CAMERA-REQ-419865/A-Trailer 360 View Configuration Values

Trailer 360 view uses FBMP for requesting the various camera views. The values below are the various configuration values associated with their respective Trailer View.

| Config Value | Specific View |
| --- | --- |
| 0x31 | Trailer 360 |
| 0x32 | Trailer Rear Normal |
| 0x33 | Trailer Interior View 1 |
| 0x34 | Trailer Interior View 2 |
| 0x35 | Trailer Left |
| 0x36 | Trailer Right |
| 0x3D | Trailer Rear Normal at Speed |

To request a particular view the client should transmit the below:

Feature_Rq(Operation = Set, FeatureID = 081B[Camera Showing], Configuration = Config Value[Specific View], PersIndex = Vehicle)

The server should reply back. The reply back contains the View the Server is transmitting currently.

Feature_St(Operation = Set, FeatureID = 081B[Camera Showing], Configuration = Config Value[Specific View], PersIndex = Vehicle)

##### CAMERA-REQ-419879/A-Trailer 360 Views Availability

The client shall use the signal LTrlrConnect to control the availability of trailer views to the user.

When signal LTrlrConnect = 0x01 (Connected) the client shall make available specific views. Refer to HMI specification for the list of available views.

When signal LTrlrConnect = 0x00(Not Connected) the client shall deactivate Trailer 360 Views.

#### Use Cases

##### CAMERA-UC-REQ-419705/A-Trailer 360 Soft Button Activation

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Trailer 360 Trailer 360 system is not connected Vehicle in Run/Start |
| Scenario Description | Trailer 360 system becomes connected to the vehicle. |
| Post-conditions | The Trailer 360 soft buttons in Front, Rear, TBA, and TRG views shall become present. The Aux camera button shall not be present. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces | Client HMI |

##### CAMERA-UC-REQ-419706/A-Trailer 360 view selection below 10 KPH or in Reverse

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Trailer 360 Trailer 360 system is connected Vehicle in Run/Start User is in the Front Camera menu below 10 KPH or rear camera menu |
| Scenario Description | User presses the Trailer 360 view soft button |
| Post-conditions | Client displays Trailer 360 view. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces | Client HMI |

##### CAMERA-UC-REQ-419707/A-Trailer Rear Normal view selection below 10 KPH or in Reverse

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Trailer 360 Trailer 360 system is connected Vehicle in Run/Start User is in the Front Camera menu below 10 KPH or rear camera menu |
| Scenario Description | User presses the Trailer Rear Normal view soft button |
| Post-conditions | Client displays Trailer Rear Normal view. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-419708/A-Trailer Interior View 1 selection below 10 KPH or in Reverse

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Trailer 360 Trailer 360 system is connected Vehicle in Run/Start User is in the Front Camera menu below 10 KPH or rear camera menu |
| Scenario Description | User presses the Trailer Interior View 1 soft button |
| Post-conditions | Client displays Trailer Interior View 1 |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-419709/A-Trailer Interior View 2 selection below 10 KPH or in Reverse

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Trailer 360 Trailer 360 system is connected Vehicle in Run/Start User is in the Front Camera menu below 10 KPH or rear camera menu |
| Scenario Description | User presses the Trailer Interior View 2 soft button |
| Post-conditions | Client displays Trailer Interior View 2 |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-419710/A-Trailer Left View selection below 10 KPH or in Reverse

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Trailer 360 Trailer 360 system is connected Vehicle in Run/Start User is in the Front Camera menu below 10 KPH or rear camera menu |
| Scenario Description | Customer presses the Trailer Left View soft button |
| Post-conditions | Client displays Trailer Left View |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-419711/A-Trailer Right View selection below 10 KPH or in Reverse

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Trailer 360 Trailer 360 system is connected Vehicle in Run/Start User is in the Front Camera menu below 10 KPH or rear camera menu |
| Scenario Description | User presses the Trailer Right view soft button |
| Post-conditions | Client displays Trailer Right view |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-419712/A-Trailer 360 Soft Button Deactivation

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Trailer 360 Trailer 360 system is connected Vehicle in Run/Start |
| Scenario Description | Trailer 360 system disconnects from the vehicle |
| Post-conditions | The Trailer 360 soft buttons shall be deactivated, and the Aux camera button shall become active again. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-419713/A-Trailer Rear Normal View transition from below 10 KPH to above 10 KPH

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Trailer 360 Client is configured for Trailer 360 Views at Speed Trailer 360 system is connected Vehicle in Run/Start User is in the Front Camera menu below 10 KPH Trailer Rear Normal View is active |
| Scenario Description | Customer drives above 10 KPH |
| Post-conditions | The views at speed menu shall become available |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-419714/A-Trailer Rear Normal View at Speed selection above 10 KPH

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Trailer 360 Client is configured for Trailer 360 Views at Speed Trailer 360 system is connected Vehicle in Run/Start Views at Speed is active |
| Scenario Description | User presses the Trailer Rear Normal View at Speed soft button |
| Post-conditions | Client displays Trailer Rear Normal View |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-419715/A-Trailer Rear Normal View transition from above 10 KPH to below 10 KPH

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Trailer 360 Client is configured for Trailer 360 Views at Speed Trailer 360 system is connected Vehicle in Run/Start Views at Speed is active Trailer Rear Normal View at Speed is active |
| Scenario Description | Customer drives below 10 KPH |
| Post-conditions | The front camera menu shall become available. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

#### White Box Views

##### Activity Diagrams

###### CAMERA-ACT-REQ-423119/A-Trailer 360 Views

##### Sequence Diagrams

###### CAMERA-SD-REQ-423120/A-Trailer 360 Views

## PiP At Speed

### FUN-REQ-423142/A-PiP At Speed

#### Requirements

##### CAMERA-REQ-422921/A-Views Parameter Values

PiP views and swap use FBMP for requesting the various camera views. The values below are the various configuration values associated with their respective PiP and swap view.

For Pip we use FeatureID 081B.

| Config Value | Specific View |
| --- | --- |
| 0x41 | PiP 50/50 |
| 0x42 | PiP 50/50 Views At Speed |
| 0x43 | PiP Aux |
| 0x44 | PiP Aux Views At Speed |
| 0x45 | PiP Trailer Rear Normal View |
| 0x46 | PiP Trailer Rear Normal View At Speed |

To request a particular view the client should transmit the below:

Feature_Rq(Operation = Set, FeatureID = 081B[Camera Showing], Configuration = Config Value[Specific View from table above], PersIndex = Vehicle)

The server should reply back. The reply back contains the View the Server is transmitting currently.

Feature_St(Operation = Set, FeatureID = 081B[Camera Showing], Configuration = Config Value[Specific Viewfrom table above], PersIndex = Vehicle)

To Swap we request FeatureID 082A.

| Config Value | Specific View |
| --- | --- |
| 0x02 | Swap |

#### Use Cases

##### CAMERA-UC-REQ-420591/A-Requesting PiP AUX from AUX below 10kph

| Actors |  |  | Vehicle Occupant |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pre-conditions |  |  | Client is configured for Pip and PiP views at Speed Vehicle is in Drive mode and below 10kph Front camera AUX View is active |  |  |  |  |  |  |  |
| Scenario Description |  |  | PiP button is selected |  |  |  |  |  |  |  |
| Post-conditions |  |  | Client shall send a request to server for PiP Aux View Server sends the response. PiP Aux view becomes active. |  |  |  |  |  |  |  |
| List of Exception Use Cases |  |  | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |  |  |  |  |  |  |  |
| Interfaces |  |  |  |  |  |  |  |  |  |  |
| Legacy Requirement |  |  |  |  |  |  |  |  |
| Rationale |  |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |  |
| Source Req. |  |  |  |  |  | V&V Method |  |  |
| Type |  |  |  | Priority |  | Status | In-Progress |  |
|  |  |  | End of Requirement |  |  |  |  |

##### CAMERA-UC-REQ-420592/A-Requesting AUX from PiP AUX with PiP button at below 10kph

| Actors |  |  | Vehicle Occupant |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pre-conditions |  |  | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and below 10kph speed. PiP AUX View is active |  |  |  |  |
| Scenario Description |  |  | PiP Button is selected |  |  |  |  |
| Post-conditions |  |  | Client shall send a request to server for Aux view. Server sends the response Aux View is displayed. |  |  |  |  |
| List of Exception Use Cases |  |  | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |  |  |  |  |
| Interfaces |  |  |  |  |  |  |  |
| Legacy Requirement |  |  |  |  |  |  |  |  |  |
| Rationale |  |  |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |  |  |
| Source Req. |  |  |  |  |  | V&V Method |  |  |  |
| Type |  |  |  | Priority |  | Status |  | In-Progress |  |
|  |  |  | End of Requirement |  |  |  |  |  |

##### CAMERA-UC-REQ-420593/A-Requesting Swap from PiP AUX at below 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and below 10kph speed. PiP AUX View is active |
| Scenario Description | Swap Button is selected |
| Post-conditions | Client shall send a request to server for Pip Swap Server sends the response Images are swap. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420594/A-Requesting PiP 50/50 from 50/50 view , below 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and below 10kph speed. 50/50 view is active |
| Scenario Description | PiP button is selected |
| Post-conditions | Client shall send a request to server PiP 50/50 view. Server sends the response PiP 50/50 view shall be active |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420595/A-Requesting 50/50 from PiP 50/50 with PiP button at below 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and below 10kph speed. PiP 50/50 View is active |
| Scenario Description | PiP Button is selected |
| Post-conditions | Client shall send a request to server to show 50/50 view Server sends the response Client displays 50/50 views. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420596/A-Requesting Swap from PiP 50/50 at below 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and below 10kph speed. PiP 50/50 View is active |
| Scenario Description | Swap button is selected. |
| Post-conditions | Client shall send a request to server PiP Swap. Server sends the response. Client displays 50/50 pip. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420597/A-Requesting Pip Trailer Rear Normal from Trailer Rear Normal below 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and below 10kph speed. Trailer Rear Normal View is active |
| Scenario Description | PiP button is selected |
| Post-conditions | Client shall send a request to server Pip trailer rear normal. Server sends the response Client displays Pip Trailer Rear Normal View |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420598/A-Requesting Trailer Rear Normal from PiP Trailer Rear Normal with PiP button at below 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and below 10kph speed. PiP Trailer Rear Normal view is active |
| Scenario Description | PiP Button is selected |
| Post-conditions | Client shall send a request to server to show Trailer rear normal. Server sends the response. Trailer rear normal becomes active |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420599/A-Requesting Swap from Pip Trailer Rear Normal below 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and below 10kph speed. PiP Trailer Rear Normal view is active |
| Scenario Description | Swap button is selected. |
| Post-conditions | Client shall send a request to server for a PipSwap Server provides feedback to client. Client displays PIP Trailer Rear Normal view. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420600/A-Transitioning from PiP AUX to PiP AUX at Speed

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and below 10kph speed. PiP AUX View is active |
| Scenario Description | Vehicle speed is transitioning from below to above 10kph |
| Post-conditions | Client will display PIP Aux at speed. Views at speed HMI shall be displayed |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420601/A-Transitioning from PiP AUX at Speed to PiP AUX

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph PiP Aux at speed view is active |
| Scenario Description | Vehicle speed is transitioning from above to below 10kph |
| Post-conditions | Client displays Pip Aux view Front Camera HMI shall be displayed |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420602/A-Transitioning from PiP 50/50 to PiP 50/50 at speed

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and below 10kph PiP 50/50 view is active |
| Scenario Description | Vehicle speed is transitioning from below to above 10kph |
| Post-conditions | Client will display PiP 50/50 at speed. Views at speed HMI shall be displayed |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420603/A-Transitioning from PiP 50/50 at speed to PiP 50/50

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Vehicle is in drive and above 10kph PiP 50/50 at speed is active |
| Scenario Description | Vehicle speed is transitioning from above to below 10kph |
| Post-conditions | Client displays Pip 50/50 Front Camera HMI shall be displayed |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420604/A-Transitioning from PiP Trailer Rear Normal to PiP Trailer Rear Normal at speed

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and below 10kph PiP Trailer Rear Normal view is active |
| Scenario Description | Vehicle speed is transitioning from below to above 10kph |
| Post-conditions | Client displays Pip trailer rear normal at speed view. View at speed HMI shall be displayed |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420605/A-Transitioning from PiP Trailer Rear Normal at speed to PiP Trailer Rear Normal

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph PiP Trailer Rear Normal at Speed is active |
| Scenario Description | Vehicle speed is transitioning from above to below 10kph |
| Post-conditions | Client displays pip 50/50 Front Camera HMI shall be displayed |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420606/A-Requesting PiP AUX at Speed from AUX at Speed

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph AUX view at Speed is active |
| Scenario Description | PiP button is selected |
| Post-conditions | Client shall send a request to server for Pip Aux at speed Client displays Pip AUX at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420607/A-Requesting PiP 50/50 at speed from 50/50 at speed with PiP button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph 50/50 view at Speed is active |
| Scenario Description | PiP button is selected |
| Post-conditions | Client shall send a request to server for PiP 50/50 at speed Client displays Pip 50/50 at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420608/A-Requesting PiP Trailer Rear Normal at Speed from Trailer Rear Normal at Speed with PiP button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Apim is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph Trailer Rear Normal view at Speed is active |
| Scenario Description | PiP button is selected |
| Post-conditions | Client shall send a request to server for PiP Trailer Rear Normal at Speed Client displays Pip Trailer Normal at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420609/A-Requesting AUX at speed view from PiP AUX at Speed view with PiP button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph PiP AUX at Speed is active |
| Scenario Description | PiP button is selected |
| Post-conditions | Client shall send a request to server for Aux at speed Client displays Aux at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420610/A-Requesting 50/50 at speed view from PiP 50/50 at Speed view with PiP button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph PiP 50/50 at Speed is active |
| Scenario Description | PiP button is selected |
| Post-conditions | Client shall send a request to server for 50/50 at speed Client displays 50/50 at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420611/A-Requesting Trailer Rear Normal at speed from PiP Trailer Rear Normal at speed view with PiP button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph PiP Trailer Rear Normal view at Speed is active |
| Scenario Description | PiP button is selected |
| Post-conditions | Client shall send a request to server for Trailer Rear Normal at speed Client displays Trailer rear normal at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420612/A-Exit PiP Aux at Speed View with Camera Hard button

| Actors |  |  | Vehicle Occupant |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pre-conditions |  |  | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph PiP AUX view at speed is active |  |  |  |  |  |
| Scenario Description |  |  | Camera hard button is pressed |  |  |  |  |  |
| Post-conditions |  |  | Client shuts off the camera view and return to the last sync screen |  |  |  |  |  |
| List of Exception Use Cases |  |  | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |  |  |  |  |  |
| Interfaces |  |  |  |  |  |  |  |  |
| Legacy Requirement |  |  |  |  |  |  |  |  |  |
| Rationale |  |  |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |  |  |
| Source Req. |  |  |  |  |  | V&V Method |  |  |  |
| Type |  |  |  | Priority |  | Status | In-Progress |  |  |
|  |  |  | End of Requirement |  |  |  |  |  |
| Legacy Requirement |  |  |  |  |  |  |  |  |  |
| Rationale |  |  |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |  |  |
| Source Req. |  |  |  |  |  | V&V Method |  |  |  |
| Type |  |  |  | Priority |  | Status | In-Progress |  |  |
|  |  |  | End of Requirement |  |  |  |  |  |

##### CAMERA-UC-REQ-420613/A-Exit PiP 50/50 at Speed View with Camera Hard button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph PiP AUX view at speed is active |
| Scenario Description | Camera hard button is selected |
| Post-conditions | Client shall shut off the camera view and return to the last sync screen |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420614/A-Exit PiP Trailer Rear Normal at Speed View with Camera Hard button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph PiP AUX view at speed is active |
| Scenario Description | Camera hard button is pressed |
| Post-conditions | Last remembered Client screen shall be displayed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420615/A-Requesting AUX from PiP AUX view with Aux button below 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and below 10kph speed. PiP AUX View is active |
| Scenario Description | AUX view Button is selected |
| Post-conditions | Client shall send a request to server for Aux view Client displays Aux view. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420616/A-Requesting 50/50 from PiP 50/50 view with 50/50 button below 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and below 10kph speed. PiP 50/50 View is active |
| Scenario Description | 50/50 view Button is selected |
| Post-conditions | Client shall send a request to server for 50/50 view Client displays 50/50 view. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420617/A-Requesting Trailer Normal View from PiP Trailer Rear Normal view with Trailer Rear Normal View button below 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and below 10kph speed. PiP Trailer Rear Normal View is active |
| Scenario Description | Trailer Rear Normal view button is selected |
| Post-conditions | Client shall send a request to server for Trailer Rear Normal view Client displays Trailer Rear Normal. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420618/A-Requesting Swap from PiP AUX at speed above 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and above 10kph speed. PiP AUX at Speed View is active |
| Scenario Description | Swap Button is selected |
| Post-conditions | Client displays Pip Aux at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420619/A-Requesting Swap from PiP 50/50 at speed above 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and above 10kph speed. PiP AUX at Speed View is active |
| Scenario Description | Swap Button is selected |
| Post-conditions | Client displays Pip 50/50 at speed, |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420620/A-Requesting Swap from PiP Trailer Rear Normal at speed above 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and above 10kph speed. PiP AUX at Speed View is active |
| Scenario Description | Swap Button is selected |
| Post-conditions | Client displays Pip Trailer rear normal at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420621/A-Last Remembered view for PiP AUX at Speed

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and above 10kph speed. Camer view is off Last View at speed active was PiP AUX at Speed |
| Scenario Description | Camera hard button is pressed |
| Post-conditions | Client displays Pip Aux at Speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420622/A-Last Remembered view for PiP 50/50 at Speed

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and above 10kph speed. Camer view is off Last View at speed active was PiP 50/50 at Speed |
| Scenario Description | Camera hard button is pressed |
| Post-conditions | Client displays pip 50/50 at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420623/A-Last Remembered view for PiP Trailer Rear Normal at Speed

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive mode and above 10kph speed. Camer view is off Last View at speed active was PiP Trailer Rear Normal at Speed |
| Scenario Description | Camera hard button is pressed |
| Post-conditions | Client displays Pip Trailer rear normal at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420624/A-Requesting AUX at speed view from PiP 50/50 at Speed view above 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph PiP 50/50 at Speed is active |
| Scenario Description | AUX at speed view button is selected |
| Post-conditions | Client displays Aux at Speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-420625/A-Requesting 50/50 at speed view from PiP AUX at Speed view above 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph PiP AUX at Speed is active |
| Scenario Description | 50/50 at speed view button is selected |
| Post-conditions | Client displays 50/50 at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-422845/A-Requesting Trailer Rear Normal at speed view from PiP 50/50 at Speed view above 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph PiP 50/50 at Speed is active |
| Scenario Description | Trailer Rear Normal at speed view button is selected |
| Post-conditions | Client displays trailer rear normal at speed, |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### CAMERA-UC-REQ-422844/A-Requesting 50/50 at speed from PiP Trailer Rear Normal at speed view above 10kph

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | Client is configured for Pip and PiP views at Speed Vehicle is in drive and above 10kph PiP Trailer Rear Normal view at Speed is active |
| Scenario Description | 50/50 at speed view button is selected |
| Post-conditions | Client displays 50/50 at speed. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

#### White Box Views

##### Activity Diagrams

###### CAMERA-ACT-REQ-422922/A-PiP Views

##### Sequence Diagrams

###### CAMERA-SD-REQ-422923/A-PiP Views

## Tailgate RVC

### CAMERA-FUN-REQ-421101/A-Tailgate RVC

#### Overview

This section is about client behavior when vehicle tailgate is provided to the client. As described below, when tailgate is down, it affects the various views that are available by disabling them.

#### Requirements

##### CAMERA-REQ-421102/A-Tailgate Down In Reverse

If client receives, LTgateSt = 0x1 (Down) and vehicle is in Reverse Gear, client shall not display certain views. For complete list of disabled views please consult with HMI specification.

##### CAMERA-REQ-421103/A-Tailgate Down Not In Reverse

If client receives, LTgateSt = 0x1 (Down) and vehicle is running and not in Reverse Gear, client shall not display certain views. For complete list of disabled views please consult with HMI specification.

##### CAMERA-REQ-421104/A-Tailgate Down And Vehicle Speed Above 10kph

If client receives, LTgateSt = 0x1 (Down) and vehicle is driving at speed above 10 kph, client shall not display certain views. For complete list of disabled views please consult with HMI specification.

##### CAMERA-REQ-421105/A-Tailgate States

If client receives, LTgateSt =0x0 (NotAvailable) OR LTgateSt = 0x3 (NotUsed_1), then client shall use default Tailgate position as Closed (LTgateSt =0x2).

#### Use Cases

##### UC-REQ-421106/A-Suppressed Views with Tailgate Down RVC in Reverse

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for Tailgate down RPA/RVC - Vehicle is in Run/Start |
| Scenario Description | - Vehicle is in Reverse Gear - Tailgate is Down |
| Post-conditions | Some camera views shall not be available on the Camera Menu. For a complete list of the views refer to HMI specification. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-421107/A-Suppressed Views with Tailgate Down RVC not in Reverse

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for Tailgate Down RVC/RPA and it is Present. - Vehicle is in Run/Start |
| Scenario Description | - Vehicle is not in Reverse - Tailgate is down |
| Post-conditions | Some camera views shall not be available on the Camera Menu. Refer to HMI spec for complete list of these views. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-421108/A-Suppressed Views-at-speed with Tailgate Down RVC

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for Tailgate Down RVC and it is enabled. - Vehicle is in Run/Start - Vehicle is in Reverse Gear |
| Scenario Description | - Tailgate is down. - Vehicle is driving above 10 kph |
| Post-conditions | Some camera views-at-speed shall not be available on the Camera Menu. For complete list of not available views refer to HMI specification. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with ADAS module E3 – Valid camera video signal not present |
| Interfaces |  |

#### White Box Views

##### Activity Diagrams

###### ACT-REQ-422407/A-Tailgate Camera Limiting

##### Sequence Diagrams

###### SD-REQ-422408/A-Tailgate Camera Limiting

## AUX IOD

### FUN-REQ-483789/A-AUX IOD

#### Use Cases

##### UC-REQ-483740/A-AUX IOD Activation

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for AUX IOD - Vehicle in Run/Start - Vehicle is not in reverse - Camera HMI is not active (Front Views, Front Views at Speed, Rear Views, APA, TBA/TRG) |
| Scenario Description | User Selects AUX IOD |
| Post-conditions | Client request AUX IOD via FBMP: CtrStkDsplyOpRq == 0x2 (Set) CtrStkFeatNo == 0x081B CtrStkFeatConfig == 0x4C (AUXIOD) CtrStkPersIndex == 0x4 (Vehicle) Client displays AUX IOD once AUX IOD active response is received from camera module |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483741/A-AUX IOD Deactivation via Camera Button

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT is configured for AUX IOD - Vehicle in Run/Start - Vehicle is below 10 KPH - Vehicle is not in reverse - AUX IOD is active |
| Scenario Description | Front camera button is pressed |
| Post-conditions | CLIENT will receive (CamraFrntStat_D_Stat = 1) along with FBMP signals for front camera view to show from camera module. Client displays the Front Camera View per FBMP signaling from camera module. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483743/A-AUX IOD Deactivation via Shift to Reverse

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT is configured for AUX IOD - Vehicle in Run/Start - Vehicle is below 10 KPH - Vehicle is not in reverse - AUX IOD is active |
| Scenario Description | Vehicle shifts to reverse |
| Post-conditions | Client displays the Rear Camera Menu |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483744/A-AUX IOD Deactivation via Pro Trailer Assist Activation

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT is configured for AUX IOD - Vehicle in Run/Start - Vehicle is not in reverse - AUX IOD is active |
| Scenario Description | Pro Trailer Assist (TBA, Auto Hitch) becomes active. |
| Post-conditions | Client displays the Pro Trailer Assist HMI |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483745/A-AUX IOD Deactivation via APA Activation

| Actors |  |
| --- | --- |
| Pre-conditions | - CLIENT is configured for AUX IOD - Vehicle in Run/Start - Vehicle is not in reverse - AUX IOD is active |
| Scenario Description | APA becomes active |
| Post-conditions | Client displays the APA HMI |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483747/A-AUX IOD Deactivation via Ignition

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT is configured for AUX IOD - Vehicle in Run/Start - Vehicle is not in reverse - AUX IOD is active |
| Scenario Description | Ignition != Run or Start |
| Post-conditions | Aux IOD becomes unavailable. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483748/A-AUX IOD while Shifting to Park

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT is configured for AUX IOD - Vehicle in Run/Start - Vehicle is not in reverse - AUX IOD is active |
| Scenario Description | Vehicle shifts to Park |
| Post-conditions | AUX IOD remains active |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483749/A-AUX IOD Deactivation via Sync

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT is configured for AUX IOD - Vehicle in Run/Start - Vehicle is not in reverse - AUX IOD is active |
| Scenario Description | IOD changes away from Aux IOD to a non-camera state (i.e. navigation, IOD changes) |
| Post-conditions | Client requests OFF through FBMP: CtrStkDsplyOpRq == 0x2 (Set) CtrStkFeatNo == 0x081B CtrStkFeatConfig == 0x00 (Off) CtrStkPersIndex == 0x4 (Vehicle) |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483750/A-AUX IOD Deactivation via Boundary Alert

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT is configured for AUX IOD - Vehicle in Run/Start - Vehicle is not in reverse - AUX IOD is active |
| Scenario Description | Boundary alert become active |
| Post-conditions | Boundary alert HMI becomes active |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483781/A-AUX IOD Deactivation via Off-road Mode

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT is configured for AUX IOD - Vehicle in Run/Start - Vehicle is not in reverse - AUX IOD is active |
| Scenario Description | Off-road mode becomes active. |
| Post-conditions | Off-road Front Camera menu HMI becomes active |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483751/A-Returning to AUX IOD from Parking Feature

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT is configured for AUX IOD - Vehicle in Run/Start - Vehicle is in a Camera Feature (Front Camera, Rear Camera, TBA, Autohitch, APA, Boundary Alert) - Previous Client display state is AUX IOD |
| Scenario Description | Camera feature becomes inactive |
| Post-conditions | Client request AUX IOD via FBMP: CtrStkDsplyOpRq == 0x2 (Set) CtrStkFeatNo == 0x081B CtrStkFeatConfig == 0x4C (AUXIOD) CtrStkPersIndex == 0x4 (Vehicle) Client displays AUX IOD Per X36A HMI spec once AUX IOD active response is received from camera module |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483752/A-No Response from ADAS/IPMB Handling

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT is configured for AUX IOD - Vehicle in Run/Start - Vehicle is in a Camera Feature (Front Camera, Rear Camera, TBA, Autohitch, APA, Boundary Alert) - Previous Client display state is AUX IOD |
| Scenario Description | Camera feature becomes inactive |
| Post-conditions | Client request AUX IOD via FBMP: CtrStkDsplyOpRq == 0x2 (Set) CtrStkFeatNo == 0x081B CtrStkFeatConfig == 0x4C (AUXIOD) CtrStkPersIndex == 0x4 (Vehicle) Client displays AUX IOD Per X36A HMI spec once AUX IOD active response is received from camera module |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483753/A-No Video from ADAS/IPMB Handling

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - CLIENT is configured for AUX IOD - Vehicle in Run/Start - Vehicle is not in reverse - Camera HMI is not active (Front Views, Front Views at Speed, Rear Views, APA, TBA/TRG) |
| Scenario Description | User Selects AUX IOD and Client does not detect video output |
| Post-conditions | Client shall display Camera Malfunction warning. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

## RCOD

### FUN-REQ-483788/A-RCOD

#### Use Cases

##### UC-REQ-483754/A-RCOD Black Box Not Configured ADAS

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - RCOD HMI w/o Black Box is Enabled - Vehicle in Run/Start - Vehicle is not in reverse - RCOD or RCOD at Speed is Configured in CLIENT - Vehicle has RVC connected to ADAS |
| Scenario Description | User Selects RCOD View or RCOD View at Speed |
| Post-conditions | Server sends: FeatConfigIPMB = 0x27 or 0x29 (RCOD or RCOD at Speed) FeatNoIPMBActl = 0x81B CLIENT shall show the RCOD HMI without the RCOD black box. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483755/A-RCOD Black Box Is Configured ADAS Variant

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - RCOD HMI w/o Black Box is Disabled - Vehicle in Run/Start - Vehicle is not in reverse - RCOD or RCOD at Speed is Configured in CLIENT - Vehicle has RVC connected to ADAS |
| Scenario Description | User Selects RCOD View or RCOD View at Speed |
| Post-conditions | Server sends: FeatConfigIPMB = 0x27 or 0x29 (RCOD or RCOD at Speed) FeatNoIPMBActl = 0x81B CLIENT shall show the RCOD HMI with the RCOD black box. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483757/A-RCOD Black Box Is Configured Non-ADAS Variant

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - RCOD HMI w/o Black Box is Disabled - Vehicle in Run/Start - Vehicle is not in reverse - RCOD or RCOD at Speed is Configured in CLIENT - Vehicle has Standalone Digital or Standalone Analog RVC system (camera video feed is connected directly to CLIENT) |
| Scenario Description | User presses the Camera button to activate RCOD. |
| Post-conditions | CLIENT Displays RCOD view. CLIENT shall show the RCOD HMI with the RCOD black box. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483756/A-RCOD Black Box Not Configured Non-ADAS variant

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - RCOD HMI w/o Black Box is Enabled - Vehicle in Run/Start - Vehicle is not in reverse - RCOD or RCOD at Speed is Configured in CLIENT - Vehicle has Standalone Digital or Standalone Analog RVC system (camera video feed is connected directly to CLIENT) |
| Scenario Description | User presses the Camera button to activate RCOD. |
| Post-conditions | CLIENT Displays RCOD view. CLIENT shall show the RCOD HMI without the RCOD black box. |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

## Front rear auto switch

This function allows the user to see the front 360view after shifting from reverse to drive, up to a speed of 30kph.

AVM will change from rear 360 view to front 360view by shifting from R to D. but when shift from R to N, and D to N, keep last view.

#### Use Cases

##### UC-REQ-483754/A-FR AUTO SWITCH ACTIVATE

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for FRONT REAR AUTO SWITCH - Vehicle in Run/Start - Vehicle is in reverse - Camera HMI is active (Rear Views) |
| Scenario Description | User shift from reverse to drive |
| Post-conditions | AVM respond with: FeatConfigIpmbActl=07 FeatNoIpmbActl=081B(2075) PersIndexIpmb_D_Actl=Vehicle Client displays front 360camera view |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483754/A-FR AUTO SWITCH DEACTIVATE

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - Client is configured for FRONT REAR AUTO SWITCH - Vehicle in Run/Start - Vehicle is in reverse - Camera HMI is active (Rear Views) |
| Scenario Description | User shift from reverse to drive, and speed is over 30kph. |
| Post-conditions | APIM request: CtrStkDsplyOp_D_Rq=set CtrStkFeatNoActl=081B(2075) CtrStkFeatConfigActl=00 CtrStkPersIndex_D_Actl=Vehicle Servers respond: FeatConfigIpmbActl=00 FeatNoIpmbActl=081B(2075) PersIndexIpmb_D_Actl=Vehicle Client exit front 360view and display SYNC interface |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

##### UC-REQ-483754/A-FR AUTO SWITCH NOT CONFIG IN APIM

| Actors | Vehicle Occupant |
| --- | --- |
| Pre-conditions | - FR AUTO SWITCH DISABLED IN APIM - Vehicle in Run/Start - Vehicle is in reverse - SYNC display rear camera view |
| Scenario Description | User shift from reverse to drive APIM display SYNC interface |
| Post-conditions | APIM display SYNC interface |
| List of Exception Use Cases | E1 – Vehicle is not RUN/START E2 – Loss of communication with IPMB module E3 – Valid camera video signal not present |
| Interfaces |  |

# Appendix: Reference Documents

| Reference # | Document Title |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |