|  | Function Group Spec F000171-Classic_Memory-DZARATE3 Custom Scope |  |  |  |
| --- | --- | --- | --- | --- |
| Document Type | Function Specification |  |  |  |
| Template Version | 6.0 |  |  |  |
| SysML Report Template Version | O Beta (11/6/2019) |  |  |  |
| Document ID | functionspec_sysmlreporttemplate |  |  |  |
| Document Location |  |  |  |  |
| Document Owner | MyName |  |  |  |
| Document Revision | FGS0 |  |  |  |
| Document Status | Draft |  |  |  |
| Date Issued | 2021-03-04 |  |  |  |
| Date Revised | 2021-03-04 |  |  |  |
| Document Classification | GIS1 Item Number: | 27.60/35 |  |  |
|  | GIS2 Classification: | Confidential |  |  |
| Document Approval |  |  |  |  |
| Name | Role |  | Email Confirmation | Date |

Auto-Generated by MagicDraw

Printed Copies are Uncontrolled

# Disclaimer

This document contains Ford Motor Company Confidential information. Disclosure of the information contained in any portion of this document is not permitted without the expressed, written consent of a duly authorized representative of Ford Motor Company, Dearborn, Michigan, U.S.A.

Copyright, Ó 2016  Ford Motor Company

This document contains information developed and accumulated by and for FORD MOTOR COMPANY. As such, it is a proprietary document, which, if disseminated to unauthorized persons, would provide others with restricted information, data, or procedures not otherwise available, exposing the FORD MOTOR COMPANY to potential harm.

Employees and suppliers having custody of this specification or authorized to use it must be cognizant of its proprietary nature and ensure that the information herein is not made available to unauthorized persons.

FORD MOTOR COMPANY reserves the right to protect this work as an unpublished copyrighted work in the event of an inadvertent or deliberate unauthorized publication. FORD MOTOR COMPANY also reserves its rights under copyright laws to protect this work as a published work.

This document or portions thereof shall not be distributed outside FORD MOTOR COMPANY without prior written consent. Refer all questions concerning disclosure to the author(s) or to any duly authorized representative of Ford Motor Company.

# Contents

1.1.1	Decomposition of   	4

Disclaimer	16

Contents	17

2	Introduction	19

2.1	Document Purpose	19

2.2	Document Scope	19

2.3	Document Audience	19

2.3.1	Stakeholder List	19

2.4	Document Organization	19

2.4.1	Document Context	19

2.4.2	Document Structure	19

2.5	Document Conventions	20

2.5.1	Requirements Templates	20

3	Function Group Description	21

4	Functional Architecture	22

4.1	Description	22

4.2	Function List	22

4.3	Signal List	22

5	Function Specifications	23

5.1	  	23

5.1.1	Function Overview	23

5.1.2	Function Scope	25

5.1.3	Function Interfaces	26

5.1.4	Function Modeling	28

5.1.5	Function Requirements	28

6	Open Concerns	32

7	Revision History	33

8	Appendix	34

8.1	Data Dictionary	34

8.1.1	Logical Signals	34

8.1.2	Logical Parameters	34

8.1.3	Encoding Types	34

8.2	Glossary	34

8.2.1	Definitions	34

8.2.2	Abbreviations	35

List of Figures

Figure 1 Activity diagram “” depicting black box behavior collaboration	22

Figure 2: Activity Diagram of   “” calling   “”	26

Figure 3: 	28

List of Tables

Table 1:  - Decomposition Table	5

Table 2: Functions described in this specification	19

Table 3: List of Logical Functions	22

Table 4: Ford internal Documents	25

Table 5: Ford internal Documents (not specified in model)	25

Table 6: External documents and publications	25

Table 7: External documents and publications (not specified in model)	25

Table 8: Open Concerns (Not supported by MagicDraw report generation.)	32

Table 9: Definitions used in this document	35

Table 10: Abbreviations used in this document	35

# Introduction

## Document Purpose

The Function (Group) Specification (FS) specifies an individual function / a group of functions.

To get more information about the concept of feature, function and component level abstraction refer to the Ford RE Wiki.

## Document Scope

The following set of functions from the Global Feature & Function List is described in this specification.

| Function ID | Function Name | Owner | Reference |
| --- | --- | --- | --- |

Table 2: Functions described in this specification

## Document Audience

The FS is authored by the owners of the individual functions. All Stakeholders, i.e., all people who have a valid interest in the functions and their behavior should read and, if possible, review the FS. It needs to be guaranteed, that all stakeholders have access to the currently valid version of the FS.

### Stakeholder List

For the latest stakeholder list of the feature and their influence refer to VSEM ID: VDOC087225.

## Document Organization

### Document Context

Refer to the Specification Structure page in the Ford RE Wiki to understand how the FS relates to other Ford Requirements Documents and Specifications.

### Document Structure

The structure of this document is explained below:

Section 1 – 	Introduction how to use this document including responsibilities and requisite documents. Explains the terminology. Gives a clarification of the definitions, concepts and abbreviations used in the document.

Section 2 – 	Function Group Description. Gives an overview and the purpose of the function group.

Section 3 – 	Functional Architecture: Specifies the overall functional architecture of the function group

Section 4 – 	Function Specifications: Specifies the logical functions of the function group in detail

Section 5 – 	List of Open Concerns

Section 6	Revision history including a list of new or modified requirements. The requirements in this document are tagged, and this section contains different types of tables listing all, new, or changed requirements by their title and page no.

Section 7 – 	Appendix: Presenting additional data mainly in a tabular form, e.g., a data dictionary

## Document Conventions

### Requirements Templates

Each requirement, use case or scenario in this specification shall follow the corresponding template given in the document template Specification_Macros.dotm at RE Wiki - Specification Templates.

#### Identification of Requirements

#### Requirements Attributes

The templates provided by Specification_Macros.dotm define a list of attributes for each requirement. This helps to classify the requirement. The attributes are explained at RE Wiki - Requirements Attributes.

# Function Group Description

This Function Group consists of all functions identified in a custom scope with the MagicDraw project F000171-Classic_Memory-DZARATE3.

# Functional Architecture

## Description

These system functions describe how Classic Memory will perform stores and recalls for the driver and passenger. There is a system function for Easy Entry Easy Exit. Domain functions that describe inputs and outputs from Classic Memory and Easy Entry Easy Exit are not included in this document. The Classic Memory Functional Architecture and Easy Entry Easy Exit Function Architecture diagrams describe the interfaces between the system and domain functions.

## Function List

| Function ID | Function Name | Function Description | ASIL |
| --- | --- | --- | --- |
| SFn:1 | Provide Memory Input Status <<System Function>> | The purpose of this function is to indicates when and which input the user selects. This function processes some of the input depending on the type of memory input available (Memory Set). |  |
| SFn:2 | Notify of Memory Set Indicator Status <<System Function>> | The purpose of this function is to alert the user that Memory Set is active. The user has activated the Memory Set functionality and this function will notify the user for the duration that it is active. |  |
| SFn:3 | Request Recall Position <<System Function>> | The purpose of this function is to determine that the result of the user input is a recall action. From this determination this function will then trigger a Classic Memory recall. This recall will retrieve stored settings from a user selected profile and apply them to the current settings. |  |
| SFn:4 | Read Mirrors Stored Position <<System Function>> | The purpose of this function is to obtain the stored value of the mirror positions. The settings will be applied to the current mirror positions. |  |
| SFn:5 | Read Seat Stored Position <<System Function>> | The purpose of this function is to obtain the stored value of the seat positions. The settings will be applied to the current seat positions. |  |
| SFn:6 | Read Steering Stored Position <<System Function>> | The purpose of this function is to obtain the stored value of the steering positions. The settings will be applied to the current steering positions. |  |
| SFn:7 | Read Pedals Stored Position <<System Function>> | The purpose of this function is to obtain the stored value of the pedal positions. The settings will be applied to the current pedal positions. |  |
| SFn:8 | Read AHUD Stored Brightness <<System Function>> | The purpose of this function is to obtain the stored value of the AHUD brightness. The settings will be applied to the current AHUD brightness. |  |
| SFn:9 | Move Mirrors Position <<System Function>> | The purpose of this function is to adjust the position of the mirrors to a stored value. This is the final step of the Classic Memory recall process. This function receives the stored settings and applies them. |  |
| SFn:10 | Move Seat Position <<System Function>> | The purpose of this function is to adjust the position of the seats to a stored value. This is the final step of the Classic Memory recall process. This function receives the stored settings and applies them. |  |
| SFn:11 | Move Steering Position <<System Function>> | The purpose of this function is to adjust the position of the steering column to a stored value. This is the final step of the Classic Memory recall process. This function receives the stored settings and applies them. |  |
| SFn:12 | Move Pedals Position <<System Function>> | (activity) The purpose of this function is to adjust the position of the pedals to a stored value. This is the final step of the Classic Memory recall process. This function receives the stored settings and applies them. |  |
| SFn:13 | Adjust AHUD Brightness <<System Function>> | The purpose of this function is to adjust the brightness of the AHUD to a stored value. This is the final step of the Classic Memory recall process. This function receives the stored settings and applies them. |  |
| SFn:14 | Request Store Position <<System Function>> | The purpose of this function is to determine that the result of the user input is a store action. From this determination this function will then trigger a Classic Memory store. This store will retrieve current settings and store them to the user select profile. |  |
| SFn:15 | Read Mirrors Current Position <<System Function>> | The purpose of this function is to obtain the current value of the mirror positions. This function also identifies a profile that the settings should be stored to. |  |
| SFn:16 | Read Seat Current Position <<System Function>> | The purpose of this function is to obtain the current value of the seat positions. This function also identifies a profile that settings should be stored to. |  |
| SFn:17 | Read Steering Current Position <<System Function>> | The purpose of this function is to obtain the current value of the steering positions. This function also identifies a profile that settings should be stored to. |  |
| SFn:18 | Read Pedals Current Position <<System Function>> | The purpose of this function is to obtain the current value of the pedal positions. This function also identifies a profile that settings should be stored to. |  |
| SFn:19 | Read AHUD Current Brightness <<System Function>> | The purpose of this function is to obtain the current value of the AHUD brightness. This function also identifies a profile that the settings should be stored to. |  |
| SFn:20 | Store Mirrors Position <<System Function>> | The purpose of this function is to store the position of the mirrors to a profile. This is the final step of the Classic Memory store process. This function receives the current settings and applies them to the user selected profile. |  |
| SFn:21 | Store Seat Position <<System Function>> | The purpose of this function is to store the position of the seats to a profile. This is the final step of the Classic Memory store process. This function receives the current settings and applies them to the user selected profile. |  |
| SFn:22 | Store Steering Position <<System Function>> | The purpose of this function is to store the position of the steering to a profile. This is the final step of the Classic Memory store process. This function receives the current settings and applies them to the user selected profile. |  |
| SFn:23 | Store Pedals Position <<System Function>> | The purpose of this function is to store the position of the pedals to a profile. This is the final step of the Classic Memory store process. This function receives the current settings and applies them to the user selected profile. |  |
| SFn:24 | Store AHUD Brightness <<System Function>> | The purpose of this function is to store the brightness of the AHUD to a profile. This is the final step of the Classic Memory store process. This function receives the current settings and applies them to the user selected profile. |  |
| SFn:25 | Detect Remote Transmitter <<System Function>> | activity) The purpose of this function is to detect when a remote transmitter sends out a Lock or Unlock signal. This function then indicates what signal was received and by which remote transmitter. Other functions will use this data to determine if a recall should occur or if a profile should add/remove a remote transmitter. |  |
| SFn:26 | Associate Remote Transmitter <<System Function>> | (activity) The purpose of this function is to add or remove a remote transmitter from a profile. |  |
| SFn:27 | Request EEEE <<System Function>> | The purpose of this function is to determine when the user is entering and exiting the vehicle. From that determination this function will then adjust the seat and steering positions. |  |

Table 3: List of Logical Functions

## Signal List

Refer to the Data Dictionary - Logical Signals.

# Function Specifications

## Provide Memory Input Status

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Memory Input System <<Logical>>

The purpose of this function is to indicates when and which input the user selects. This function processes some of the input depending on the type of memory input available (Memory Set).

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Provide Memory Input Status” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Detect User Input”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Provide Memory Input Status”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Memory_Input_Configuration Type: Set Indicator - Set Available/Set NOT Available | Signal Description: These values indicate whether Memory Set functionality is available to the user. Usage in Function: Signal Description: Contains the configuration implemented on the vehicle. From this configuration the function can determine if the vehicle has a Set Indicator and 2 memory locations or 3 memory locations available to the user |
| User_Input Type: User Selection - Selection/No Selection | Signal Description: These values indicate the input of the user. Usage in Function: Signal Description: Indicates when the user is giving input (selection) Received from: - Store/Recall Positional Settings |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Memory_Input_Status Type: Input Status - Recall/Set/Pers/None | Signal Description: These values indicate how the input from the user was perceived by the Classic Memory system. When Memory Set is selected then the input function will wait until a profile is selected before transmitting the information. Otherwise the input function relays the input from the user to the rest of the Classic Memory system. Usage in Function: Signal Description: Contains the value of the Memory Input. Indicates if a memory location was selected. Additionally may indicated if a recall or store was selected. Sent to: - Request Recall Position - Request Store Position |
| Memory_Set_Indicator_Status Type: Set Indicator Status - Active/Inactive | Signal Description: These values indicate when Memory Set is active. When active the indicator will notify the user for the duration that Memory Set is active. Usage in Function: Signal Description: Contains the value to determine if the Memory Set Indicator should be active Sent to: - Notify of Memory Set Indicator Status - Display HMI - Notify Audio |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Provide Memory Input Status

## Notify of Memory Set Indicator Status

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Memory Input System <<Logical>>

The purpose of this function is to alert the user that Memory Set is active. The user has activated the Memory Set functionality and this function will notify the user for the duration that it is active.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Notify of Memory Set Indicator Status” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Detect User Input”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Notify of Memory Set Indicator Status”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Memory_Set_Indicator_Status Type: Set Indicator Status - Active/Inactive | Signal Description: These values indicate when Memory Set is active. When active the indicator will notify the user for the duration that Memory Set is active. Usage in Function: Signal Description: Contains the value to determine if the Memory Set Indicator should be active Received from: - Provide Memory Input Status |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Notify of Memory Set Indicator Status

### Function Requirements

#### Functional Requirements

##### Normal Operation

28 Personalization - Memory Set Illuminate 1

While the 'Notify of Memory Set Indicator Status' function receives 'Memory_Set_Indicator_Status' as 'Active' then the 'Notify of Memory Set Indicator Status' function shall illuminate the Set Indicator until the 'Notify of Memory Set Indicator Status' function receives 'Memory_Set_Indicator_Status' as 'Inactive'

| Requirement ID: 28 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_21 Personalization - Memory Set Illuminate |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

29 Personalization – Memory Set Illuminate 2

While the 'Notify of Memory Set Indicator Status' function receives 'Memory_Set_Indicator_Status' as 'Inactive' then the 'Notify of Memory Set Indicator Status' function shall cease illuminating the Set Indicator

| Requirement ID: 29 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request Recall Position

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

The purpose of this function is to determine that the result of the user input is a recall action. From this determination this function will then trigger a Classic Memory recall. This recall will retrieve stored settings from a user selected profile and apply them to the current settings.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Recall Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Easy Entry Easy Exit Functional Architecture”

- – “Recall Positional Settings”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Request Recall Position”

Description of “Easy Entry Easy Exit Functional Architecture” diagram:

This diagram contains all the functions required to adjust seats and steering to provide additional space for the user to enter and exit. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Easy Entry Easy Exit Functional Architecture” calling   “Request Recall Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Manual_Adjustment Type: Manual Adjustment - Active/Inactive | Signal Description: These values indicate when a memory controller system has determined that a manual adjustment occurred. Manual adjustments cause automatic movement to cancel. Usage in Function: Signal Description: Indicates that positional system which has a memory controller system made a manual adjustment Received from: - Notify Mirrors Manual Adjustement - Notify Seat Manual Adjustment - Notify Multicontour Seat Manual Adjustment - Notify Steering Manual Adjustment - Notify AHUD Manual Adjustment - Notify Pedals Manual Adjustment |
| Memory_Input_Status Type: Input Status - Recall/Set/Pers/None | Signal Description: These values indicate how the input from the user was perceived by the Classic Memory system. When Memory Set is selected then the input function will wait until a profile is selected before transmitting the information. Otherwise the input function relays the input from the user to the rest of the Classic Memory system. Usage in Function: Signal Description: Contains the value of the Memory Input. Indicates if a memory location was selected. Additionally may indicated if a recall or store was selected. Received from: - Provide Memory Input Status |
| Feature_Configuration_Signal Type: Feature Configuration - Classic/Advanced | Signal Description: These values indicate whether the standard or variant configuration should be executed. The Advanced Classic Memory variant provides remote transmitter functionality and modified prompts. Usage in Function: Indicates the configuration of Classic Memory Received from: - Provide Feature Configuration |
| Remote_Transmitter_Id Type: Integer | Received from: - Detect Remote Transmitter |
| Restriction Settings Status Type: Restriction Settings - Factory/Vehicle/Moveable/Non-Moveable | Signal Description: These values indicate which restrictions are applied to profiles. Usage in Function: Contains the value for the Restriction Status. The Restriction Statuses limit profiles, stores, and recalls. Received from: - Provide Restriction Settings Status |
| Vehicle_Speed Type: Vehicle Speed - Slow/NOT Slow | Signal Description: These values indicate whether the State Management System has determined that the current vehicle speed is at a speed where it would be safe to trigger a Classic Memory recall. When the speed is Slow then a Classic Memory recall may trigger. Usage in Function: Signal Description: Indicates if the vehicle speed is SLOW Received from: - Provide Vehicle Speed |
| EM_Recall Type: EM Recall - Request/Inactive | Signal Description: These values indicate that an Enhanced Memory recall has occurred and for which memory location. This is to indicate to the Classic Memory system that it should perform a Classic Memory recall for the same memory location. Usage in Function: Signal Description: Indicates that Enhanced Memory is performing a recall Received from: - Notify of Enhanced Memory Recall |
| Gear Type: Gear Status - Safe/NOT Safe | Signal Description: These values indicate whether the State Management System has determined that the current transmission gear is in a position where it would be safe to trigger a Classic Memory recall. Usage in Function: Indicates if it is Safe to do a recall Received from: - Provide Transmission Status |
| Remote_Transmitter_Status Type: Remote Transmitter Status - Lock/Unlock Pers 1-4 | Received from: - Detect Remote Transmitter |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Memory_Command Type: Memory Command - Store/Recall | Signal Description: These values indicate whether memory controller systems should perform a Classic Memory store or Classic Memory recall. The value indicates the memory location to be used in the operation. Usage in Function: Signal Description: Contains the value of the memory location to be affected. Indicates if the memory location should be recalled or updated (store). Sent to: - Request EEEE - Read Seat Stored Position - Read Pedals Stored Position - Read Steering Stored Position - Recall Multicontour Seat - Read Mirrors Stored Position - Read AHUD Stored Brightness |
| Deny_Recall Type: Deny Recall - Deny/Inactive | Sent to: - Display HMI |
| CM_Recall Type: CM Recall - Request/Inactive | Signal Description: These values indicate that a Classic Memory recall has occurred and for which memory location. This is to indicate to the Enhanced Memory system that it should perform an Enhanced Memory recall for the same memory location. Usage in Function: Indicates that Classic Memory is doing a recall Sent to: - Request Recall Settings |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Request Recall Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

8 Personalization - Classic Memory Recall Methods 1

"When the 'Request Recall Position' function receives one of the following

'Memory_Input Status' value of:

- 'Recall 1'

- 'Recall 2'

- 'Store 1'

- 'Store 2'

- 'Pers 1' for less than Memory Selection Period

- 'Pers 2' for less than Memory Selection Period

- 'Pers 3' for less than Memory Selection Period

- 'Pers 4' for less than Memory Selection Period

'EM Recall' value of

- 'Request 1'

- 'Request 2'

- 'Request 3'

- 'Request 4'

Then 'Request Recall Position' shall trigger a Classic Memory recall from the memory location indicated in 'Memory_Input_Status'"

| Requirement ID: 8 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

9 Personalization - Classic Memory Recall Methods 2

"When the 'Request Recall Position' function triggers a Classic Memory recall, received signal 'Gear' is 'Recall Available', and received signal 'Vehicle Speed' is 'Slow' then it shall ouput 'Memory_Command' as 'Recall' + the memory location indicated in 'Memory_Input_Status' to the following functions

- Read Mirrors Stored Position

- Read Seat Stored Position

- Read Steering Stored Position

- Read Pedals Stored Position

- Read AHUD Stored Brightness"

| Requirement ID: 9 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

42 Enhanced Memory Recall Notification

When the 'Request Recall Position' function receives 'EM Recall' as 'Request 1', 'Request 2', 'Request 3', or 'Request 4' then the 'Request Recall Position' function shall trigger a Classic Memory recall

| Requirement ID: 42 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

43 Classic Memory Recall Notification

When 'Request Recall Settings' function receives 'CM Recall' as 'Request 1', 'Request 2', 'Request 3', or 'Request 4' then the 'Request Recall Settings' function shall trigger a Enhanced Memory recall

| Requirement ID: 43 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

44 Recall Prevention - CM Gear

When the 'Request Recall Position' function triggers a Classic Memory recall and received signal 'Gear' is 'Recall Restricted' then it shall output 'Deny Recall' as 'Deny'

| Requirement ID: 44 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

45 Recall Prevention - CM Speed

When the 'Request Recall Position' function triggers a Classic Memory recall and received signal 'Vehicle Speed' is ' NotSlow' then it shall output 'Deny Recall' as 'Deny'

| Requirement ID: 45 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

46 Recall Prevention - EEEE Speed

When the 'Request EEEE' function receives 'Vehicle Speed' as 'Not Slow' then the '' function shall restrict EEEE requests from triggering for the driver

| Requirement ID: 46 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

59 Classic Memory Restriction Settings - Factory Setting 1

When the 'Request Store Position' function receives 'Restriction Setting Status' as 'Factory' then the 'Request Store Position' function shall set all memory locations to Default Profile

| Requirement ID: 59 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_34 Classic Memory Restriction Settings - Factory Setting |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

60 Classic Memory Restriction Settings - Factory Setting 2

While a profile is a Default Profile the 'Request Store Position' function shall inhibit the trigger of a Classic Memory store for that profile

| Requirement ID: 60 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

61 Classic Memory Restriction Settings - Vehicle Setting 1

When the 'Request Store Position' function receives 'Restriction Setting Status' as 'Vehicle' then the 'Request Store Position' function shall set all memory locations to Vehicle Profile

| Requirement ID: 61 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_35 Classic Memory Restriction Settings - Vehicle Setting |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

62 Classic Memory Restriction Settings - Vehicle Setting 2

While a profile is a Vehicle Profile when a Classic Memory store triggers then the 'Request Store Position' function shall trigger a Classic Memory store for all Vehicle Profiles

| Requirement ID: 62 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

63 Classic Memory Restriction Settings - Moveable Person Setting 1

When the 'Request Store Position' function receives 'Restriction Setting Status' as 'Moveable_Person' then the 'Request Store Position' function shall set all memory locations to Moveable_Person Profile

| Requirement ID: 63 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_36 Classic Memory Restriction Settings - Moveable Person Setting |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

64 Classic Memory Restriction Settings - Non-Moveable Person Setting 1

When the 'Request Store Position' function receives 'Restriction Setting Status' as 'Non_Moveable_Person' then the 'Request Store Position' function shall set all memory locations to Non_Moveable_Person Profile

| Requirement ID: 64 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_37 Classic Memory Restriction Settings - Non-Moveable Person Setting |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

65 Classic Memory Restriction Settings - Non-Moveable Person Setting 2

While a profile is a Non_Moveable_Person Profile the 'Request Store Position' function shall inhibit the the transfer of setting from that profile to an external storage location and vice versa

| Requirement ID: 65 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

32 Personalization - Memory Selection Period

Memory Selection Period is a configurable parameter with a default duration of 1.5 seconds

| Requirement ID: 32 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

#### Uncategorized Requirements

58 Classic Memory Restriction Settings - Settings 1

"The valid Restriction Settings for the Classic Memory feature are:

- Factory

- Vehicle

- Moveable_Person

- Non_Moveable_Person"

| Requirement ID: 58 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_33 Classic Memory Restriction Settings - Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Read Mirrors Stored Position

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

The purpose of this function is to obtain the stored value of the mirror positions. The settings will be applied to the current mirror positions.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Read Mirrors Stored Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Recall Positional Settings”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Read Mirrors Stored Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Memory_Command Type: Memory Command - Store/Recall | Signal Description: These values indicate whether memory controller systems should perform a Classic Memory store or Classic Memory recall. The value indicates the memory location to be used in the operation. Usage in Function: Signal Description: Indicates that the function should transmit the Mirror Position values stored in the corresponding memory location Received from: - Request Recall Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Stored_Mirrors_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the stored values of the Mirror Positions Sent to: - Store Mirrors Position - Move Mirrors Position |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Read Mirrors Stored Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

9 Personalization - Classic Memory Recall Methods 2

"When the 'Request Recall Position' function triggers a Classic Memory recall, received signal 'Gear' is 'Recall Available', and received signal 'Vehicle Speed' is 'Slow' then it shall ouput 'Memory_Command' as 'Recall' + the memory location indicated in 'Memory_Input_Status' to the following functions

- Read Mirrors Stored Position

- Read Seat Stored Position

- Read Steering Stored Position

- Read Pedals Stored Position

- Read AHUD Stored Brightness"

| Requirement ID: 9 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

#### Uncategorized Requirements

94 Personalization - Memory Location Settings 1

Classic Memory shall provide only one set of positional settings per memory location per memory commodity

| Requirement ID: 94 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_64 Personalization - Memory Location Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

96 Memory Seat - Memory Mirrors

Where a vehicle has Driver Seat memory then that vehicle shall also have the Memory Mirrors (Power Exterior Side Mirrors)

| Requirement ID: 96 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

97 Memory Mirrors - Settings 1

"Where Power Exterior Side Mirrors is supported, then the Classic Memory feature shall include the following positional settings for:

- Memory Driver Side Mirror

- Memory Passenger Side Mirror"

| Requirement ID: 97 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_68 Memory Mirrors - Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

98 Memory Seat - Memory Locations 1

Classic Memory shall provide up to 4 memory locations per user

| Requirement ID: 98 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_65 Memory Seat - Memory Locations |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Read Seat Stored Position

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

The purpose of this function is to obtain the stored value of the seat positions. The settings will be applied to the current seat positions.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Read Seat Stored Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Recall Positional Settings”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Read Seat Stored Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Memory_Command Type: Memory Command - Store/Recall | Signal Description: These values indicate whether memory controller systems should perform a Classic Memory store or Classic Memory recall. The value indicates the memory location to be used in the operation. Usage in Function: Signal Description: Indicates that the function should transmit the Seat values stored in the corresponding memory location Received from: - Request Recall Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Stored_Seat_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the stored values of the Seat Sent to: - Move Seat Position - Store Seat Position |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Read Seat Stored Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

9 Personalization - Classic Memory Recall Methods 2

"When the 'Request Recall Position' function triggers a Classic Memory recall, received signal 'Gear' is 'Recall Available', and received signal 'Vehicle Speed' is 'Slow' then it shall ouput 'Memory_Command' as 'Recall' + the memory location indicated in 'Memory_Input_Status' to the following functions

- Read Mirrors Stored Position

- Read Seat Stored Position

- Read Steering Stored Position

- Read Pedals Stored Position

- Read AHUD Stored Brightness"

| Requirement ID: 9 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

#### Uncategorized Requirements

94 Personalization - Memory Location Settings 1

Classic Memory shall provide only one set of positional settings per memory location per memory commodity

| Requirement ID: 94 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_64 Personalization - Memory Location Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

96 Memory Seat - Memory Mirrors

Where a vehicle has Driver Seat memory then that vehicle shall also have the Memory Mirrors (Power Exterior Side Mirrors)

| Requirement ID: 96 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

98 Memory Seat - Memory Locations 1

Classic Memory shall provide up to 4 memory locations per user

| Requirement ID: 98 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_65 Memory Seat - Memory Locations |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

99 Memory Seat - Driver Settings 1

Where the Power Driver Seat is supported then the Classic Memory feature shall include positional settings for Memory Driver Seat

| Requirement ID: 99 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_69 Memory Seat - Driver Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

100 Memory Seat - Passenger Settings 1

Where the Power Passenger Seat is supported then the Classic Memory feature shall allow positional settings for Memory Passenger Seat

| Requirement ID: 100 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_70 Memory Seat - Passenger Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Read Steering Stored Position

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

The purpose of this function is to obtain the stored value of the steering positions. The settings will be applied to the current steering positions.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Read Steering Stored Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Recall Positional Settings”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Read Steering Stored Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Memory_Command Type: Memory Command - Store/Recall | Signal Description: These values indicate whether memory controller systems should perform a Classic Memory store or Classic Memory recall. The value indicates the memory location to be used in the operation. Usage in Function: Signal Description: Indicates that the function should transmit the Steering values stored in the corresponding memory location Received from: - Request Recall Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Stored_Steering_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the stored values of the Steering Sent to: - Move Steering Position - Store Steering Position |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Read Steering Stored Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

9 Personalization - Classic Memory Recall Methods 2

"When the 'Request Recall Position' function triggers a Classic Memory recall, received signal 'Gear' is 'Recall Available', and received signal 'Vehicle Speed' is 'Slow' then it shall ouput 'Memory_Command' as 'Recall' + the memory location indicated in 'Memory_Input_Status' to the following functions

- Read Mirrors Stored Position

- Read Seat Stored Position

- Read Steering Stored Position

- Read Pedals Stored Position

- Read AHUD Stored Brightness"

| Requirement ID: 9 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

#### Uncategorized Requirements

94 Personalization - Memory Location Settings 1

Classic Memory shall provide only one set of positional settings per memory location per memory commodity

| Requirement ID: 94 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_64 Personalization - Memory Location Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

98 Memory Seat - Memory Locations 1

Classic Memory shall provide up to 4 memory locations per user

| Requirement ID: 98 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_65 Memory Seat - Memory Locations |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

101 Memory Steering Column - Settings 1

Where Power Tilt/Telescoping Memory column is supported, then the Classic Memory feature shall include positional settings for the Memory Steering Column

| Requirement ID: 101 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_71 Memory Steering Column - Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Read Pedals Stored Position

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

The purpose of this function is to obtain the stored value of the pedal positions. The settings will be applied to the current pedal positions.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Read Pedals Stored Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Recall Positional Settings”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Read Pedals Stored Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Memory_Command Type: Memory Command - Store/Recall | Signal Description: These values indicate whether memory controller systems should perform a Classic Memory store or Classic Memory recall. The value indicates the memory location to be used in the operation. Usage in Function: Signal Description: Indicates that the function should transmit the Pedal values stored in the corresponding memory location Received from: - Request Recall Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Stored_Pedals_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the stored values of the Pedals Sent to: - Store Pedals Position - Move Pedals Position |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Read Pedals Stored Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

9 Personalization - Classic Memory Recall Methods 2

"When the 'Request Recall Position' function triggers a Classic Memory recall, received signal 'Gear' is 'Recall Available', and received signal 'Vehicle Speed' is 'Slow' then it shall ouput 'Memory_Command' as 'Recall' + the memory location indicated in 'Memory_Input_Status' to the following functions

- Read Mirrors Stored Position

- Read Seat Stored Position

- Read Steering Stored Position

- Read Pedals Stored Position

- Read AHUD Stored Brightness"

| Requirement ID: 9 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

#### Uncategorized Requirements

94 Personalization - Memory Location Settings 1

Classic Memory shall provide only one set of positional settings per memory location per memory commodity

| Requirement ID: 94 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_64 Personalization - Memory Location Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

95 Memory Pedals - Settings 1

"Where Power Adjustable Pedals is supported, then the Classic Memory feature shall include the following positional settings:

- Memory Gas Pedal

- Memory Brake Pedal

- Memory Clutch Pedal (on vehicles with manual transmission only)"

| Requirement ID: 95 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_66 Memory Pedals - Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

98 Memory Seat - Memory Locations 1

Classic Memory shall provide up to 4 memory locations per user

| Requirement ID: 98 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_65 Memory Seat - Memory Locations |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Read AHUD Stored Brightness

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

The purpose of this function is to obtain the stored value of the AHUD brightness. The settings will be applied to the current AHUD brightness.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Read AHUD Stored Brightness” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Recall Positional Settings”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Read AHUD Stored Brightness”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Memory_Command Type: Memory Command - Store/Recall | Signal Description: These values indicate whether memory controller systems should perform a Classic Memory store or Classic Memory recall. The value indicates the memory location to be used in the operation. Usage in Function: Signal Description: Indicates that the function should transmit the AHUD Brightness values stored in the corresponding memory location Received from: - Request Recall Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Stored_AHUD_Setting Type: Brightness | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the stored value of the AHUD Brightness Sent to: - Store AHUD Brightness - Adjust AHUD Brightness |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Read AHUD Stored Brightness

### Function Requirements

#### Functional Requirements

##### Normal Operation

9 Personalization - Classic Memory Recall Methods 2

"When the 'Request Recall Position' function triggers a Classic Memory recall, received signal 'Gear' is 'Recall Available', and received signal 'Vehicle Speed' is 'Slow' then it shall ouput 'Memory_Command' as 'Recall' + the memory location indicated in 'Memory_Input_Status' to the following functions

- Read Mirrors Stored Position

- Read Seat Stored Position

- Read Steering Stored Position

- Read Pedals Stored Position

- Read AHUD Stored Brightness"

| Requirement ID: 9 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

#### Uncategorized Requirements

94 Personalization - Memory Location Settings 1

Classic Memory shall provide only one set of positional settings per memory location per memory commodity

| Requirement ID: 94 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_64 Personalization - Memory Location Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

98 Memory Seat - Memory Locations 1

Classic Memory shall provide up to 4 memory locations per user

| Requirement ID: 98 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_65 Memory Seat - Memory Locations |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

102 Memory HUD - Settings 1

Where the HUD feature is supported, then the Classic Memory feature shall include positional settings for the Advanced Heads Up Display (AHUD)

| Requirement ID: 102 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_72 Memory HUD - Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Move Mirrors Position

### Function Overview

#### Description

Function is allocated to:

- Vehicle Body System <<Logical>>

- Vehicle Door System <<Logical>>

The purpose of this function is to adjust the position of the mirrors to a stored value. This is the final step of the Classic Memory recall process. This function receives the stored settings and applies them.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Move Mirrors Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Recall Side Mirrors Position”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Move Mirrors Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Current_Mirrors_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Contains the current value of the Mirror Positions Received from: - Read Mirrors Current Position |
| Stored_Mirrors_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the value that the current position will be updated to Received from: - Read Mirrors Stored Position |
| Manual_Adjustment Type: Manual Adjustment - Active/Inactive | Signal Description: These values indicate when a memory controller system has determined that a manual adjustment occurred. Manual adjustments cause automatic movement to cancel. Usage in Function: Signal Description: Indicates that positional system which has a memory controller system made a manual adjustment Received from: - Notify Mirrors Manual Adjustement - Notify Seat Manual Adjustment - Notify Multicontour Seat Manual Adjustment - Notify Steering Manual Adjustment - Notify AHUD Manual Adjustment - Notify Pedals Manual Adjustment |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Mirrors_Settings Type: Mirrors Movement | Sent to: - Receive Positional Status |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Move Mirrors Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

10 Personalization - Classic Memory Recall Methods 3

"When the function reading the stored setting for a memory controller system obtains the stored setting then it shall output the stored setting to the corresponding move functions

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Adjust AHUD Brightness"

| Requirement ID: 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

13 Personalization - Recall Cancels Automatic Movement 1

"While moving to a saved position (recall) when a new saved position (recall) is received then the following functions shall cease the current movement (recall) and process the new movement (recall)

- 'Move Mirrors Position' function

- 'Move Seat Position' function

- 'Move Steering Position' function

- 'Move Pedals Position' function

- 'Adjust AHUD Brightness' function"

| Requirement ID: 13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_12 Personalization - Recall Cancels Automatic Movement |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

14 Personalization - Manual Move Cancels Automatic Movement 1

"When the following functions determine their respective commodity was manually adjusted then the following functions shall set their output as 'Active'

- 'Notify Mirrors Manual Adjustment' function

- 'Notify Seat Manual Adjustment' function

- 'Notify Multicontour Seat Manual Adjustment' function

- 'Notify Steering Manual Adjustment' function

- 'Notify Pedals Manual Adjustment' function

- 'Notify AHUD Manual Adjustment' function"

| Requirement ID: 14 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_10 Personalization - Manual Move Cancels Automatic Movement |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

15 Personalization - Manual Move Cancels Automatic Movement 2

"While processing a Classic Memory recall when the following functions are notifiied of any manual adjustment as 'Active' then the following functions shall cease the current recall

- 'Move Mirrors Position' function

- 'Move Seat Position' function

- 'Move Steering Position' function

- 'Move Pedals Position' function

- 'Adjust AHUD Brightness' function"

| Requirement ID: 15 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

40 Recall Prevention - Compare Data 1

"When receiving the stored setting the following functions shall compare the stored setting to the current setting

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Move AHUD Brightness"

| Requirement ID: 40 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

41 Recall Prevention - Matching Data 1

"When the function determines from the comparison of the current setting and stored setting that the settings do not match then the following functions shall update the current setting with the stored setting

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Move AHUD Brightness"

| Requirement ID: 41 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified

## Move Seat Position

### Function Overview

#### Description

Function is allocated to:

- Seat System <<Logical>>

- Vehicle Body System <<Logical>>

The purpose of this function is to adjust the position of the seats to a stored value. This is the final step of the Classic Memory recall process. This function receives the stored settings and applies them.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Move Seat Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Easy Entry Easy Exit Functional Architecture”

- – “Recall Seat Position”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Move Seat Position”

Description of “Easy Entry Easy Exit Functional Architecture” diagram:

This diagram contains all the functions required to adjust seats and steering to provide additional space for the user to enter and exit. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Easy Entry Easy Exit Functional Architecture” calling   “Move Seat Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Current_Seat_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Contains the current values of the Seat Received from: - Read Seat Current Position |
| Stored_Seat_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the value that the current position will be updated to Received from: - Read Seat Stored Position |
| Manual_Adjustment Type: Manual Adjustment - Active/Inactive | Signal Description: These values indicate when a memory controller system has determined that a manual adjustment occurred. Manual adjustments cause automatic movement to cancel. Usage in Function: Signal Description: Indicates that positional system which has a memory controller system made a manual adjustment Received from: - Notify Mirrors Manual Adjustement - Notify Seat Manual Adjustment - Notify Multicontour Seat Manual Adjustment - Notify Steering Manual Adjustment - Notify AHUD Manual Adjustment - Notify Pedals Manual Adjustment |
| EEEE_Request Type: EEEE Request - EEEE/Drive | Signal Description: These values indicate whether the request wants Easy Entry Easy Exit controlled commodities to move to Easy Entry Easy Exit position or the Drive Position. Usage in Function: Signal Description: Contains the EEEE value that the current position will be updated to Received from: - Request EEEE |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Seat_Settings Type: Seat Movement | Sent to: - Receive Positional Status |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Move Seat Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

10 Personalization - Classic Memory Recall Methods 3

"When the function reading the stored setting for a memory controller system obtains the stored setting then it shall output the stored setting to the corresponding move functions

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Adjust AHUD Brightness"

| Requirement ID: 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

13 Personalization - Recall Cancels Automatic Movement 1

"While moving to a saved position (recall) when a new saved position (recall) is received then the following functions shall cease the current movement (recall) and process the new movement (recall)

- 'Move Mirrors Position' function

- 'Move Seat Position' function

- 'Move Steering Position' function

- 'Move Pedals Position' function

- 'Adjust AHUD Brightness' function"

| Requirement ID: 13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_12 Personalization - Recall Cancels Automatic Movement |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

14 Personalization - Manual Move Cancels Automatic Movement 1

"When the following functions determine their respective commodity was manually adjusted then the following functions shall set their output as 'Active'

- 'Notify Mirrors Manual Adjustment' function

- 'Notify Seat Manual Adjustment' function

- 'Notify Multicontour Seat Manual Adjustment' function

- 'Notify Steering Manual Adjustment' function

- 'Notify Pedals Manual Adjustment' function

- 'Notify AHUD Manual Adjustment' function"

| Requirement ID: 14 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_10 Personalization - Manual Move Cancels Automatic Movement |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

15 Personalization - Manual Move Cancels Automatic Movement 2

"While processing a Classic Memory recall when the following functions are notifiied of any manual adjustment as 'Active' then the following functions shall cease the current recall

- 'Move Mirrors Position' function

- 'Move Seat Position' function

- 'Move Steering Position' function

- 'Move Pedals Position' function

- 'Adjust AHUD Brightness' function"

| Requirement ID: 15 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

40 Recall Prevention - Compare Data 1

"When receiving the stored setting the following functions shall compare the stored setting to the current setting

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Move AHUD Brightness"

| Requirement ID: 40 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

41 Recall Prevention - Matching Data 1

"When the function determines from the comparison of the current setting and stored setting that the settings do not match then the following functions shall update the current setting with the stored setting

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Move AHUD Brightness"

| Requirement ID: 41 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

76 Easy Entry Easy Exit - Manual Move - Cancel Automatic Movement 1

While the 'Move Seat Position' function or the 'Move Steering Position' function are in the process of automatic movement triggered by an Easy Entry Easy Exit request when either receive 'Seat_Manual_Adjustment' as 'Active' or 'Steering_Manual_Adjustment' as 'Active' then both functions shall cease the automatic movement

| Requirement ID: 76 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_51 Easy Entry Easy Exit - Manual Move - Cancel Automatic Movement |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

77 Easy Entry Easy Exit - Manual Move - Drive Position 1

When the 'Move Seat Position' function or the 'Move Steering Position' function receive 'Seat_Manual_Adjustment' as 'Active' or 'Steering_Manual_Adjustment' as 'Active' then both functions shall consider the current position the drive position

| Requirement ID: 77 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_52 Easy Entry Easy Exit - Manual Move - Drive Position |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

80 Easy Entry Easy Exit - Seat Offset 1

When the 'Move Seat Position' function receives 'EEEE_Request_Seat' as 'EEEE' then the 'Move Seat Position' function shall remember the current position and move the seat rearward by EE Seat Horizontal Offset

| Requirement ID: 80 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_55 Easy Entry Easy Exit - Seat Offset |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

81 Easy Entry Easy Exit - Seat Offset 2

While the seat position is in the Easy Entry Easy Exit offset when the 'Move Seat Position' function receives 'EEEE_Request_Seat' as 'Drive' then the 'Move Seat Position' function shall move the seat to the remembered position

| Requirement ID: 81 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

82 Easy Entry Easy Exit - EE Seat Horizontal Offset

EE Seat Horizintal Offset is a configurable parameter with a default distance of 50mm

| Requirement ID: 82 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Move Steering Position

### Function Overview

#### Description

Function is allocated to:

- Steering Wheel System <<Logical>>

- Vehicle Body System <<Logical>>

The purpose of this function is to adjust the position of the steering column to a stored value. This is the final step of the Classic Memory recall process. This function receives the stored settings and applies them.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Move Steering Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Easy Entry Easy Exit Functional Architecture”

- – “Recall Steering Column Position”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Move Steering Position”

Description of “Easy Entry Easy Exit Functional Architecture” diagram:

This diagram contains all the functions required to adjust seats and steering to provide additional space for the user to enter and exit. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Easy Entry Easy Exit Functional Architecture” calling   “Move Steering Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| EEEE_Request Type: EEEE Request - EEEE/Drive | Signal Description: These values indicate whether the request wants Easy Entry Easy Exit controlled commodities to move to Easy Entry Easy Exit position or the Drive Position. Usage in Function: Signal Description: Contains the EEEE value that the current position will be updated to Received from: - Request EEEE |
| Stored_Steering_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the value that the current position will be updated to Received from: - Read Steering Stored Position |
| Current_Steering_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Contains the current values of the Steering wheel Received from: - Read Steering Current Position |
| Manual_Adjustment Type: Manual Adjustment - Active/Inactive | Signal Description: These values indicate when a memory controller system has determined that a manual adjustment occurred. Manual adjustments cause automatic movement to cancel. Usage in Function: Signal Description: Indicates that positional system which has a memory controller system made a manual adjustment Received from: - Notify Mirrors Manual Adjustement - Notify Seat Manual Adjustment - Notify Multicontour Seat Manual Adjustment - Notify Steering Manual Adjustment - Notify AHUD Manual Adjustment - Notify Pedals Manual Adjustment |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Steering_Setting Type: Steering Movement | Sent to: - Receive Positional Status |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Move Steering Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

10 Personalization - Classic Memory Recall Methods 3

"When the function reading the stored setting for a memory controller system obtains the stored setting then it shall output the stored setting to the corresponding move functions

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Adjust AHUD Brightness"

| Requirement ID: 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

13 Personalization - Recall Cancels Automatic Movement 1

"While moving to a saved position (recall) when a new saved position (recall) is received then the following functions shall cease the current movement (recall) and process the new movement (recall)

- 'Move Mirrors Position' function

- 'Move Seat Position' function

- 'Move Steering Position' function

- 'Move Pedals Position' function

- 'Adjust AHUD Brightness' function"

| Requirement ID: 13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_12 Personalization - Recall Cancels Automatic Movement |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

14 Personalization - Manual Move Cancels Automatic Movement 1

"When the following functions determine their respective commodity was manually adjusted then the following functions shall set their output as 'Active'

- 'Notify Mirrors Manual Adjustment' function

- 'Notify Seat Manual Adjustment' function

- 'Notify Multicontour Seat Manual Adjustment' function

- 'Notify Steering Manual Adjustment' function

- 'Notify Pedals Manual Adjustment' function

- 'Notify AHUD Manual Adjustment' function"

| Requirement ID: 14 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_10 Personalization - Manual Move Cancels Automatic Movement |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

15 Personalization - Manual Move Cancels Automatic Movement 2

"While processing a Classic Memory recall when the following functions are notifiied of any manual adjustment as 'Active' then the following functions shall cease the current recall

- 'Move Mirrors Position' function

- 'Move Seat Position' function

- 'Move Steering Position' function

- 'Move Pedals Position' function

- 'Adjust AHUD Brightness' function"

| Requirement ID: 15 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

40 Recall Prevention - Compare Data 1

"When receiving the stored setting the following functions shall compare the stored setting to the current setting

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Move AHUD Brightness"

| Requirement ID: 40 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

41 Recall Prevention - Matching Data 1

"When the function determines from the comparison of the current setting and stored setting that the settings do not match then the following functions shall update the current setting with the stored setting

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Move AHUD Brightness"

| Requirement ID: 41 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

76 Easy Entry Easy Exit - Manual Move - Cancel Automatic Movement 1

While the 'Move Seat Position' function or the 'Move Steering Position' function are in the process of automatic movement triggered by an Easy Entry Easy Exit request when either receive 'Seat_Manual_Adjustment' as 'Active' or 'Steering_Manual_Adjustment' as 'Active' then both functions shall cease the automatic movement

| Requirement ID: 76 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_51 Easy Entry Easy Exit - Manual Move - Cancel Automatic Movement |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

77 Easy Entry Easy Exit - Manual Move - Drive Position 1

When the 'Move Seat Position' function or the 'Move Steering Position' function receive 'Seat_Manual_Adjustment' as 'Active' or 'Steering_Manual_Adjustment' as 'Active' then both functions shall consider the current position the drive position

| Requirement ID: 77 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_52 Easy Entry Easy Exit - Manual Move - Drive Position |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

83 Easy Entry Easy Exit - Steering Column Tilt Offset 1

When the 'Move Steering Position' function receives 'EEEE_Request_Steering' as 'EEEE' then the 'Move Steering Position' function shall remember the current position and move the steering column upward to the maximum setting

| Requirement ID: 83 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_57 Easy Entry Easy Exit - Steering Column Tilt Offset |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

84 Easy Entry Easy Exit - Steering Column Telescope Offset 1

When the 'Move Steering Position' function receives 'EEEE_Request_Steering' as 'EEEE' then the 'Move Steering Position' function shall remember the current position and move the steering column inward to the minimum setting

| Requirement ID: 84 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_58 Easy Entry Easy Exit - Steering Column Telescope Offset |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

85 Easy Entry Easy Exit - Steering Column Telescope Offset 2

While the seat position is in the Easy Entry Easy Exit offset when the 'Move Steering Position' function receives 'EEEE_Request_Steering' as 'Drive' then the 'Move Steering Position' function shall move the steering column to the remembered position

| Requirement ID: 85 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

8 Personalization - Classic Memory Recall Methods 1

"When the 'Request Recall Position' function receives one of the following

'Memory_Input Status' value of:

- 'Recall 1'

- 'Recall 2'

- 'Store 1'

- 'Store 2'

- 'Pers 1' for less than Memory Selection Period

- 'Pers 2' for less than Memory Selection Period

- 'Pers 3' for less than Memory Selection Period

- 'Pers 4' for less than Memory Selection Period

'EM Recall' value of

- 'Request 1'

- 'Request 2'

- 'Request 3'

- 'Request 4'

Then 'Request Recall Position' shall trigger a Classic Memory recall from the memory location indicated in 'Memory_Input_Status'"

| Requirement ID: 8 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

16 Personalization - IGN ON - Memory Set Indicator Trigger 1

"When the 'Provide Memory Input Status' function receives 'User_Input' as 'Set' then the 'Provide Memory Input Status' function shall

- Output 'Memory_Set_Indicator_Status' as 'Active'

- Enter Wait Mode for additional 'User_Input'"

| Requirement ID: 16 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_14 Personalization - IGN ON - Memory Set Indicator Trigger |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

17 Personalization - IGN ON - Memory Set Indicator Trigger 2

When the 'Display HMI' function receives 'Memory_Set_Indicator_Status' as 'Active' then the 'Display HMI' function shall display a prompt to the user with instructions for selecting a memory location until the 'Display HMI' function receives 'Memory_Set_Indicator_Status' as 'Inactive'

| Requirement ID: 17 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

18 Personalization - IGN ON - Memory Set Indicator Trigger 3

When then 'Notify Audio' function receives 'Memory_Set_Indicator_Status' output 'Active' then the 'Notify Audio' function shall notify the user with audible alert

| Requirement ID: 18 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

19 Personalization - IGN ON - Memory Set Indicator Store 1

"While the 'Provide Memory Input Status' function is in Wait Mode for additional 'User_Input' when it receives 'User_Input' as 'Select 1' or 'Select 2' then the 'Provide Memory Input Status' function shall exit Wait Mode and output 'Memory_Input_Status' to

- 'Store 1' if 'User_Input' was 'Select 1'

- 'Store 2' if 'User_Input' was 'Select 2'"

| Requirement ID: 19 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_16 Personalization - IGN ON - Memory Set Indicator Store |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

20 Personalization - IGN ON - Memory Set Indicator Store 2

When the 'Display HMI' function receives 'Memory_Set_Indicator_Status' as 'Inactive' then the 'Display HMI' function shall cease displaying a prompt to the user with instructions for selecting a memory location

| Requirement ID: 20 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

21 Personalization - IGN ON - Memory Set Indicator Store 3

When then 'Notify Audio' function receives 'Memory_Set_Indicator_Status' as 'Store 1' or 'Store 2' then the 'Notify Audio' function shall notify the user with an audible alert

| Requirement ID: 21 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

22 Personalization - Memory Set Indicator Cease Waiting 1

While the 'Provide Memory Input Status' function is in Wait Mode when Set Selection Period elapses then the 'Provide Memory Input Status' function shall exit Waiting Mode

| Requirement ID: 22 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_19 Personalization - Memory Set Indicator Cease Waiting |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

26 Personalization - Memory Set Indicator Second Trigger 1

While the 'Provide Memory Input Status' function is in Wait Mode for additional 'User_Input' when it receives 'User_Input' as 'Set' then the 'Provide Memory Input Status' function shall exit Wait Mode

| Requirement ID: 26 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_20 Personalization - Memory Set Indicator Second Trigger |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

28 Personalization - Memory Set Illuminate 1

While the 'Notify of Memory Set Indicator Status' function receives 'Memory_Set_Indicator_Status' as 'Active' then the 'Notify of Memory Set Indicator Status' function shall illuminate the Set Indicator until the 'Notify of Memory Set Indicator Status' function receives 'Memory_Set_Indicator_Status' as 'Inactive'

| Requirement ID: 28 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_21 Personalization - Memory Set Illuminate |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

29 Personalization – Memory Set Illuminate 2

While the 'Notify of Memory Set Indicator Status' function receives 'Memory_Set_Indicator_Status' as 'Inactive' then the 'Notify of Memory Set Indicator Status' function shall cease illuminating the Set Indicator

| Requirement ID: 29 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

30 Personalization - Memory Switch Input 1

While the 'Provide Memory Input Status' function receives 'User_Input' then the 'Provide Memory Input Status' function shall output 'Memory_Input_Status'

| Requirement ID: 30 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

23 Personalization - Set Selection Period Duration 1

Set Selection Period is a configurable parameter with a default duration of 5 seconds

| Requirement ID: 23 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

32 Personalization - Memory Selection Period

Memory Selection Period is a configurable parameter with a default duration of 1.5 seconds

| Requirement ID: 32 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Move Pedals Position

### Function Overview

#### Description

Function is allocated to:

- Pedals System <<Logical>>

- Vehicle Body System <<Logical>>

(activity) The purpose of this function is to adjust the position of the pedals to a stored value. This is the final step of the Classic Memory recall process. This function receives the stored settings and applies them.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Move Pedals Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Recall Pedals Position”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Move Pedals Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Manual_Adjustment Type: Manual Adjustment - Active/Inactive | Signal Description: These values indicate when a memory controller system has determined that a manual adjustment occurred. Manual adjustments cause automatic movement to cancel. Usage in Function: Signal Description: Indicates that positional system which has a memory controller system made a manual adjustment Received from: - Notify Mirrors Manual Adjustement - Notify Seat Manual Adjustment - Notify Multicontour Seat Manual Adjustment - Notify Steering Manual Adjustment - Notify AHUD Manual Adjustment - Notify Pedals Manual Adjustment |
| Current_Pedals_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Contains the current value of the Pedals Received from: - Read Pedals Current Position |
| Stored_Pedals_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the value that the current position will be updated to Received from: - Read Pedals Stored Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Pedals_Settings Type: Pedals Movement | Sent to: - Receive Positional Status |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Move Pedals Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

10 Personalization - Classic Memory Recall Methods 3

"When the function reading the stored setting for a memory controller system obtains the stored setting then it shall output the stored setting to the corresponding move functions

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Adjust AHUD Brightness"

| Requirement ID: 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

13 Personalization - Recall Cancels Automatic Movement 1

"While moving to a saved position (recall) when a new saved position (recall) is received then the following functions shall cease the current movement (recall) and process the new movement (recall)

- 'Move Mirrors Position' function

- 'Move Seat Position' function

- 'Move Steering Position' function

- 'Move Pedals Position' function

- 'Adjust AHUD Brightness' function"

| Requirement ID: 13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_12 Personalization - Recall Cancels Automatic Movement |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

14 Personalization - Manual Move Cancels Automatic Movement 1

"When the following functions determine their respective commodity was manually adjusted then the following functions shall set their output as 'Active'

- 'Notify Mirrors Manual Adjustment' function

- 'Notify Seat Manual Adjustment' function

- 'Notify Multicontour Seat Manual Adjustment' function

- 'Notify Steering Manual Adjustment' function

- 'Notify Pedals Manual Adjustment' function

- 'Notify AHUD Manual Adjustment' function"

| Requirement ID: 14 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_10 Personalization - Manual Move Cancels Automatic Movement |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

15 Personalization - Manual Move Cancels Automatic Movement 2

"While processing a Classic Memory recall when the following functions are notifiied of any manual adjustment as 'Active' then the following functions shall cease the current recall

- 'Move Mirrors Position' function

- 'Move Seat Position' function

- 'Move Steering Position' function

- 'Move Pedals Position' function

- 'Adjust AHUD Brightness' function"

| Requirement ID: 15 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

40 Recall Prevention - Compare Data 1

"When receiving the stored setting the following functions shall compare the stored setting to the current setting

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Move AHUD Brightness"

| Requirement ID: 40 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

41 Recall Prevention - Matching Data 1

"When the function determines from the comparison of the current setting and stored setting that the settings do not match then the following functions shall update the current setting with the stored setting

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Move AHUD Brightness"

| Requirement ID: 41 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Adjust AHUD Brightness

### Function Overview

#### Description

#Hint: Some descriptive text to explain the purpose and functionality of the function.

Function is allocated to:

- HUD System <<Logical>>

- Infotainment System <<Logical>>

The purpose of this function is to adjust the brightness of the AHUD to a stored value. This is the final step of the Classic Memory recall process. This function receives the stored settings and applies them.

#### Variants

#Classification: Mandatory (State “Not applicable”, if not used)

#Hint: If different variants of the same function are specified in this section, list those variants in the table below.

Variants on Function level could be driven technology or feature content. Example: There could be a “Low Content” and a “High Content” variant of some exterior lighting function. The Low Content variant applies for Conventional Headlight technology, the High Content variant applies for LED and Xenon technology. In this case we call the different technologies the Variant Options (for the time being you could think of them as Logical Parameters) which the Variant depends on. The optional column “Variant condition” allows to express the dependency of a Variant based on Variant Options/Logical Parameters.

If requirements/signals are not applicable for all variants/variant options, those requirements should state explicitly, which function variant/variant option they apply to.

#Link: RE Wiki – Variant Management.

Not supported by MagicDraw report generation.

#### Input Requirements

#Classification: Mandatory (State “Not applicable”, if not used)

#Hint: List any input requirements here (legal, Trustmark), which need to be taken into account, beyond what is specified in the corresponding Feature Documents.

Not supported by MagicDraw report generation.

#### Assumptions

#Classification: Mandatory (State “Not applicable”, if not used)

#Hint: A list of known assumptions concerning the effects of the function’s behavior on other functions or elements (i.e., dependencies) as well as assumptions on the behavior expected by the function (e.g. known limitations). During the course of the development most of those assumptions are typically either converted into actual requirements or discarded at some point – such that this chapter remains mostly empty.

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

#Hint: You may refer to IEEE Citation Reference on how to format a reference.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Adjust AHUD Brightness” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Recall HUD Brightness”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Adjust AHUD Brightness”

### Function Interfaces

#Link: RE Wiki – Adding a Logical Signal or Parameter

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Manual_Adjustment Type: Manual Adjustment - Active/Inactive | Signal Description: These values indicate when a memory controller system has determined that a manual adjustment occurred. Manual adjustments cause automatic movement to cancel. Usage in Function: Signal Description: Indicates that positional system which has a memory controller system made a manual adjustment Received from: - Notify Mirrors Manual Adjustement - Notify Seat Manual Adjustment - Notify Multicontour Seat Manual Adjustment - Notify Steering Manual Adjustment - Notify AHUD Manual Adjustment - Notify Pedals Manual Adjustment |
| Stored_AHUD_Setting Type: Brightness | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the value that the current brightness will be updated to Received from: - Read AHUD Stored Brightness |
| Current_AHUD_Setting Type: Brightness | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Contains the current value of the AHUD Brightness Received from: - Read AHUD Current Brightness |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| AHUD_Settings Type: AHUD Adjustment | Sent to: - Receive Visual Status |

#### Logical Parameters

#Hint: Put requirements for parameters here, which are implemented as configuration parameters using Method 2 or 3 or as parameters for calibration.

Not supported by MagicDraw report generation.

### Function Modeling

#Classification: Mandatory

#Hint: Typical modeling artifacts in this section are State Machines, Activity Diagrams / Flow Charts, Decision Tables, and possibly Sequence Diagrams, which can all be used as techniques to analyze the function requirements.

#Links: Analyze / Model Requirements: RE Wiki – Analyze / Model Requirements

#### Use Cases

#Classification: Infotainment Only (remove section, if not used)

#Hint: Some Domains (e.g. Infotainment) use not only Customer Use Cases (in the Feature Doc), but refine Use Case descriptions down to function level. In general, the RE approach encourages the use of Use Cases on Feature Level but not on Function Level. Activity Diagrams are a more suitable way to express the same on Function Level.

#Links: Infotainment – “Harmony Systems Engineering” Approach

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

#Classification: Mandatory

#Hint: It is highly recommended to use at least one of the following modeling techniques for modeling and analyzing the Function behavior and derived requirements (refer to sample diagrams below):

State Machines, Activity Diagrams / Flow Charts, or Decision Tables

Optionally, Sequence Diagrams might help to analyze the interaction between Functions.

#Links: Analyze / Model Requirements: RE Wiki – Analyze / Model Requirements

State Charts RE Wiki – State Charts

Activity Diagrams: RE Wiki – Activity Diagram, SysML User Group – Activity Diagram Basics

Sequence Diagrams: RE Wiki – Sequence Chart, SysML User Group – Sequence Diagram Basics

Figure 3: Adjust AHUD Brightness

### Function Requirements

#Link: RE Wiki – How to write good requirements

#### Functional Requirements

#Hint: Please also consider specific situations like Initialization (Startup) and Deinitialization (Shutdown) apart from Normal Operation and Error Handling. E.g. a state chart or activity diagram might help for better understanding.

##### Normal Operation

10 Personalization - Classic Memory Recall Methods 3

"When the function reading the stored setting for a memory controller system obtains the stored setting then it shall output the stored setting to the corresponding move functions

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Adjust AHUD Brightness"

| Requirement ID: 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

13 Personalization - Recall Cancels Automatic Movement 1

"While moving to a saved position (recall) when a new saved position (recall) is received then the following functions shall cease the current movement (recall) and process the new movement (recall)

- 'Move Mirrors Position' function

- 'Move Seat Position' function

- 'Move Steering Position' function

- 'Move Pedals Position' function

- 'Adjust AHUD Brightness' function"

| Requirement ID: 13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_12 Personalization - Recall Cancels Automatic Movement |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

14 Personalization - Manual Move Cancels Automatic Movement 1

"When the following functions determine their respective commodity was manually adjusted then the following functions shall set their output as 'Active'

- 'Notify Mirrors Manual Adjustment' function

- 'Notify Seat Manual Adjustment' function

- 'Notify Multicontour Seat Manual Adjustment' function

- 'Notify Steering Manual Adjustment' function

- 'Notify Pedals Manual Adjustment' function

- 'Notify AHUD Manual Adjustment' function"

| Requirement ID: 14 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_10 Personalization - Manual Move Cancels Automatic Movement |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

15 Personalization - Manual Move Cancels Automatic Movement 2

"While processing a Classic Memory recall when the following functions are notifiied of any manual adjustment as 'Active' then the following functions shall cease the current recall

- 'Move Mirrors Position' function

- 'Move Seat Position' function

- 'Move Steering Position' function

- 'Move Pedals Position' function

- 'Adjust AHUD Brightness' function"

| Requirement ID: 15 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

40 Recall Prevention - Compare Data 1

"When receiving the stored setting the following functions shall compare the stored setting to the current setting

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Move AHUD Brightness"

| Requirement ID: 40 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

41 Recall Prevention - Matching Data 1

"When the function determines from the comparison of the current setting and stored setting that the settings do not match then the following functions shall update the current setting with the stored setting

- Move Mirrors Position

- Move Seat Position

- Move Steering Position

- Move Pedals Position

- Move AHUD Brightness"

| Requirement ID: 41 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

#Hint: FMEA counter measures could be considered as requirements in this chapter

No Error Handling Requirements specified.

#### Non-Functional Requirements

#Hint: Non-functional requirements specify some performance criteria in addition to the functional behavior given defined by the functional requirements. Timing (if not already included in the functional requirements), security details (e.g. how secure does an algorithm have to be) or reliability (e.g. mean time between failure) could be specified in this section.

No Non-Functional Requirements specified.

#### Functional Safety Requirements

#Classification: Functional Safety only – If not used, remove content and state “Not Applicable”

#Hint: Add Functional Safety Requirements (FSRs) derived for this function.

#Link:	Functional Safety Sharepoint – Functional Safety Concept

RE Wiki - Requirements Attributes

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

#Classification: Functional Safety only – If not used, remove content and state “Not Applicable”

#Hint:  Sometimes an ASIL decomposition of Functional Safety Requirements is required. The decomposed FSRs should be listed beneath each ASIL Decomposition table below and referenced inside the table by ID and Title

#Link:	Functional Safety Sharepoint – Functional Safety Concept

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

#Hint: Requirements of a Logical Function should be typically agnostic of their SW/HW implementation. If for specific reasons the function owner needs to define explicitly design constraints, it can be done in this chapter.

No Design Requirements specified.

## Request Store Position

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

The purpose of this function is to determine that the result of the user input is a store action. From this determination this function will then trigger a Classic Memory store. This store will retrieve current settings and store them to the user select profile.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Store Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Store Positional Settings”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Request Store Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Auto_Save_Store Type: Auto Save Store - Pers/Inactive | Signal Description: These values indicate when the Auto Save system has determined a Classic Memory store should occur. The value indicates which memory location the current settings should be stored to. Usage in Function: Signal Description: Indicates that Auto Save is requesting Classic Memory to update the active memory location (store) Received from: - Auto Save Store |
| Memory_Input_Status Type: Input Status - Recall/Set/Pers/None | Signal Description: These values indicate how the input from the user was perceived by the Classic Memory system. When Memory Set is selected then the input function will wait until a profile is selected before transmitting the information. Otherwise the input function relays the input from the user to the rest of the Classic Memory system. Usage in Function: Contains the value of the Memory Input. Indicates if a memory location was selected. Additionally may indicated if a recall or store was selected. Received from: - Provide Memory Input Status |
| Feature_Configuration Type: Feature Configuration - Classic/Advanced | Signal Description: These values indicate whether the standard or variant configuration should be executed. The Advanced Classic Memory variant provides remote transmitter functionality and modified prompts. Usage in Function: Indicates the configuration of Classic Memory Received from: - Provide Feature Configuration |
| EM_Store Type: Profile Association: Select Location - Associate/Inactive | Signal Description: These values indicate when the Enhanced Memory system is creating a new profile. The Classic Memory system waits for the user to select (associate) a memory location. Usage in Function: Signal Description: Enhanced Memory is indicating that Classic Memory should perform a special store Received from: - Create New Profile |
| Restriction_Settings_Status Type: Restriction Settings - Factory/Vehicle/Moveable/Non-Moveable | Signal Description: These values indicate which restrictions are applied to profiles. Usage in Function: Contains the value for the Restriction Status. The Restriction Statuses limit profiles, stores, and recalls. |
| New_Profile_Store Type: New Profile Store - Associate 1-4/Inactive | Received from: - EM_Store |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Memory_Command Type: Memory Command - Store/Recall | Signal Description: These values indicate whether memory controller systems should perform a Classic Memory store or Classic Memory recall. The value indicates the memory location to be used in the operation. Usage in Function: Signal Description: Contains the value of the memory location to be affected. Indicates if the memory location should be recalled or updated (store). Sent to: - Display HMI - Store Multicontour Seat - Read Mirrors Current Position - Read Pedals Current Position - Read AHUD Current Brightness - Read Seat Current Position - Read Steering Current Position |
| Store_Status Type: Store Status - Active/Inactive | Sent to: - Associate Remote Transmitter |
| ACM_Prompt_Status | Sent to: - Display HMI |
| Chime_Status Type: Set Indicator Status - Active/Inactive | Signal Description: These values indicate when Memory Set is active. When active the indicator will notify the user for the duration that Memory Set is active. Sent to: - Notify Audio |
| CM_Association Type: Profile Association - Pers/Inactive | Signal Description: These values indicate which memory location the user selected to use for their new profile. Usage in Function: Indicates that Classic Memory is doing a store Sent to: - EM_Store |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: EM Store

Figure 3: Request Store Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

1 RKE Automatic Trigger 1

"While

- 'Provide Feature Configuration' is 'Advanced Classic Memory'

- There is no remote transmitter associated to the memory location being stored

When the 'Request Store Position' function triggers a Classic Memory store then the 'Request Store Position' function shall

- Output 'Memory_Command' as 'Store' to the memory location indicated in 'Memory_Input_Status'

- Output 'ACM Prompt_Status' as 'Store' + the number of the profile stored too + 'RKE Help'

- Output 'Chime_Status' as 'Chime'

- Enter Wait Mode for a valid remote transmitter signal"

| Requirement ID: 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_1 RKE Automatic Trigger |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

2 RKE Manual  Trigger 1

"While

- 'Provide Feature Configuration' is 'Advanced Classic Memory'

- There is at least one remote transmitter associated to the memory location being stored

When the 'Request Store Position' function triggers a Classic Memory store then the 'Request Store Position' function shall

- Output 'Memory_Command' as 'Store' to the memory location indicated in 'Memory_Input_Status'

- Output 'ACM Prompt_Status' as 'Store' + the number of the profile stored too

- Enter Wait Mode for a valid remote transmitter signal"

| Requirement ID: 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_2 RKE Manual Trigger |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

5 RKE Incomplete - Cease Waiting 1

"While the 'Request Store Position' function is in Wait Mode for a valid remote transmitter signal when Transmitter Wait Period elapses then the 'Request Store Position' function shall

- Exit Wait Mode"

| Requirement ID: 5 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_6 RKE Incomplete |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

31 Personalization - Manual Memory Trigger 1

When the 'Request Store Position' function receives 'Memory_Input_Status' as 'Pers 1', 'Pers 2', 'Pers 2', or 'Pers 4' for Memory Selection Period then the 'Request Store Position' function shall trigger a Classic Memory store

| Requirement ID: 31 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

33 Personalization - Memory Store 1

When the 'Request Store Position' function triggers a Classic Memory store then it shall output 'Memory_Command' as 'Store' + the memory location indicated in 'Memory_Input_Status'

| Requirement ID: 33 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_22 Personalization - Memory Store |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

35 Personalization - Memory Store 3

"When the following functions receive 'Memory_Command' as 'Store' then the following functions shall read the current settings of any available positional memory axis

- Read Mirrors Current Position

- Read Seat Current Position

- Read Steering Current Position

- Read Pedals Current Position

- Read AHUD Current Brightness"

| Requirement ID: 35 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

36 Personalization - Memory Store 4

"When the following functions complete reading the current settings then the following functions shall output the current settings and 'Profile' as the memory location indicated in 'Memory_Command'

- Read Seat Current Position

- Read Steering Current Position

- Read Pedals Current Position

- Read AHUD Current Brightness"

| Requirement ID: 36 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

52 Driver Profile Creation - Enter Button Association Mode

When the 'Request Store Position' function receives 'Button Association Mode' as 'Associate' then the 'Request Store Position' function shall enter Wait Mode for association selection

| Requirement ID: 52 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

53 Driver Profile Creation - Exit Button Association Mode

When the 'Request Store Position' function receives 'Button Association Mode' as 'Inactive' then the 'Request Store Position' function shall enter Wait Mode for association selection

| Requirement ID: 53 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

54 Driver Profile Creation - Cease Waiting 1

While the 'Request Store Position' function is in Wait Mode for association selection and Association Selection Period elapses then the 'Request Store Position' function shall exit Waiting Mode

| Requirement ID: 54 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_28 Driver Profile Creation - Cease Waiting |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

56 Driver Profile Creation - Store 1

While the 'Request Store Position' function is in Wait Mode for association selection when 'Memory_Input_Status' as 'Pers 1', 'Pers 2', 'Pers 3', or 'Pers 4' is received then the 'Request Store Position' function shall output 'Classic Memory Association' as the memory location indicated in 'Memory_Input_Status'

| Requirement ID: 56 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_27 Driver Profile Creation - Store |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

57 Auto Save Feature Store 1

When the 'Request Store Position' function receives 'Auto_Save_Store' as 'Store' then the 'Request Store Position' function shall trigger a Classic Memory store for the current profile

| Requirement ID: 57 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_30 Auto Save Feature Store |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

59 Classic Memory Restriction Settings - Factory Setting 1

When the 'Request Store Position' function receives 'Restriction Setting Status' as 'Factory' then the 'Request Store Position' function shall set all memory locations to Default Profile

| Requirement ID: 59 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_34 Classic Memory Restriction Settings - Factory Setting |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

60 Classic Memory Restriction Settings - Factory Setting 2

While a profile is a Default Profile the 'Request Store Position' function shall inhibit the trigger of a Classic Memory store for that profile

| Requirement ID: 60 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

61 Classic Memory Restriction Settings - Vehicle Setting 1

When the 'Request Store Position' function receives 'Restriction Setting Status' as 'Vehicle' then the 'Request Store Position' function shall set all memory locations to Vehicle Profile

| Requirement ID: 61 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_35 Classic Memory Restriction Settings - Vehicle Setting |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

62 Classic Memory Restriction Settings - Vehicle Setting 2

While a profile is a Vehicle Profile when a Classic Memory store triggers then the 'Request Store Position' function shall trigger a Classic Memory store for all Vehicle Profiles

| Requirement ID: 62 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

63 Classic Memory Restriction Settings - Moveable Person Setting 1

When the 'Request Store Position' function receives 'Restriction Setting Status' as 'Moveable_Person' then the 'Request Store Position' function shall set all memory locations to Moveable_Person Profile

| Requirement ID: 63 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_36 Classic Memory Restriction Settings - Moveable Person Setting |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

64 Classic Memory Restriction Settings - Non-Moveable Person Setting 1

When the 'Request Store Position' function receives 'Restriction Setting Status' as 'Non_Moveable_Person' then the 'Request Store Position' function shall set all memory locations to Non_Moveable_Person Profile

| Requirement ID: 64 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_37 Classic Memory Restriction Settings - Non-Moveable Person Setting |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

65 Classic Memory Restriction Settings - Non-Moveable Person Setting 2

While a profile is a Non_Moveable_Person Profile the 'Request Store Position' function shall inhibit the the transfer of setting from that profile to an external storage location and vice versa

| Requirement ID: 65 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

6 Transmitter Wait Period 1

Transmitter Wait Period is a configurable parameter with a default time of 10 seconds

| Requirement ID: 6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

32 Personalization - Memory Selection Period

Memory Selection Period is a configurable parameter with a default duration of 1.5 seconds

| Requirement ID: 32 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

55 Driver Profile Creation - Associate Selection Time 1

Association Selection Period is a configurable parameter with a default duration of 70 seconds

| Requirement ID: 55 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

#### Uncategorized Requirements

58 Classic Memory Restriction Settings - Settings 1

"The valid Restriction Settings for the Classic Memory feature are:

- Factory

- Vehicle

- Moveable_Person

- Non_Moveable_Person"

| Requirement ID: 58 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_33 Classic Memory Restriction Settings - Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Read Mirrors Current Position

### Function Overview

#### Description

Function is allocated to:

- Vehicle Body System <<Logical>>

- Vehicle Door System <<Logical>>

The purpose of this function is to obtain the current value of the mirror positions. This function also identifies a profile that the settings should be stored to.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Read Mirrors Current Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Current Side Mirrors Position”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Read Mirrors Current Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Memory_Command Type: Memory Command - Store/Recall | Signal Description: These values indicate whether memory controller systems should perform a Classic Memory store or Classic Memory recall. The value indicates the memory location to be used in the operation. Usage in Function: Signal Description: Indicates to the function that is should read the current values of the Mirrors Position Received from: - Request Store Position |
| Actual_Mirrors_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Contains the actual value of the side mirrors position Received from: - Provide Actual Side Mirrors Settings |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Profile Type: Profiles - Memory Location 1-4 | Signal Description: These values indicate which memory location to use for the accompanying operation being execute. Usage in Function: Signal Description: Contains the memory location to be used in the settings update Sent to: - Update Profile with Positional Settings - Store Mirrors Position |
| Current_Mirrors_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Contains the current value of the Mirror Positions Sent to: - Update Profile with Positional Settings - Store Mirrors Position - Move Mirrors Position |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Read Mirrors Current Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

34 Personalization - Memory Store 2

When the 'Display HMI' function receives 'Memory Command' as 'Store + the memory location' then the 'Display HMI' function shall display a Classic Memory store prompt to the user indicating which profile successfully stored

| Requirement ID: 34 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

39 Personalization - Memory Store and Personal Portable Profile

"When the following functions receive 'Memory_Command' as 'Store' and the current position then the following functions shall output the profile and position to be stored to the 'Update Profile with Positional Settings' function

- Read Mirrors Current Position

- Read Seat Current Position

- Read Steering Current Position

- Read Pedals Current Position

- Read AHUD Current Brightness"

| Requirement ID: 39 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_22 Personalization - Memory Store |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Read Seat Current Position

### Function Overview

#### Description

Function is allocated to:

- Seat System <<Logical>>

- Vehicle Body System <<Logical>>

The purpose of this function is to obtain the current value of the seat positions. This function also identifies a profile that settings should be stored to.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Read Seat Current Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Current Seat Position”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Read Seat Current Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Actual_Seat_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Contains the actual value of the driver seat position Received from: - Provide Actual Seat Settings |
| Memory_Command Type: Memory Command - Store/Recall | Signal Description: These values indicate whether memory controller systems should perform a Classic Memory store or Classic Memory recall. The value indicates the memory location to be used in the operation. Usage in Function: Signal Description: Indicates to the function that is should read the current values of the Seat Received from: - Request Store Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Profile Type: Profiles - Memory Location 1-4 | Signal Description: These values indicate which memory location to use for the accompanying operation being execute. Usage in Function: Signal Description: Contains the memory location to be used in the settings update Sent to: - Update Profile with Positional Settings - Store Seat Position |
| Current_Seat_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the current values of the Seat Sent to: - Update Profile with Positional Settings - Move Seat Position - Store Seat Position |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Read Seat Current Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

34 Personalization - Memory Store 2

When the 'Display HMI' function receives 'Memory Command' as 'Store + the memory location' then the 'Display HMI' function shall display a Classic Memory store prompt to the user indicating which profile successfully stored

| Requirement ID: 34 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

39 Personalization - Memory Store and Personal Portable Profile

"When the following functions receive 'Memory_Command' as 'Store' and the current position then the following functions shall output the profile and position to be stored to the 'Update Profile with Positional Settings' function

- Read Mirrors Current Position

- Read Seat Current Position

- Read Steering Current Position

- Read Pedals Current Position

- Read AHUD Current Brightness"

| Requirement ID: 39 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_22 Personalization - Memory Store |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Read Steering Current Position

### Function Overview

#### Description

Function is allocated to:

- Steering Wheel System <<Logical>>

- Vehicle Body System <<Logical>>

The purpose of this function is to obtain the current value of the steering positions. This function also identifies a profile that settings should be stored to.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Read Steering Current Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Current Steering Column Position”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Read Steering Current Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Actual_Steering_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Contains the actual value of the Steering Column position Received from: - Provide Actual Steering Settings |
| Memory_Command Type: Memory Command - Store/Recall | Signal Description: These values indicate whether memory controller systems should perform a Classic Memory store or Classic Memory recall. The value indicates the memory location to be used in the operation. Usage in Function: Signal Description: Indicates to the function that is should read the current values of the Steering wheel Received from: - Request Store Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Current_Steering_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the current values of the Steering wheel Sent to: - Move Steering Position - Update Profile with Positional Settings - Store Steering Position |
| Profile Type: Profiles - Memory Location 1-4 | Signal Description: These values indicate which memory location to use for the accompanying operation being execute. Usage in Function: Signal Description: Contains the memory location to be used in the settings update Sent to: - Update Profile with Positional Settings - Store Steering Position |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Read Steering Current Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

34 Personalization - Memory Store 2

When the 'Display HMI' function receives 'Memory Command' as 'Store + the memory location' then the 'Display HMI' function shall display a Classic Memory store prompt to the user indicating which profile successfully stored

| Requirement ID: 34 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

39 Personalization - Memory Store and Personal Portable Profile

"When the following functions receive 'Memory_Command' as 'Store' and the current position then the following functions shall output the profile and position to be stored to the 'Update Profile with Positional Settings' function

- Read Mirrors Current Position

- Read Seat Current Position

- Read Steering Current Position

- Read Pedals Current Position

- Read AHUD Current Brightness"

| Requirement ID: 39 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_22 Personalization - Memory Store |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Read Pedals Current Position

### Function Overview

#### Description

Function is allocated to:

- Pedals System <<Logical>>

- Vehicle Body System <<Logical>>

The purpose of this function is to obtain the current value of the pedal positions. This function also identifies a profile that settings should be stored to.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Read Pedals Current Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Current Power Pedals Position”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Read Pedals Current Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Memory_Command Type: Memory Command - Store/Recall | Signal Description: These values indicate whether memory controller systems should perform a Classic Memory store or Classic Memory recall. The value indicates the memory location to be used in the operation. Usage in Function: Signal Description: Indicates to the function that is should read the current values of the Pedals Received from: - Request Store Position |
| Actual_Pedals_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Contains the actual value of the Pedals position Received from: - Provide Actual Pedals Settings |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Current_Pedals_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the current value of the Pedals Sent to: - Store Pedals Position - Update Profile with Positional Settings - Move Pedals Position |
| Profile Type: Profiles - Memory Location 1-4 | Signal Description: These values indicate which memory location to use for the accompanying operation being execute. Usage in Function: Signal Description: Contains the memory location to be used in the settings update Sent to: - Store Pedals Position - Update Profile with Positional Settings |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Read Pedals Current Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

34 Personalization - Memory Store 2

When the 'Display HMI' function receives 'Memory Command' as 'Store + the memory location' then the 'Display HMI' function shall display a Classic Memory store prompt to the user indicating which profile successfully stored

| Requirement ID: 34 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

39 Personalization - Memory Store and Personal Portable Profile

"When the following functions receive 'Memory_Command' as 'Store' and the current position then the following functions shall output the profile and position to be stored to the 'Update Profile with Positional Settings' function

- Read Mirrors Current Position

- Read Seat Current Position

- Read Steering Current Position

- Read Pedals Current Position

- Read AHUD Current Brightness"

| Requirement ID: 39 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_22 Personalization - Memory Store |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Read AHUD Current Brightness

### Function Overview

#### Description

Function is allocated to:

- HUD System <<Logical>>

- Infotainment System <<Logical>>

The purpose of this function is to obtain the current value of the AHUD brightness. This function also identifies a profile that the settings should be stored to.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Read AHUD Current Brightness” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Current HUD Brightness”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Read AHUD Current Brightness”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Memory_Command Type: Memory Command - Store/Recall | Signal Description: These values indicate whether memory controller systems should perform a Classic Memory store or Classic Memory recall. The value indicates the memory location to be used in the operation. Usage in Function: Signal Description: Indicates to the function that is should read the current values of the AHUD Brightness Received from: - Request Store Position |
| Actual_AHUD_Setting Type: Brightness | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Contains the actual value of brightness of the AHUD Received from: - Provide Actual AHUD Settings |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Profile Type: Profiles - Memory Location 1-4 | Signal Description: These values indicate which memory location to use for the accompanying operation being execute. Usage in Function: Signal Description: Contains the memory location to be used in the settings update Sent to: - Store AHUD Brightness - Update Profile with Positional Settings |
| Current_AHUD_Setting Type: Brightness | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the current value of the AHUD Brightness Sent to: - Store AHUD Brightness - Update Profile with Positional Settings - Adjust AHUD Brightness |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Read AHUD Current Brightness

### Function Requirements

#### Functional Requirements

##### Normal Operation

34 Personalization - Memory Store 2

When the 'Display HMI' function receives 'Memory Command' as 'Store + the memory location' then the 'Display HMI' function shall display a Classic Memory store prompt to the user indicating which profile successfully stored

| Requirement ID: 34 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

39 Personalization - Memory Store and Personal Portable Profile

"When the following functions receive 'Memory_Command' as 'Store' and the current position then the following functions shall output the profile and position to be stored to the 'Update Profile with Positional Settings' function

- Read Mirrors Current Position

- Read Seat Current Position

- Read Steering Current Position

- Read Pedals Current Position

- Read AHUD Current Brightness"

| Requirement ID: 39 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_22 Personalization - Memory Store |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Store Mirrors Position

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

The purpose of this function is to store the position of the mirrors to a profile. This is the final step of the Classic Memory store process. This function receives the current settings and applies them to the user selected profile.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Store Mirrors Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Store Positional Settings”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Store Mirrors Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Stored_Mirrors_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Received from: - Read Mirrors Stored Position |
| Current_Mirrors_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the current values of the Mirror Positions Received from: - Read Mirrors Current Position |
| Profile Type: Profiles - Memory Location 1-4 | Signal Description: These values indicate which memory location to use for the accompanying operation being execute. Usage in Function: Signal Description: Contains the memory location to be used in the settings update Received from: - Read Mirrors Current Position |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Store Mirrors Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

#### Uncategorized Requirements

94 Personalization - Memory Location Settings 1

Classic Memory shall provide only one set of positional settings per memory location per memory commodity

| Requirement ID: 94 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_64 Personalization - Memory Location Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

96 Memory Seat - Memory Mirrors

Where a vehicle has Driver Seat memory then that vehicle shall also have the Memory Mirrors (Power Exterior Side Mirrors)

| Requirement ID: 96 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

97 Memory Mirrors - Settings 1

"Where Power Exterior Side Mirrors is supported, then the Classic Memory feature shall include the following positional settings for:

- Memory Driver Side Mirror

- Memory Passenger Side Mirror"

| Requirement ID: 97 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_68 Memory Mirrors - Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

98 Memory Seat - Memory Locations 1

Classic Memory shall provide up to 4 memory locations per user

| Requirement ID: 98 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_65 Memory Seat - Memory Locations |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Store Seat Position

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

The purpose of this function is to store the position of the seats to a profile. This is the final step of the Classic Memory store process. This function receives the current settings and applies them to the user selected profile.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Store Seat Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Store Positional Settings”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Store Seat Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Current_Seat_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the current values of the Seat Positions Received from: - Read Seat Current Position |
| Stored_Seat_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Received from: - Read Seat Stored Position |
| Profile Type: Profiles - Memory Location 1-4 | Signal Description: These values indicate which memory location to use for the accompanying operation being execute. Usage in Function: Signal Description: Contains the memory location to be used in the settings update Received from: - Read Seat Current Position |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Store Seat Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

#### Uncategorized Requirements

94 Personalization - Memory Location Settings 1

Classic Memory shall provide only one set of positional settings per memory location per memory commodity

| Requirement ID: 94 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_64 Personalization - Memory Location Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

96 Memory Seat - Memory Mirrors

Where a vehicle has Driver Seat memory then that vehicle shall also have the Memory Mirrors (Power Exterior Side Mirrors)

| Requirement ID: 96 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

98 Memory Seat - Memory Locations 1

Classic Memory shall provide up to 4 memory locations per user

| Requirement ID: 98 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_65 Memory Seat - Memory Locations |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

99 Memory Seat - Driver Settings 1

Where the Power Driver Seat is supported then the Classic Memory feature shall include positional settings for Memory Driver Seat

| Requirement ID: 99 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_69 Memory Seat - Driver Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

100 Memory Seat - Passenger Settings 1

Where the Power Passenger Seat is supported then the Classic Memory feature shall allow positional settings for Memory Passenger Seat

| Requirement ID: 100 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_70 Memory Seat - Passenger Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Store Steering Position

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

The purpose of this function is to store the position of the steering to a profile. This is the final step of the Classic Memory store process. This function receives the current settings and applies them to the user selected profile.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Store Steering Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Store Positional Settings”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Store Steering Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Current_Steering_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the current values of the Steering Positions Received from: - Read Steering Current Position |
| Stored_Steering_Setting Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Received from: - Read Steering Stored Position |
| Profile Type: Profiles - Memory Location 1-4 | Signal Description: These values indicate which memory location to use for the accompanying operation being execute. Usage in Function: Signal Description: Contains the memory location to be used in the settings update Received from: - Read Steering Current Position |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Store Steering Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

#### Uncategorized Requirements

94 Personalization - Memory Location Settings 1

Classic Memory shall provide only one set of positional settings per memory location per memory commodity

| Requirement ID: 94 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_64 Personalization - Memory Location Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

98 Memory Seat - Memory Locations 1

Classic Memory shall provide up to 4 memory locations per user

| Requirement ID: 98 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_65 Memory Seat - Memory Locations |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

101 Memory Steering Column - Settings 1

Where Power Tilt/Telescoping Memory column is supported, then the Classic Memory feature shall include positional settings for the Memory Steering Column

| Requirement ID: 101 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_71 Memory Steering Column - Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Store Pedals Position

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

The purpose of this function is to store the position of the pedals to a profile. This is the final step of the Classic Memory store process. This function receives the current settings and applies them to the user selected profile.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Store Pedals Position” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Store Positional Settings”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Store Pedals Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Stored_Pedals_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Received from: - Read Pedals Stored Position |
| Current_Pedals_Settings Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the current values of the Pedal Positions Received from: - Read Pedals Current Position |
| Profile Type: Profiles - Memory Location 1-4 | Signal Description: These values indicate which memory location to use for the accompanying operation being execute. Usage in Function: Signal Description: Contains the memory location to be used in the settings update Received from: - Read Pedals Current Position |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Store Pedals Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

#### Uncategorized Requirements

94 Personalization - Memory Location Settings 1

Classic Memory shall provide only one set of positional settings per memory location per memory commodity

| Requirement ID: 94 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_64 Personalization - Memory Location Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

95 Memory Pedals - Settings 1

"Where Power Adjustable Pedals is supported, then the Classic Memory feature shall include the following positional settings:

- Memory Gas Pedal

- Memory Brake Pedal

- Memory Clutch Pedal (on vehicles with manual transmission only)"

| Requirement ID: 95 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_66 Memory Pedals - Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

98 Memory Seat - Memory Locations 1

Classic Memory shall provide up to 4 memory locations per user

| Requirement ID: 98 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_65 Memory Seat - Memory Locations |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Store AHUD Brightness

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

The purpose of this function is to store the brightness of the AHUD to a profile. This is the final step of the Classic Memory store process. This function receives the current settings and applies them to the user selected profile.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Store AHUD Brightness” function is called by the following functions:

- – “Classic Memory Functional Architecture”

- – “Store Positional Settings”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Store AHUD Brightness”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Stored_AHUD_Setting Type: Brightness | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Received from: - Read AHUD Stored Brightness |
| Profile Type: Profiles - Memory Location 1-4 | Signal Description: These values indicate which memory location to use for the accompanying operation being execute. Usage in Function: Signal Description: Contains the memory location to be used in the settings update Received from: - Read AHUD Current Brightness |
| Current_AHUD_Setting Type: Brightness | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the current values of the AHUD Brightness Received from: - Read AHUD Current Brightness |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Store AHUD Brightness

### Function Requirements

#### Functional Requirements

##### Normal Operation

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

#### Uncategorized Requirements

94 Personalization - Memory Location Settings 1

Classic Memory shall provide only one set of positional settings per memory location per memory commodity

| Requirement ID: 94 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_64 Personalization - Memory Location Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

98 Memory Seat - Memory Locations 1

Classic Memory shall provide up to 4 memory locations per user

| Requirement ID: 98 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_65 Memory Seat - Memory Locations |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

102 Memory HUD - Settings 1

Where the HUD feature is supported, then the Classic Memory feature shall include positional settings for the Advanced Heads Up Display (AHUD)

| Requirement ID: 102 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_72 Memory HUD - Settings |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Detect Remote Transmitter

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

activity) The purpose of this function is to detect when a remote transmitter sends out a Lock or Unlock signal. This function then indicates what signal was received and by which remote transmitter. Other functions will use this data to determine if a recall should occur or if a profile should add/remove a remote transmitter.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Detect Remote Transmitter” function is called by the following functions:

- – “Arbitrate Remote Transmitters”

- – “Classic Memory Functional Architecture”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Detect Remote Transmitter”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Lock/Unlock Type: Lock/Unlock | Received from: - Lock/Unlock via Remote Transmitter |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Remote_Transmitter_Id Type: Integer | Sent to: - Request Recall Position - Associate Remote Transmitter |
| Remote_Transmitter_Status Type: Remote Transmitter Status - Lock/Unlock Pers 1-4 | Sent to: - Request Recall Position - Associate Remote Transmitter |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Detect Remote Transmitter

### Function Requirements

#### Functional Requirements

##### Normal Operation

3 RKE Association 1

"While the 'Request Store Position' function is in Wait Mode for a valid remote transmitter signal when the 'Request Store Position' function receives 'RKE Lock Status' as 'Lock' then the 'Request Store Position' function shall

- Associate the remote transmitter that sent the 'Lock' signal with the current memory location which was stored too

- Exit Wait Mode"

| Requirement ID: 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_4 RKE Association |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

4 RKE Dissociation 1

"While the 'Request Store Position' function is in Wait Mode for a valid remote transmitter signal when the 'Request Store Position' function receives 'RKE Lock Status' as 'Unlock' then the 'Request Store Position' function shall

- Disassociate the remote transmitter that sent the 'Unlock' signal with the current memory location which was stored too

- Exit Wait Mode"

| Requirement ID: 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_5 RKE Dissociation |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Associate Remote Transmitter

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Classic Memory Settings System <<Logical>>

(activity) The purpose of this function is to add or remove a remote transmitter from a profile.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Associate Remote Transmitter” function is called by the following functions:

- – “Arbitrate Remote Transmitters”

- – “Classic Memory Functional Architecture”

Description of “Classic Memory Functional Architecture” diagram:

This diagram contains all the functions required to Store and Recall the positional settings. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Classic Memory Functional Architecture” calling   “Associate Remote Transmitter”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Remote_Transmitter_ID Type: Integer | Received from: - Detect Remote Transmitter |
| Remote_Transmitter_Status Type: Remote Transmitter Status - Lock/Unlock Pers 1-4 | Received from: - Detect Remote Transmitter |
| Store_Status Type: Store Status - Active/Inactive | Received from: - Request Store Position |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Associate Remote Transmitter

### Function Requirements

#### Functional Requirements

##### Normal Operation

3 RKE Association 1

"While the 'Request Store Position' function is in Wait Mode for a valid remote transmitter signal when the 'Request Store Position' function receives 'RKE Lock Status' as 'Lock' then the 'Request Store Position' function shall

- Associate the remote transmitter that sent the 'Lock' signal with the current memory location which was stored too

- Exit Wait Mode"

| Requirement ID: 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_4 RKE Association |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

4 RKE Dissociation 1

"While the 'Request Store Position' function is in Wait Mode for a valid remote transmitter signal when the 'Request Store Position' function receives 'RKE Lock Status' as 'Unlock' then the 'Request Store Position' function shall

- Disassociate the remote transmitter that sent the 'Unlock' signal with the current memory location which was stored too

- Exit Wait Mode"

| Requirement ID: 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_5 RKE Dissociation |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

6 Transmitter Wait Period 1

Transmitter Wait Period is a configurable parameter with a default time of 10 seconds

| Requirement ID: 6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

.

## Request EEEE

### Function Overview

#### Description

Function is allocated to:

- Classic Memory Logical <<Logical>>

- Easy Entry Easy Exit System <<Logical>>

The purpose of this function is to determine when the user is entering and exiting the vehicle. From that determination this function will then adjust the seat and steering positions.

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request EEEE” function is called by the following functions:

- – “Easy Entry Easy Exit Functional Architecture”

- – “Recall EEEE Position”

Description of “Easy Entry Easy Exit Functional Architecture” diagram:

This diagram contains all the functions required to adjust seats and steering to provide additional space for the user to enter and exit. This includes inputs and outputs to the user, other features, and external systems.

Figure 2: Activity Diagram of   “Easy Entry Easy Exit Functional Architecture” calling   “Request EEEE”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Park Status Type: Park Status Park/NOT Park | Signal Description: These values indicate whether the State Management System has determined that the vehicle is in a safe state to trigger a Classic Memory recall. When the vehicle is in Park then a Classic Memory recall may trigger. Usage in Function: Signal Description: Indicates if the vehicle is in PARK Received from: - Provide Park Status |
| Seatblet Status Type: Seatbelt Status - Engaged/Disengaged | Signal Description: These values indicate whether the Seatbelt System has determined that the driver seatbelt is engaged or disengaged. Usage in Function: Signal Description: Indicates if the seatbelt is Engaged Received from: - Provide Seatbelt Status |
| EEEE_Profile_Status Type: EEEE Profile Status - Enabled/Disabled | Signal Description: These values indicate whether the user has selected to enable or disable the Easy Entry Easy Exit system. Usage in Function: Signal Description: Indicates if the active profile has EEEE set to ON Received from: - Provide EEEE Profile Status |
| Memory_Command Type: EM Recall - Request/Inactive | Signal Description: These values indicate that an Enhanced Memory recall has occurred and for which memory location. This is to indicate to the Classic Memory system that it should perform a Classic Memory recall for the same memory location. Usage in Function: Contains the value of the memory location to be affected. Indicates if the memory location should be recalled or updated (store). Received from: - Request Recall Position |
| Stowable_Steering_Wheel_Status Type: Stowable Steering Wheel - Active/Inactive | Signal Description: These values indicate when the Stowable Steering Wheel system is active and thus the Easy Entry Easy Exit system should be inhibited. Usage in Function: Signal Description: Indicates if Stowable Steering Wheel is currently active Received from: - Provide Stowable Steering Wheel Status |
| Rejuvenate_Status Type: Rejuvenate Status - Active/Inactive | Signal Description: These values indicate when the Rejuvenate system is active and thus the Easy Entry Easy Exit system should be inhibited. Usage in Function: Signal Description: Indicates if Rejuvenate is currently active Received from: - Provide Rejuvenate Status |
| EEEE_Configuration Type: Strategy Configuration - DCO/Standard | Signal Description: These values indicate whether Drive Control Optimization (DCO) is configured. DCO removes the key slot and PEPS from a vehicle thus making Easy Entry Easy Exit rely on different triggers. Usage in Function: Signal Description: Contains the configuration implemented on the vehicle. From this configuration the function can determine if DCO strategy is implemented. Received from: - Provide EEEE Configuration |
| Door_Status Type: Driver Door Status - Ajar/Closed | Signal Description: These values indicate whether the Door System has determined that the driver door is ajar (open) or closed. Usage in Function: Signal Description: Indicates the status of the vehicle door Received from: - Provide Door Status |
| Key_In_Ignition_Status Type: Key In Igniton - In/Out | Signal Description: These values indicate whether the Key System has determined that the Key (or PEPS) in IN (ON) or OUT (OFF). Usage in Function: Signal Description: Indicates is the key is IN or OUT Received from: - Provide Key Status |
| Detection_Status Type: Remote Transmitter Status - Inside/Other | Signal Description: These values indicate where the Key System has determined the Key is detected. Usage in Function: Signal Description: Indicates the location of a Remote Transmitter Received from: - Detect RKE |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| EEEE_Request_Steering Type: Position | Signal Description: These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored. Usage in Function: Signal Description: Contains the position the Steering should adjust to for EEEE Sent to: - Move Steering Position |
| EEEE_Request_Seat Type: EEEE Request - EEEE/Drive | Signal Description: These values indicate whether the request wants Easy Entry Easy Exit controlled commodities to move to Easy Entry Easy Exit position or the Drive Position. Usage in Function: Signal Description: Contains the position the Seat should adjust to for EEEE Sent to: - Move Seat Position |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Request EEEE

### Function Requirements

#### Functional Requirements

##### Normal Operation

67 Easy Entry Easy Exit - Toggle Activation 1

When the 'Provide EEEE Profile Status' function receives 'EEEE Toggle Status' as 'Enabled' then the 'Provide EEEE Profile Status' function  shall output 'EEEE Profile Status' as 'Enable'

| Requirement ID: 67 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_44 Easy Entry Easy Exit - Toggle Activation |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

68 Easy Entry Easy Exit - Toggle Activation 2

When the 'Provide EEEE Profile Status' function receives 'EEEE Toggle Status' as 'Disabled' then the 'Provide EEEE Profile Status' function  shall output 'EEEE Profile Status' as 'Disable'

| Requirement ID: 68 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

69 Easy Entry Easy Exit - Toggle Activation 3

While 'EEEE Profile Status' is 'Enable' then the 'Request EEEE' function shall allow Easy Entry Easy Exit functionality

| Requirement ID: 69 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

70 Easy Entry Easy Exit - Toggle Activation 4

While 'EEEE Profile Status' is 'Disable' then the 'Request EEEE' function shall inhibit Easy Entry Easy Exit functionality

| Requirement ID: 70 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

71 Easy Entry Easy Exit - Key Trigger to Offset Position 1

"While

- 'EEEE Profile Status' is 'Enable' for active profile

- 'Vehicle Speed' is 'Slow'

- 'Gear' is 'Recall Available'

- Easy Entry Easy Exit controlled commodities are in the drive position

When the 'Request EEEE' function receives 'Key_In_Ignition Status' as 'OUT' then the 'Request EEEE' function shall output 'EEEE_Request_Seat' and 'EEEE_Request_Steering' as 'EEEE'"

| Requirement ID: 71 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_45 Easy Entry Easy Exit - Key Trigger to Offset Position |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

72 Easy Entry Easy Exit - Key Trigger to Drive Position 1

"While

- 'EEEE Profile Status' is 'Enable' for active profile

- 'Vehicle Speed' is 'Slow'

- 'Gear' is 'Recall Available'

- Easy Entry Easy Exit controlled commodities are in the Easy Entry Easy Exit offset position

When the 'Request EEEE' function receives 'Key_In_Ignition Status' as 'IN' then the 'Request EEEE' function shall output 'EEEE_Request_Seat' and 'EEEE_Request_Steering' as 'Drive'"

| Requirement ID: 72 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_46 Easy Entry Easy Exit - Key Trigger to Drive Position |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

73 Easy Entry Easy Exit - Drive Control Optimization - Offset Position - Park 1

"While

- 'Configuration' is 'DCO'

- Easy Entry Easy Exit is set Enabled for the active profile

- Easy Entry Easy Exit controlled commodities are in the drive position

- 'Vehicle Speed' is 'Slow'

- 'Gear' is 'Recall Available'

- 'Seatbelt Status' is 'Belted'

When the 'Request EEEE' function receives transition of 'Park Status' from 'Not Park' to 'Park' then the 'Request EEEE' function shall output 'EEEE_Request_Seat' and 'EEEE_Request_Steering' as 'EEEE'"

| Requirement ID: 73 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_47 Easy Entry Easy Exit - Drive Control Optimization - Easy Exit Offset Position - Park Transition |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

74 Easy Entry Easy Exit - Drive Control Optimization - Offset Position - Seat Belt 1

"While

- 'Configuration' is 'DCO'

- Easy Entry Easy Exit is set Enabled for the active profile

- Easy Entry Easy Exit controlled commodities are in the drive position

- 'Vehicle Speed' is 'Slow'

- 'Gear' is 'Recall Available'

- 'Park Status' is 'Park'

When the 'Request EEEE' function receives transition of 'Seatbelt Status' as 'Unbelted' to 'Belted' then the 'Request EEEE' function shall output 'EEEE_Request_Seat' and 'EEEE_Request_Steering' as 'EEEE'"

| Requirement ID: 74 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_48 Easy Entry Easy Exit - Drive Control Optimization - Easy Exit Offset Position - Seatbelt Transition |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

75 Easy Entry Easy Exit - Drive Control Optimization - Drive Position 1

"While

- 'Configuration' is 'DCO'

- Easy Entry Easy Exit is set Enabled for the active profile

- Easy Entry Easy Exit controlled commodities are in the Easy Entry Easy Exit offset position

- 'Vehicle Speed' is 'Slow'

- 'Gear' is 'Recall Available'

When the 'Request EEEE' function receives

- 'Detection Status' as 'Inside

- 'Door Status' as 'Closed'

Then the 'Request EEEE' function shall output 'EEEE_Request_Seat' and 'EEEE_Request_Steering' as 'Drive'"

| Requirement ID: 75 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_50 Easy Entry Easy Exit - Drive Control Optimization - Drive Position |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

78 Easy Entry Easy Exit - Store to Drive Position 1

When the 'Request EEEE' function receives 'Memory_Command' as 'Store' then the 'Request EEEE' function shall consider the current position the drive position

| Requirement ID: 78 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_53 Easy Entry Easy Exit - Store to Drive Position |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

79 Easy Entry Easy Exit - Recall to Offset Position 1

While the Easy Entry Easy Exit controlled commodities are in the Easy Entry Easy Exit offset position when the 'Request EEEE' function receives 'Memory_Command' as 'Recall' for a profile that has Easy Entry Easy Exit enabled then the 'Request EEEE' function shall output 'EEEE_Request_Seat' and 'EEEE_Request_Steering' as 'EEEE'

| Requirement ID: 79 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_54 Easy Entry Easy Exit - Recall to Offset Position |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

86 Easy Entry Easy Exit - Inhibit Auto Save 1

When the 'Request EEEE' function outputs either 'EEEE_Request_Seat' or 'EEEE_Request_Steering' as 'EEEE' then the 'Request EEEE' function shall also output 'Auto Save Inhibit' as 'Inhibit'

| Requirement ID: 86 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_59 Easy Entry Easy Exit - Inhibit Auto Save |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

87 Easy Entry Easy Exit - Resume Auto Save

When either

- 'Request EEEE' function outputs either 'EEEE_Request_Seat' or 'EEEE_Request_Steering' as 'Drive'

- 'Request EEEE' function considers Easy Entry Easy Exit commodities in the drive position

Then the 'Request EEEE' function shall also output 'Auto Save Inhibit' as 'Normal'

| Requirement ID: 87 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

90 Stowable Steering Wheel - Inhibit EEEE 1

When the 'Request EEEE' function receives 'Stowable Steering Wheel Status' as 'Active' then the 'Request EEEE' function shall inhibit Easy Entry Easy Exit functionality

| Requirement ID: 90 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_62 Stowable Steering Wheel - Inhibit EEEE |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

91 Stowable Steering Wheel - Inhibit EEEE 2

When the 'Request EEEE' function receives 'Stowable Steering Wheel Status' as 'Inactive' then the 'Request EEEE' function shall allow Easy Entry Easy Exit functionality

| Requirement ID: 91 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

92 Rejuvenate - Inhibit EEEE 1

When the 'Request EEEE' function receives 'Rejuvenate Status' as 'Active' then the 'Request EEEE' function shall inhibit Easy Entry Easy Exit functionality

| Requirement ID: 92 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

93 Rejuvenate - Inhibit EEEE 2

When the 'Request EEEE' function receives 'Rejuvenate Status' as 'Inactive' then the 'Request EEEE' function shall allow Easy Entry Easy Exit functionality

| Requirement ID: 93 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

103 Diagnostics - Technician 1

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall apply the request

| Requirement ID: 103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

104 Diagnostics - Technician 2

When receiving a request to write from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall apply the request

| Requirement ID: 104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

105 Diagnostics - Technician 3

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Classic Memory Settings System shall send the requested data

| Requirement ID: 105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

106 Diagnostics - Technician 4

When receiving a request to read from the Diagnostics System while in Diagnostic Mode the Easy Entry Easy Exit System shall send the requested data

| Requirement ID: 106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Feature_Req_73 Diagnostics - Technician |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

# Open Concerns

| ID | Concern Description | e-Tracker / Reference | Responsible | Status | Solution |
| --- | --- | --- | --- | --- | --- |
| 1 | DCO timing |  |  |  |  |

Table 8: Open Concerns (Not supported by MagicDraw report generation.)

# Revision History

| VSEM Revision | Date | Description | Approved by | Responsible |
| --- | --- | --- | --- | --- |
| A | 12-21-20 | Initial version |  |  |
| B | 03-04-21 | - Added function IDs - Updated Requirements - Updated Functional Architecture |  |  |

## Template Revisions

#Important: Do not change this section

| Version | Rev. | Date | Description | Responsible |
| --- | --- | --- | --- | --- |
| 1 | 0 | 2016-02-26 | Initial version, derived from FDS | Jbaden1 |
| 1 | 1 | 2016-02-26 | Word properties corrected | Jbaden1 |
| 1 | 2 | 2016-03-10 | Clean up of document meta data (Word properties) | Jbaden1 |
| 1 | 3 | 2016-03-22 | - Footer formatting corrected (Issue 19) - “Constraints” chapter renamed to “Input Requirements” (Issue 20) | Jbaden1 |
| 1 | 4 | 2016-04-20 | - Broken Wiki links repaired | Jbaden1 |
| 2 | 0 | 2016-06-10 | - Document metadata adapted. Prepared for new macros - DTC table removed - HMI function added as a chapter (details still to be refined) - Signal / Parameter IDs column deleted interface tables | Jbaden1 |
| 2 | 1 | 2016-07-14 | - Converted to SysML diagrams - HMI section further elaborated - Template version added to footer - Dedicated Startup / Shutdown sections removed (only hints added) - Data Dictionary reworked and Signal / Parameter IDs column re-introduced | Jbaden1 |
| 2 | 2 | 2016-12-07 | - Minor formatting changes | Jbaden1 |
| 3 |  |  | Skipped to synchronize with Specification_Macros.dotm |  |
| 4 |  |  |  |  |
| 5 | 0 | 2017-01-13 | - Meta data updated for specification macros, version 3.1 - SW Unit chapter removed for the time being - Green boxes added for user hints | Jbaden1 |
| 5 | 1 | 2017-01-18 | - Some additional hints. - Hyperlinks highlighted in hints | Jbaden1 |
| 6 | 0 | 2017-04-28 | - Editorial change. Hints added to chapter 4.1.4 - Chapter “Traceability Matrix” removed | Jbaden1 |
| 6 | 0 | 2018-04-28 | - CR69/63: New chapters added for Functional Safety (FTTI and Technical Safety Requirements) - CR53: New coversheet + additional meta-data - CR76: merge sections for configuration and for calibration parameters into one on Function Level | Jbaden1 |
| 6 | 0 | 2018-08-06 | - CR66: Fix version numbering in footer of Function Spec | Jbaden1 |
| 6 | 0 | 2018-09-28 | - Broken links to RE Wiki repaired | Jbaden1 |
| 6 | 0 | 2018-10-31 | - Minor corrections on cover sheet and in footer to be more GIS compliant and VSEM aligned - “Overview” and “Description” exchanged in headings (following common sense) | Jbaden1 |
| 6 | 0 | 2018-11-12 | - Explanatory text in Variants” section revised - Functional Safety modifications as agreed with FuSa core team (Baseline: November 2018 Dearborn On-Site) | Jbaden1 |
| M |  | 2019-04-02 | - Initial version of SysML report template | snuesch |
| M |  | 2019-04-05 | - Improved dialog boxes to select function group | snuesch |
| M |  | 2019-04-26 | - Improved function interfaces (support of additional elements (e.g., send signal action, add structural feature action, merge node, decision node, activity parameter nodes, etc.) and pins) | snuesch |
| N |  | 2019-08-19 | - For each function now also requirements satisfied by activity parameter nodes and function outputs are populated. | snuesch |
| N |  | 2019-08-21 | - Improved glossary and acronym tables | snuesch |
| N |  | 2019-09-20 | - Updated Function Interfaces tables with better wording. Added a report template variable that allows to disable senders and receivers. Sorted requirements by ID. | snuesch |
| N |  | 2019-09-27 | - Green hints now only show up for first function. - labelTag variable can be used to filter revision history. | snuesch |

# Appendix

## Data Dictionary

### Logical Signals

Auto Save Store - Pers/Inactive

These values indicate when the Auto Save system has determined a Classic Memory store should occur. The value indicates which memory location the current settings should be stored to.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Store - Inactive |
| Unit |  |  |

Auto Save Inhibit

These values indicate when the Easy Entry Easy Exit system is active and thus Auto Save should inhibit automatic saving.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Inhibit - Normal |
| Unit |  |  |

Brightness

These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value | 0% |
|  | Max Value | 100% |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

CM Recall – Request/Inactive

These values indicate that a Classic Memory recall has occurred and for which memory location. This is to indicate to the Enhanced Memory system that it should perform an Enhanced Memory recall for the same memory location.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Request 1 - Request 2 - Request 3 - Request 4 - Inactive |
| Unit |  |  |

Deny Recall - Deny/Inactive

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Deny Recall - Inactive |
| Unit |  |  |

Driver Door Status - Ajar/Closed

These values indicate whether the Door System has determined that the driver door is ajar (open) or closed.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Closed - Ajar |
| Unit |  |  |

EEEE Profile Status – Enabled/Disabled

These values indicate whether the user has selected to enable or disable the Easy Entry Easy Exit system.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Enabled - Disabled |
| Unit |  |  |

EEEE Request - EEEE/Drive

These values indicate whether the request wants Easy Entry Easy Exit controlled commodities to move to Easy Entry Easy Exit position or the Drive Position.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - EEEE - Drive |
| Unit |  |  |

EM Recall - Request/Inactive

These values indicate that an Enhanced Memory recall has occurred and for which memory location. This is to indicate to the Classic Memory system that it should perform a Classic Memory recall for the same memory location.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Request 1 - Request 2 - Request 3 - Request 4 - Inactive |
| Unit |  |  |

Feature Configuration - Classic/Advanced

These values indicate whether the standard or variant configuration should be executed. The Advanced Classic Memory variant provides remote transmitter functionality and modified prompts.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Classic - Advanced |
| Unit |  |  |

Gear Status - Safe/NOT Safe

These values indicate whether the State Management System has determined that the current transmission gear is in a position where it would be safe to trigger a Classic Memory recall.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Safe - Not Safe |
| Unit |  |  |

Input Status - Recall/Set/Pers/None

These values indicate how the input from the user was perceived by the Classic Memory system. When Memory Set is selected then the input function will wait until a profile is selected before transmitting the information. Otherwise the input function relays the input from the user to the rest of the Classic Memory system.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Recall 1 - Recall 2 - Set 1 - Set 2 - Pers 1 - Pers 2 - Pers 3 - Pers 4 - No Selection |
| Unit |  |  |

Key In Igniton - In/Out

These values indicate whether the Key System has determined that the Key (or PEPS) in IN (ON) or OUT (OFF).

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - IN - OUT |
| Unit |  |  |

Manual Adjustment - Active/Inactive

These values indicate when a memory controller system has determined that a manual adjustment occurred. Manual adjustments cause automatic movement to cancel.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Active - Inactive |
| Unit |  |  |

Memory Command - Store/Recall

These values indicate whether memory controller systems should perform a Classic Memory store or Classic Memory recall. The value indicates the memory location to be used in the operation.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Store 1 - Store 2 - Store 3 - Store 4 - Recall 1 - Recall 2 - Recall 3 - Recall 4 - Inactive |
| Unit |  |  |

Park Status – Park/NOT Park

These values indicate whether the State Management System has determined that the vehicle is in a safe state to trigger a Classic Memory recall. When the vehicle is in Park then a Classic Memory recall may trigger.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Park - Not Park |
| Unit |  |  |

Position

These values indicate the actual settings being transferred. These can be stored settings to be recalled or current settings to be stored.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value | 0% |
|  | Max Value | 100% |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Profile Association - Pers/Inactive

These values indicate which memory location the user selected to use for their new profile.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Pers1 - Pers2 - Pers3 - Pers4 - Inactive |
| Unit |  |  |

Profile Association: Select Location - Associate/Inactive

These values indicate when the Enhanced Memory system is creating a new profile. The Classic Memory system waits for the user to select (associate) a memory location.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Associate - Inactive |
| Unit |  |  |

Profiles - Memory Location 1-4

These values indicate which memory location to use for the accompanying operation being execute.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Pers1 - Pers2 - Pers3 - Pers4 - Inactive |
| Unit |  |  |

Rejuvenate Status - Active/Inactive

These values indicate when the Rejuvenate system is active and thus the Easy Entry Easy Exit system should be inhibited.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Active - Inactive |
| Unit |  |  |

Remote Transmitter Status - Lock/Unlock Pers 1-4

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Lock 1 - Lock 2 - Lock 3 - Lock 4 - Unlock 1 - Unlock 2 - Unlock 3 - Unlock 4 |
| Unit |  |  |

Restriction Settings – Factory/Vehicle/Moveable/Non-Moveable

These values indicate which restrictions are applied to profiles.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Factory - Vehicle - Moveable Person - Non-Moveable Person |
| Unit |  |  |

Seatbelt Status - Engaged/Disengaged

These values indicate whether the Seatbelt System has determined that the driver seatbelt is engaged or disengaged.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Engaged (belted) - Disengaged (unbelted) |
| Unit |  |  |

Set Indicator - Set Available/Set NOT Available

These values indicate whether Memory Set functionality is available to the user.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Set Available - Set NOT Available |
| Unit |  |  |

Set Indicator Status - Active/Inactive

These values indicate when Memory Set is active. When active the indicator will notify the user for the duration that Memory Set is active.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Active - Inactive |
| Unit |  |  |

Stowable Steering Wheel - Active/Inactive

These values indicate when the Stowable Steering Wheel system is active and thus the Easy Entry Easy Exit system should be inhibited.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Active - Inactive |
| Unit |  |  |

Strategy Configuration - DCO/Standard

These values indicate whether Drive Control Optimization (DCO) is configured. DCO removes the key slot and PEPS from a vehicle thus making Easy Entry Easy Exit rely on different triggers.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - DCO - Standard |
| Unit |  |  |

User Selection - Selection/No Selection

These values indicate the input of the user.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Select 1 - Select 2 - Select 3 - Select 4 - No Selection - Set |
| Unit |  |  |

Vehicle Speed - Slow/NOT Slow

These values indicate whether the State Management System has determined that the current vehicle speed is at a speed where it would be safe to trigger a Classic Memory recall. When the speed is Slow then a Classic Memory recall may trigger.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  | - Slow - NOT Slow |
| Unit |  |  |

### Logical Parameters

### Encoding Types

## Glossary

### Definitions

| Definition | Description |
| --- | --- |
| Drive Position | The position from which the driver plans to drive the vehicle. Positional settings are by default in the Drive Position unless a feature changes the position. |
| EEEE Offset Position | The position which provide additional space and allows the user to enter and exit the vehicle more easily. A special position of the Easy Entry Easy Exit system (of Classic Memory feature). |
| Memory Positional Controller | When the vehicle has memory then each positional commodity (elements with adjustable positions such as seat, mirrors, etc.) that has memory will have a controller for arbitrating settings |
| Memory Location | Space within a memory positional controller allocated for the storage of positional settings |
| Profile | A profile is the collection of settings stored in a memory location. Each profile typically stores setting for one user. |
| Multicontour Seat | A seat with additional functionality such as massage and additional comfort settings |
| Chime | An audible alert |
| Prompt | A visual alert |
| Wait Mode | When a function enters a wait mode it then waits for a specific trigger to complete. A function may complete waiting when specific time has elapsed. |
| SLOW | Indicates the vehicle is at a speed where it is safe to trigger a Classic Memory recall |
| NOT SLOW | Indicates the vehicle is above a speed where it is safe to trigger a Classic Memory recall |
| SAFE | Indicates the vehicle is in a transmission gear where it is safe to trigger a Classic Memory recall |
| NOT SAFE | Indicates the vehicle is transmission gear where it is NOT safe to trigger a Classic Memory recall |
| Personality | Synonymous with ‘profile’ |
| Restriction Settings: Factory | In this restriction setting all profile are set to Default Profile. The Default Profile cannot be updated. All recalls will recall the Default Profile. |
| Restriction Settings: Vehicle | In this restriction setting all profile are set to Vehicle Profile. All recalls will recall the Vehicle Profile. |
| Restriction Settings: Non-Moveable Person | In this restriction setting all profile are set to Non-Moveable Person Profile. Setting cannot be loaded from or to an external device. |
| Restriction Settings: Moveable Person | In this restriction setting all profile are set to Moveable Person Profile. No restrictions. |
| Drive Control Optimization (DCO) | This is an effort to remove the dedicated key, key slot, and push to start button. As the key slot was a trigger for Easy Entry Easy Exit (EEEE) functionality then new triggers are needed when DCO is implemented. EEEE will then use PaaK, Door, Seatbelt, and Park as triggers. |

Table 9: Definitions used in this document

### Abbreviations

| Abbr. | Stands for |
| --- | --- |
| HUD | Head-Up Display |
| AHUD | Advanced Head-Up Display |
| CM | Classic Memory |
| ACM | Advanced Classic Memory |
| EEEE | Easy Entry / Easy Exit |
| EM | Enhanced Memory |
| PPP | Personal and Portable Profile |
| SSW | Stowable Steering Wheel |
| DCO | Drive Control Optimization |
| RKE | Remote Keyless Entry |

Table 10: Abbreviations used in this document

Document ends here.