|  | Function Group Spec EM Registers <<Logical>> (Allocated) |  |  |  |
| --- | --- | --- | --- | --- |
| Document Type | Function Specification |  |  |  |
| Template Version | 6.1a |  |  |  |
| SysML Report Template Version | M (03/09/2021) |  |  |  |
| Document ID | functionspec_sysmlreporttemplate |  |  |  |
| Document Location |  |  |  |  |
| Document Owner |  |  |  |  |
| Document Revision | FGS1 |  |  |  |
| Document Status | Draft |  |  |  |
| Date Issued | 2021/03/04 |  |  |  |
| Date Revised | 2021/03/04 |  |  |  |
| Document Classification | GIS1 Item Number: | 27.60/35 |  |  |
|  | GIS2 Classification: | Confidential |  |  |
| Document Approval |  |  |  |  |
| Name | Role |  | Email Confirmation | Date |

Auto-Generated by MagicDraw

Printed Copies are Uncontrolled

# Disclaimer

This document contains Ford Motor Company Confidential information. Disclosure of the information contained in any portion of this document is not permitted without the expressed, written consent of a duly authorized representative of Ford Motor Company, Dearborn, Michigan, U.S.A.

Copyright, Ó 2021  Ford Motor Company

This document contains information developed and accumulated by and for FORD MOTOR COMPANY. As such, it is a proprietary document, which, if disseminated to unauthorized persons, would provide others with restricted information, data, or procedures not otherwise available, exposing the FORD MOTOR COMPANY to potential harm.

Employees and suppliers having custody of this specification or authorized to use it must be cognizant of its proprietary nature and ensure that the information herein is not made available to unauthorized persons.

FORD MOTOR COMPANY reserves the right to protect this work as an unpublished copyrighted work in the event of an inadvertent or deliberate unauthorized publication. FORD MOTOR COMPANY also reserves its rights under copyright laws to protect this work as a published work.

This document or portions thereof shall not be distributed outside FORD MOTOR COMPANY without prior written consent. Refer all questions concerning disclosure to the author(s) or to any duly authorized representative of Ford Motor Company.

# Contents

1.1.1	Decomposition of   	4

Disclaimer	16

Contents	17

2	Introduction	19

2.1	Document Purpose	19

2.2	Document Scope	19

2.3	Document Audience	19

2.3.1	Stakeholder List	19

2.4	Document Organization	19

2.4.1	Document Context	19

2.4.2	Document Structure	19

2.5	Document Conventions	20

2.5.1	Requirements Templates	20

3	Function Group Description	21

4	Functional Architecture	22

4.1	Description	22

4.2	Function List	22

4.3	Signal List	22

5	Function Specifications	23

5.1	  	23

5.1.1	Function Overview	23

5.1.2	Function Scope	25

5.1.3	Function Interfaces	26

5.1.4	Function Modeling	28

5.1.5	Function Requirements	28

6	Open Concerns	32

7	Revision History	33

8	Appendix	34

8.1	Data Dictionary	34

8.1.1	Logical Signals	34

8.1.2	Logical Parameters	34

8.1.3	Encoding Types	34

8.2	Glossary	34

8.2.1	Definitions	34

8.2.2	Abbreviations	35

List of Figures

Figure 1 Activity diagram “” depicting black box behavior collaboration	22

Figure 2: Activity Diagram of   “” calling   “”	26

Figure 3: 	28

List of Tables

Table 1:  - Decomposition Table	5

Table 2: Functions described in this specification	19

Table 3: List of Logical Functions	22

Table 4: Ford internal Documents	25

Table 5: Ford internal Documents (not specified in model)	25

Table 6: External documents and publications	25

Table 7: External documents and publications (not specified in model)	25

Table 8: Open Concerns (Not supported by MagicDraw report generation.)	32

Table 9: Definitions used in this document	35

Table 10: Abbreviations used in this document	35

# Introduction

## Document Purpose

The Function (Group) Specification (FS) specifies an individual function / a group of functions.

To get more information about the concept of feature, function and component level abstraction refer to the Ford RE Wiki.

## Document Scope

The following set of functions from the Global Feature & Function List is described in this specification.

| Function ID | Function Name | Owner | Reference |
| --- | --- | --- | --- |

Table 2: Functions described in this specification

## Document Audience

The FS is authored by the owners of the individual functions. All Stakeholders, i.e., all people who have a valid interest in the functions and their behavior should read and, if possible, review the FS. It needs to be guaranteed, that all stakeholders have access to the currently valid version of the FS.

### Stakeholder List

For the latest list of the feature stakeholder and their roles & responsibilities refer to <Put VSEM Link here>.

| Name | CDSID | Contact Info | Role | Stakeholder Group |
| --- | --- | --- | --- | --- |
| Vikaram Gokhale | vgokhale |  | Feature Delivery Supervisor |  |
| Landie Berry | cberry37 |  | HMI UX |  |
| William Johnston | wjohnst2 |  | Supervisor Occupant Features |  |
| Robert Paquette | rpaquet2 |  | Infotainment Systems Engr |  |
| Dinh Tran | dtran39 |  | Infotainment Systems Engr |  |
| Jason Richardson | jricha41 |  | Climate Core Engr | VESC |
| Michelle Churchill | mchurc35 |  | D & R Engineer |  |
| Andrew Zourob | azourob |  | Climate Control Core Engr | VESC |
| Ken Cunningham | kcunni16 |  | FuSa Coach |  |
| George Smith | GSMIT560 |  | Feature Owner | Interior Cockpit and Trim Core Engineerint |
| Steven Jacobson | sjcobse |  | Feature Champion |  |
| Christopher Van Auken | cvanauke |  | RCCM Engineer | VESC |
| Andres Portillo Reyes | aportil9 |  | Climate Hardware Engr | VESC |
| Aamir Pasha | apasha |  | Feature Delivery Engineer |  |
| Martin Lindell | mlindel4 |  | Climate CFD |  |
| Mitali Chakrabarti | mchakrab |  | Climate Features Supervisor |  |
| Neelima Majjiga | nmajjiga |  | Infotainment Systems Engr |  |
| Julee Harlow | jharlow6 |  | HMI Supervisor |  |
| Jamie Liao | jliao |  | Climate Hardware Engr | VESC |
| Andrew Zourob | azourob |  | Climate Control Core Engr |  |
| Laura Check | LBUREK |  | SYNC3 Supervisor |  |
| Jorge Aaujo | jarauj59 |  | Register Engineer |  |
| Joe Qussar | jqussar |  | FuSa Engr |  |
| Demetrius Johnson-Gault | djohn840 |  | HMI Core Engr |  |
| Joshua Cerda | jcerda7 |  | Modeling Team Member |  |
| Greg Kopp | gkopp6 |  | HMI Engineer |  |
| Martin Imhof | mimhof4 |  | Model Architect |  |
| Tao Zhou | tzhou32 |  | Climate Core Engr | VESC |
| Fernando OVando | joveaneo |  | Register Engineer |  |
| Cindy Rutyna | crutyna |  | Supervisor Climate E/E | VESC |

## Document Organization

### Document Context

Refer to the Specification Structure page in the Ford RE Wiki to understand how the FS relates to other Ford Requirements Documents and Specifications.

### Document Structure

The structure of this document is explained below:

Section 1 – 	Introduction how to use this document including responsibilities and requisite documents. Explains the tterminology. Gives a clarification of the definitions, concepts and abbreviations used in the document.

Section 2 – 	Function Group Description. Gives an overview and the purpose of the function group.

Section 3 – 	Functional Architecture: Specifies the overall functional architecture of the function group

Section 4 – 	Function Specifications: Specifies the logical functions of the function group in detail

Section 5 – 	List of Open Concerns

Section 6	Revision history including a list of new or modified requirements. The requirements in this document are tagged, and this section contains different types of tables listing all, new, or changed requirements by their title and page no.

Section 7 – 	Appendix: Presenting additional data mainly in a tabular form, e.g., a data dictionary

## Document Conventions

### Requirements Templates

Refer to “How to use the Specification Templates” on how to use the specification templates and the VBA macros to create/edit the requirements in the specifications.

The VBA macro enable the import of the specification to VSEM (refer to "How to import specifications into VSEM as separate requirements").

#### Identification of Requirements

The unique requirement ID given in the headline of any requirement follows the requirement throughout the development process. The requirement ID format follows a well-defined syntax.

All identifiers in a FS shall be composed of 4 parts:

- A leading prefix, which indicates the type of requirement (R=Requirement, UC=Use Case, SC=Scenario, …)

- A prefix, which indicates the abstraction level (F=Feature, FNC=Function, CMP = component).

- Followed by a name, indicating the scope, which the requirement belongs to (e.g. feature or function name )

- Ending with the actual requirement number

Example:

R_FNC_LockArbitrator_00004	This is the fourth requirement on function level for the function Lock Arbitrator.

#### Requirements Attributes

The templates provided by Specification_Macros.dotm define a list of attributes for each requirement. This helps to classify the requirement. The attributes are explained at RE Wiki - Requirements Attributes.

## References

### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Document Location | Revision |
| --- | --- | --- | --- | --- |
| Ford GIS Standard | Ford GIS Standard |  |  |  |

Table 2: Ford Documents

### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication | Document Location |
| --- | --- | --- |
| IEEE Std 1012-2004 IEEE Standard for Software Verification and Validation |  |  |
| ISO/IEC 19500-2:2003 | Information technology -- Open Distributed Processing -- Part 2 |  |
| UML Testing Profile (UTP), v1.2 |  |  |
| Wikipedia |  |  |

Table 3: External Documents and Publications

## Glossary

### Definitions

No terms specified.

| Definition | Description |
| --- | --- |

Table 4: Definitions relevant for “Logical Function A”

### Abbreviations

No acronyms specified.

| Abbr. | Meaning | Description |
| --- | --- | --- |
| FS | Function Requirements Specification / Function Group Specification | The document describing, collecting and developing the requirements of a function or a group of functions. |

Table 5: Abbreviations relevant for “Logical Function A”

# Function Group Description

This Function Group consists of all functions allocated to   EM Registers <<Logical>> including all functions in their corresponding call trees.

Description of EM Registers:

Feature block

# Functional Decomposition and Architecture

## Description

## Function List

| Function ID | Function Name | Function Description | ASIL |
| --- | --- | --- | --- |
|  | Provide Climate system with User Input <<System Function>> | Senses user input and gives said input/selection to the Climate system. |  |
|  | Retrieve LastPosition Value <<Subsystem Function>> | Retrieve the LastPosition position value from memory. |  |
|  | Save Positions <<System Function>> | Saves the registers position every time the registers are moved by the user through "manually" moving the registers or through selecting a preset. |  |
|  | Save LastPosition of Corresponding Registers to Selected Preset <<System Function>> | Saves the LastPosition to the selected preset. This allows for the presets to be programmed with the current register positions. |  |
|  | Provide Register Positons <<Subsystem Function>> |  |  |
|  | Send Move Command to Actuators <<Subsystem Function>> | Register Controller sends move request to the register actuators to move the registers. |  |
|  | Display Visual Feedback <<System Function>> | Shows the user the HMI that visually depicts the register positions and various selections. |  |
|  | Move Corresponding Registers to Received Position <<System Function>> | General Function that will move the registers to the directed position. |  |
|  | Detect User Input <<Subsystem Function>> | senses user input from HMI |  |
|  | Move Registers to HOME <<System Function>> | Will move the registers to the HOME position every time the vehicle state transitions to OFF. |  |
|  | Track Registers' Positions <<Subsystem Function>> | Sense the registers physical position and gives that position to the Climate system |  |
|  | Save to LastPosition <<Subsystem Function>> | Saves the new register position to the LastPosition every time it receives a move register command |  |
|  | Move to Requested Position <<Subsystem Function>> | Registers move to the requested position. |  |
|  | Send Feedback to HMI <<Subsystem Function>> | Sends feedback to the HMI to display when the HMI display is shown next. |  |
|  | Provide User move register move commands <<Subsystem Function>> | provides move register move commands |  |
|  | Save LastPosition to BeforeClosedPosition <<Subsystem Function>> | Saves LastPosition to BeforeClosedPosition every time ButtonPressed is a close command |  |
|  | Detect Register Feedback Positions <<Subsystem Function>> | Senses register feedback positions |  |
|  | Retrieve HOME Value <<Subsystem Function>> | Retrieve the HOME position value from memory. |  |
|  | Program Presets <<Subsystem Function>> | Function that stores the desired register setting under a preset button |  |
|  | Move Registers to LastPosition <<System Function>> | Will move the registers to the LastPosition every time the vehicle state transitions to the ON state. |  |
|  | Provide Display Feedback Positions <<Subsystem Function>> | detects register positions feedback from register move command |  |

Table 3: List of Logical Functions

## Signal List

Refer to the Data Dictionary - Logical Signals.

# Function Specifications

## Logical Function   Provide Climate system with User Input

### Function Overview

#### Function Description

Function is allocated to:

- Climate System <<Logical>>

- EM Registers <<Logical>>

- HMI System <<Logical>>

Senses user input and gives said input/selection to the Climate system.

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Provide Climate system with User Input” function is called by the following functions:

- – “Move Registers”

- – “Save LastPosition in Preset”

- – “Store Registers' Positions”

Figure 2: Activity Diagram of   “Move Registers” calling   “Provide Climate system with User Input”

Figure 2: Activity Diagram of   “Save LastPosition in Preset” calling   “Provide Climate system with User Input”

Figure 2: Activity Diagram of   “Store Registers' Positions” calling   “Provide Climate system with User Input”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| userInput | User input from HMI screen display |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| MoveRegisterCommand | Move register commands based on register postion settings or button options Sent to: - Save Positions - Save LastPosition of Corresponding Registers to Selected Preset - Move Corresponding Registers to Received Position |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

Figure 3: Provide Climate system with User Input

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

HMI System Functionality

When "Display Visual Feedback" system function receives "RegistersPositionsHMIFeedback, it shall provide "userFeedback".

Provide Climate System with User Input  delete function?????

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Provide user move register selections feedback |  |  |  |  |  |  |
| Acceptance Criteria | Observation of feedback on HMI Screen |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | George Smith |
| Source Req. | - EM Reg HMI Functionality - HMI Feedback - EM Reg Move Command - EM Reg Preset Aim Memo - HVAC Vent Air Flow Direction Interface - EM Reg HMI Feedback - EM Reg Preset Command |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 2 - Medium | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

HMI to Climate System User Input

When "Provide Climate System With User Input" system function receives "MoveRegisterCommand" user input , it shall provide the climate system with "MoveRegisterCommand"

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | George Smith |
| Source Req. | - EM Reg HMI Functionality - Personalizable - EM Reg Preset Aim Memo - HVAC Vent Air Flow Direction Interface |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 2 - Medium | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Retrieve LastPosition Value

### Function Overview

#### Function Description

Function is allocated to:

- EM Registers <<Logical>>

- Register Controller <<Logical>>

Retrieve the LastPosition position value from memory.

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Retrieve LastPosition Value” function is called by the following functions:

- – “Move Registers to LastPosition”

Figure 2: Activity Diagram of   “Move Registers to LastPosition” calling   “Retrieve LastPosition Value”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| vehicleStatus | Vehicle on off status used by register move commands |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| MoveRegisterCommand | Move register commands based on register postion settings or button options Sent to: - Send Move Command to Actuators |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Register Controller Functionality

Register Controller shall output Registers'PositionsAcuatorCommand, "Registers'PostionsHMIFeedback" and input "Registers'PositionsSensorFeedback", "MoveRegisterCommand".

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Climate System Functionality - HMI to Climate System User Input - Preset Memorization - Save LastPositions |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Retrieve LastPosition Value

"Retrieve LastPosition Value" subsystem function shall output the LastPosition value as "MoveRegisterCommand" when it receives "vehicleStatus"==ON.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - LastPosition |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Save Positions

### Function Overview

#### Function Description

Function is allocated to:

- Climate System <<Logical>>

- EM Registers <<Logical>>

Saves the registers position every time the registers are moved by the user through "manually" moving the registers or through selecting a preset.

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Save Positions” function is called by the following functions:

- – “Store Registers' Positions”

Figure 2: Activity Diagram of   “Store Registers' Positions” calling   “Save Positions”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| MoveRegisterCommand | Move register commands based on register postion settings or button options Received from: - Provide Climate system with User Input |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

Figure 3: Save Positions

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Save BeforeClosedPositions

When the "Saved Positions" system function receives the signal "MoveRegisterCommand", it shall store the old registers positions(LastPosition) in BeforeClosedPosition then store the new registers positions in LastPosition when it receives "ButtonPressed"==(Close_LHOB||Close_LHIB||Close_RHIB||Close_RHOB

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | George Smith |
| Source Req. | - Registers remember users last chosen position Memory - EM Reg Memo Postion |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 2 - Medium | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Save LastPositions

When"Saved Positions" system function receives the signal "MoveRegisterCommand", it shall store the new registers positions in LastPosition  "ButtonPressed"!=(Close_LHOB||Close_LHIB||Close_RHIB||Close_RHOB

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Store the most recent register move request position as the last position stored |  |  |  |  |  |  |
| Acceptance Criteria | Observation of last stored register position |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | George Smith |
| Source Req. | - Registers remember users last chosen position Memory - EM Reg Memo Postion |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 2 - Medium | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Save LastPosition of Corresponding Registers to Selected Preset

### Function Overview

#### Function Description

Function is allocated to:

- Climate System <<Logical>>

- EM Registers <<Logical>>

Saves the LastPosition to the selected preset. This allows for the presets to be programmed with the current register positions.

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Save LastPosition of Corresponding Registers to Selected Preset” function is called by the following functions:

- – “Save LastPosition in Preset”

Figure 2: Activity Diagram of   “Save LastPosition in Preset” calling   “Save LastPosition of Corresponding Registers to Selected Preset”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| MoveRegisterCommand | Move register commands based on register postion settings or button options Received from: - Provide Climate system with User Input |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

Figure 3: Save LastPosition of Corresponding Registers to Selected Preset

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Preset Memorization

When "Save LastPosition of Selected Registers to Selected Preset" system function receives the signal "MoveRegisterCommand", it  shall save LastPosition of the selected StaticSelection to:  "MoveRegisterCommand"==PRESET_ONE_SAVED || "MoveRegisterCommand"==PRESET_TWO_SAVED.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | George Smith |
| Source Req. | - Personalizable - EM Reg Preset Aim Memo |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 2 - Medium | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Provide Register Positons

### Function Overview

#### Function Description

Function is allocated to:

- Register Sensor <<Logical>>

No description provided for this function.

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Provide Register Positons” function is called by the following functions:

- – “Move Registers to HOME”

Figure 2: Activity Diagram of   “Move Registers to HOME” calling   “Provide Register Positons”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Actuate Register Command | Signal requesting actuation of register position changes Received from: - Move to Requested Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Registers'PostionsHMIFeedback | Register position feedback to HMI Sent to: - Track Registers' Positions |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

No Normal Operation Requirements specified.

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Send Move Command to Actuators

### Function Overview

#### Function Description

Function is allocated to:

- EM Registers <<Logical>>

- Register Controller <<Logical>>

Register Controller sends move request to the register actuators to move the registers.

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Send Move Command to Actuators” function is called by the following functions:

- – “Move Corresponding Registers to Received Position”

- – “Move Registers to HOME”

- – “Move Registers to LastPosition”

Figure 2: Activity Diagram of   “Move Corresponding Registers to Received Position” calling   “Send Move Command to Actuators”

Figure 2: Activity Diagram of   “Move Registers to HOME” calling   “Send Move Command to Actuators”

Figure 2: Activity Diagram of   “Move Registers to LastPosition” calling   “Send Move Command to Actuators”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| RegistersPositionsFeedback | Register sensor position feedback to HMI Received from: - Track Registers' Positions |
| RegHomePositions | Received from: - Retrieve LastPosition Value - Retrieve HOME Value |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| RegHomePositions | Sent to: - Move to Requested Position |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Register Controller Functionality

Register Controller shall output Registers'PositionsAcuatorCommand, "Registers'PostionsHMIFeedback" and input "Registers'PositionsSensorFeedback", "MoveRegisterCommand".

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Climate System Functionality - HMI to Climate System User Input - Preset Memorization - Save LastPositions |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Send Move Command to Actuators

"Send Move Command to Actuators" subsystem function shall send "Registers'PositionsActuatorCommand" to the corresponding actuators anytime it receives "moveRegisterCommand".

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - HOME - Move Registers - LastPosition |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Display Visual Feedback

### Function Overview

#### Function Description

Function is allocated to:

- EM Registers <<Logical>>

- HMI System <<Logical>>

Shows the user the HMI that visually depicts the register positions and various selections.

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Display Visual Feedback” function is called by the following functions:

- – “Display Visual Feedback”

- – “Move Registers”

Figure 2: Activity Diagram of   “Move Registers” calling   “Display Visual Feedback”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Registers'PostionsHMIFeedback | Register position feedback to HMI Received from: - Move Corresponding Registers to Received Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| userFeedback | User feedback sent to the HMI visual output |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

Figure 3: Display Visual Feedback

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

HMI System Functionality

When "Display Visual Feedback" system function receives "RegistersPositionsHMIFeedback, it shall provide "userFeedback".

Provide Climate System with User Input  delete function?????

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Provide user move register selections feedback |  |  |  |  |  |  |
| Acceptance Criteria | Observation of feedback on HMI Screen |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | George Smith |
| Source Req. | - EM Reg HMI Functionality - HMI Feedback - EM Reg Move Command - EM Reg Preset Aim Memo - HVAC Vent Air Flow Direction Interface - EM Reg HMI Feedback - EM Reg Preset Command |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 2 - Medium | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Display Visual Feedback

When "Display Visual Feedback" system function receives the input "RegistersPositionsHMIFeedback, it shall display the status of EM registers to the user through HMI.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Provide user with acknowledgement of requested register move commands |  |  |  |  |  |  |
| Acceptance Criteria | Observation of feedback to HMI screen |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | George Smith |
| Source Req. | - HMI Feedback - EM Reg HMI Feedback |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 2 - Medium | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Move Corresponding Registers to Received Position

### Function Overview

#### Function Description

Function is allocated to:

- Climate System <<Logical>>

- EM Registers <<Logical>>

General Function that will move the registers to the directed position.

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Move Corresponding Registers to Received Position” function is called by the following functions:

- – “Move Corresponding Registers to Received Position”

- – “Move Corresponding Registers to Received Position”

- – “Move Corresponding Registers to Received Position”

- – “Move Registers”

Figure 2: Activity Diagram of   “Move Registers” calling   “Move Corresponding Registers to Received Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| MoveRegisterCommand | Move register commands based on register postion settings or button options Received from: - Provide Climate system with User Input |
| MoveRegisterCommand | Move register commands based on register postion settings or button options |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Registers'PostionsHMIFeedback | Register position feedback to HMI Sent to: - Display Visual Feedback |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

Figure 3: Move Corresponding Registers to Received Position

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Move Registers

When "Move Registers to Received Position" system function receives "MoveRegisterCommand", it shall move the registers to the said position.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Provide option to change register aiming position |  |  |  |  |  |  |
| Acceptance Criteria | Detect airflow in desired direction |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | George Smith |
| Source Req. | - EM Reg Cyclical Move Command - EM Reg Move Command - Electrically Actuated - EM Reg Preset Command |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 2 - Medium | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Move Registers Feedback

When "Move Registers to Received Position" system function receives "MoveRegisterCommand", it shall update Registers'PositionsHMIFeedback to HMI.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | George Smith |
| Source Req. | - EM Reg Cyclical Move Command - EM Reg Move Command - HVAC Vent Air Flow Direction Interface - EM Reg Preset Command |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 2 - Medium | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Detect User Input

### Function Overview

#### Function Description

Function is allocated to:

- EM Registers <<Logical>>

- Register Controller <<Logical>>

senses user input from HMI

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Detect User Input” function is called by the following functions:

- – “Provide Climate system with User Input”

Figure 2: Activity Diagram of   “Provide Climate system with User Input” calling   “Detect User Input”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| userInput | User input from HMI screen display |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| userInput | User input from HMI screen display Sent to: - Provide User move register move commands |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Detect User Input

When subsystem function receives the signal "userinput", it shall forward the signal to the subsystem function "Provide User Input"

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - HMI System Functionality - Display Visual Feedback - Body Control System Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Move Registers to HOME

### Function Overview

#### Function Description

Function is allocated to:

- Body Control System <<Logical>>

- Climate System <<Logical>>

- EM Registers <<Logical>>

Will move the registers to the HOME position every time the vehicle state transitions to OFF.

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Move Registers to HOME” function is called by the following functions:

- – “Move Registers”

- – “Move Registers to HOME”

Figure 2: Activity Diagram of   “Move Registers” calling   “Move Registers to HOME”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| vehicleStatus | Vehicle on off status used by register move commands |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

Figure 3: Move Registers to HOME

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

HOME

When "Move Registers to HOME" system function receives "VehicleStatus==OFF", it shall move the registers to the HOME position.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Provide a reference register position on vehicle startup |  |  |  |  |  |  |
| Acceptance Criteria | Detect airflow at nominal register positions |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | George Smith |
| Source Req. | - Electrically Actuated - EM Reg Home Position |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 2 - Medium | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Body Control System Functionality

"Body Control System" shall provide the "vehicleStatus" to the "Climate System".

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | When vehicle status is on register positions can be stored |  |  |  |  |  |  |
| Acceptance Criteria | Observation of last position is stored on vehicle sartup |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | George Smith |
| Source Req. | - EM Reg Vehicle ON - EM Reg Home Position |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 2 - Medium | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Track Registers' Positions

### Function Overview

#### Function Description

Function is allocated to:

- EM Registers <<Logical>>

- Register Controller <<Logical>>

- Register Sensor <<Logical>>

Sense the registers physical position and gives that position to the Climate system

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Track Registers' Positions” function is called by the following functions:

- – “Move Corresponding Registers to Received Position”

- – “Move Registers to HOME”

Figure 2: Activity Diagram of   “Move Corresponding Registers to Received Position” calling   “Track Registers' Positions”

Figure 2: Activity Diagram of   “Move Registers to HOME” calling   “Track Registers' Positions”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Registers'PostionsHMIFeedback | Register position feedback to HMI Received from: - Provide Register Positons |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| RegistersPositionsFeedback | Register sensor position feedback to HMI Sent to: - Send Feedback to HMI - Send Move Command to Actuators |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Retrieve Registers' Positions

"Retrieve Registers' Positions" subsystem function shall sense the registers positions after any registers are moved and output that information as "Registers'PositionsSensorFeedback".

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Move Registers Feedback |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Register Sensor Functionality

Register Sensor shall output "Registers'PositionsSensorFeedback" to Register Control every time a Register Actuator moves.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Move Registers Feedback |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Save to LastPosition

### Function Overview

#### Function Description

Function is allocated to:

- EM Registers <<Logical>>

- Register Controller <<Logical>>

Saves the new register position to the LastPosition every time it receives a move register command

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Save to LastPosition” function is called by the following functions:

- – “Save Positions”

Figure 2: Activity Diagram of   “Save Positions” calling   “Save to LastPosition”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| MoveRegisterCommand | Move register commands based on register postion settings or button options |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Save to LastPosition

"Save to LastPosition" subsystem function shall save the new register position to LastPosition every time it receives "MoveRegisterCommand".

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Save LastPositions |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Move to Requested Position

### Function Overview

#### Function Description

Function is allocated to:

- EM Registers <<Logical>>

- Register Acuator <<Logical>>

Registers move to the requested position.

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Move to Requested Position” function is called by the following functions:

- – “Move Corresponding Registers to Received Position”

- – “Move Registers to HOME”

- – “Move Registers to LastPosition”

Figure 2: Activity Diagram of   “Move Corresponding Registers to Received Position” calling   “Move to Requested Position”

Figure 2: Activity Diagram of   “Move Registers to HOME” calling   “Move to Requested Position”

Figure 2: Activity Diagram of   “Move Registers to LastPosition” calling   “Move to Requested Position”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| RegHomePositions | Received from: - Send Move Command to Actuators |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Actuate Register Command | Signal requesting actuation of register position changes Sent to: - Provide Register Positons |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Move to Requested Position

"Move to Requested Position" subsystem function shall move registers to requested position when it receives "Registers'PositionsActuatorCommand".

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - HOME - Move Registers - LastPosition |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Register Actuator Functionality

Register Actuator shall move itself to the correct position whenever it receives "Registers'PositionsActuatorCommand" from Register Controller.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - HOME - Move Registers - LastPosition |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Send Feedback to HMI

### Function Overview

#### Function Description

Function is allocated to:

- EM Registers <<Logical>>

- Register Controller <<Logical>>

Sends feedback to the HMI to display when the HMI display is shown next.

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Send Feedback to HMI” function is called by the following functions:

- – “Move Corresponding Registers to Received Position”

Figure 2: Activity Diagram of   “Move Corresponding Registers to Received Position” calling   “Send Feedback to HMI”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| RegistersPositionsFeedback | Register sensor position feedback to HMI Received from: - Track Registers' Positions |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Registers'PostionsHMIFeedback | Register position feedback to HMI |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Register Controller Functionality

Register Controller shall output Registers'PositionsAcuatorCommand, "Registers'PostionsHMIFeedback" and input "Registers'PositionsSensorFeedback", "MoveRegisterCommand".

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Climate System Functionality - HMI to Climate System User Input - Preset Memorization - Save LastPositions |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Send Feedback to HMI

"Send Feedback to HMI" subsystem function shall send "Registers'PositionsHMIFeedback" to HMI every 50ms.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Move Registers Feedback |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Provide User move register move commands

### Function Overview

#### Function Description

Function is allocated to:

- EM Registers <<Logical>>

- Register Controller <<Logical>>

provides move register move commands

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Provide User move register move commands ” function is called by the following functions:

- – “Provide Climate system with User Input”

Figure 2: Activity Diagram of   “Provide Climate system with User Input” calling   “Provide User move register move commands ”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| userInput | User input from HMI screen display Received from: - Detect User Input |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| MoveRegisterCommand | Move register commands based on register postion settings or button options |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Provide User Input

When subsytem function receives the signal 'userinput', it shall provide the signal 'MoveRegisterCommand'

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - HMI System Functionality - Display Visual Feedback |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Save LastPosition to BeforeClosedPosition

### Function Overview

#### Function Description

Function is allocated to:

- EM Registers <<Logical>>

- Register Controller <<Logical>>

Saves LastPosition to BeforeClosedPosition every time ButtonPressed is a close command

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Save LastPosition to BeforeClosedPosition” function is called by the following functions:

- – “Save Positions”

Figure 2: Activity Diagram of   “Save Positions” calling   “Save LastPosition to BeforeClosedPosition”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| MoveRegisterCommand | Move register commands based on register postion settings or button options |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| MoveRegisterCommand | Move register commands based on register postion settings or button options |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Save LastPosition to BeforeClosedPosition

"Save LastPosition to BeforeClosedPosition" subsystem function shall save the current register positions(LastPosition) to BeforeClosedPosition every time "ButtonPressed"==(Close_LHOB||Close_LHIB||Close_RHIB||Close_RHOB).

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Save BeforeClosedPositions |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Detect Register Feedback Positions

### Function Overview

#### Function Description

Function is allocated to:

- EM Registers <<Logical>>

- Register Controller <<Logical>>

Senses register feedback positions

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Detect Register Feedback Positions” function is called by the following functions:

- – “Display Visual Feedback”

Figure 2: Activity Diagram of   “Display Visual Feedback” calling   “Detect Register Feedback Positions”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Registers'PostionsHMIFeedback | Register position feedback to HMI |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| RegistersPositionsFeedback | Register sensor position feedback to HMI Sent to: - Provide Display Feedback Positions |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Detect Register Feedback Positions

When system subfunction "Detect Register Feedback Positions" receives the signal "Detect Feedback" it shall forward the signal to the "Provide Display Feedback Positons"

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - HMI System Functionality - Display Visual Feedback |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Retrieve HOME Value

### Function Overview

#### Function Description

Function is allocated to:

- EM Registers <<Logical>>

- Register Controller <<Logical>>

Retrieve the HOME position value from memory.

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Retrieve HOME Value” function is called by the following functions:

- – “Move Registers to HOME”

Figure 2: Activity Diagram of   “Move Registers to HOME” calling   “Retrieve HOME Value”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| vehicleStatus | Vehicle on off status used by register move commands |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| RegHomePositions | Sent to: - Send Move Command to Actuators |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Register Controller Functionality

Register Controller shall output Registers'PositionsAcuatorCommand, "Registers'PostionsHMIFeedback" and input "Registers'PositionsSensorFeedback", "MoveRegisterCommand".

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - Climate System Functionality - HMI to Climate System User Input - Preset Memorization - Save LastPositions |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Retrieve HOME Value

"Retrieve HOME Value" subsystem function shall output the HOME value as "MoveRegisterCommand" when it receives "vehicleStatus"==OFF

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - HOME |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Program Presets

### Function Overview

#### Function Description

Function that stores the desired register setting under a preset button

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Program Presets” function is called by the following functions:

- – “Save LastPosition of Corresponding Registers to Selected Preset”

Figure 2: Activity Diagram of   “Save LastPosition of Corresponding Registers to Selected Preset” calling   “Program Presets”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| MoveRegisterCommand | Move register commands based on register postion settings or button options |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

No Normal Operation Requirements specified.

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Move Registers to LastPosition

### Function Overview

#### Function Description

Function is allocated to:

- Body Control System <<Logical>>

- Climate System <<Logical>>

- EM Registers <<Logical>>

Will move the registers to the LastPosition every time the vehicle state transitions to the ON state.

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Move Registers to LastPosition” function is called by the following functions:

- – “Move Registers”

- – “Move Registers to LastPosition”

Figure 2: Activity Diagram of   “Move Registers” calling   “Move Registers to LastPosition”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| vehicleStatus | Vehicle on off status used by register move commands |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

Figure 3: Move Registers to LastPosition

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

LastPosition

When "Move Registers to LastPosition" system function receives "VehicleStatus==ON" or "VehicleStatus==Accessory", it shall move the registers to the LastPosition

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | George Smith |
| Source Req. | - EM Reg Vehicle ON - Electrically Actuated |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 2 - Medium | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Body Control System Functionality

"Body Control System" shall provide the "vehicleStatus" to the "Climate System".

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | When vehicle status is on register positions can be stored |  |  |  |  |  |  |
| Acceptance Criteria | Observation of last position is stored on vehicle sartup |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | George Smith |
| Source Req. | - EM Reg Vehicle ON - EM Reg Home Position |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 2 - Medium | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Logical Function   Provide Display Feedback Positions

### Function Overview

#### Function Description

Function is allocated to:

- EM Registers <<Logical>>

- Register Controller <<Logical>>

detects register positions feedback from register move command

#### Function Variants

Not supported by MagicDraw report generation.

| Variant Name | Variant Description | Variant Condition (optional) |
| --- | --- | --- |
| e.g. “High Content” |  | e.g. ExtLightTechnology = LED OR ExtLightTechnology = Xenon |

#### Input Requirements/Documents

| Reference (Reference as listed in ch. ”References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Function Requirements”) |
| --- | --- | --- | --- |
| Feature Requirements |  |  |  |
|  | <Example: id + title of relevant Feature Docs> | <Example: “Requirements of Feature …”> | <Note: If you reference a requirement in this column, that requirement should have a trace link in its “Source”/”Source Req.” attribute field pointing back to the input requirement (or to a requirement inside the input document) given in this table row> |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system shall be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |
|  | Air Register Life Cycle from IP-0114 | Registers shall withstand at least 7000 cycles. |  |
|  | Air Register Applied Loads from IP-0117 | Register shall withstand loads of 100N and remain functional. |  |
|  | Personalizable | EM Registers shall be personalizable. (PPP and/or Preset) |  |
|  | HMI Feedback | EM Registers shall give visual feedback to the User through the HMI. |  |
|  | Registers remember users last chosen position Memory | EM Registers shall remember Users last chosen registers directions. |  |
|  | HVAC Vent Air Flow Direction Interface | EM Registers shall control the direction of air exiting the Registers by interfacing with the HMI. |  |
|  | Fast and Accurate | EM Registers shall provide fast and accurate control of the air vents. |  |
|  | Electrically Actuated | EM Registers has electrically actuated and not human actuated air vents. |  |

Table 7: Input Requirements/Documents

#### Assumptions

No assumptions specified for this function.

### Function Scope

The  – “Provide Display Feedback Positions” function is called by the following functions:

- – “Display Visual Feedback”

Figure 2: Activity Diagram of   “Display Visual Feedback” calling   “Provide Display Feedback Positions”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| RegistersPositionsFeedback | Register sensor position feedback to HMI Received from: - Detect Register Feedback Positions |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| userFeedback | User feedback sent to the HMI visual output |

#### Logical Parameters

Not supported by MagicDraw report generation.

| Parameter Name | Description |
| --- | --- |
| <(Mandatory) Word reference to the “Logical Parameters” name bookmark in the Data Dictionary> | <(Optional) Word reference to the “Logical Parameters” description bookmark in the Data Dictionary> |

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts

No diagrams internal to function specified.

Figure 4: State Machine of Function A

#### Activity Diagrams

No diagrams internal to function specified.

Figure 5: Activity Diagram of Function A

#### Sequence Diagrams

No diagrams internal to function specified.

Figure 6: Sequence Diagram of Function A

#### Decision Tables

### Function Requirements

#### Functional Requirements

##### Normal Operation

Provide Display Feedback Positions

When subsystem function receives the signal 'Register Positions Feedback', it shall forward the signal "UserFeedback' to the HMI display

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. | - HMI System Functionality - Display Visual Feedback |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

# Open Concerns

| ID | Concern Description | e-Tracker / Reference | Responsible | Status | Solution |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |

Table 8: Open Concerns (Not supported by MagicDraw report generation.)

# Revision History

| Rev. (revision) | Date | Description | Approved by | Responsible |
| --- | --- | --- | --- | --- |
| FS1 | 2021-03-04 | 3/4/2021 added revision level to generated documents |  | mimhof4 |

## Template Revisions

| Version | Rev. | Date | Description | Responsible |
| --- | --- | --- | --- | --- |
| 1 | 0 | 2016-02-26 | Initial version, derived form FDS | Jbaden1 |
| 1 | 1 | 2016-02-26 | Word properties corrected | Jbaden1 |
| 1 | 2 | 2016-03-10 | Clean up of ocument meta data (Word properties) | Jbaden1 |
| 1 | 3 | 2016-03-22 | - Footer formating corrected (Issue 19) - “Constraints” chapter renamed to “Input Requirements” (Issue 20) | Jbaden1 |
| 1 | 4 | 2016-04-20 | - Broken Wiki links repaired | Jbaden1 |
| 2 | 0 | 2016-06-10 | - Document metadata adapted. Prepared for new macros - DTC table removed - HMI function added as a chapter (details still to be refined) - Signal / Parameter IDs column deleted interface tables | Jbaden1 |
| 2 | 1 | 2016-07-14 | - Converted to SysML diagrams - HMI section further elaborated - Template version added to footer - Dedicated Startup / Shutdown sections removed (only hints added) - Data Dictionary reworked and Signal / Parameter IDs column re-introduced | Jbaden1 |
| 2 | 2 | 2016-12-07 | - Minor formatting changes | Jbaden1 |
| 3 |  |  | Skipped to synchronize with Specification_Macros.dotm |  |
| 4 |  |  |  |  |
| 5 | 0 | 2017-01-13 | - Meta data updated for specification macros, version 3.1 - SW Unit chapter removed for the time being - Green boxes added for user hints | Jbaden1 |
| 5 | 1 | 2017-01-18 | - Some additional hints. - Hyperlinks highlighted in hints | Jbaden1 |
| 6 | 0 | 2017-04-28 | - Editorial change. Hints added to chapter 4.1.4 - Chapter “Traceability Matrix” removed | Jbaden1 |
| 6 | 0 | 2018-04-28 | - CR69/63: New chapters added for Functional Safety (FTTI and Technical Safety Requirements) - CR53: New coversheet + additional meta-data - CR76: merge sections for configuration and for calibration parameters into one on Function Level | Jbaden1 |
| 6 | 0 | 2018-08-06 | - CR66: Fix version numbering in footer of Function Spec | Jbaden1 |
| 6 | 0 | 2018-09-28 | - Broken links to RE Wiki repaired | Jbaden1 |
| 6 | 0 | 2018-10-31 | - Minor corrections on cover sheet and in footer to be more GIS compliant and VSEM aligned - “Overview” and “Description” exchanged in headings (following common sense) | Jbaden1 |
| 6 | 0 | 2018-11-12 | - Explanatory text in Variants” section revised - Functional Safety modifications as agreed with FuSa core team (Baseline: November 2018 Dearborn On-Site) | Jbaden1 |
| 6 | 0b | 2020-02-10 | Bugfix release: - Chapter “Decomposed FSRs” renamed to Functional Safety Requirements”. ASIL decomposition table removed, no longer supported on Function Level. Got also be corrupted, when imported to VSEM - Refinement of FSRs no longer supported by Function Specification (as requested by Functional Safety team). FSR chapter just forwards FSRs from the Feature Docs 1:1 to the Implemented Function(s). | Jbaden1 |
| 6 | 1a | 2019-01-02 | - Editorial changes (in “Variants” section) | Jbaden1 |
| 6 | 1a | 2019-01-21 | - Template Id set to 2 | Jbaden1 |
| 6 | 1a | 2019-03-22 | - Chapter “Decomposed FSRs” renamed to Functional Safety Requirements” A new chapter “ASIL Decomposition of of Functional Safety Requirements” added as a subsection to that chapter. | Jbaden1 |
| 6 | 1a | 2019-04-05 | - Some wording in ASIL decomposition table modified. Description of fields in that table improved. | Jbaden1 |
| 6 | 1a | 2019-04-05 | - ASIL decomposition table removed (ASIL decomposition only allowed on Feature Level or on Component Level in the FIS or ECU Functional Spec) | Jbaden1 |
| 6 | 1a | 2019-07-02 | - "Important" box added on cover sheet which points to the macros - Chapters “References” and “Glossary” moved back up to section “Introduction - Chapter “Inputs Requirements” reworked | Jbaden1 |
| 6 | 1a | 2019-09-10 | - Chapter 4.1.4 has now one section per modeling technique again. This is to allow more intuitive tailoring of the section (driven by AV team request). | Jbaden1 |
| 6 | 1a | 2019-09-10 | - Copyright notice shortened and moved to cover sheet and added to footer (to be compliant with Ford copyright guidelines) - Term “Disclaimer” no longer used for what is actually only a copyright notice | Jbaden1 |
| 6 | 1a | 2019-05-11 | - Per FuSa core team request: Subsection “Functional Safety Requirements” removed completely from Function Spec. FSRs are only captured in the Feature Doc - Chapter “Input Requirements” renamed to “Input Requirements/Documents” | Jbaden1 |
| 6 | 1a | 2019-12-05 | - Upstream Documents section added to “Input Requirements/Documents” table - Custom style table formatting removed - Per FuSa core team request: Subsection “Functional Safety Requirements” to remain in (but no ASIL Decomposition in Function Spec) | Jbaden1 |
| 6 | 1a | 2019-12-09 | - Term “Upstream Documents” replaced by “Feature Requirements” in “Input Requirements/Documents” table - ASIL Decomposition table replaced by a version, which get not corrupted during VSEM import. | Jbaden1 |
| 6 | 1a | 2019-12-10 | - Refinement of FSRs no longer supported by Function Specification (as requested by Functional Safety team). FSR chapter just forwards FSRs from the Feature Docs 1:1 to the Implemented Function(s). | Jbaden1 |
| 6 | 1a | 2019-12-10 | Minor changes made to enable use of the function group spec for specification of system services (system service spec is to be removed): - Chapter “Functional Architecture” renamed to “Functional Decomposition and Architecture” - Functional Decomposition diagram added to renamed chapter. | Jbaden1 |
| 6 | 1a | 2020-02-12 | Minor rewording of hint for FSR table after review by FuSa team. | Jbaden1 |
| 6 | 1a | 2020-03-09 | - Missing doc property “LatestSigMappingID” and “LatestAisInterfaceID” added - doc property “CopyrightDate” re-formatted to text and copyright date field in footer corrected - Version numbering re-initialized as 0.1 - Init value of version/revision date set to “yyyy/mm/dd” instead of “yyyy-mm-dd” to be in line with the “Edit Document Property” dialog - Copyright date field on cover sheet corrected | Jbaden1 |

# Appendix

## Data Dictionary

### Logical Signals

### Logical Parameters

### Encoding Types

ButtonPressdLHS_D_Stat

Button on left hand side for presets and mode man selections

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | 0x0 nonePressed | Button pressed discrete signal for nonepressed event |
|  | 0x1 onBodyLHS | Button pressed discrete signal for on body LHS selection |
|  | 0x2 offBodyLHS | Button pressed discrete signal for off body LHS selection |
|  | 0x3 presetOneLHS | Button pressed discrete signal for preset one LHS selection |
|  | 0x4 presetTwoLHS | Button pressed discrete signal for preset two LHS selection |
|  | 0x5 presetOneSaveLHS | Button pressed discrete signal for preset one saved LHS selection |
|  | 0x6 presetTwoSaveLHS | Button pressed discrete signal for preset two saved LHS selection |
|  | 0x7 cycle_8_LHS | Button pressed discrete signal for cycling figure 8 LHS selection |
|  | 0x8 cycle_C_LHS | Button pressed discrete signal for closed vanes LHS selection |
|  | 0x9 cycle_O_LHS | Button pressed discrete signal for open vanes LHS selection |
|  | 0x10 cycle_--_LHS | Button pressed discrete signal for cycling horizontal vanes LHS selection |
|  | 0x11 cycle_¦_LHS | Button pressed discrete signal for cycling vertical vanes LHS selection |
| Unit |  |  |

Table: Encoding Details of ButtonPressdLHS_D_Stat

SensFdbkLHIBHoriz

Sense left hand horizontal inboard register vane positions and forward to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of SensFdbkLHIBHoriz

Register_RHIB

Register location passenger inboard

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Register_RHIB_Horizontal | Logical signal for register RHIB horizontal position |
|  | Register_RHIB_Vertical | Logical signal for register RHIB vertical position |
|  | NoChange | Logical signal for register RHIB no change position |
|  | ManualChange | Logical signal for register RHIB manual change position |
|  | FeatureChange | Logical signal for register RHIB Feature change position |
|  | Open | Logical signal for register RHIB open position |
|  | Close | Logical signal for register RHIB close position |
| Unit |  |  |

Table: Encoding Details of Register_RHIB

SensorFeedback_LHOB

Register sensor position feedback to HMI for Left hand outboard register

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | SensorFeedback_LHOB_Horizontal | Logical signal register position sensor feedback LHOB horizontal |
|  | SensorFeedBack_LHOB_Vertical | Logical signal register position sensor feedback LHOB vertical |
| Unit |  |  |

Table: Encoding Details of SensorFeedback_LHOB

ActiveButtonsLHS

Feedback active button options for the left side of the vehicle

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | NonePressed | Logical signal active button LHS for none pressed |
|  | OnBodyLHS | Logical signal active button LHS for on body |
|  | OffBodyLHS | Logical signal active button LHS for off body |
|  | PresetOneLHS | Logical signal active button LHS for preset one |
|  | PresetTwoLHS | Logical signal active button LHS for preset two |
|  | PresetOneSaveLHS | Logical signal active button LHS for preset one save |
|  | PresetTwoSaveLHS | Logical signal active button LHS for preset two save |
|  | Cycle_8_LHS | Logical signal active button LHS for cycling figure 8 |
|  | Cycle_C_LHS | Logical signal active button LHS for close |
|  | Cycle_O_LHS | Logical signal active button LHS for open |
|  | Cycle_--_LHS | Logical signal active button LHS for cycling horizontal |
|  | Cycle_¦_LHS | Logical signal active button LHS for cycling vertical |
| Unit |  |  |

Table: Encoding Details of ActiveButtonsLHS

SensorFeedBack_RHOB_Vertical

Register sensor position feedback to HMI for right hand outboard register vertical voltage

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal register position sensor feedback RHOB vertical voltage percentage |
| Unit |  |  |

Table: Encoding Details of SensorFeedBack_RHOB_Vertical

ActuatorCommand_RHIB

Command to move registers in horizontal and vertical directions

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | ActuatorCommand_RHIB_Horizontal | Logical signal RHIB actuator command horizontal |
|  | ActuatorCommand_RHIB_Vertical | Logical signal RHIB actuator command vertical |
| Unit |  |  |

Table: Encoding Details of ActuatorCommand_RHIB

SensFdbkRHIBHoriz

Sense right hand horizontal inboard register vane positions and forward to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of SensFdbkRHIBHoriz

ActCmdLHIBHoriz

Signal from RCCM to left hand inboard horizontal vane register actuator

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of ActCmdLHIBHoriz

ActiveButtonsRHS

Feedback for active button options for the right side of the vehicle

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | NonePressed | Logical signal active button RHS for none pressed |
|  | OnBodyLHS | Logical signal active button RHS for on body |
|  | OffBodyLHS | Logical signal active button RHS for off body |
|  | PresetOneLHS | Logical signal active button RHS for preset one |
|  | PresetTwoLHS | Logical signal active button RHS for preset two |
|  | PresetOneSaveLHS | Logical signal active button RHS for preset one save |
|  | PresetTwoSaveLHS | Logical signal active button RHS for preset two save |
|  | Cycle_8_LHS | Logical signal active button RHS for cycling figure 8 |
|  | Cycle_C_LHS | Logical signal active button RHS for close |
|  | Cycle_O_LHS | Logical signal active button RHS for open |
|  | Cycle_--_LHS | Logical signal active button RHS for cycling horizontal |
|  | Cycle_¦_LHS | Logical signal active button RHS for cycling vertical |
| Unit |  |  |

Table: Encoding Details of ActiveButtonsRHS

ActCmdRHOBVert

Signal from RCCM to right hand outboard vertical vane register actuator

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of ActCmdRHOBVert

LHUpOBRegVert_Deg_Actl

Position of left hand outboard register vertical vanes from HMI and communicated to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of LHUpOBRegVert_Deg_Actl

Register_RHOB_Horizontal

Register airflow direction up or down

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Percentage | Logical signal for register RHOB horizontal percentage change position horizontal |
| Unit |  |  |

Table: Encoding Details of Register_RHOB_Horizontal

Sensed_Actuator_Position

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of Sensed_Actuator_Position

RgstrSetIbl_D_Stat

Signal from HMI to RCCM that provides enumerations of no change, manual change, feature manual change, feature button change, open, and close

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | 0x0 noChange | Implementation signal for LHIB register status for no change |
|  | 0x1 manualChange | Implementation signal for LHIB register status for manual change |
|  | 0x2 featureManualChange | Implementation signal for LHIB register status for feature manual change |
|  | 0x3 featureButtonChange | Implementation signal for LHIB register status for feature button change |
|  | 0x4 open | Implementation signal for LHIB register status for open |
|  | 0x5 close | Implementation signal for LHIB register status for close |
| Unit |  |  |

Table: Encoding Details of RgstrSetIbl_D_Stat

RHUpOBVeHMIFB_Deg_Actl

Feedback from RCCM to HMI, Position of right hand outboard register vertical Vanes: 0% to 100%

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of RHUpOBVeHMIFB_Deg_Actl

ActCmdLHOBHorizLin

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of ActCmdLHOBHorizLin

RgstrSetIbr_D_Stat

Signal from HMI to RCCM that provides enumerations of no change, manual change, feature manual change, feature button change, open, and close for right hand upper outboard register

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | 0x0 noChange | implementation signal for RHOB Reg position no change |
|  | 0x1 manualChange | implementation signal for RHOB Reg position manual change |
|  | 0x2 featureManualChange | implementation signal for RHOB Reg position feature manual change |
|  | 0x3 featureButtonChange | implementation signal for RHOB Reg position feature button change |
|  | 0x4 open | implementation signal for RHOB Reg position open |
|  | 0x5 close | implementation signal for RHOB Reg position closed |
| Unit |  |  |

Table: Encoding Details of RgstrSetIbr_D_Stat

LHUpIBRegVert_Deg_Actl

Position of left hand inboard register vertical vanes from HMI and communicated to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of LHUpIBRegVert_Deg_Actl

Registers'PostionsHMIFeedback

Register position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | HMIFeedback_LHOB | Logical signal HMI position feedback LHOB |
|  | HMIFeedback_LHIB | Logical signal HMI position feedback LHIB |
|  | HMIFeedback_RHIB | Logical signal HMI position feedback RHIB |
|  | HMIFeedback_RHOB | Logical signal HMI position feedback RHOB |
| Unit |  |  |

Table: Encoding Details of Registers'PostionsHMIFeedback

SensFdbkLHOBVert

Sense left hand vertical outboard register vane positions and forward to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of SensFdbkLHOBVert

SensorFeedback_RHOB

Register sensor position feedback to HMI for right hand outboard register

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | SensorFeedback_RHOB_Horizontal | Logical signal register position sensor feedback RHOB horizontal |
|  | SensorFeedBack_RHOB_Vertical | Logical signal register position sensor feedback RHIB vertical |
| Unit |  |  |

Table: Encoding Details of SensorFeedback_RHOB

ActCmdLHIBHorizLin

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of ActCmdLHIBHorizLin

SensorFeedBack_LHIB_Vertical

Register sensor position feedback to HMI for Left hand inboard register voltage

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal register position sensor feedback LHIB vertical voltage percentage |
| Unit |  |  |

Table: Encoding Details of SensorFeedBack_LHIB_Vertical

Register_RHIB_Horizontal

Register airflow direction up or down

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Percentage | Logical signal for register RHIB horizontal percentage change position horizontal |
| Unit |  |  |

Table: Encoding Details of Register_RHIB_Horizontal

ActCmdLHIBVert

Signal from RCCM to left hand inboard vertica vane register actuator

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of ActCmdLHIBVert

ButtonPressdRHS_D_Stat

Button on right hand side for preset and mode man register movement selections

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | 0x0 nonePressed | Button pressed discrete signal for none pressed event |
|  | 0x1 onBodyRHS | Button pressed discrete signal for on body RHS selection |
|  | 0x2 offBodyRHS | Button pressed discrete signal for off body RHS selection |
|  | 0x3 presetOneRHS | Button pressed discrete signal for preset one RHS selection |
|  | 0x4 presetTwoRHS | Button pressed discrete signal for preset two RHS selection |
|  | 0x5 presetOneSaveRHS | Button pressed discrete signal for preset one save RHS selection |
|  | 0x6 presetTwoSaveRHS | Button pressed discrete signal for preset two save RHS selection |
|  | 0x7 cycle_8_RHS | Button pressed discrete signal for cycling in figure 8 RHS selection |
|  | 0x8 cycle_C_RHS | Button pressed discrete signal for close RHS selection |
|  | 0x9 cycle_O_RHS | Button pressed discrete signal for open RHS selection |
|  | 0x10 cycle_--_RHS | Button pressed discrete signal for horizontal cycling RHS selection |
|  | 0x11 cycle_¦_RHS | Button pressed discrete signal for vertical cycling RHS selection |
| Unit |  |  |

Table: Encoding Details of ButtonPressdRHS_D_Stat

HMIFeedback_RHIB_Vertical

Register position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of HMIFeedback_RHIB_Vertical

LHUpIBVeHMIFB_Deg_Actl

Feedback from RCCM to HMI, Position of left hand inboard vertical Vanes: 0% to 100%

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of LHUpIBVeHMIFB_Deg_Actl

SensorFeedback_RHIB_Horizontal

Register sensor position feedback to HMI for right hand inboard register horizontal voltage

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal register position sensor feedback RHIB horizontal voltage percentage |
| Unit |  |  |

Table: Encoding Details of SensorFeedback_RHIB_Horizontal

LHUpOBRegHorz_Deg_Actl

Position of left hand outboard register horizontal vanes from HMI and communicated to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of LHUpOBRegHorz_Deg_Actl

RHUpIBVeHMIFB_Deg_Actl

Feedback from RCCM to HMI, Position of right hand inboard register vertical Vanes: 0% to 100%

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of RHUpIBVeHMIFB_Deg_Actl

SensFdbkLHIBVert

Sense left hand vertical inboard register vane positions and forward to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of SensFdbkLHIBVert

SensorFeedback_LHIB_Horizontal

Register sensor position feedback to HMI for left hand inboard register voltage

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal register position sensor feedback LHIB horizontal voltage percentage |
| Unit |  |  |

Table: Encoding Details of SensorFeedback_LHIB_Horizontal

SensFdbkRHOBVert

Sense right hand vertical inboard register vane positions and forward to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of SensFdbkRHOBVert

userInput

User input from HMI screen display

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | HMI Touch Input | Logical signal that provides user input from HMI Touch screen |
| Unit |  |  |

Table: Encoding Details of userInput

RHUpIBHoHMIFB_Deg_Actl

Feedback from RCCM to HMI, Position of right hand inboard register Horizontal Vanes: 0% to 100%

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of RHUpIBHoHMIFB_Deg_Actl

ActCmdRHIBHoriz

Signal from RCCM to right hand inboard horizontal vane register actuator

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of ActCmdRHIBHoriz

Register_LHIB_Horizontal

Register up or down airflow direction

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Percentage | Logical signal for register LHIB horizontal percentage change position horizontal |
| Unit |  |  |

Table: Encoding Details of Register_LHIB_Horizontal

SensFdbkRHOBHoriz

Sense right hand horizontal outboard register vane positions and forward to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of SensFdbkRHOBHoriz

HMIFeedback_LHIB_Vertical

Register position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of HMIFeedback_LHIB_Vertical

HMIFeedback_RHOB

Register position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | horizontal feedback | Logical signal HMI feedback RHOB horizontal |
|  | vertical feedback | Logical signal HMI feedback RHOB vertical |
|  | Position | Logical signal HMI feedback RHOB position |
|  | Close | Logical signal HMI feedback RHOB close |
| Unit |  |  |

Table: Encoding Details of HMIFeedback_RHOB

ButtonPressedRHS

Move register commands for button options for the right side of the vehicle

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | NonePressed | Logical signal button pressed RHS for none pressed |
|  | OnBodyRHS | Logical signal button pressed RHS on body |
|  | OffBodyRHS | Logical signal button pressed RHS for off body |
|  | PresetOneRHS | Logical signal button pressed RHS for preset one |
|  | PresetTwoRHS | Logical signal button pressed RHS for preset two |
|  | PresetOneSaveRHS | Logical signal button pressed RHS for preset one save |
|  | PresetTwoSaveRHS | Logical signal button pressed RHS for preset two save |
|  | Cycle_8_RHS | Logical signal button pressed RHS for cycling figure 8 |
|  | Cycle_C_RHS | Logical signal button pressed RHS for close |
|  | Cycle_O_RHS | Logical signal button pressed RHS for open |
|  | Cycle_--_RHS | Logical signal button pressed RHS for cycling horizontal |
|  | Cycle_¦_RHS | Logical signal button pressed RHS for cycling vertical |
| Unit |  |  |

Table: Encoding Details of ButtonPressedRHS

SensorFeedback_RHIB

Register sensor position feedback to HMI for right hand inboard register

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | SensorFeedback_RHIB_Horizontal | Logical signal register position sensor feedback RHIB horizontal |
|  | SensorFeedBack_RHIB_Vertical | Logical signal register position sensor feedback RHIB vertical |
| Unit |  |  |

Table: Encoding Details of SensorFeedback_RHIB

LHUpOBVeHMIFB_Deg_Actl

Feedback from RCCM to HMI, Position of left hand outboard vertical register Vanes: 0% to 100%

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of LHUpOBVeHMIFB_Deg_Actl

ActCmdLHIBVertLin

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of ActCmdLHIBVertLin

SensFdbkRHIBVert

Sense right hand vertical inboard register vane positions and forward to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of SensFdbkRHIBVert

RHUpIBRegHorz_Deg_Actl

Position of right hand inboard register horizontal vanes from HMI and communicated to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of RHUpIBRegHorz_Deg_Actl

LHUpOBRegHMIFB_D_Stat

Signal from RCCM to HMI regarding position and closure status of left hand outboard register

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | 0x0 position | Signal for feedback to HMI for LHOB vane position |
|  | 0x1 close | Signal for HMI feedback of LHOB vane closed position |
| Unit |  |  |

Table: Encoding Details of LHUpOBRegHMIFB_D_Stat

ActuatorCommand_LHIB_Horizontal

Left hand inboard register voltage for up down movement

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal for LHIB horizontal percentage change |
| Unit |  |  |

Table: Encoding Details of ActuatorCommand_LHIB_Horizontal

Register_LHOB_Horizontal

Register up or down airflow direction

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Percentage | Logical signal for register LHOB horizontal percentage change position horizontal |
| Unit |  |  |

Table: Encoding Details of Register_LHOB_Horizontal

ActuatorCommand_LHOB_Horizontal

Left hand inboard register voltage for up down movement

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal for LHOB horizontal percentage change |
| Unit |  |  |

Table: Encoding Details of ActuatorCommand_LHOB_Horizontal

RHUpIBRegVert_Deg_Actl

Position of right hand inboard register vertical vanes from HMI and communicated to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of RHUpIBRegVert_Deg_Actl

ActCmdLHOBHoriz

Signal from RCCM to left hand inboard horizontal vane register actuator

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of ActCmdLHOBHoriz

LHUpIBHoHMIFB_Deg_Actl

Feedback from RCCM to HMI, Position of left hand inboard Horizontal Vanes: 0% to 100%

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of LHUpIBHoHMIFB_Deg_Actl

ActuatorCommand_LHOB

Register move command horizontal and vertical directions

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | ActuatorCommand_LHOB_Horizontal | Logical signal LHOB actuator command horizontal |
|  | ActuatorCommand_LHOB_Vertical | Logical signal LHOB actuator command vertical |
| Unit |  |  |

Table: Encoding Details of ActuatorCommand_LHOB

RHUpIBRegHMIFB_D_Stat

Signal from RCCM to HMI regarding position and closure status of right hand inboard register

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | 0x0 position | Signal for RHIB vane position feedback to HMI |
|  | 0x1 close | Signal for RHIB vane closed position feedback to HMI |
| Unit |  |  |

Table: Encoding Details of RHUpIBRegHMIFB_D_Stat

Register_LHOB

Register location driver outboard

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Register_LHOB_Vertical | Logical signal for register LHOB vertical position |
|  | Register_LHOB_Horizontal | Logical signal for register LHOB horizontal position |
|  | NoChange | Logical signal for register LHOB no changel position |
|  | ManualChange | Logical signal for register LHOB manual change position |
|  | FeatureChange | Logical signal for register LHOB Feature change position |
|  | Open | Logical signal for register LHOB open position |
|  | Close | Logical signal for register LHOB close position |
| Unit |  |  |

Table: Encoding Details of Register_LHOB

ActuatorCommand_RHOB

Register movement commands for horizontal and vertical directions

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | ActuatorCommand_RHOB_Horizontal | Logical signal RHOB actuator command horizontal |
|  | ActuatorCommand_RHOB_Vertical | Logical signal RHOB actuator command vertical |
| Unit |  |  |

Table: Encoding Details of ActuatorCommand_RHOB

ActuatorCommand_LHIB

Register move commands horizontal, vertical

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | ActuatorCommand_LHIB_Horizontal | Logical signal LHIB actuator command horizontal |
|  | ActuatorCommand_LHIB_Vertical | Logical signal LHIB actuator command vertical |
| Unit |  |  |

Table: Encoding Details of ActuatorCommand_LHIB

SensorFeedBack_LHOB_Vertical

Register sensor position feedback to HMI for Left hand inboard register vertical voltage

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal register position sensor feedback LHOB vertical voltage percentage |
| Unit |  |  |

Table: Encoding Details of SensorFeedBack_LHOB_Vertical

LHUpIBRegHorz_Deg_Actl

Position of left hand inboard register horizontal vanes from HMI and communicated to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of LHUpIBRegHorz_Deg_Actl

RCCM to Actuator

Signals sent from RCCM to register actuator

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | ActCmdLHOBHoriz | Implementation signal for ActCmdLHOBHoriz |
|  | ActCmdLHOBVert | Implementation signal for ActCmdLHOBVert |
|  | ActCmdLHIBHoriz | Implementation signal for ActCmdLHIBHoriz |
|  | ActCmdLHIBVert | Implementation signal for ActCmdLHIBVert |
|  | ActCmdRHIBHoriz | Implementation signal for ActCmdRHIBHoriz |
|  | ActCmdRHIBVert | Implementation signal for ActCmdRHIBVert |
|  | ActCmdRHOBHoriz | Implementation signal for ActCmdRHOBHoriz |
|  | ActCmdRHOBVert | Implementation signal for ActCmdRHOBVert |
|  | LIN |  |
|  | Hardwire |  |
| Unit |  |  |

Table: Encoding Details of RCCM to Actuator

ActuatorCommand_RHOB_Vertical

Right hand outboard register voltage for left right movement

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal for RHOB vertical percentage change |
| Unit |  |  |

Table: Encoding Details of ActuatorCommand_RHOB_Vertical

RHUpOBHoHMIFB_Deg_Actl

Feedback from RCCM to HMI, Position of right hand outboard register Horizontal Vanes: 0% to 100%

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of RHUpOBHoHMIFB_Deg_Actl

ActiveButtonLHS_D_Stat

Feedback signal from RCCM to HMI that provides the following enumerations of signal content:

*NonePressed

*OnBodyLHS

*OffBodyLHS

*PresetOneLHS

*PresetTwoLHS

*PresetOneSaveLHS

*PresetTwoSaveLHS

*Cycle_8_LHS

*Cycle_C_LHS

*Cycle_O_LHS

*Cycle_--_LHS

*Cycle_I_LHS

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | 0x0 nonePressed | Button pressed discrete signal for none pressed LHS selection |
|  | 0x1 onBodyLHS | Button pressed discrete signal for on body LHS selection |
|  | 0x2 offBodyLHS | Button pressed discrete signal for off body LHS selection |
|  | 0x3 presetOneLHS | Button pressed discrete signal for preset one LHS selection |
|  | 0x4 presetTwoLHS | Button pressed discrete signal for preset two LHS selection |
|  | 0x6 presetTwoSaveLHS | Button pressed discrete signal for preset two saved LHS selection |
|  | 0x5 presetOneSaveLHS | Button pressed discrete signal for preset one saved LHS selection |
|  | 0x5 cycle_8_LHS | Button pressed discrete signal for cycling figure 8 vanes LHS selection |
|  | 0x6 cycle_C_LHS | Button pressed discrete signal for closed vanes LHS selection |
|  | 0x7 cycle_O_LHS | Button pressed discrete signal for open vanes LHS selection |
|  | 0x8 cycle_--_LHS | Button pressed discrete signal for cycling horizontal vanes LHS selection |
|  | 0x9 cycle_¦_LHS | Button pressed discrete signal for cycling vertical vanes LHS selection |
| Unit |  |  |

Table: Encoding Details of ActiveButtonLHS_D_Stat

HMIFeedback_LHOB

Register position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Horizontal Feedback | Logical signal HMI feedback LHOB horizontal |
|  | Vertical Feedback | Logical signal HMI feedback LHOB vertical |
|  | Position | Logical signal HMI feedback LHOB position |
|  | Close | Logical signal HMI feedback LHOB close |
| Unit |  |  |

Table: Encoding Details of HMIFeedback_LHOB

Register_RHOB

Register location passenger outboard

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Register_RHOB_Horizontal | Logical signal for register RHOB horizontal position |
|  | Register_RHOB_Vertical | Logical signal for register RHOB vertical position |
|  | NoChange | Logical signal for register RHOB no change position |
|  | ManualChange | Logical signal for register RHOB manual change position |
|  | FeatureChange | Logical signal for register RHOB feature change position |
|  | Open | Logical signal for register RHOB open position |
|  | Close | Logical signal for register RHOB close position |
| Unit |  |  |

Table: Encoding Details of Register_RHOB

Actuate Register Command

Signal requesting actuation of register position changes

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | ActuatorCommand_LHOB | Register actuator command LHOB |
|  | ActuatorCommand_LHIB | Register actuator command LHIB |
|  | ActuatorCommand_RHIB | Register actuator command RHIB |
|  | ActuatorCommand_RHOB | Register actuator command RHOB |
| Unit |  |  |

Table: Encoding Details of Actuate Register Command

ActuatorCommand_RHIB_Horizontal

Right hand inboard register voltage for up down movement

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal for RHIB horizontal percentage change |
| Unit |  |  |

Table: Encoding Details of ActuatorCommand_RHIB_Horizontal

SensFdbkLHOBHoriz

Sense left hand horizontal outboard register vane positions and forward to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of SensFdbkLHOBHoriz

LHUpIBRegHMIFB_D_Stat

Signal from RCCM to HMI regarding position and closure status of left hand inboard register

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | 0x0 position | Signal for HMI feedback position |
|  | 0x1 close | Signal for feedback to HMI for close position |
| Unit |  |  |

Table: Encoding Details of LHUpIBRegHMIFB_D_Stat

ActCmdLHOBVert

Signal from RCCM to left hand outboard vertical vane register actuator

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of ActCmdLHOBVert

HMIFeedback_LHOB_Horizontal

Register position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of HMIFeedback_LHOB_Horizontal

ActCmdRHOBHoriz

Signal from RCCM to right hand outboard horizontal vane register actuator

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of ActCmdRHOBHoriz

RHUpOBRegVert_Deg_Actl

Position of right hand outboard vertical vanes from HMI and communicated to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of RHUpOBRegVert_Deg_Actl

HMIFeedback_RHOB_Vertical

Register position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of HMIFeedback_RHOB_Vertical

ActuatorCommand_RHIB_Vertical

Right hand inboard register voltage for left right movement

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal for RHIB vertical percentage change |
| Unit |  |  |

Table: Encoding Details of ActuatorCommand_RHIB_Vertical

HMIFeedback_RHIB_Horizontal

Register position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of HMIFeedback_RHIB_Horizontal

Actuator to RCCM

Signals sent from register actuator to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | SensFdbkLHOBHoriz | Implementation signal for SensFdbkLHOBHoriz |
|  | SensFdbkLHOBVert | Implementation signal for SensFdbkLHOBVert |
|  | SensFdbkLHIBHoriz | Implementation signal for SensFdbkLHIBHoriz |
|  | SensFdbkLHIBVert | Implementation signal for SensFdbkLHIBVert |
|  | SensFdbkRHIBHoriz | Implementation signal for SensFdbkRHIBHoriz |
|  | SensFdbkRHIBVert | Implementation signal for SensFdbkRHIBVert |
|  | SensFdbkRHOBHoriz | Implementation signal for SensFdbkRHOBHoriz |
|  | SensFdbkRHOBVert | Implementation signal for SensFdbkRHOBVert |
| Unit |  |  |

Table: Encoding Details of Actuator to RCCM

LHUpOBHoHMIFB_Deg_Actl

Feedback from RCCM to HMI, Position of left hand outboard register Horizontal Vanes: 0% to 100%

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of LHUpOBHoHMIFB_Deg_Actl

RHUpOBRegHMIFB_D_Stat

Signal from RCCM to HMI regarding position and closure status of right hand outboard register

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | 0x0 position | Signal for RHOB vane position feedback to HMI |
|  | 0x1 close | Signal for RHOB vane closed position feedback to HMI |
| Unit |  |  |

Table: Encoding Details of RHUpOBRegHMIFB_D_Stat

ActCmdRHIBVert

Signal from RCCM to right hand inboard vertical vane register actuator

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | voltage | voltage property |
| Unit |  |  |

Table: Encoding Details of ActCmdRHIBVert

VehStat

Vehicle status signals used to store register positions when vehicle is on

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | ON | Implementation signal for vehicle status=on |
|  | OFF | Implementation signal for vehicle status=OFF |
|  | ACCESSORY | Implementation signal for vehicle status=ACCESSORY |
| Unit |  |  |

Table: Encoding Details of VehStat

Register_RHOB_Vertical

Register airflow direction left or right

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Percentage | Logical signal for register RHOB horizontal percentage change position vertical |
| Unit |  |  |

Table: Encoding Details of Register_RHOB_Vertical

LHUpperOBReg_D_Stat

Signal from HMI to RCCM that provides enumerations of no change, manual change, feature manual change, feature button change, open, and close

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | 0x0 noChange | Implementation signal for LHOB reg position no change |
|  | 0x1 manualChange | Implementation signal for LHOB reg position manual change |
|  | 0x2 featureManualChange | Implementation signal for LHOB reg position feature manual change |
|  | 0x3 featureButtonChange | Implementation signal for LHOB reg position feature button change |
|  | 0x4 open | Implementation signal for LHOB reg position open |
|  | 0x5 close | Implementation signal for LHOB reg position close |
| Unit |  |  |

Table: Encoding Details of LHUpperOBReg_D_Stat

ButtonPressedLHS

Move register commands for button options for the left side of the vehicle

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | NonePressed | Logical signal button pressed LHS for none pressed |
|  | OnBodyLHS | Logical signal button pressed LHS for on body |
|  | OffBodyLHS | Logical signal button pressed LHS for off body |
|  | PresetOneLHS | Logical signal button pressed LHS for preset one |
|  | PresetTwoLHS | Logical signal button pressed LHS for preset two |
|  | PresetOneSaveLHS | Logical signal button pressed LHS for preset one save |
|  | PresetTwoSaveLHS | Logical signal button pressed LHS for preset two save |
|  | Cycle_8_LHS | Logical signal button pressed LHS for cycling figure 8 |
|  | Cycle_C_LHS | Logical signal button pressed LHS for closed |
|  | Cycle_O_LHS | Logical signal button pressed LHS for open |
|  | Cycle_--_LHS | Logical signal button pressed LHS for cycling horizontal |
|  | Cycle_¦_LHS | Logical signal button pressed LHS for cycling vertical |
| Unit |  |  |

Table: Encoding Details of ButtonPressedLHS

userFeedback

User feedback sent to the HMI visual output

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | HMI Visual Output | logical signal for HMI visual output |
| Unit |  |  |

Table: Encoding Details of userFeedback

SensorFeedback_RHOB_Horizontal

Register sensor position feedback to HMI for right hand outboard register horizontal voltage

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal register position sensor feedback RHOB horizontal voltage percentage |
| Unit |  |  |

Table: Encoding Details of SensorFeedback_RHOB_Horizontal

RegistersPositions

Register location in IP

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Register_LHOB | Logical signal for Register LHOB position |
|  | Register_LHIB | Logical signal for Register LHIB position |
|  | Register_RHIB | Logical signal for Register RHIB position |
|  | Register_RHOB | Logical signal for Register RHOB position |
| Unit |  |  |

Table: Encoding Details of RegistersPositions

RCCM to APIM

Signals sent from RCCM to APIM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | ActiveButtonLHS_D_Stat | Implementation signal from RCCM to APIM for ActiveButtonsLHS_D_Stat |
|  | ActiveButtonRHS_D_Stat | Implementation signal from RCCM to APIM for ActiveButtonsRHS_D_Stat |
|  | LHUpOBRegHMIFB_D_Stat | Implementation signal from RCCM to APIM for LHUpOBRegHMIFB_D_Stat |
|  | LHUpIBRegHMIFB_D_Stat | Implementation signal from RCCM to APIM for LHUpIBRegHMIFB_D_Stat |
|  | RHUpOBRegHMIFB_D_Stat | Implementation signal from RCCM to APIM for RHUpOBRegHMIFB_D_Stat |
|  | RHUpIBRegHMIFB_D_Stat | Implementation signal from RCCM to APIM for RHUpIBRegHMIFBS_D_Stat |
|  | LHUpOBHoHMIFB_Deg_Actl | Implementation signal from RCCM to APIM for LHUpOBHoHMIFB_Deg_Actl |
|  | LHUpOBVeHMIFB_Deg_Actl | Implementation signal from RCCM to APIM for LHUpOBVeHMIFB_Deg_Actl |
|  | LHUpIBHoHMIFB_Deg_Actl | Implementation signal from RCCM to APIM forLHUpIBHoHMIFB_Deg_Actl |
|  | LHUpIBVeHMIFB_Deg_Actl | Implementation signal from RCCM to APIM for LHUpIBVeHMIFB_Deg_Actl |
|  | RHUpIBHoHMIFB_Deg_Actl | Implementation signal from RCCM to APIM for RHUpOBVeHMIFB_Deg_Actl |
|  | RHUpOBVeHMIFB_Deg_Actl | Implementation signal from RCCM to APIM for RHUpOBVeHMIFB_Deg_Actl |
|  | RHUpOBHoHMIFB_Deg_Actl | Implementation signal from RCCM to APIM for RHUpOBHoHMIFB_Deg_Actl |
|  | RHUpIBVeHMIFB_Deg_Actl | Implementation signal from RCCM to APIM forRHUpIBVeHMIFB_Deg_Actl |
| Unit |  |  |

Table: Encoding Details of RCCM to APIM

ActuatorCommand_RHOB_Horizontal

Right hand outboard register voltage for up down movement

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal for RHOB horizontal percentage change |
| Unit |  |  |

Table: Encoding Details of ActuatorCommand_RHOB_Horizontal

HMIFeedback_RHOB_Horizontal

Register position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of HMIFeedback_RHOB_Horizontal

HMIFeedback_LHIB_Horizontal

Register position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of HMIFeedback_LHIB_Horizontal

HMIFeedback_RHIB

Register position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | horizontal feedback | Logical signal HMI feedback RHIB horizontal |
|  | vertical feedback | Logical signal HMI feedback RHIB vertical |
|  | Position | Logical signal HMI feedback RHIB position |
|  | Close | Logical signal HMI feedback RHIB close |
| Unit |  |  |

Table: Encoding Details of HMIFeedback_RHIB

ActuatorCommand_LHOB_Vertical

Left hand outboard register voltage for up down movement

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal for LHOB vertical percentage change |
| Unit |  |  |

Table: Encoding Details of ActuatorCommand_LHOB_Vertical

Register_LHIB_Vertical

Register vane left or right airflow direction

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Percentage | Logical signal for register LHIB horizontal percentage change position vertical |
| Unit |  |  |

Table: Encoding Details of Register_LHIB_Vertical

SensorFeedback_LHOB_Horizontal

Register sensor position feedback to HMI for Left hand outboard register horizontal voltage

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal register position sensor feedback LHOB horizontal voltage percentage |
| Unit |  |  |

Table: Encoding Details of SensorFeedback_LHOB_Horizontal

SensorFeedBack_RHIB_Vertical

Register sensor position feedback to HMI for right hand inboard register vertical voltage

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal register position sensor feedback RHIB vertical voltage percentage |
| Unit |  |  |

Table: Encoding Details of SensorFeedBack_RHIB_Vertical

Register_RHIB_Vertical

Register airflow direction left or right

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Percentage | Logical signal for register RHIB horizontal percentage change position vertical |
| Unit |  |  |

Table: Encoding Details of Register_RHIB_Vertical

Register_LHIB

Register location driver inboard

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Register LHIB Horizontal | Logical signal for register LHIB horizontal position |
|  | Register_LHIB_Vertical | Logical signal for register LHIB vertical position |
|  | NoChange | Logical signal for register LHIB no changel position |
|  | ManualChange | Logical signal for register LHIB no change position |
|  | FeatureChange | Logical signal for register LHIB Feature change position |
|  | Open | Logical signal for register LHIB open position |
|  | Close | Logical signal for register LHIB close position |
| Unit |  |  |

Table: Encoding Details of Register_LHIB

APIM to RCCM

Signals sent from APIM to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | LHUpperOBReg_D_Stat | Implementation signal for LHUpperOBReg_D_Stat |
|  | LHUpperIBReg_D_Stat | Implementation signal for LHUpperIBReg_D_Stat |
|  | RHUpperOBReg_D_Stat | Implementation signal for RHUpperOBReg_D_Stat |
|  | RHUpperIBReg_D_Stat | Implementation signal for RHUpperIBReg_D_Stat |
|  | LHUpOBRegHorz_Deg_Actl | Implementation signal for LHUpOBRegHorz_Deg_Actl |
|  | LHUpOBRegVert_Deg_Actl | Implementation signal for LHUpOBRegVert_Deg_Actl |
|  | LHUpIBRegHorz_Deg_Actl | Implementation signal for LHUpIBRegHorz_Deg_Actl |
|  | LHUpIBRegVert_Deg_Actl | Implementation signal for LHUpIBRegVert_Deg_Actl |
|  | RHUpIBRegHorz_Deg_Actl | Implementation signal for RHUpIBRegHorz_Deg_Actl |
|  | RHUpIBRegVert_Deg_Actl | Implementation signal for RHUpIBRegVert_Deg_Actl |
|  | RHUpOBRegHorz_Deg_Actl | Implementation signal for RHUpOBRegHorz_Deg_Actl |
|  | RHUpOBRegVert_Deg_Actl | Implementation signal for RHUpOBRegVert_Deg_Actl |
|  | ButtonPressdLHS_D_Stat | Implementation signal for ButtonPressdLHS_D_Stat |
|  | ButtonPressdRHS_D_Stat | Implementation signal for ButtonPressdRHS_D_Stat |
| Unit |  |  |

Table: Encoding Details of APIM to RCCM

ActuatorCommand_LHIB_Vertical

Register left hand inboard voltage for vertical movement

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Voltage | Logical signal for LHIB vertical percentage change |
| Unit |  |  |

Table: Encoding Details of ActuatorCommand_LHIB_Vertical

RHUpOBRegHorz_Deg_Actl

Position of right hand outboard horizontal vanes from HMI and communicated to RCCM

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of RHUpOBRegHorz_Deg_Actl

MoveRegisterCommand

Move register commands based on register postion settings or button options

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Registers'Positions | logical signal to move register positions |
|  | ButtonPressedLHS | logical signal to move register positions using button presets for LHS |
|  | ButtonPressedRHS | logical signal to move register positions using button pressed for LHS |
| Unit |  |  |

Table: Encoding Details of MoveRegisterCommand

Clmt_Button_Stat4

Climate button signal

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | None_Pressed | Implementation signal for climate button none pressed |
|  | Front_Power_Pressed | Implementation signal for climate button front power pressed |
| Unit |  |  |

Table: Encoding Details of Clmt_Button_Stat4

RegHomePositions

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Reg_LHIB_Horz_Home |  |
|  | Reg_LHIB_Vert_Home |  |
|  | Reg_LHOB_Horz_Home |  |
|  | Reg_LHOB_Vert_Home |  |
|  | Reg_RHIB_Horz_Home |  |
|  | Reg_RHIB_Vert_Home |  |
|  | Reg_RHOB_Horz_Home |  |
|  | Reg_RHOB_Vert_Home |  |
|  | Reg_LHIB_Horz_Angle_Percentage |  |
|  | Reg_LHIB_Vert_Angle_Percentage |  |
|  | Reg_LHOB_Horz_Angle_Percentage |  |
|  | Reg_LHOB_Vert_Angle_Percentage |  |
|  | Reg_RHIB_Horz_Angle_Percentage |  |
|  | Reg_RHOB_Vert_Angle_Percentage |  |
|  | Reg_RHIB_Vert_Angle_Percentage |  |
|  | Reg_RHOB_Horz_Angle_Percentage |  |
| Unit |  |  |

Table: Encoding Details of RegHomePositions

Register_LHOB_Vertical

Register airflow left or right airflow direction

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Percentage | Logical signal for register LHOB horizontal percentage change position vertical |
| Unit |  |  |

Table: Encoding Details of Register_LHOB_Vertical

vehicleStatus

Vehicle on off status used by register move commands

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | ON_TO_OFF | vehicle status on |
|  | OFF_TO_ON | vehicle status off |
|  | Accessory | vehicle status accessory |
| Unit |  |  |

Table: Encoding Details of vehicleStatus

ActiveButtonRHS_D_Stat

Feedback signal from RCCM to HMI that provides the following enumerations of signal content:

*NonePressed

*OnBodyLHS

*OffBodyLHS

*PresetOneLHS

*PresetTwoLHS

*PresetOneSaveLHS

*PresetTwoSaveLHS

*Cycle_8_LHS

*Cycle_C_LHS

*Cycle_O_LHS

*Cycle_--_LHS

*Cycle_I_LHS

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | 0x0 nonePressed | Button pressed discrete signal for none pressed RHS selection |
|  | 0x1 onBodyRHS | Button pressed discrete signal for on body RHS selection |
|  | 0x2 offBodyRHS | Button pressed discrete signal for off body RHS selection |
|  | 0x3 presetOneRHS | Button pressed discrete signal for preset one RHS selection |
|  | 0x4 presetTwoRHS | Button pressed discrete signal for preset two RHS selection |
|  | 0x6 presetTwoSaveRHS | Button pressed discrete signal for preset one saved RHS selection |
|  | 0x5 cycle_8_RHS | Button pressed discrete signal cycling in figure 8 RHS selection |
|  | 0x5 presetOneSaveRHS | Button pressed discrete signal for preset one saved RHS selection |
|  | 0x6 cycle_C_RHS | Button pressed discrete signal for closed vanes RHS selection |
|  | 0x7 cycle_O_RHS | Button pressed discrete signal open vanes RHS selection |
|  | 0x8 cycle_--_RHS | Button pressed discrete signal for cycling vanes in horizontal direction RHS selection |
|  | 0x9 cycle_¦_RHS | Button pressed discrete signal for cycling vanes in vertical direction RHS selection |
| Unit |  |  |

Table: Encoding Details of ActiveButtonRHS_D_Stat

SensorFeedback_LHIB

Register sensor position feedback to HMI for left hand inboard register

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | SensorFeedback_LHIB_Horizontal | Logical signal register position sensor feedback LHIB horizontal |
|  | SensorFeedBack_LHIB_Vertical | Logical signal register position sensor feedback LHIB vertical |
| Unit |  |  |

Table: Encoding Details of SensorFeedback_LHIB

RgstrSetObr_D_Stat

Signal from HMI to RCCM that provides enumerations of no change, manual change, feature manual change, feature button change, open, and close for right hand upper inboard register

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | 0x0 noChange | implementation signal for RHIB Reg position no change |
|  | 0x1 manualChange | implementation signal for RHIB Reg position manual change |
|  | 0x2 featureManualChange | implementation signal for RHIB Reg position feature manual change |
|  | 0x3 featureButtonChange | implementation signal for RHIB Reg position feature button change |
|  | 0x4 open | implementation signal for RHIB Reg position open |
|  | 0x5 close | implementation signal for RHIB Reg position close |
| Unit |  |  |

Table: Encoding Details of RgstrSetObr_D_Stat

RegistersPositionsFeedback

Register sensor position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | SensorFeedback_LHOB | Logical signal register position sensor feedback LHOB |
|  | SensorFeedback_LHIB | Logical signal register position sensor feedback LHIB |
|  | SensorFeedback_RHIB | Logical signal register position sensor feedback RHIB |
|  | SensorFeedback_RHOB | Logical signal register position sensor feedback RHOB |
| Unit |  |  |

Table: Encoding Details of RegistersPositionsFeedback

MoveAirFlowCommand

Move register commands from user input or another feature

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | MoveRegisterCommand | Logical signal for move airflow command to move registers |
|  | userInput | Logical signal for move airflow command for user input to move registers |
| Unit |  |  |

Table: Encoding Details of MoveAirFlowCommand

HMIFeedback_LHIB

Register position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
|  | Horizontal feedback | Logical signal HMI feedback LHIB horizontal |
|  | Vertical feedback | Logical signal HMI feedback LHIB vertical |
|  | Position | Logical signal HMI feedback LHIB position |
|  | Close | Logical signal HMI feedback LHIB close |
| Unit |  |  |

Table: Encoding Details of HMIFeedback_LHIB

HMIFeedback_LHOB_Vertical

Register position feedback to HMI

Note: An encoding is either discrete or continuous. Delete those fields, which are not needed.

| Value (Continuous Encoding) | Min Value |  |
| --- | --- | --- |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Table: Encoding Details of HMIFeedback_LHOB_Vertical

Document ends here.