|  | Function Group Spec Stowable Steering and Deployable Tray System <<Logical>> (Allocated) |  |  |  |
| --- | --- | --- | --- | --- |
| Document Type | Function Specification |  |  |  |
| Template Version | 6.0 |  |  |  |
| SysML Report Template Version | O Beta (11/6/2019) |  |  |  |
| Document ID | functionspec_sysmlreporttemplate |  |  |  |
| Document Location |  |  |  |  |
| Document Owner | Tok Lau |  |  |  |
| Document Revision | FGS0 |  |  |  |
| Document Status | Complete |  |  |  |
| Date Issued | 2021-06-09 |  |  |  |
| Date Revised | 2021-06-09 |  |  |  |
| Document Classification | GIS1 Item Number: | 27.60/35 |  |  |
|  | GIS2 Classification: | Confidential |  |  |
| Document Approval |  |  |  |  |
| Name | Role |  | Email Confirmation | Date |

Auto-Generated by MagicDraw

Printed Copies are Uncontrolled

# Disclaimer

This document contains Ford Motor Company Confidential information. Disclosure of the information contained in any portion of this document is not permitted without the expressed, written consent of a duly authorized representative of Ford Motor Company, Dearborn, Michigan, U.S.A.

Copyright, Ó 2016  Ford Motor Company

This document contains information developed and accumulated by and for FORD MOTOR COMPANY. As such, it is a proprietary document, which, if disseminated to unauthorized persons, would provide others with restricted information, data, or procedures not otherwise available, exposing the FORD MOTOR COMPANY to potential harm.

Employees and suppliers having custody of this specification or authorized to use it must be cognizant of its proprietary nature and ensure that the information herein is not made available to unauthorized persons.

FORD MOTOR COMPANY reserves the right to protect this work as an unpublished copyrighted work in the event of an inadvertent or deliberate unauthorized publication. FORD MOTOR COMPANY also reserves its rights under copyright laws to protect this work as a published work.

This document or portions thereof shall not be distributed outside FORD MOTOR COMPANY without prior written consent. Refer all questions concerning disclosure to the author(s) or to any duly authorized representative of Ford Motor Company.

# Contents

1.1.1	Decomposition of   	4

Disclaimer	16

Contents	17

2	Introduction	19

2.1	Document Purpose	19

2.2	Document Scope	19

2.3	Document Audience	19

2.3.1	Stakeholder List	19

2.4	Document Organization	19

2.4.1	Document Context	19

2.4.2	Document Structure	19

2.5	Document Conventions	20

2.5.1	Requirements Templates	20

3	Function Group Description	21

4	Functional Architecture	22

4.1	Description	22

4.2	Function List	22

4.3	Signal List	22

5	Function Specifications	23

5.1	  	23

5.1.1	Function Overview	23

5.1.2	Function Scope	25

5.1.3	Function Interfaces	26

5.1.4	Function Modeling	28

5.1.5	Function Requirements	28

6	Open Concerns	32

7	Revision History	33

8	Appendix	34

8.1	Data Dictionary	34

8.1.1	Logical Signals	34

8.1.2	Logical Parameters	34

8.1.3	Encoding Types	34

8.2	Glossary	34

8.2.1	Definitions	34

8.2.2	Abbreviations	35

List of Figures

Figure 1 Activity diagram “” depicting black box behavior collaboration	22

Figure 2: Activity Diagram of   “” calling   “”	26

Figure 3: 	28

List of Tables

Table 1:  - Decomposition Table	5

Table 2: Functions described in this specification	19

Table 3: List of Logical Functions	22

Table 4: Ford internal Documents	25

Table 5: Ford internal Documents (not specified in model)	25

Table 6: External documents and publications	25

Table 7: External documents and publications (not specified in model)	25

Table 8: Open Concerns (Not supported by MagicDraw report generation.)	32

Table 9: Definitions used in this document	35

Table 10: Abbreviations used in this document	35

# Introduction

## Document Purpose

The Function (Group) Specification (FS) specifies an individual function / a group of functions.

To get more information about the concept of feature, function and component level abstraction refer to the Ford RE Wiki.

## Document Scope

The following set of functions from the Global Feature & Function List is described in this specification.

| Function ID | Function Name | Owner | Reference |
| --- | --- | --- | --- |

Table 2: Functions described in this specification

## Document Audience

The FS is authored by the owners of the individual functions. All Stakeholders, i.e., all people who have a valid interest in the functions and their behavior should read and, if possible, review the FS. It needs to be guaranteed, that all stakeholders have access to the currently valid version of the FS.

#Hint: The FS template has the IP Classification “Proprietary” by default. IP Classification “Confidential” might be required in some cases, e.g. by Ford Functional Safety.

### Stakeholder List

For the latest list of the feature stakeholder and their roles & responsibilities refer to <Put VSEM Link here>.

#Hint: Refer to Ford RE Wiki – Stakeholder List on how to create a stakeholder list. The stakeholder list should be stored in VSEM in the pseudo folder “General Data Artifacts” of the corresponding function.

## Document Organization

### Document Context

Refer to the Specification Structure page in the Ford RE Wiki to understand how the FS relates to other Ford Requirements Documents and Specifications.

### Document Structure

The structure of this document is explained below:

Section 1 – 	Introduction how to use this document including responsibilities and requisite documents. Explains the terminology. Gives a clarification of the definitions, concepts and abbreviations used in the document.

Section 2 – 	Function Group Description. Gives an overview and the purpose of the function group.

Section 3 – 	Functional Architecture: Specifies the overall functional architecture of the function group

Section 4 – 	Function Specifications: Specifies the logical functions of the function group in detail

Section 5 – 	List of Open Concerns

Section 6	Revision history including a list of new or modified requirements. The requirements in this document are tagged, and this section contains different types of tables listing all, new, or changed requirements by their title and page no.

Section 7 – 	Appendix: Presenting additional data mainly in a tabular form, e.g., a data dictionary

#Hint: All sections are mandatory, unless explicitly marked by the tag “#Classification” as “optional” or as applicable e.g. to certain domains like “Functional Safety”.

## Document Conventions

### Requirements Templates

Each requirement, use case or scenario in this specification shall follow the corresponding template given in the document template Specification_Macros.dotm at RE Wiki - Specification Templates.

#### Identification of Requirements

#### Requirements Attributes

The templates provided by Specification_Macros.dotm define a list of attributes for each requirement. This helps to classify the requirement. The attributes are explained at RE Wiki - Requirements Attributes.

# Function Group Description

#Classification: Function Group only (remove section, if only a single Function is specified in this document)

#Hint: Provide an overview / a description of the Function Group

This Function Group consists of all functions allocated to   Stowable Steering and Deployable Tray System <<Logical>> including all functions in their corresponding call trees.

Description of Stowable Steering and Deployable Tray System:

Logical Realization of the Conceptual Feature Entity

# Functional Architecture

#Classification: Function Group only (remove section, if only a single Function is specified in this document)

#Hint: The Functional Architecture shall reflect the result of the functional decomposition for a given feature or parts of it.

Refer to the RE Wiki – Functional Decomposition for some guidance on how to decompose a feature into functions, i.e., how to find the right functional partitioning for the function level. The functions shown here are those, which are specified in section 5 “Function Specifications”.

## Description

#Hint: Provide some informal description of the characteristics of the chosen architecture. Also give some graphical representation of the Functional Architecture. Either SysML Internal Block diagrams or Data Flow Diagrams could be used to depict such a Functional Architecture.

#Link: SysML – Internal Block Diagrams or RE Wiki - Data Flow Diagrams

## Function List

| Function ID | Function Name | Function Description | ASIL |
| --- | --- | --- | --- |
|  | Request Airbag Enabling <<Subsystem Function>> | Depending upon State Selection Signal this function sends a request to DeInhibit Airbag |  |
|  | Check Feature Availability <<Subsystem Function>> | This function checks the vehicle configuration and sends the State Selection Signal |  |
|  | Provide Tray Lock status <<Subsystem Function>> | This Function provides Tray Lock status from Tray Lock Sensor |  |
|  | provide cause of failure <<Subsystem Function>> | Based on Termination signal Information ,the cause of failure is conveyed to the Driver through this function |  |
|  | Move SC in Tele Direction <<Subsystem Function>> | A subsystem Function responsible for Moving Steering Column in Tele Direction |  |
|  | Check for Transition Termination <<System Function>> | This system Functions check all the terminate signals information from Seat , steering Column and Tray Table and verifies if termination of transition is necessary |  |
|  | Move SC in Rake Direction <<Subsystem Function>> | A subsystem Function responsible for Moving Steering Column in Rake Direction |  |
|  | Request Tray Arbitration <<System Function>> | This System Function is responsible for preparing the Tray to Arbitrate and send the respective command for Arbitrating tray to other functions |  |
|  | Request Locomotion Disabling <<Subsystem Function>> | Depending upon State Selection Signal this function sends a request to Inhibit Locomotion |  |
|  | Check Inhibit status <<Subsystem Function>> | This Subsystem function verifies the Locomotion and Air bag Inhibit status and Outputs the Inhibit status |  |
|  | Request Tray Deployment <<Subsystem Function>> | This System Function is responsible for preparing the Tray to Arbitrate and send the respective command to Arbitrate tray to next functions |  |
|  | Issue Termination Signal <<Subsystem Function>> | This subsystem function issues Terminate signal based on other status |  |
|  | Control Tray Functionality <<Subsystem Function>> | This function decides the Tray Actuation based on sensor inputs and Commands |  |
|  | Control Steering Column Functionality <<Subsystem Function>> | This subsystem function sends out actuate /Move commands for respective actuators and reads the signals from sensors |  |
|  | Provide steering column Rake position <<Subsystem Function>> | Subsystem Function responsible for sending Steering Column Rake Position |  |
|  | Provide Tray Stow Instructions <<Subsystem Function>> | A subsystem Function which provides instructions to HMI when User requests stow or to exit work mode |  |
|  | Request Seat Arbitration <<System Function>> | This System Function responsible for preparing the Seat to Arbitrate and send the respective request to Driver Seat System |  |
|  | Control SSC Functionality <<System Function>> | When received State selection signal for mode transition , this function issues individual requests to Seat, steering Column and Tray Table after verifying the Inhibit status and Provide a cause of Failure if the transition is not possible |  |
|  | Sense Tray Overload <<Subsystem Function>> | This function provides the status of Overload on the Tray Table |  |
|  | Request Tray Unlock <<Subsystem Function>> | A subsystem Function which sends Tray Unlock request based on its inputs |  |
|  | Request Airbag Disabling <<Subsystem Function>> | Depending upon State Selection Signal this function sends a request to Inhibit Airbag |  |
|  | Request Drive Control Inhibition <<System Function>> | Depending upon State Selection Signal this function sends a request to Inhibit Drive Control |  |
|  | Request SteeringColumn Deployment <<Subsystem Function>> | This Sub System Function is responsible for sending Steering Column Stow Request based on its Inputs requests |  |
|  | Request to Move Seat <<Subsystem Function>> | This Sub System Function is responsible for sending Seat Movement Request based on its Inputs requests |  |
|  | Request Steering Column Arbitration <<System Function>> | A System Function responsible for preparing the Steering Column to Arbitrate and send the respective command to Arbitrate Steering Column to next functions |  |
|  | Arbitrate SSC Functionality <<Subsystem Function>> | Arbitrate SSC Functionality requests the other sub systems in the systems when state selection signal is received for Mode transition |  |
|  | Actuate Tray Locking <<Subsystem Function>> | This Function Actuates the Tray Lock and Unlock based on requests |  |
|  | Provide steering column Tele position <<Subsystem Function>> | Subsystem Function responsible for sending Steering Column Tele Position |  |
|  | Provide Tray Position <<Subsystem Function>> | This Function provides Tray position through Tray Position sensor |  |
|  | Provide HMI Feedback <<Subsystem Function>> | This Function Checks the Vehicle Validity and state selection signal and provide User his selected Mode and Vehicle status Validity |  |
|  | Verify Feature Status <<System Function>> | This Function verifies the User Input , vehicle Status and Vehicle Configuration and provides State Selection signal and HMI Feedback |  |
|  | Check Tray Status <<Subsystem Function>> | This sub system Function checks the status of Tray upon receiving the move request and issue request respectively |  |
|  | Request Drive Control DeInhibition <<System Function>> | Depending upon State Selection Signal this function sends a request to DeInhibit Drive Control |  |
|  | Actuate Tray <<System Function>> | System Function responsible for Actuating Tray based on requests |  |
|  | Verify Drive Control Status <<System Function>> | System Function responsible for Verify the Drive Control Status Before Actuating the feature |  |
|  | Request SteeringColumn Stowing <<Subsystem Function>> | This Sub System Function is responsible for sending Steering Column Stow Request based on its Inputs requests |  |
|  | Check vehicle status <<Subsystem Function>> | This Function checks the Vehicle status and outputs the Vehicle status Validity |  |
|  | Evaluate Drivability status <<Subsystem Function>> | This function checks all the status ( Tray , Steering Column, Driver seat) before sending a request to enable the drive control( Locomotion and Driver Airbag) |  |
|  | Actuate Steering Column <<System Function>> | System Function Responsible for Actuating Steering Column as per request from Feature Controller |  |
|  | Request Tray Lock <<Subsystem Function>> | A subsystem Function which sends Tray Lock request based on its inputs |  |
|  | Request Locomotion Enabling <<Subsystem Function>> | Depending upon State Selection Signal this function sends a request to DeInhibit Locomotion |  |
|  | Actuate Tray Movement <<Subsystem Function>> | This Function Deploys / Stows the Tray based on request |  |

Table 3: List of Logical Functions

## Signal List

Refer to the Data Dictionary - Logical Signals.

# Function Specifications

## Actuate Steering Column

### Function Overview

#### Description

#Hint: Some descriptive text to explain the purpose and functionality of the function.

Function is allocated to:

- Stowable Steering and Deployable Tray System <<Logical>>

System Function Responsible for Actuating Steering Column as per request from Feature Controller

#### Variants

#Classification: Mandatory (State “Not applicable”, if not used)

#Hint: If different variants of the same function are specified in this section, list those variants in the table below.

Variants on Function level could be driven technology or feature content. Example: There could be a “Low Content” and a “High Content” variant of some exterior lighting function. The Low Content variant applies for Conventional Headlight technology, the High Content variant applies for LED and Xenon technology. In this case we call the different technologies the Variant Options (for the time being you could think of them as Logical Parameters) which the Variant depends on. The optional column “Variant condition” allows to express the dependency of a Variant based on Variant Options/Logical Parameters.

If requirements/signals are not applicable for all variants/variant options, those requirements should state explicitly, which function variant/variant option they apply to.

#Link: RE Wiki – Variant Management.

Not supported by MagicDraw report generation.

#### Input Requirements

#Classification: Mandatory (State “Not applicable”, if not used)

#Hint: List any input requirements here (legal, Trustmark), which need to be taken into account, beyond what is specified in the corresponding Feature Documents.

Not supported by MagicDraw report generation.

#### Assumptions

#Classification: Mandatory (State “Not applicable”, if not used)

#Hint: A list of known assumptions concerning the effects of the function’s behavior on other functions or elements (i.e., dependencies) as well as assumptions on the behavior expected by the function (e.g. known limitations). During the course of the development most of those assumptions are typically either converted into actual requirements or discarded at some point – such that this chapter remains mostly empty.

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

#Hint: You may refer to IEEE Citation Reference on how to format a reference.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Actuate Steering Column” function is called by the following functions:

- – “Make Rest space Available”

- – “Make Work Space Available”

Figure 2: Activity Diagram of   “Make Rest space Available” calling   “Actuate Steering Column”

Figure 2: Activity Diagram of   “Make Work Space Available” calling   “Actuate Steering Column”

### Function Interfaces

#Link: RE Wiki – Adding a Logical Signal or Parameter

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Steering Column Request Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Received from: - Request Steering Column Arbitration |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| steering column rake position Type: SteeringColumnRakePosition | Signal Description: Steering Column Position in Rake Position Sent to: - Request Steering Column Arbitration |
| steering column tele position Type: SteeringColumnTelePosition | Signal Description: Steering Column Position in Tele Position Sent to: - Request Steering Column Arbitration |

#### Logical Parameters

#Hint: Put requirements for parameters here, which are implemented as configuration parameters using Method 2 or 3 or as parameters for calibration.

Not supported by MagicDraw report generation.

### Function Modeling

#Classification: Mandatory

#Hint: Typical modeling artifacts in this section are State Machines, Activity Diagrams / Flow Charts, Decision Tables, and possibly Sequence Diagrams, which can all be used as techniques to analyze the function requirements.

#Links: Analyze / Model Requirements: RE Wiki – Analyze / Model Requirements

#### Use Cases

#Classification: Infotainment Only (remove section, if not used)

#Hint: Some Domains (e.g. Infotainment) use not only Customer Use Cases (in the Feature Doc), but refine Use Case descriptions down to function level. In general, the RE approach encourages the use of Use Cases on Feature Level but not on Function Level. Activity Diagrams are a more suitable way to express the same on Function Level.

#Links: Infotainment – “Harmony Systems Engineering” Approach

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

#Classification: Mandatory

#Hint: It is highly recommended to use at least one of the following modeling techniques for modeling and analyzing the Function behavior and derived requirements (refer to sample diagrams below):

State Machines, Activity Diagrams / Flow Charts, or Decision Tables

Optionally, Sequence Diagrams might help to analyze the interaction between Functions.

#Links: Analyze / Model Requirements: RE Wiki – Analyze / Model Requirements

State Charts RE Wiki – State Charts

Activity Diagrams: RE Wiki – Activity Diagram, SysML User Group – Activity Diagram Basics

Sequence Diagrams: RE Wiki – Sequence Chart, SysML User Group – Sequence Diagram Basics

Figure 3: Actuate Steering Column

### Function Requirements

#Link: RE Wiki – How to write good requirements

#### Functional Requirements

#Hint: Please also consider specific situations like Initialization (Startup) and Deinitialization (Shutdown) apart from Normal Operation and Error Handling. E.g. a state chart or activity diagram might help for better understanding.

##### Normal Operation

1117 Steering Column Position

Stowable Steering and Deployable System shall provide steering column position to Feature Controller

| Requirement ID: 1117 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

#Hint: FMEA counter measures could be considered as requirements in this chapter

No Error Handling Requirements specified.

#### Non-Functional Requirements

#Hint: Non-functional requirements specify some performance criteria in addition to the functional behavior given defined by the functional requirements. Timing (if not already included in the functional requirements), security details (e.g. how secure does an algorithm have to be) or reliability (e.g. mean time between failure) could be specified in this section.

No Non-Functional Requirements specified.

#### Functional Safety Requirements

#Classification: Functional Safety only – If not used, remove content and state “Not Applicable”

#Hint: Add Functional Safety Requirements (FSRs) derived for this function.

#Link:	Functional Safety Sharepoint – Functional Safety Concept

RE Wiki - Requirements Attributes

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

#Classification: Functional Safety only – If not used, remove content and state “Not Applicable”

#Hint:  Sometimes an ASIL decomposition of Functional Safety Requirements is required. The decomposed FSRs should be listed beneath each ASIL Decomposition table below and referenced inside the table by ID and Title

#Link:	Functional Safety Sharepoint – Functional Safety Concept

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

#Hint: Requirements of a Logical Function should be typically agnostic of their SW/HW implementation. If for specific reasons the function owner needs to define explicitly design constraints, it can be done in this chapter.

No Design Requirements specified.

#### Uncategorized Requirements

#Hint: Requirements that are allocated to this function but do not fit in any of the previous categories are populated below.

1120 Actuate Steering Column

Actuate Steering Column System Function shall Actuate The steering Column position according to the request from Steering Column Controller

| Requirement ID: 1120 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Actuate Tray

### Function Overview

#### Description

Function is allocated to:

- Stowable Steering and Deployable Tray System <<Logical>>

System Function responsible for Actuating Tray based on requests

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Actuate Tray” function is called by the following functions:

- – “Make Work Space Available”

Figure 2: Activity Diagram of   “Make Work Space Available” calling   “Actuate Tray”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Move Tray Command Type: TrayMoveCommand | Signal Description: Tray Move Command is the command signal which has various Tray Control requests in it. Received from: - Request Tray Arbitration |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Tray Overload Status Type: TrayOverloadStatus | Sent to: - Request Tray Arbitration |
| Tray Lock Status Type: TrayLockStatus | Signal Description: Tray lock status is given by Tray Lock sensor Sent to: - Request Drive Control DeInhibition - Request Tray Arbitration |
| Tray Position Status Type: TrayPostionStatus | Signal Description: Tray Position status is the feedback for Tray position given by Tray position Sensor Sent to: - Request Tray Arbitration |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Actuate Tray

### Function Requirements

#### Functional Requirements

##### Normal Operation

1096 Tray Movement Functionality

When requested, Stowable Tray System shall manage Stowing / Deploying Functionality of the Stowable Tray

| Requirement ID: 1096 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1097 Tray Position sense

While Activated, Stowable Tray System shall detect position of the Stowable Tray

| Requirement ID: 1097 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1098 Tray Lock Status Sensing

While Activated, Stowable Tray System shall detect the Lock Status of the Stowable Tray

| Requirement ID: 1098 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1099 Tray Lock Functionality

When requested, Stowable Tray System shall manage Locking / Unlocking Functionality of the Stowable Tray

| Requirement ID: 1099 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1100 Tray Obstacle Sensing

Stowable Tray system shall Detect for any obstacles present on Tray before Stowing

| Requirement ID: 1100 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1106 Actuating Tray

Upon receiving Move Tray Command , Actuate Tray Function shall send

1 . Tray position Status

2. Tray Obstruction status

3. Tray Lock status

| Requirement ID: 1106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Actuate Tray Locking

### Function Overview

#### Description

Function is allocated to:

- Actuate Tray Locking Mechanism <<Logical>>

- Tray Locking Actuator <<Logical>>

This Function Actuates the Tray Lock and Unlock based on requests

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Actuate Tray Locking” function is called by the following functions:

- – “Actuate Tray”

Figure 2: Activity Diagram of   “Actuate Tray” calling   “Actuate Tray Locking”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Tray Lock Request Type: TrayMoveCommand | Signal Description: Tray Move Command is the command signal which has various Tray Control requests in it. Received from: - Control Tray Functionality |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

919 Actuate Tray Locking Definition

The Actuate Tray Locking function shall LOCK, UNLOCK or remain stationary or NONE the Feature Tray Table when the Tray Lock Command is issued.

| Requirement ID: 919 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Actuate Tray Movement

### Function Overview

#### Description

Function is allocated to:

- Tray Movement Actuator <<Logical>>

This Function Deploys / Stows the Tray based on request

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Actuate Tray Movement” function is called by the following functions:

- – “Actuate Tray”

Figure 2: Activity Diagram of   “Actuate Tray” calling   “Actuate Tray Movement”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Tray Move Command Type: TrayMoveCommand | Signal Description: Tray Move Command is the command signal which has various Tray Control requests in it. Received from: - Control Tray Functionality |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Tray Motor Load Type: TrayMotorLoad | Sent to: - Sense Tray Overload |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

862 Actuate Tray Movement Definition

The Actuate Tray Movement function shall DEPLOY or remain stationary or NONE the Feature Tray Table when the Tray Move Command provides an input command.

| Requirement ID: 862 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Arbitrate SSC Functionality

### Function Overview

#### Description

Function is allocated to:

- Stowable Feature Controller <<Logical>>

Arbitrate SSC Functionality requests the other sub systems in the systems when state selection signal is received for Mode transition

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Arbitrate SSC Functionality” function is called by the following functions:

- – “Control SSC Functionality”

Figure 2: Activity Diagram of   “Control SSC Functionality” calling   “Arbitrate SSC Functionality”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Inhibit status Type: InhibitStatus | Signal Description: Inhibit status of driver control which is verified by the feature systems Received from: - Activity Parameter Node: Inhibit Status |
| state selection signal Type: StateSelectionSignal | Signal Description: Signal which contains the Mode that User selected after the Vehicle status is Valid Received from: - Activity Parameter Node: State Selection Signal |
| Termination signal Type: Termination Signal | Signal Description: Logical signal for termination of Mode Arbitration when system did not act according to requests Received from: - Activity Parameter Node: Termination Signal |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Tray Move request Type: TrayMoveRequest | Signal Description: Internal Request of Feature Controller to Move Tray Table Sent to: - Activity Parameter Node: Tray Move request |
| Seat Movement Request Type: SeatMovementRequest | Signal Description: Command to Move seat which goes to the Driver seat System Sent to: - Activity Parameter Node: Seat movement Request |
| Steering Column Request Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Sent to: - Activity Parameter Node: SC Request |
| State of Feature Type: StateOfFeature | Signal Description: Current state of Feature and other warnings Sent to: - Activity Parameter Node: Publish state |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

828 Normal End of Deployment

WHEN Inhibit Status is DC_INHIBITED AND State Selection Signal is DRIVE AND Termination Signal is NONE THE Arbitrate SSC Functionality Subfunction SHALL set State of Feature to DRIVE_STATE_ENTERED

| Requirement ID: 828 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

830 Normal Rest Mode Entry

WHEN Inhibit Status is DC_INHIBITED AND State Selection Signal is REST AND Termination Signal is NONE THE Arbitrate SSC Functionality Subfunction SHALL set State of Feature to REST_STATE_ENTERED

| Requirement ID: 830 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

832 Normal Work Mode Entry

WHEN Inhibit Status is DC_INHIBITED AND State Selection Signal is WORK AND Termination Signal is NONE THE Arbitrate SSC Functionality Subfunction SHALL set State of Feature to DRIVE_STATE_ENTERED

| Requirement ID: 832 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

834 Drive State Entered

WHEN Inhibit Status is DC_DEINHIBITED AND Termination Signal is NONE THE Arbitrate SSC Functionality Subfunction SHALL set State of Feature to DRIVE_STATE_ENTERED

| Requirement ID: 834 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

836 Arbitrate SSC Functionality function Output State of Feature Definition

The Arbitrate SSC Functionality function output State of Feature shall define states WORK_STATE_ENTERED, FAILED_TO_ENTER_WORK_STATE, REST_STATE_ENTERED, FAILED_TO_ENTER_REST_STATE and DRIVE_STATE_ENTERED, FAILED_TO_ENTER_DRIVE_STATE

| Requirement ID: 836 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

840 Start Steering Column Stow

WHEN Inhibit Status is DC_INHIBITED AND State Selection Signal is NOT DRIVE AND Termination Signal is NONE THEN Arbitrate SSC Functionality Subfunction SHALL set SC Request to STOW

| Requirement ID: 840 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality - 1096 Tray Movement Functionality - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

869 End Tray Deployment

WHEN Inhibit Status is DC_INHIBITED AND State Selection Signal is DRIVE AND Termination Signal is NONE THE Arbitrate SSC Functionality Subfunction SHALL set Tray Request to LOCK

| Requirement ID: 869 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

872 Deploy Tray

WHEN Inhibit Status is DC_INHIBITED AND State Selection Signal is NOT DRIVE AND Termination Signal is NONE THE Arbitrate SSC Functionality Subfunction SHALL set Tray Request to Unlock+Deploy

| Requirement ID: 872 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

873 Maintain Tray in Drive Position

WHEN Inhibit Status is DC_DEINHIBITED AND Termination Signal is NONE THE Arbitrate SSC Functionality Subfunction SHALL set Tray Request to LOCK

| Requirement ID: 873 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

874 Arbitrate SSC Functionality TrayRequest Output Definition

The Arbitrate SSC Functionality function output Tray Request shall define states NONE, LOCK and Unlock+Deploy or UC.

| Requirement ID: 874 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

932 End Steering Column Stow

WHEN Inhibit Status is DC_INHIBITED AND State Selection Signal is DRIVE AND Termination Signal is NONE THE Arbitrate SSC Functionality Subfunction SHALL set SC Request to DEPLOY

| Requirement ID: 932 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality - 1096 Tray Movement Functionality - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

940 Deploy Seat

WHEN Inhibit Status is DC_INHIBITED AND State Selection Signal is NOT DRIVE AND Termination Signal is NOT TERMINATE_SEAT THE Arbitrate SSC Functionality Subfunction SHALL set Seat Request to MOVE_REST

| Requirement ID: 940 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

941 Maintain Seat in Drive Position

WHEN Inhibit Status is DC_DEINHIBITED AND Termination Signal is NONE THE Arbitrate SSC Functionality Subfunction SHALL set Seat Request to MOVE_DRIVE

| Requirement ID: 941 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

942 Arbitrate SSC Functionality function output Seat Request Definition

The Arbitrate SSC Functionality function output Seat Request shall define output states NONE, MOVE_WORK, MOVE_REST and MOVE_DRIVE

| Requirement ID: 942 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

987 Maintain Steering Column in Drive Mode

WHEN Inhibit Status is DC_DEINHIBITED AND Termination Signal is NONE THEN Arbitrate SSC Functionality Subfunction SHALL set SC Request to DEPLOY

| Requirement ID: 987 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality - 1096 Tray Movement Functionality - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

988 End Seat Deployment

WHEN Inhibit Status is DC_INHIBITED AND State Selection Signal is DRIVE AND Termination Signal is NONE THE Arbitrate SSC Functionality Subfunction SHALL set Seat Request to MOVE_DRIVE

| Requirement ID: 988 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality - 1096 Tray Movement Functionality - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1059 Arbitrate SSC Functionality Steering Column Request Output

The Arbitrate SSC Functionality function output Steering Column Request or SC Request shall define NONE, STOW and DEPLOY

| Requirement ID: 1059 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

829 Failed to End Deployment

IF Inhibit Status is DC_INHIBITED AND State Selection Signal is DRIVE AND Termination Signal is NOT NONE THEN the Arbitrate SSC Functionality Subfunction SHALL set State of Feature to FAILED_TO_ENTER_DRIVE_STATE

| Requirement ID: 829 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

831 Failed to Enter Rest State

IF Inhibit Status is DC_INHIBITED AND State Selection Signal is REST AND Termination Signal is NOT NONE THEN the Arbitrate SSC Functionality Subfunction SHALL set State of Feature to FAILED_TO_ENTER_REST_STATE

| Requirement ID: 831 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

833 Failed to Enter Work State

IF Inhibit Status is DC_INHIBITED AND State Selection Signal is WORK AND Termination Signal is NOT NONE THEN the Arbitrate SSC Functionality Subfunction SHALL set State of Feature to FAILED_TO_ENTER_WORK_STATE

| Requirement ID: 833 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

835 Failed to Enter Drive State

IF Inhibit Status is DC_DEINHIBITED AND Termination Signal is NOT NONE THEN the Arbitrate SSC Functionality Subfunction SHALL set State of Feature to FAILED_TO_ENTER_DRIVE_STATE

| Requirement ID: 835 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

839 Halt SC Movement

IF Termination Signal is TERMINATE_SC THEN Arbitrate SSC Functionality Subfunction SHALL set SC Request to NONE

| Requirement ID: 839 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality - 1096 Tray Movement Functionality - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

870 Halt Tray Movement

IF Termination Signal is TERMINATE_TRAY THEN the Arbitrate SSC Functionality Subfunction SHALL set Tray Request to NONE

| Requirement ID: 870 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

871 Return Tray to Safe State

IF Termination Signal is TERMINATE_SC OR Termination Signal is TERMINATE_SEAT THEN the Arbitrate SSC Functionality Subfunction SHALL set the Tray Request to LOCK

| Requirement ID: 871 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

933 Return Steering Column to Safe State

IF Termination Signal is TERMINATE_TRAY OR Termination Signal is TERMINATE_SEAT THEN Arbitrate SSC Functionality Subfunction SHALL set SC Request to DEPLOY

| Requirement ID: 933 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality - 1096 Tray Movement Functionality - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

989 Return Seat to Safe State

IF Termination Signal is TERMINATE_TRAY OR Termination Signal is TERMINATE_SC THEN Arbitrate SSC Functionality Subfunction SHALL set Seat Request to MOVE_DRIVE

| Requirement ID: 989 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality - 1096 Tray Movement Functionality - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

990 Halt Seat Movement

IF Termination Signal is TERMINATE_SEAT THEN Arbitrate SSC Functionality Subfunction SHALL set Seat Request to NONE

| Requirement ID: 990 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality - 1096 Tray Movement Functionality - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Check Feature Availability

### Function Overview

#### Description

Function is allocated to:

- Stowable Feature Controller <<Logical>>

This function checks the vehicle configuration and sends the State Selection Signal

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Check Feature Availability” function is called by the following functions:

- – “Verify Feature Status”

Figure 2: Activity Diagram of   “Verify Feature Status” calling   “Check Feature Availability”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Vehicle Configuration Type: VehicleConfig | Signal Description: Configuration of the Feature and Vehicle Specific Received from: - Activity Parameter Node: Vehicle Configuration |
| User Input Type: UserInput | Signal Description: Input given by user through HMI Received from: - Activity Parameter Node: User Input |
| Vehicle Status Validity Type: VehicleStatusValidity | Signal Description: This is the signal which contains the validity of the Vehicle status before Arbitrating into any Mode Received from: - Check vehicle status |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Unavailabilty Type: ModeAcceptenceStatus | Signal Description: This signal contains the information whats not available in the vehicle configuration Sent to: - Provide HMI Feedback |
| State Selection signal Type: StateSelectionSignal | Signal Description: Signal which contains the Mode that User selected after the Vehicle status is Valid Sent to: - Activity Parameter Node: state selection signal |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

1069 Check Feature Availability State Selection Signal Set Work State

The Check Feature Availability function shall set State Selection Signal to WORK when User Input is set to WORKMODE AND Vehicle Configuration is set to ENABLE AND Vehicle Status Validity is set to VALID

| Requirement ID: 1069 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1070 Check Feature Availability Vehicle Status Validity Set to INVALID

The Check Feature Availability function shall set State Selection Signal to DRIVE when Vehicle Status Validity is set to INVALID

| Requirement ID: 1070 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1071 Check Feature Availability Vehicle SConfiguration Set to DISABLE

The Check Feature Availability function shall set State Selection Signal to DRIVE when Vehicle Configuration is set to DISABLE

| Requirement ID: 1071 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1072 Check Feature Availability User Input Set to NONE

The Check Feature Availability function shall set State Selection Signal to DRIVE when User Input is set to NONE

| Requirement ID: 1072 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1073 Check Feature Availability State Selection Signal Set Rest State

The Check Feature Availability function shall set State Selection Signal to REST when User Input is set to RESTMODE AND Vehicle Configuration is set to ENABLE AND Vehicle Status Validity is set to VALID

| Requirement ID: 1073 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1074 Check Feature Availability State Selection Signal Set Play State

The Check Feature Availability function shall set State Selection Signal to PLAY when User Input is set to PLAYMODE AND Vehicle Configuration is set to ENABLE AND Vehicle Status Validity is set to VALID

| Requirement ID: 1074 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1075 Check Feature Availability User Input Set to NOT_AVAILABLE

The Check Feature Availability function shall set State Selection Signal to DRIVE when User Input is set to NOT_AVAILABLE

| Requirement ID: 1075 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1076 Check Feature Availability User Input Set to RESET

The Check Feature Availability function shall set State Selection Signal to DRIVE when User Input is set to RESET

| Requirement ID: 1076 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Check for Transition Termination

### Function Overview

#### Description

Function is allocated to:

- Stowable Steering and Deployable Tray System <<Logical>>

This system Functions check all the terminate signals information from Seat , steering Column and Tray Table and verifies if termination of transition is necessary

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Check for Transition Termination” function is called by the following functions:

- – “Make Rest space Available”

- – “Make Work Space Available”

Figure 2: Activity Diagram of   “Make Rest space Available” calling   “Check for Transition Termination”

Figure 2: Activity Diagram of   “Make Work Space Available” calling   “Check for Transition Termination”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Seat Movement status Internal Type: SeatMovementStatusInternal | Signal Description: Status of the Seat movement according to the current position and external position Received from: - Request Seat Arbitration |
| SC Status Type: SteeringColumnStatusInternal | Signal Description: Status of SteeringColumn based on Feature Decisions and current steering Column Position Received from: - Request Steering Column Arbitration |
| Tray Status Type: TrayStatus | Signal Description: This status is decided and fed by the Feature systems based on Requests and Input feedbacks fed Received from: - Request Tray Arbitration |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Termination Signal Type: Termination Signal | Signal Description: Logical signal for termination of Mode Arbitration when system did not act according to requests Sent to: - Control SSC Functionality |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Check for Transition Termination

### Function Requirements

#### Functional Requirements

##### Normal Operation

No Normal Operation Requirements specified.

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Check Inhibit status

### Function Overview

#### Description

Function is allocated to:

- check Inhibit Status <<Logical>>

- Stowable Feature Controller <<Logical>>

This Subsystem function verifies the Locomotion and Air bag Inhibit status and Outputs the Inhibit status

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Check Inhibit status” function is called by the following functions:

- – “Verify Drive Control Status”

Figure 2: Activity Diagram of   “Verify Drive Control Status” calling   “Check Inhibit status”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Locmotion Inhibit Status Type: LocomotionInhibitStatus | Signal Description: Status of the Locomotion Inhibition Received from: - Activity Parameter Node: Locomotion Inhibit status |
| Driver Airbag Inhibit Status Type: DriverAirbagInhibitStatus | Signal Description: Status of the Driver Airbag Inhibition Received from: - Activity Parameter Node: Driver Airbag Inhibit status |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Inhibit Status Type: InhibitStatus | Signal Description: Inhibit status of driver control which is verified by the feature systems Sent to: - Activity Parameter Node: Inhibit status |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

976 Check Inhibit status Timer INHIBITED

The Check Inhibit status Function shall set the Next Timer to INIT when Driver Airbag Inhibit Status is set to INHIBITED AND Locomotion Inhibit Status is set to INHIBITED

| Requirement ID: 976 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

977 Check Inhibit status Timer PENDING

The Check Inhibit status Function shall set the Next Timer to PENDING when Driver Airbag Inhibit Status is not set equal to Locomotion Inhibit Status

| Requirement ID: 977 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

978 Check Inhibit status Timer DEINHIBITED

The Check Inhibit status Function shall set the Next Timer to INIT when Driver Airbag Inhibit Status is set to DEINHIBITED AND Locomotion Inhibit Status is set to DEINHIBITED

| Requirement ID: 978 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

979 Check Inhibit status Timer TIMEOUT

The Check Inhibit status Function shall set the Next Timer to INIT when Timer is set to TIMEOUT

| Requirement ID: 979 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

980 Check Inhibit status Function Inhibit Status INHIBITED

The Check Inhibit status Function shall set the Inhibit Status to DC_INHIBITED when Driver Airbag Inhibit Status is set to INHIBITED AND Locomotion Inhibit Status is set to INHIBITED

| Requirement ID: 980 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

981 Check Inhibit status Function Timer Not TIMEOUT

The Check Inhibit status Function shall set the Inhibit Status to the current State Memory value when Driver Airbag Inhibit Status is not set equal to Locomotion Inhibit Status AND the Timer is NOT TIMEOUT

| Requirement ID: 981 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

982 Check Inhibit status Function Inhibit Status DEINHIBITED

The Check Inhibit status Function shall set the Inhibit Status to DC_DEINHIBITED when Driver Airbag Inhibit Status is set to DEINHIBITED AND Locomotion Inhibit Status is set to DEINHIBITED

| Requirement ID: 982 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

983 Check Inhibit status Function Inhibit Status Failed to Inhibit

The Check Inhibit status Function shall set the Inhibit Status to DC_FAILED_TO_INHIBIT when Timer is set to TIMEOUT.

| Requirement ID: 983 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

984 Check Inhibit status Funciton Inhibit Status Failed to De-Inhibit

The Check Inhibit status Function shall set the Inhibit Status to DC_FAILED_TO_DEINHIBITED when Driver Airbag Inhibit Status and Locomotion Inhibit Status are NOT equal AND State Memory is set to DC_FAILED_TO_DEINHIBIT and Timer is set to TIMEOUT.

| Requirement ID: 984 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

985 Check Inhibit status Function Inhibit Status transition to De-Inhibit

The Check Inhibit status Function shall set the Inhibit Status to DC_FAILED_TO_DEINHIBITED when Driver Airbag Inhibit Status and Locomotion Inhibit Status are NOT equal AND State Memory is set to DC_INHIBIT and Timer is set to TIMEOUT.

| Requirement ID: 985 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

986 Check Inhibit status Function Inhibit Status transition to Inhibit

The Check Inhibit status Function shall set the Inhibit Status to DC_FAILED_TO_INHIBITED when Driver Airbag Inhibit Status and Locomotion Inhibit Status are NOT equal AND State Memory is set to DC_FAILED_TO_INHIBIT and Timer is set to TIMEOUT.

| Requirement ID: 986 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1061 The Check Inhibit status functions Timer Definition

The Check Inhibit status function Timer shall indicate INIT, PENDING and TIMEOUT.

| Requirement ID: 1061 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1062 The Check Inhibit status functions Timer Time-Out Definition

The Check Inhibit status function Timer shall indicate TIMEOUT when the timer reaches its stored limit time.

| Requirement ID: 1062 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1063 The Check Inhibit status function Inhibit Status Definition

The Check Inhibit status function Inhibit Status shall indicate DC_INHIBITED, DC_FAILED_TO_INHIBIT, DC_DEINHIBITED and DC_FAILED_TO_DEINHIBIT

| Requirement ID: 1063 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1064 The Check Inhibit status function State Memory Definition

The Check Inhibit status function State Memory shall store the current DC_INHIBITED, DC_FAILED_TO_INHIBIT, DC_DEINHIBITED or DC_FAILED_TO_DEINHIBIT value output by Inhibit Status

| Requirement ID: 1064 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1068 The Manage Seat Save Request function Save Seat Request Definition

The Manage Seat Save Request function Save Seat Request shall indicate REQUEST_SEAT_SAVE and NO_REQUEST.

| Requirement ID: 1068 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1095 Drive Control Inhibit Status Verification |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Check Tray Status

### Function Overview

#### Description

Function is allocated to:

- Stowable Feature Controller <<Logical>>

This sub system Function checks the status of Tray upon receiving the move request and issue request respectively

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Check Tray Status” function is called by the following functions:

- – “Request Tray Arbitration”

Figure 2: Activity Diagram of   “Request Tray Arbitration” calling   “Check Tray Status”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Tray Lock status Type: TrayLockStatus | Signal Description: Tray lock status is given by Tray Lock sensor Received from: - Activity Parameter Node: Tray Lock status |
| Tray Position status Type: TrayPostionStatus | Signal Description: Tray Position status is the feedback for Tray position given by Tray position Sensor Received from: - Activity Parameter Node: Tray Position status |
| Tray Overload status Type: TrayOverloadStatus | Received from: - Activity Parameter Node: Tray Overload status |
| Tray Move request Type: TrayMoveRequest | Signal Description: Internal Request of Feature Controller to Move Tray Table Received from: - Activity Parameter Node: Tray Move Request |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Tray Lock request Type: TrayMoveRequest | Signal Description: Internal Request of Feature Controller to Move Tray Table Sent to: - Request Tray Lock |
| Tray Unlock request Type: TrayMoveRequest | Signal Description: Internal Request of Feature Controller to Move Tray Table Sent to: - Request Tray Unlock |
| provide HMI instructions Type: HMIInsWar | Signal Description: HMI Instructions to Stow and Warning display if Tray is not pushed in properly Sent to: - Provide Tray Stow Instructions |
| Tray Deploy Request Type: TrayMoveRequest | Signal Description: Internal Request of Feature Controller to Move Tray Table Sent to: - Request Tray Deployment |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

875 Tray Locked With Position Error

WHEN Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_LOCKED  THE Check Tray Status Subfunction SHALL set Tray Deploy Request to NONE

| Requirement ID: 875 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

876 Tray Locked and Waiting to Unlock

WHEN Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_LOCKED AND Tray Position Status is IN_MOTION THE Check Tray Status Subfunction SHALL set Tray Deploy Request to NONE

| Requirement ID: 876 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

877 Tray Fully Deployed

WHEN Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_UNLOCKED AND Tray Position Status is DEPLOYED THE Check Tray Status Subfunction SHALL set Tray Deploy Request to NONE

| Requirement ID: 877 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

878 Tray Unlocked and Deploying

WHEN Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_UNLOCKED AND Tray Position Status is EITHER IN_MOTION OR STOWED THE Check Tray Status Subfunction SHALL set Tray Deploy Request to DEPLOY

| Requirement ID: 878 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

881 Tray Failed to Unlock and Going to Safe State

WHEN Tray Move Request is STOW AND Tray Lock Status is TRAY_LOCKED AND Tray Position Status is EITHER DEPLOYED OR IN_MOTION THE Check Tray Status Subfunction SHALL set Tray Deploy Request to STOW

| Requirement ID: 881 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

882 Tray Fully Stowed

WHEN Tray Move Request is STOW AND Tray Lock Status is EITHER TRAY_LOCKED OR TRAY_UNLOCKED AND Tray Position Status is STOWED THE Check Tray Status Subfunction SHALL set Tray Deploy Request to NONE

| Requirement ID: 882 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

883 Tray Unlocked and Tray Stowing

WHEN Tray Move Request is STOW AND Tray Lock Status is TRAY_UNLOCKED AND Tray Position Status is EITHER DEPLOYED OR IN_MOTION THE Check Tray Status Subfunction SHALL set Tray Deploy Request to STOW

| Requirement ID: 883 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

885 Tray Lock Request is None and Waiting to Unlock

WHEN Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_LOCKED AND Tray Position Status is EITHER STOWED OR IN_MOTION OR FAILED_TO_DEPLOY THE Check Tray Status Subfunction SHALL set Tray Lock Request to NONE

| Requirement ID: 885 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

886 Tray Lock Request is Locked and Failed to Stow

WHEN Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_LOCKED AND Tray Position Status is FAILED_TO_STOW THE Check Tray Status Subfunction SHALL set Tray Lock Request to TRAY_LOCKED

| Requirement ID: 886 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

887 Tray Lock Request is Fully Deployed

WHEN Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_UNLOCKED THE Check Tray Status Subfunction SHALL set Tray Lock Request to NONE

| Requirement ID: 887 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

890 Tray Locked Request Locked and Fully Stowed

WHEN Tray Move Request is STOW AND Tray Lock Status is TRAY_LOCKED AND Tray Position Status is EITHER STOWED OR IN_MOTION OR FAILED_TO_DEPLOY THE Check Tray Status Subfunction SHALL set Tray Lock Request to TRAY_LOCKED

| Requirement ID: 890 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

891 Tray Locked Request Unlocked and Tray Stowing

WHEN Tray Move Request is STOW AND Tray Lock Status is TRAY_UNLOCKED AND Tray Position Status is DEPLOYED THE Check Tray Status Subfunction SHALL set Tray Lock Request to NONE

| Requirement ID: 891 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

892 Tray Locked Request Locked and Tray Stowed

WHEN Tray Move Request is STOW AND Tray Lock Status is TRAY_UNLOCKED AND Tray Position Status is STOWED THE Check Tray Status Subfunction SHALL set Tray Lock Request to TRAY_LOCKED

| Requirement ID: 892 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

897 Tray Unlock Request is Tray Locked and Waiting to Unlock

WHEN Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_LOCKED AND Tray Position Status is EITHER STOWED OR IN_MOTION THE Check Tray Status Subfunction SHALL set Tray Lock Request to TRAY_UNLOCKED

| Requirement ID: 897 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

899 Tray Unlock Request is Unlocked and Fully Deployed

WHEN Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_UNLOCKED THE Check Tray Status Subfunction SHALL set Tray Lock Request to TRAY_UNLOCKED

| Requirement ID: 899 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

902 Tray Unlock Request is None and Fully Stowed

WHEN Tray Move Request is STOW AND Tray Lock Status is TRAY_LOCKED AND Tray Position Status is EITHER STOWED OR IN_MOTION THE Check Tray Status Subfunction SHALL set Tray Lock Request to NONE

| Requirement ID: 902 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

903 Tray Unlock Request is Unlocked and Tray Stowing

WHEN Tray Move Request is STOW AND Tray Lock Status is TRAY_LOCKED AND Tray Position Status is NEITHER STOWED NOR FAILED_TO_STOW THE Check Tray Status Subfunction SHALL set Tray Lock Request to TRAY_UNLOCKED

| Requirement ID: 903 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

904 Tray Unlock Request is Locking but Tray Stowed

WHEN Tray Move Request is STOW AND Tray Lock Status is TRAY_UNLOCKED AND Tray Position Status is STOWED THE Check Tray Status Subfunction SHALL set Tray Lock Request to NONE

| Requirement ID: 904 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

905 HMI Tray Locked With Position Error

WHEN Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_LOCKED  THE Check Tray Status Subfunction SHALL set Provide HMI Instructions is Tray Locked With Position Error

| Requirement ID: 905 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

906 HMI Tray Fully Deployed

WHEN Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_LOCKED AND Tray Position Status is IN_MOTION THE Check Tray Status Subfunction SHALL set Provide HMI Instructions is Tray Locked and Waiting to Unlock

| Requirement ID: 906 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

907 Provide HMI Tray Fully Deployed

WHEN Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_UNLOCKED AND Tray Position Status is DEPLOYED THE Check Tray Status Subfunction SHALL set Provide HMI Instructions is Tray Fully Deployed

| Requirement ID: 907 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

908 Provide HMI Tray Unlocked and Deploying

WHEN Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_UNLOCKED AND Tray Position Status is EITHER IN_MOTION OR STOWED THE Check Tray Status Subfunction SHALL set Provide HMI Instructions is Tray Unlocked and Deploying

| Requirement ID: 908 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

911 Provide HMI Tray Failed to Unlock and Going to Safe State

WHEN Tray Move Request is STOW AND Tray Lock Status is TRAY_LOCKED AND Tray Position Status is EITHER DEPLOYED OR IN_MOTION THE Check Tray Status Subfunction SHALL set Provide HMI Instructions is Tray Failed to Unlock and Going to Safe State

| Requirement ID: 911 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

912 Provide HMI Tray Fully Stowed

WHEN Tray Move Request is STOW AND Tray Lock Status is EITHER TRAY_LOCKED OR TRAY_UNLOCKED AND Tray Position Status is STOWED THE Check Tray Status Subfunction SHALL set Provide HMI Instructions is Tray Fully Stowed

| Requirement ID: 912 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

913 Provide HMI Tray Unlocked and Tray Stowing

WHEN Tray Move Request is STOW AND Tray Lock Status is TRAY_UNLOCKED AND Tray Position Status is EITHER DEPLOYED OR IN_MOTION THE Check Tray Status Subfunction SHALL set Provide HMI Instructions is Tray Unlocked and Tray Stowing

| Requirement ID: 913 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

914 Define Check Tray Status Function Output Tray Deploy Request

The Check Tray Status Function Output Tray Deploy Request shall define values NONE, STOW and DEPLOY.

| Requirement ID: 914 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

915 Check Tray Status Function Output Tray Lock Request Definition

The Check Tray Status Function Output Tray Lock Request Shall Define States NONE, TRAY_LOCKED and TRAY_FAILED_TO_LOCK

| Requirement ID: 915 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

916 Check Tray Status Output Tray Unlock Request State Definition

Check Tray Status Function Output Tray Unlock Request States SHALL define NONE, TRAY_UNLOCKED and TRAY_FAILED_TO_UNLOCK

| Requirement ID: 916 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

917 Check Tray Status Function Output Provide HMI Instructions Definition

Check Tray Status Function Output Provide HMI Instructions shall define States 'Tray Locked With Position Error, 'Tray Locked and Waiting to Unlock'', 'Tray Fully Deployed', 'Tray Unlocked and Deploying', 'Tray Failed to Lock and Halting Motion', 'Tray Failed to Lock and Going to Safe State', 'Tray Failed to Unlock and Going to Safe State', 'Tray Fully Stowed', 'Tray Unlocked and Tray Stowing'

| Requirement ID: 917 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

879 Tray Failed to Lock and Halting Motion

IF Tray Position Status is EITHER FAILED_TO_DEPLOY OR FAILED_TO_STOW THEN Check Tray Status Subfunction SHALL set Tray Deploy Request to NONE

| Requirement ID: 879 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

880 Tray Failed to Lock and Going to Safe State

IF Tray Lock Status is EITHER TRAY_FAILED_TO_LOCK OR TRAY_FAILED_TO_UNLOCK AND Tray Position Status is NEITHER FAILED_TO_DEPLOY NOR FAILED_TO_STOW THEN Check Tray Status Subfunction SHALL set Deploy Request to STOW

| Requirement ID: 880 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

884 Tray Lock Request is None with Position Error

IF Tray Lock Status is TRAY_LOCKED AND Tray Position Status is DEPLOYED THEN Check Tray Status Subfunction SHALL set Tray Lock Request to NONE

| Requirement ID: 884 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

888 Tray Lock Request is Failed to Lock and Go to Safe State

IF Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_FAILED_TO_LOCK THEN Check Tray Status Subfunction SHALL set Tray Lock Request to TRAY_FAILED_TO_LOCK

| Requirement ID: 888 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

889 Tray Unlock Request is Failed to Lock and Go to Safe State

IF Tray Move Request is DEPLOY AND Tray Lock Status is TRAY_FAILED_TO_UNLOCK THEN Check Tray Status Subfunction SHALL set Tray Lock Request to NONE

| Requirement ID: 889 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

893 Tray Locked Request Failed to Lock and Halt Motion

IF Tray Move Request is STOW AND Tray Lock Status is TRAY_UNLOCKED AND Tray Position Status is FAILED_TO_DEPLOY THEN Check Tray Status Subfunction SHALL set Tray Lock Request to NONE

| Requirement ID: 893 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

894 Tray Locked Request  to Lock and Halt Motion Again

IF Tray Move Request is STOW AND Tray Lock Status is TRAY_FAILED_TO_LOCK AND Tray Position Status is FAILED_TO_STOW THEN Check Tray Status Subfunction SHALL set Tray Lock Request to TRAY_FAILED_TO_LOCK

| Requirement ID: 894 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

895 Tray Locked Request Failed to Unlock and Tray Failed to Stow

IF Tray Move Request is STOW AND Tray Lock Status is TRAY_FAILED_TO_UNLOCK THEN Check Tray Status Subfunction SHALL set Tray Lock Request to NONE

| Requirement ID: 895 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

896 Tray Unlock Request is Tray Failed to Unlock with Position Error

IF Tray Lock Status is TRAY_LOCKED AND Tray Position Status is EITHER DEPLOYED OR FAILED_TO_DEPLOY THEN Check Tray Status Subfunction SHALL set Tray Unlock Request to TRAY_FAILED_TO_UNLOCK

| Requirement ID: 896 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

898 Tray Unlock Request is Locked But Failed to Stow

IF Tray Lock Status is TRAY_LOCKED AND Tray Position Status is FAILED_TO_STOW THEN Check Tray Status Subfunction SHALL set Tray Unlock Request to NONE

| Requirement ID: 898 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

900 Tray Unlock Request is Tray Failed to Lock and Go to Safe State

IF Tray Lock Status is TRAY_FAILED_TO_LOCK  THEN Check Tray Status Subfunction SHALL set Tray Unlock Request to NONE

| Requirement ID: 900 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

901 Tray Unlock Request is Tray Failed to Unlock and Go to Safe State

IF Tray Lock Status is TRAY_FAILED_TO_UNLOCK  THEN Check Tray Status Subfunction SHALL set Tray Unlock Request to TRAY_FAILED_TO_UNLOCK

| Requirement ID: 901 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

909 Provide HMI Tray Failed to Lock and Halting Motion

IF Tray Position Status is EITHER FAILED_TO_DEPLOY OR FAILED_TO_STOW THEN Check Tray Status Subfunction SHALL set Provide HMI Instructions is Tray Failed to Lock and Halting Motion

| Requirement ID: 909 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

910 Provide HMI Tray Failed to Lock and Going to Safe State

IF Tray Lock Status is EITHER TRAY_FAILED_TO_LOCK OR TRAY_FAILED_TO_UNLOCK AND Tray Position Status is NEITHER FAILED_TO_DEPLOY NOR FAILED_TO_STOW THEN Check Tray Status Subfunction SHALL set Provide HMI Instructions is Tray Failed to Lock and Going to Safe State

| Requirement ID: 910 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Check vehicle status

### Function Overview

#### Description

Function is allocated to:

- Stowable Feature Controller <<Logical>>

- Verify Availability of Drive State <<Logical>>

- Verify Availability of Rest State <<Logical>>

- Verify Availabilty of Work State <<Logical>>

This Function checks the Vehicle status and outputs the Vehicle status Validity

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Check vehicle status” function is called by the following functions:

- – “Verify Feature Status”

Figure 2: Activity Diagram of   “Verify Feature Status” calling   “Check vehicle status”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| vehicle status Type: VehicleStatus | Signal Description: This Signal is the collectivity approval signal which includes Parking status, battery state of charge and Vehicle Speed Received from: - Activity Parameter Node: vehicle status |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Vehicle status Validity Type: VehicleStatusValidity | Signal Description: This is the signal which contains the validity of the Vehicle status before Arbitrating into any Mode Sent to: - Check Feature Availability |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

1077 Check Vehicle Status Function Vehicle Status Validity Definition

The Check Vehicle Status Function Vehicle Status Validity Definition

| Requirement ID: 1077 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1078 Check Vehicle Status Vehicle Status Validity Invalid Criteria

The Check Vehicle Status function shall set Vehicle Status Validity to INVALID when Vehicle Status is NOT set to PARKED.

| Requirement ID: 1078 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1079 Check Vehicle Status Vehicle Status Validity Valid Criteria

The Check Vehicle Status function shall set Vehicle Status Validity to VALID when Vehicle Status is set to PARKED.

| Requirement ID: 1079 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

1092 Vehicle Speed Inhibit

Stowable Feature Controller shall Inhibit Work,Play and Rest Modes if Vehicle Speed is not Idle

| Requirement ID: 1092 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Control SSC Functionality

### Function Overview

#### Description

Function is allocated to:

- Stowable Steering and Deployable Tray System <<Logical>>

When received State selection signal for mode transition , this function issues individual requests to Seat, steering Column and Tray Table after verifying the Inhibit status and Provide a cause of Failure if the transition is not possible

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Control SSC Functionality” function is called by the following functions:

- – “Drive to Rest”

- – “Make Rest space Available”

- – “Make Work Space Available”

- – “Request Work Mode”

Figure 2: Activity Diagram of   “Drive to Rest” calling   “Control SSC Functionality”

Figure 2: Activity Diagram of   “Make Rest space Available” calling   “Control SSC Functionality”

Figure 2: Activity Diagram of   “Make Work Space Available” calling   “Control SSC Functionality”

Figure 2: Activity Diagram of   “Request Work Mode” calling   “Control SSC Functionality”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Inhibit Status Type: InhibitStatus | Signal Description: Inhibit status of driver control which is verified by the feature systems Received from: - Activity Parameter Node: Inhibit Status - Verify Drive Control Status - Activity Parameter Node: Inhibit Status |
| Termination Signal Type: Termination Signal | Signal Description: Logical signal for termination of Mode Arbitration when system did not act according to requests Received from: - Check for Transition Termination |
| State Selection Signal Type: StateSelectionSignal | Signal Description: Signal which contains the Mode that User selected after the Vehicle status is Valid Received from: - Activity Parameter Node: state selection signal - Activity Parameter Node: State Selection Signal - Activity Parameter Node: State Selection Signal |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Seat movement Request Type: SeatMovementRequest | Signal Description: Command to Move seat which goes to the Driver seat System Sent to: - Request Seat Arbitration |
| SC Request Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Sent to: - Request Steering Column Arbitration |
| Tray Move request Type: TrayMoveRequest | Signal Description: Internal Request of Feature Controller to Move Tray Table Sent to: - Request Tray Arbitration |
| Cause of Failure Type: CauseOfFailure | Signal Description: Cause for which status is not appropriate in the vehicle to arbitrate the feature mode Sent to: - Manage Feature Arbitration Feedback |
| Publish state Type: StateOfFeature | Signal Description: Current state of Feature and other warnings Sent to: - Manage Feature Arbitration Feedback |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Control SSC Functionality

### Function Requirements

#### Functional Requirements

##### Normal Operation

No Normal Operation Requirements specified.

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Control Steering Column Functionality

### Function Overview

#### Description

Function is allocated to:

- Deploy Steering Column <<Logical>>

- Stow Steering Column <<Logical>>

- Stowable Feature Controller <<Logical>>

This subsystem function sends out actuate /Move commands for respective actuators and reads the signals from sensors

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Control Steering Column Functionality” function is called by the following functions:

- – “Actuate Steering Column”

Figure 2: Activity Diagram of   “Actuate Steering Column” calling   “Control Steering Column Functionality”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Steering Column Rake Position Type: SteeringColumnRakePosition | Signal Description: Steering Column Position in Rake Position Received from: - Provide steering column Rake position |
| input Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Received from: - Activity Parameter Node: Steering Column Request |
| Steering Column Tele Position Type: SteeringColumnTelePosition | Signal Description: Steering Column Position in Tele Position Received from: - Provide steering column Tele position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Stering Column Command Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Sent to: - Move SC in Tele Direction |
| Steering Column Command Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Sent to: - Move SC in Rake Direction |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

997 Control Steering Column Function Steering Column Comand Tele Output Definition

The Control Steering Column Functionality function output Steering Column Command Tele shall indicate NONE for halt, DEPLOY to deploy and STOW to stow.

| Requirement ID: 997 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

998 Control Steering Column Functionality function Steering Column Tele Command Deploy Case One

The Control Steering Column Functionality function  shall set Steering Column Tele Command to DEPLOY if Steering Column Tele is set to STOWED AND Steering Column Command is set to DEPLOY

| Requirement ID: 998 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

999 Control Steering Column Functionality function Steering Column Tele Command Halt Case One

The Control Steering Column Functionality function  shall set Steering Column Tele Command to NONE if Steering Column Tele is set to STOWED AND Steering Column Command is set to STOW

| Requirement ID: 999 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1000 Control Steering Column Functionality function Steering Column Tele Command Halt Case Two

The Control Steering Column Functionality function  shall set Steering Column Tele Command to NONE if Steering Column Command is set to NONE

| Requirement ID: 1000 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1001 Control Steering Column Functionality function Steering Column Tele Command Halt Case Three

The Control Steering Column Functionality function  shall set Steering Column Tele Command to NONE if Steering Column Tele is set to DEPLOYED AND Steering Column Command is set to DEPLOY

| Requirement ID: 1001 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1002 Control Steering Column Functionality function Steering Column Tele Command Stow Case One

The Control Steering Column Functionality function  shall set Steering Column Tele Command to STOW if Steering Column Tele is set to DEPLOYED AND Steering Column Command is set to STOW

| Requirement ID: 1002 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1003 Control Steering Column Functionality function Steering Column Tele Command Deploy Case Two

The Control Steering Column Functionality function  shall set Steering Column Tele Command to DEPLOY if Steering Column Tele is set to IN_MOTION AND Steering Column Command is set to DEPLOY

| Requirement ID: 1003 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1004 Control Steering Column Functionality function Steering Column Tele Command Stow Case Two

The Control Steering Column Functionality function  shall set Steering Column Tele Command to STOW if Steering Column Tele is set to IN_MOTION AND Steering Column Command is set to STOW

| Requirement ID: 1004 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1005 Control Steering Column Functionality function Steering Column Tele Command Error Case One

The Control Steering Column Functionality function  shall set Steering Column Tele Command to NONE if Steering Column Tele is set to ERROR OR Steering Column Command is set to DEPLOY

| Requirement ID: 1005 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1006 Control Steering Column Function Steering Column Comand Rake Output Definition

The Control Steering Column Functionality function output Steering Column Command Rake shall indicate NONE for halt, DEPLOY to deploy and STOW to stow.

| Requirement ID: 1006 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1007 Control Steering Column Function Steering Column Comand Rake DEPLOY case one

The Control Steering Column Functionality function  shall set Steering Column Command Rake to DEPLOY when Steering Column Rake Position is set to STOWED AND Steering Column Command is set to DEPLOY

| Requirement ID: 1007 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1008 Control Steering Column Function Steering Column Comand Rake Halt case one

The Control Steering Column Functionality function  shall set Steering Column Command Rake to NONE when Steering Column Rake Position is set to STOWED AND Steering Column Command is set to STOW

| Requirement ID: 1008 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1009 Control Steering Column Function Steering Column Comand Rake Halt case two

The Control Steering Column Functionality function  shall set Steering Column Command Rake to NONE when Steering Column Command is set to NONE

| Requirement ID: 1009 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1010 Control Steering Column Function Steering Column Comand Rake Halt case three

The Control Steering Column Functionality function  shall set Steering Column Command Rake to NONE when Steering Column Rake is set to DEPLOYED AND Steering Column Command is set to DEPLOY

| Requirement ID: 1010 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1011 Control Steering Column Function Steering Column Comand Rake STOW case one

The Control Steering Column Functionality function  shall set Steering Column Command Rake to STOW when Steering Column Rake is set to DEPLOYED AND Steering Column Command is set to STOW

| Requirement ID: 1011 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1012 Control Steering Column Function Steering Column Comand Rake DEPLOY case two

The Control Steering Column Functionality function  shall set Steering Column Command Rake to DEPLOY when Steering Column Rake is set to IN_MOTION AND Steering Column Command is set to DEPLOY

| Requirement ID: 1012 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1013 Control Steering Column Function Steering Column Comand Rake STOW case two

The Control Steering Column Functionality function  shall set Steering Column Command Rake to STOW when Steering Column Rake is set to IN_MOTION AND Steering Column Command is set to STOW

| Requirement ID: 1013 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1014 Control Steering Column Function Steering Column Comand Rake ERROR case two

The Control Steering Column Functionality function  shall set Steering Column Command Rake to NONE if Steering Column Rake is set to FAILED_RAKE_DEPLOY OR Steering Column Rake is set to FAILED_RAKE_STOW

| Requirement ID: 1014 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Control Tray Functionality

### Function Overview

#### Description

Function is allocated to:

- Command Tray Table Deploy <<Logical>>

- Command Tray Table Stow <<Logical>>

- Lock Tray Table <<Logical>>

- Stowable Feature Controller <<Logical>>

This function decides the Tray Actuation based on sensor inputs and Commands

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Control Tray Functionality” function is called by the following functions:

- – “Actuate Tray”

Figure 2: Activity Diagram of   “Actuate Tray” calling   “Control Tray Functionality”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Tray Position status Type: TrayPostionStatus | Signal Description: Tray Position status is the feedback for Tray position given by Tray position Sensor Received from: - Provide Tray Position |
| Move Tray Command Type: TrayMoveCommand | Signal Description: Tray Move Command is the command signal which has various Tray Control requests in it. Received from: - Activity Parameter Node: Move Tray Command |
| Tray Lock Status Type: TrayLockStatus | Signal Description: Tray lock status is given by Tray Lock sensor Received from: - Provide Tray Lock status |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Tray Lock Command Type: TrayMoveCommand | Signal Description: Tray Move Command is the command signal which has various Tray Control requests in it. Sent to: - Actuate Tray Locking |
| Tray Move Command Type: TrayMoveCommand | Signal Description: Tray Move Command is the command signal which has various Tray Control requests in it. Sent to: - Actuate Tray Movement |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

841 Control Tray Functionality function Tray Move Command Output Definition

Control Tray Functionality function Tray Move Command shall indicate DEPLOY, STOW and NONE or halt movement.

| Requirement ID: 841 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

842 Control Tray Functionality function Tray Move Command When Not Unlocked

The  Control Tray Functionality function shall set Tray Move Command to NONE when Tray Lock Status is NOT set to UNLOCKED

| Requirement ID: 842 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

843 Control Tray Functionality function Tray Move Command Halt Commanded

The  Control Tray Functionality function shall set Tray Move Command is NONE when Move Tray Command is set to NONE

| Requirement ID: 843 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

844 Control Tray Functionality function Tray Move Command Begin Normal Deployment

The  Control Tray Functionality function shall set Tray Move Command to DEPLOY when Tray Position is set to STOWED AND Tray Lock Status is set to UNLOCKED AND Move Tray Command is set to DEPLOY

| Requirement ID: 844 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

845 Control Tray Functionality function Tray Move Command Stow Complete

The  Control Tray Functionality function shall set Tray Move Command to NONE when Tray Position is set to STOWED AND Move Tray Command is set to STOW

| Requirement ID: 845 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

846 Control Tray Functionality function Tray Move Command Error Response

The  Control Tray Functionality function shall set Tray Move Command to NONE when Tray Lock Status is set to ERROR OR Move Tray Command is set to ERROR

| Requirement ID: 846 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

847 Control Tray Functionality function Tray Move Command Deployment Complete

The  Control Tray Functionality function shall set Tray Move Command to NONE when Tray Position is set to DEPLOYED AND Move Tray Command is set to DEPLOY

| Requirement ID: 847 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

848 Control Tray Functionality function Tray Move Command Begin Normal Stowing

The  Control Tray Functionality function shall set Tray Move Command to STOW when Tray Position is set to DEPLOYED AND Tray Lock Status is set to UNLOCKED AND Move Tray Command is set to STOW

| Requirement ID: 848 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

849 Control Tray Functionality function Tray Move Command Deploying In Progress

The  Control Tray Functionality function shall set Tray Move Command is set to DEPLOY when Tray Position is set to IN_MOTION AND Tray Lock Status is set to LOCKED AND Move Tray Command is set to DEPLOY

| Requirement ID: 849 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

850 Control Tray Functionality function Tray Move Command Stowing In Progress

The  Control Tray Functionality function shall set Tray Move Command to STOW if Tray Position is set to IN_MOTION AND Tray Lock Status is set to UNLOCKED AND Move Tray Command is set to STOW

| Requirement ID: 850 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

851 Tray Lock Command function Tray Lock Command Output Definition

Tray Lock Command function Tray Lock Command shall indicate LOCK, UNLOCK and NONE or un-powered, default position.

| Requirement ID: 851 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

852 Tray Lock Command function Tray Lock Command Start Normal Stowed to Deploy Unlock

The Control Tray Functionality function shall set Tray Lock Command to UNLOCK  when Tray Position is set to STOWED AND Tray Lock Status is NOT set to TRAY_FAILED_TO_LOCK AND Tray Lock Status is NOT set to TRAY_FAILED_TO_UNLOCK AND Move Tray Command is set to DEPLOY

| Requirement ID: 852 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

853 Tray Lock Command function Tray Lock Command Normal Locking

The Control Tray Functionality function shall set Tray Lock Command to LOCK when Tray Position is set to STOWED AND Tray Lock Status is NOT set to TRAY_FAILED_TO_LOCK AND Tray Lock Status is NOT set to TRAY_FAILED_TO_UNLOCK AND Move Tray Command is set to STOW

| Requirement ID: 853 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

854 Tray Lock Command function Tray Lock Command Maintain Lock

The Control Tray Functionality function shall set Tray Lock Command is set to LOCK when Tray Position is NOT set to FAILED_TO_DEPLOY AND Tray Position is NOT set to FAILED_TO_STOW AND Tray Lock Status is set to LOCKED AND Move Tray Command is set to NONE

| Requirement ID: 854 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

855 Tray Lock Command function Tray Lock Command Maintain Unlock

The Control Tray Functionality function shall set Tray Lock Command is set to UNLOCK if Tray Position is NOT set to  ERROR AND Tray Lock Status is set to UNLOCKED AND Move Tray Command is set to NONE

| Requirement ID: 855 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

857 Tray Lock Command function Tray Lock Command Maintain Lock on Deploy

The Control Tray Functionality function shall set Tray Lock Command is set to LOCK if Tray Position is set to DEPLOYED AND Tray Lock Status is NOT set to TRAY_FAILED_TO_LOCK AND Tray Lock Status is NOT set to TRAY_FAILED_TO_UNLOCK AND Move Tray Command is set to DEPLOY

| Requirement ID: 857 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

858 Tray Lock Command function Tray Lock Command Unlock to Stow

The Control Tray Functionality function shall set Tray Lock Command is set to UNLOCK if Tray Position is set to DEPLOYED AND Tray Lock Status is NOT set to TRAY_FAILED_TO_LOCK AND Tray Lock Status is NOT set to TRAY_FAILED_TO_UNLOCK AND Move Tray Command is set to STOW

| Requirement ID: 858 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

859 Tray Lock Command function Tray Lock Command Ignore Tray Position When Locked

The Control Tray Functionality function shall set Tray Lock Command is set to LOCK when Tray Position is set to IN_MOTION AND Tray Lock Status is set to LOCKED

| Requirement ID: 859 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

860 Tray Lock Command function Tray Lock Command Ignore Tray Position When Unlocked

The Control Tray Functionality function shall set Tray Lock Command to UNLOCK when Tray Position is set to IN_MOTION AND Tray Lock Status is set to UNLOCKED

| Requirement ID: 860 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

868 Tray Lock Status Normal Definition

The Tray Lock Status shall be identified as NORMAL when Tray Lock Status is neither set to TRAY_FAILED_TO_LOCK nor TRAY_FAILED_TO UNLOCK.

| Requirement ID: 868 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

856 Tray Lock Command function Tray Lock Command Error Response

The Control Tray Functionality function shall set Tray Lock Command is set to NONE if Tray Position is  ERROR OR Tray Lock Status is ERROR

| Requirement ID: 856 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Evaluate Drivability status

### Function Overview

#### Description

Function is allocated to:

- Stowable Feature Controller <<Logical>>

- Verify Availability of Drive State <<Logical>>

This function checks all the status ( Tray , Steering Column, Driver seat) before sending a request to enable the drive control( Locomotion and Driver Airbag)

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Evaluate Drivability status” function is called by the following functions:

- – “Request Drive Control DeInhibition”

Figure 2: Activity Diagram of   “Request Drive Control DeInhibition” calling   “Evaluate Drivability status”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Seat Position Type: SeatPosition | Signal Description: Current Seat Position from the Driver Seat System Received from: - Activity Parameter Node: Seat Posotion |
| Tray Lock Status Type: TrayLockStatus | Signal Description: Tray lock status is given by Tray Lock sensor Received from: - Activity Parameter Node: Tray Lock Status |
| Tray Lock Status Type: StateSelectionSignal | Signal Description: Signal which contains the Mode that User selected after the Vehicle status is Valid Received from: - Activity Parameter Node: state selection signal |
| Steering Column Position Type: SteeringColumnStatus | Signal Description: Position of the Steering Column provided by the steering column system Received from: - Activity Parameter Node: Steering Column Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Drivability Status Type: Drivability Status | Signal Description: The logical signal which validates all systems component status and has the calculated the validity to enter into drive mode Sent to: - Request Airbag Enabling - Request Locomotion Enabling |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

1065 Normal Disallow De-Inhibition

The Evaluate Drivability Status function shall set Drivability Status to DON’T_ALLOW_DC_DEINHIBITION when Tray Lock Status is set to NORMAL AND Steering Column Status is set to NORMAL AND Seat Position is NOT set to ERROR AND State Selection Signal is NOT set to STOW

| Requirement ID: 1065 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1108 Verify Deinhibit status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1067 Normal De-Inhibit on Stow

The Evaluate Drivability Status function shall set Drivability Status to DON’T_ALLOW_DC_DEINHIBITION when Tray Lock Status is set to NORMAL AND Steering Column Status is set to NORMAL AND Seat Position is NOT set to ERROR AND State Selection Signal is set to STOW

| Requirement ID: 1067 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1108 Verify Deinhibit status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

931 Do Not Allow De-Inhibition

The Evaluate Drivability Status function shall set Drivability Status to DON’T_ALLOW_DC_DEINHIBITION when Tray Lock Status is set to ERROR OR Steering Column Status is set to ERROR OR Seat Position is set to ERROR.

| Requirement ID: 931 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Issue Termination Signal

### Function Overview

#### Description

Function is allocated to:

- Stowable Feature Controller <<Logical>>

This subsystem function issues Terminate signal based on other status

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Issue Termination Signal” function is called by the following functions:

- – “Check for Transition Termination”

Figure 2: Activity Diagram of   “Check for Transition Termination” calling   “Issue Termination Signal”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Steering Column internal status Type: SteeringColumnStatusInternal | Signal Description: Status of SteeringColumn based on Feature Decisions and current steering Column Position Received from: - Activity Parameter Node: SC Status |
| Tray Status Type: TrayStatus | Signal Description: This status is decided and fed by the Feature systems based on Requests and Input feedbacks fed Received from: - Activity Parameter Node: Tray Status |
| Seat Movement status Internal Type: SeatMovementStatusInternal | Signal Description: Status of the Seat movement according to the current position and external position Received from: - Activity Parameter Node: Seat Movement status Internal |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Termination signal Type: Termination Signal | Signal Description: Logical signal for termination of Mode Arbitration when system did not act according to requests Sent to: - Activity Parameter Node: Termination Signal |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

1083 SSW Normal Operation Detection

WHEN Tray Status is NOT TRAY_FAILED_TO_STOW, TRAY_FAILED_TO_DEPLOY, TRAY_FAILED_TO_LOCK, TRAY_FAILED_TO_UNLOCK, OBSTACLE_DETECTED  AND SC Status is NOT STEERING_COLUMN_FAILED_TO_DEPOY, STEERING_COLUMN_FAILED_TO_STOW AND Seat Movement Status Internal is NOT SEAT_FAILED_TO_MOVE the IssueTerminationSignal function shall set Termination Signal NONE

| Requirement ID: 1083 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality - 1096 Tray Movement Functionality - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

1084 Tray Failed to Stow

IF Tray Status is either TRAY_FAILED_TO_STOW, TRAY_FAILED_TO_DEPOY, TRAY_FAILED_TO_LOCK, TRAY_FAILED_TO_UNLOCK, OBSTACLE_DETECTED THEN the IssueTerminationSignal function SHALL set Termination Signal to TERMINATE_TRAY

| Requirement ID: 1084 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1085 Steering Column Failed to Deploy

IF SC Status is either STEERING_COLUMN_FAILED_TO_DEPOY, STEERING_COLUMN_FAILED_TO_STOW THEN the IssueTerminationSignal function SHALL set Termination Signal TERMINATE_SC

| Requirement ID: 1085 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1086 Seat Failed to Move

IF Seat Movement Status Internal is SEAT_FAILED_TO_MOVE THEN the IssueTerminationSignal function SHALL set Termination Signal TERMINATE_SEAT

| Requirement ID: 1086 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Move SC in Rake Direction

### Function Overview

#### Description

Function is allocated to:

- Stowable Steering Rake Actuator <<Logical>>

A subsystem Function responsible for Moving Steering Column in Rake Direction

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Move SC in Rake Direction” function is called by the following functions:

- – “Actuate Steering Column”

Figure 2: Activity Diagram of   “Actuate Steering Column” calling   “Move SC in Rake Direction”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| input Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Received from: - Control Steering Column Functionality |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

1102 Steering Column Rake Movement Actuation

Steering Column Rake Actuator shall Actuate the steering column in Rake Direction as per Stowable Feature Controller's Request

| Requirement ID: 1102 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | For steering Column Movement |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1120 Actuate Steering Column |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Move SC in Tele Direction

### Function Overview

#### Description

Function is allocated to:

- Stowable Steering Tele Actuator <<Logical>>

A subsystem Function responsible for Moving Steering Column in Tele Direction

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Move SC in Tele Direction” function is called by the following functions:

- – “Actuate Steering Column”

Figure 2: Activity Diagram of   “Actuate Steering Column” calling   “Move SC in Tele Direction”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| input Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Received from: - Control Steering Column Functionality |

#### Logical Outputs

No Logical Outputs specified.

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

No Normal Operation Requirements specified.

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

#### Uncategorized Requirements

1103 Steering Column Tele Movement Actuation

Steering Column Tele Actuator shall Actuate the steering column in Tele Direction as per Stowable Feature Controller's Request

| Requirement ID: 1103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## provide cause of failure

### Function Overview

#### Description

Function is allocated to:

- Stowable Feature Controller <<Logical>>

Based on Termination signal Information ,the cause of failure is conveyed to the Driver through this function

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “provide cause of failure” function is called by the following functions:

- – “Control SSC Functionality”

Figure 2: Activity Diagram of   “Control SSC Functionality” calling   “provide cause of failure”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Termination signal Type: Termination Signal | Signal Description: Logical signal for termination of Mode Arbitration when system did not act according to requests Received from: - Activity Parameter Node: Termination Signal |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| cause of failure Type: CauseOfFailure | Signal Description: Cause for which status is not appropriate in the vehicle to arbitrate the feature mode Sent to: - Activity Parameter Node: Cause of Failure |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

1087 Cause of Failure Definition

The Cause of Failure output from the Provide Cause Of Failure function shall set Cause of Failure to EITHER TRAY_OBSTRUCTION_DETECTED OR SC_ERROR OR DRIVER_SEAT_ERROR OR NONE

| Requirement ID: 1087 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1088 Normal Tray Cause of Failure

When Termination Signal is NONE the Provide Cause Of Failure function shall set Cause of Failure to NONE

| Requirement ID: 1088 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

1089 Tray Obstruction Cause of Failure

If Termination Signal set to TERMINATE_TRAY THEN the Provide Cause Of Failure function shall set Cause of Failure to TRAY_OBSTRUCTION_DETECTED

| Requirement ID: 1089 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1090 Seat Cause of Failure

If Termination Signal set to TERMINATE_SEAT THEN the Provide Cause Of Failure function shall set Cause of Failure to DRIVER_SEAT_ERROR

| Requirement ID: 1090 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1091 Steering Column Cause of Failure

If Termination Signal set to TERMINATE_SC THEN the Provide Cause Of Failure function shall set Cause of Failure to SC_ERROR

| Requirement ID: 1091 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Provide HMI Feedback

### Function Overview

#### Description

Function is allocated to:

- Notify User to Low Sli SOC <<Logical>>

- Stowable Feature Controller <<Logical>>

This Function Checks the Vehicle Validity and state selection signal and provide User his selected Mode and Vehicle status Validity

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Provide HMI Feedback” function is called by the following functions:

- – “Verify Feature Status”

Figure 2: Activity Diagram of   “Verify Feature Status” calling   “Provide HMI Feedback”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Unavailabality Type: ModeAcceptenceStatus | Signal Description: This signal contains the information whats not available in the vehicle configuration Received from: - Check Feature Availability |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| HMI Feedback Type: HMIFeedback | Signal Description: Feedback given to HMI Sent to: - Activity Parameter Node: HMI Feedback |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

1080 Provide HMI Feedback function HMI Feedback Definition

The Provide HMI Feedback Function HMI Feedback shall indicate by an HMI message that the Stowable Steering Column and Deployable Tray feature is availavle  when HMI Feedback is set to VEHICLE_STATUS_MESSAGE and shall produce no HMI message when it is NO_VEHICLE_STATUS_MESSAGE

| Requirement ID: 1080 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1081 NO_VEHICLE_STATUS_MESSAGE

The Provide HMI Feedback Function shall set HMI_Feedback to NO_VEHICLE_STATUS_MESSAGE when Vehicle Status Validity is set to INVALID

| Requirement ID: 1081 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1082 Provide HMI Feedback function HMI Feedback Message

The Provide HMI Feedback Function shall set HMI_Feedback to VEHICLE_STATUS_MESSAGE when Vehicle Status Validity is set to VALID

| Requirement ID: 1082 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1112 Verify Feature Status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Provide steering column Rake position

### Function Overview

#### Description

Function is allocated to:

- Stowable Steering Rake Sensor <<Logical>>

Subsystem Function responsible for sending Steering Column Rake Position

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Provide steering column Rake position” function is called by the following functions:

- – “Actuate Steering Column”

Figure 2: Activity Diagram of   “Actuate Steering Column” calling   “Provide steering column Rake position”

### Function Interfaces

#### Logical Inputs

No Logical Inputs specified.

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Steering Column Rake Position Type: SteeringColumnRakePosition | Signal Description: Steering Column Position in Rake Position Sent to: - Activity Parameter Node: steering column rake position - Control Steering Column Functionality |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

1105 Provide Steering Column Rake Position

Steering Column Rake Sensor shall provide Steering Column Rake Position to Stowable Feature Controller

| Requirement ID: 1105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | For Steering Column Position |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1117 Steering Column Position |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Provide steering column Tele position

### Function Overview

#### Description

Function is allocated to:

- Steering Column Tele Sensor <<Logical>>

Subsystem Function responsible for sending Steering Column Tele Position

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Provide steering column Tele position” function is called by the following functions:

- – “Actuate Steering Column”

Figure 2: Activity Diagram of   “Actuate Steering Column” calling   “Provide steering column Tele position”

### Function Interfaces

#### Logical Inputs

No Logical Inputs specified.

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Steering Column Tele Position Type: SteeringColumnTelePosition | Signal Description: Steering Column Position in Tele Position Sent to: - Control Steering Column Functionality - Activity Parameter Node: steering column tele position |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

1106 Steering Column Tele Position

Steering Column Tele Sensor shall provide Steering Column Tele Position to Stowable Feature Controller

| Requirement ID: 1106 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | For Steering Column Position |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1117 Steering Column Position |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Provide Tray Lock status

### Function Overview

#### Description

Function is allocated to:

- Actuate Tray Locking Mechanism <<Logical>>

- Tray Lock Sensor <<Logical>>

This Function provides Tray Lock status from Tray Lock Sensor

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Provide Tray Lock status” function is called by the following functions:

- – “Actuate Tray”

Figure 2: Activity Diagram of   “Actuate Tray” calling   “Provide Tray Lock status”

### Function Interfaces

#### Logical Inputs

No Logical Inputs specified.

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Tray Lock Status Type: TrayLockStatus | Signal Description: Tray lock status is given by Tray Lock sensor Sent to: - Activity Parameter Node: Tray Lock Status - Control Tray Functionality |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

861 Provide Tray Lock Status function Tray Lock Status Output Definition

The Provide Tray Lock Status function Tray Lock Status shall indicate LOCKED, UNLOCKED TRAY_FAILED_TO_LOCK or TRAY_FAILED_TO_UNLOCK status for the Feature Tray Lock.

| Requirement ID: 861 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

863 Provide Tray Lock Status Error Defintion

The Provide Tray Lock Status function Tray Lock Status shall define TRAY_FAILED_TO_LOCK OR TRAY_FAILED_TO_UNLOCK as ERROR.

| Requirement ID: 863 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Provide Tray Position

### Function Overview

#### Description

Function is allocated to:

- Provide Cross car Tray Position <<Logical>>

- Tray Position Sensor <<Logical>>

This Function provides Tray position through Tray Position sensor

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Provide Tray Position” function is called by the following functions:

- – “Actuate Tray”

Figure 2: Activity Diagram of   “Actuate Tray” calling   “Provide Tray Position”

### Function Interfaces

#### Logical Inputs

No Logical Inputs specified.

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Tray Position Status Type: TrayPostionStatus | Signal Description: Tray Position status is the feedback for Tray position given by Tray position Sensor Sent to: - Activity Parameter Node: Tray Position Status - Control Tray Functionality |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

864 Provide Tray Position Tray Position Definition

The Provide Tray Position function Tray Position shall indicate STOWED, DEPLOYED, IN_MOTION, FAILED_TO_DEPLOY or FAILED_TO_STOW status for the Feature Tray Position sensor.

| Requirement ID: 864 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

865 Provide Tray Position Error Defintion

The Provide Tray Position function Tray Position shall define FAILED_TO_DEPLOY OR FAILED_TO_STOW as ERROR.

| Requirement ID: 865 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1096 Tray Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Provide Tray Stow Instructions

### Function Overview

#### Description

Function is allocated to:

- Command Tray Table Stow <<Logical>>

- Stowable Feature Controller <<Logical>>

A subsystem Function which provides instructions to HMI when User requests stow or to exit work mode

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Provide Tray Stow Instructions” function is called by the following functions:

- – “Request Tray Arbitration”

Figure 2: Activity Diagram of   “Request Tray Arbitration” calling   “Provide Tray Stow Instructions”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Provide HMI Instructions Type: HMIInsWar | Signal Description: HMI Instructions to Stow and Warning display if Tray is not pushed in properly Received from: - Check Tray Status |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| HMI instructions and warning Type: HMIInsWar | Signal Description: HMI Instructions to Stow and Warning display if Tray is not pushed in properly Sent to: - Activity Parameter Node: HMI instructions |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

944 Request Tray Stowing function Stow Tray Command Halt Due to Obstacle Detection

The Request Tray Stowing function Shall Set Stow Tray Command to NONE when Tray Obstruction Status NOT set to NO_OBSTACLE_DETECTED

| Requirement ID: 944 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

945 Request Tray Stowing function Stow Tray Command Successful Completion of Drive Request

The Request Tray Stowing function Shall Set Stow Tray Command to NONE when Tray Table Position is set to STOWED AND Tray Obstruction Status is set to NO_OBSTACLE_DETECTED

| Requirement ID: 945 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

946 Request Tray Stowing Function Stow Tray Command Normal Start of Stowing

The Request Tray Stowing function Shall Set Stow Tray Command is STOW when State Selection Signal is set to DRIVE AND Tray Table Position is set to DEPLOYED AND Tray Obstruction Status is set to NO_OBSTACLE_DETECTED

| Requirement ID: 946 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

947 Request Tray Stowing function Stow Tray Command Normal Progress of Stowing

The Request Tray Stowing function Shall Set Stow Tray Command is set to STOW when State Selection Signal is set to DRIVE AND Tray Table Position is set to IN_MOTION AND Tray Obstruction Status is set to NO_OBSTACLE_DETECTED

| Requirement ID: 947 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

951 Request Tray Stowing function Tray Status Stowed Definition

The Request Tray Stowing function Tray Status Stowed Output shall indicate Tray DEPLOYED, FAILED_TO_DEPLOY, STOWED, FAILED_TO_STOW, DEPLOYING and STOWING

| Requirement ID: 951 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

952 Request Tray Stowing function Tray Status Stowed

The Request Tray Stowing function Shall Set Tray Status to TRAY_STOWED when State Selection Signal is set to DRIVE and Tray Table Position is set to STOWED

| Requirement ID: 952 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

953 Request Tray Stowing function Tray Status Failed To Stow Case One

The Request Tray Stowing function Shall Set Tray Status to TRAY_FAILED_TO_STOW when State Selection Signal is set to DRIVE AND Tray Table Position NOT set to STOWED AND Tray Obstruction Status NOT set to NO_OBSTACLE_DETECTED

| Requirement ID: 953 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

954 Request Tray Stowing function Tray Status Stowing Case One

The Request Tray Stowing function Shall Set Tray Status to TRAY_STOWING when State Selection Signal is set to DRIVE AND Tray Table Position is set to DEPLOYED AND Tray Obstruction Status is set to NO_OBSTACLE_DETECTED

| Requirement ID: 954 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

955 Request Tray Stowing function Tray Status Failed To Stow Case Two

The Request Tray Stowing function Shall Set Tray Status to TRAY_STOWING when State Selection Signal is set to DRIVE AND Tray Table Position is set to IN_MOTION AND Tray Obstruction Status is set to NO_OBSTACLE_DETECTED

| Requirement ID: 955 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

956 Request Tray Stowing function Tray Status Failed To Deploy Case One

The Request Tray Deployment function Shall Set Tray Status to TRAY_FAILED_TO_DEPLOY when State Selection Signal is set to NOT_DRIVE AND Tray Table Position NOT set to DEPLOYED AND Tray Obstruction Status NOT set to NO_OBSTACLE_DETECTED

| Requirement ID: 956 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

957 Request Tray Stowing function Tray Status Deploying Case One

The Request Tray Deployment function Shall Set Tray Status to TRAY_DEPLOYING when State Selection Signal is set to NOT_DRIVE AND Tray Table Position is set to STOWED AND Tray Obstruction Status is set to NO_OBSTACLE_DETECTED

| Requirement ID: 957 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

958 Request Tray Stowing function Tray Status Deployed

The Request Tray Deployment function Shall Set Tray Status to TRAY_DEPLOYED when State Selection Signal is set to NOT_DRIVE AND Tray Table Position is set to DEPLOYED

| Requirement ID: 958 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

959 Request Tray Stowing function Tray Status Failed To Deploy Case Two

The Request Tray Deployment function Shall Set Tray Status to TRAY_DEPLOYING when State Selection Signal is set to NOT_ DRIVE AND Tray Table Position is set to IN_MOTION AND Tray Obstruction Status is set to NO_OBSTACLE_DETECTED

| Requirement ID: 959 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request Airbag Disabling

### Function Overview

#### Description

Function is allocated to:

- Request Airbag Inhibit <<Logical>>

- Stowable Feature Controller <<Logical>>

Depending upon State Selection Signal this function sends a request to Inhibit Airbag

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Airbag Disabling” function is called by the following functions:

- – “Request Drive Control Inhibition”

Figure 2: Activity Diagram of   “Request Drive Control Inhibition” calling   “Request Airbag Disabling”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| state selection signal Type: StateSelectionSignal | Signal Description: Signal which contains the Mode that User selected after the Vehicle status is Valid Received from: - Activity Parameter Node: State selection signal |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Request Inhibit Driver Airbag Type: DriverAirbagRequest | Signal Description: Request sent to the Driver Airbag system / Passive Restraint System for either Inhibit or Deinhibit Sent to: - Activity Parameter Node: Inhibit Driver Airbag |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

967 The Request Driver_Airbag Disabling function Request Inhibit Driver_Airbag Definition

The Request DRIVER_AIRBAG Disabling function shall set the Request Inhibit DRIVER_AIRBAG to INHIBIT_DRIVER_AIRBAG_REQUEST, DEINHIBIT_DRIVER_AIRBAG_REQUEST

| Requirement ID: 967 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1110 Provide Airbag Inhibit/ DeInhibit Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

968 Request Driver_Airbag Disabling De-Inhibit

The Request DRIVER_AIRBAG Disabling function shall set Request Inhibit DRIVER_AIRBAG to DEINHIBIT_DRIVER_AIRBAG_REQUEST when State Selection Signal is set to DRIVE.

| Requirement ID: 968 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1110 Provide Airbag Inhibit/ DeInhibit Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

969 Request Driver_Airbag Disabling Inhibit

The Request DRIVER_AIRBAG Disabling function shall set Request Inhibit DRIVER_AIRBAG to INHIBIT_DRIVER_AIRBAG_REQUEST when State Selection Signal is NOT set to DRIVE.

| Requirement ID: 969 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1110 Provide Airbag Inhibit/ DeInhibit Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

972 Request Driver_Airbag Enabling Inhibit

The Request DRIVER_AIRBAG Disabling function shall set Request Inhibit DRIVER_AIRBAG to INHIBIT_DRIVER_AIRBAG_REQUEST when State Selection Signal is NOT set to DRIVE.

| Requirement ID: 972 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1110 Provide Airbag Inhibit/ DeInhibit Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request Airbag Enabling

### Function Overview

#### Description

Function is allocated to:

- Request Driver Airbag DisInhibit <<Logical>>

- Stowable Feature Controller <<Logical>>

Depending upon State Selection Signal this function sends a request to DeInhibit Airbag

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Airbag Enabling” function is called by the following functions:

- – “Request Drive Control DeInhibition”

Figure 2: Activity Diagram of   “Request Drive Control DeInhibition” calling   “Request Airbag Enabling”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Drivability Status Type: Drivability Status | Signal Description: The logical signal which validates all systems component status and has the calculated the validity to enter into drive mode Received from: - Evaluate Drivability status |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| DeInhibit Airbag Release Request Type: DriverAirbagRequest | Signal Description: Request sent to the Driver Airbag system / Passive Restraint System for either Inhibit or Deinhibit Sent to: - Activity Parameter Node: DeInhibit Airbag Release Request |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

970 The Request Driver_Airbag Enabling function Request Inhibit Driver_Airbag Definition

The Request DRIVER_AIRBAG Enabling function shall set the Request Inhibit DRIVER_AIRBAG to INHIBIT_DRIVER_AIRBAG_REQUEST, DEINHIBIT_DRIVER_AIRBAG_REQUEST

| Requirement ID: 970 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1110 Provide Airbag Inhibit/ DeInhibit Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

971 Request Driver_Airbag Enabling De-Inhibit

The Request DRIVER_AIRBAG Enabling function shall set Request Inhibit DRIVER_AIRBAG to DEINHIBIT_DRIVER_AIRBAG_REQUEST when State Selection Signal is set to DRIVE.

| Requirement ID: 971 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1110 Provide Airbag Inhibit/ DeInhibit Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request Drive Control DeInhibition

### Function Overview

#### Description

Function is allocated to:

- Stowable Steering and Deployable Tray System <<Logical>>

Depending upon State Selection Signal this function sends a request to DeInhibit Drive Control

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Drive Control DeInhibition” function is called by the following functions:

- – “Make Rest space Available”

- – “Make Work Space Available”

- – “Request Drive Control Arbitration”

- – “Request Drive Control DeInhibition”

- – “Request Drive Control DeInhibition”

- – “Request Drive Control DeInhibition”

- – “Request Drive Control DeInhibition”

Figure 2: Activity Diagram of   “Make Rest space Available” calling   “Request Drive Control DeInhibition”

Figure 2: Activity Diagram of   “Make Work Space Available” calling   “Request Drive Control DeInhibition”

Figure 2: Activity Diagram of   “Request Drive Control Arbitration” calling   “Request Drive Control DeInhibition”

Figure 2: Activity Diagram of   “Request Drive Control DeInhibition” calling   “Request Drive Control DeInhibition”

Figure 2: Activity Diagram of   “Request Drive Control DeInhibition” calling   “Request Drive Control DeInhibition”

Figure 2: Activity Diagram of   “Request Drive Control DeInhibition” calling   “Request Drive Control DeInhibition”

Figure 2: Activity Diagram of   “Request Drive Control DeInhibition” calling   “Request Drive Control DeInhibition”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| state selection signal Type: StateSelectionSignal | Signal Description: Signal which contains the Mode that User selected after the Vehicle status is Valid Received from: - Activity Parameter Node: state selection signal - Activity Parameter Node: State Selection Signal - Activity Parameter Node: state selection signal |
| Tray Lock Status Type: TrayLockStatus | Signal Description: Tray lock status is given by Tray Lock sensor Received from: - Activity Parameter Node: Tray Lock status - Actuate Tray |
| Seat Posotion Type: SeatPosition | Signal Description: Current Seat Position from the Driver Seat System Received from: - Activity Parameter Node: Seat Position - Activity Parameter Node: Seat Position - Activity Parameter Node: Seat Position |
| Steering Column Position Type: SteeringColumnStatus | Signal Description: Position of the Steering Column provided by the steering column system |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| DeInhibit Airbag Release Request Type: DriverAirbagRequest | Signal Description: Request sent to the Driver Airbag system / Passive Restraint System for either Inhibit or Deinhibit Sent to: - Activity Parameter Node: Driver Airbag Request - Activity Parameter Node: Driver Airbag Request - Activity Parameter Node: Driver Airbag Request |
| Deinhibit Locomotion Request Type: LocomotionRequest | Signal Description: Request to Powertrain / Locomotion to Inhibit /Deinhibit Sent to: - Activity Parameter Node: Locomotion Request - Activity Parameter Node: Locomotion Request - Activity Parameter Node: Locomotion Request |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Request Drive Control DeInhibition

### Function Requirements

#### Functional Requirements

##### Normal Operation

1114 Request Drive Control DeInhition

When Request Drive Control DeInhibition function receives StateSelectionSignal = DRIVE, it shall verify SeatPosition= AT_DRIVE , TrayStatus= LOCKED , SteeringColumnStatus= Deployed and then set

LocomotionRequest=DEINHIBIT_LOCOMOTION_REQUEST

DriverAirbagRequest=DEINHIBIT_DRIVER_AIRBAG_REQUEST

| Requirement ID: 1114 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request Drive Control Inhibition

### Function Overview

#### Description

Function is allocated to:

- Stowable Steering and Deployable Tray System <<Logical>>

Depending upon State Selection Signal this function sends a request to Inhibit Drive Control

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Drive Control Inhibition” function is called by the following functions:

- – “Inhibit and Verify Drive Control”

- – “Make Rest space Available”

- – “Make Work Space Available”

- – “Request Drive Control Arbitration”

- – “Request Drive Control Inhibition”

Figure 2: Activity Diagram of   “Inhibit and Verify Drive Control” calling   “Request Drive Control Inhibition”

Figure 2: Activity Diagram of   “Make Rest space Available” calling   “Request Drive Control Inhibition”

Figure 2: Activity Diagram of   “Make Work Space Available” calling   “Request Drive Control Inhibition”

Figure 2: Activity Diagram of   “Request Drive Control Arbitration” calling   “Request Drive Control Inhibition”

Figure 2: Activity Diagram of   “Request Drive Control Inhibition” calling   “Request Drive Control Inhibition”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| State selection signal Type: StateSelectionSignal | Signal Description: Signal which contains the Mode that User selected after the Vehicle status is Valid Received from: - Activity Parameter Node: state selection signal - Activity Parameter Node: State Selection signal - Activity Parameter Node: State Selection Signal - Activity Parameter Node: state selection signal |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Request Inhibit Locomotion Type: LocomotionRequest | Signal Description: Request to Powertrain / Locomotion to Inhibit /Deinhibit Sent to: - Activity Parameter Node: Locomotion request - Activity Parameter Node: Locomotion Request - Activity Parameter Node: Locomotion Request - Activity Parameter Node: Locomotion Request |
| Request Inhibit Driver Airbag Type: DriverAirbagRequest | Signal Description: Request sent to the Driver Airbag system / Passive Restraint System for either Inhibit or Deinhibit Sent to: - Activity Parameter Node: Driver Airbag Request - Activity Parameter Node: Driver Airbag Request - Activity Parameter Node: Driver Airbag Request - Activity Parameter Node: Driver Airbag Request |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Request Drive Control Inhibition

### Function Requirements

#### Functional Requirements

##### Normal Operation

1113 Request Drive Control Inhibition

When Request Drive Control Inhibition function receives StateSelectionSignal = WORK

or  StateSelectionSignal = REST or StateSelectionSignal = PLAY, it shall set LocomotionRequest=INHIBIT_LOCOMOTION_REQUEST

DriverAirbagRequest=INHIBIT_DRIVER_AIRBAG_REQUEST

| Requirement ID: 1113 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request Locomotion Disabling

### Function Overview

#### Description

Function is allocated to:

- Request Locomotion Inhibit <<Logical>>

- Stowable Feature Controller <<Logical>>

Depending upon State Selection Signal this function sends a request to Inhibit Locomotion

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Locomotion Disabling” function is called by the following functions:

- – “Request Drive Control Inhibition”

Figure 2: Activity Diagram of   “Request Drive Control Inhibition” calling   “Request Locomotion Disabling”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| state selection signal Type: StateSelectionSignal | Signal Description: Signal which contains the Mode that User selected after the Vehicle status is Valid Received from: - Activity Parameter Node: State selection signal |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Request Inhibit Locomotion Type: LocomotionRequest | Signal Description: Request to Powertrain / Locomotion to Inhibit /Deinhibit Sent to: - Activity Parameter Node: Request Inhibit Locomotion |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

963 The Request Locomotion Disabling function Request Inhibit Locomotion Definition

The Request Locomotion Disabling function shall set the Request Inhibit Locomotion to INHIBIT_LOCOMOTION_REQUEST, DEINHIBIT_LOCOMOTION_REQUEST

| Requirement ID: 963 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1109 Provide Locomotion Inhibit/Deinhibit Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

965 Request Locomotion Disabling Inhibit

The Request Locomotion Disabling function shall set Request Inhibit Locomotion to INHIBIT_LOCOMOTION_REQUEST when State Selection Signal is NOT set to DRIVE.

| Requirement ID: 965 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1109 Provide Locomotion Inhibit/Deinhibit Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request Locomotion Enabling

### Function Overview

#### Description

Function is allocated to:

- Request Locomotion Inhibit Release <<Logical>>

- Stowable Feature Controller <<Logical>>

Depending upon State Selection Signal this function sends a request to DeInhibit Locomotion

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Locomotion Enabling” function is called by the following functions:

- – “Request Drive Control DeInhibition”

Figure 2: Activity Diagram of   “Request Drive Control DeInhibition” calling   “Request Locomotion Enabling”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Drivability Status Type: Drivability Status | Signal Description: The logical signal which validates all systems component status and has the calculated the validity to enter into drive mode Received from: - Evaluate Drivability status |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| DeiInhibit Locomotion Request Type: LocomotionRequest | Signal Description: Request to Powertrain / Locomotion to Inhibit /Deinhibit Sent to: - Activity Parameter Node: Deinhibit Locomotion Request |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

960 The Request Locomotion Enabling function Request Inhibit Locomotion Definition

The Request Locomotion Enabling function shall set the Request Inhibit Locomotion to INHIBIT_LOCOMOTION_REQUEST, DEINHIBIT_LOCOMOTION_REQUEST

| Requirement ID: 960 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1113 Request Drive Control Inhibition |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

961 Request Locomotion Enabling Inhibit

The Request Locomotion Disabling function shall set Request Inhibit Locomotion to INHIBIT_LOCOMOTION_REQUEST when State Selection Signal is NOT set to DRIVE.

| Requirement ID: 961 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1113 Request Drive Control Inhibition |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

962 Request Locomotion Enabling De-Inhibit

The Request Locomotion Enabling function shall set Request Inhibit Locomotion to DEINHIBIT_LOCOMOTION_REQUEST when State Selection Signal is set to DRIVE.

| Requirement ID: 962 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1114 Request Drive Control DeInhition |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

964 Request Locomotion Disabling De-Inhibit

The Request Locomotion Disabling function shall set Request Inhibit Locomotion to DEINHIBIT_LOCOMOTION_REQUEST when State Selection Signal is set to DRIVE.

| Requirement ID: 964 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1109 Provide Locomotion Inhibit/Deinhibit Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request Seat Arbitration

### Function Overview

#### Description

Function is allocated to:

- Stowable Steering and Deployable Tray System <<Logical>>

This System Function responsible for preparing the Seat to Arbitrate and send the respective request to Driver Seat System

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Seat Arbitration” function is called by the following functions:

- – “Drive to Rest”

- – “From work to drive mode”

- – “Make Rest space Available”

- – “Make Work Space Available”

- – “Request Work Mode”

- – “Rest to Drive”

- – “Rest to Work”

- – “Work to Rest”

Figure 2: Activity Diagram of   “Drive to Rest” calling   “Request Seat Arbitration”

Figure 2: Activity Diagram of   “From work to drive mode” calling   “Request Seat Arbitration”

Figure 2: Activity Diagram of   “Make Rest space Available” calling   “Request Seat Arbitration”

Figure 2: Activity Diagram of   “Make Work Space Available” calling   “Request Seat Arbitration”

Figure 2: Activity Diagram of   “Request Work Mode” calling   “Request Seat Arbitration”

Figure 2: Activity Diagram of   “Rest to Drive” calling   “Request Seat Arbitration”

Figure 2: Activity Diagram of   “Rest to Work” calling   “Request Seat Arbitration”

Figure 2: Activity Diagram of   “Work to Rest” calling   “Request Seat Arbitration”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Seat Movement Request Type: SeatMovementRequest | Signal Description: Command to Move seat which goes to the Driver seat System Received from: - Activity Parameter Node: Seat Movement Request - Control SSC Functionality - Activity Parameter Node: Seat movement Request - Activity Parameter Node: Seat movement Request - Activity Parameter Node: Seat Movement Request - Activity Parameter Node: Seat movement Request - Activity Parameter Node: Seat movement Request |
| Seat Position Type: SeatPosition | Signal Description: Current Seat Position from the Driver Seat System Received from: - Activity Parameter Node: Seat Position - Activity Parameter Node: Seat Position - Activity Parameter Node: Seat Position - Activity Parameter Node: Seat Position - Activity Parameter Node: Seat Position - Activity Parameter Node: Seat Position - Activity Parameter Node: Seat Position - Activity Parameter Node: Seat Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Seat Movement Command Type: SeatMovementRequest | Signal Description: Command to Move seat which goes to the Driver seat System Sent to: - Activity Parameter Node: Seat Movement Command - Activity Parameter Node: Seat movement request - Activity Parameter Node: Seat Movement Command - Activity Parameter Node: Seat Movement Command - Activity Parameter Node: Seat Movement Command - Activity Parameter Node: Seat Movement Command - Activity Parameter Node: Seat movement request - Activity Parameter Node: Seat Movement Command |
| Seat Movement Status Internal Type: SeatMovementStatusInternal | Signal Description: Status of the Seat movement according to the current position and external position Sent to: - Check for Transition Termination |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Arbitrate Seat Position

### Function Requirements

#### Functional Requirements

##### Normal Operation

1101 Drive seat PreSet Positions

Driver seat system shall support 4 positions for different Modes of Operation ; Drive Mode seat position , Play Mode seat Position , Work Mode seat position and Rest Mode seat position

| Requirement ID: 1101 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1102 Request Seat Arbitration

When Request Seat Arbitration Function receives state selection signal , it shall set Seat Movement Command as follows

1. StateSelectionSignal==WORK , SeatMovementCommand=WORK

2.StateSelectionSignal==PLAY , SeatMovementCommand=PLAY

3.StateSelectionSignal==REST , SeatMovementCommand=REST

4.StateSelectionSignal==DRIVE , SeatMovementCommand=DRIVE

| Requirement ID: 1102 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1107 Set Seat Movement status

While Activated, Request Seat Arbitration function shall Monitor the Seat Position through SeatPosition and set SeatMovementStatus to

1.SEAT_MOVED

2.SEAT_MOVING

3.SEAT_FAILED_TO_MOVE

| Requirement ID: 1107 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request Steering Column Arbitration

### Function Overview

#### Description

Function is allocated to:

- Stowable Steering and Deployable Tray System <<Logical>>

A System Function responsible for preparing the Steering Column to Arbitrate and send the respective command to Arbitrate Steering Column to next functions

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Steering Column Arbitration” function is called by the following functions:

- – “Drive to Rest”

- – “From work to drive mode”

- – “Make Rest space Available”

- – “Make Work Space Available”

- – “Request Work Mode”

- – “Rest to Drive”

- – “Rest to Work”

- – “Work to Rest”

Figure 2: Activity Diagram of   “Drive to Rest” calling   “Request Steering Column Arbitration”

Figure 2: Activity Diagram of   “From work to drive mode” calling   “Request Steering Column Arbitration”

Figure 2: Activity Diagram of   “Make Rest space Available” calling   “Request Steering Column Arbitration”

Figure 2: Activity Diagram of   “Make Work Space Available” calling   “Request Steering Column Arbitration”

Figure 2: Activity Diagram of   “Request Work Mode” calling   “Request Steering Column Arbitration”

Figure 2: Activity Diagram of   “Rest to Drive” calling   “Request Steering Column Arbitration”

Figure 2: Activity Diagram of   “Rest to Work” calling   “Request Steering Column Arbitration”

Figure 2: Activity Diagram of   “Work to Rest” calling   “Request Steering Column Arbitration”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Steering Column Tele Position Type: SteeringColumnTelePosition | Signal Description: Steering Column Position in Tele Position Received from: - Actuate Steering Column |
| Steering Column Rake Position Type: SteeringColumnRakePosition | Signal Description: Steering Column Position in Rake Position Received from: - Actuate Steering Column |
| SC Request Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Received from: - Activity Parameter Node: Steering Column Request - Activity Parameter Node: Steering Column Request - Activity Parameter Node: Steering Column Request - Control SSC Functionality - Activity Parameter Node: SteeringColumn request - Activity Parameter Node: Steering Column Request - Activity Parameter Node: SteeringColumn request |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Move Steering Column Command Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Sent to: - Activity Parameter Node: steeringcolumn request - Activity Parameter Node: Steering Column request - Activity Parameter Node: Steering Column request - Activity Parameter Node: steeringcolumn request - Activity Parameter Node: steeringcolumn request - Actuate Steering Column - Activity Parameter Node: Steering Column Command - Activity Parameter Node: steeringcolumn request |
| SteeringColumn Status Type: SteeringColumnStatusInternal | Signal Description: Status of SteeringColumn based on Feature Decisions and current steering Column Position Sent to: - Activity Parameter Node: SteeringColumn Status - Activity Parameter Node: SteeringColumn Status - Check for Transition Termination - Activity Parameter Node: SteeringColumn Status - Activity Parameter Node: SteeringColumn Status - Activity Parameter Node: Steering Column Status - Activity Parameter Node: Steering Column Status |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Request Steering Column Arbitration

### Function Requirements

#### Functional Requirements

##### Normal Operation

1105 Request Steering Column Arbitration

When Request Steering Column Arbitration function receives StateSelectionSignal = WORK or StateSelectionSignal = PLAY or StateSelectionSignal = REST , it shall verify

SteeringColumnRakePosition=STOWED , SteeringColumnTelePosition=STOWED it shall set SteeringColumnCommand = DEPLOY_RAKE and SteeringColumnCommand = DEPLOY_DEPLOY and SteeringColumnStatus=STEERING_COLUMN_DEPLOYING

| Requirement ID: 1105 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request SteeringColumn Deployment

### Function Overview

#### Description

Function is allocated to:

- Deploy Steering Column <<Logical>>

- Stowable Feature Controller <<Logical>>

This Sub System Function is responsible for sending Steering Column Stow Request based on its Inputs requests

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request SteeringColumn Deployment” function is called by the following functions:

- – “Request Steering Column Arbitration”

Figure 2: Activity Diagram of   “Request Steering Column Arbitration” calling   “Request SteeringColumn Deployment”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Steering Column Request Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Received from: - Activity Parameter Node: SC Request |
| Steering Column Rake position Type: SteeringColumnRakePosition | Signal Description: Steering Column Position in Rake Position Received from: - Activity Parameter Node: Steering Column Rake Position |
| Steering Column Tele Position Type: SteeringColumnTelePosition | Signal Description: Steering Column Position in Tele Position Received from: - Activity Parameter Node: Steering Column Tele Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| steering column deploy status Type: SteeringColumnStatusInternal | Signal Description: Status of SteeringColumn based on Feature Decisions and current steering Column Position Sent to: - Activity Parameter Node: SteeringColumn Status |
| Deploy Steering Column Command Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Sent to: - Activity Parameter Node: Move Steering Column Command |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

991 Request Actuate Function Actuate Steering Column Command Definition

The Request Actuation Function Actuate Steering Column Command shall indicate DEPLOY, STOW and NONE states.

| Requirement ID: 991 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

992 Request Actuation Function Actuate Steering Column Command

The Request Actuation Function Actuate Steering Column Command shall be set to the Move Steering Column Command input when the Actuate Signal is set to ACTUATE

| Requirement ID: 992 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

993 Request Actuation Function Actuate Steering Column Command Default

The Request Actuation Function Actuate Steering Column Command shall be set to DEPLOY when the Actuate Signal is set to TERMINATE

| Requirement ID: 993 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1017 Execute Request SteeringColumn Deployment Function

The Request SteeringColumn Deployment function shall execute only when State Selection Signal is set to DRIVE.

| Requirement ID: 1017 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1038 Request SteeringColumn Deployment Function Deploy Steering Column Command Movement Starting

The Request SteeringColumn Deployment Function shall set Deploy Steering Column Command to DEPLOY when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to STOWED and Steering Column Rake Position is set to STOWED.

| Requirement ID: 1038 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1039 Request SteeringColumn Deployment Function Deploy Steering Column Command Tele Position Error

The Request SteeringColumn Deployment Function shall set Deploy Steering Column Command to NONE when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to STOWED and Steering Column Rake Position is set to DEPLOYED.

| Requirement ID: 1039 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1040 Request SteeringColumn Deployment Function Deploy Steering Column Command Deployment Commenced

The Request SteeringColumn Deployment Function shall set Deploy Steering Column Command to DEPLOY when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to STOWED and Steering Column Rake Position is set to IN_MOTION.

| Requirement ID: 1040 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1041 Request SteeringColumn Deployment Function Deploy Steering Column Command Position Error

The Request SteeringColumn Deployment Function shall set Deploy Steering Column Command to NONE when Steering Column Tele Position is set to ERROR or Steering Column Rake Position is set to ERROR

| Requirement ID: 1041 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1042 Request SteeringColumn Deployment Function Deploy Steering Column Command Rake Position Error

The Request SteeringColumn Deployment Function shall set Deploy Steering Column Command to NONE when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to STOWED

| Requirement ID: 1042 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1043 Request SteeringColumn Deployment Function Deploy Steering Column Command Deployment Completed

The Request SteeringColumn Deployment Function shall set Deploy Steering Column Command to NONE when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to DEPLOYED.

| Requirement ID: 1043 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1044 Request SteeringColumn Deployment Function Deploy Steering Column Command Deployment Completing

The Request SteeringColumn Deployment Function shall set Deploy Steering Column Command to DEPLOYT when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to IN_MOTION.

| Requirement ID: 1044 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1045 Request SteeringColumn Deployment Function Deploy Steering Column Command Commence Deploy

The Request SteeringColumn Deployment Function shall set Deploy Steering Column Command to DEPLOY when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to IN_MOTION.

| Requirement ID: 1045 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1046 Request SteeringColumn Deployment Function Deploy Steering Column Command Start Deployment

The Request SteeringColumn Deployment Function shall set Deploy Steering Column Command to DEPLOY when Inhibit Status is set to NOT_INHIBITED and both Steering Column Tele Position and Steering Column Rake Position is set to DEPLOYED or either  Steering Column Tele Position or Steering Column Rake Positionis is set to ERROR.

| Requirement ID: 1046 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1047 Request SteeringColumn Deployment Function Deploy Steering Column Command Normal Deployment Completion

The Request SteeringColumn Deployment Function shall set Deploy Steering Column Command to NONE when Inhibit Status is set to NOT_INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to DEPLOYED.

| Requirement ID: 1047 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1049 Request SteeringColumn Deployment Function Steering Column Deploy Status Movement Starting

The Request SteeringColumn Deployment Function shall set Steering Column Deploy Status to STEERING_COLUMN_DEPLOYING when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to STOWED and Steering Column Rake Position is set to STOWED.

| Requirement ID: 1049 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1050 Request SteeringColumn Deployment Function Steering Column Deploy Status Tele Position Error

The Request SteeringColumn Deployment Function shall set Steering Column Deploy Status to STEERING_COLUMN_FAILED_TO_DEPLOY when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to STOWED and Steering Column Rake Position is set to DEPLOYED.

| Requirement ID: 1050 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1051 Request SteeringColumn Deployment Function Steering Column Deploy Status Deployment Commenced

The Request SteeringColumn Deployment Function shall set Steering Column Deploy Status to STEERING_COLUMN_DEPLOYING when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to STOWED and Steering Column Rake Position is set to IN_MOTION.

| Requirement ID: 1051 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1052 Request SteeringColumn Deployment Function Steering Column Deploy Status Position Error

The Request SteeringColumn Deployment Function shall set Steering Column Deploy Status to STEERING_COLUMN_FAILED_TO_DEPLOY when Steering Column Tele Position is set to ERROR or Steering Column Rake Position is set to ERROR

| Requirement ID: 1052 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1053 Request SteeringColumn Deployment Function Steering Column Deploy Status Rake Position Error

The Request SteeringColumn Deployment Function shall set Steering Column Deploy Status to STEERING_COLUMN_FAILED_TO_DEPLOY when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to STOWED

| Requirement ID: 1053 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1054 Request SteeringColumn Deployment Function Steering Column Deploy Status Deployment Completed

The Request SteeringColumn Deployment Function shall set Steering Column Deploy Status to STEERING_COLUMN_DEPLOYED when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to DEPLOYED.

| Requirement ID: 1054 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1055 Request SteeringColumn Deployment Function Steering Column Deploy Status Deployment Completing

The Request SteeringColumn Deployment Function shall set Steering Column Deploy Status to STEERING_COLUMN_DEPLOYING when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to IN_MOTION.

| Requirement ID: 1055 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1056 Request SteeringColumn Deployment Function Steering Column Deploy Status Commence Deploy

The Request SteeringColumn Deployment Function shall set Steering Column Deploy Status to STEERING_COLUMN_DEPLOYING when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to IN_MOTION.

| Requirement ID: 1056 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1057 Request SteeringColumn Deployment Function Steering Column Deploy Status Start Deployment

The Request SteeringColumn Deployment Function shall set Steering Column Deploy Status to STEERING_COLUMN_DEPLOYING when Inhibit Status is set to NOT_INHIBITED and both Steering Column Tele Position and Steering Column Rake Position is set to DEPLOYED or either  Steering Column Tele Position or Steering Column Rake Positionis is set to ERROR.

| Requirement ID: 1057 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1058 Request SteeringColumn Deployment Function Steering Column Deploy Status Normal Deployment Completion

The Request SteeringColumn Deployment Function shall set Steering Column Deploy Status to STEERING_COLUMN_DEPLOYED when Inhibit Status is set to NOT_INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to DEPLOYED.

| Requirement ID: 1058 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request SteeringColumn Stowing

### Function Overview

#### Description

Function is allocated to:

- Stow Steering Column <<Logical>>

- Stowable Feature Controller <<Logical>>

This Sub System Function is responsible for sending Steering Column Stow Request based on its Inputs requests

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request SteeringColumn Stowing” function is called by the following functions:

- – “Request Steering Column Arbitration”

Figure 2: Activity Diagram of   “Request Steering Column Arbitration” calling   “Request SteeringColumn Stowing”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Steering Column Request Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Received from: - Activity Parameter Node: SC Request |
| Steering Column Rake Position Type: SteeringColumnRakePosition | Signal Description: Steering Column Position in Rake Position Received from: - Activity Parameter Node: Steering Column Rake Position |
| Steering Column Tele Position Type: SteeringColumnTelePosition | Signal Description: Steering Column Position in Tele Position Received from: - Activity Parameter Node: Steering Column Tele Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| steering column stow status Type: SteeringColumnStatusInternal | Signal Description: Status of SteeringColumn based on Feature Decisions and current steering Column Position Sent to: - Activity Parameter Node: SteeringColumn Status |
| Stow Steering Column Command Type: SteeringColumnRequest | Signal Description: Request to Move steering Column with respect to selected Mode Sent to: - Activity Parameter Node: Move Steering Column Command |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

991 Request Actuate Function Actuate Steering Column Command Definition

The Request Actuation Function Actuate Steering Column Command shall indicate DEPLOY, STOW and NONE states.

| Requirement ID: 991 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

992 Request Actuation Function Actuate Steering Column Command

The Request Actuation Function Actuate Steering Column Command shall be set to the Move Steering Column Command input when the Actuate Signal is set to ACTUATE

| Requirement ID: 992 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1015 Request SteeringColumn Stowing functions Stow Steering Column Command Definition

The Request SteeringColumn Stowing functions Stow Steering Column Command shall indicate DEPLOY, STOW and NONE.  NONE shall indicate a halt to movement.

| Requirement ID: 1015 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1016 Execute Request SteeringColumn Stowing Function

The Request SteeringColumn Stowing function shall execute only when State Selection Signal is set to NOT_DRIVE.

| Requirement ID: 1016 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1018 Request SteeringColumn Stowing Function Stow Steering Column Command Movement Done

The Request SteeringColumn Stowing Function shall set Stow Steering Column Command to NONE when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to STOWED and Steering Column Rake Position is set to STOWED.

| Requirement ID: 1018 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1019 Request SteeringColumn Stowing Function Stow Steering Column Command Rake Error

The Request SteeringColumn Stowing Function shall set Stow Steering Column Command to NONE when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to STOWED and Steering Column Rake Position is set to DEPLOYED.

| Requirement ID: 1019 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1020 Request SteeringColumn Stowing Function Stow Steering Column Command Stow Near Completion

The Request SteeringColumn Stowing Function shall set Stow Steering Column Command to STOW when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to STOWED and Steering Column Rake Position is set to IN_MOTION.

| Requirement ID: 1020 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1021 Request SteeringColumn Stowing Function Stow Steering Column Command Rake Position Error

The Request SteeringColumn Stowing Function shall set Stow Steering Column Command to NONE when Steering Column Tele Position is set to ERROR or Steering Column Rake Position is set to ERROR

| Requirement ID: 1021 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1022 Request SteeringColumn Stowing Function Stow Steering Column Command Tele Position Error

The Request SteeringColumn Stowing Function shall set Stow Steering Column Command to NONE when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to STOWED

| Requirement ID: 1022 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1023 Request SteeringColumn Stowing Function Stow Steering Column Command Begin Stowing

The Request SteeringColumn Stowing Function shall set Stow Steering Column Command to STOW when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to DEPLOYED.

| Requirement ID: 1023 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1024 Request SteeringColumn Stowing Function Stow Steering Column Command Rake Stow Commenced

The Request SteeringColumn Stowing Function shall set Stow Steering Column Command to STOW when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to IN_MOTION.

| Requirement ID: 1024 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1025 Request SteeringColumn Stowing Function Stow Steering Column Command Tele Stow Near Completion

The Request SteeringColumn Stowing Function shall set Stow Steering Column Command to STOW when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to IN_MOTION.

| Requirement ID: 1025 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1026 Request SteeringColumn Stowing Function Stow Steering Column Command DEPLOY Case

The Request SteeringColumn Stowing Function shall set Stow Steering Column Command to DEPLOY when Inhibit Status is set to NOT_INHIBITED and both Steering Column Tele Position and Steering Column Rake Position is set to DEPLOYED or either  Steering Column Tele Position or Steering Column Rake Positionis is set to ERROR.

| Requirement ID: 1026 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1027 Request SteeringColumn Stowing Function Stow Steering Column Command DEPLOYED Case

The Request SteeringColumn Stowing Function shall set Stow Steering Column Command to NONE when Inhibit Status is set to NOT_INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to DEPLOYED.

| Requirement ID: 1027 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1028 Request SteeringColumn Stowing Function Steering Column Stow Status Definition

The Request SteeringColumn Stowing functions Stow Steering Column Command shall indicate Steering Column DEPLOYED, FAILED_TO_DEPLOY, STOWED, FAILED_TO_STOW, DEPLOYING, STOWING

| Requirement ID: 1028 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1029 Request SteeringColumn Stowing Function Steering Column Stow Status Rake Error

The Request SteeringColumn Stowing Function shall set Steering Column Stow Status to STEERING_COLUMN_FAILED_TO_STOW when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to STOWED and Steering Column Rake Position is set to DEPLOYED.

| Requirement ID: 1029 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1032 Request SteeringColumn Stowing Function Steering Column Stow Status Failed to Stow Tele Position Error

The Request SteeringColumn Stowing Function shall set Steering Column Stow Status to STEERING_COLUMN_FAILED_TO_STOW when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to STOWED

| Requirement ID: 1032 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1033 Request SteeringColumn Stowing Function Steering Column Stow Status Begin Stowing

The Request SteeringColumn Stowing Function shall set Steering Column Stow Status to STEERING_COLUMN_STOWING when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to DEPLOYED.

| Requirement ID: 1033 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1034 Request SteeringColumn Stowing Function Steering Column Stow Status Rake Stow Commenced

The Request SteeringColumn Stowing Function shall set Steering Column Stow Status to STEERING_COLUMN_STOWING when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to IN_MOTION.

| Requirement ID: 1034 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1035 Request SteeringColumn Stowing Function Steering Column Stow Status Tele Stow Near Completion

The Request SteeringColumn Stowing Function shall set Steering Column Stow Status to STEERING_COLUMN_STOWING when Inhibit Status is set to INHIBITED and Steering Column Tele Position is set to IN_MOTION.

| Requirement ID: 1035 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1036 Request SteeringColumn Stowing Function Steering Column Stow Status Deploying

The Request SteeringColumn Stowing Function shall set Steering Column Stow Status to STEERING_COLUMN_DEPLOYING when Inhibit Status is set to NOT_INHIBITED and both Steering Column Tele Position and Steering Column Rake Position is set to DEPLOYED or either  Steering Column Tele Position or Steering Column Rake Positionis is set to ERROR.

| Requirement ID: 1036 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1037 Request SteeringColumn Stowing Function Steering Column Stow Status Deployed

The Request SteeringColumn Stowing Function shall set Steering Column Stow Status to STEERING_COLUMN_DEPLOYED when Inhibit Status is set to NOT_INHIBITED and Steering Column Tele Position is set to DEPLOYED and Steering Column Rake Position is set to DEPLOYED.

| Requirement ID: 1037 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1048 Request SteeringColumn Stowing Function Steering Column Deployment Status Definition

The Request SteeringColumn Stowing functions Stow Steering Column Command shall indicate Steering Column DEPLOYED, FAILED_TO_DEPLOY, STOWED, FAILED_TO_STOW, DEPLOYING, STOWING

| Requirement ID: 1048 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

1030 Request SteeringColumn Stowing Function Steering Column Stow Status Stow Near Completion

The Request SteeringColumn Stowing Function shall set Steering Column Stow Status to STEERING_COLUMN_FAILED_TO_STOW when Steering Column Tele Position is set to ERROR or Steering Column Rake Position is set to ERROR

| Requirement ID: 1030 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1031 Request SteeringColumn Stowing Function Steering Column Stow Status Failed to Stow Rake Position Error

The Request SteeringColumn Stowing Function shall set Steering Column Stow Status to STEERING_COLUMN_FAILED_TO_STOW when Steering Column Tele Position is set to ERROR or Steering Column Rake Position is set to ERROR

| Requirement ID: 1031 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - Column Movement Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request to Move Seat

### Function Overview

#### Description

Function is allocated to:

- Request Maneuver Driver Seat To Stored Position <<Logical>>

- Stowable Feature Controller <<Logical>>

This Sub System Function is responsible for sending Seat Movement Request based on its Inputs requests

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request to Move Seat” function is called by the following functions:

- – “Request Seat Arbitration”

Figure 2: Activity Diagram of   “Arbitrate Seat Position” calling   “Request to Move Seat”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Seat Movement Request Type: SeatMovementRequest | Signal Description: Command to Move seat which goes to the Driver seat System Received from: - Activity Parameter Node: Seat Movement Request |
| seat positon Type: SeatPosition | Signal Description: Current Seat Position from the Driver Seat System Received from: - Activity Parameter Node: Seat Position |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Seat Movement Command Type: SeatMovementRequest | Signal Description: Command to Move seat which goes to the Driver seat System Sent to: - Activity Parameter Node: Seat Movement Command |
| Seat Movement status Internal Type: SeatMovementStatusInternal | Signal Description: Status of the Seat movement according to the current position and external position Sent to: - Activity Parameter Node: Seat Movement Status Internal |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

934 Request to Move Seat Seat Movement Command Definition

The Request to Move Seat function Shall Set Seat Movement Command to WORK, PLAY, REST or DRIVE

| Requirement ID: 934 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

935 Request to Move Seat Normal Command

The Request to Move Seat function shall set Seat Movment Command to the equivilent State Selection Signal input or DRIVE, REST, PLAY, WORK.

| Requirement ID: 935 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1107 Set Seat Movement status |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request Tray Arbitration

### Function Overview

#### Description

Function is allocated to:

- Stowable Steering and Deployable Tray System <<Logical>>

This System Function is responsible for preparing the Tray to Arbitrate and send the respective command for Arbitrating tray to other functions

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Tray Arbitration” function is called by the following functions:

- – “From work to drive mode”

- – “Make Work Space Available”

- – “Request Work Mode”

- – “Rest to Work”

- – “Work to Rest”

- – “WorkToandFroRest”

Figure 2: Activity Diagram of   “From work to drive mode” calling   “Request Tray Arbitration”

Figure 2: Activity Diagram of   “Make Work Space Available” calling   “Request Tray Arbitration”

Figure 2: Activity Diagram of   “Request Work Mode” calling   “Request Tray Arbitration”

Figure 2: Activity Diagram of   “Rest to Work” calling   “Request Tray Arbitration”

Figure 2: Activity Diagram of   “Work to Rest” calling   “Request Tray Arbitration”

Description of “WorkToandFroRest” diagram:

Feature behavior from work to rest mode

Figure 2: Activity Diagram of   “WorkToandFroRest” calling   “Request Tray Arbitration”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Tray Move Request Type: TrayMoveRequest | Signal Description: Internal Request of Feature Controller to Move Tray Table Received from: - Activity Parameter Node: Requst Tray Move - Activity Parameter Node: Requst Tray Move - Control SSC Functionality - Activity Parameter Node: Requst Tray Move - Activity Parameter Node: Requst Tray Move |
| Tray Position status Type: TrayPostionStatus | Signal Description: Tray Position status is the feedback for Tray position given by Tray position Sensor Received from: - Actuate Tray |
| Tray Lock status Type: TrayLockStatus | Signal Description: Tray lock status is given by Tray Lock sensor Received from: - Actuate Tray |
| Tray Overload status Type: TrayOverloadStatus | Received from: - Actuate Tray |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Tray status Type: TrayStatus | Signal Description: This status is decided and fed by the Feature systems based on Requests and Input feedbacks fed Sent to: - Activity Parameter Node: Tray Status - Activity Parameter Node: Tray Status - Check for Transition Termination - Activity Parameter Node: Tray Status - Activity Parameter Node: Tray Status |
| Tray Move command Type: TrayMoveCommand | Signal Description: Tray Move Command is the command signal which has various Tray Control requests in it. Sent to: - Actuate Tray |
| HMI instructions Type: HMIInsWar | Signal Description: HMI Instructions to Stow and Warning display if Tray is not pushed in properly Sent to: - Manage Feature Arbitration Feedback |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Request Tray Arbitration

### Function Requirements

#### Functional Requirements

##### Normal Operation

1104 Request Tray Arbitration

When Request Tray Arbitration function receives StateSelectionSignal = WORK, it shall verify TrayObstructionStatus= NO_OBSTACLE_DETECTED , TrayPositionStatus= STOWED , TrayLockStatus=LOCKED and then set

TrayMoveCommand= DEPLOY && TrayMoveCommand= UNLOCK && TrayStatus= DEPLOYING and Look for HMIInsWar while stowing if requested

| Requirement ID: 1104 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request Tray Deployment

### Function Overview

#### Description

Function is allocated to:

- Command Tray Table Deploy <<Logical>>

- Stowable Feature Controller <<Logical>>

This System Function is responsible for preparing the Tray to Arbitrate and send the respective command to Arbitrate tray to next functions

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Tray Deployment” function is called by the following functions:

- – “Request Tray Arbitration”

Figure 2: Activity Diagram of   “Request Tray Arbitration” calling   “Request Tray Deployment”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Tray Deploy Request Type: TrayMoveRequest | Signal Description: Internal Request of Feature Controller to Move Tray Table Received from: - Check Tray Status |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Deploy Tray Command Type: TrayMoveCommand | Signal Description: Tray Move Command is the command signal which has various Tray Control requests in it. Sent to: - Activity Parameter Node: Tray Move Command |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

943 Deploy/Stow Tray Command Output Definition

The Request Tray Deployment/Stowing function(s) Shall Set Stow/Deploy Tray Command to NONE or Halt, Deploy or STOW

| Requirement ID: 943 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

948 Request Tray Deployment function Deploy Tray Command Normal Tray Deployment Start

The Request Tray Deployment function Shall Set Deploy Tray Command is set to DEPLOY when State Selection Signal is set to NOT_DRIVE AND Tray Table Position is set to STOWED AND Tray Obstruction Status is set to NO_OBSTACLE_DETECTED

| Requirement ID: 948 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

949 Request Tray Deployment function Deploy Tray Command Normal Tray Deployment Completed

The Request Tray Deployment/Stowing function(s) Shall Set Stow/Deploy Tray Command is set to NONE when Tray Table Position is set to DEPLOYED AND Tray Obstruction Status is set to NO_OBSTACLE_DETECTED

| Requirement ID: 949 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

950 Request Tray Deployment/Stowing function Deploy/Stow Tray Command Error Detected

The Request Tray Deployment/Stowing function(s) Shall Set Stow/Deploy Tray Command to NONE when Tray Table Position is set to ERROR

| Requirement ID: 950 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1104 Request Tray Arbitration |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request Tray Lock

### Function Overview

#### Description

Function is allocated to:

- Lock Tray Table <<Logical>>

- Stowable Feature Controller <<Logical>>

A subsystem Function which sends Tray Lock request based on its inputs

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Tray Lock” function is called by the following functions:

- – “Request Tray Arbitration”

Figure 2: Activity Diagram of   “Request Tray Arbitration” calling   “Request Tray Lock”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Tray Move request Type: TrayMoveRequest | Signal Description: Internal Request of Feature Controller to Move Tray Table Received from: - Check Tray Status |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Request Lock Tray Type: TrayMoveCommand | Signal Description: Tray Move Command is the command signal which has various Tray Control requests in it. Sent to: - Activity Parameter Node: Tray Move Command |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

926 Request Tray Lock Normal Lock

The Request Tray Lock function shall set Request Lock Tray to LOCK when State Selection Signal is set to DRIVE and Tray Lock Status is set to TRAY_LOCKED

| Requirement ID: 926 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

927 Request Tray Lock Normal Unlock

The Request Tray Lock function shall set Request Lock Tray to UNLOCK when State Selection Signal is set to DRIVE and Tray Lock Status is set to TRAY_UNLOCKED

| Requirement ID: 927 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

928 Request Tray Failure to Lock

The Request Tray Lock function shall set Request Lock Tray to NONE when State Selection Signal is set to DRIVE and Tray Lock Status is set to TRAY_FAILED_TO_LOCK

| Requirement ID: 928 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

930 Request Tray Lock Not Drive

The Request Tray Lock function shall set Request Lock Tray to NONE when State Selection Signal is set to NOT DRIVE

| Requirement ID: 930 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

966 Request Lock Tray Definition

The Request Tray Lock function Shall Set  Request Lock Tray to LOCK, UNLOCK or NONE

| Requirement ID: 966 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1110 Provide Airbag Inhibit/ DeInhibit Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

929 Request Tray Failure to Unlock

The Request Tray Lock function shall set Request Lock Tray to NONE when State Selection Signal is set to DRIVE and Tray Lock Status is set to TRAY_FAILED_TO_UNLOCKED

| Requirement ID: 929 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Handling |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Request Tray Unlock

### Function Overview

#### Description

Function is allocated to:

- Stowable Feature Controller <<Logical>>

- Unlock Tray Table <<Logical>>

A subsystem Function which sends Tray Unlock request based on its inputs

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Request Tray Unlock” function is called by the following functions:

- – “Request Tray Arbitration”

Figure 2: Activity Diagram of   “Request Tray Arbitration” calling   “Request Tray Unlock”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| tray unlock request Type: TrayMoveRequest | Signal Description: Internal Request of Feature Controller to Move Tray Table Received from: - Check Tray Status |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Tray Move command Type: TrayMoveCommand | Signal Description: Tray Move Command is the command signal which has various Tray Control requests in it. Sent to: - Activity Parameter Node: Tray Move Command |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

918 Unlock Tray Table

When stowing of the Tray Table is commanded, the Unlock Tray Table function shall use the obstacle status to  provide a status used to determine if stowing of the Tray Table shall proceed.

| Requirement ID: 918 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

920 Request Tray Unlock Pass Through

The Request Tray Unlock function shall pass through the Tray Lock Status value unchanged to the Tray Lock Status output.

| Requirement ID: 920 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

921 UnLock Tray Request Definition

The Request Tray Unlock function Shall Set Unlock Tray Request to LOCK, UNLOCK or NONE

| Requirement ID: 921 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Interface Specification |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

922 Normal Lock Tray Request

The Request Tray Unlock function shall set Unlock Tray Request to LOCK when Tray Lock Status is set to TRAY_LOCKED and State Selection Signal is set to WORK.

| Requirement ID: 922 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

923 Normal Unlock Tray Request

The Request Tray Unlock function shall set Unlock Tray Request to UNLOCK when Tray Lock Status is set to TRAY_UNLOCKED and State Selection Signal is set to WORK.

| Requirement ID: 923 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

924 Failed Tray Request

The Request Tray Unlock function shall set Unlock Tray Request to NONE when Tray Lock Status is set to either TRAY_FAILED_TO_LOCK or TRAY_FAILED_TO_UNLOCK and State Selection Signal is set to WORK.

| Requirement ID: 924 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

925 Not Work Tray Lock Request

The Request Tray Unlock function shall set Unlock Tray Request to NONE when State Selection Signal is set to NOT_WORK.

| Requirement ID: 925 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1099 Tray Lock Functionality |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Sense Tray Overload

### Function Overview

#### Description

Function is allocated to:

- Tray Load Sensor <<Logical>>

This function provides the status of Overload on the Tray Table

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Sense Tray Overload” function is called by the following functions:

- – “Actuate Tray”

Figure 2: Activity Diagram of   “Actuate Tray” calling   “Sense Tray Overload”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Tray Motor Load Type: TrayMotorLoad | Received from: - Actuate Tray Movement |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Tray Overload Status Type: TrayOverloadStatus | Sent to: - Activity Parameter Node: Tray Overload Status |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

No diagrams internal to function specified.

### Function Requirements

#### Functional Requirements

##### Normal Operation

837 Sense Tray Overload Tray Load Status Definition

The Sense Tray Overload function Tray Load Status shall indicate OVERLOAD_DETECTED, NO_OVERLOAD_DETECTED or UNKNOWN in regard to the status of the force or weight exerted on the Feature Tray.

| Requirement ID: 837 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality. |  |  |  |  |  |  |
| Acceptance Criteria | Unit Test |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com> |
| Source Req. | - 1100 Tray Obstacle Sensing |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Verify Drive Control Status

### Function Overview

#### Description

Function is allocated to:

- Stowable Steering and Deployable Tray System <<Logical>>

System Function responsible for Verify the Drive Control Status Before Actuating the feature

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Verify Drive Control Status” function is called by the following functions:

- – “Inhibit and Verify Drive Control”

- – “Make Rest space Available”

- – “Make Work Space Available”

Figure 2: Activity Diagram of   “Inhibit and Verify Drive Control” calling   “Verify Drive Control Status”

Figure 2: Activity Diagram of   “Make Rest space Available” calling   “Verify Drive Control Status”

Figure 2: Activity Diagram of   “Make Work Space Available” calling   “Verify Drive Control Status”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| Locomotion Inhibit status Type: LocomotionInhibitStatus | Signal Description: Status of the Locomotion Inhibition Received from: - Activity Parameter Node: Locomotion Inhibit status - Activity Parameter Node: Locomotion Inhibit status - Activity Parameter Node: Locomotion Inhibit Status |
| Driver Airbag Inhibit status Type: DriverAirbagInhibitStatus | Signal Description: Status of the Driver Airbag Inhibition Received from: - Activity Parameter Node: Driver Airbag Inhibit Status - Activity Parameter Node: Driver Airbag Inhibit status - Activity Parameter Node: Driver Airbag Inhibit status |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| Inhibit status Type: InhibitStatus | Signal Description: Inhibit status of driver control which is verified by the feature systems Sent to: - Control SSC Functionality - Activity Parameter Node: Inhibit Status |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Verify Drive Control Status

### Function Requirements

#### Functional Requirements

##### Normal Operation

1108 Verify Deinhibit status

when deinhibit drive mode is requested, Verify Drive control status function shall monitor and update drive mode inhibition status as follows:

1. if LocomotionInhibitStatus==DEINHIBITED and DriverAirbagInhibitStatus==DEINHIBITED , set InhibitStatus=DC_INHIBITED

2. Otherwise, set InhibitStatus=DC_FAIL_TO_DEINHIBIT

| Requirement ID: 1108 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

1095 Drive Control Inhibit Status Verification

Stowable Steering and Deployable Tray System shall Verify the Inhibit status of Drive Control before entering Play , Work or Rest Modes

| Requirement ID: 1095 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Error Response |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Non-Functional Requirements

1103 Verify Inhibit status

when inhibit drive mode is requested, Verify Drive control status function shall monitor and update drive mode inhibition status as follows:

1. if LocomotionInhibitStatus==INHIBITED and DriverAirbagInhibitStatus==INHIBITED , set InhibitStatus=DC_INHIBITED

2. Otherwise, set InhibitStatus=DC_FAIL_TO_INHIBIT

| Requirement ID: 1103 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

## Verify Feature Status

### Function Overview

#### Description

Function is allocated to:

- Stowable Steering and Deployable Tray System <<Logical>>

This Function verifies the User Input , vehicle Status and Vehicle Configuration and provides State Selection signal and HMI Feedback

#### Variants

Not supported by MagicDraw report generation.

#### Input Requirements

Not supported by MagicDraw report generation.

#### Assumptions

No assumptions specified for this function.

#### References

##### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Reference | Title | Doc. ID | Revision |
| --- | --- | --- | --- |

Table 5: Ford internal Documents (not specified in model)

##### External Documents and Publications

The list of external documents could include books, reports and online sources.

| Reference | Document / Publication |
| --- | --- |

Table 7: External documents and publications (not specified in model)

#### Glossary

See Appendix for Definitions and Abbreviations.

### Function Scope

The  – “Verify Feature Status” function is called by the following functions:

- – “Check Feature Status”

Figure 2: Activity Diagram of   “Check Feature Status” calling   “Verify Feature Status”

### Function Interfaces

#### Logical Inputs

| Signal Name | Description |
| --- | --- |
| vehicle status Type: VehicleStatus | Signal Description: This Signal is the collectivity approval signal which includes Parking status, battery state of charge and Vehicle Speed Received from: - Activity Parameter Node: Vehicle Status |
| Vehicle Configuration Type: VehicleConfig | Signal Description: Configuration of the Feature and Vehicle Specific Received from: - Activity Parameter Node: Vehicle Configuration |
| User Input Type: UserInput | Signal Description: Input given by user through HMI Received from: - Manage User Selection |

#### Logical Outputs

| Signal Name | Description |
| --- | --- |
| state selection signal Type: StateSelectionSignal | Signal Description: Signal which contains the Mode that User selected after the Vehicle status is Valid Sent to: - Activity Parameter Node: state selection signal |
| HMI Feedback Type: HMIFeedback | Signal Description: Feedback given to HMI Sent to: - Provide ModeSelection Feedback |

#### Logical Parameters

Not supported by MagicDraw report generation.

### Function Modeling

#### Use Cases

Not supported by MagicDraw report generation.

#### State Charts / Activity Diagrams / Sequence Diagrams / Decision Tables

Figure 3: Verify Feature Status

### Function Requirements

#### Functional Requirements

##### Normal Operation

1093 Accept User Input

HMI System shall allow selection of "Work Mode", "Rest Mode", "Play Mode" and "Drive Mode".

| Requirement ID: 1093 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes | Tray Table Mode deploys the Tray Table. Rest Mode keeps the Tray Table stowed. Drive Mode is the normal vehicle drive state with normal seat position. |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

1112 Verify Feature Status

When Activated, Verify Feature status functions shall read UserInput, VehicleStatus and VehicleConfig it shall set the StateSelectionSignal and HMIFeedback Accordingly

| Requirement ID: 1112 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Basic Functionality |  |  |  |  |  |  |
| Acceptance Criteria | Inspection |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner | "Lau, Tok (T. W.) <tlau4@ford.com>, Gajjela, Vinay Sai (V.) <vgajjela@ford.com>, Gray, Wallace (W. M.) <wgray45@ford.com>, Loucks, Charles (C.H.) <cloucks2@ford.com>" |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority | 1 - High | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

##### Error Handling

No Error Handling Requirements specified.

#### Non-Functional Requirements

No Non-Functional Requirements specified.

#### Functional Safety Requirements

No Functional Safety Requirements specified.

##### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

#### Other Requirements

##### Design Requirements

No Design Requirements specified.

# Open Concerns

#Hint: The following list presents open concerns, which have to be discussed or clarified over the course of the on-going requirements engineering.

| ID | Concern Description | e-Tracker / Reference | Responsible | Status | Solution |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |

Table 8: Open Concerns (Not supported by MagicDraw report generation.)

# Revision History

#Hint: A new version number is assigned to a document with a given revision each time it is checked in to Team Center (TCSE). After release of a revision, the document cannot be edited and no new versions can be created on that revision. When updating the document after that, a new revision has to be created and new versions on that revision will be created upon checking in.

No Revision History found.

## Template Revisions

#Important: Do not change this section

| Version | Rev. | Date | Description | Responsible |
| --- | --- | --- | --- | --- |
| 1 | 0 | 2016-02-26 | Initial version, derived from FDS | Jbaden1 |
| 1 | 1 | 2016-02-26 | Word properties corrected | Jbaden1 |
| 1 | 2 | 2016-03-10 | Clean up of document meta data (Word properties) | Jbaden1 |
| 1 | 3 | 2016-03-22 | - Footer formatting corrected (Issue 19) - “Constraints” chapter renamed to “Input Requirements” (Issue 20) | Jbaden1 |
| 1 | 4 | 2016-04-20 | - Broken Wiki links repaired | Jbaden1 |
| 2 | 0 | 2016-06-10 | - Document metadata adapted. Prepared for new macros - DTC table removed - HMI function added as a chapter (details still to be refined) - Signal / Parameter IDs column deleted interface tables | Jbaden1 |
| 2 | 1 | 2016-07-14 | - Converted to SysML diagrams - HMI section further elaborated - Template version added to footer - Dedicated Startup / Shutdown sections removed (only hints added) - Data Dictionary reworked and Signal / Parameter IDs column re-introduced | Jbaden1 |
| 2 | 2 | 2016-12-07 | - Minor formatting changes | Jbaden1 |
| 3 |  |  | Skipped to synchronize with Specification_Macros.dotm |  |
| 4 |  |  |  |  |
| 5 | 0 | 2017-01-13 | - Meta data updated for specification macros, version 3.1 - SW Unit chapter removed for the time being - Green boxes added for user hints | Jbaden1 |
| 5 | 1 | 2017-01-18 | - Some additional hints. - Hyperlinks highlighted in hints | Jbaden1 |
| 6 | 0 | 2017-04-28 | - Editorial change. Hints added to chapter 4.1.4 - Chapter “Traceability Matrix” removed | Jbaden1 |
| 6 | 0 | 2018-04-28 | - CR69/63: New chapters added for Functional Safety (FTTI and Technical Safety Requirements) - CR53: New coversheet + additional meta-data - CR76: merge sections for configuration and for calibration parameters into one on Function Level | Jbaden1 |
| 6 | 0 | 2018-08-06 | - CR66: Fix version numbering in footer of Function Spec | Jbaden1 |
| 6 | 0 | 2018-09-28 | - Broken links to RE Wiki repaired | Jbaden1 |
| 6 | 0 | 2018-10-31 | - Minor corrections on cover sheet and in footer to be more GIS compliant and VSEM aligned - “Overview” and “Description” exchanged in headings (following common sense) | Jbaden1 |
| 6 | 0 | 2018-11-12 | - Explanatory text in Variants” section revised - Functional Safety modifications as agreed with FuSa core team (Baseline: November 2018 Dearborn On-Site) | Jbaden1 |
| M |  | 2019-04-02 | - Initial version of SysML report template | snuesch |
| M |  | 2019-04-05 | - Improved dialog boxes to select function group | snuesch |
| M |  | 2019-04-26 | - Improved function interfaces (support of additional elements (e.g., send signal action, add structural feature action, merge node, decision node, activity parameter nodes, etc.) and pins) | snuesch |
| N |  | 2019-08-19 | - For each function now also requirements satisfied by activity parameter nodes and function outputs are populated. | snuesch |
| N |  | 2019-08-21 | - Improved glossary and acronym tables | snuesch |
| N |  | 2019-09-20 | - Updated Function Interfaces tables with better wording. Added a report template variable that allows to disable senders and receivers. Sorted requirements by ID. | snuesch |
| N |  | 2019-09-27 | - Green hints now only show up for first function. - labelTag variable can be used to filter revision history. | snuesch |

# Appendix

## Data Dictionary

### Logical Signals

#Macro: Add Ins -> Add Requirement macro (select “Logical Signal” as type)

CauseOfFailure

Cause for which status is not appropriate in the vehicle to arbitrate the feature mode

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Drivability Status

The logical signal which validates all systems component status and has the calculated the validity to enter into drive mode

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

DriverAirbagInhibitStatus

Status of the Driver Airbag Inhibition

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

DriverAirbagRequest

Request sent to the Driver Airbag system / Passive Restraint System for either Inhibit or Deinhibit

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

HMIFeedback

Feedback given to HMI

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

HMIInsWar

HMI Instructions to Stow and Warning display if Tray is not pushed in properly

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

InhibitStatus

Inhibit status of driver control which is verified by the feature systems

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

LocomotionInhibitStatus

Status of the Locomotion Inhibition

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

LocomotionRequest

Request to Powertrain / Locomotion to Inhibit /Deinhibit

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

ModeAcceptenceStatus

This signal contains the information whats not available in the vehicle configuration

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

SeatMovementRequest

Command to Move seat which goes to the Driver seat System

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

SeatMovementStatusInternal

Status of the Seat movement according to the current position and external position

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

SeatPosition

Current Seat Position from the Driver Seat System

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

StateOfFeature

Current state of Feature and other warnings

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

StateSelectionSignal

Signal which contains the Mode that User selected after the Vehicle status is Valid

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

SteeringColumnRakePosition

Steering Column Position in Rake Position

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

SteeringColumnRequest

Request to Move steering Column with respect to selected Mode

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

SteeringColumnStatus

Position of the Steering Column provided by the steering column system

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

SteeringColumnStatusInternal

Status of SteeringColumn based on Feature Decisions and current steering Column Position

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

SteeringColumnTelePosition

Steering Column Position in Tele Position

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

Termination Signal

Logical signal for termination of Mode Arbitration when system did not act according to requests

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

TrayLockStatus

Tray lock status is given by Tray Lock sensor

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

TrayMotorLoad

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

TrayMoveCommand

Tray Move Command is the command signal which has various Tray Control requests in it.

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

TrayMoveRequest

Internal Request of Feature Controller to Move Tray Table

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

TrayOverloadStatus

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

TrayPostionStatus

Tray Position status is the feedback for Tray position given by Tray position Sensor

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

TrayStatus

This status is decided and fed by the Feature systems based on Requests and Input feedbacks fed

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

UserInput

Input given by user through HMI

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

VehicleConfig

Configuration of the Feature and Vehicle Specific

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

VehicleStatus

This Signal is the collectivity approval signal which includes Parking status, battery state of charge and Vehicle Speed

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

VehicleStatusValidity

This is the signal which contains the validity of the Vehicle status before Arbitrating into any Mode

| ASIL |  |  |
| --- | --- | --- |
| Encoding Type Name |  |  |
| Note: An encoding is either discrete or continuous. Delete fields below which are not needed |  |  |
| Value (Continuous Encoding) | Min Value |  |
|  | Max Value |  |
|  | Resolution |  |
|  | Offset |  |
| Value (Discrete Encoding) |  |  |
| Unit |  |  |

### Logical Parameters

#Macro: Add Ins -> Add Requirement macro (select “Logical Parameter” as type)

### Encoding Types

#Macro: Add Ins -> Add Requirement macro (select “Encoding Type” as type)

## Glossary

#Hint: Terms, concepts and abbreviations used in the document shall be defined and illustrated here. Note that changes to terms and/or concepts described in this section tend to cause major updates to this document.

The tables below have feature specific definitions and abbreviations. For additional, non-feature specific terms please refer to the RE Glossary

### Definitions

#Hint: The table below has definitions and abbreviations relevant for the functions in this document. For additional terms please refer to the RE Glossary

| Definition | Description |
| --- | --- |
| Deploy | Come out / Move away from Dashboard or Instrument Panel Cluster |
| Drivability Status | Status to make sure all the component's status are good before allowing the drivability to user for safe drive |
| Drive Control | Drive Control is the Control status of Locomotion and Passive Restraint ( Drive Airbag)) |
| Drive Mode | The original Mode of the Vehicle in which the work surface for productivity is inside the instrument panel assembly ( stowed and locked) and the Driver seat is at drive seat position and Steering Column is Deployed |
| Drive state | The original Mode of the Vehicle in which the work surface for productivity is inside the instrument panel assembly ( stowed and locked) and the Driver seat is at drive seat position and Steering Column is Deployed |
| FuSa | Functional Safety |
| Rest Mode | The mode in which driver will have Driver seat is moved back (Work mode seat position) and Steering Column is stowed inside( Work Surface will not be Arbitrated) |
| Rest state | The mode in which driver will have Driver seat is moved back (Work mode seat position) and Steering Column is stowed inside( Work Surface will not be Arbitrated) |
| Stow | Move in / Move towards to Dashboard or Instrument Panel Cluster |
| Tray | Work surface where a User / Driver can use |
| Work Mode | The mode in which driver will have a work surface for productivity and the Driver seat is moved back (Work mode seat position) and Steering Column is stowed inside. |
| Work state | The mode in which driver will have a work surface for productivity and the Driver seat is moved back (Work mode seat position) and Steering Column is stowed inside. |

Table 9: Definitions used in this document

### Abbreviations

No acronyms specified.

Document ends here.