|  | My Seat Space <<Feature>> (F003519) |  |  |  |
| --- | --- | --- | --- | --- |
| Document Type | Feature Document (FD) |  |  |  |
| Template Version | 6.1b |  |  |  |
| SysML Report Template Version | O Beta (2021/06/01) |  |  |  |
| Document ID | ffst01.10_featuredocument_sysmlreporttemplate |  |  |  |
| Document Location |  |  |  |  |
| Document Owner | Cintron, Austin. |  |  |  |
| Document Revision | FD0 |  |  |  |
| Document Status | Draft |  |  |  |
| Date Issued | 2021/08/13 |  |  |  |
| Date Revised | 2021/08/13 |  |  |  |
| Document Classification | GIS1 Item Number: | 27.60/35 |  |  |
|  | GIS2 Classification: | Confidential |  |  |
| Document Approval |  |  |  |  |
| Person | Role |  | Email Confirmation | Date |

Auto-Generated by MagicDraw

Printed Copies Are Uncontrolled

# Disclaimer

This document contains Ford Motor Company Confidential information. Disclosure of the information contained in any portion of this document is not permitted without the expressed, written consent of a duly authorized representative of Ford Motor Company, Dearborn, Michigan, U.S.A.

Copyright, Ó 2021  Ford Motor Company

This document contains information developed and accumulated by and for FORD MOTOR COMPANY. As such, it is a proprietary document, which, if disseminated to unauthorized persons, would provide others with restricted information, data, or procedures not otherwise available, exposing the FORD MOTOR COMPANY to potential harm.

Employees and suppliers having custody of this specification or authorized to use it must be cognizant of its proprietary nature and ensure that the information herein is not made available to unauthorized persons.

FORD MOTOR COMPANY reserves the right to protect this work as an unpublished copyrighted work in the event of an inadvertent or deliberate unauthorized publication. FORD MOTOR COMPANY also reserves its rights under copyright laws to protect this work as a published work.

This document or portions thereof shall not be distributed outside FORD MOTOR COMPANY without prior written consent. Refer all questions concerning disclosure to the author(s) or to any duly authorized representative of Ford Motor Company.

Copyright © 2021 Ford Motor Company

# Contents

1.1.2	Decomposition of Functional Safety Requirement 	13

Disclaimer	22

Contents	23

2	Introduction	26

2.1	Document Purpose	26

2.2	Document Scope	26

2.3	Document Audience	26

2.3.1	Stakeholder List	26

2.4	Document Organization	26

2.4.1	Document Context	26

2.4.2	Document Structure	27

2.5	Document Conventions	27

2.5.1	Requirements Templates	27

3	Feature Overview	28

3.1	Purpose and Description of Feature	28

3.2	Feature Variants	28

3.2.1	Regions & Markets	29

3.3	Input Requirements	29

3.3.1	Legal Requirements	30

3.3.2	Trustmark Requirements	30

3.3.3	Industry Standards	30

3.3.4	Attribute Requirements	30

3.4	Lessons Learned	30

3.5	Assumptions	31

3.6	References	31

3.6.1	Ford Documents	32

3.6.2	External Documents and Publications	32

3.7	Glossary	33

3.7.1	Parameters / Values	33

4	Feature Context	34

4.1	Feature Context Diagram	34

4.2	List of Influences	34

5	Feature Modeling	36

5.1	Operation Modes and States	36

5.2	Use Cases	38

5.2.1	Use Case Diagram	38

5.2.2	Actors	39

5.2.3	Use Case Descriptions	39

5.3	Driving and Operation Scenarios	43

5.4	Decision Tables	45

6	Feature Requirements	46

6.1	Functional Requirements	46

6.1.1	Error Handling	47

6.2	Non-Functional Requirements	47

6.2.1	Safety	47

6.2.2	Security	47

6.2.3	Reliability	47

6.3	HMI Requirements	47

6.4	Other Requirements	48

6.4.1	Design Requirements	48

6.4.2	Manufacturing Requirements	48

6.4.3	Service Requirements	48

6.4.4	After Sales Requirements	48

6.4.5	Process Requirements	48

6.4.6	Uncategorized Requirements	49

7	Functional Safety	50

7.1	System Behaviors for HARA	50

7.2	Safety Assumptions	50

7.3	Safety Goals	51

7.4	Functional Safety Requirements	53

7.4.1	Safety Goal:  	53

7.4.2	Derivation of Functional Safety Requirements on Assumptions	55

7.5	ASIL Decomposition of Functional Safety Requirements	56

8	Architecture	57

8.1	Functional Architecture	57

8.1.1	List of Functions	57

8.2	Logical Architecture	58

8.2.1	Logical Interfaces	59

9	Open Concerns	61

10	Revision History	62

11	Appendix	63

11.1	Definitions	63

11.2	Abbreviations	63

List of Figures

Figure 1. 	12

Figure 2: 	28

Figure 3:  - 	28

Figure 4: 	34

Figure 5: 	36

Figure 6: 	38

Figure 7: 	55

Figure 8: 	57

Figure 9: 	58

List of Tables

Table 1: Features described in this FD	26

Table 2: Feature Variants	29

Table 3: Regions & Markets	29

Table 4: Ford internal Documents	32

Table 5: Ford internal Documents (not specified in SysML model)	32

Table 6: External documents and publications	32

Table 7: External documents and publications (not specified in SysML model)	33

Table 8: Parameters / Values used in this document (Not supported by MagicDraw report generation)	33

Table 9: List of Influences	35

Table 10: Operation Modes and States on 	36

Table 11: Transitions between Operation Modes and States on 	38

Table 12: List of Actors	39

Table 13: System Behaviors for HARA	50

Table 14: Functional Safety Assumptions	51

Table 15: Functional Safety Goals	53

Table 16: List of Functions	58

Table 17: List of Functions on 	58

Table 18: Feature Interactions	59

Table 19: Feature Interactions on 	59

Table 20: Open Concerns (Not supported by MagicDraw report generation)	61

Table 21: Definitions used in this document	63

Table 22: Abbreviations used in this document	64

# Introduction

## Document Purpose

A Feature Document (FD) document defines a Feature on Concept Level. It specifies what the feature shall do and how it shall behave from customer perspective. It should also provide reasoning and background why we have the feature in the vehicle.

The FD also serves as an Item Definition as defined by ISO26262 for those features, which follow the Ford Functional Safety process. Refer FFSG01.10 Feature Document Guideline for how to apply the Feature Doc template for Functional Safety.

## Document Scope

This Feature Document (FD) specifies the following features:

| Feature ID | Feature Name | Owner | Reference |
| --- | --- | --- | --- |
| F003519 | My Seat Space (Program(s): GE2 MY24 CDX746/747 ) | Cintron, Austin. |  |

Table 1: Features described in this FD

## Document Audience

The FD is written by the feature owner of

Cintron, Austin.

. All Stakeholders, i.e., all people who have a valid interest in the feature should read and, if possible, review the FD. It needs to be guaranteed, that all stakeholders have access to the currently valid version of the FD.

### Stakeholder List

For the latest list of stakeholder of the feature and their influence refer to <VDOC099532>.

| Name | CDSID | Contact Info | Role | Stakeholder Group |
| --- | --- | --- | --- | --- |

## Document Organization

### Document Context

Refer to the Specification Structure page in the Ford RE Wiki to understand how the FD relates to other Ford Requirements Documents and Specifications.

### Document Structure

The structure of this document is explained below:

Introduction		– 	Explains how to use this document including responsibilities and requisite documents. Explains the terminology. Gives a clarification of the definitions, concepts and abbreviations used in the document.

Feature Overview		– 	States briefly the background and the purpose of the feature, feature variants and corresponding regions and markets. Also includes input requirements, assumptions and constraints.

Feature Context 		– 	describes all external entities, which have an influence on the feature.

Feature Modeling 	– 	Contains Use Case, Driving Scenarios, State Charts to describe the functional behavior of the feature.

Feature Requirements 	– 	Lists functional and non-functional requirements of the feature.

Functional Safety		– 	Lists System Behaviors, Safety Goals and Safety Requirements of the feature.

Cybersecurity	 	– 	Lists Security Goals and Security Requirements of the feature.

Architecture 		– 	Shows the coarse architecture, which the feature requirements are deployed to. Describes the elements and the boundary of the feature as well as the decomposition and distribution of associated functions.

Traceability Matrix 	– 	Traceability Matrix.

Open Concerns		– 	List of Open Concerns

Revision History 		– 	Document Change History including a list of new or modified requirements. The requirements in this document are tagged, and this section contains different types of tables listing all, new, or changed requirements by their title and page no.

Appendix		– 	Appendix

## Document Conventions

### Classification of Chapters

A chapter is considered mandatory, unless the chapter or its parent chapter(s) are categorized by using the tag:

#Classification: Some Condition

If no requirement/other content is known for a mandatory chapter, leave a statement “Not Applicable”

Some chapters have a follow certain rules in context of specific Ford processes, e.g. Functional Safety. This is indicated at the beginning of the corresponding chapter by the tags:

#Functional Safety: Some process specific explanation

#Cybersecurity: Some process specific explanation

### Requirements Templates

Refer to “How to use the Specification Templates” on how to use the specification templates and the VBA macros to create/edit the requirements in the specifications.

#### Requirements Attributes

The templates provided by Specification_Macros.dotm define a list of attributes for each requirement. This helps to classify the requirement. The attributes are explained at RE Wiki - Requirements Attributes.

## References

### Ford Documents

List here all Ford internal documents, which are directly related to the feature.

| Document Name | Document Owner | Identifier Number |
| --- | --- | --- |
| MSS CIED ARL | Raven Fields | FEDE: RQT-002004- 712583 |
| URC Feature Document | Gregory Reed | VSEM:VDOC080630 |
| URC Functional Specifications | Gregory Reed | VSEM:VDOC080631 |
| URC Feature Implementation Specifications | Gregory Reed | VSEM:VDOC080632 |
| MyKey Feature Specifications | Ankisha Kudyar | VSEM:VDOC093428 |
| Passenger Phone Call SPSS | Balakrishnan Ganesan | VSEM:VDOC099175 |
| Zone Manager SPSS | Balakrishnan Ganesan | Pending |
| Media Share SPSS | Balakrishnan Ganesan | Pending |
| ICC SPSS | Savitha Emani | Pending |
| CA SPSS | Savitha Emani | Pending |

### External Documents and Publications

The list of external documents could include books, reports and online sources.

External documents and publications not specified in SysML model

## Glossary

See Appendix for Definitions and Abbreviations.

### Definitions

### Abbreviations

### Parameters / Values

| Name | Description | Range / Resolution |
| --- | --- | --- |

Table 8: Parameters / Values used in this document

# Feature Overview

## Purpose and Description of Feature

Feature to improve each customers’ individual entertainment and communication experience. This is accomplished by giving customers an isolated immerse audio experience, enabling zones in the vehicle to have to their own audio source while minimizing distraction to others in the vehicle. This feature further enables in-car communication, private phone calls, media stream sharing, and audio enhancements.

1. Multiple simultaneous connections/controls

•       System allows for per zone personalized audio experiences.

•       Audio and media streaming as well as private phone calls are contained to one seat without affecting others

•       Chimes and prompts can be sent to the appropriate seat without interrupting the entire vehicle.

2. Media sharing and enhanced in-vehicle communication

•       Allows for improved two-way row-to-row communication by amplifying occupant speech.

•       Allows for one-way communication for driver announcements

•       Users can share their media content with others in the vehicle

3. Realtime control inside the vehicle

•       Allows control of personalized seat environment

•       Climate, lighting and seat controls can be managed by an app on user’s phone(URC).

Figure 2: Feature Image Here

## Feature Variants

No Feature Variants specified.

### Regions & Markets

No Feature Variants specified.

## Input Requirements/Documents

| Reference (Reference as listed in ch. “References”) | Section/Requirement | Description | Derived Requirement (optional – reference to requirement in ch. “Feature Implementation Requirements”) |
| --- | --- | --- | --- |
| Attribute Requirements |  |  |  |
|  | Device Authorization Settings | Within the center stack HMI, the driver can authorize unknown devices to utilize passenger space commands via a brought in device until the setting is turned off |  |
|  | Occupant Override | SYNC center stack HMI shall have a screen to adjust second and third row passenger settings (all those noted in section 3 of this document) |  |
|  | URC Lag Requirement | Post input from user there shall be no more than a 300 millisecond delay before the user receives HMI feedback, and no more than an 1500 millisecond delay before the command is completed |  |
|  | CA Activation | Driver shall have the ability to initiate a Captains Announcement from either a button on the steering wheel or via the center stack HMI. |  |
|  | RACM and MSS | The RACM screen shall display the following when MSS and URC are in a vehicle and active in the second row: · When URC is active in only one second row passenger seat the cabin audio will be displayed on the RACM screen · When URC is active in both second row passenger seats an HMI notification will reflect this status change and no controls shall be available via the RACM screen |  |
|  | URC Passenger Push Notification | When using a brought in device, upon first time use as a passenger, FordPass or URC application must be downloaded and open for the vehicle to provide a push notification asking if the passenger would like to be connected to the system |  |
|  | CA Audio Interruption | Captain’s Announcement will play a chime and then pause audio for all passengers |  |
|  | Do Not Disturb Activated | Do Not Disturb (3.6.4.1) mode shall not allow media sharing or In Car Communication to devices that have this feature active |  |
|  | URC Lockout | Driver shall have the ability to lockout (3.6.4.3) specific controls brought in devices |  |
|  | Unified Visual HMI | There shall be a unified approach to controls and harmonious synergy between all HMI screens in the vehicle |  |
|  | MUCReq_012 | The passenger’s device ID will be saved automatically post connecting to the vehicle (assuming authorization) |  |
|  | ICC Initiation | Passengers will be able to use their brought in device to initiate In Car Communication (3.6.4.4) to talk back to the driver via the microphone in the passenger’s seat. The sound can come from either the speakers in the headrest of the seat. |  |
|  | Initiating CA | Driver must be able to accept or decline media and navigation sharing requests via the center stack HMI |  |
|  | Re-entry Device Connection | When a passenger leaves the vehicle and returns (Key off), If their phone has previously been paired, it shall automatically pair upon access of the FordPass app. |  |
|  | Centralized Content Screen | When using a brought in device, the HMI shall contain a single screen that centralizes basic controls and displays current media source. |  |
|  | Phone Call Drive Default | In coming phone calls shall default to driver only speakers in headrest with the option to switch into full cabin speakers via the HMI |  |
|  | CA Audio Routing | During the Captain’s Announcement, the drivers voice will not play over the front row speakers |  |
|  | ICC Accessibility | In Car Communication (3.6.4.4) button/setting must be accessible (turned on/off) in all rows whether in a vehicle screen (center stack) or on a brought in device |  |
|  | Using FordPass for URC | If FordPass is the environment that URC lives within, there must be an easily accessible passenger section where an occupant can use the URC features without creating an account and having to input VIN data |  |
| Ford Engineering Standards |  |  |  |
|  | <Example: some SDS (requirement)> |  |  |
| Legal Regulations |  |  |  |
|  | Compliance with FMVSS101 | The Feature shall comply with FMVSS101. |  |
| Industry Standards |  |  |  |
|  | ISO 26262 | The system should be developed according to Ford's implementation of Functional Safety. |  |
| Other Sources |  |  |  |

Table 2-1: Input Requirements/Documents

## Lessons Learned

No lessons learned specified.

## Assumptions

No Assumptions specified.

# Feature Context

## Feature Context Diagram

Figure 4: Feature Context

## List of Influences

| ID | External Entity | Influence Description |
| --- | --- | --- |
| Accept_AudioSource | Driver To HMI | Accept Audio source requests |
| Activation Status | My Seat Space To Rear Audio Control Subscriber | When MSS is activated the RACM will be locked out for the second row passengers |
| AudioZoneReq | Ultimate Remote Control Info Provider To My Seat Space | Request to control audio |
| Audio_St | My Seat Space To Ultimate Remote Control Info Provider | Simple and centralized way for all passengers to control of audio features in their seating zone |
| BluetoothConnection_Notification | HMI To 2nd Row Passengers | Bluetooth connection notification |
|  | HMI To 3rd Row Passenger | Bluetooth connection notification |
|  | HMI To Driver | Bluetooth connection notification |
|  | HMI To Front Row Passenger | Bluetooth connection notification |
| Bluetooth[FrontSeats]_ConnectionReq | HMI To My Seat Space | Request to pair bluetooth |
| Captain's Announcement | Driver To HMI | Enable communication |
|  | Front Row Passenger To HMI | Enable communication |
| Control_Share | HMI To My Seat Space | Share content to other available seats |
| Disable_GeneralSoundZones | My Seat Space To Audio System Provider | Enable Do Not Disturb (DND) |
| Disable_SoundZones | 2nd Row Passengers To HMI | Disable sound on specific vehicle zone |
|  | 3rd Row Passenger To HMI | Disable sound on specific vehicle zone |
|  | Driver To HMI | Disable sound on specific vehicle zone |
|  | Front Row Passenger To HMI | Disable sound on specific vehicle zone |
| Driver's Announcement | CA Info Provider To My Seat Space | CA will play across the cabin speakers, delivering the driver announcement. |
| DriverSeatVolume_Ctrl | Driver To HMI | Adjust volume of driver seat |
| Duck/Mute Audio | CA Info Provider To My Seat Space | CA can duck/mute audio throughout the vehicle to share an announcement to passengers. |
| Enable_GeneralSoundZones | My Seat Space To Audio System Provider | Disable DND |
| Enable_SoundZones | 2nd Row Passengers To HMI | Enable sound on specific vehicle zone |
|  | 3rd Row Passenger To HMI | Enable sound on specific vehicle zone |
|  | Driver To HMI | Enable sound on specific vehicle zone |
|  | Front Row Passenger To HMI | Enable sound on specific vehicle zone |
| FrontSeatVolume_Ctrl | HMI To My Seat Space | Control Front passenger seat volume |
| HMI Status | HMI To My Seat Space | HMI Status (Enabled/Disabled) |
| In-CarCommunications_Req | 2nd Row Passengers To HMI | Request for in car communication activation |
|  | 3rd Row Passenger To HMI | Request for in car communication activation |
|  | Driver To HMI | Request for in car communication activation |
|  | Front Row Passenger To HMI | Request for in car communication activation |
| In-VehicleNoises | Host Vehicle Environment To My Seat Space | -Bluetooth Interference -Ignition System -Seat Belt Status -Door Ajar Status -Other Occupants |
| IndividualSeatVolume_Ctrl | 2nd Row Passengers To HMI | Adjust individual seat zone volume |
|  | 3rd Row Passenger To HMI | Adjust individual seat zone volume |
| Media | External Audio Device To My Seat Space | Audio or video |
| MSS Lockout | Driver To HMI | •There will be a button on the MSS Sync Application that can disable the feature •This “disable” will prevent the vehicle from going into zone mode and grey out actions for passengers via URC. •Once the button is activated, the vehicle will enter/stay in Full Cabin Mode. •Users will not be able to share audio to a zone or take a phone call in their seat. •CA and ICC will function as normal, as this doesn’t change the sound mode. |
|  | Front Row Passenger To HMI | •There will be a button on the MSS Sync Application that can disable the feature •This “disable” will prevent the vehicle from going into zone mode and grey out actions for passengers via URC. •Once the button is activated, the vehicle will enter/stay in Full Cabin Mode. •Users will not be able to share audio to a zone or take a phone call in their seat. •CA and ICC will function as normal, as this doesn’t change the sound mode. |
| NaviPromt_Notification | HMI To Driver | Navigation prompt notification |
| On-Vehicle_Noises | External Environment To My Seat Space | -Android Auto/Apple CarPlay -3rd Party Apps -Bluetooth Interference |
| PassengerSeatVolume_Ctrl | Front Row Passenger To HMI | Adjust passenger seat volume |
| PhoneCall | External Communication Device To My Seat Space | Incoming/outgoing Phone call |
| Play_Music | My Seat Space To Audio System Provider | Play Music |
| Prevent ZoneMode | Mykey Info Provider To My Seat Space | MyKey will prevent the system from going into zone mode, restricting how audio can be shared from URC |
| RearSeat_HMIConnection | Ultimate Remote Control Info Provider To My Seat Space | For sharing/shared audio sources |
| Reduce Audio | Mykey Info Provider To My Seat Space | MyKey will reduce maximum audio levels |
| Restrict Communication | Mykey Info Provider To My Seat Space | Captain’s Announcement and In-Car Communication will work as before, but only activated by the driver. |
| Share_ AudioSource | 2nd Row Passengers To HMI | Share audio source command |
|  | 3rd Row Passenger To HMI | Share audio source command |
|  | Driver To HMI | Share audio source command |
|  | Front Row Passenger To HMI | Share audio source command |
| SoundZones_Cancelation | HMI To My Seat Space | Enable DND |
| SoundZones_Setup | HMI To My Seat Space | Setup for each sound zone |
| Source_Data | Microphone/Speech Processing To My Seat Space | Provides information/audio source information delivered by microphone network (first row, second row, third row) |
| Speaker_Command | My Seat Space To Seat Speaker Sys Provider | Enable speaker |
| SurroundSoundVolume_Ctrl | Driver To HMI | Control of the surround sound volume |
| TrafficAnnouncements | Traffic Announcements provider To My Seat Space | •Once this is activated by the driver, traffic announcements will play roughly every 30 minutes. •This will duck the audio for the listener and play the traffic announcement. •Typically this will play only for the driver. •Whoever is listen to the driver’s media will also receive the announcement, i.e. cases of everyone listening to the same source, or in zone mode. |
| VehicleNavigationSt | Vehicle Navigation Info Provider To My Seat Space | Vehicle's Navigation information |

Table 9: List of Influences

# Feature Modeling

## Operation Modes and States

## Use Cases

### Use Case Diagram

Figure 6: Feature Use Cases

### Actors

| Actor | Description |
| --- | --- |
| Audio System Provider | System responsible of audio arbitration |
| CA Info Provider | Captain's Announcement Info Provider |
| Driver | User seating on the driver's seat |
| External Communication Device | Any device capable of cellular communication |
| Front Row User | User seating on 1st row (driver's or front passenger's seat) |
| HMI | HMI interface, may be SYNC or brought in device. |
| ICC's Allowed User | Entity representing the Front Row and/or 3rd Row users |
| Microphone/Speech Processing | Microphones on each seat |
| Rear Zone Passengers | The Rear Zone Passengers represent the 2nd and 3rd row passengers |
| Seat Speaker Sys Provider | Speakers on each row |
| Ultimate Remote Control Info Provider | Ultimate Remote Control. Reductive MSS design requires it to control seat zones |
| User | Vehicle occupants |

Table 12: List of Actors

### Use Case Descriptions

UC7_ Manage Do Not Disturb

| Actors | Primary | User |
| --- | --- | --- |
|  | Secondary | HMI |
|  | Secondary | Ultimate Remote Control Info Provider |
|  | Secondary | Audio System Provider |
| Subject |  | My Seat Space |
| Description |  |  |
| Preconditions | PreC1 | HMI Enabled |
|  | PreC2 | User phones are connected to the vehicle through URC. |
| Main Flow Description |  | Do Not Disturb is a feature that allows passengers to auto-deny media share requests or changes being made to their sound zone. In some cases, Do Not Disturb becomes limited to muting specific seats. The driver is able to activate/deactivate Do Not Disturb only for users that do not have URC. |
| Main Flow | M1 | Passenger activates Do Not Disturb via URC while listening to their own media. |
| Alternative Flow Description |  | A1. Driver activates Do Not Disturb for a user without URC. (Can only do this for users without URC) |
| Alternative Flow Description |  | A2. Passenger activates Do Not Disturb while media is being shared to their zone. (Not from the "2 Zone Audio Share" request from the driver) |
| Alternative Flow Steps | A1 | A1.1 Driver activates Do Not Disturb for a user without URC via the front SYNC screen. |
|  | A2 | A2.1 Passenger activates Do Not Disturb via URC. |
|  | A3 | A2.2 Audio stops sharing to their zone, the passenger zone goes silent until the passenger selects audio via URC. |
| Exceptional Flow Description |  | E1. Driver deactivates Do Not Disturb for a user without URC. (Can only do this for users without URC) |
| Exceptional Flow Description |  | E2. Passenger deactivates Do Not Disturb for their zone. |
| Exceptional Flow Description |  | E3. The driver activates "2 Zone Audio Share", Do Not Disturb is disabled. |
| Exceptional Flow Steps | E1 | E1.1 Driver deactivates Do Not Disturb for a user without URC via the front SYNC screen. |
|  | E2 | E1.2 Sharing to seats/zones without URC is automatically accepted while the vehicle is in zone mode. |
|  | E3 | E2.1 Passenger deactivates Do Not Disturb for their zone via URC. |
|  | E4 | E2.2 Sharing media will prompt a request to users who have disabled Do Not Disturb. |
|  | E5 | E3.1 Vehicle locks into 2 zone sharing configuration on URC. Cannot be undone except by driver. |
|  | E6 | E3.2 Do Not Disturb is disabled for URC. Muting is still allowed. |
| Postconditions | PostC1 | Sharing and changes to the passenger seat/zone is automatically denied for users with the Do Not Disturb status active. Driver is unable to push the vehicle to 2 zone configuration, option is greyed out on Driver HMI. ICC still plays for a zone in DND. |

UC1_Change Sound Mode

| Actors | Primary | Driver |
| --- | --- | --- |
|  | Secondary | Ultimate Remote Control Info Provider |
|  | Secondary | HMI |
| Subject |  | My Seat Space |
| Description |  |  |
| Preconditions | PreC1 | HMI Enabled |
|  | PreC2 | MSS Global Feature Mode is on |
| Main Flow Description |  | The feature allows users to enable sound zones for an immersive experience. When zone mode is activated, users will be able to adjust audio levels in their seat relative to their zone. Users will be able to share media to the specified zone and the other users within the zone. Zone mode transitioned are initiated by use cases such as audio sharing, phone calls, and turning off the vehicle. |
| Main Flow | M1 | Driver/Passengers connect phones to vehicle. |
|  | M2 | The vehicle is in Full Cabin Mode. |
|  | M3 | The driver shares a separate audio source to the rear zone via the front SYNC screen. |
| Alternative Flow Description |  | A1 These cases are all alternative actions that can change the vehicle from Full Cabin Mode to Zone Mode. These steps would replace M3 and continue to the post condition. |
| Alternative Flow Description |  | A2 The vehicle is in Zone Mode and this alternate flow replaces "zone mode" for "cabin mode" for M2. The following steps will each replace M3 and continue to the post condition. |
| Alternative Flow Steps | A1 | A1.1 Passenger selects a separate audio source for the passenger zone via URC application or front SYNC screen. |
|  | A2 | A1.2 The driver accepts the audio share transition. |
|  | A3 | A2.1 Driver selects a single audio source for the entire vehicle via the front SYNC screen. |
|  | A4 | A2.2 A passenger requests to share audio with the entire vehicle, driver accepts. |
|  | A5 | A2.3 The vehicle is shut off and turned back on. |
|  | A6 | A2.4 The driver/front row passenger turns off MSS Zone Mode functionality via front SYNC screen. |
|  | A7 | A2.5 The driver/front row passenger selects the cabin mode button via front SYNC screen. |
| Exceptional Flow Description |  | The vehicle is in Zone Mode, specifically not in 2 zones. The driver switches zone mode from 4 zones to 2 zones with audio share shortcut. Continuation from M1. |
| Exceptional Flow Steps | E1 | E1.1 Passenger/Driver activates Zone Mode by sharing audio to a zone. |
|  | E2 | E1.2 Driver/Front row passenger switches zone by activating "2 Zone Audio Share" |
|  | E3 | E1.3 A beep plays across the cabin speakers, alerting driver/passengers of the MSS mode/media change. |
|  | E4 | E1.4 Driver or front row passenger audio plays for the entire front row, Seat 3 or Seat 4 audio plays for second and third rows. |
|  | E5 | E1.5 Passenger audio share selections are locked into 2 zone configuration (front/rear) via URC. |
| Postconditions | PostC1 | A beep plays across the cabin speakers, alerting driver/passengers of the MSS mode change. Vehicle switches audio modes. (Zone/Cabin) |

UC5_Make a Captain's Announcement

| Actors | Primary | Front Row User |
| --- | --- | --- |
|  | Secondary | CA Info Provider |
|  | Secondary | HMI |
|  | Secondary | External Communication Device |
|  | Secondary | Microphone/Speech Processing |
| Subject |  | My Seat Space |
| Description |  |  |
| Preconditions | PreC1 | HMI Enabled |
|  | PreC2 | Vehicle is turned on. |
| Main Flow Description |  | The feature allows the driver to easily speak with the entire vehicle. When Captain's Announcement is activated, audio mutes/pauses throughout the cabin and the driver's speech is played through the vehicle speakers. |
| Main Flow | M1 | Voice recognition is turned off throughout the vehicle. |
|  | M2 | Audio is muted/paused throughout the vehicle. |
|  | M3 | A beep occurs over the cabin speakers, alerting passenger(s) of an incoming captain’s announcement. |
|  | M4 | CA picks up Driver microphone and plays audio through the entire cabin speakers. |
|  | M5 | CA is turned off by Driver/1st Row Passenger. |
|  | M6 | Driver/1st Row Passenger activates CA |
| Alternative Flow Description |  | CA cannot be activated when a phone call is active in the vehicle. |
| Alternative Flow Steps | A1 | There is an active phone call in the vehicle. |
|  | A2 | User is alerted via SYNC screen that there is an active phone call in the vehicle and CA cannot be activated at this time. |
| Postconditions | PostC1 | Audio returns to previous state before Captain's Announcement was activated. |

UC6_Lock MSS

| Actors | Primary | Driver |
| --- | --- | --- |
|  | Secondary | HMI |
|  | Secondary | Ultimate Remote Control Info Provider |
| Subject |  | My Seat Space |
| Description |  |  |
| Preconditions | PreC1 | HMI Enabled |
| Main Flow Description |  | MySeatSpace offers multiple options for the driver to lockout features for the passengers. The driver can lock URC features for passengers or force the vehicle into full cabin mode. |
| Main Flow | M1 | Driver wants to lock audio/climate/seat position control features from passengers. |
|  | M2 | Turns off URC control from the rear seat controls lockout menu in SYNC. |
| Alternative Flow Description |  | A1 Driver turns Global MySeatSpace button off via the MySeatSpace SYNC screen. Continuation from M1. |
| Alternative Flow Steps | A1 | A1.1 Driver turns Global MySeatSpace button off via the MySeatSpace SYNC screen. |
|  | A2 | A1.2 Zone mode is disabled within the vehicle. In Car Communication and other URC controls are still available for passengers. |
|  | A3 | A1.3 URC actions related to audio control such as sharing music to a specific zone, selecting music for a specific zone, or taking a phone call are greyed out via URC HMI. |
| Exceptional Flow Description |  | Driver turns off audio control from the rear seat controls lockout menu in SYNC. Continuation from M1. |
| Exceptional Flow Steps | E1 | E1 Driver turns off audio control from rear seat controls lockout menu in SYNC. |
|  | E2 | E2 Passengers lose audio control via URC. Still have access to In Car Communication and all other URC controls. Results in exception flow E1. |
|  | E3 | E3 URC actions related to audio control such as sharing music, selecting music, or taking a phone call are greyed out via URC HMI. |
| Postconditions | PostC1 | All passenger options within URC are made unavailable. |

UC2_Make a Phone Call

| Actors | Primary | User |
| --- | --- | --- |
|  | Secondary | Audio System Provider |
|  | Secondary | Ultimate Remote Control Info Provider |
|  | Secondary | HMI |
|  | Secondary | Microphone/Speech Processing |
|  | Secondary | Seat Speaker Sys Provider |
|  | Secondary | External Communication Device |
| Subject |  | My Seat Space |
| Description |  |  |
| Preconditions | PreC1 | HMI Enabled |
|  | PreC2 | MSS Global Feature is turned on. |
|  | PreC3 | URC functionality is not locked out. |
|  | PreC4 | User phones are connected to the vehicle through URC. |
| Main Flow Description |  | MSS allows users to take phone calls directly through their seat speakers. A user will be able to accept their call, speak into the microphone dedicated to their seat, and listen to their phonecall through their dedicated seat speakers. |
| Main Flow | M1 | Audio is playing throughout the vehicle. |
|  | M2 | Driver receives a phone call. |
|  | M3 | Driver takes phone call. |
|  | M4 | Audio pauses/mutes throughout the cabin. |
|  | M5 | Driver chooses to send current driver SYNC source to other passengers (at the same time) |
|  | M6 | Vehicle enters zone mode. Driver listens to the phone call, passengers listening to driver selected audio source. |
|  | M7 | Driver phone call ends. |
| Alternative Flow Description |  | A1: The driver decides to do nothing after taking the phone call. Continuation from M4 |
| Alternative Flow Description |  | A2: Driver takes the phone call while the vehicle is already in zone mode. Continuation from M2. |
| Alternative Flow Description |  | A3: Passenger receives a phone call while sharing audio in zone mode. Continuation from M1. |
| Alternative Flow Description |  | A4: Passenger receives a phone call while not sharing audio (zone/cabin mode). Continuation from M1. |
| Alternative Flow Description |  | A5: The vehicle is in Zone Mode. First row passenger receives a phone call and takes a phone call. Driver then receives and takes a phone call. Continuation from M1. |
| Alternative Flow Description |  | A6: The vehicle is in Zone Mode. First row passenger receives a phone call and takes a phone call. Driver then receives and denies a phone call. Continuation from M1. |
| Alternative Flow Description |  | A7: The vehicle is in Zone Mode. Rear zone passenger receives and takes a phone call. Another rear zone passenger receives a phone call. Continuation from M1. |
| Alternative Flow Description |  | A8: A user takes a phone call in zone mode. The system takes too long to play the phone call in the seat speakers (X seconds), pushes the phone call to the phone. |
| Alternative Flow Steps | A1 | A1.1 Driver does nothing, driver phone call plays over cabin. |
|  | A2 | A1.2 Driver phone call ends. |
|  | A3 | A2.1 Vehicle is in zone mode. Driver takes the phone call. |
|  | A4 | A2.2 Audio is muted for the driver, and anyone listening to driver media. Phone call only plays through driver headrest speakers. |
|  | A5 | A2.3 Driver phone call ends. |
|  | A6 | A3.1 Passenger is sharing audio to their current zone. Passenger receives a phone call. |
|  | A7 | A3.2 Passenger takes the phone call. Audio stops playing for the rest of the zone. Passenger receives the phone call through the headrest speaker. |
|  | A8 | A3.3 Driver/Passengers do nothing. Passenger phone call ends. |
|  | A9 | A4.1 Passenger receives a phone call and is not the user sharing audio. |
|  | A10 | A4.2 Passenger takes the phone call. If the vehicle is in zone mode and the passenger is in Seat 3 or 4, Seat 5 or 6 is muted respectively. |
|  | A11 | A4.3 If in Cabin Mode, the phone call is pushed to the phone. If in Zone mode, the phone call audio is pushed to the passenger zone speakers |
|  | A12 | A4.4 Phone call ends. |
|  | A13 | A5.1 First Row Passenger receives/takes a phone call. |
|  | A14 | A5.2 The driver receives a phone call. |
|  | A15 | A5.3 Driver takes the phone call, it will play over the driver's headrest speakers. This will force the phone call of the first row passenger to the phone only. |
|  | A16 | A5.4 Driver phone call ends. If the first row passenger phone call is still active, the phone call remains playing over the phone only. |
|  | A17 | A6.1 First Row Passenger receives/takes a phone call. |
|  | A18 | A6.2 The driver receives a phone call. |
|  | A19 | A6.3 Driver denies the phone call. First Row Passenger phone call continues unchanged. |
|  | A20 | A6.4 First row passenger phone call ends. |
|  | A21 | A7.1 Rear zone passenger receives/takes a phone call. |
|  | A22 | A7.2 Another rear zone passenger receives a phone call. |
|  | A23 | A7.3 The second rear zone passenger is alerted that the phone call will be pushed to their phone and that there is another active phone call in session already. |
|  | A24 | A7.4 Both phone call ends, another phone call can now be taken in the rear zone. No active secondary phone calls will switch to headrest speakers, only new phone calls after the first call ends. |
|  | A25 | A8.1 A user receives/takes a phone call. |
|  | A26 | A8.2 X seconds pass and the user phone call is not playing through the user's seat speakers. |
|  | A27 | A8.3 The system pushes the phone call directly to the phone after passing the wait threshold. |
| Exceptional Flow Description |  | E1. The driver sends a different SYNC source to any zone passengers. Continuation from M4 |
| Exceptional Flow Description |  | E2. Passenger(s) select their own audio source for the their zone. Continuation from M4 |
| Exceptional Flow Description |  | E3. Passenger receives a phone call while sharing audio in zone mode. Continuation from M1. |
| Exceptional Flow Steps | E1 | E1.1 The driver selects a different SYNC audio source to any passenger zone. |
|  | E2 | E1.2 Vehicle enters zone mode. Driver listens to phone call, driver selected zone listens to driver selected audio source |
|  | E3 | E1.3 Driver phone call ends. |
|  | E4 | E1.4 Audio remains in zone mode as media sharing has changed from prior to the phone call. |
|  | E5 | E2.1 Passengers select audio source for the rear zone via URC |
|  | E6 | E2.2 Vehicle enters zone mode. Driver listens to phone call, passenger listen to passenger selected audio source. |
|  | E7 | E2.3 Driver phone call ends. |
|  | E8 | E2.4 Audio remains in zone mode as media sharing has changed from prior to the phone call. |
|  | E9 | E3.1 Passenger is sharing audio to their current zone. Passenger receives a phone call. |
|  | E10 | E3.2 Passenger takes the phone call. Audio stops playing for the rest of the zone. Passenger receives the phone call through the headrest speaker. |
|  | E11 | E3.3 Driver/Passengers select a new audio source. Passenger phone call ends. |
|  | E12 | E3.4 Audio remains in zone mode as media sharing has changed from prior to the phone call. |
| Postconditions | PostC1 | Audio returns to original state, in terms of audio mode and audio that was playing prior to the phone call. |

UC3_Share Audio

| Actors | Primary | Rear Zone Passengers |
| --- | --- | --- |
|  | Secondary | HMI |
|  | Secondary | Ultimate Remote Control Info Provider |
| Subject |  | My Seat Space |
| Description |  |  |
| Preconditions | PreC1 | HMI Enabled |
|  | PreC2 | MySeatSpace Global Functionality is turned on |
|  | PreC3 | No lockouts are activated for audio or URC feature |
|  | PreC4 | User phones are connected to the vehicle through URC. |
| Main Flow Description |  | MySeatSpace allows users to share audio to specific zones or to the entire vehicle. Passengers can share media from their phones via the URC application while the driver and first row passenger can share audio via the SYNC screen. |
| Main Flow | M1 | Passenger selects to play audio for their zone via URC. |
|  | M2 | In Cabin Mode, the driver receives and accepts the audio share to switch to zone mode. In zone mode, the audio does not need approval. |
|  | M3 | All other zones will continue to listen to the previous audio played for their zone/cabin. |
| Alternative Flow Description |  | A3 Any user requests to share audio with the entire vehicle via URC/SYNC screen. |
| Alternative Flow Description |  | A4 The driver activates the "2 Zone Audio Share" after users have shared audio for a four zone configuration. Both Seat 3 and Seat 4 (2nd row left and 2nd row right) passengers are sharing media via URC. Both the driver and front row passenger have separate media sources as well. |
| Alternative Flow Description |  | A1 The passenger chooses to share a media source with another zone. |
| Alternative Flow Description |  | A2 Driver/ 1st Row Passenger selects audio for the front zone and a separate audio source for another zone via SYNC screen. (2nd and 3rd row passengers are unable to select SYNC sources via URC application. Only front row can choose this for them.) |
| Alternative Flow Steps | A1 | A1.1 Passenger requests to share audio with specified zone via the URC app. |
|  | A2 | A1.2 The zone and driver accepts share request if this vehicle is in Cabin Mode. Only the zone accepts if the vehicle is in zone mode. |
|  | A3 | A1.3 If vehicle is in cabin mode, vehicle switches to zone mode. If not, vehicle stays in zone mode. |
|  | A4 | A2.1 2nd/3rd row passenger verbally requests to listen to SYNC source in the rear zone. |
|  | A5 | A2.2 Driver/1st Row Passenger selects an audio source for the front zone and a separate audio source for the rear zone via the SYNC screen. |
|  | A6 | A2.3 If vehicle is in cabin mode, vehicle switches to zone mode. If not, vehicle stays in zone mode. |
|  | A7 | A3.1 Any user requests to share audio with the entire vehicle via URC/SYNC screen. |
|  | A8 | A3.2 Driver accepts the share request if it is a passenger request. |
|  | A9 | A3.3 If vehicle is in zone mode, vehicle switches to full cabin mode. If not, vehicle stays in full cabin mode. |
|  | A10 | A4.1 Passenger requests to share audio to a user/zone without URC. |
|  | A11 | A4.2 MSS auto-accepts the share request. |
| Exceptional Flow Description |  | E1 User denies a share request. Continuation from M1. |
| Exceptional Flow Description |  | E2 The driver activates the "2 Zone Audio Share" after users have shared audio for a four zone configuration. Both Seat 3 and Seat 4 (2nd row left and 2nd row right) passengers are sharing media via URC. Both the driver and front row passenger have separate media sources as well. |
| Exceptional Flow Description |  | E3 The driver activates the "2 Zone Audio Share" after users have shared audio for a four zone configuration. Only one user is sharing media via URC in the second row (does not include a zone listening to a SYNC source such as AM/FM), and only one user is sharing media in the front row (does include a zone listening to a SYNC source.) |
| Exceptional Flow Description |  | E4 The driver activates the "2 Zone Audio Share" after users have shared audio for a four zone configuration. Both rear zones (Zone 3 and Zone 4) are playing separate SYNC sources such as AM/FM. |
| Exceptional Flow Description |  | E5 The driver selects "Stop Sharing" for a zone listening to a SYNC source. |
| Exceptional Flow Description |  | E6 A passenger presses "Stop Sharing" via URC. |
| Exceptional Flow Description |  | E7 A passenger presses "Stop Being Shared To" via URC. |
| Exceptional Flow Steps | E1 | E1.1 Passenger requests to share audio. (Either to entire cabin or to specified zone.) |
|  | E2 | E1.2 User denies the request. |
|  | E3 | E1.3 If the user denies a share request, the vehicle stays in the same audio mode and the current audio that is playing throughout the cabin/zone continues to play unchanged. |
|  | E4 | E2.1 Driver activates 2 Zone Audio Share. |
|  | E5 | E2.2 Rear zone audio will default to the Seat 3 media being shared prior to the Driver request. |
|  | E6 | E2.3 If there are two media sources playing in the front row, the entire first row will now play the Driver media source. |
|  | E7 | E2.4 URC sharing seat configuration changes/locks into 2 zone configuration for passengers. |
|  | E8 | E3.1 Driver activates 2 Zone Audio Share. |
|  | E9 | E3.2 Rear zone audio defaults to 2nd Row passenger sharing via URC. |
|  | E10 | E3.3 Front zone audio defaults to the only source playing from the front row prior to activation. |
|  | E11 | E3.4 URC sharing seat configuration changes/locks into 2 zone configuration for passengers. |
|  | E12 | E4.1 Driver activates 2 Zone Audio Share. |
|  | E13 | E4.2 Rear zone audio defaults to Seat 3/Zone 3 media. |
|  | E14 | E4.3 URC sharing seat configuration changes/locks into 2 zone configuration for passengers. |
|  | E15 | E5.1Driver selects a zone listening to a SYNC source. |
|  | E16 | E5.2Driver presses "Stop Sharing" button. |
|  | E17 | E5.3The selected zone stops receiving media and goes silent. |
|  | E18 | E6.1A passenger presses "Stop Sharing" via URC. |
|  | E19 | E6.2Any zone listening to the shared passenger media will go silent and stop receiving media. |
|  | E20 | E7.1A passenger presses "Stop Being Shared To" via URC. |
|  | E21 | E7.2Any media playing for the passenger's zone will stop sharing and the zone will go silent |
| Postconditions | PostC1 | A beep occurs over the cabin/zone speakers, alerting driver/passenger(s) of the audio change. Shared audio starts to play in specified seats or the entire cabin. |

UC4_Manage In-Car Communication

| Actors | Primary | ICC's Allowed User |
| --- | --- | --- |
|  | Secondary | Audio System Provider |
|  | Secondary | Seat Speaker Sys Provider |
|  | Secondary | Microphone/Speech Processing |
|  | Secondary | Ultimate Remote Control Info Provider |
|  | Secondary | HMI |
| Subject |  | My Seat Space |
| Description |  |  |
| Preconditions | PreC1 | HMI Enabled |
|  | PreC2 | User phones are connected to the vehicle through URC. |
| Main Flow Description |  | The feature allows ease of communication between the front row and the third row. When In-Car Communication is activate, speech picked up by the first and third row, and played for the other row via the headrest speakers. This will play concurrently with media to simulate a conversation while listening to music. ICC is planned to be on passively at all times to enhance communciation, rather than a toggle on/off for specific messages. There is the capability to turn off ICC for all users. |
| Main Flow | M1 | Driver/First Row Passenger/Third Row Passenger activates ICC through URC app or SYNC screen. |
|  | M2 | Voice recognition is turned off throughout the vehicle. |
|  | M3 | MSS picks up speech from first and third rows, play ICC audio through headrest speakers in the first and third rows. |
|  | M4 | Media plays concurrently with ICC audio. |
| Alternative Flow Description |  | A1. ICC is active within the vehicle. A phone call occurs which would deactivate ICC until the phone call is completed. Continuation from M1 |
| Alternative Flow Description |  | A2. ICC is active within the vehicle. A voice recognition session is started, disabling ICC for the duration of the voice recognition session. Continuation from M1. |
| Alternative Flow Steps | A1 | A1.1 Any user takes a phone call through SYNC/URC within the vehicle. |
|  | A2 | A1.2 ICC is disabled for the duration of the phone call. |
|  | A3 | A1.3 Phone call ends, ICC is enabled again. Continue to PostC1. |
|  | A4 | A2.1 Driver/First Row passenger activates voice recognition session. |
|  | A5 | A2.2 ICC is disabled for the duration of the VR session. |
|  | A6 | A2.3 Voice recognition session ends, ICC is enabled again. Continue to PostC1. |
| Exceptional Flow Description |  | Users turn off ICC. Continuation from M4. |
| Exceptional Flow Steps | E1 | Any user in the vehicle turns off ICC. |
|  | E2 | ICC audio no longer plays over headrest speakers. |
| Postconditions | PostC1 | ICC status is saved between different drive sessions. As this is a passive feature, users are anticipated to leave ICC on at all times. ICC would continue to play concurrently with media. ICC still plays if DND is active for a zone. |

## Driving and Operation Scenarios

Additional Driving Scenario 1

Description of the scenario in the Documentation field on the Feature Scenario Diagram.

UC1_Change Sound Mode

UC2_Make a Phone Call

UC3_Share Audio

UC4_Use In-Car Communication

UC5_Make a Captain's Announcement

UC6_Lock MSS

UC7_ Mange Do Not Disturb

## Decision Tables

Not supported by MagicDraw report generation.

# Feature Requirements

## Functional Requirements

Phone Calls 1.2

When there is a second phone call incoming in the rear zone (second/third row), MSS shall push the second phone call to the user's phone.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If there is an active phone call in the rear zone, the next phone call can only play through the phone. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Row to Row Communication 1.8

MSS shall only play In-Car Communication speech audio for the other row. (1st Row speech plays for the 3rd Row, 3rd Row speech plays for the 1st Row.)

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | First row speech will play for the third row, while third row speech will play for the first row. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1.13

If the front row passenger has an active phone call and the driver takes/receives a phone call, MSS shall push the front row passenger phone call to the phone.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Row to Row Communication 1.1

MSS shall allow users to activate and deactivate In-Car Communication via Ultimate Remote Control

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Passengers need a button to activate/deactivate ICC. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Do Not Disturb 1.10

If a user activates Do Not Disturb when media is being shared to their zone, media will stop being share to their zone and the zone will go silent until the user plays media for their zone. (Does not include driver forcing the "2 Zone Locked Configuration")

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If a passenger activates DND, active media to the passenger should stop playing. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Zone Transition 1.3

When the vehicle switches between zone/cabin mode, MSS shall activate a beep to alert driver/passenger(s).

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | When the vehicle is turned back on, the system will start in cabin mode |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Traffic Announcements 1

Traffic Announcments shall play for the driver and any passengers listening to driver audio.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | TA audio will play for the driver and any one listening to the driver audio source. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MSS Lockout 1

MSS shall allow the driver to lock passengers out of audio sharing controls.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | The driver can lock passengers from sharing audio. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1.7

If a passenger takes a phone call in Cabin Mode, MSS shall push the phone call to the handrest.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If the vehicle is in Cabin Mode, passenger phone calls will play through their phone. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Captain's Announcement 1.2

MSS shall activate a beep to alert passenger(s) when CA is activated.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | When Captain's Announcement is activated, a beep will occur. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Shared Media 1.1

MSS shall allow users listening to content from another sound zone to exit that audio stream.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Passenger can stop listening to music that is shared to them. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MyKey 1

When MyKey is turned on, MSS shall remove functionality to limit driver distraction.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If MyKey is active, functionality will be limited to limit driver distraction. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MSS Lockout 1.2

When the global MSS button is turned off, MSS shall disable the ability for users to share audio or take phonecalls through the vehicle.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | The driver can lock the ability to play audio in individual seats. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1.8

If a passenger takes a phone call in Cabin Mode, MSS shall keep the phone call in privacy mode if the vehicle switches to Zone Mode mid-call.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Active phone calls will continue to play through the phone, even when the vehicle switches to Zone Mode. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Do Not Disturb 1.3

When Do Not Disturb is active for a zone, MSS shall still play alerts and chimes audio direct to the zone.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Alerts and Chimes will still play when a user activates DND. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1

MSS shall allow each sound zone to receive its own phone call.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Users should be able to play a phone call from their seat speakers. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Zone Transition 1.2

When cabin mode is activated, MSS shall push audio to the cabin speakers.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Audio will play throughout the entire vehicle |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MSS Lockout 1.3

When the global MSS button is turned off, MSS shall grey out options within URC for sharing audio and taking phone calls.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MSS Audio 1.4

MSS shall match audio levels for Seat 3 and Seat 5, and match audio levels for Seat 4 and Seat 6.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Seat 3 and Seat 5 will have the same audio level. Seat 4 and Seat 6 will have the same audio level. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MSS Audio 1

MSS shall support individual sound zones for each seat.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Users can play different media in different seats. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MSS Audio 1.3

When a seat is muted, In-Car Communication, Captain's Announcement, and Phone Calls are not muted.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | ICC, CA, and Phone Call audio still plays if a seat is muted. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Zone Transition 1.5

When there is an active phone call within the vehicle, the vehicle cannot switch from zone mode to cabin mode.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Traffic Announcements 1.3

If a passenger is listening to driver media, traffic announcements shall play at the same audio level as media volume.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | TA audio volume level will not change for passengers |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1.3

When no other user is sharing media within the vehicle and the driver is taking a phone call, media shall pause/mute throughout the vehicle.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | When the driver has a phone call in cabin mode, media will stop playing throughout the vehicle. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Row to Row Communication 1

MSS shall allow users in the front and rear of the vehicle to be able to communicate back and forth effortlessly without raising their voice or changing their seating position.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | First and third row users will be able to talk to eachother effortlessly without having to yell. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status |  |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Captain's Announcement 1.1

When a captain’s announcement is made, MSS shall mute all media throughout the vehicle.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | When Captain's Announcement is activated, music will mute throughout the vehicle. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Do Not Disturb 1.2

When Do Not Disturb is active for a zone, MSS shall still play In-Car Communication audio for the zone.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | ICC audio will still play when a user activates DND. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1.11

If the front row passenger has an active phone call and the driver takes/receives a phone call, MSS shall push the front row passenger phone call to the phone.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If the driver takes a phone call while the front passenger has a phone call, the front passenger phone call will be pushed to the phone for the ideal customer experience. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1.5

MSS shall prioritize phone calls over Captain's Announcement and In-Car Communication

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Phone calls will disable CA and ICC. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MSS Audio 1.5

If Seat 3 is muted, MSS shall mute Seat 5. If Seat 4 is muted, MSS shall mute Seat 6.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Seat 5 must be muted if Seat 3 is muted. Seat 6 must be muted if Seat 4 is muted. Technical limitation. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Traffic Announcements 1.1

MSS shall duck/pause audio when a traffic announcment occurs.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Media audio levels will drop when a Traffic Announcement plays. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Do Not Disturb 1.9

If the vehicle is in a "2 Zone Locked Configuration", users cannot activate Do Not Disturb.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Passengers cannot activate DND if the "2 Zone Audio Share" is active. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Do Not Disturb

MSS shall allow a user to lock out in-car communication and media share requests in their sound zone.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Passengers can lock out other users from sharing media to them. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Row to Row Communication 1.4

MSS shall make Voice Recognition inactive when In-Car Communication is activated

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Voice recognition will be turned off when ICC is activated. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Navigation Prompts 1

MSS shall send navigation prompts and alerts to only the driver’s seat zone.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Navigation prompts will only play for the driver. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1.1

If the front row passenger receives a phone call while the driver has an active phone call, MSS shall push the first row passenger's phone call to the handset.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If the driver is in a phone call, the front passenger can only play their phone call through their phone. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Do Not Disturb 1.1

When Do Not Disturb is active for a zone, MSS shall still play Captain's Annoucement audio for the zone.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Captain's Announcement will still play when a user activates DND. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Zone Transition 1.4

When HMI status is reset (on/off), MSS shall default to cabin mode.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If a user is connected to the vehicle and is in a phone call, the vehicle cannot enter cabin mode. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1.4

When someone sharing audio receives/takes a phone call, MSS shall pause audio for all other users in that zone.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If someone is sharing media and takes a phone call, the media will stop playing for the duration of the phone call. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MyKey 1.1

When MyKey is turned on, MSS shall disable the ability to transition into zone mode.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If MyKey is active, zone mode is unavailable. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Traffic Announcements 1.2

MSS shall play traffic announcements at an increased audio level for the driver.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | TA audio needs to play at a higher volume level for the driver. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MSS Audio 1.1

MSS shall provide a means for a user to control their audio (volume, play/pause, etc).

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Users can adjust their seat volume with the URC application. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Captain's Announcement 1.5

If a phone call is active in the vehicle, MSS shall prompt the user that Captain's Announcement is not available when they attempt to activate Captain's Announcement.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Shared Media 1.4

MSS shall continue shared audio once a phone call completes, as long as there was no audio change by other users to the zone that the original audio was being shared to during the phone call.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Once a user ends a phone call, their music will continue to play |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MSS Lockout 1.1

When the global MSS button is turned off, MSS shall disable the ability for the vehicle to enter zone mode.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | The driver can lock the ability to enter zone mode. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MyKey 1.3

When MyKey is turned on, MSS shall only allow In-Car Communication to be activated by the front HMI screen.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Only the driver can activate ICC if MyKey is active. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Captain's Announcement 1.3

MSS shall play Captain's Announcement audio in vehicle speakers for the second and third rows only.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Captain's Announcement should not play for the first row, as they are the speaker |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1.14

MSS shall continue to play privacy phone calls through the phone, even if the headrest phone call option becomes available during the phone call.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale |  |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1.12

MSS shall continue to play privacy phone calls through the phone, even if the headrest phone call option becomes available during the phone call.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Phone calls will continue to play through the phone, even if the ability to play through seat speakers becomes available. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Navigation Prompts 1.1

MSS shall duck audio for the driver when Navigation Prompts play.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Media audio levels will drop when a navigation prompt plays. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MSS Audio 1.2

Muting shall only affect media streams.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Muting a seat will only mute the media playing in the seat. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Zone Transition 1.1

When zone mode is activated, MSS shall push audio to headrest speakers and zonal cabin speakers for each seat.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Audio will play in the headrest speakers for different users |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

MyKey 1.2

When MyKey is turned on, maximum audio level shall be reduced by 45%.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If MyKey is active, the maximum volume is reduced. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1.6

For a passenger to take a phone call through the headrest speakers and seat microphone, the vehicle must already be in Zone Mode.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If a passenger wishes to take a phone call through the seat speakers, the vehicle must be in zone mode. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Shared Media 1

MSS shall allow user to share audio content from their external audio device to any other sound zone.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Passengers can share media from their phone to other users. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Shared Media 1.5

Only the first and second row users shall have media sharing control.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Third row users cannot share media due to technical limitations. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

### Error Handling

No Error Handling Requirements specified.

## Non-Functional Requirements

### Security

No Security Requirements specified.

### Reliability

No Reliability Requirements specified.

### Performance

No Performance Requirements specified.

## HMI Requirements

Zone Transition 1

MSS shall allow the vehicle to switch between Full Cabin Mode and Zone Mode (full cabin audio and zonal cabin speakers).

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Need to be able to change the settings to play different audio streams in different speakers |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Shared Media 1.7

For the any passenger media share request to switch the vehicle from cabin mode to zone mode, MSS shall prompt the driver to accept/deny the share request.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Any passenger share requests in cabin mode will prompt the driver to accept/deny. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Shared Media 1.6

AM/FM/SDARS/SYNC sources are only selectable by driver/front row passenger. For rear zone passengers, must be chosen for them via the SYNC screen.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Vehicle audio sources can only be selected via the front HMI screen. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Shared Media 1.2

MSS shall allow driver to accept and listen to shared content.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | The driver needs to receive a prompt to accept music shares. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Row to Row Communication 1.7

MSS shall temporarily disable In-Car Communication when a phone call is received and will resume In-Car Communication once the phone call is completed.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | ICC will be turned off when a phone call is taken, |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Do Not Disturb 1.7

If Do Not Disturb is active, MSS shall allow the driver to disable zone mode within the vehicle.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | The driver can still push the vehicle to cabin mode, even if a passenger has activated DND. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Captain’s Announcement 1

When a captain’s announcement is made, MSS shall duck the volume of any media playing in each sound zone. (Example)

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | The driver can easily talk to the entire vehicle without having to raise their voice or turn. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Do Not Disturb 1.4

When Do Not Disturb is activated by a passenger via URC, MSS shall prevent the driver from deactivating Do Not Disturb for this passenger.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | The driver cannot disable another user's DND status. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Row to Row Communication 1.3

MSS shall maintain In-Car Communication status (on/off) between SYNC screen cycles.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | ICC status shall be saved between drive sessions. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Row to Row Communication 1.2

MSS shall allow the first row to activate and deactivate In-Car Communication via the front HMI.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | The driver needs a button to activate/deactivate ICC. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Do Not Disturb 1.6

If Do Not Disturb is active in the vehicle, MSS shall prevent the "2 Zone Audio Share" from being activated.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If any passenger has DND active, the driver cannot activate "2 Zone Audio Share" |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Shared Media 1.8

After the driver has accepted a share request to transition to zone mode, MSS shall approve all future audio share requests for passengers only as long as the vehicle is in zone mode.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If the vehicle is in zone mode, the driver will only receives prompts for share requests that affect the driver. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Captain's Announcement 1.4

If a phone call is active in the vehicle, MSS shall prompt the user that Captain's Announcement is not available when they attempt to activate Captain's Announcement.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If there is a phone call in the vehicle, Captain's Announcement cannot be activated. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Shared Media 1.3

MSS shall allow driver to decline a shared content request.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | The driver needs to be able to deny music shares. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Do Not Disturb 1.11

If Do Not Disturb is deactivated, MSS shall allow media share requests to the zone again.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | Users can share media with the passenger once they disable DND. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Do Not Disturb 1.5

If a zone has no users with URC, MSS shall allow the driver to activate/deactivate Do Not Disturb for this zone.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | The driver can activate DND for a passenger if they do not have their phone connected to do it themselves. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Do Not Disturb 1.8

If Do Not Disturb is active, MSS shall allow the driver to disable URC functionality for passengers.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | The driver can disable URC actions, even if a passenger has activated DND. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1.10

When there is an active phone call in Zone Mode, MSS shall prevent the driver from pushing the vehicle into the "2 Zone Configuration"

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If there is a phone call in the vehicle, "2 Zone Audio Share" cannot be activated |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Row to Row Communication 1.5

MSS shall temporarily disable In-Car Communication when Voice Recognition is activated and will resume In-Car Communication once Voice Recognition session is completed.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | ICC will be turned off when Voice Recogniton is activated. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Phone Calls 1.9

When there is an active phone call in Zone Mode, MSS shall prevent the driver from pushing the vehicle to Cabin Mode.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | If there is a phone call in the vehicle, Cabin mode cannot be activated. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Shared Media 1.9

MSS shall prompt the driver for a share request for any requests that changes driver audio.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | The driver will receive a prompt whenever a share request affects driver audio. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Row to Row Communication 1.9

If a passenger activates ICC via URC, the driver shall receive a prompt alerting for the activation.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | The driver needs to be alerted if a passenger activates ICC |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

Row to Row Communication 1.6

MSS shall temporarily disable In-Car Communication when Captain's Announcement is activated, and will resume In-Car Communication when Captain's Announcement is completed.

| Requirement ID: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rationale | ICC will be turned off when CA is activated. |  |  |  |  |  |  |
| Acceptance Criteria |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Source |  |  |  |  |  | Owner |  |
| Source Req. |  |  |  |  |  | V&V Method |  |
| Type |  |  |  | Priority |  | Status | In-Progress |
| Req. Template Version |  | 6.0 | End of Requirement |  |  |  |  |

## Other Requirements

### Design Requirements

Not supported by MagicDraw report generation.

### Manufacturing Requirements

No Manufacturing Requirements specified.

### Service Requirements

No Service Requirements specified.

#### Cloud Connectivity Data Analytics Requirements

### After Sales Requirements

No After Sales Requirements specified.

### Process Requirements

No Process Requirements specified.

# Functional Safety

## System Behaviors for HARA

| ID | Name | Description |
| --- | --- | --- |
|  | Activate Share In Audio Stream |  |
|  | Deactivate Nagviation Prompts |  |
|  | Deactivate Do Not Disturb |  |
|  | Display MSS Status |  |
|  | Activate Make an MSS Phone Call |  |
|  | Activate Listen to Own Audio |  |
|  | Deactivate Make an MSS Phone Call |  |
|  | Deactivate Share In Audio Stream |  |
|  | Activate Do Not Disturb |  |
|  | Deactivate In-Car Communication |  |
|  | Adjust MSS Volume |  |
|  | Deactivate Listen to Own Audio |  |
|  | Activate Captain's Announcement |  |
|  | Activate In-Car Communication |  |
|  | Deactivate Captain's Announcement |  |
|  | Activate Nagivation Prompts |  |

Table 13: System Behaviors for HARA

## Functional Safety Assumptions

| ID | Assumption |  |
| --- | --- | --- |
|  | Name | MSS Headrest Speaker Volume |
|  | Description | MSS Headrest Speakers have the power and volume capability of personal headphones, which have a very low potential to emit hazardous noise or radiation. |
|  | Purpose | Establish potentail for MSS speakers to create hazard |
|  | Category |  |
|  | Related Requirement IDs |  |

Table 14: Functional Safety Assumptions

## Safety Goals

| ID | Goal |  |  |  |
| --- | --- | --- | --- | --- |
|  | Goal Name | Feature/System must not expose Driver or Passenger to Loud or Harmful Sounds or Noises. |  |  |
|  | Description |  |  |  |
|  | Safety Goal Concept | Safety Goal Concept: Warning & Recovery Concept: |  |  |
|  | ASIL |  | FTTI |  |
|  | Related FSR IDs |  |  |  |

Table 15: Functional Safety Goals

## Functional Safety Requirements

### Safety Goal:  Feature/System must not expose Driver or Passenger to Loud or Harmful Sounds or Noises.

Name: Feature/System must not expose Driver or Passenger to Loud or Harmful Sounds or Noises.

Purpose:

Text:

ASIL:

#### Safety Goal Concept

Figure 1: SG #1 Diagram – Feature/System must not expose Driver or Passenger to Loud or Harmful Sounds or Noises.

Note: The authoritative source for the Safety Goals is document “FFSD 02 Hazard Analysis and Risk Assessment”. The documentation of Safety Goals in this chapter (In the Argumentation for Safety Goal achievement) is for information purposes only.

The authoritative source for the Functional Safety Requirements is section 2.1.x.3: of this document. The documentation of Functional Safety Requirements in the following chapter (complete or summarised) is for information purposes only.

#### Warning and Recovery Concept

Figure 7: W&RC #1 Diagram – Feature/System must not expose Driver or Passenger to Loud or Harmful Sounds or Noises.

### Derivation of Functional Safety Requirements on Assumptions

No Functional Safety Requirements tracing to Assumptions specified.

### ASIL Decomposition of Functional Safety Requirements

No Functional Safety Requirements with ASIL Decompositions specified.

# CyberSecurity

## Security Goals

| ID | Goal |
| --- | --- |

Table 18: Cybersecurity Goals

## Cybersecurity Requirements

# Architecture

## Functional Decomposition

### Functions

## Logical Architecture

Description of diagram and content on logical architecture in Documentation field of Structural Boundary Diagram.

Figure 9: Logical Architecture

### Logical Elements

| Element Name | Description | Allocated Functions | Comments |
| --- | --- | --- | --- |
| Amplifier | Logical Entity Provider of the Amplifier Status | - System Function |  |
| Headrest/Cabin Speakers | MSS logical abstraction of an entity that provides the speaker status |  |  |
| Key | Logical Abstraction of a vehicle Key (Passive Key, PaaK, etc) |  |  |
| Microphone Status Provider | MSS logical abstraction of an entity that provides the microphone status |  |  |
| MSS HMI | MSS internal logical entity representing the Human -Machine Interface with the feature | - Request CA Status Change - Request DND Status Change - Request ICC Status Change ON/OFF - Request Audio Share Response - Request Share - Request Source Change - Request Volume Change - Request Sound Mode Change - Request Zone Lock Out Status Change - Request CA Status Change - Request DND Status Change - Request ICC Status Change ON/OFF - Request Audio Share Response - Request Share - Request Source Change - Request Sound Mode Change - Request Zone Lock Out Status Change - Request Volume Change |  |
| MSS Logical Database | MSS internal database entity | - Check HMI Status - Update HMI Status |  |
| MSS Logical Manager | MSS logical controller, it receives the request, executes and transmits the result. | - Request Share - Request Audio Share Response - Route Audio - Play Audio - Request Call - Request DND Status Change - Request Source Change - Request Volume Change - Request ICC Status Change ON/OFF - Request CA Status Change - Connect BT Audio - Request BT Status Change - Check HMI Status - Update HMI Status - Change Volume - Request Sound Mode Change - Request Zone Lock Out Status Change - Request Sound Mode Change - Request Zone Lock Out Status Change - Change Volume |  |
| My Seat Space |  | - System Function |  |
| Smartphone/Media Device | MSS logical abstraction for user's phone/Media device (not associated as PaaK) |  |  |
| User Interactions | Logical Abstraction of the several user interactions with the feature |  |  |

Table 19: Logical Elements

### Logical Interfaces

| Interface | Direction | Description | Value Range |
| --- | --- | --- | --- |
| AudioShareResp_Command | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Audio Share Resp Command (Physical Touch) |  |
|  | OF1_p1 (User Interactions) To FN2_p1 (MSS HMI) | Audio Share Resp Command (Physical Touch) |  |
|  | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Audio Share Resp Command (Physical Touch) |  |
| AudioShareResp_Req | FN3_p5 (MSS Logical Manager) To OF2_p1 (Smartphone/Media Device) | Internal Signal for Audio SHare Response | Accept Decline |
|  | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Internal Signal for Audio SHare Response | Accept Decline |
| Audio_Vol_Level | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal for Audio volume in full cabin mode | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal for Audio volume in full cabin mode | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
| Audio_Vol_Level_Zone | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal is for the Media volume in Zone 1 to Zone 6. | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal is for the Media volume in Zone 1 to Zone 6. | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
| Audio_Vol_Updated | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal indicates if audio volume was updated because of a user activated event in full cabin mode | Updated NoUpdate |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal indicates if audio volume was updated because of a user activated event in full cabin mode | Updated NoUpdate |
| Audio_Vol_Zone_Updated | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal indicates if the Media volume was updated because of a user activated event in Zone 1 to Zone 6 | Updated NoUpdate |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal indicates if the Media volume was updated because of a user activated event in Zone 1 to Zone 6 | Updated NoUpdate |
| AutomotiveAudio_Req | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Internal Signal for Automotive Audio Req |  |
|  | OF5_p2 (Amplifier) To OF4_p1 (Headrest/Cabin Speakers) | Internal Signal for Automotive Audio Req |  |
| BluetoothAudioSource | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Internal Signal for Bluetooth Audio Source | Connected |
| BluetoothAudioSource_Connection | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Internal Signal for Bluetooth Audio Source | Connected |
|  | FN3_p5 (MSS Logical Manager) To OF2_p1 (Smartphone/Media Device) | Internal Signal for Bluetooth Audio Source | Connected |
|  | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Internal Signal for Bluetooth Audio Source | Connected |
| BTConnectionReq | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Internal Signal for Bluetooth Connection Request | Connect Disconnect |
| BT_Command | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Bluetooth Command (Physical Touch) |  |
|  | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Bluetooth Command (Physical Touch) |  |
| CA.Rq | FN1_p1 (MSS Logical Database) To FN3_p6 (MSS Logical Manager) | Signal to request activation or deactivation of Captain Annoucement | Null ActivateCA CancelCA |
| CA.St | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Signal to provide status feedback of Captain Annoucement State. | Inactive CA Active CA Disabled |
|  | FN3_p6 (MSS Logical Manager) To FN1_p1 (MSS Logical Database) | Signal to provide status feedback of Captain Annoucement State. | Inactive CA Active CA Disabled |
| CallRing_Vol_Level | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal for Call Ring volume in full cabin mode | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal for Call Ring volume in full cabin mode | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
| CallRing_Vol_Updated | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal indicates if Call ring volume was updated because of a user activated event in full cabin mode | Updated NoUpdate |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal indicates if Call ring volume was updated because of a user activated event in full cabin mode | Updated NoUpdate |
| CA_Command | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Captain's Announcement Command (Physical Touch) |  |
|  | OF1_p1 (User Interactions) To FN2_p1 (MSS HMI) | Captain's Announcement Command (Physical Touch) |  |
|  | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Captain's Announcement Command (Physical Touch) |  |
| ChangeVolume_Req | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Internal Signal for Change Volume |  |
| DND.Rq | FN3_p5 (MSS Logical Manager) To OF2_p1 (Smartphone/Media Device) | Internal Signal for Do Not Disturb | Enable Disable |
|  | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Internal Signal for Do Not Disturb | Enable Disable |
| DND_Command | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Do Not Disturb Command (Physical Touch) |  |
|  | OF1_p1 (User Interactions) To FN2_p1 (MSS HMI) | Do Not Disturb Command (Physical Touch) |  |
| HMI_Captains Announcement_ Status | FN3_p1 (MSS Logical Manager) To FN2_p2 (MSS HMI) | HMI Status update for CA |  |
| HMI_DND Status | FN3_p1 (MSS Logical Manager) To FN2_p2 (MSS HMI) | HMI Status update for DND Status |  |
| HMI_In-Car Communication Status | FN3_p1 (MSS Logical Manager) To FN2_p2 (MSS HMI) | HMI Status update for ICC |  |
| HMI_MediashareRequest_Feedback | FN3_p1 (MSS Logical Manager) To FN2_p2 (MSS HMI) | HMI Feedback update for Media Share Request |  |
| HMI_Phone Call_Status | FN3_p1 (MSS Logical Manager) To FN2_p2 (MSS HMI) | HMI Status update for Phone Call |  |
| HMI_Source Selection_Feedback | FN3_p1 (MSS Logical Manager) To FN2_p2 (MSS HMI) | HMI Feedback update for Source Selection |  |
| HMI_Volume Control_Feedback | FN3_p1 (MSS Logical Manager) To FN2_p2 (MSS HMI) | HMI Feedback update for Volume Control |  |
| HMI_ZoneLockOut_Feedback | FN3_p1 (MSS Logical Manager) To FN2_p2 (MSS HMI) | HMI Feedback update for Zone Lockout |  |
| HMI_Zone_Cabin_Mode_Status | FN3_p1 (MSS Logical Manager) To FN2_p2 (MSS HMI) | HMI Status update for Zone/Cabin Mode |  |
| ICC_Command | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | In Car's Communication Command (Physical Touch) |  |
|  | OF1_p1 (User Interactions) To FN2_p1 (MSS HMI) | In Car's Communication Command (Physical Touch) |  |
|  | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | In Car's Communication Command (Physical Touch) |  |
| ICC_Rq | FN1_p1 (MSS Logical Database) To FN3_p6 (MSS Logical Manager) | Signal to request activation or deactivation of InCar Communication | Inactive Active |
| ICC_St | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Signal to provide status feedback of InCar Communication | Inactive Enabled Temporarily Disabled |
|  | FN3_p6 (MSS Logical Manager) To FN1_p1 (MSS Logical Database) | Signal to provide status feedback of InCar Communication | Inactive Enabled Temporarily Disabled |
| KeyType | OF6_p1 (Key) To FN3_p2 (MSS Logical Manager) | Key Type (MyKey/NotMyKey) |  |
| MicrophoneStatus | OF3_p1 (Microphone Status Provider) To FN3_p3 (MSS Logical Manager) | Microphone Status (Enabled/Disabled) |  |
| MixableZonePrompts | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal indicates if mixable zone prompts are active or inactive for Zone 1 to Zone 6. | Inactive Active |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal indicates if mixable zone prompts are active or inactive for Zone 1 to Zone 6. | Inactive Active |
| MixableZonePromptsChannel | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal specifies audio channel for the mixable zone prompts on zone 1 to Zone 6 | Inactive A2B_ID1 A2B_ID2 A2B_ID3 A2B_ID4 A2B_ID5 A2B_ID6 A2B_ID7 A2B_ID8 A2B_ID9 A2B_ID10 A2B_ID11 A2B_ID12 A2B_ID13 A2B_ID14 A2B_ID15 A2B_ID16 A2B_ID17 A2B_ID18 A2B_ID19 A2B_ID20 A2B_ID21 A2B_ID22 A2B_ID23 A2B_ID24 A2B_ID25 A2B_ID26 A2B_ID27 A2B_ID28 A2B_ID29 A2B_ID30 A2B_ID31 A2B_ID32 Reserved for future expansion |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal specifies audio channel for the mixable zone prompts on zone 1 to Zone 6 | Inactive A2B_ID1 A2B_ID2 A2B_ID3 A2B_ID4 A2B_ID5 A2B_ID6 A2B_ID7 A2B_ID8 A2B_ID9 A2B_ID10 A2B_ID11 A2B_ID12 A2B_ID13 A2B_ID14 A2B_ID15 A2B_ID16 A2B_ID17 A2B_ID18 A2B_ID19 A2B_ID20 A2B_ID21 A2B_ID22 A2B_ID23 A2B_ID24 A2B_ID25 A2B_ID26 A2B_ID27 A2B_ID28 A2B_ID29 A2B_ID30 A2B_ID31 A2B_ID32 Reserved for future expansion |
| Offset_Vol_Zone | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal is for the offset volume in Zone 1 to Zone 6. | -3 -2 -1 0 1 2 3 Unused |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal is for the offset volume in Zone 1 to Zone 6. | -3 -2 -1 0 1 2 3 Unused |
| Offset_Vol_Zone_Updated | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal indicates if the Offset volume was updated because of a user activated event | Updated NoUpdate |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal indicates if the Offset volume was updated because of a user activated event | Updated NoUpdate |
| PhoneCall_Zone_Rq | FN3_p6 (MSS Logical Manager) To FN1_p1 (MSS Logical Database) | Signal requesting to switch the active phone call. This message is intended only for the Phones connected and mapped to zone 2. | NoCall CallRinging CallActive |
| PhoneCall_Zone_St | FN1_p1 (MSS Logical Database) To FN3_p6 (MSS Logical Manager) | Signal indicating the call status for the Phones connected and mapped to zone 2 | Inactive Privacy Handsfree |
| Phone_Vol_Level | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal for Phone volume in full cabin mode | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal for Phone volume in full cabin mode | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
| Phone_Vol_Updated | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal indicates if phone volume was updated because of a user activated event in full cabin mode | Updated NoUpdate |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal indicates if phone volume was updated because of a user activated event in full cabin mode | Updated NoUpdate |
| PlayAudio_Req | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Internal Signal for Play Audio Req |  |
|  | OF5_p2 (Amplifier) To OF4_p1 (Headrest/Cabin Speakers) | Internal Signal for Play Audio Req |  |
| Prompt_Vol_Level | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal for Prompt volume in full cabin mode | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal for Prompt volume in full cabin mode | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
| Prompt_Vol_Level_Zone | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal is for the Prompt volume in Zone 1 to Zone 6. | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal is for the Prompt volume in Zone 1 to Zone 6. | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
| Prompt_Vol_Updated | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal indicates if prompt volume was updated because of a user activated event in full cabin mode | Updated NoUpdate |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal indicates if prompt volume was updated because of a user activated event in full cabin mode | Updated NoUpdate |
| Prompt_Vol_Zone_Updated | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal is to indicate if the Prompt volume was updated because of a user activated event | Updated NoUpdate |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal is to indicate if the Prompt volume was updated because of a user activated event | Updated NoUpdate |
| RA_Vol_Level | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal for Radio Announcement volume in full cabin mode | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal for Radio Announcement volume in full cabin mode | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
| RA_Vol_Updated | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal indicates if RA volume was updated because of a user activated event in full cabin mode | Updated NoUpdate |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal indicates if RA volume was updated because of a user activated event in full cabin mode | Updated NoUpdate |
| Share_Command | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Request Share Command (Physical Touch) |  |
|  | OF1_p1 (User Interactions) To FN2_p1 (MSS HMI) | Request Share Command (Physical Touch) |  |
|  | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Request Share Command (Physical Touch) |  |
| Share_Rq | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Internal Signal for Audio Share Response | Accept Decline |
| SoundMode_Command | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Sound Mode Command (Physical Touch) |  |
|  | OF1_p1 (User Interactions) To FN2_p1 (MSS HMI) | Sound Mode Command (Physical Touch) |  |
|  | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Sound Mode Command (Physical Touch) |  |
| SourceType | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal specifies selection of source type of audio used for zone 1 to zone 6 | Inactive / Audio OFF Aux_Media AM FM SDARS SAT SDARS IP DAB Phone Call Ring Radio Announcement VR Priority Assist Reserved |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal specifies selection of source type of audio used for zone 1 to zone 6 | Inactive / Audio OFF Aux_Media AM FM SDARS SAT SDARS IP DAB Phone Call Ring Radio Announcement VR Priority Assist Reserved |
| SourceTypeChannel | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal specifies selection of audio channel for the audio source for zone 1 to zone 6 | Inactive BT_Zone2 Reserved for future expansion A2B_ID1 A2B_ID2 A2B_ID3 A2B_ID4 A2B_ID5 A2B_ID6 A2B_ID7 A2B_ID8 A2B_ID9 A2B_ID10 A2B_ID11 A2B_ID12 A2B_ID13 A2B_ID14 A2B_ID15 A2B_ID16 A2B_ID17 A2B_ID18 A2B_ID19 A2B_ID20 A2B_ID21 A2B_ID22 A2B_ID23 A2B_ID24 A2B_ID25 A2B_ID26 A2B_ID27 A2B_ID28 A2B_ID29 A2B_ID30 A2B_ID31 A2B_ID32 BT_Zone3 BT_Zone4 BT_Zone5 BT_Zone6 |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal specifies selection of audio channel for the audio source for zone 1 to zone 6 | Inactive BT_Zone2 Reserved for future expansion A2B_ID1 A2B_ID2 A2B_ID3 A2B_ID4 A2B_ID5 A2B_ID6 A2B_ID7 A2B_ID8 A2B_ID9 A2B_ID10 A2B_ID11 A2B_ID12 A2B_ID13 A2B_ID14 A2B_ID15 A2B_ID16 A2B_ID17 A2B_ID18 A2B_ID19 A2B_ID20 A2B_ID21 A2B_ID22 A2B_ID23 A2B_ID24 A2B_ID25 A2B_ID26 A2B_ID27 A2B_ID28 A2B_ID29 A2B_ID30 A2B_ID31 A2B_ID32 BT_Zone3 BT_Zone4 BT_Zone5 BT_Zone6 |
| SourceTypeStatus | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal specifies the source type status of audio source used for zone 1 to zone 6 | Inactive Deallocated Stacked Granted |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal specifies the source type status of audio source used for zone 1 to zone 6 | Inactive Deallocated Stacked Granted |
| Source_Command | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Source Command (Physical Touch) |  |
|  | OF1_p1 (User Interactions) To FN2_p1 (MSS HMI) | Source Command (Physical Touch) |  |
|  | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Source Command (Physical Touch) |  |
| TempSource_Vol_Level_Zone | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal is for the Phone, Call Ring, and Radio Announcement volume in Zone 1 to Zone 6. | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal is for the Phone, Call Ring, and Radio Announcement volume in Zone 1 to Zone 6. | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
| TempSource_Vol_Zone_Updated | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal indicates if the TempSource volume was updated because of a user activated event | Updated NoUpdate |
|  | OF5_p1 (Amplifier) To FN3_p4 (MSS Logical Manager) | Signal indicates if the TempSource volume was updated because of a user activated event | Updated NoUpdate |
| VehicleAudioMode | FN1_p1 (MSS Logical Database) To FN3_p6 (MSS Logical Manager) | Signal to request to select Cabin mode or Zone Mode. | Null Cabin Zone |
| VehicleAudioMode_Rsp | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Signal provides feedback on Audio mode selection | Null Cabin Zone |
|  | FN3_p6 (MSS Logical Manager) To FN1_p1 (MSS Logical Database) | Signal provides feedback on Audio mode selection | Null Cabin Zone |
| VolCntlr_Audio_Vol_Level_Zone | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Signal provides status of media volume that is active in zones 1 - 6 | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
|  | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal provides status of media volume that is active in zones 1 - 6 | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
| VolCntlr_Offset_Vol_Level_Zone | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Signal provides status of the offset volume that is active in zones 1 - 6 | -3 -2 -1 0 1 2 3 Unused |
|  | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal provides status of the offset volume that is active in zones 1 - 6 | -3 -2 -1 0 1 2 3 Unused |
| VolCntlr_TempSource_Vol_Level_Zone | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Signal provides status of Phone, Call Ring or Radio Announcement volume that is active in zones 1 - 6 | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
|  | FN3_p4 (MSS Logical Manager) To OF5_p1 (Amplifier) | Signal provides status of Phone, Call Ring or Radio Announcement volume that is active in zones 1 - 6 | NoVolume Vol_Step1 Vol_Step2 Vol_Step3 Vol_Step4 Vol_Step5 Vol_Step6 Vol_Step7 Vol_Step8 Vol_Step9 Vol_Step10 Vol_Step11 Vol_Step12 Vol_Step13 Vol_Step14 Vol_Step15 Vol_Step16 Vol_Step17 Vol_Step18 Vol_Step19 Vol_Step20 Vol_Step21 Vol_Step22 Vol_Step23 Vol_Step24 Vol_Step25 Vol_Step26 Vol_Step27 Vol_Step28 Vol_Step29 Vol_Step30 |
| Volume_Command | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Volume Command (Physical Touch) |  |
|  | OF1_p1 (User Interactions) To FN2_p1 (MSS HMI) | Volume Command (Physical Touch) |  |
|  | OF2_p1 (Smartphone/Media Device) To FN3_p5 (MSS Logical Manager) | Volume Command (Physical Touch) |  |
| ZoneLockReq | FN3_p5 (MSS Logical Manager) To OF2_p1 (Smartphone/Media Device) | Internal Signal for Zone Lock Request | Activate Deactivate |
| ZoneLock_Command | FN2_p2 (MSS HMI) To FN3_p1 (MSS Logical Manager) | Zone Lock Command (Physical Touch) |  |
|  | OF1_p1 (User Interactions) To FN2_p1 (MSS HMI) | Zone Lock Command (Physical Touch) |  |

Table 18: Feature Interactions

# Traceability Matrix

# Open Concerns

| ID | Concern Description | e-Tracker / Reference | Responsible | Status | Solution |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |

Table 20: Open Concerns (Not supported by MagicDraw report generation)

# Revision History

No Revision History found.

## Template Revisions

| Version | Rev. | Date | Description | Responsible |
| --- | --- | --- | --- | --- |
| 0 | 6 | 2015-05-26 | - Chapter “Feature Overview” and made a 2nd level heading. - Chapter “Feature Modeling” divided into 3 subchapter (“Scenarios”, “Use Cases”, “State Machines”) for different modeling methods | Jbaden1 |
| 0 | 7 | 2015-05-27 | - Table of Content updated - Template Revision History chapter added | Jbaden1 |
| 0 | 8 | 2015-07-02 | - Section “Unsettled Issues” added | Alevin7 |
| 0 | 9 | 2015-08-04 | - Section “Feature Variants” added - Section “Feature Boundary Diagram” renamed to “Feature Context Diagram” - Document Properties adapted to match needs of VBA macros | Jbaden1, Awegman1 |
| 1 | 0 | 2015-09-11 | - Section “Feature Variants” reworked - Feature Goals removed. Only “Safety Goals“ chapter remains. - Heading 2 formatting issues corrected. - Requirements / Use Cases Listing removed from traceability chapter. - Formatting of attribute table in Notation chapter corrected - Open Topics / Known Issues chapter moved to the end | Jbaden1 |
| 1 | 1 | 2015-11-16 | - Table-Styles removed (for smooth VSEM import) - Some clean-up of sections “Purpose” and “Audience” | Awegman1, jbaden1 |
| 1 | 2 | 2016-02-26 | - Minor corrections based on lessons learned from CC and PCL pilot (e.g. section market/regions) and discussion with Functional Safety Team (purpose of feature) - Footer corrected - Boundary diagram interface chapter renamed to influences. | Jbaden1 |
| 1 | 3 | 2016-02-26 | - Minor corrections after review with Whitney Keith from Functional Safety team | Jbaden1 |
| 1 | 4 | 2016-03-10 | - Some cleanup of meta-data in Word Properties | Jbaden1 |
| 1 | 5 | 2016-03-10 | - Footer formatting corrected (Issue 19) - Results from review with Functional Safety Team incorporated (Issue 20). | jbaden1 |
| 1 | 6 | 2016-04-18 | - Scenario Template added | Jbaden1 |
| 1 | 7 | 2016-04-18 | - Chapter “Operation Modes and States” moved before “Use Case” section. | Jbaden1 |
| 1 | 8 | 2016-04-18 | - Broken Wiki links repaired. | Jbaden1 |
| 2 | 0 | 2016-05-19 | - Adapted to Specification_Macros.dotm V2.0 - Requirements Templates chapter (ch. 1.7.1) no longer has an attribute table, but refers directly to the Wiki.. | Jbaden1 |
| 2 | 1 | 2016-06-10 | - Table for Context Diagram modified (lists external entities and Influence Description only) | Jbaden1 |
| 2 | 2 | 2016-07-08 | - Template version added to footer - Several hints added to the various sections - Findings from Functional Safety Team incorporated. - RE_SafetyRequirement style added | Jbaden1 |
| 2 | 3 | 2016-09-21 | - Update from Functional Safety Team incorporated (“Lessons Learned”, “System Behaviors for HARA”) | Jbaden1 |
| 2 | 4 | 2016-11-15 | - Update from Functional Safety Team incorporated (“Lessons Learned”, “System Behaviors for HARA”) - Explanatory notes made more formal | Jbaden1 |
| 3 |  |  | Skipped to synchronize with Specification_Macros.dotm |  |
| 4 |  |  |  |  |
| 5 | 0 | 2017-01-13 | - Meta data updated for specification macros, version 3.1 - SW Unit chapter removed for the time being - Green boxes added for user hints | Jbaden1 |
| 5 | 1 | 2017-01-18 | - Minor editorial changes | Jbaden1 |
| 6 | 0 | 2017-02-03 | - CR48: Chapter 6 renamed from “Safety” to “Functional Safety”. New sub-chapter “Safety” introduced in Non-Functional Requirements section | Jbaden1 |
| 6 | 0 | 2017-04-28 | - CR7: “RequirementsTraceability” chapter removed | Jbaden1 |
| 6 | 0 | 2017-11-15 | - CR32/53: New Cover Sheet + Disclaimer replaces FAP-150 like ones. - CR75: Some rewording -> Terminology to Glossary, Notation -> Document Conventions - CR49: Rename “Assumptions & Constraints” to “Assumptions” - CR74: Safety Assumptions added to chapter 6. - CR58: Add function allocation column to Logical Architecture chapter | Jbaden1 |
| 6 | 0 | 2018-01-31 | - CR63: Updated links to Functional Safety Sharepoint | Jbaden1 |
| 6 | 0 | 2018-07-24 | - CR69: Add FSR to FeatureDoc - CR64: Add new section "Design Requirements" to Function Spec and Feature Spec | Jbaden1 |
| 6 | 0 | 2018-08-06 | - CR53: some corrections for metada and formatting | Jbaden1 |
| 6 | 0 | 2018-09-28 | - Broken links to RE Wiki repaired | Jbaden1 |
| 6 | 0 | 2018-10-31 | - Cover sheet and footer more GIS like. Functional Safety team feedback incorporated: - New subsections “Functional Safety Requirements, (Decomposed) FSRs and Parameters / Values - Removal of “Logical Architecture” | Jbaden1 |
| 6 | 0 | 2018-12-12 | - FSR template removed, now as a macro in the Specification_Macros.dotm | Jbaden1 |
| 6 | 0a | 2019-05-23 | - Re-introduce “Logical Architecture” (for Functional Safety) | Jbaden1 |
| 6 | 0b | 2019-06-26 | - Chapter “Logical Elements” in “Logical Architecture” section added (FuSa CR 15136240) | Jbaden1 |
| 6 | 0c | 2019-03-22 | - Chapter “Decomposed FSRs” renamed to “ASIL Decomposition of Functional Safety Requirements” and moved beneath Chapter “Functional Safety Requirements”. Explanatory text improved. | Jbaden1 |
| 6 | 0c | 2019-04-05 | - Some wording in ASIL decomposition table modified. Description of fields in that table improved. | Jbaden1 |
| 6 | 0c | 2019-06-24 | - “Input Requirements” section modified (table approach as for the other RE templates). - “References” and “Glossary” chapter moved to the “Introduction” chapter. | Jbaden1 |
| 6 | 0c | 2019-07-02 | - "Important" box added on cover sheet which points to the macros | Jbaden1 |
| 6 | 0c | 2019-07-02 | - Subsection “Error Handling” removed form chapter “Feature Requirements”->”Functional Requirements” (teams are free to create their own substructure of that section). Note tells author not to forget about error handling. - Hint for chapter “Feature Variants” improved reworded upon request from Functional Safety Team. | Jbaden1 |
| 6 | 0c | 2019-05-11 | - Copyright notice shortened and moved to cover sheet and added to footer (to be compliant with Ford copyright guidelines) - Term “Disclaimer” no longer used for what is actually only a copyright notice | Jbaden1 |
| 6 | 0c | 2019-22-11 | - Chapter “Input Requirements/Documentst: minor modifications (examples added), Word comment removed” | Jbaden1 |
| 6 | 0c | 2019-12-05 | - Upstream Documents section added to “Input Requirements/Documents” table - Custom style table formatting removed - Hint on system behaviors modified as requested from FuSa team | Jbaden1 |
| 6 | 0c | 2019-12-09 | - Term “Upstream Documents” replaced by “Attribute Requirements” in “Input Requirements/Documents” table - ASIL Decomposition table replaced by a version, which get not corrupted during VSEM import. | Jbaden1 |
| 6 | 0c | 2019-12-10 | - In ch. “Functional Safety Requirements” Word reference Id by Word reference text replaced.. | Jbaden1 |
| 6 | 1a | 2020-02-12 | - New chapter “Cybersecurity” added. | Jbaden1 |
| 6 | 1a | 2020-03-03 | - All User Hints formatted using style “RE_UserHint” to enable automatic removal by a macro. | Jbaden1 |
| 6 | 1a | 2020-03-04 | - Chapter “Cloud Connectivity Data Analytics Requirements” added upon request by D. Crockett/J. Rawlings | Jbaden1 |
| 6 | 1a | 2020-03-09 | - Missing doc property “LatestSigMappingID” and “LatestAisInterfaceID” added - doc property “CopyrightDate” re-formatted to text and copyright date field in footer corrected - Version numbering re-initialized as 0.1 - Init value of version/revision date set to “yyyy/mm/dd” instead of “yyyy-mm-dd” to be in line with the “Edit Document Property” dialog - type of document property for latest IDs changed to number instead of text | Jbaden1 |
| 6 | 1b | 2020-03-17 | - Chapter “Functional Architecture” renamed to “Functional Decomposition” - New MBSE terminology introduced: “Feature Level”, “Function Level” and “Component Level” renamed to “Concept Level”, “Logical Level” and “Technology Level” | Jbaden1 |
| 6 | 1b | 2020-07-03 | - CR31: Chapter “Traceability Matrix” added. | Jbaden1 |
| 6 | 1b | 2020-23-09 | - CR28: Alignment to FFSG01.10 Feature Document Guideline for how to apply the Feature Doc template for Functional Safety. New section “Classification of Chapters” added. “Active Tilt Control” Example in section “Logical Architecture” updated based on input from HARA training. | Jbaden1 |
| 6 | 1b | 2020-25-11 | - Reference to process definition in Stages added to “How to Use” section on cover sheet. User hints removed from “Document Purpose” chapter. - RE-Wiki links mostly replaced by Stages links, links to Functional Safety Sharepoint updated | Jbaden1 |

# Appendix

## Definitions

| Definition | Description |
| --- | --- |
| 2 Zone Audio Share | A lockout that can be activated via the front HMI screen. The lockout pushes the zone configuration from 4 zones to 2 zones (The front zone and the rear zone) |
| Cabin Mode | The audio mode that plays audio through the cabin speakers. This involves a single source playing through the vehicle. |
| Duck | Audio is reduced to allow another audio source to play at a heightened level |
| Front Zone | A zone configuration that is exclusive to "2 Zone Audio Share": This zone is allocated to the driver and first row passenger |
| Kick/Push | Allocation is removed from a user and an alternative solution is provided |
| Rear Zone | A zone configuration that is exclusive to "2 Zone Audio Share": This zone is allocated to the second and third row passengers |
| Zone 1 | The zone allocated to the driver. |
| Zone 2 | The zone allocated to the first row passenger. |
| Zone 3 | The zone allocated to the second row left and third row left passengers. |
| Zone 4 | The Zone allocated to the second row right and third row right passengers. |
| Zone Mode | The audio mode that plays audio through zone speakers. Different media will play through different zone speakers, allowing for multiple sources to play throughout the vehicle. |

Table 21: Definitions used in this document

## Abbreviations

| Abbr. | Stands for |
| --- | --- |
| A2B | Automotive Audio Bus |
| BLE | Bluetooth Low Energy |
| BT | Bluetooth |
| CA | Captain's Announcement |
| DAB | Digital Audio Broadcasting |
| DND | Do Not Disturb |
| DSP | Digital Signal Processing |
| HMI | Human-Machine Interface |
| ICC | In Car Communication |
| LVDS | Low Voltage Differential Signaling |
| MSS | My Seat Space |
| NMK | Not MyKey |
| PAC | Phoenix Audio Controller Module: ACM(AHU) Module |
| PDC | Phoenix Domain Controller Module: APIM_CDC |
| RACM | Rear Audio Control Module |
| SDARS | Satellite Digital Audio Receiver System |
| TA | Traffic Announcement |
| VR | Voice Recognition |

Table 22: Abbreviations used in this document

Document ends here.