Ford ID in IVI with DuerOS

Product Requirements Document (PRD)

V 2.0.7

Document Status: Draft

Document Type: Product Requirements Document (PRD)

Classification: Confidential

Department: CVPP

Author: Johnny Wang

| Reviewing Manager: |  |
| --- | --- |
| Document Location: |  |
| Date Released: | <Date the reviewing manager approved this document MM/DD> |

Project: DuerOS IVI Phase 3&4

Contents

1	Background and Objectives	4

2	Target Audience of Specification	5

3	Scope of Specification	6

4	Terminology	7

4.1	Definitions	7

4.2	Acronyms and Abbreviations	7

5	General Assumptions	9

5.1	User assumptions	9

5.2	Vehicle assumptions	9

6	Supported vehicle programs	10

7	Related features and customer touch points	10

7.1	Related features	10

7.2	Related customer touch points	10

7.3	Context diagram	11

8	Login flow on IVI	12

8.1	User account login	12

8.2	Login via Face ID	15

9	Function list	16

10	General Requirements	17

10.1	Owner app download guide	17

10.2	Account sign up	17

10.3	Account binding	17

10.4	Ford account and Baidu account bilateral login and logout	17

10.5	Login QR code requirements	18

10.6	In-vehicle Face ID	19

10.7	Login verification via Bluetooth (TBD)	22

10.8	Multiple accounts switch	23

10.9	Account and personal profiles	23

10.10	Accounts have separate settings	24

10.11	Remove an account from sign-in (delete an account)	24

10.12	Account logout	25

10.13	Visitor mode(Not available based on discussion)	25

10.14	Account SDK high level requirements	25

10.15	In vehicle app account access management	26

10.16	Master/Factory Reset	26

10.17	Account profile	26

10.18	Third party app binding management	27

10.19	Account login settings	27

11	System and Architectural Design	28

11.1	Related vehicle components EE architecture	28

11.2	IVI account SDK architecture	28

11.3	IVI login QR code architecture	28

11.4	Related backend and cloud services architecture	28

12	Non-functional requirements	29

12.1	Performance requirements	29

12.2	Service-level agreements (SLA)	29

12.3	Data plan requirements	29

12.4	Data tagging requirements	29

12.5	Statistics, reports and operation requirements	29

12.6	Face ID security and privacy requirements	29

13	Use cases	31

13.1	Scan QR code to download owner’s app	31

13.2	Account binding	33

13.3	Login (based on one QR code solution)	37

13.4	Hardware swap	38

13.5	Face ID login	40

14	IVI app list that need work without login	41

15	Classification Key	42

16	Document Status Key	43

17	Changes	44

18	Contacts	45

19	Appendix. Reference Documents	46

# Background and Objectives

Ford is introducing SYNC+ IVI system in China, to embrace the rapid changes in telematics market. SYNC+ is an IVI system, deeply customized DuerOS system, which is developed by Baidu.

On SYNC+, there are many features developed by Ford, Baidu and third-party partners. Some of the features such as natural language recognition,  Baidu Maps, iQiYi (an online video application),  etc., are from Baidu ecosystem which require Baidu account support,   some other features, such as online service booking, online mall, car wallet , etc., require Ford account, and other 3 party apps such as QQ music, Ximalaya Radio requires their own accounts  login on IVI.

To satisfy the business requirements, both IVI and cloud solutions are needed for account login and binding.

To meet the program timeline, the account functions on IVI are prioritized and implemented literately through phase 1, 2, 3 and 4.

In phase 1, customers will need to download FordPass/Lincoln Way/Lincoln Way app, add the VIN, complete SIM card/s real name registration (a separate process), complete Ford and account binding and scan the QR code on IVI to login. In this phase, Ford sends the combination of FordPass/Lincoln Way/Lincoln Way mobile phone number and VIN during FordPass/Lincoln Way/Lincoln Way account and Baidu account binding process to Baidu as OAuth 2.0 is not supported and only Baidu account is logged in on IVI, technically.

In phase 2, there is an enhancement that enables pulling Ford account profile, e.g. vehicle nickname onto IVI. In addition, add CX483/U611 Lincoln program support.

In phase 3, there will be some login security verification updates added.

In phase 4, Ford account login and Face ID are added and contains lots of improvements. (Face ID is only available on vehicles which equip in-vehicle camera).

The objectives of the SYNC+ account include:

- Enable Ford, Baidu and 3rd parties’ ecosystems current and future functions to work normally, such as cloud voice recognition, OSB, Ford credit, Face ID log in, membership system, etc.

- Build the account ID system to support as a basic function for data statistics, data report and data sharing between Ford and Baidu

- Basis for service customization

- Basis for service recommendation

- Used for big data personas and labeling

- Expansibility to bind with more 3rd party apps

- Meet high standard security requirements

While the login process onto IVI

- Shall be user friendly, seamlessly and effortlessly as possible

- Trustfully and securely

# Target Audience of Specification

The target audience of this document includes:

- Engineers responsible for writing the specifications of Enhanced Memory and Face ID sub-systems and related components

- Software business analyst and product designer

- IVI and cloud software architect

- HMI and UX designer

- System and IVI validation team

- Hardware and software suppliers that develop and deliver the features

# Scope of Specification

The scope of this document is:

- General feature Requirements

- Functional Requirements including Functional Decomposition Diagram, General HMI Flow Chart, Use Cases

- High level HMI Requirements

Note: No physical level or program specific requirements, such as SDK or cloud software APIs, will be included in this document.  Important physical level and program specific specifications and information can be found in the Reference section.

# Terminology

## Definitions

| Enhanced Memory | The personalization feature defined in this SPSS which allows a user to create, manage, and recall up to 4 Driver Profiles all containing different positional and non-positional settings configurations. |
| --- | --- |
| Classic Memory | The original personalization feature which allows a user to store and recall positional settings for 3 drivers. |
| My Key | Feature allowing a user to restrict a set of vehicle settings for specific vehicle keys |
| Key fob | A vehicle key with lock, unlock, and alarm functionalities |
| Door Unlock Event | Event from Key fob/phone Unlock button that triggers a recall request |
| Remote Start Event | Event from Key fob/phone Remote Start button that triggers a recall request |
| Driving Restriction | HMI restrictions imposed to reduce the distraction risk connected to electronic devices built into a vehicle |
| Driver Profile | A collection of personalized vehicle settings that can be recalled by a user in an Enhanced Memory vehicle |
| Key fob Association | The process by which a key fob is linked to a Driver Profile |
| key fob Disassociation | The process by which a key fob is unlinked from a Driver Profile |
| Driver Memory Seat Button Association | The process by which a Driver Memory Seat Button is linked to a Driver Profile |
| Positional Settings | Settings personalized by Classic Memory such as driver seat, exterior mirrors, adjustable pedals, the tilt/telescope steering column and Heads-up Display features. The complete list is vehicle dependent. |
| Non-Positional Settings | Settings personalized by Enhanced Memory such as radio presets, language, time, distance units, temperature, navigation, etc. |
| Phone as a Key | Feature that makes phone as a key to remote start vehicle, lock, unlock doors, start Engine, recall Enhanced Memory Driver Profiles and many other key fob functions |
| Phone Association | The process by which a Phone is linked to a Driver Profile |
| Phone Disassociation | The process by which a Phone is unlinked from a Driver Profile |

## Acronyms and Abbreviations

| Term or Acronym | Definition |
| --- | --- |
| FordPass/Lincoln Way/Lincoln Way | Ford customer owner app installed on customer’s small phone |
| DuerOS | A set of functions and capabilities provided by Baidu, including Baidu navigation, voice recognition, Baidu online music, etc. |
| VIN | Vehicle Identification Number, which is the unique ID for a vehicle |
| POI | Point of interest |
| HMI | Human Machine Interface. The infotainment displays and controls provided by the vehicle. |
| IVI | The in-vehicle infotainment unit. |
| VR | Voice recognition |
| OSB | Online service booking |
| BCM | Body Control Module |
| BTMBF | Bass-Treble-Middle-Balance-Fade (Audio Adjustment Settings) |
| CAN | Controller Area Network |
| CGEA 1.3 | Common Global Electrical Architecture 1.3 |
| D&R | Design and Release Engineer |
| DDM | Driver Door Module |
| DSM | Door Seat Module |
| DSP AMP | Digital Signal Processing Amplifier |
| DTC | Diagnostic Trouble Code |
| ECU | Electronic Control Unit |
| FCIM | Front Control Interface Module |
| FS | Feature Specification Document |
| GSDB | Global Signal Database |
| HMI | Human Machine Interface |
| IPC | Instrument Panel Cluster |
| IPMA | Image Processing Module A |
| OCC Mode | Occupancy Mode |
| PEPS | Passive Entry Passive Start |
| PaaK | Phone as a Key Feature |

# General Assumptions

## User assumptions

- The customer has registered and set up a FordPass/Lincoln Way/Lincoln Way account and Ford mini programs in WeChat if needed, e.g. account profile image, customer name, vehicle nickname, etc...

- The customer has completed vehicle SIM card/s real name registration

- Customer has a valid phone number so that a FordPass/Lincoln Way/Lincoln Way account can be created

The above features/processes have been defined in separate documents, thus these are beyond the scope of this PRD.

## Vehicle assumptions

- The vehicle has valid internet access.

- Face ID is supported when referring to Face ID related functions

- Classic memory/enhanced memory is supported so that seat positions, mirror positions, steering column can be memorized and recalled when referring to related features

- Vehicle architecture support 3 driver profiles based on classic memory/enhanced memory

# Supported vehicle programs

- CD542

- CX727CBEV

- U554

- P702

- U625ICA

- U725

To be updated from program team.

# Related features and customer touch points

## Related features

- Rocket setup

- Account binding

- Account login on IVI

- Face ID

- Classic memory

- Enhanced memory

- PAAK

- Authorization (No account related change in Phase 4)

- Smart Greetings

## Related customer touch points

- Owner’s app, i.d. FordPass/Lincoln Way, Lincoln Way app

- IVI

- Ford mini programs (TBD), i.e. 电动派

## Context diagram

### System context diagram

Note: Pers 4 is reserved and shall not be used since classic memory only supports 3 driver profiles.

### Account and enhanced memory profile context diagram

Note: Pers 4 is reserved and shall not be used since classic memory only supports 3 driver profiles.

Visitor mode is not available in phase 4. In the case, personal profiles will switch to guest profile when account is nog logged in.

# Login flow on IVI

## User account login

### Single account login and multiple accounts login

The system shall enable multiple accounts login.

Once an account is logged in onto IVI, it shall creates a set of account profile data and synchronize necessary account profile (e.g. account profile image, customer name) and vehicle personal profile (e.g. ambient light setting for rocket setup feature) from cloud.

Log out of the account shall not delete the account profile data. The profile data shall be when

- A user deletes the account

- A factory reset is performed

Except the user deleted the account, the account shall be displayed on the account login list.

When an account is logged out on IVI, it shall return to the account login list. User can login the account again via loin verification:

  - Via Face ID

  - Via PIN (Not available in Phase 4)

  - Via scanning QR code. Please note by scanning the QR code, the IVI may login to another o

Each account that logged in onto IVI shall have an option:

- Verify  user identity during each ignition cycle

- The default value is ON

- If there is only accounts logged in onto IVI, user can turn off the option (be a setting for all accounts rather than a spefic one)

- If there are 2 accounts or more logged in onto IVI, the option will be ON and cannot be turn off.

To match enhanced memory profile numbers which is 3, only 3 accounts are allowed to bind to a personal profile which memorizes seat positions, mirror positions, steering wheel column and other IVI and vehicle soft settings. For a complete list of these features/functions that are memorized, please refer to enhanced memory/personal profiles spec. The initial binding is first comes first.

If the 4th user wants to log in onto IVI, user shall be prompted to replace an account – personal profile binding or remove at least 1 account (please note NOT log out) after the 4th account is logged in.

- The 4th user can skip the personal profile binding. If this is the case, the user will not have personal profile feature to memorize enhanced memorized features but only accounts related settings that are synchronized to the cloud, e.g. POI favorites in Maps, online music preferences, for example.

- The 4th user selects to replace an existing personal profile binding. In this case, the personal profile binding will hand over from the existing account to the new account, i.e. the 4th user will have the personal profile to memorize enhanced memorized features while the replaced one will unbind the personal profile.

The maximum number of accounts that can log in IVI is 10. If the 11th account attempts to login IVI, the user shall be notified the number of login accounts has reached to limit and be advised to delete one or several accounts to continue.

### Ford and Baidu accounts bilateral login, Face ID association and driver profile association

The system shall have a configuration that indicates whether Face ID is supported or not. After an account is logged in onto IVI, it shall check the configuration,

- If the vehicle supports Face ID and Face ID is not created for the account, it shall prompt and guide uses to create a Face ID

- If the vehicle does not supports Face ID, the process above shall be ignored.

After user logs in onto IVI,

- The QR code that supports login Ford ID and Baidu ID can be one QR code if technically feasible.

- Account FA automatically links to Profile 1, which memorizes personal settings. Account FB links to FB, etc.…

- When seat position changes, it saves/prompts to save to current driver profile, e.g. Pers 1（FA to pers 1, FB to pers2）

- During next ignition cycle,  accounts are standing by waiting for identity verification

- When account login verification is passed, e.g. FA, Pers 1 driver profile shall be recalled

## Login via Face ID

Please be noted at most 3 users + guest for personal profiles since only 3 personal profiles supported by vehicle architecture.

When Ford ID is logged via Face ID verification, it shall inform Baidu to login IVI as well.

# Function list

| Category | Function | Description | Owner app/WeChat mini program/Baidu mobile app/SDK | IVI | Ford cloud | Baidu cloud | Priority |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Before login | Guide to download owner app |  |  | √ |  |  | Must have |
|  | Ford account sign up |  | √ |  | √ |  | Must have |
|  | Baidu account sign up |  | √ |  |  | √ | Must have |
|  | Account binding |  | √ |  | √ | √ | Must have |
|  | Visitor mode |  |  | √ |  |  | Must have |
| During login | Login Ford/Lincoln account on IVI via QR code scanning |  | √ | √ | √ | √ | Must have |
|  | Login Ford/Lincoln account on IVI via input credentials |  | √ | √ | √ | √ | Nice to have |
|  | Login Baidu account on IVI |  | √ | √ | √ | √ | Must have |
|  | Login and auth |  | √ | √ | √ | √ | Must have |
|  | Multiple accounts login switch |  |  | √ | √ | √ | Must have |
|  | Login verification via Face ID |  |  | √ | √ | √ | Must have |
|  | Login verification via Bluetooth |  | √ | √ | √ | √ | Nice to have |
| After login | Login success welcome |  |  | √ |  |  | Must have |
|  | Account profile display and synchronization |  | √ | √ | √ | √ | Must have |
|  | Personalization settings recall |  |  | √ | √ | √ | Must have |
|  | Account log out |  |  | √ | √ | √ | Must have |
| Misc. | Account unbinding |  | √ |  | √ | √ | Must have |
|  | Account binding change |  | √ |  | √ | √ | Must have |
|  | Account management |  |  | √ |  |  | Must have |
|  | Delete an account |  |  | √ |  |  |  |
|  | Replace an account on IVI |  |  | √ |  |  |  |
|  | Face ID management |  |  | √ |  |  |  |
|  | Profile binding |  |  | √ |  |  |  |
|  | 3rd party service authorization management |  |  | √ |  | √ | Nice to Have |
|  | 3rd party app account binding |  |  | √ | √ |  | Nice to Have |
|  | Mini program support (TBD) |  |  | √ | √ |  | TBD |
|  | IoT support (TBD) |  |  | √ | √ |  | TBD |

# General Requirements

## Owner app download guide

Follow existing design.

## Account sign up

Follow existing design.

## Account binding

Follow existing design.

## Ford account and Baidu account bilateral login and logout

The main accounts including Ford account and Baidu account will need to bind together before they login onto IVI in pair under the one QR code login both Ford and Baidu account solution.

Once either account is logged in on IVI, the other shall be asked to login also. The IVI shall be considered as logged successfully once both accounts are logged in.

Login method

Scan one QR code to login both FordPass/Lincoln Way/Lincoln account and Baidu account.

Once either account log out on IVI, the other shall be logged out as well (同登同出).

Below are some reference HMI wireframes:

## Login QR code requirements

One FordPass/Lincoln Way account can only be bound to one Baidu account. System shall only enable the linked Baidu account to log in, where the linked account means the Baidu account has bound to the FordPass/Lincoln Way account and the VIN of the target vehicle has added to that FordPass/Lincoln Way account. If the VIN is not added to FordPass/Lincoln Way, user shall be asked to add the VIN first.

Terms can conditions shall be provided and agreed by the user before logging in.

The QR code should be refreshed regularly (10 minutes by default,  to conform to security requirements. It can be refreshed manually on IVI also.

The IVI shall only allow one active user to log in, that means if other user wants to log in the IVI, the current logged in account needs to log out first.

The user will be automatically logged in on the next ignition if the account is not logged out before ignition is off depending on user’s option.

System shall record the time and VIN when the account is logged in.

System shall collect the phone metadata (Mac address, IMEI, etc.) if Bluetooth/USB is paired and connected.

Once logged in on IVI, system shall allow user profile synchronized between FordPass/Lincoln Way and IVI.

## In-vehicle Face ID

In-vehicle Face ID (hereafter as Face ID in short) uses face recognition technology to identify the driver by scanning the face of the driver. Face ID provides an intuitive and secure authentication approach to login account or recall personal settings, such as seat and mirror positions.

To start using Face ID, customer needs to first enroll face. User may do this during the initial set up process, or later by going to Face ID settings. And later customer can login the IVI or recall personal settings simply by glancing at the camera.

Please note Face ID is only available to vehicles equipped with designated camera and software. For details, please consulate specific vehicle programs. This document is a general document which is not specified for each vehicle program.

For Face ID details, please refer to Face ID PRD.

The table below shows the possible Face ID and memory seat button combination existence on different programs (To be confirmed by each program)

| SYNC+ Phase 4 program | Face ID | Memory seat button |
| --- | --- | --- |
| CX727 | √ | √ |
|  | √ |  |
|  |  | √ |
| CD542 | √ | ❌ |
|  |  | ❌ |
| U554 | √ | √ |
|  | √ |  |
|  |  | √ |

Assumption

For cost and UX perspective, the Face ID and memory seat button coexistence shall not be recommended, the document will not describe the scenario unless program confirm the existence.

For vehicles equipped with memory seat button but no Face ID, the overall design can follow SYNC+ phase 2 design.

Therefore, the following only depicts the scenario when Face ID is equipped, and memory seat buttons are removed.

### Set-up Face ID

The system shall have a configuration to indicate whether the IVI supports the Face ID function or not. The set-up Face ID process and related Face ID management shall only be available when Face ID is available. The HMI shall distinguish related design elements and user flow based on the availability of Face ID.

Customer will need to first set-up Face ID. Considering the user experience, the HMI user flow is suggested having at least 2 entries to initiate the Face ID set-up process in terms of IVI account login.

- When the account is logging into the IVI without Face ID created for this account, the Face ID set- up process shall be prompted so that the user can learn the feature and continue the set-up process. The user may skip the set-up process.

- In case the user skips the set-up process, the HMI shall provide the user another entry to set-up the Face ID, e.g. system settings, account settings depend on HMI and user flow design.

Please note the Face ID set-up process alone will be clarified in the Face ID/Image Recognition PRD, hence it will not be elaborated in this document.

### Face ID and account binding/unbinding

To enable IVI account login via face recognition, one account shall be only associated with one Face ID and one Face ID can only be associated with one account, hence the binding relationship of Face ID and account is:

Face ID data is stored offline, hence so the binding relationship is only valid on the vehicle where it is created.

The Face ID and account binding can be removed when

- Face ID is no longer exist, i.e. Face ID is deleted from IVI

- Account is no longer exist, i.e.  the account is deleted from backend server database

- The account is removed from IVI (not logged out).

- A factory reset action is performed

### Face recognition and account login verification

Before face recognition

When IVI system starts, system shall check whether there is any account logged in. If yes, the system shall verify the user identity before entering the home screen (The user can change the account login verification option is system settings, i.e. whether it requires identify verification during every ignition cycle).

- No account existing on IVI. The system shall ask user to login an account.

- There is/are existing account/s logged in (currently not active after system starts), but none of the accounts has associated with a Face ID. In the case, the system shall inform the user to verify the identity by alternative login methods, e.g. QR code scanning, password, which is the same as the scenario when the Face ID is not available on the vehicle.

- There is/are existing account/s logged in (currently not active after system starts), and some of these accounts are associated with Face IDs, and some don’t. For those accounts that have associated with Face ID, the system shall enable face recognition as user identity verification method. For those don’t, the system shall enable alternative login method.

- All the existing account/s logged in (currently not active after system starts) are associated with Face IDs. The system shall enable face recognition as user identity verification method for all these accounts.

Alternatively, the user can also login a new account rather than the existing ones.

Initialize face recognition

The face reception shall start as user requested from HMI soft button, voice commands or defined by system settings (e.g. initiate at every ignition cycle)

During face recognition

During face recognition, user shall be guided complete gestures required by the face recognition.

The system shall allow the user a defined number of retries, each limited to a defined amount of time, to prevent system lock-up.

In case the system cannot recognize the face, it shall responses accordingly:

- No valid face detected

- Valid face detected and cannot match local Face ID data

- Valid face detected and can match local Face ID data

- Time out

After face recognition

No matter whether the face recognition is successful or not, it shall output the recognition results to account module. The HMI shall provide the user an indication of the face recognition results.

- Face ID recognition is successful, the account login verification will pass and accept the login request. The HMI shall provide a loading animation/indicator matching the login process.

- Face ID recognition is not successful, the account login verification will not pass and reject the login request. The system shall allow the user a defined number of retries, each limited to a defined amount of time. If the number of retries reaches the number of maxima retries, the system shall prohibit the use of Face recognition for a limited time period. Once the user logins in via an alternative login method, or reaches the time limit, the system shall enable Face ID login again.

### Face ID management

User shall be able to create a Face ID after account login in case no Face ID is associated with this account on the vehicles that Face ID is supported.

The Face ID HMI shall provide the user an indication of the availability of Face ID on the vehicle, whether the Face ID is enabled/disabled and the interface to delete the Face ID.

The system shall not allow the user to associate an already associated Face ID to the account.

In short, the following functions are required:

- Create a Face ID

- Enable/Disable Face ID account login verification

- Delete Face ID

## Login verification via Bluetooth (TBD)

Customer can enable phone Bluetooth connection to vehicle as the verification method. This could be set by customer when the account has logged in.

## Multiple accounts switch

There are two options for account login via face recognition.

Assumption

- There is at least one valid user has logged in the IVI.

- The number of accounts that can be logged in onto IVI is limited to the number of personal profiles that vehicle supports (suppose 3 customizable personal profiles + vehicle/guest profile).

### Option 1- Detect face, identify which account the face is associated and then login (Preferred and selected)

When system starts, Face ID camera starts to detect whether there is a valid face entering and match whether the face match with the Face ID data stored in IVI.

- If the face match one of the Face ID stored in IVI, then return the account ID that associated with this Face ID and account starts to login.

- If the face does not match any of the Face ID stored in IVI，then

  - If the face recognition has not reached the maximum attempts, then ask the user to try again.

  - If the face recognition has reached the maximum attempts, then tell the user to login with other methods (i.e. QR code scanning).

### Option 2 – Select account first, validate face associated with the account, and then login

When system starts, IVI shall list all the user that has currently logged into the IVI, but an account validation is required to access the system. The last used account is set as default account which is selected as current account automatically. And the Face ID is validation method by default. User shall be able to switch to another validation method such as QR code scanning or manually entering the account password.

## Account and personal profiles

Once account is logged in, it shall bind one of the personal profiles available. In personal profiles (enhanced memory) server there are 3 customizable profiles available + vehicle profile (guest). One account logged in shall use up one of the profiles. Once the profile is occupied by one account, it cannot be associated with another.

The profile shall be released once the account is removed (not logged) from IVI or a factory reset is performed.

Once the account is logged in on IVI, it shall recall the personal profile that binds to it.

In the case the profile is recalled by hard button, the corresponding account shall not be activated to login.  E.g. hard button 1 is linked to driver profile Pers 1 and account A is also linked to Pers 1, the account A shall not be invoked to login as pressing a hard does not meet account verification requirements.

## Accounts have separate settings

All the account profile data and settings are only for the access of the account of their own, except specific clarification.

One possible example of the exempt data is vehicle license plate number and engine number in the personal center. These data can be input and modified by any user, in login or non-login status.

## Remove an account from sign-in (delete an account)

User shall be able to remove an account from the account login list. This will remove the binding between the account and personal profile index. After the unbinding, the profile is released the profile index can be associated to a new account. Please note performing a factory reset will remove all the accounts.

Note: 访客 not available. To be updated.

The maximum number of accounts on IVI is 3. If there is a fourth account login attmpts, customers will asked to replace an account.

## Account logout

The system shall not dissociate the Face ID and account when the account is logged out, so that the Face ID can be used to login the IVI next time.

## Visitor mode(Not available based on discussion)

Visitor mode is a mode under which user can use most of the functions of the vehicle but no actual account login. In this case, no personal preference will be saved.

The visitor mode shall match the personal profile/enhanced memory feature as guest.

In the mode IVI can be used temporarily without real customer account login during one ignition cycle. In this mode, users are advised to connect to a Wi-Fi hotspot.

## Account SDK high level requirements

- Account binding with Baidu

- Account unbinding with Baidu

- Checking account binding status

- Checking Ford account login status

- Checking Baidu account login status

- Receive and respond login requests from IVI apps

- Log out synchronization

- Bind to Face ID

- Unbind to Face ID

- Bind to personal profiles

- Unbind to personal profiles

- Recall personal profiles

- Store and update personal profile binding status

- Get current profile index

- IVI app access permission management

- Factory reset

The account SDK shall be able to distinguish the source that requests the profile recall, i.e. from Face ID or tapping the soft button.

It shall also adapt the vehicle hardware availability status such as:

- Face ID availability

- Memory seat button availability

- SYNC+ version

Regarding OTA, it shall not log out the current accounts as possible to prevent from asking customer to relog in IVI.

## In vehicle app account access management

The Ford account shall provide a management mechanism to control the in-vehicle app access to Ford cloud and ecosystems.

## Master/Factory Reset

When a Master Reset operation is executed, system shall:

- Dissociate Face ID and account

- Delete the Face ID

- Log out the account

- Unbind profile index with account and release pers signal

- All account logs out and settings to default

## Account profile

The accounts, vehicle information and other necessary items shall be organized in the account profile (by UX).

### Ford ID formation

- Phone number

- Account profile picture

- Customer name

- Home address (TBD, pending synchronization solution)

- Company address (TBD, pending synchronization solution)

- Account creation time

- Last login time on IVI

|  | FordPass/Lincoln Way |  |  | IVI |  |
| --- | --- | --- | --- | --- | --- |
| Customer Data for Ford Account | Priority | Edit | View | Edit | View |
| FordPass/Lincoln Way user name (mobile number) | High | x | x |  | x |
| Title | High | x | x | x | x |
| First Name | High | x | x | x | x |
| Last Name | High | x | x | x | x |
| Associated Baidu account username/mobile number | High | x | x |  | x |
| Ford account profile picture | High | x | x | x | x |
| Vehicle model/make/year/ | High |  | x |  | x |
| Vehicle nickname | High | x | x | x | x |
| VIN | High |  | x |  | x |
| Preferred Dealer | Mid | x | x |  | x |
| Home Address | TBD |  |  | x | x |
| Company Address | TBD |  |  | x | x |

The IVI account profile picture shall come from Ford account. The default image shall be applied when

- Not logged in

- The customer has not set a profile picture

Also, in visitor mode, a special profile image is needed.

### Baidu ID information

- Baidu ID phone number

- Baidu ID nickname

### Vehicle information

- VIN

- Make/Model/Year

- Color

- License plate number

- License plate color

- Engine number

- Next annual inspection date

- Insurance expiration date

These items shall be stored on IVI by VIN (account independent) as primary source if any data synchronization occurs.

### Face ID management

- Face ID setup

- Disable Face ID

### Others (per UX)

- Order center

- Third party services (authorization) management

## Third party app binding management

## Account login settings

- Login verification method

  - Face ID

  - Bluetooth(TBD)

  - PIN (TBD)

  - Verification during each ignition (by default)

# System and Architectural Design

## Related vehicle components EE architecture

TBD.

## IVI account SDK architecture

TBD.

## IVI login QR code architecture

TBD.

## Related backend and cloud services architecture

TBD.

# Non-functional requirements

## Performance requirements

The log in confirmation notification shall be available on FordPass/Lincoln Way/Lincoln Way in less than 2 seconds.

Once confirmed by the user, the FordPass/Lincoln Way account shall be logged in on IVI in less than 2 seconds.

## Service-level agreements (SLA)

Service availability:  99.99 %

## Data plan requirements

The data consumed by this feature shall be covered by the basic data plan (paid by Ford).

Data consumption estimation (TBD)

## Data tagging requirements

TBD

埋点事件

通过开机自检弹出扫码登录页面

通过其他app调用弹出扫码登录页面

扫码登录页面关闭点击pv uv

账号登录成功（只有在首次登录才会上报登录成功状态）

退出当前账号

在系统登录二维码界面点击“没有福特派”/’没有林肯之道’

二维码登录时长

二维码失效次数

个人中心各页面进入、按钮点击pv uv

## Statistics, reports and operation requirements

- Inquiry and export Ford IVI account login status by VIN, vehicle make/model, nameplate, date, account,

- Inquiry and export Ford IVI account binding status with all parties by VIN, vehicle make/model, nameplate, partner name, date, account

- Account binding status management

- Account login statistics

- Login performance reports and monitor

## Face ID security and privacy requirements

For Face ID functional and unfunctional requirements, please refer to Face ID PRD.

For Face ID related security requirements, please refer to separate requirements document from Ford Cybersecurity team.

For Face ID related privacy requirements and how Face ID data is protected, please refer to Ford terms and conditions/privacy policy requirements.

# Use cases

## Scan QR code to download owner’s app

| Use Case ID | ### Ford brand vehicle, iOS |
| --- | --- |
| Use Case | The user scans the QR code of FordPass/Lincoln Way download link on IVI with an app installed on an iPhone. |
| Pre-Conditions | - Ignition is on and IVI is launched. - No accounts logged in on IVI. |
| Trigger | - The user selects to view FordPass/Lincoln Way QR code download screen on IVI. - The user scans the QR code with an app installed on the phone. |
| Expected Behavior | The user is directed to FordPass/Lincoln Way download page in Apple App store on the iPhone. |
| Post Conditions | The user downloads the FordPass/Lincoln Way app in App Store. |
| Exceptions | If the vehicle is moving, the QR code screen should be disabled to display. |

| Use Case ID | ### Ford brand vehicle, Android |
| --- | --- |
| Use Case | The user scans the QR code of FordPass/Lincoln Way download link on IVI with an app installed on an Android phone. |
| Pre-Conditions | - Ignition is on and IVI is launched. - No accounts logged in on IVI. |
| Trigger | - The user selects to view FordPass/Lincoln Way QR code download screen on IVI. - The user scans the QR code with an app installed on the phone. |
| Expected Behavior | The user is directed to FordPass/Lincoln Way android version download page on the Android phone. |
| Post Conditions | The user downloads the FordPass/Lincoln Way app. |
| Exceptions | If the vehicle is moving, the QR code screen should be disabled from display. |

| Use Case ID | ### Lincoln brand vehicle, iOS |
| --- | --- |
| Use Case | The user scans the QR code of Lincoln Way download link on IVI with an app installed on an iPhone. |
| Pre-Conditions | - Ignition is on and IVI is launched. - No accounts logged in on IVI. |
| Trigger | - The user selects to view Lincoln Way QR code download screen on IVI. - The user scans the QR code with an app installed on the phone. |
| Expected Behavior | The user is directed to Lincoln Way download page in Apple App store on the iPhone. |
| Post Conditions | The user downloads the Lincoln Way app in App Store. |
| Exceptions | If the vehicle is moving, the QR code screen should be disabled to display. |

| Use Case ID | ### Lincoln brand vehicle, Android |
| --- | --- |
| Use Case | The user scans the QR code of Lincoln Way download link on IVI with an app installed on an Android phone. |
| Pre-Conditions | - Ignition is on and IVI is launched. - No accounts logged in on IVI. |
| Trigger | - The user selects to view Lincoln Way QR code download screen on IVI. - The user scans the QR code with an app installed on the phone. |
| Expected Behavior | The user is directed to Lincoln Way android version download page on the Android phone. |
| Post Conditions | The user downloads the Lincoln Way app. |
| Exceptions | If the vehicle is moving, the QR code screen should be disabled from display. |

| Use Case ID |  |
| --- | --- |
| Use Case | The user adds the VIN in FordPass/ Lincoln Way via scanning the QR code/bar code on IVI |
| Pre-Conditions | - Ignition is on and IVI is launched. - The user has logged in FordPass/Lincoln Way app. |
| Trigger | - The user selects to view the VIN code screen. - The user scans the VIN code with FordPass/Lincoln Way app. |
| Expected Behavior | - The VIN is recognized, displayed and filled the VIN field automatically in FordPass/Lincoln Way. |
| Post Conditions | - The user confirm the VIN information and confirm adding the VIN. - The VIN is associated with the FordPass/Lincoln Way account. |
| Exceptions | - If the vehicle is moving, the QR code screen should be disabled from display. - The user shall be notified to check and scan again or input it manually if the VIN fails to recognize or is recognized as an invalid VIN |

## Account binding

| Use Case ID |  |
| --- | --- |
| Use Case | The user binds FordPass/Lincoln Way account to an existing Baidu account |
| Pre-Conditions | - The user has logged in FordPass/Lincoln Way app. - The SYNC+ VIN has added to the FordPass/Lincoln Way account. - No existing Baidu account has been bound to the FordPass/Lincoln Way/Lincoln Way account. |
| Trigger | - The user adds a VIN of a SYNC+ vehicle in FordPass/Lincoln Way. - The user goes to the SYNC+ vehicle screen in FordPass/Lincoln Way app. |
| Expected Behavior | - The user is prompted to bind a Baidu account to use the full features of the vehicle. - The user confirms terms and conditions - The user logs in Baidu account in FordPass/Lincoln Way. - The user confirms the binding. |
| Post Conditions | - Baidu server will record of FordPass/Lincoln Way account and VIN combination. - FordPass/Lincoln Way will have Baidu account token. |
| Exceptions | If the Baidu account has been bound with other FordPass/Lincoln Way account, system shall reject the binding request and prompt the user to bind another Baidu account or unbind the previous account first and try again. |

| Use Case ID |  |
| --- | --- |
| Use Case | The user binds FordPass/Lincoln Way account to a new Baidu account |
| Pre-Conditions | - The user has logged in FordPass/Lincoln Way app. - The SYNC+ VIN has added to the FordPass/Lincoln Way account. - No existing Baidu account has been bound to the FordPass/Lincoln Way account. |
| Trigger | - The user adds a VIN of a SYNC+ vehicle in FordPass/Lincoln Way Way. - The user goes to the SYNC+ vehicle screen in FordPass/Lincoln Way app. |
| Expected Behavior | - The user is prompted to bind a Baidu account to use the full features of the vehicle. - The user confirms terms and conditions - The user creates a Baidu account in FordPass/Lincoln Way app. - The user logs in Baidu account in FordPass/Lincoln Way app. - The user confirms the binding. |
| Post Conditions | - Baidu server will the record of binding status of FordPass/Lincoln Way account and VIN combination. - FordPass/Lincoln Way will receive Baidu account token. |
| Exceptions | If the Baidu account has been bound with other FordPass/Lincoln Way account, system shall reject the binding request and prompt the user to bind another Baidu account or unbind the previous account first and try again. |

| Use Case ID |  |
| --- | --- |
| Use Case | The user removes the VIN of a SYNC+ vehicle in FordPass/Lincoln Way after accounts binding |
| Pre-Conditions | - The user has logged in FordPass/Lincoln Way app. - The SYNC+ VIN has added to the FordPass/Lincoln Way account. - The FordPass/Lincoln Way account has bound to a Baidu account. |
| Trigger | - The user removes the VIN of a SYNC+ vehicle in FordPass/Lincoln Way/Lincoln Way. |
| Expected Behavior | - The user is prompted that all the SYNC+ related features will be disabled. - The user confirms the VIN removal. - The user will be logged out on IVI during next ignition if it is the current logged in account on IVI. |
| Post Conditions | - Both Ford and Baidu server shall update the VIN combination relationship. - The FordPass/Lincoln Way and Baidu account binding status remains as it is (there could be no vehicle association existing if all the VINs are removed). |
| Exceptions | None. |

| Use Case ID |  |
| --- | --- |
| Use Case | The user adds a VIN of a SYNC+ vehicle again after the VIN is removed from FordPass/Lincoln Way. |
| Pre-Conditions | - The user has logged in FordPass/Lincoln Way app. - The FordPass/Lincoln Way account has bound to a Baidu account (no SYNC+ vehicle is associated with the account at the moment) |
| Trigger | - The user adds a VIN of SYNC+ vehicle. |
| Expected Behavior | - The user is prompted to bind the Baidu account that currently the FordPass/Lincoln Way account is bound to. - The user accepts the terms and conditions and confirms the binding. |
| Post Conditions | - The FordPass/Lincoln Way records new SYNC+ vehicle binding and updates the binding status of both vehicles - Baidu backend server records new SYNC+ vehicle binding and updates the binding status of both vehicles. |
| Exceptions | None. |

| Use Case ID |  |
| --- | --- |
| Use Case | User changes the mobile number of FordPass/Lincoln Way account |
| Pre-Conditions | - The user has logged in FordPass/Lincoln Way app. - The FordPass/Lincoln Way account has bound to a Baidu account. |
| Trigger | - The user changes the phone number of FordPass/Lincoln Way account. |
| Expected Behavior | - The FordPass/Lincoln Way mobile number is set as the new number. - IVI update - The mobile number when account screen refresh or next ignition. |
| Post Conditions | - FordPass/Lincoln Way sends the new phone number of the FordPass/Lincoln Way account to Baidu. - Baidu update the phone number associated with the FordPass/Lincoln Way account. |
| Exceptions | None. |

| Use Case ID |  |
| --- | --- |
| Use Case | User changes to bind another Baidu account |
| Pre-Conditions | - The user has logged in FordPass/Lincoln Way app. - The FordPass/Lincoln Way account has bound to a Baidu account. |
| Trigger | - User selects to bind another Baidu account |
| Expected Behavior | - User will be prompted that current data based on the account will be removed and all data will be linked to that of the new Baidu account. - User is logout out on next ignition. |
| Post Conditions | Both Ford and Baidu shall synchronize the binding status change. |
| Exceptions | None |

| Use Case ID |  |
| --- | --- |
| Use Case | User logs in FordPass/Lincoln Way on another phone |
| Pre-Conditions | - The user has logged in FordPass/Lincoln Way app. - The FordPass/Lincoln Way account has bound to a Baidu account. |
| Trigger | - User logs in FordPass/Lincoln Way account on another phone - Or user uninstall FordPass/Lincoln Way app and reinstall the app on the same phone. |
| Expected Behavior | - User logs in FordPass/Lincoln Way. - FordPass/Lincoln Way gets the token of the bound Baidu account. |
| Post Conditions | FordPass/Lincoln Way and Baidu shall record the log in of the new device. |
| Exceptions | None |

| Use Case ID |  |
| --- | --- |
| Use Case | User views bound Baidu account information in FordPass/Lincoln Way. |
| Pre-Conditions | - The user has registered a FordPass/Lincoln Way account, added a SYNC+ vehicle. - The FordPass/Lincoln Way account has bound to a Baidu account. |
| Trigger | - User views bound Baidu account information. |
| Expected Behavior | - User views the bound Baidu account information successfully. |
| Post Conditions | N/A. |
| Exceptions | N/A. |

| Use Case ID |  |
| --- | --- |
| Use Case | The user who has bound a SYNC+ vehicle with his/her Baidu account adds a new SYNC+ vehicle. |
| Pre-Conditions | - The user has registered a FordPass/Lincoln Way account. - The FordPass/Lincoln Way account has bound to a Baidu account. - The FordPass/Lincoln Way account has added one or multiple SYNC+ vehicles. |
| Trigger | - User adds the VIN of another SYNC+ vehicle. |
| Expected Behavior | - The user is prompted to bind the Baidu account that currently the FordPass/Lincoln Way account is bound to. - The user accepts the terms and conditions and confirms the binding. |
| Post Conditions | - The FordPass/Lincoln Way records new SYNC+ vehicle binding and updates the binding status of both vehicles - Baidu backend server records new SYNC+ vehicle binding and updates the binding status of both vehicles. |
| Exceptions | N/A. |

## Login (based on one QR code solution)

| Use Case ID |  |
| --- | --- |
| Use Case | User logs in IVI via scanning QR code displayed on IVI before vehicle auth is completed. |
| Pre-Conditions | - The FordPass/Lincoln Way remote control features are not authorized yet. - The user is logged in FordPass/Lincoln Way app. - Vehicle is not in motion during log in process - Ignition is on and IVI is running and switched to sign up screen - No other account logged in on IVI. |
| Trigger | - User selects the log in screen on IVI. - User scans the QR code via FordPass/Lincoln Way. |
| Expected Behavior | - User will be prompted on FordPass/Lincoln Way to log in both Ford(Lincoln) account and Baidu account onto IVI on the vehicle. - User confirms the log in and vehicle auth on FordPass/Lincoln Way app. - Log in on IVI succeeds and IVI goes to home screen. |
| Post Conditions | - The FordPass/Lincoln Way app should receive a notification that the account is logged in on IVI. - The driver profile associated is recalled on enhanced memory supported vehicles. - The vehicle is authorized successfully regarding the FordPass/Lincoln Way remote control features. |
| Exceptions | - The user should be notified on FordPass/Lincoln Way if the QR code is not valid. - The user should be notified on FordPass/Lincoln Way if the QR code has expired and it need to be refreshed on IVI. - The user should be notified if the vehicle is not added to the FordPass/Lincoln Way yet. - The user should be notified if the vehicle has not bound to the FordPass/Lincoln Way account yet. - The user can check the auth status and resend the auth request if it fails. |

| Use Case ID |  |
| --- | --- |
| Use Case | User logs in IVI via scanning QR code displayed on IVI after vehicle auth is completed. |
| Pre-Conditions | - The FordPass/Lincoln Way remote control features have been authorized. - The user is logged in FordPass/Lincoln Way app. - Vehicle is not in motion during log in process - Ignition is on and IVI is running and switched to sign up screen - No other account logged in on IVI. |
| Trigger | - User selects the log in screen on IVI. - User scans the QR code via FordPass/Lincoln Way. |
| Expected Behavior | - User will be prompted on FordPass/Lincoln Way to log in both Ford (Lincoln) account and Baidu account onto IVI on the vehicle. - User confirms the log in on FordPass/Lincoln Way app. - Log in on IVI succeeds and IVI goes to home screen. |
| Post Conditions | The FordPass/Lincoln Way app should receive a notification that the account is logged in on IVI. |
| Exceptions | - The user should be notified on FordPass/Lincoln Way if the QR code is not valid. - The user should be notified on FordPass/Lincoln Way if the QR code has expired and it need to be refreshed on IVI. - The user should be notified if the vehicle is not added to the FordPass/Lincoln Way yet. - The user should be notified if the vehicle has not bound to the FordPass/Lincoln Way account yet. |

## Hardware swap

| Use Case ID |  |
| --- | --- |
| Use Case | User logs in the DuerOS vehicle after SYNC +head unit/TCU swap |
| Pre-Conditions | - The user is logged in on IVI before the SYNC+ head unit/TCU swap - Internet access is available - DuerOS has been activated after the hardware swap. - Vehicle is not in motion during log in process |
| Trigger | - Hardware swap |
| Expected Behavior | - User scans the QR code on IVI log in screen via FordPass/Lincoln Way. - User confirms the log in on FordPass/Lincoln Way app. - Log in on IVI succeeds and IVI goes to home screen. |
| Post Conditions | The customer settings are retrieved automatically after the log in. |
| Exceptions | N/A |

| Use Case ID |  |
| --- | --- |
| Use Case | User who has added a DuerOS vehicle to her/her FordPass account scans the QR code of another DuerOS vehicle to log in. |
| Pre-Conditions | - The user is logged in FordPass app. - Vehicle is not in motion during log in process - Ignition is on and IVI is running and switched to sign up screen - No other account logged in on IVI. |
| Trigger | - User selects the log in screen on IVI. - User scans the QR code via FordPass. |
| Expected Behavior | - User will be prompted on FordPass to log in IVI on the vehicle. - User confirms the log in and vehicle auth (if auth not done yet) on FordPass app. - Log in on IVI succeeds and IVI goes to home screen. |
| Post Conditions | - The FordPass app should receive a notification that the account is logged in on IVI. - The vehicle is authorized successfully regarding the FordPass remote control features (if not authorized before log in). |
| Exceptions | - The user should be notified on FordPass if the QR code is not valid. - The user should be notified on FordPass if the QR code has expired and it need to be refreshed on IVI. - The user should be notified if the VIN is not added to the FordPass account yet. - The user should be notified if the vehicle is added to the FordPass but not bound to the Baidu account yet. - The user can check the auth status and resend the auth request if it fails. |

| Use Case ID |  |
| --- | --- |
| Use Case | User logs out IVI |
| Pre-Conditions | - Ignition is on and IVI is running - Vehicle is not moving. - The user has logged in on IVI. |
| Trigger | - User logs out on IVI. |
| Expected Behavior | - User will be prompted that all the related features will be disabled for the account. - User confirms the log out - Both Ford and Baidu account are logged out. - IVI goes to home screen and all the settings under the user’s profile will be disabled to view. |
| Post Conditions | The FordPass app should receive a notification that the account is logged out on IVI. The account binding status shall not change and the user can log on IVI by scanning the QR code on IVI again. |
| Exceptions | None. |

| Use Case ID | ### Switch account |
| --- | --- |
| Use Case | The user switches an account on IVI |
| Pre-Conditions | At least 2 accounts are logged in onto IVI |
| Trigger | The user switches accounts on IVI |
| Expected Behavior | - The user selects the accounts to login - The user will be prompted to verify the login via Face ID or PIN |
| Post Conditions | 1. When verification is passed, switched to the selected account 2. When verification fails, user is prompted verification fails and if try again (at most 3 times). If tries up to limit (3 times), the user will no longer be able to try during the ignition. |
| Exceptions | None. |

| Use Case ID | ### Delete an account |
| --- | --- |
| Use Case | The user deletes an account on IVI |
| Pre-Conditions | The account is in the list displayed on IVI |
| Trigger | The user deletes an account on IVI |
| Expected Behavior | - The user selects the accounts to delete - The user will be prompted to confirm and informed that all the personal settings will be removed. |
| Post Conditions | 1. When confirmed, delete the account, unlink all the Face ID and driver profiles if any. 2. Not confirmed or cancelled the action, keep the account. |
| Exceptions | None. |

| Use Case ID | ### Master reset |
| --- | --- |
| Use Case | The user performs master reset on IVI |
| Pre-Conditions | The user is logged in on IVI |
| Trigger | The user performs master reset on IVI. |
| Expected Behavior | - The user confirms master reset action on the IVI popup - The user will be prompted all accounts will be removed and personal settings will be lost. - User will be notified in FordPass that the vehicle auth is removed and the account is logged out. |
| Post Conditions | - Vehicle auth shall be all removed from the associated accounts.. - The VIN association will not change on all the related FordPass accounts. - All IVI accounts will be removed. - All driver profiles binding will be removed. - All Face ID and binding will be removed. - The FordPass and Baidu accounts binding status will not change after master reset. |
| Exceptions | None. |

## Face ID login

TBD

# IVI app list that need work without login

To be updated based on newly added apps from phase 2, mostly vehicle control features in principle.

# Classification Key

| Classification | Notes |
| --- | --- |
| Proprietary | Information created or obtained in the normal course of business and not classified as Secret or Confidential |
| Confidential | Information that provides the Company with a competitive advantage, that supports its technical or financial position, and which, if disclosed without authorization, could cause damage to the Company. |
| Secret | Information of a strategic or highly sensitive nature that, if disclosed without authorization, would cause substantial, severe, or irreparable damage to the Company or its relationships. |

# Document Status Key

| Status | Notes |
| --- | --- |
| DRAFT | Document currently being worked on. Shall not be used as a solid reference to information included in this document. |
| AFR | Available For Review. Document information is not eligible for changes. Approving manager will revise this document and if all the information is found to be completely valid, then the document will change to REL status. If the document is found to have errors, the document will change to DRAFT status. |
| REL | Released. Document is completely valid at time of review, and is now available to be used as a solid reference of information. |

# Changes

| Author | Date (YYYY/MM/DD) | Status | Notes |
| --- | --- | --- | --- |
| Johnny W. | 2018/10/19 | DRAFT | Version 1.0 – Initial draft at compiling all use cases, available information and existing requirements |
| Johnny W. | 2018/11/03 | DRAFT | Version 1.1 – Revise account log in process and update use cases, remove personalization (will have a separate PRD) |
| Johnny W. | 2018/11/11 | DRAFT | Version 1.2 – Revise log in wireframe and update use cases after internal discussion |
| Johnny W. Fei Ya | 2018/11/13 | DRAFT | Version 1.3 – Architecture solution update |
| Johnny W | 2019/12/11 | DRAFT | Version 2.0 – Add Face ID and multiple account login support for face ID equipped vehicles |
| Johnny W | 2019/12/13 | DRAFT | Version 2.0.1 – Add context diagram |
| Johnny W | 2019/12/16 | DRAFT | Version 2.0.2 – Add account and profile index context diagram, general requirements update |
| Johnny W | 2019/12/18 | DRAFT | Version 2.0.3 – Error fix |
| Johnny W | 2019/12/24 | DRAFT | Version 2.0.4 – Error fix |
| Johnny W | 2020/01/11 | DRAFT | Version 2.0.5 – Typo correction, design update and add use cases |
| Johnny W | 2020/02/17 | DRAFT | Version 2.0.6 – Add replace account and other minor changes according to review meeting discussion |
| Johnny W | 2020/02/25 | DRAFT | Version 2.0.7 – Change max login account number to 10 |

# Contacts

For assistance or correction, please contact any of the following:

Johnny Wang (jwang308@ford.com), Account Feature Owner

Elaine Jia (tjia2@ford.com) ，Face ID Feature Owner

Andy Xu (xxu71@ford.com ), Enhanced Memory Feature Owner

Calvin Zhou (QZHOU31@ford.com ), Smart Greeting Feature owner

# Appendix. Reference Documents

| Reference # | Document Title |
| --- | --- |
| 1 | Enhanced Memory SPSS |
| 2 | FBMP SPSS (Feature Based Message Protocol SPSS spec). |
| 3 | SYNC+ account PRD for phase 1&2 |
| 4 | Face ID spec. |
| 5 | Master reset SPSS and PRD |
| 6 | PaaK Feature SPSS and PRD |
| 7 | Place holder for Ford account SDK API documents |
| 8 | Place holder for Baidu account SDK API documents |
| 9 | Events tracking, statistics and reports requirements list |
| 10 | DUEROS_CVPP_Smart Greeting_PRD |